| # FortnaPlus Control Scanner Control Configuration |  |
| --- | --- |
|  | Document Title |
| Document Revision: 1.5 |  |
| Date: 16-Mar-2022 |  |

Revision; Date; Author; Revision Summary. 1.1; 10-Oct-2013; B C Conklin; Added Transfer description and updates. 1.2; 09-Sep-2020; T R Kremser; Updates and general refresh. 1.3; 29-Oct-2020; T R Kremser; Update the CONCAT feature.. 1.4; 16-Nov-2020; T R Kremser; Additional improvements to CONCAT feature.. 1.5; 16-Mar-2022; T R Kremser; Add note regarding SrtTrack and SrtDevice table sizes..

Contents

1.	General Description	4

2.	Receiving Scans	4

2.1.	Machine Table	5

2.1.1.	Serial Connections	6

2.1.2.	TCP Connections	7

2.2.	MsgMap Table	8

3.	Scan Zone Configuration	9

3.1.	Scan Zone Error IO Configuration	13

3.1.1.	Additional Details for the CommErrorMsg Field	15

4.	Scan Device Configuration	16

4.1.	Scan Device Error IO Configuration	20

4.2.	Sorter NoScan Processing	22

5.	Transfer Scan Processing	23

6.	Debugging Scanner Connections	26

7.	Example 1 – Single Scanner on a Sorter	26

8.	Example 2 – Single Scanner on a Transfer	28

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# General Description

The Scanner Control module in FortnaPlus Control (FPC) works with transfers and sorters using special logic to process scanner and scale inputs.  The module can work with many possible scanner configurations.  It can be used to parse multi-part scans, or to concatenate multiple individual scans into a multi-part scan based on delimiter characters.  It can be used to compare inputs from redundant scanners.

The configuration is organized using a scan zone, which contains scan devices.  Zones and devices have separate error handling capabilities.

The module understands certain special types of scanner input data.  Scale data, and tote IDs, for instance, are received just like scan data, but the data can be directed to special fields within messages as is appropriate.

The following diagram depicts two of the most common configurations as two scan zones.  Zone 1 is composed of a scanner and a scale.  Zone 2 contains two scanners that might read a barcode on the side of a container regardless of orientation, optionally checking that if two scans are received, they are both the same.

Scale

Scanner

Zone 1

Scan 2

Scan 3

Zone 2

# Receiving Scans

The configuration for receiving scanner and scale data is managed in two parts.  The Machine table defines the connection to the actual device.  The software behind it manages the connection, restoring it if it is lost.  The MsgMap table directs the received data to the ScnScanZone or ScnScanDevice tables where the data is acted upon and processed.  This section describes the Machine and MsgMap tables.

  - Machine Table

The Machine table is used to define connections to external devices, such as scanners and scales.  Many connection types are supported.

| Machine Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Machine_Name | String | A unique name for the entry. For TCP client connections, the name must match an entry in the /etc/hosts file to define the related IP address. |
| Status | String | A message filled in by the software. |
| Error | String | A message filled in by the software if an error occurs. |
| Machine_Index | Int | Not used. |
| Machine_Pid | Int | Process ID number assigned by the operating system for the connection. You can set this to -1 to force the connection to be dropped. It will automatically attempt to reconnect. |
| Connection_Type | PopUp | Define the type of connection by choosing from the list. Use CONNECT_SERIAL for serial port and RocketPort connections. Use CONNECT_TCPCLIENT for TCP client-style connections. Use TCPSERV_LISTEN and TCPSERV_RDWR for TCP server-style connections. These are the most common. Others are possible. |
| Communication_Protocol | PopUp | Define how the incoming text should be interpreted by choosing from the list. The most common choices for scanners are AlphaNum for serial connections and STXETX for TCP connections. Both filter the received data to remove invalid characters according to the definition in table AsciiMnu, but do not manipulate the text in any other way. |
| Machine_Port | String | For serial connections, enter the name of the QNX device file, like /dev/ser1 or /dev/ser-rckt1. For TCP connections, this will be a numeric port number like 9301 that the scanner expects to use for its messages. |
| Transmit_Retry | Int | Use a value of 0. |
| Offline | Y/N | Set by the software to indicate the current state of the connection. |
| Process | PopUp | Use the CTRL process for scanner connections. |
| UseDB | Y/N | Not applicable for scanner connections. |
| IgnoreDupes | Y/N | Not applicable for scanner connections. |
| NoCircBuff | Y/N | Not applicable for scanner connections. |
| CircSize | Int | Not applicable for scanner connections. |
| Trig_Size | Int | For serial connections only. Set this if you want the scan to be processed when the specified number of characters has been received. |
| Trig_InterChar | Int | For serial connections only. Once the first character of a message has been received, this value determines how long the software will wait until the next character arrives before declaring the message “complete” and processing the data. The value is in milliseconds. The default is 10 milliseconds. In the QNX4 versions, this value is in tenths of a second. |
| Trig_Timeout | Int | This field is not used for scanner connections. (It is used only for ActiveMQ connections). In QNX4 versions, for serial connections only, it represents the total time elapsed between arrival of the first and last characters of a message. |
| Trig_MsgEndChar | PopUp | Choose the character that indicates the end of a message. Defaults to ETX (Ctrl-C, or 0x03). For serial connections, use Ctrl-J (Line Feed). The message is processed when this character is received. |
| Log File | PopUp | Rarely used. Defines a separate log file for this machine connection. |
| NumCbItems | Int | Not applicable for scanner connections. |
| LastMsgAttempts | Int | Not applicable for scanner connections. |
| MaxAttempts | Int | Not applicable for scanner connections. |
| StripNulls | Int | Not applicable for scanner connections. |
| TimerId | Int | Not applicable for scanner connections. |
| RetryCount | Int | Not applicable for scanner connections. |
| MaxRetries | Int | Not applicable for scanner connections. |
| FortnaMachine | Y/N | Not applicable for scanner connections. |
| NeedACK | Int | Not applicable for scanner connections. |
| LogLevel | PopUp | Choose a level to determine how much detail is written to the CTRL log file. Level 2 is usually a good choice. |
| Username | String | Not applicable for scanner connections. |
| Password | String | Not applicable for scanner connections. |
| CloseDelay(ms) | Int | Not applicable for scanner connections. |
| OpenTimeout(ms) | Int | Not applicable for scanner connections. |
| DontSetNeedAck | Y/N | Not applicable for scanner connections. |
| pCBuff | Blob | Not applicable for scanner connections. |
| pCBuffLen | Int | Not applicable for scanner connections. |
| pCBMaxLen | Int | Not applicable for scanner connections. |
| pCTimeout | Int | Not applicable for scanner connections. |
| pCharSet | Blob | Maintained by the software. A list of valid characters, as defined through the AsciiMnu table. |
| pSendSequence | Int | Not applicable for scanner connections. |
| pRecvSequence | Int | Not applicable for scanner connections. |

    - Serial Connections

Serial devices use individual hardware ports supporting the RS232 or RS422 standards.  They may be “native” QNX ports like /dev/ser1, or ports generated by add-on hardware like a RocketPort.  Beckhoff’s modular approach allows adding serial port hardware in pairs.

      - Setting Baud Rates

The data rates for serial ports are configured at the operating system level, not within FPC.  There are slight differences between QNX versions in the names of built-in serial ports.  Add-on ports, such as RocketPort cards also define serial port devices (in /dev) with different names depending on the model used.

RocketPort Universal devices are named like /dev/ser-rckt1 in QNX6, and /dev/rkt1 in QNX4.

RocketPort Infinity devices (QNX6 only) are named like /dev/ser-inf1.

Built-in QNX serial ports are named like /dev/ser1 in all QNX versions.  Beckhoff serial ports follow this naming convention.

The /etc/rc.d/rc.local file defines how QNX will start (in QNX4, it is the /etc/config/sysinit file).  Search for ROCKETPORT or SERIALPORT near the top of the file.  Follow the pattern to define a default baud rate for each AC and individual baud rates per port, if needed.  The FORTNA_ROCKETPORT_BAUD variable defines a default baud rate for all RocketPort ports.  FORTNA_SERIALPORT_OVERRIDE is used to define individual port settings, and can be used to set other port parameters, if needed, as well.  The stty command is used to issue the parameter settings to QNX.  It can also be used to check the settings of a port.  For example: “stty < /dev/ser1” will show all the port settings for the first built-in serial port.  Any of the parameters can be adjusted by adding them to the command line in the form shown in the stty output.  For example, to set the baud rate to 19200 on /dev/ser1:  “stty baud=19200 < /dev/ser1”

Note that in the example FORTNA_SERIALPORT_OVERRIDE given in the rc.local file, the command has a trailing semicolon.  This is to remind you that if multiple overrides are needed, the stty commands must be separated by semicolons:

FORTNA_SERIALPORT_OVERRIDE=”stty baud=9600 < /dev/ser1; stty baud=57600 < /dev/ser2;”

    - TCP Connections

The trend for scanner devices is toward TCP connections.  TCP connections are often easier to manage, as they use standardized cables, and leverage ubiquitous network infrastructure to support very long runs..  The devices themselves may also support web interfaces that make diagnosis of problems and remote support much easier.  Configuration for Connection_Type CONNECT_TCPCLIENT connections at the QNX OS level only requires definition of a name-to-IP-address cross-reference in the /etc/hosts file.  Choose a name for the device and enter it in /etc/hosts with the IP address like this:

192.168.10.51	SCANNER1

Then create the Machine table entry using SCANNER1 as the Machine_Name.

FPC may act as either a client or server when connecting to a TCP scanner device.  If FPC is the client, a single entry in the Machine table with Connection_Type CONNECT_TCPCLIENT is needed, along with an entry in /etc/hosts, as described above.  If FPC is the server, two entries are needed in table Machine for Connection_Types TCPSERV_LISTEN and TCPSERV_RDWR.  TCPSERV_LISTEN listens for the scanner’s connection request, and initiates the actual connection using the TCPSERV_RDWR line.  No /etc/hosts entry is needed in this case, as the scanner (client, in this case) will contact FPC on the defined Machine_Port.

Both types of connections require the Machine_Port to be defined to the port number the scanner will use to send the scan data.

  - MsgMap Table

The MsgMap table creates the connection between the device as defined in the Machine table, and the tables used by the scanner control software (ScnScanZone and ScnScanDevice).  MsgMap defines where the incoming data will be written.

| MsgMap Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Message_Name | String | Enter a unique name for the map entry. |
| Machine_Name | PopUp | Choose the Machine table connection that this map entry will act against. |
| Recv_Msg_Type | String | Set to “ALL”. The Msg_Type fields make more sense in the context of named messages exchanged with CommCore. Here, we want to process all received data from the scanner. |
| Send_Msg_Type | String | Leave blank. We don’t send messages to scanners. |
| Ack_Msg_Type | String | Leave blank. We don’t acknowledge scanner messages. |
| Menu_Name | PopUp | Choose the table to which the scans will be delivered. The next section of this document explains how to choose between the ScnScanZone and ScnScanDevice tables. |
| Low_Cursor | Int | Enter the column number (count from 1) in the destination table of the column that will receive the scan data. These are the RcvData fields. For ScnScanDevice, use 2. For ScnScanZone, use 4, 6, 8, or 10. See the next section. |
| High_Cursor | Int | Enter the same value chosen for Low_Cursor. Low_Cursor and High_Cursor define the range of fields to be populated. Scanners always deliver one field of data. |
| Stat_Cursor | Int | Enter the column number of the RxStat column in the destination table that goes with the RcvData column chosen. For ScnScanDevice, use 3. For ScnScanZone, use 5, 7, 9, or 11. |
| Last_Record | Int | Not used in this context. |
| Low_Head | Int | Not used in this context. |
| High_Head | Int | Not used in this context. |
| Key_Cursor | Int | Not used in this context. |
| Id | Int | Used internally. |
| LastId | Int | Used internally. |
| Record | Int | Enter the row number in the destination table that will receive the scan data. The Low_Cursor/High_Cursor column number and this value define a single field in the destination table where the scan message will be delivered. |
| Delimiter | PopUp | Used only with blob field types to indicate the end of the data. |
| FieldLength | Int | Used only with blob field types to indicate the length of the expected data. |
| AckLen | Int | Used to define the added length of an Ack response in the HostAck and HostEcho protocols only. |
| CheckFile | Y/N | Used only by the Disk connection type to generate additional logging. |
| pMsgType | Int | Used internally for QNX node-to-node messages only. |

# Scan Zone Configuration

The scan zone configuration table ScnScanZone is the top-level configuration table that defines a scanner or group of scanners that act together logically to gather data at a particular point on the conveyor path.

Scans are usually received directly to the subordinate table ScnScanDevice, described in section 4, but if a scanner delivers a compound scan (multiple scan results separated by a delimiter character) the scans can be received directly to ScnScanZone, then parsed into ScnScanDevice.

A scan zone may contain multiple scan devices.  Some examples:

- A scanner and a scale.

- Two scanners that are combined to look for two copies of a single label, such as a UPC code on the left and right side of a container.

- Redundant scanners scanning the same barcode to provide backup and increase the read rate.

- A Print and Apply scan tunnel with four scanners for shipping label, carton contents label, national retailer label, and SKU label.

NOTE:  To activate the scanner sontrol processing software, you must add the Batches entry for “ScnApp CTRL”.

Based on the zone’s configuration, the software will determine when all of the scans for a zone have been received and will set the “ProcFlag” in the tracking table to indicate to the application that all scans have been received for the zone so processing can commence.

NOTE:  Versions prior to V11.47.0 have a bug that requires the TrackingTable and DeviceTrkTable pair chosen to be defined with the same number of records for Print and Apply applications.  If DeviceTrkTable is larger than TrackingTable, the symptom is LRM messages will stop being generated when DeviceTrkTable row numbers greater than the last row of TrackingTable are used.  This is not a problem for Sorter applications.

| ScnScanZone Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the scan zone. |
| Desc | String | Optional description. |
| pStatusMsg | String | Set by the software to show the current status including error messages. |
| RcvDataA | String | Use table MsgMap to receive data to the RcvData fields here if compound scans must be parsed or read conflict processing must be performed on redundant scanners. |
| RxStatA | PopUp | The receive status field for RcvDataA. Set by the protocol selected in the Machine table. |
| RcvDataB | String | A field to receive the second scan, if any. |
| RxStatB | PopUp | The receive status field for RcvDataB. Set by the protocol selected in the Machine table. |
| RcvDataC | String | A field to receive the third scan, if any. |
| RxStatC | PopUp | The receive status field for RcvDataC. Set by the protocol selected in the Machine table. |
| RcvDataD | String | A field to receive the fourth scan, if any. |
| RxStatD | PopUp | The receive status field for RcvDataD. Set by the protocol selected in the Machine table. |
| Delimiter | String | Set a single character for parsing compound scans, or one or more characters for concatenating individual scans. |
| LogLevel | PopUp | Choose the logging level. Level 0 records only error messages. Each higher level includes the lower level’s messages, plus additional detail. Level 1 records most of the relevant scan-oriented events. Level 2 has some additional detail and is a good choice. Higher levels may produce excessive logging and are useful for troubleshooting for short periods only. |
| ScanZoneID | String | Enter the agreed-upon string that represents this scan zone in the CommCore system – the “Decision Point”. The string is included in the sScanner field of CRR, CSM, CSU and DCM messages. |
| CompareScans | Y/N | Set to Y if the software must be used to detect read conflicts. Redundant scanners would be defined to use the RcvData fields, and the results would be compared with this setting. In most cases, the scanners themselves are configured to detect and report read conflicts, so this feature is not needed. |
| ConcatScans | Int | If multiple individual scans must be concatenated to a single delimited string, set this field to a value between 1 and 4. The value defines how many scans are expected. Use the Scan Types “Concat A” through “Concat D” in ScnScanDevice_ScanType to denote the individual scans to be concatenated and define their order (A, B, C, D). The scans will be processed when the update point defined in SrtScanBoss_ConcatScansUpdatePt is reached. The scans will be concatenated with the Delimiter inserted between them and the result will be delivered to the tracking table ConfirmScan field. Be sure to allow enough room between the last scan point and the update point so the longest box can tail out of the scan point before reaching the update point so the NoScan processing will work correctly. Each element of the scan will be processed for NoRead and NoScan events before the concatenation is performed. Setting ConcatScans to 0 disables this feature. |
| TrackingTable | PopUp | Choose a tracking table to store information about containers that are in process on sorters or transfers. For sorters, choose a separate table for each sorter controlled by the AC. Use tables SrtTrack[1-5]. For transfers, many transfers can share the same tracking table. Use table XfrTrack. |
| DeviceTrkTable | PopUp | Choose a device tracking table to store scan information for containers as they are processed on sorters or transfers. For sorters, choose a separate table for each sorter controlled by the AC. Use tables SrtDevice[1-5]. For transfers, many transfers can share the same tracking table. Use table XfrDevice. |
| PrinterTrkTable | PopUp | Choose a printer tracking table for print and apply applications. Use tables PrtTrkTable[1-5]. Separate tables are only needed if an AC is configured with more than one print and apply line. |
| PrinterErrTable | PopUp | Choose a printer error table for print and apply applications. Use tables PrtTrkErrTbl[1-5]. Separate tables are only needed if an AC is configured with more than one print and apply line. An update point type of “Gather Errors” is required. |
| CTRLProcess | PopUp | Used for special applications (like Print and Apply). The function selected is called to process special device types. The function will be called for each scan event. |
| ScanAlgorithm | PopUp | Used to activate special scan processing algorithms for specific customers: - MSC Ship – Scan data is received as three delimited fields. The fields are stored as a CartonID and a ToteID (Carrier Tracking Number) using the standard parsing and device processing configurations. If the first character of the CartonID is “M”, the container will not have a Carrier Tracking Number. If the CartonID does not start with “M”, a Carrier Tracking Number is required, and it is blank-filled to 18 characters. The two strings are then sent to CommCore as a single string (Carton Tracking Number – 18, Carton ID). - MSC PnA – Scan data is received as two delimited fields. The fields are stored as a CartonID and a ToteID (box size) using standard parsing and device processing configurations. If the CartonID has 6 characters, the string is for a tote and the “box size” is not needed. If the CartonID is not 6 characters long, the box size is checked against the “Box Type Control” table to determine if the box is an old- or new-world box type. New-world box types must have a CartonID. - MSC OIVP – Scan data is received in two fields. If the scan data length matches the number of characters defined in field Alg_P1, (6 characters) the data is a Tote Scan. The second field is the box size, and also indicates if the box is an old- or new-world box type. An error is returned if both fields are NoRead. - McKesson PnA – Used with McKesson PnA verification process. - McKesson Xfr Induct – Used for McKesson ESM message processing. The message includes both scan data and a time stamp. - Xfr Scan Tote – If selected, this algorithm expects two scan fields, and treats each scan separately and returns the barcode or errors for each. If this option is not selected, the default Transfer Carton Scan and Tote Scan logic will send both barcodes to CommCore (however if either barcode has an error, the good scan is sent as the Carton ID). |
| Alg_P1 | Int | This parameter field is used by certain customer-specific algorithms that can be chosen in the ScanAlgorithm field: - MSC Ship – Set to 18 to indicate the number of characters required for the Carrier Tracking Number. - MSC PnA – Set to 6 to indicate the number of characters required to identify a tote. |
| XfrZoneID | String | Used by transfer print and apply applications to link a verify scan zone to a prior printer applicator zone. |
| PrintLine | PopUp | Used to link a scan zone to a print apply line to check for pass-through mode. If a scan zone entry is selected here, and the print apply line is set to “Pass Through Mode” and PassThruCRR = N in table PrintApplyLine, a CRR message will not be sent. |
| pRecordProcessed | Int | Used by the software to indicate the record was processed when a reset is being checked. |
| pTrnDataRcv | Int | Used by the software for NoScan event processing. |
| pTrnTrkRecord | Int | Used by the software to refer to the tracking table row currently being processed for transfers. The field is set when a transfer scan event occurs. The field is cleared when the required number of scans have been processed. The field is also cleared if a NoScan event occurs. |
| pTrnLastTrk | Int | Used by the software to refer to the tracking table row after all scans have been received. The field is cleared when the presence photoeye is activated for a transfer. The field is also cleared if a NoScan event occurs. |

  - Scan Zone Error IO Configuration

The ScnZoneErrIO table defines IO control and history gathering for groups of scanners in scan zone events.  Note that the history gathering configuration is best handled by using the history configuration wizard.  The IO control aspects must be configured in the table manually.

| ScnZoneErrIO Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the scan zone error event. |
| ScanZone | PopUp | Choose the scan zone associated with this entry. |
| ErrorType | PopUp | Choose the event type to be captured by this entry: - No Scan – NoScan event. No scan data could be associated with the container. - Good Read – Good read event. - Bad Read – The system detected an error with the scan data. - No Read – The scanner sent the NoRead string to the system. - Conflict – The scanner sent the ReadConflict string to the system. - No Data – The “No Data” event is defined by the CommCore server and may be captured by setting the CommErrorMsg column to the string received from CommCore. - Any Type – This event type will capture error types NoScan, BadRead, Conflict, NoRead, but it will not capture CommCore errors (NoData). - Any Device – This event type will capture other device errors, but it will not capture CommCore errors (NoData). |
| UpdateHistory | Y/N | Set to Y to update the history record. The history may be processed by the application software either in the CTRL process or in the process that is triggered by the UpdFlag in the tracking record. Set to N to disable history updates for this record. |
| HRRecord | PopUp | Select the HistConfig table record that will be updated by this event. |
| AutoResetCons | Y/N | Set to Y to reset the consecutive event count when a good scan event occurs, and the count has reached the MaximumErrors value. Set to N to disable automatic consecutive event count reset – the count must be reset manually if this option is selected. |
| MaximumErrors | Int | Set the number of errors that must occur before the IO configuration is processed. A value of 0 will disable IO processing. |
| ErrorIO | PopUp | Choose an output that will be activated when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO. The inverse action will occur when the event clears. |
| ErrorBIT | Int | If Enable is chosen for ErrorACTION, define the bit number to use. |
| ErrorSignal | PopUp | Choose an IO signal that will be turned ON when the MaximumErrors value is reached. The signal is usually configured as a memory bit and is also referenced in the Errors table. This is processed in the exact same way as ErrorIO, but always uses FIRE for the action. |
| ResetCons | Y/N | Set this field to Y to manually reset the count of consecutive errors found in pConsecErrors. It will automatically revert to N when the action is complete. |
| ResetConsSignal | PopUp | Choose an IO signal that will reset the count of consecutive errors found in pConsecErrors. The reset will occur when the signal is ON. |
| ResetSeconds | Int | Enter the number of seconds to wait before automatically clearing the error and resetting the count of consecutive errors found in pConsecErrors. Once the MaximumErrors value has been reached, the timer will start. Once the time has elapsed, the error will be cleared and pConsecErrors will be reset. This value and the MaximumErrors value must be greater than zero to enable this feature. |
| CommErrorMsg | String | Used with ErrorType “No Data”. The sStatus field in the CRR message response received from CommCore indicates the error status. If an error is detected, the string found in the sStatusMsg is compared to this field. If a match is found, the pConsecErrors field is incremented. If MaximumErrors is reached, the error processing feature will be activated. If ScnScanZone_ScanAlgorithm is configured to use an application-specific algorithm, special strings must be sent in this field: - ApplGoodScan – good scan, no error detected - ApplNoReadScan – scan error detected - Destination Found – indicates destination was found - Destination NOT Found – indicates no destination was found - ALL – special case representing all error types See section 3.1.1 at the end of this table for more detail. |
| AppSorter | PopUp | For sorter control applications, select the SrtAppControl record that contains the motor to stop when MaximumErrors is reached. |
| PrintLine | PopUp | For print and apply configurations, select the PrintApplyLine record that contains the motor to stop when MaximumErrors is reached. |
| Xfer | PopUp | For transfer control applications, select the XfRouteBoss record that contains the motor to stop when MaximumErrors is reached. |
| ErrorLane | PopUp | For sorter control applications where a container is to be assigned to a special lane when an error occurs, select a sorter lane from the Outpoints table. This is typically used only for print and apply applications based on a sorter that has an error correction lane as part of the individual print and apply station. |
| pConsecErrors | Int | Set by the software. The current count of consecutive error events. |
| pResetSecsLeft | Int | The number of CTRL process scans left before auto-reset occurs. Used with the ResetSeconds field. |

    - Additional Details for the CommErrorMsg Field

The ScnZoneErrIO_CommErrorMsg field has a number of special features that can be activated by defining records using the ErrorType “No Data” to trap specific error message strings returned by CommCore in the CRR message response sStatusMsg field.

      - “Destination Found” – Special CRR sStatusMsg literal entry that can be detected indicating there was no routing error.  This can be used to light a beacon for 1 second to indicate valid routing was received for a container (this special case does not increment the sequential errors count).  This is not implemented for LRM messages.

      - “Destination NOT Found” – Special CRR sStatusMsg literal entry that can be detected indicating a routing error has occurred.  This can be used to light a beacon for 1 second to indicate valid routing was NOT received for a container (this special case does not increment the sequential errors count).  This is not implemented for LRM messages.

      - “ALL” – Special CRR sStatusMsg literal entry that can be detected to increment the sequential error count for any error received from CommCore.  This does NOT include NOREAD, NOSCAN, SCANCONFLICT and BADREAD errors, which are generated locally.

      - Routing module errors – Any string that is returned from CommCore in the sStatusMsg field to indicate an error condition can be trapped by defining an entry in table ScnZoneErrIO with the CommErrorMsg field set to that string.  Multiple error messages can be combined in a single entry, if desired, by entering them with a “pipe” (“|”) delimiter.  Some examples: “No Destination defined”, “No Host Info”, “Unknown Container”, “No Destinations Found”.

      - Application-specific scan errors – Several application-specific algorithms are available by selecting from the list in table ScnScanZone, field ScanAlgorithm.  See the definitions in section 3.  Entries in ScnZoneErrIO can be used to act on the custom algorithm results.  The ErrorType “No Data” must be selected, and the literal strings described here must be entered in CommErrorMsg.
“ApplGoodScan” – Application-specific error entry that is updated when a good scan is processed and the message reports no error.
“ApplNoReadScan” – Application-specific error entry that is updated when a scan error is detected.

# Scan Device Configuration

The ScnScanDevice table is used to configure each scan device in a scan zone.  It provides a place for the incoming scan data and has fields to track the scan as it is processed.  It is where NoRead and ReadConflict strings are defined, and has features for parsing received strings, and checking weights (for scales).  It is used to tie scan data to the transfer and sorter logic.  In short, it is a pretty important table.

| ScnScanDevice Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the scan device. |
| RcvData | String | A place for the raw incoming scan string. The protocol defined in the Machine table for the device links the incoming data to this field as defined in the MsgMap table. |
| RxStat | PopUp | The status of the incoming scan message. Set to state Recvd when a scan arrives by the protocol defined in the Machine table. The scan processing module will reset this to state Empty after the data has been processed. |
| ParsedData | String | A place to contain the parsed string after the scan processing module has applied its rules. This is a good place to observe the string as it will be used in messages and by other modules. Scans received and parsed in the ScnScanZone table are delivered here, as well. If a device tracking table is selected in the ScnScanZone table, this string will be copied to the device tracking record, as well. |
| ParsedStatus | PopUp | The status of the parsed message, such as Good Scan or No Read. If a device tracking table is selected in ScnScanZone, this status is copied to the device tracking record. |
| pStatusMsg | String | A status message for the current scan from the scan processing module. Also shows configuration errors. |
| ScanZone | PopUp | Select the scan zone associated with this device from table ScnScanZone. |
| ScanType | PopUp | Select a scan type for the device. The scan type defines how the data will be handled. There are many options: - Scan – Basic scanner. - Ship Scan – Designates a ship scanner, same as type Scan. - Tote Scan – Designates a Tote Scanner. Tote Scan data is stored in a separate Level II message field if an additional scan type, such as a Scan is defined for the same zone. This allows applications with dual scanners for the shipping label and tote label to generate a single Level II transaction. - Scale – Special device type for scales. The received value is checked against the MinWeight and MaxWeight columns. - If a NoRead string or a ReadConflict string is received for a scale, the weight value sent in the CRR is converted to “ “ (a single space character). - Scan Confirm – The scanner is configured as a confirmation scanner. Scans using this scan type generate a divert confirm (DCM) message. - RFID Scan – Designates the scanner as a RFID scanner. Used with Print Apply. - SKU Scan – Designates the scanner as a SKU scanner. Used with Print Apply. - LPN Scan – Designates the scanner as an LPN scanner. Used with Print Apply. - Verify RF – Designates the scanner as an RFID verify scanner. Used with Print Apply. - Verify SKU – Designates the scanner as a SKU verify scanner. Used with Print Apply. - Verify LPN – Designates the scanner as an LPN verify scanner. Used with Print Apply. - Verify Top – Designates the scanner as a top verify scanner. Used with Print Apply. - Verify Left – Designates the scanner as a left verify scanner. Used with Print Apply. - Verify Right – Designates the scanner as a right verify scanner. Used with Print Apply. - Verify Front – Designates the scanner as a front verify scanner. Used with Print Apply. - Scan1 – Designates the scanner as the first scanner in a scanner group. The combined scan is sent as the sContainerNum in a CRR message. If multiple scan devices are configured for a single scan zone, Scan1 will have priority over the other scanners. - Scan2 – Designates the scanner as the second scanner in a scanner group. - Scan3 – Designates the scanner as the third scanner in a scanner group. - Scan4 – Designates the scanner as the fourth scanner in a scanner group. - Tote1 – Designates the scanner as the first scanner in a scanner group. The combined scan is sent as a tote scan (sToteID) in the CRR message. If multiple scan devices are configured for a single scan zone, Tote1 will have priority over the other scanners. - Tote2 – Designates the scanner as the second scanner in a scanner group. - Tote3 – Designates the scanner as the third scanner in a scanner group. - Tote4 – Designates the scanner as the fourth scanner in a scanner group. - SKU 1 – Designates the scanner as the first scanner in a scanner group. The combined scan is stored as a SKU scan (sSKU) in the LRM message for Print Apply. If multiple scan devices are configured for a single scan zone, SKU 1 will have priority over the other scanners. - SKU 2 – Designates the scanner as the second scanner in a scanner group. - SKU 3 – Designates the scanner as the third scanner in a scanner group. - SKU 4 – Designates the scanner as the fourth scanner in a scanner group. - SpcGoodRead1 – Designates a special device type that may be configured to assign a lane whenever a Good Read is received by the scanner. The lane assigned is configured in the SrtScanBoss table, column SpcLane. The feature is used to pre-assign a lane for a two-sided shoe sorter based on which scanner receives a Good Read. Example: If one side of a sorter handles cartons and the other side handles totes, and if the tote side of the sorter requires the shoes to be puller over, select the tote scanner as SpcGoodRead1, select one of the tote lanes in column SpcLane and the shoes will always be pulled over when a Good Scan occurs on the tote scanner. - SpcGoodRead2 – Designates a special device type that may be configured to assign a lane whenever a No Read is received by the SpcGoodRead2 scanner, and Good Read is received by the SpcGoodRead1 scanner. When a scan zone is configured with both a SpcGoodRead1 and a SpcGoodRead2, the scan zone will combine the data from the two scanners in the scanner statistics. The lane assigned is configured in the SrtScanBoss table, column SpcLane. - Sim Scan – Designates the device as a simulated scanner. Scan data is input from the simulation configuration. - Sim Scale – Designates the device as a simulated scale. Scan data is input from the simulation configuration. - UPC 1 – Designates a device that reads UPC codes. - UPC 2 – Designates a device that reads UPC codes. - UPC 3 – Designates a device that reads UPC codes. - UPC 4 – Designates a device that reads UPC codes. - UPC 5 – Designates a device that reads UPC codes. - UPC 6 – Designates a device that reads UPC codes. - UPC 7 – Designates a device that reads UPC codes. - UPC 8 – Designates a device that reads UPC codes. - UPC 9 – Designates a device that reads UPC codes. - UPC 10 – Designates a device that reads UPC codes. - UPC 11 – Designates a device that reads UPC codes. - UPC 12 – Designates a device that reads UPC codes. - UPC 13 – Designates a device that reads UPC codes. - UPC 14 – Designates a device that reads UPC codes. - UPC 15 – Designates a device that reads UPC codes. - UPC 16 – Designates a device that reads UPC codes. - UPC 17 – Designates a device that reads UPC codes. - UPC 18 – Designates a device that reads UPC codes. - UPC 19 – Designates a device that reads UPC codes. - UPC 20 – Designates a device that reads UPC codes. - Carton Length – Designates a device that reads carton length. Used with Print Apply. A dimension check can be configured in the Print Apply HeightWidth table and the dimensional check is completed when the scan data is received. - Carton Width – Designates a device that reads the carton width. Used with Print Apply. - Carton Height – Designates a device that reads the carton height. Used with Print Apply. - Scan Length – Designates a device that reads the carton length. Used with Sorter. - Scan Width – Designates a device that reads the carton width. Used with sorters. - Scan Height – Designates a device that reads the carton height. Used with sorters. - Ship 1 – Used with transfers to capture a scan event. - Ship 2 – Used with transfers to capture a scan event. - Ship 3 – Used with transfers to capture a scan event. - Ship 4 – Used with transfers to capture a scan event. - Pack 1 – Used with transfers to capture a scan event. The scan is stored in XfRouteBoss_pLastPack, and can be used to send the data in one of the Param[1-5] columns of the CRR message. The column is chosen using the XfrParamConfig_Parameter field. - Pack 2 – Used with transfers to capture a scan event. - Pack 3 – Used with transfers to capture a scan event. - Pack 4 – Used with transfers to capture a scan event. - Concat A – Used with the ScnScanZone_ConcatScans feature for the first scan in the concatenation. - Concat B – The second scan in a concatenated scan. - Concat C – The third scan in a concatenated scan. - Concat D – The fourth scan in a concatenated scan. |
| Field | PopUp | For use in applications where compound scans are received into the ScnScanZone table. The Field is used to designate the device entries in ScnScanDevice that will receive each element as the compound scan is parsed. Example: A camera scanner reads the carton ID and up to 20 SKU codes. The MsgMap configuration directs the compound scan to ScnScanZone_RcvDataA. ScnScanDevice is configured with 21 entries – one for the carton ID, and one for each of the 20 SKU codes. |
| UpdatePt | PopUp | If this scanner is associated with a shifter, select the appropriate update point. |
| NoReadString | String | Enter the string that the scanner will return if it cannot read a barcode. The string is compared to the ParsedData field and will generate a NoRead event on an exact match. |
| ReadConflictStr | String | Enter the string that the scanner will return if it detects a read conflict. The string is compared to the ParsedData field and will generate a ReadConflict event on an exact match. The meaning of a read conflict depends on the scanner type and configuration. Two line scanners might be arranged to read opposite sides of a tote, expecting to read the same barcode and generating a read conflict if they don’t agree. A camera scanner might be set to read all the codes it can “see”. It could be configured to return a read conflict if unexpected codes are read. Common causes of read conflict errors include extraneous labels on a carton or cartons side by side on the conveyor. |
| StartChar | Int | Enter the character position within the received string where the data of interest starts. Used to discard unwanted leading characters. A value of 0 or 1 will start with the first character. |
| QtyChars | Int | Enter the maximum number of characters to be extracted from the received string, starting at the StartChar position. If set to 0, all characters to the end of the string are extracted. Works with the ChkLength field. |
| ChkLength | Y/N | Set to Y to check the parsed string is at least QtyChars long. If the parsed string, after adjusting for StartChar, is shorter that QtyChars, the ParsedStatus will be set to BadRead to indicate a length error. If the parsed string is longer than QtyChars, no error is generated. Set to N to disable length checking. |
| MinWeight | Double | Used to detect scale errors. Set to the minimum allowed weight. If the value is below MinWeight, the ParsedStatus will be set to MinWeight and the error will be recorded in the scan statistics as a BadRead. Range weight errors are processed by CommCore. |
| MaxWeight | Double | Used to detect scale errors. Set to the maximum allowed weight. If the value is above MaxWeight, the ParsedStatus will be set to MaxWeight and the error will be recorded in the scan statistics as a BadRead. |
| pLeadEdgeNdx | Int | Used internally to indicate a leading-edge event. |
| pLeadEdgeShifter | Int | Used internally to indicate a leading-edge event. |
| pTrailEdgeNdx | Int | Used internally to indicate a trailing-edge event. |
| pTrailEdgeShifter | Int | Used internally to indicate a trailing-edge event. |
| pTrnDataRcv | Int | Used internally to record a scan processing event for transfers. |
| pSrtDataRcv | Int | Used internally to record a scan processing event for shifters. |

  - Scan Device Error IO Configuration

The ScnDeviceErrIO table defines IO control and history gathering for individual scan device events.  Note that the history gathering configuration is best handled by using the history configuration wizard.  The IO control aspects must be configured in the table manually.

| ScnDeviceErrIO Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the scan device error event. |
| ScanDevice | PopUp | Choose the scan device associated with this entry. |
| ErrorType | PopUp | Choose the event type to be captured by this entry: - No Scan – NoScan event. No scan data could be associated with the container. - Good Read – Good Read event. - Bad Read – The system detected an error with the scan data. - No Read – The scanner sent the NoRead string to the system. - Conflict – The scanner sent the ReadConflict string to the system. - No Data – The “No Data” event is defined by the CommCore server and may be captured by setting the CommErrorMsg column to the string received from CommCore. - Any Type – The event will capture other error types: NoScan, BadRead, Conflict, NoRead, but it will not capture CommCore errors (NoData) - Any Device – The event will capture other device errors, but it will not capture CommCore errors (NoData). |
| UpdateHistory | Y/N | Set to Y to update the history record. The history may be processed by the application software either in the CTRL process or in the process that is triggered by the UpdFlag in the tracking record. Set to N to disable history updates for this record. |
| HRRecord | PopUp | Select the HistConfig table record that will be updated by this event. |
| AutoResetCons | Y/N | Set to Y to reset the consecutive event count when a good scan event occurs, and the count has reached the MaximumErrors value. Set to N to disable automatic consecutive event count reset – the count must be reset manually if this option is selected. |
| MaximumErrors | Int | Set the number of errors that must occur before the IO configuration is processed. A value of 0 will disable IO processing. |
| ErrorIO | PopUp | Choose an output that will be activated when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO. The inverse action will occur when the event clears. |
| ErrorBIT | Int | If Enable is chosen for ErrorACTION, define the bit number to use. |
| ErrorSignal | PopUp | Choose an IO signal that will be turned ON when the MaximumErrors value is reached. The signal is usually configured as a memory bit and is also referenced in the Errors table. This is processed in the exact same way as ErrorIO, but always uses FIRE for the action. |
| ResetCons | Y/N | Set this field to Y to manually reset the count of consecutive errors found in pConsecErrors. It will automatically revert to N when the action is complete. |
| ResetConsSignal | PopUp | Choose an IO signal that will reset the count of consecutive errors found in pConsecErrors. The reset will occur when the signal is ON. |
| ResetSeconds | Int | Enter the number of seconds to wait before automatically clearing the error and resetting the count of consecutive errors found in pConsecErrors. Once the MaximumErrors value has been reached, the timer will start. Once the time has elapsed, the error will be cleared and pConsecErrors will be reset. This value and the MaximumErrors value must be greater than zero to enable this feature. |
| CommErrorMsg | String | Used with ErrorType “No Data”. The sStatus field in the CRR message response received from CommCore indicates the error status. If an error is detected, the string found in the sStatusMsg is compared to this field. If a match is found, the pConsecErrors field is incremented. If MaximumErrors is reached, the error processing feature will be activated. If ScnScanZone_ScanAlgorithm is configured to use an application-specific algorithm, special strings must be sent in this field: - ApplGoodScan – good scan, no error detected - ApplNoReadScan – scan error detected - Destination Found – indicates destination was found - Destination NOT Found – indicates no destination was found - ALL – special case representing all error types See section 3.1.1 for more detail. |
| AppSorter | PopUp | For sorter control applications, select the SrtAppControl record that contains the motor to stop when MaximumErrors is reached. |
| PrintLine | PopUp | For print and apply configurations, select the PrintApplyLine record that contains the motor to stop when MaximumErrors is reached. |
| Xfer | PopUp | For transfer control applications, select the XfRouteBoss record that contains the motor to stop when MaximumErrors is reached. |
| ErrorLane | PopUp | For sorter control applications where a container is to be assigned to a special lane when an error occurs, select a sorter lane from the Outpoints table. This is typically used only for print and apply applications based on a sorter that have an error correction lane as part of the individual print and apply station. |
| pConsecErrors | Int | Set by the software. The current count of consecutive error events. |
| pResetSecsLeft | Int | The number of CTRL process scans left before auto-reset occurs. Used with the ResetSeconds field. |

  - Sorter NoScan Processing

When configured for use with a sorter, a NoScan event is detected if the scan does not arrive before the container shifts entirely through the update point associated with the scan.  Each ScnScanDevice entry will need to have UpdatePt defined to enable NoScan event processing.

# Transfer Scan Processing

If a scan device does not have an update point selected in the ScnScanDevice_UpdatePt column, it is treated as a transfer.  A transfer requires additional configuration in the transfer scan processing table XfRouteBoss.

When a scanner sends data for a transfer, the data is captured and stored in a tracking table row.  The tracking table row in use can be found in the ScnScanZone table pTrnTrkRecord field.  After all scans have been received, the tracking table reference is moved to the pTrnLastTrk field.

The XfRouteBoss table has entries required for use with transfers.  Detailed descriptions of the columns in XfRouteBoss are given here for items related to scanner configuration, shown in boldface in the Field column of the table, below.  The rest of the columns are described in minimal detail here.  See the FPC-Transfer-Configuration document for details on configuring transfers.

| XfRouteBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the transfer route. |
| Desc | String | Optional description |
| pScanStatus | String | Status messages generated by the software. May help find configuration errors. |
| pXfrErrorMsg | String | Status messages pertaining to the state of the running transfer. |
| LogLevel | PopUp | Set the detail level for log messages. The wizard sets this to Level 1 by default. |
| ScanZone | PopUp | Choose the scan zone associated with this entry. |
| EsmMsgTable | PopUp | To activate ESM message processing, choose a message table such as MsgGP1. |
| EssMsgTable | PopUp | To activate ESS message processing, choose a message table such as MsgGP1. |
| CrrMsgTable | PopUp | To activate CRR message processing, choose a message table such as MsgCR1. |
| CsmMsgTable | PopUp | To activate CSM message processing, choose a message table such as MsgGP1. |
| DcmMsgTable | PopUp | To activate DCM message processing, choose a message table such as MsgDC1. |
| CsuMsgTable | PopUp | To activate CSU message processing, choose a message table such as MsgGP1. |
| SendAvailStatus | Y/N | Set to Y to send information to CommCore in the CRR sZone1 field about the current availability status of transfer zones. |
| AvailTransfer | Int | Used to include availability status information from other related transfers. |
| Straight | PopUp | Defines the “go straight” route. |
| CommErr | PopUp | Defines the “communications error” route. |
| ScanError | PopUp | Defines the “scan error” route. |
| NotDefined | PopUp | Defines the route for cartons that don’t have a matching record in the route table. |
| NoData | PopUp | Defines the “no data” route. |
| RouteFull | PopUp | Defines the “route full” route. Also used for routes that are not enabled. |
| ESMRoute | PopUp | If a route needs to be executed with the ESM message, select it here. |
| MaxWaitTime | Int | No longer used. |
| NoScanTmr | PopUp | Automatically populated. This timer’s duration can be adjusted to prevent spurious NoScan events if the presence photoeye is activated before the scan is delivered. If no scan is received by the time this timer expires, a NoScan event will be generated. The timer is stopped if the SystemRun signal goes OFF or if no SystemRun device is selected. This feature can be disabled by setting DontChkNoScan to Y. |
| NoRouteTmr | PopUp | Automatically populated. This timer’s duration can be adjusted to prevent spurious “No Route” events by allowing CommCore some time to respond to a destination request. The timer is stopped if the SystemRun signal goes OFF or if no SystemRun device is selected. The NoRouteRetry field determines what will occur when this timer expires. |
| NoCartonTmr | PopUp | Automatically populated. This timer’s duration can be adjusted to prevent spurious “No Carton” events if a scan is received before the presence photoeye is activated. If a “No Carton” event occurs, queued transfer moves and scan information is cleaned up. The timer is stopped if the SystemRun signal goes OFF or if no SystemRun device is selected. This feature may be disabled in two parts by setting DontResetQue and DontResetRcv. |
| ClrQueDlyTmr | PopUp | Automatically populated. Used to detect if a carton is manually removed from a transfer, or if “push through” occurs. The default is 0.05 seconds. |
| DontChkNoScan | Y/N | If set to N, a NoScan event will occur if NoScanTmr expires while the presence photoeye is blocked and no scan has been received. Set to Y to disable generation of a NoScan event. |
| DontResetQue | Y/N | If set to Y, when the NoCartonTmr timer expires, queued transfer moves are left in place. If set to N, any queued transfer moves will be cleaned up. |
| DontResetRcv | Y/N | If set to Y, when the NoCartonTmr timer expires, unprocessed received scans are left in place. If set to N, any unprocessed scans are cleaned up. |
| NoRouteRetry | Y/N | If set to Y, when the NoRouteRetry timer expires, a new routing request message will be sent to CommCore. If set to N, when the NoRouteRetry timer expires, the CommErr field is checked to determine if a NoRoute route has been defined. If so, the container will be sent to that route. |
| SystemRun | PopUp | Choose a part that will indicate if the system is enabled to run. When it is OFF, timers will be halted. They will resume when this signal comes back ON. |
| WaitPresence | Y/N | For old transfers, Y will override the default behavior and wait for the presence photoeye to come ON before executing the transfer move. This is the default behavior for new transfers. |
| XferCnvMtr | PopUp | Choose the motor that will be disabled if a transfer error occurs. These may include consecutive scan errors and consecutive comm errors. The configuration in ScnZoneErrIO and ScnDeviceErrIO determines when this bit is activated. See sections 3.1 and 4.1 for full details. |
| MtrACTION | PopUp | Choose the action to take when a transfer error occurs. The opposite action will be taken when the error clears. |
| MtrBIT | Int | If Enable is chosen for MtrACTION, set the bit that will be used. |
| ScanErrSig | PopUp | Choose the memory bit to use when a transfer scan error occurs. These may include consecutive scan errors and consecutive comm errors. The configuration in ScnZoneErrIO and ScnDeviceErrIO determines when this bit is activated. See sections 3.1 and 4.1 for full details. |
| ScanACTION | PopUp | Choose the action to take when a transfer scan error occurs. The opposite action will be taken when the error clears. |
| ScanErrBIT | Int | If Enable is chosen for ScanACTION, set the bit that will be used. |
| DataErrSig | PopUp | Choose the memory bit to use when a transfer data error occurs. These are errors with an ErrorType of Data. The configuration in ScnZoneErrIO and ScnDeviceErrIO determines when this bit is activated. See sections 3.1 and 4.1 for full details. |
| DataACTION | PopUp | Choose the action to take when a transfer data error occurs. The opposite action will be taken when the error clears. |
| DataBIT | Int | If Enable is chosen for DataACTION, set the bit that will be used. |
| RunNewXfr | Y/N | Set to Y to use the “New Transfer” configuration, or N for the “Old Transfer” configuration. |
| CRRReadyToSend | PopUp | Choose a signal to indicate when the CRR message should be sent. |
| CRRSendSense | PopUp | Choose the state of CRRReadyToSend that will initiate a CRR message. |
| SendOneCRR | Y/N | Set to N to allow a CRR message to be generated each time a scan is received, including repeat scans resulting from push-through events. Set to Y to limit to a single CRR message, which helps prevent “out of sync” conditions for transfer Print Apply configurations. |
| ToteSensor | PopUp | Choose an input here to activate tote detection. |
| TotePresentStr | String | Set the string to send when a tote has been detected by ToteSensor. |
| ToteAbsentStr | String | Set the string to send when a tote has not been detected by ToteSensor. |
| pProductState | Y/N | Set by the software to indicate the state of the product input sensor. |
| pRouteTreeQue | Int | Set by the software to track route tree assignments. |
| pTrkMenuQue | PopUp | Set by the software to store a tracking table entry while waiting for a route. |
| pTrkIndxQue | Int | Set by the software to store a tracking table row while waiting for a route. |
| pProcFlag | Int | Set by the software to trigger the SERV process when scanner data is available. |
| pXfrError | Int | Set by the software to store the error status of the transfer. |
| pGrtHalfSecond | Int | Set by the software to track the number of responses that took more than 0.5 seconds. |
| pGrtQtrSecond | Int | Set by the software to track the number of responses that took more than 0.25 seconds. |
| pMaxResponse | Double | Holds the maximum response time value for interactions with CommCore. |
| pNumSamples | Int | Holds the number of responses processed. |
| pScanError | Int | Set by the software when determining the state of ScanErrSig. |
| pDataError | Int | Set by the software when determining the state of DataErrSig. |
| pWrkScanError | Int | Used internally when processing scan errors. |
| pWrkDataError | Int | Used internally when processing data errors. |
| pLastScan | String | Used internally to record the last scan received while performing verify processing. |
| pLastScanCnt | Int | Used internally to track scan events involving pLastScan. |
| pLastScanProc | Int | Used internally to indicate if the pLastScan scan was processed. |
| pParamConfigError | Int | Used internally for parameter processing. |
| pTimeoutError | Int | Used internally when evaluating timeout conditions. |
| pESMAck | Int | Used internally to trigger sending of ESS messages. |
| pTotePresent | Int | Used internally to latch the state of ToteSensor. Resets when a CRR or CSM message is sent. |
| pLastPack | String | Used internally to record the last scan received while performing verify processing. |
| pLastPackCnt | Int | Used internally to track scan events involving pLastPack. |
| pLastPackProc | Int | Used internally to indicate if the pLastPack scan was processed. |

# Debugging Scanner Connections

Serial ports:  Use “cat /dev/ser1” (for example) to see the input coming to the first built-in serial port on the console.  This will break the connection in FPC, so you will need to restart FPC afterward to get the port to connect again.

TCP ports:  Scanners will respond to a ping.  Some will allow connection using telnet.

# Example 1 – Single Scanner on a Sorter

This example describes the simplest case of a single scanner on a sorter.  The diagram shows a section of conveyor with the scanner and the positions of the scanner, photoeye and update point.  The containers move from left to right.  The following tables show settings needed to capture and process the scans.  Only the fields that are pertinent to the scanner configuration are shown.  Additional fields may need to be configured for a working setup.

Scanner photoeye – Scanner sends scan on tail-out.

Scanner – SCN201

Scan Zone 1 – SHIP

Update Point – Scan expected by Automation Controller at any time while the container blocks the beam.

Machine table settings

This defines a connection to a serial scanner called SCN201 that will filter the input according to the AlphaNum protocol, which, by default, keeps only letters and numbers, carriage return, line feed and a few others.

| Field | Value |
| --- | --- |
| Machine_Name | SCN201 |
| Connection_Type | CONNECT_SERIAL |
| Communication_Protocol | AlphaNum |
| Machine_Port | /dev/ser-rckt1 |
| Tansmit_Retry | 0 |
| Process | CTRL |

MsgMap table settings:

| Field | Value |
| --- | --- |
| Message_Name | Scan 201 |
| Machine_Name | SCN201 |
| Recv_Msg_Type | ALL |
| Send_Msg_Type |  |
| Ack_Msg_Type |  |
| Menu_Name | ScnScanDevice |
| Low_Cursor | 2 |
| High_Cursor | 2 |
| Stat_Cursor | 3 |
| Record | 1 |

ScnScanZone table settings:

| Field | Value |
| --- | --- |
| Name | Scan 201 |
| Delimiter |  |
| LogLevel | Level 1 |
| ScanZoneID | SHIP |
| CompareScans | N |
| TrackingTable | SrtTrack1 |
| DeviceTrkTable | SrtDevice1 |
| PrinterTrkTable |  |
| PrinterErrTable |  |
| CTRLProcess |  |
| ScanAlgorithm |  |
| Alg_P1 |  |

ScnScanDevice table settings:

| Field | Value |
| --- | --- |
| Name | Scan 201 |
| ScanZone | Scan 201 |
| ScanType | Scan |
| Field |  |
| UpdatePt | ScanUpd |
| NoReadStr | ???????? |
| ReadConflictStr | ******** |
| StartChar |  |
| QtyChars |  |
| ChkLength |  |
| MinWeight |  |
| MaxWeight |  |

SrtScanBoss table settings:

| Field | Value |
| --- | --- |
| Name | Scan 201 |
| ScanZone | Scan 201 |

# Example 2 – Single Scanner on a Transfer

This example describes the simplest case of a single scanner on a transfer.  The diagram shows a section of conveyor with the scanner and the positions of the scanner, its photoeye and the FPC presence eye.  The containers move from left to right.  The following tables show settings needed to capture and process the scans.  Only the fields that are pertinent to the scanner configuration are shown.  Additional fields may need to be configured for a working setup.

Scanner – SCN401

Scan Zone 1 – PICK1

Presence PE – PE401PRS

Carton is scanned as it moves into the transfer

Scanner photoeye

Machine table settings:

This defines a connection to a serial scanner called SCN401 that will filter the input according to the AlphaNum protocol, which, by default, keeps only letters and numbers, carriage return, line feed and a few others.

| Field | Value |
| --- | --- |
| Machine_Name | SCN401 |
| Connection_Type | CONNECT_SERIAL |
| Communication_Protocol | AlphaNum |
| Machine_Port | /dev/ser-rckt2 |
| Tansmit_Retry | 0 |
| Process | CTRL |

MsgMap table settings:

| Field | Value |
| --- | --- |
| Message_Name | Scan 401 |
| Machine_Name | SCN401 |
| Recv_Msg_Type | ALL |
| Send_Msg_Type |  |
| Ack_Msg_Type |  |
| Menu_Name | ScnScanDevice |
| Low_Cursor | 2 |
| High_Cursor | 2 |
| Stat_Cursor | 3 |
| Record | 2 |

ScnScanZone table settings:

| Field | Value |
| --- | --- |
| Name | Scan 401 |
| Delimiter |  |
| LogLevel | Level 1 |
| ScanZoneID | PICK1 |
| CompareScans | N |
| TrackingTable | XfrTrack |
| DeviceTrkTable | XfrDevice |
| PrinterTrkTable |  |
| PrinterErrTable |  |
| CTRLProcess |  |
| ScanAlgorithm |  |
| Alg_P1 |  |

ScnScanDevice table settings:

| Field | Value |
| --- | --- |
| Name | Scan 401 |
| ScanZone | Scan 401 |
| ScanType | Scan |
| Field |  |
| UpdatePt |  |
| NoReadStr | ???????? |
| ReadConflictStr | ******** |
| StartChar |  |
| QtyChars |  |
| ChkLength |  |
| MinWeight |  |
| MaxWeight |  |

XfRouteBoss table settings:

| Field | Value |
| --- | --- |
| Name | Xfr 401 |
| ScanZone | Scan 401 |