# FortnaPlus Control Security Upgrade Procedure

Document Revision: 1.1

Date: 01-Dec-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 26-Mar-2013 | T R Kremser | Document created |
| 1.1 | 01-Dec-2022 | T R Kremser | Update and refresh |

Table of Contents

Glossary of Terms	4

1.	Introduction	5

2.	Background	5

3.	Changes	7

4.	Upgrade Procedure	9

5.	Conversion Tools	10

6.	Mnu File Information	11

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Introduction

Changes associated with Redmine Issue #6216 (and supporting issues #6210, #6211, and #6212) require additional upgrade steps when upgrading from FortnaPlus configurations prior to V10.0R0.

Issue #6216 changes the treatment of security within FortnaPlus from a numeric level-based security model to a role-based model.

# Background

Level-based Model

Under the level-based model, each securable entity had a numeric security level value assigned to it.  A numeric security privilege level was associated with each user.  When the user attempted to access a securable entity, the user’s value was checked against the entities’ value.  If the user’s value was less than or equal to the entities’ value, access was granted.  (Exceptions will be explained below).  This system resulted in a hierarchy of user privilege – users with “higher” privilege (lower value) had access to all “their” items, plus all of those associated with users of “lower” privilege (higher value).

The FortnaPlus menu structure lead naturally to four default user roles: Operator, Maintenance, Administrator and Configuration.  A fifth role, the Fortna Super-User, with full access to all items was also defined.  These roles are listed here in order of increasing privilege under the level-based model.

Deficiencies

The job functions of the default roles do not match the level-based model’s structure – an Administrator does not necessarily possess the skill set of a Maintenance person, for example.  It is preferable to have a model where a user can be granted access to the securable entities associated with a role or roles as independent sets, removing the implied user hierarchy.

Storage

There are several types of securable entities within FortnaPlus.  The security level values for these entities are stored in two different places.  The security information for the users is also stored in multiple places.  Menus, tables, and columns within tables each have security values associated with them and may be independently set.  These values are stored within the “mnu” files, in the Integer column DSECR.  Security values are also set for callable procedures via the Security_Level column in table procnames, but these values are stored separately, encoded as a C array, in file project.sss in the RUN directory (Note that this has since been rectified – the security information in table procnames is stored within the table in the normal way and project.sss is no longer used.  TRK).  The user’s security values are also stored as integer values in the database table password, column LEVEL.  The Fortna super-user value is stored within the program itself, and if the Remote Authentication module is used with a level-based configuration, an integer value is stored in the level II system and returned upon successful authentication of a user.

Additional issues

The DSECR column had a few exception cases that must be considered.  First, in the level-based model, the Fortna Super-User was assigned the special security level value of 9999, which breaks the “lower value is more privilege” scheme.  There was really no reason for this, and it resulted in a lot of extra code to deal with the special case.  In many spots, a value of 1 was treated as Super-User, as well.  Second, the security level value of zero for securable entities was treated as “no security”, but was used in the hierarchy of menus to provide a sort of inheritance from the parent menu.  The third issue was that the DSECR column was also used to store information for database tables concerning additional special SQL directives  that were associated with the column.  These items did not belong in the DSECR column.  Issue #6216 moved them to a new column, DSQLCLS (new variable datasqlsclause in the source code), in the “mnu” files.  The values and their meanings were not changed:

| DSQLCLS Value | Meaning |
| --- | --- |
| 8999 | add FOREIGN KEY clause |
| 9990 | DROP INDEX statement |
| 9991 | CREATE INDEX statement |
| 9992 | add UNIQUE clause |
| 9993 | add ‘UNIQUE (‘ clause |
| 9994 | add AUTOINCREMENT clause |
| 9995 | add PRIMARY KEY and AUTOINCREMENT clauses |
| 9996 | add PRIMARY KEY clause |
| 9997 | designate as a ‘Calculated’ field that is not to be written to the database |
| 9998 | not used |

Only a few fields in a few tables, as defined in the standard set of tables, used these special sql directives:

| Table | Column | SQL Directive Value |
| --- | --- | --- |
| DbHistConfig | Name1 | 9991 |
| DbHistConfig | Name2 | 9991 |
| DbHistData | Name1 | 9991 |
| DbHistData | Name2 | 9991 |
| Error_Log | ErrNumber | 9991 |
| Error_Log | Time_Stop | 9991 |
| EventLog | Description | 9991 |
| EventLog | EventTimeStamp | 9991 |
| MsgTrack | TxSeq | 9996 |
| MsgTrack | SubSeq | 9996 |
| TIMElist | TName | 9996 |
| TIMElist | Description | 9996 |
| TIMElist | Owner | 9991 |
| TIMElist | Changed | 9991 |

Also note that in the context of security, these values were interpreted according to the usual scheme, so their security level could not be set independently, and was interpreted as a very low security level – in effect, granting all users access to change them.

# Changes

Changes implemented in Issue #6216 use the same Integer data storage for the security information but assign each bit to a role.  Each securable entity can be assigned one or more role bits.  Each user can be assigned one or more roles.  Access is granted to a securable entity if the user has any of the role bits set that are set for the securable entity.

Role Editor

To manage this new behavior, a Role Editor has been added in conjunction with a new data type.  Type 38, INTEGER_AS_ROLESET, stores the data and otherwise acts the same as type 1, INTEGER, except it also adds access to the Role Editor, which can be activated by double-clicking on any type 38 field.  The role editor shows the roles by name and allows selection of roles by checkbox.  Explanatory text is also displayed on the proper use of the type 38 fields.  The integer value representing the bits chosen is shown in the editor and is copied to the field when the “Accept” button is clicked.  Once the user knows the appropriate value for a given role, it may be entered directly in the type 38 fields in the same way as an INTEGER without using the Role Editor, if desired, for speed.

Four table columns have been changed to type 38:  MenuColumn_SecurityLevel, procnames_Security_Level, password_LEVEL, and Conveyor_Gen_Security.  Note that the Designer Pane for DDS windows, like the Print Apply Status screen, also have a Security field that can access the Role Editor.

Remote Authentication

If the optional Remote Authentication module is used, the level II side must be configured to return a list of role strings.  Under the level-based model, the level II side was configured to return a single numeric value representing the user’s security level.  The level II side already supports returning a list of roles.  The valid role strings expected by FortnaPlus are:

| Remote Authentication Role String | Meaning |
| --- | --- |
| FPCSU | FortnaPlus Super-User. FortnaPlus grants the Super-User role and all others. |
| CONFG | Configuration Action Role |
| CFGRO | Configuration Read-Only |
| ADMIN | Administrator Action Role |
| ADMRO | Administrator Read-Only |
| MAINT | Maintenance Action Role |
| MNTRO | Maintenance Read-Only |
| OPRTR | Operator Action Role |
| OPRRO | Operator Read-Only |
| USER1 | User-defined role 1 |
| USER2 | User-defined role 2 |
| USER3 | User-defined role 3 |
| USER4 | User-defined role 4 |
| USER5 | User-defined role 5 |

FortnaPlus interprets the FPCSU role token to mean “assign all roles”.

The USER1 – USER5 role tokens are not assigned to any securable entities within FortnaPlus by default but are available to be used to configure for special circumstances, if needed.

The Read-Only roles allow access to view all items under that role’s top-level menu item.

The Action roles allow changes to all items under that role’s top-level menu item that we have deemed appropriate for that role.

All the security settings may be changed easily, as need requires, using the Role Editor on MenuColumn_SecurityLevel (accessible via Alt-K on the table in F2 – MenuMenu), procnames_Security_Level, password_LEVEL, and Conveyor_Gen_Security.  The Designer Pane (Ctrl-D) has a Security field that can be used to access the Role Editor for DDS windows, like the Print Apply Status screen.

MNU File Changes

An additional column, DSQLCLS, has been added, as described above, to the end of the table column definition lines.  It contains the special directives for SQL database clause control that were formerly embedded in the DSECR column in the range of 8999 to 9998.

Four table columns have been changed to type 38:  MenuColumn_SecurityLevel, procnames_Security_Level, password_LEVEL, and Conveyor_Gen_Security.  The data entry upper limit value (DHIGH or MenuColumn_HighValue) has been changed to 16383 – the value obtained if all role bits are set.

All the DSECR values stored in the MNU files have been reviewed and adjusted to suit the role-based model.

MySQL Database Changes

User’s security level information is stored in the LEVEL column of the password table in the MySQL “Fortna” database.  The values stored there must be converted to integer values representing the role bits granted to each user.

Level to Role conversions

Level to Role conversions (and the reverse) can be made automatically if some assumptions are made about the levels assigned and their meanings in the standard FortnaPlus level-based configuration.  Tools provided to aid in the conversion process are described below.  They all follow this scheme for conversion:

| Level | Role | Level |
| --- | --- | --- |
| 401 to 8998 | OPRRO | 450 |
| 400 | OPRTR | 400 |
| 301 to 399 | MNTRO | 350 |
| 300 | MAINT | 300 |
| 201 to 299 | ADMRO | 250 |
| 200 | ADMIN | 200 |
| 100 to 199 | CFGRO | 100 |
| 2 to 99 | CONFG | 50 |
| 1 and 9999 | FPCSU | 1 |

Note that 8999 through 9998 represent SQL directives and have been moved to their own column.

The tools described below for conversion may be used to convert in either direction.  Converting from role to level is most likely an academic exercise but may help illuminate some useful details.

The process is not reversible, since the level 9999 is lost, and any “odd” values are normalized to the values shown.  However, a level-based configuration that is converted to role-based and back will be functionally equivalent if the user levels are set to the defaults of Operator – 400, Maintenance – 300, Administration – 200, and Configuration – 100.  Note that the Configuration user, at level 100, is equivalent in the role-based scheme to CFGRO.

Compilation from Source

Note that the makeinits utility has been updated (Redmine Issue #6123).  The added column in the mnu files put the line length over the limit, so the limit has been increased.  You will need the updated makeinits to compile this version.

ROLE_NONE

The special significance of a security value of zero (ROLE_NONE) for a securable entity has been preserved.  This is useful for read-only access to tables, since multiple roles may then view a table, while only one role has privilege to change its contents.  This has been used extensively in the re-defined default security settings.

# Upgrade Procedure

To upgrade from a version of FortnaPlus prior to version 10.0R0 (Issue #6216), some additional steps must be taken.  This is necessary to add the new DSQLCLS column to the MNU files, and to adjust the DSECR settings in the MNU files and the security values encoded in the project.sss file.  The extra upgrade steps are only necessary to switch from level-based to role-based security – once the change is made, further upgrades are done with the usual procedure.

- Use “rundircheck.pl” with the –SR options.  The –S “SECURITY OVERRIDE” option preserves the DSECR settings in the new MNU files.  The default behavior is to keep the old DSECR settings, which is undesirable in this case.  Run “rundircheck.pl –help” for more explanation of the options available.  The –R “Repair” option is needed, as usual, to write the changes to the mnu files.  As always, be sure to capture the output of the rundircheck.pl script to a file and analyze it carefully to be sure there are no unexpected surprises.

- Use the new “convert_security” program (Issue #6212) to convert the project.sss file to role-based security.  Run the command without arguments to see help on its use.  To convert from level-based to role-based:

  - cd /SortPlus/RUN (or wherever your RUN directory resides)

  - cp project.sss level.project.sss

  - convert_security –r –V level.project.sss role.project.sss

  - Inspect the output of the command.

  - cp role.project.sss project.sss

- Use the new level2role.sql script to convert the level values stored in table “password”, column “LEVEL” to role values.  Alternatively, if you have only the default set of users, you may drop the password table and let FortnaPlus re-create it, though you must also remove /usr/local/var/fortna/control.fil or FortnaPlus will re-construct the table using the values stored there.  The contents of this table may also be edited from within FortnaPlus via the “Set User Passwords” menu items found in the Admin and Config user menus.  Run the scripts as in this example (more details in the next section):

mysql –u root –p fortna < level2role.sql

- If using the Remote Authentication module, be sure to update the tokens returned from the Level II portion to return appropriate role strings instead of a level value.  (See above)

NOTE: Subsequent upgrades do not require any special steps – just the usual use of rundircheck.pl.

# Conversion Tools

A few additional tools are needed to make the level to role conversion, as described above.  Here are a few additional details on these tools.

rundircheck.pl

Two new options have been added to the rundircheck.pl script.  The –r option converts a set of mnu files from level-based to role-based, following the conversion scheme described above.  It also moves the SQL directive entries from the DSECR column to their own column, DSQLCLS, at the end of each table column definition line.  The –l option converts a role-based set of mnu files back to level-based, removing the extra DSQLCLS column in the process and returning the special sql directive values to the DSECR column.

The mnu files stored in the repository from V10.0R0 (Issue #6216) forward have already been converted, so these options are not needed when using them.  The –S option is needed to preserve the updated DSECR values from the new “mnu” files.

convert_security

The convert_security program is used to adjust the values stored in the project.sss file.  It can be used to convert from level-based to role-based using the –r option, or the other way using the –l option.  The conversion follows the conversion scheme described above.  Run it without arguments to see details on what it will do.  Use the –V option for verbose output, including the starting and ending values.  Role values are shown both as decimal and binary representations.  Note that the options cannot be combined: -rV will not work.  Use –r –V instead.  Both an input and an output file name must be given to capture the output in the proper form.  If only an input file is given, the –V option will show what would be done without doing it.

MySQL Conversion Scripts

The user privilege values are stored in the MySQL table “password”, column “LEVEL” in the “fortna” database.  These values must be converted to values that represent the role bits granted to each user.  The scripts may be run as in the following example:

mysql –u root –p fortna < level2role.sql

This will update the password_LEVEL values according to the conversion scheme described above.  The role2level.sql reverses the process.

The command connects to the local MySQL instance as user root (-u root), prompts for a password (-p), “uses” database fortna (fortna), and takes the command input from the level2role.sql file (find it in /usr/local/bin, or the source repository).  The sql files are text, so you may inspect them if you wish.

# Mnu File Information

The following table shows some information for the data elements stored in the mnu files.

| MNU Header Name | Program Datastructure Name | Table Column Name | Data Type |
| --- | --- | --- | --- |
| MNUNAME | mnuparms[a].mnuname | MenuMenu_Menu_Name | String |
| ROW | mnuparms[a].mrow | MenuMenu_Row | Int |
| COL | mnuparms[a].mcol | MenuMenu_Col | Int |
| FG | mnuparms[a].mfg | MenuMenu_FG_Color | Int |
| BG | mnuparms[a].mbg | MenuMenu_BG_Color | Int |
| HFG | mnuparms[a].mhfg | MenuMenu_HFG_Color | Int |
| HBG | mnuparms[a].mhbg | MenuMenu_HBG_Color | Int |
| TY | mnuparms[a].starty | MenuMenu_Start_Row | Int |
| TX | mnuparms[a].startx | MenuMenu_Start_Col | Int |
|  |  | MenuMenu_Start_Text_Row | Int |
|  |  | MenuMenu_Start_Text_Col | Int |
| TYPE | mnuparms[a].type | MenuMenu_Menu_Type | Int |
| #RECS | mnuparms[a].arrays | MenuMenu_Data_Recs | Int |
| LOC | mnuparms[a].pointermemb | MenuMenu_Data_Location | Int |
| FONT | mnuparms[a].font | MenuMenu_Menu_Font | PopUp |
| CURR | mnuparms[a].mycurr | MenuMenu_Current_Cursor | Int |
| MXCR | mnuparms[a].mymaxcurr | MenuMenu_Last_Cursor | Int |
| DREC | mnuparms[a].datarec | MenuMenu_Current_Record | Int |
|  |  | MenuMenu_Last_Record | Int |
| VERP | mnuparms[a].vertp | MenuMenu_Vertical_Position | Int |
|  |  | MenuMenu_Last_Vertical | Int |
|  |  | MenuMenu_Max_Cursor_Row | Int |
| BRODCAST | mnuparms[a].broadcast | MenuMenu_Broadcast_Menu | Int |
| DISPMAX | mnuparms[a].menudispmax | MenuMenu_Max_Display | Int |
| FILTERED | mnuparms[a].filtered | MenuMenu_Filtered_Menu | Int |
| DSTYPE | mnuparms[a].disptype | MenuMenu_Display_Type | Int |
| NOBOX | mnuparms[a].nobox | MenuMenu_NoBox | Int |
| FLAGS | mnuparms[a].flags | MenuMenu_Flags | Binary |
| *** |  |  |  |
| COLNAME | mnu[a][b] | MenuColumn_Column_Name | String |
| DTYPE | datatype[a][b] | MenuColumn_Data_Type | Int |
| DLEN | datalen[a][b] | MenuColumn_Length | Int |
| DLOW | datalow[a][b] | MenuColumn_LowValue | Int |
| DHIGH | datahigh[a][b] | MenuColumn_HighValue | Int |
| DLIST | dataselect[a][b] | MenuColumn_Select_Str | PopUp |
|  |  | MenuColumn_Select_StrCol | Trilogy |
|  |  | MenuColumn_Select_StrRec | PopUp |
| DSECR | datasecure[a][b] | MenuColumn_SecurityLevel | Int |
| DHIDE | datahide[a][b] | MenuColumn_Hide_Col | Int |
| DFONT | datafont[a][b] | MenuColumn_Font | PopUp |
| DX | dataxcord[a][b] | MenuColumn_Xcord | Int |
| DY | dataycord[a][b] | MenuColumn_Ycord | Int |
| DRTYPE | datarealtype[a][b] | MenuColumn_RealType | Int |
| DORD | dataorder[a][b] | MenuColumn_Order | Int |
| DFG | datafg[a][b] | MenuColumn_ForGndCol | PopUp |
| DBG | databg[a][b] | MenuColumn_BakGndCol | PopUp |
| DHFG | datahfg[a][b] | MenuColumn_HilitFGCol | PopUp |
| DHBG | datahbg[a][b] | MenuColumn_HilitBGCol | PopUp |
| DSRC | datasource[a][b] | MenuColumn_Source | Int |
| DDLEN | datadisplen[a][b] | MenuColumn_Displaylen | Int |
| DSPACE | dataspace[a][b] | MenuColumn_PopUpMenu | PopUp |
| DSTRING | datastring[a][b] | MenuColumn_DataString | String |
| DSELHIDE | dataselhide[a][b] | MenuColumn_HideInSelection | Int |
| DSQLCLS | datasqlclause[a][b] | MenuColumn_SQLClause | Int |