# FortnaPlus Control Servo Gap Optimizer

Document Revision: 1.1

Date: 18-Jan-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 15-Aug-2014 | B C Conklin | New document |
| 1.1 | 18-Jan-2023 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	The Servo Gap Optimizer Module	5

2.	Determining Container and Belt Length	5

2.1.	Example Configuration	7

2.2.	Example Gap Optimizer Requirements and Control Logic	7

2.3.	Gap Calculations	9

3.	Design Concerns	9

4.	Servo Gap Optimizer Configuration	10

5.	Servo Gap Optimizer Tables	10

5.1.	Configuring the Shifter Portion	10

5.2.	Locating the Induct Photoeyes	11

5.3.	Configuring Induction Points	11

5.4.	Configuring Update Points	11

5.5.	Configuring the Encoders Table	12

5.6.	Configuring Logical Belts	12

5.7.	Configuring Belt Speeds	12

5.8.	The Exit Belt	12

5.9.	Verifying the Shifter Configuration	13

5.10.	Checking Operational Container Counts	14

6.	The Servo Gap Optimizer Table	14

7.	The Servo Gap Optimizer Simulator	17

8.	An Example Gap Optimizer	18

9.	Gap Optimizer Simulator Example	19

10.	Encoders Table Configuration for the Example	22

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# The Servo Gap Optimizer Module

The Servo Gap Optimizer module uses standard shifters linked to sequential servo-controlled belts.  The shifters contain images of the containers.  Encoders are used to measure where containers are located for gap calculations.  Standard shifters, in combination with the Servo Gap module, allow detection of containers that have slipped out of their respective window, increasing the gap in front of the container and reducing the gap behind the container.  The Servo Gap Optimizer restores the location of the container that has fallen back by increasing the speed of the belt, pulling it forward in the space between the two containers.

# Determining Container and Belt Length

The induct photoeye may be located as close as two inches from the start of the belt.

Effective belt length = Total belt length – Photoeye location.

Optimum Belt Length = Carton length + Gap required.

Maximum belt length = Carton Length + (2 * Gap required).  If the belt length is greater than or equal to the Maximum belt length the carton gap cannot be adjusted.

Minimum belt length = Carton length.  If the belt length is less than or equal to the Minimum belt length the carton gap cannot be adjusted.

Optimum Length =

Carton Length + Gap

Maximum Length =

Carton Length + (2 * Gap)

Carton position can’t be adjusted

Minimum Length =

Carton Length

Carton position can’t be adjusted.

Example with four belts:

Infeed

420 ft/min

Belt 1

Exit

420 ft/min

Belt 2

Belt 3

Belt 4

Induct

PE

Belt 1

Belt 1

Encoder

Induct

PE

Belt 2

Induct

PE

Belt 3

Induct

PE

Belt 4

Exit Belt

Encoder

Example Configuration

- Four 36 in. belts – average product 20 inches in length

- Each belt has an encoder

- Each belt has an Induct PE located 3/4 largest product length from the beginning of the belt

- Each belt has a VFD for speed control

- Exit belt also has an encoder to detect the location and speed of product.

# Example Configuration

The following configuration has four physical belts.  The system runs at 420 ft/min and the maximum speed is set at 525 ft/min.

A logical belt and shifter are required to measure and detect the movement of containers as they transition from belt to belt.  Containers move to the logical belt when they hit the “End of Belt” update point and are copied to the logical belt by the software.

Containers are inducted onto the next physical belt when they are at the Induct PE.

As a container moves onto the next belt, there is a period of time when it has not yet reached the “Induct PE” on the belt, but it has moved far enough onto the next belt to begin to be pulled ahead (if the new belt is running faster).

The control logic runs the belt at Max Speed if the belt does not have a container between the Induct PE and the “End of Belt” (the space between the Induct PE and the “End of Belt” is the “Belt Length”) as long as the “Target Gap” is less than the “Belt Length”.

To prevent the possible “Loss of Gap”, the speed will match the speed of the “Next Belt” if the “Target Gap” is larger than the “Belt Length”.

# Example Gap Optimizer Requirements and Control Logic

- Gap Optimizer – Belt 1

  - Target Gap – 12 inches (1 ft)

  - Design Container Size – 20 inches

- Containers are inducted onto “Belt 1”, when they reach the Induct PE.

- Containers are inducted onto “Logical Belt 1” when they reach the “End of Belt” update point.

- If Belt 1 does not have a container present, it will run at maximum speed unless the “Target Gap” is larger than the “Belt Length”, in which case it will match the speed of Belt 2 – the next belt.

- If Belt 1 has only one container present, it will calculate the speed needed for the gap.

- If Belt 1 has more than one container present, it will match the speed of the upstream belt.

Infeed

420 ft/min

Belt 1

Exit

420 ft/min

Belt 2

Belt 3

Belt 4

Induct PE

Logical Belt 1

(Ticks from Belt 2)

End of Belt

Update Point

Box 1

Box 2

Same Encoder

(DupToMem Bit)

Belt Length

The Example requires eight shifters.

- Four shifters are used for the four physical belts.

- Four logical shifters are linked to the physical belts.

- Note: The encoder for logical belt 1 gets its ticks from a “Dup To Memory” of the encoder for Belt 2.

# Gap Calculations

Box 2

Box 1

Current Gap

Gap Error

Desired Gap

Ticks Left on Belt 1 (between Belts)

Speed = Max Speed * (Ticks Left / (Ticks Left + (Gap Error * Gain)))

Gap Error = (Desired Gap – Gap between boxes)

Box 2 – Located on Belt 1

Box 1 – Located on Logical Belt 1

# Design Concerns

    - The Servo Gap Optimizer only corrects the position of a container.  It does not provide additional gapping if it is needed in a slug of containers.

    - If two sequential containers are too close together, and if the first container cannot be pulled forward enough to correct the gap, the second container’s gap cannot be corrected either.

Infeed

420 ft/min

Exit

420 ft/min

Gap 1

Gap 3

Gap 2

# Servo Gap Optimizer Configuration

The Servo Gap Optimizer configuration requires only one table.  One additional table is available to configure for simulation.

      - The ServoGapOpt table requires an entry for each belt controlled by the module.

      - The GapOptSim table contains the configuration parameters used to run the belts in simulation mode.

Note:  Activate the Servo Gap Optimizer module in the Batches table:

InternalProcCtrl = ServoGapOpt (SORT)
Process = SORT

# Servo Gap Optimizer Tables

# Configuring the Shifter Portion

Determine the size of the shifter that will be required for the gap optimizer.  Configure the shifter (Sorter Configuration Menu) and add the entries required for each column.  If the Servo Gap Optimizer uses a timer-based encoder, set the “TimeBasedEncoder” column to “Y”.  The Servo Gap Optimizer will automatically start and stop the timer (if used) for generating encoder ticks based on the state of the input defined in the Gap Optimizer Menu RunIO column.

To Create the Belt Shifter:

- Calculate the size of the shifter based on the length of each belt.  If using the wizard, it will round up to the nearest 500 ticks.

- Create two update points for each shifter.  Only one is needed for the “End of Belt” point for operation.  The second update point is needed for the simulator “Copy to Next Belt” point.

Note:  Refer to the Tracking Utilities section for more information on configuring, testing, and debugging the shifter.

# Locating the Induct Photoeyes

Each belt has an Induct PE located as close as two inches from the start of the belt.  The photoeye location can be adjusted up to “3/4 of the Largest Product”.  The photoeye location is adjusted to accommodate the target carton size.

# Configuring Induction Points

Locate the Induct Point at 30 ticks for both the physical belt and the logical belt.  The shifter links to the physical belt location based on where the Induct PE is located.  The physical belt induct point will be selected as an entry in the GapOptSim table Inpoint and NextInpoint columns.

The logical induct point is configured with a memory bit.  The memory bit is controlled by the software as a container passes through the “End of Belt” update point.

Note:  If using the simulator and the boxes are larger than 29 ticks, set the tick location of the induct point to the (Largest Box + 1 tick).

# Configuring Update Points

The “End of Belt” update point is located at the end of the gapper belt.  The “Belt Length” is calculated to determine the “End of Belt Location” for the update point.  “Belt Length” is the distance from the induct point (3/4 Longest Carton) to the “End of Belt” in ticks.

Example:

- Encoder resolution = 12 ticks per foot

- Total Belt Length = 36 inches.  Belt (36 ticks)

- Longest Carton = 24 inches (24 ticks)

- ¾ Longest Carton = 18 ticks

- Induct location = ¾ Longest Carton = 18 ticks

- Belt Length = Total Belt Length (36 ticks) – Induct Location (18 ticks) = 18 ticks.  The actual belt length should be measured.  Measure the distance from the induct PE to the point where the container touches the next belt.

- End of Belt update point location = Induct offset (30 ticks) + Belt Length (18 ticks) = 48

- Copy to Next Belt update point location (only used for simulation) = Induct offset (30 ticks) + Belt Length (18 ticks) + ¾ Longest Carton (18 ticks) = 66

# Configuring the Encoders Table

If the shifter is configured with a timer-based encoder, an Encoders table entry may be configured, however it is not required.

If an encoder is selected in the Sorters table (Encoder ioName column) and the “Encoder Type” is set to a timer-based encoder:

- The timer-based encoder entry will update the speed columns.

- It can be used to create a “DupToMemBit” encoder signal.

- The EnableBit column can be used to start and stop the timer-based encoder.

# Configuring Logical Belts

Each physical belt requires a corresponding logical belt.  The encoder for the logical belt is linked through the Encoders table DupToMemBit column.  For example, Belt 1’s logical belt gets its encoder signal using the DupToMemBit of Belt 2, etc.

# Configuring Belt Speeds

Belt speeds are adjusted as a percentage of the maximum speed.

For example:

- If the conveyor feeding the Servo Gapper is running at 425 ft/min

- The maximum belt speed of the Servo Gapper belts is 500 ft/min.

- The minimum belt speed of the Servo Gapper belt would be set to 85%.

  - MinSpeed = 425 ft/min / 500 ft/min = 85%.

  - MaxSpeed = 100%.

# The Exit Belt

The “Exit Belt” is configured with an encoder that provides ticks for “Logical Belt 4”.  The actual gap between cartons on “Belt 4” and the “Exit Belt” is calculated from the gap between cartons on “Belt 4” and cartons on “Logical Belt 4”.

An entry for the “Exit Belt” is added to the ServoGapOptimizer table to define the maximum speed of the “Exit Belt”.  The “Exit Belt” is selected in the NextBelt column for “Belt 4”.

The “Exit Belt” speed is controlled from the ServoGapOptimizer table and is set in the Minimum Speed column and the Maximum Speed column.  Both are set to the same value – using the example:  Exit Belt Speed = 85%.

When the gap on “Belt 4” matches the target (12 inches in the example), “Belt 4” will match the speed of the “Exit Belt” (85%).

# Verifying the Shifter Configuration

The shifter configuration is used to calculate the gap based on the encoder settings.  To properly calculate the gap, the encoder settings must be correct.

- Add a memory bit to the MaintIn column of the SpdControl table.  Set the target speed (for example 85%) in the OnPercent column.

- Force the memory bit ON.

- All drives should go to the speed entered (for example 85%).

- Position five containers on the conveyor separated by the target gap (for example 12 inches).

- Run the five containers through the Belt 4 Induct PE, and stop the conveyor when they all appear on the shifter screen for the last belt (for example Belt 4 Gap Belt).

- Calculate the number of ticks in the gap.

  - The container number is found in the first column of the “Boxes Info” tab.  The Carton Number corresponds to the “Data Position” column of the SortData table.  The length is found in the “Box Size” column.

  - The distance between containers is calculated by using the “Boxes Info” tab on the shifter screen.  The second column contains the location of each container. The space between containers is calculated by subtracting the location of container 1 from the location of container 2.

  - The gap is found by subtracting the length from the space between containers.

  - Verify the gap in ticks matches the pGapInTicks value found in the ServoGapOptimizer table.

# Checking Operational Container Counts

The software keeps a count of the containers that have attained the required gap for each belt.  The statistics help determine the number of containers where gap issues are occurring.

The columns:

- pGoodCount		– The number of containers that have attained the required gap.

- pOneOrZeroCnt	– The number of containers that have attained the required gap or are within 1 tick of the required gap.

- pTtlCount		– The total number of containers processed on the belt.

The container counts may be reset individually, or they may all be reset at once by selecting:  Configuration Menu | Gapping Utilities | Gap Optimizer | Clear Carton Counts

# The Servo Gap Optimizer Table

The ServoGapOpt table (Configuration Menu | Gapping Utilities | Gap Optimizer | Servo Gap Opt Table) is used to define the belts that comprise the gap optimizer.

| ServoGapOpt Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this line. |
| EnableFlag | Y/N | Used to manually enable or disable this entry. |
| pStatus | String | The software will display status and error messages here. |
| pBeltState | PopUp | The software will indicate the current state of the Gap Optimizer here. - No Box – Belt does not currently have a box. Follow speed of the next Belt. - More 1 Box – Belt has more than 1 box. Follow speed of the next belt. - Gap Okay – Follow the speed of the next belt. - Adding Gap – Slow down to the speed needed for the gap. - Reduce Gap – Speed up to the speed needed for the gap. Transferring – Follow the speed of the next belt. |
| UpdTimerFlag | Y/N | Set the column to “Y” to allow the module to update the shifter timer. A shifter timer may be used instead of a physical encoder, or it may be used if running in simulation mode. |
| LogLevel | PopUp | Select the desired log level. 0 – Log only errors. 1 – Log warnings and errors. 2 – Log warnings, errors, and debugging information. |
| GapInches | Int | Enter the required gap in inches. The software converts the gap to ticks based on the configuration in the Encoders table. |
| PitchFlag | Y/N | N = The module will set the gap. Set GapInches to the desired gap. Y = The module will set the pitch. Set GapInches to the desired pitch. |
| Allow2Cartons | Y/N | N = USE THIS SETTING. Expect a single container on each belt. The physical and logical configuration still must be set up as defined in the document to get good results. Y = Attempt to adjust the container position when more than one container can fit on the belt at a time. This setting produces poor results in most cases. |
| Gain | Float | Enter the desired gain. The default value is 1.00. |
| NextBelt | PopUp | Select the next downstream belt. |
| UpdPntBeltEnd | PopUp | Choose the update point that is located at the end of the belt. For example, if the induct point is located at ¾ the length of the largest carton, this update point will be located at (“Length of the Belt” - “¾ largest carton”) |
| IndPntLogical | PopUp | Select the induct point created for the logical belt. The logical belt must be created with a “Memory Bit” for the Induct PE. The software will induct a container onto the logical belt when it passes through the UpdPntBeltEnd point. |
| Machine | PopUp | Choose the AC that will control this belt. |
| RunIO | PopUp | Choose the IO signal that will indicate if the belt is running or stopped. If configured, the belt will stop when this signal is OFF. |
| MinSpeed | Float | Enter the minimum operating speed for the belt as a percentage of the maximum speed. Typically set to the speed of the exit belt. |
| MaxSpeed | Float | Enter the maximum operating speed for the belt. Usually 100%. |
| AccelMilliSec | Float | Enter the acceleration time in Milliseconds. The software converts this into iterations of the SORT process, and adjusts the speed based on the rate. |
| SpdControl | PopUp | Choose the entry from table SpdControl that will be used to control the speed of the motor. The OffPercent column is updated with the calculated speed. The SpdControl table scales the value and outputs it to the VFD. |
| TimeBasedEncoder | Y/N | N = Use a physical encoder. A timer-based encoder is only used for simulation. Y = Use a timer-based encoder for the belt. The RunIO signal will start and stop the timer. Note: If set to Y, the RunIO signal must be turned ON for the simulator to run. |
| EncTmrPreset | Time | Enter the time value needed to set the timer-based encoder for 100% speed. The entry is used for both timer-based encoder configurations and for simulation. If the belt uses a physical encoder, the entry is only used for simulation. |
| pGapInTicks | Int | The software will set this to the current gap converted to tick. |
| pNumBoxes | Int | The software sets this to the current number of containers found on the belt. |
| pTicksLeft | Int | The software sets this to the number of ticks between the last container found and the “End of Belt” update point position. |
| pTicksToBox | Int | The software sets this to the number of ticks from the logical belt induct point to the first container. |
| pCurrGapTicks | Int | The software sets this to the number of ticks for the gap. It is the sum of pTicksLeft and pTicksToBox. |
| pErr | Int | The software sets this to the calculated error for the gap. |
| pSpeed1 | Float | The software sets this to the calculated speed needed to create the required gap. |
| pSpeed2 | Float | The software sets this by applying MinSpeed and MaxSpeed to the pSpeed1 value. |
| pSpeed3 | Float | The software sets this by applying the acceleration constraints to pSpeed2. |
| pBoxPresent | Int | The software sets this based on the state of the induct photoeye for the logical belt. The memory bit for the logical belt is set ON or OFF as a container moves through the End of Belt update point. |
| pCurrBoxIndx | Int | Not used |
| pAccPerScan | Float | The software calculates the speed change for each iteration of the SORT process and shows the value here. It is derived from the SORT process time setting (usually 5 mSec) and the acceleration setting. |
| pLastEncCnt | Int | The software sets this to to last encoder count processed. It is used to detect encoder pulses. |
| pPresetValue | Time | The software sets this to the timer value needed to adjust the timer-based encoder. |
| pEnc2Factor | Float | The software sets this with the ratio of the current belt encoder and its NextBelt encoder, as found in the Encoders table. |
| pFactor1 | Float | The software sets this to the calculated speed factor before the limit checks are applied or the downstream belt speed is checked. |
| pFactor2 | Float | The software sets this to the calculated speed factor after the limit checks are applied and the downstream belt speeds are checked. |
| pGoodCount | Int | The software sets this to the number of containers that have attained the required gap. |
| pOneOrZeroCnt | Int | The software sets this to the number of containers that have attained the required gap or are within one tick of the required gap. |
| pTtlCount | Int | The software sets this to the total number of containers processed on the belt. |

# The Servo Gap Optimizer Simulator

The GapOptSim table (Configuration Menu | Gapping Utilities | Gap Optimizer | Servo Gap Opt Sim) is used to start and stop the gap optimization simulator, and to define the size of the boxes created on each infeed lane.

| GapOptSim Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Create a unique name to match each entry in the ServoGapOpt table. |
| EnableSrt | Y/N | Use this to enable or disable each entry. |
| CreateBoxes | Radio | Control creation of simulated containers. For use on the first belt. |
| pStatus | String | The software will display status and error messages here. |
| GapBelt | PopUp | Choose the row from table ServoGapOpt to associate with this row. |
| FirstBelt | Y/N | Use this to define the first belt in the sequence. Containers are only created on the first belt. They will be copied to the downstream belts as they move through the simulation when they reach the CpyNextUpd update point. |
| CreateSorterTmr | Radio | Select this to create an encoder timer for the associated shifter. |
| LengthInches | Int | Enter the length in inches for the simulated containers. |
| GapInches1 | Int | Set the length, in inches, of gaps for the simulation. You must define all four. They will be used round-robin style. You might use these to test the effect of gap errors. If the design gap is 12 inches, you might set one value here to 16 to simulate the container slipping back 4 inches, and one to 8 to simulate a too-small gap. Set any unused ones to the design gap. |
| GapInches2 | Int |  |
| GapInches3 | Int |  |
| GapInches4 | Int |  |
| Inpoint | PopUp | Choose the induct point for the belt. The simulator will create a container on the first belt and copy it to each downstream belt as it shifts through the gap optimizer. |
| CpyNextUpd | PopUp | Choose the update point for the belt. This represents the location where the simulated carton will be copied to the next belt. If the induct point of the next belt is set to the three-quarters length point for the largest container, add that length to the “End of Belt” update point. |
| NextInpoint | PopUp | Choose the induct point for the next belt. The simulator will copy the simulated container to the next belt when it reaches the CpyNextUpd update point. |
| pStateForcePE | Int | Set by the software to track the force status so it can be restored when the simulator is disabled. |
| pCurrLength | Int | Set by the software to track the length of the box as it is created. |
| pCurrGap | Int | Set by the software to track the length of the gap as it is created. |
| pLastGapIndx | Int | Used by the software to track which GapInches value was last used. |
| pStartIndxCurr | Int | Used by the software to track the start position of the container as it is copied to the next belt. |
| pStartIndxNext | Int | Used by the software to track the induction point index on the next belt as the container is copied to it. |

# An Example Gap Optimizer

This example is for a configuration with four belts.  The exit belt encoder provides ticks for the fourth logical belt’s shifter.

Infeed belt

420 ft/min

Belt 1

Exit belt

420 ft/min

Belt 2

Belt 3

Belt 4

Induct PE

Logical belt 1

End of belt update point

Box 1

Box 2

Sim belt

Logical belt 2

Logical belt 3

Logical belt 4

| ServoGapOpt (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | EnableFlag | UpdTimerFlag | LogLevel | GapInches |
| Belt 1 | Y | N | Level 0 | 12 |
| Belt 2 | Y | N | Level 0 | 12 |
| Belt 3 | Y | N | Level 0 | 12 |
| Belt 4 | Y | N | Level 0 | 12 |
| Exit Belt | Y | N | Level 0 |  |

| ServoGapOpt (part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PitchFlag | Gain | NextBelt | UpdPntBeltEnd | IndPntLogical | Machine |
| N | 1.0 | Belt 2 | Belt 1 End of Belt | Logical Belt 1 | CP2 |
| N | 1.0 | Belt 3 | Belt 2 End of Belt | Logical Belt 2 | CP2 |
| N | 1.0 | Belt 4 | Belt 3 End of Belt | Logical Belt 3 | CP2 |
| N | 1.0 | Belt 5 | Belt 4 End of Belt | Logical Belt 4 | CP2 |
| N | 1.0 | N/A | Invalid | Invalid | CP2 |

| ServoGapOpt (part 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| RunIO | MinSpeed | MaxSpeed | AccelMilliSec | SpdControl |
| Belt 1 RUN | 90.00 | 100.00 | 100.00 | Belt 1 |
| Belt 2 RUN | 90.00 | 100.00 | 100.00 | Belt 2 |
| Belt 3 RUN | 90.00 | 100.00 | 100.00 | Belt 3 |
| Belt 4 RUN | 90.00 | 100.00 | 100.00 | Belt 4 |
| Exit Belt RUN | 90.00 | 90.0 | 100.00 | Exit Belt |

| ServoGapOpt (part 4) |  |  |  |
| --- | --- | --- | --- |
| TimeBasedEncoder | EncTmrPreset | pGapInTicks | pNumBoxes |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |

# Gap Optimizer Simulator Example

This example is for a simulation configuration with four belts.  The first belt is used to create the containers.  The exit belt is used to create a shifter where the results of the previous four belts can be reviewed.

Infeed belt

420 ft/min

Belt 1

Exit belt 420 ft/min

Belt 2

Belt 3

Belt 4

Induct PE

Logical belt 1

End of belt update point

Box 1

Box 2

Sim belt

Logical belt 2

The Optimizer configuration:

| ServoGapOpt (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | EnableFlag | UpdTimerFlag | LogLevel | GapInches |
| Sim Belt | Y | Y | Level 0 | 12 |
| Belt 1 | Y | Y | Level 0 | 12 |
| Belt 2 | Y | Y | Level 0 | 12 |
| Belt 3 | Y | Y | Level 0 | 12 |
| Belt 4 | Y | Y | Level 0 | 12 |
| Exit Belt | Y | N | Level 0 |  |

| ServoGapOpt (part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PitchFlag | Gain | NextBelt | UpdPntBeltEnd | IndPntLogical | Machine |
| N | 1.0 | Belt 1 | Sim Belt End of Belt | Sim Logical Belt 1 | CP2 |
| N | 1.0 | Belt 2 | Belt 1 End of Belt | Logical Belt 1 | CP2 |
| N | 1.0 | Belt 3 | Belt 2 End of Belt | Logical Belt 2 | CP2 |
| N | 1.0 | Belt 4 | Belt 3 End of Belt | Logical Belt 3 | CP2 |
| N | 1.0 | Exit Belt | Belt 4 End of Belt | Logical Belt 4 | CP2 |
| N | 1.0 | Belt 1 | Sim Belt End of Belt | Sim Logical Belt 1 | CP2 |

| ServoGapOpt (part 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| RunIO | MinSpeed | MaxSpeed | AccelMilliSec | SpdControl |
| Sim Belt RUN | 90.00 | 90.00 | 100.00 | Sim Belt |
| Belt 1 RUN | 90.00 | 100.00 | 100.00 | Belt 1 |
| Belt 2 RUN | 90.00 | 100.00 | 100.00 | Belt 2 |
| Belt 3 RUN | 90.00 | 100.00 | 100.00 | Belt 3 |
| Belt 4 RUN | 90.00 | 100.00 | 100.00 | Belt 4 |
| Exit Belt RUN | 90.00 | 90.0 | 100.00 | Exit Belt |

| ServoGapOpt (part 4) |  |  |  |
| --- | --- | --- | --- |
| TimeBasedEncoder | EncTmrPreset | pGapInTicks | pNumBoxes |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |
| N | 1.00 | 0 | 0 |

The simulation configuration:

| GapOptSim (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | EnableSrt | CreateBoxes | GapBelt | FirstBelt | CreateSorterTmr | LengthInches |
| Sim Belt | Y | Y | Sim Belt | Y | Sim Belt | 20 |
| Belt 1 | Y | N | Belt 1 | N | Belt 1 | 0 |
| Belt 2 | Y | N | Belt 2 | N | Belt 2 | 0 |
| Belt 3 | Y | N | Belt 3 | N | Belt 3 | 0 |
| Belt 4 | Y | N | Belt 4 | N | Belt 4 | 0 |
| Exit Belt | Y | N | Exit Belt | N | Exit Belt | 0 |

| GapOptSim (part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| GapInches1 | GapInches2 | InPoint | CpyNextUpd | NextInpoint |
| 8 | 16 | Sim Belt | Sim Belt Cpy to Belt 1 | Gap Opt Belt 1 |
| 0 | 0 | GapOpt Belt 1 | Belt 1 Cpy to Belt 2 | Gap Opt Belt 2 |
| 0 | 0 | GapOpt Belt 2 | Belt 2 Cpy to Belt 3 | Gap Opt Belt 3 |
| 0 | 0 | GapOpt Belt 3 | Belt 3 Cpy to Belt 4 | Gap Opt Belt 4 |
| 0 | 0 | GapOpt Belt 4 | Belt 4 Cpy to Belt 5 | Exit Belt |
| 0 | 0 | Exit Belt | Exit Belt Cpy to Nowhere | Invalid |

# Encoders Table Configuration for the Example

Building upon the previous example, the Encoders table is configured with twelve encoders.  Six encoders are associated with the six belts:

- Sim Belt

- Belt 1

- Belt 2

- Belt 3

- Belt 4

- Exit Belt

The additional six encoders are associated with the six logical belts:

- Sim Logical Belt

- Belt 1 Logical Belt

- Belt 2 Logical Belt

- Belt 3 Logical Belt

- Belt 4 Logical Belt

- Exit Logical Belt

The logical belt encoders are configured using the DupToMemBit feature, where they are linked to their corresponding physical belts.

| Encoders (part 1) |  |  |  |
| --- | --- | --- | --- |
| Encoder Name | Encoder I/O | Encoder Timer | Ticks Per Foot |
| Sim Belt | Sim Belt Encoder | tmSimBeltEncoder | 12 |
| Belt 1 | Belt 1 Encoder | tmBelt1Encoder | 12 |
| Belt 2 | Belt 2 Encoder | tmBelt2Encoder | 12 |
| Belt 3 | Belt 3 Encoder | tmBelt3Encoder | 12 |
| Belt 4 | Belt 4 Encoder | tmBelt4Encoder | 12 |
| Exit Belt | Exit Encoder | tmExitBeltEncoder | 12 |
| Sim Log Encoder | Sim Log Encoder | tmSimLogEncoder | 12 |
| Log 1 Encoder | Log 1 Encoder | tmLog1Encoder | 12 |
| Log 2 Encoder | Log 2 Encoder | tmLog2Encoder | 12 |
| Log 3 Encoder | Log 3 Encoder | tmLog3Encoder | 12 |
| Log 4 Encoder | Log 4 Encoder | tmLog4Encoder | 12 |
| Exit Log Encoder | Exit Log Encoder | tmExitLogEncoder | 12 |

Note the DupToMemBit selections linking each physical and logical belt:

| Encoders (part 2) |  |  |  |
| --- | --- | --- | --- |
| Target FPM | Tolerance_FPM | DupToMemBit | Delay Timer |
| 420 | 105 |  | tmDelaySimBelt |
| 420 | 105 | Sim Log Encoder | tmDelayBelt1 |
| 420 | 105 | Log 1 Encoder | tmDelayBelt2 |
| 420 | 105 | Log 2 Encoder | tmDelayBelt3 |
| 420 | 105 | Log 3 Encoder | tmDelayBelt4 |
| 420 | 105 | Log 4 Encoder | tmDelayExitBelt |
| 420 | 105 |  | tmDelaySimLog |
| 420 | 105 |  | tmDelayLog1 |
| 420 | 105 |  | tmDelayLog2 |
| 420 | 105 |  | tmDelayLog3 |
| 420 | 105 | Exit Log Encoder | tmDelayLog4 |
| 420 | 105 |  | tmDelayExitLog |

| Blank Table |  |  |
| --- | --- | --- |
| Field | Type | Description |