| # FortnaPlus Control Sorter Control Module |  |
| --- | --- |
| Document Revision: 1.8 |  |
| Date: 25-Nov-2020 |  |

Revision; Date; Author; Revision Summary. 1.2; 04/18/2013; B C Conklin; Added SrtSimConfig SimCrrCsu notes. 1.3; 04/22/2013; P Rotter; Added description of external simulation scan list feature. 1.4; 06/17/2013; T R Kremser; Add information about SrtFPLite protocol. 1.5; 12/03/2013; B C Conklin; Updated the options for Dynamic Re-assignment and Sorter Stops when lanes are full.. 1.6; 02/26/2014; B C Conklin; Updated SrtScanBoss to add a feature to modify the CRR Message based on a Tote PE. Added Update Point to collect the carton length.. 1.7; 16-Nov-2020; T R Kremser; Describe the CONCAT scans feature.. 1.8; 25-Nov-2020; T R Kremser; Updates and general refresh.

Table of Contents

1.	Sorter Control	4

2.	Activating the Module	4

3.	The Sorter Application Configuration Table – Top Level	4

4.	The Destination Lane Configuration Table	8

5.	The Scan Exceptions Configuration Table	11

6.	The Lane Not Avail Configuration Table	16

6.1.	Option 1 – Stop or reassign on sorters without recirculation – No update points needed	16

6.2.	Option 2 – Stop or reassign on sorters without recirculation – Update points for each divert	18

6.3.	Option 3 – Dynamically reassign on sorters with recirculation	20

7.	The Bad Gap Configuration Table	24

8.	The Round Robin Config Table	25

9.	Sorter Tracking Tables	25

10.	Message Interface Configuration Example	29

11.	CommCore Interface	30

11.1.	Status and Error Messages	30

11.2.	FPC Message Tables	32

11.2.1.	CRR – Container Route Request	33

11.2.2.	CSM – Container Scan Message	35

11.2.3.	CSU – Container Status Update	35

11.2.4.	DCM – Divert Confirm Message	36

11.2.5.	RHS – Request Host Status	37

12.	Sorter Simulation	38

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Sorter Control

The Sorter Control Module uses standardized tracking control functions (shifter / sorter), along with specific sorter logic to provide scanner input and error processing, container tracking, and management of full conditions, bad gap processing and WCS interface management.

Multiple sorters may be configured within a single AC.

# Activating the Module

The sorter module is activated in several separate portions as “Batch jobs” using table Batches:

- Srt App SERV – Required to run the SERV portion for managing messages to and from the WCS business logic layer (CommCore).

- Srt App CTRL – Required to run the CTRL portion for managing hardware control logic.

- Srt Sim SORT – Required to run the simulation features using the SORT process.

The various tables required to configure and use these features are described below.

# The Sorter Application Configuration Table – Top Level

The “App Sorter Configuration” or SrtAppConfig table is the top-level definition of a sorter.  It creates the logical representation of the sorter entity.  Other tables are used to define details about the physical and logical characteristics of the sorter.  They will all reference the entry in SrtAppConfig.  This top-level table is used to define the interface to CommCore, how containers with no valid destination are to be handled, the amount of log file detail and the motor that controls the main sorter.

| SrtAppConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a descriptive name for the sorter. |
| Status | String | Indicates the current status of the Sorter including any configuration errors detected. |
| MtrStatus | String | Indicates the current status of the sorter motor, based on the lane full processing logic. The motor may be stopped by other logic that is not included in the StrAppConfig status. |
| SendAvailStatus | Y/N | Set to Y to include the list of unavailable lanes in the CRR Message. CommCore can use the list to assist in making routing decisions. The unavailable lanes will appear as a comma-separated list in the sZone1 field of the CRR message (column Dat12). Set to N for a CRR message that does not include the list of unavailable lanes. A lane is unavailable if it is full or disabled. |
| AvailAppSorter | Int | This field may be used to report the list of unavailable lanes from a different sorter via the CRR message. A value of 0 will report the unavailable lanes for this sorter. If the unavailable lanes status from another sorter defined on this AC is needed, set the value to the row number in this table of the desired sorter. For example, if routing to the shipping sorter from the picking sorter may only occur when the shipping zone is not full, the status of the shipping sorter is needed to route the container to the correct location. |
| ErrConfig | PopUp | Select the error processing method for the sorter. This will determine how a container will be handled if the destination lane or lanes is/are not available because they are full or disabled. - N/A: The conveyor will not be stopped, and cartons will not be dynamically re-assigned. - Stop if Not Avail, Single lane assigned: For applications where the sorter needs to stop if the lane is unavailable. Also set AssignIfFull to Y. The sorter will stop when the container reaches the window defined by the SrtLaneNotAvail table. - Stop if Not Avail, Multiple lanes assigned: For applications where the sorter needs to stop if none of the assigned lanes are available. The sorter is stopped to wait for one of the lanes to become available. Once a destination becomes available, the container will be assigned, and the conveyor will start. Note: The SrtLaneNotAvail table must have at least one entry for this sorter or the sorter motor will not be stopped. In addition, the sorter will be stopped if a container is in transit to a lane and the lane is no longer available and the container cannot be re-assigned to a different destination. The SrtZoneLane table can be configured to define the type of lane (TwoSidedShoe) and the side of the sorter the lane is on (RightSideDivert). Some reassignments may not be physically possible. - Reassign and Recirc: For applications where the sorter has recirculation and containers are assigned to multiple destinations. Containers will be dynamically reassigned to down-stream lanes if the current assignment becomes unavailable. The SrtZoneLane table can be configured to define the type of lane (TwoSidedShoe) and the side of the sorter the lane is on (RightSideDivert). Some reassignments may not be physically possible. |
| LogLevel | PopUp | Select the log level for the sorter. Level 2 is a good choice. |
| CartonIdUpdate | Radio | Select this to change the ConfirmScan value in the tracking table to the string received in the CSM or CRR response message. Deselect this to leave the ConfirmScan string unchanged. This feature may be used if the CartonID string must be parsed by CommCore and returned as a new string. This feature may be tested using a CRR message and the “SrtTest” or “SrtTestSeq” protocol. These protocols simulate CommCore. If the SendTestCRR table, Param1 column, Row 1, is set to the “CartonIdUpdate1” string, the first parameter of a comma-delimited string will be returned. If the column is set to the “CartonIdUpdate2” string, the second parameter of a comma delimited string will be returned. |
| CrrMsgTable | PopUp | Select the table to use for CRR messages. Each sorter should use its own separate message table to prevent delays in processing sorter lane assignment. Select the MsgCR1 table for the 1st sorter. Nine tables are available to choose from. |
| DcmMsgTable | PopUp | Select the table to use for DCM messages. Start with the MsgDC1 table. |
| CsuMsgTable | PopUp | Select the table to use for CSU messages. It is OK to share the MsgDC1 table for this use. If the CommCore configuration expects CSU messages on the same port as DCM messages, use MsgDC1, otherwise choose another table such as one of the MsgGP tables not in use elsewhere. |
| CsmMsgTable | PopUp | Select the table to use for CSM messages. Each sorter should use its own separate message table to prevent delays in processing sorter lane assignment. Select the MsgGP1 table for the 1st Sorter on this PC. CSM Messages are only sent if the SendCRRUpdatePt is selected in the SrtScanBoss table for the sorter. This feature is used in applications where the scan event needs to send a CSM first, then later send a CRR. The CSM is sent at the scan update point, the CRR is sent at the update point selected in the SendCRRUpdatePt column. This is done to give CommCore time to retrieve routing information from the customer’s host system. |
| SecondCrrMsgTable | PopUp | This is used if a second scan point is designated on a sorter. It is activated if SrtScanBoss_SecondScanZone is populated. Choose a message table to handle the second CRR messages. |
| SecondDcmMsgTable | PopUp | This is used if a second scan point is designated on a sorter. It is activated if SrtScanBoss_SecondScanZone is populated. Choose a message table to handle the second DCM messages. |
| SecondCsuMsgTable | PopUp | This is used if a second scan point is designated on a sorter. It is activated if SrtScanBoss_SecondScanZone is populated. Choose a message table to handle the second CSU messages. |
| ControlMachine | PopUp | Select the AC that controls this sorter. |
| SorterCnvMtr | PopUp | If a motor is selected here, and the Stop if Not Avail option is selected in the ErrConfig field, the system will be able to start and stop the conveyor. Similarly, the scan errors tables ScnDeviceErrIO and/or ScnZoneErrIO can be configured to stop the AppSorter when an error occurs. |
| MtrACTION | PopUp | Select the action required to run the conveyor under normal conditions. The control logic will use the inverse when it needs to stop the conveyor. |
| MtrBIT | Int | If Enable/Disable is chosen for MtrACTION, enter the bit number here to use for the disable. |
| NotAvailErrSig | PopUp | Select a memory bit that is configured in the Errors table to indicate the sorter has stopped because a lane is not available. |
| NotAvailACTION | PopUp | Select the action required to activate the NotAvailErrSig error. |
| NotAvailBIT | Int | If Enable/Disable is chosen for NotAvailACTION, enter the bit number here to use for the disable. |
| ScanErrSig | PopUp | Select a memory bit that is configured in the Errors table to indicate the sorter has stopped because a scan error has occurred. |
| ScanACTION | PopUp | Select the action required to activate the ScanErrSig error. |
| ScanBIT | Int | If Enable/Disable is chosen for ScanACTION, enter the bit number here to use for the disable. |
| DataErrLight | PopUp | Select a beacon light output or a memory bit that is configured in the Errors table to indicate that a data error has occurred. |
| DataACTION | PopUp | Select the action required to activate the DataErrLight error. |
| DataBIT | Int | If Enable/Disable is chosen for DataACTION, enter the bit number here to use for the disable. |
| MtrMinStopTime | PopUp | Select a timer that will ensure the sorter motor in SorterCnvMtr stays stopped for a minimum amount of time when a lane is not available or when scan errors occur. This delay is used to protect the sorter hardware by ensuring that the sorter comes to a complete stop when an error occurs, even if the error clears in a shorter time. |
| SndShipInCRR | PopUp | Take data from ConfirmShip in the tracking record and load it into the CRR (or CSM) message Param1 thru Param5 fields (Dat22 thru Dat26), as defined by the selected value. |
| SndPackInCRR | PopUp | Take data from ConfirmPack in the tracking record and load it into the CRR (or CSM) message Param1 thru Param5 fields (Dat22 thru Dat26) , as defined by the selected value. |
| pErrorFlag | Int | Used by the software to signal an error has occurred. |
| pResetFault | Int | Used by the software to reset an error. |
| pMaxResponse | Float | Used by the software to indicate the maximum response time from the CommCore server. |
| pGrtHalfSecond | Int | Used by the software to indicate the number of responses that have exceeded 0.5 second. |
| pGrtQtrSecond | Int | Used by the software to indicate the number of responses that have exceeded 0.25 second. |
| pNumSamples | Int | Used by the software to indicate the number of responses processed. |
| pTrigError | Int | Used by the software to trigger an error (associated with error messages from the CommCore server) |
| pErrDesc | String | Used by the software to store the error description triggered by the error message from the CommCore server. |
| pScanError | Int | Used by the software to manage scan errors. |
| pDataError | Int | Used by the software to manage data errors. |
| pWrkScanError | Int | Used by the software to manage scan errors. |
| pWrkDataError | Int | Used by the software to manage data errors. |

# The Destination Lane Configuration Table

The “Destination Lane Configuration” or SrtZoneLane table defines the sorter lanes.  It creates the links between the sorter Outpoints and the CommCore host zones.

Multiple host zones may be assigned to the same Outpoint.

SrtZoneLane also is used to define physical and logical attributes of each lane and its behavior.

| SrtZoneLane Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description of each sorter lane. |
| Enabled | Radio | When set, the lane is available for assignment. Note: If the sorter is configured to “Stop if Not Avail” disabling a lane will stop the sorter if a container is assigned to that lane. |
| Status | String | Indicates the current status of the lane and any configuration errors detected. |
| AppSorter | PopUp | Select the sorter associated with this lane. |
| AssignIfFull | Radio | When set, the lane assignment will be made even if the lane is full. When unset, the lane assignment will not be made when the lane is full unless the sorter is configured to send a negative divert confirm. - In applications where only one lane choice is provided for a given container, and the sorter needs to stop if the lane is disabled or full, set AssignIfFull, and set Stop if Not Avail in SrtAppControl_ErrConfig. The sorter will stop when the container reaches the window defined in the SrtLaneNotAvail table and wait for the lane to become available. - In applications where more than one lane choice is provided and the sorter needs to stop if none of the lanes can be assigned, do not set AssignIfFull, but set Stop if Not Avail in SrtAppControl_ErrConfig. The sorter will stop if none of the lanes can be assigned. The sorter will wait for one of the lanes to become available. |
| AssignLaneZero | Y/N | When set to Y, the system will assign lane zero if the Lane column has no Outpoint selected. This option is used to define routing instructions to send a container straight. When set to N, the lane selection of N/A will be processed as not available. |
| Lane | PopUp | Select the Outpoint associated with this lane. |
| FullClearTimer | PopUp | If a timer is selected here instead of in the Outpoints table, the container will divert into the lane even if the lane goes full while the container is in transit. This is used in applications where a sorter does not have recirculation and the container must be diverted into the lane even if the Outpoint is full. However, the sorter must be configured to stop if the lane is full and a container is in transit to the lane. - If the lane is full when another container is assigned to the lane, the system will stop if the Stop if Not Avail option is selected, as described in the previous section. - If Outpoints_Full_Clr_Timer_Name is set instead of using this field, the container will not divert if the timer completes as the container reaches the fire point for the lane. |
| LaneEnableSignal | PopUp | If this field is used instead of Outpoints_EnableIO, the container will divert into the lane even if the signal changes to not enabled while the container is in transit. This is typically used in applications where a sorter does not have recirculation and the container must be diverted into the lane even if the Outpoint is not enabled. However, the sorter must be configured to stop if the lane is full and a container is in transit to the lane. - If the lane is not enabled when a container is assigned to the lane, the system will stop if the Stop if Not Avail option is selected, as described in the previous section. - If Outpoints_EnableIO is defined, the container will not divert if the enable signal turns off as the container reaches the fire point for the lane. |
| HostZone | String | The HostZone string is the agreed-upon name that uniquely identifies the lane for both FPC and CommCore. In CommCore, this is referred to as the Divert Point. When using the SrtFPLite protocol, the HostZone string must match the lane name encoded in the tote barcode. |
| RestoreLane | PopUp | This column is used to store the original lane assignment for a lane. Lanes may be temporarily assigned to a different Outpoint. The function “Srt App RstLane” may be configured in a Batch to restore the lanes at a specific time or it may be configured in a menu to restore lanes from operator control. Example: An operator may want to re-assign the zone UPS from Lane1 to Lane2. A batch runs every day at midnight to restore the lanes back to their original settings. The operator would select Lane2 in the lane column and the “Srt App RstLane” function would be added to the Batches table to trigger at midnight. |
| RndRobinEnabled | Y/N | This column is used to enable a lane for assignment by the Round Robin Assignment Algorithm, as configured in table SrtRndRobin. SrtRndRobin is used in applications where a sorter does not have a scanner, but lanes need to be selected based on a round-robin algorithm. This column is only to be used if table SrtRndRobin is configured and enabled. Example: A tape dunnage area with two tape dunnage lanes may be assigned product on a round-robin basis to spread the load between them. |
| AvailForAssign | PopUp | If a part is selected in this column, the state of the IO will determine if the lane will be considered for assignment. The column is used to disable lane assignment without stopping the sorter (unless none of the lane choices are available). This feature is used in applications where a sorter is configured to Stop if Not Avail and physical “Lane Enable” switches are provided. - Example: When the Stop if Not Avail option is selected in SrtAppControl_ErrConfig, the sorter will stop if a container is in transit to the lane when the assignment is made and the lane is full or is not enabled. The AvailForAssign signal is only used for assignment and will not stop the sorter if the signal turns off while a carton is in transit after the assignment has been made. - However, if a container cannot be assigned to a lane because none of the lanes are AvailForAssign at the time of assignment, the sorter will be stopped. - If AvailForAssign is set to INVALID, or if the part selected is ON, the lane will be considered in making the assignment. |
| AssignIfNotEnabled | Radio | If set, a container will be assigned to the lane even if the LaneEnableSignal is OFF or if the EnableIO is OFF in the Outpoints table, otherwise the lane will not be considered for the assignment. Example: If the energy management module disables a lane and the EnableIO signal is selected in the Outpoints table, a container will be assigned to the Outpoint even if the EnableIO signal is OFF. Once the container is assigned to the Outpoint, the wake-up function in the energy management module will restart the motor for the lane when the container reaches the wake-up point on the shifter. |
| DSRRequest | PopUp | If the lane status is to be read from the CommCore server, select the DSR message from table DSRConfig that will read the status of this lane. Each DSR message may contain up to 10 lanes of status information. If the sorter has more than 10 lanes, the status must be read from the CommCore server using multiple messages. |
| TwoSidedShoe | Y/N | If a sorter is configured to stop in the SrtLaneNotAvail table, the system will check if the lane can be reassigned. If it can be reassigned, the allowed lanes for reassignment are determined by checking if the sorter is a two-sided shoe sorter and if the new lane is on the same side as the old one. Column RightSideDivert indicates the side associated with a lane. |
| RightSideDivert | Y/N | This field indicates the side associated with a lane for a two-sided shoe sorter. |
| ResetExpected | Y/N | If a sorter is configured with a second CRR, set this column to Y. This is a special case where a sorter is configured with a second CRR to process a failed divert. Example: A sorter may be configured with a second CRR to determine if a diverter has failed (the carton cannot recirculate). The second CRR can be configured to stop the sorter if the carton does not belong on the sorter, or if it is an unknown carton. |

# The Scan Exceptions Configuration Table

The Scan Exceptions Configuration, or SrtScanBoss table defines how a sorter will handle disposition of containers with scan or size errors.

Note:  ScanError and ScanConflict routing assignments are only used by the SrtFPLite protocol.  These errors are normally handled by CommCore and are reflected in the lane assignment choices returned in the CRR response.

| SrtScanBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for the scanner or scale. |
| ErrorMsg | String | Updated by the software to indicate any errors detected during processing of the last message. |
| ScanStatus | String | Updated by the software to indicate the status of the scan zone. |
| AppSorter | PopUp | Select the sorter associated with this device. |
| ScanZone | PopUp | Select the scan zone associated with this shifter. |
| AssignUpdatePt | PopUp | Select an update point if the sorter consists of more than one shifter and if the response from CommCore is occasionally received after the container has shifted from the first shifter to the second shifter. The update point should be located on the second shifter at least one tick prior to the first divert point. If this update point is defined, an attempt will be made to find data associated with cartons that has not been passed properly along the chain of shifters. This is handled automatically as of V10.36.0. The logic is still present and active, however. |
| ScanError | PopUp | Only used by the SrtFPLite protocol. Scan errors are normally handled by CommCore. For the SrtFPLite protocol, select the lane that will be used to handle scan errors (Scan conflicts are not included). |
| ScanConflict | PopUp | Only used by the SrtFPLite protocol. Scan errors are normally handled by CommCore. For the SrtFPLite protocol, select the lane that will be used to handle scan conflicts. In some systems the ScanConflict column is set to N/A and scan conflicts are handled by sending the container(s) around for a second attempt. This technique is sometimes used in systems where “side by side” containers can occur. However, in that case, containers with two labels will recirculate forever. |
| CommErr | PopUp | Select the lane that will be used to handle CommCore errors. If a lane is defined here, it will be assigned to each container when routing data is requested. If CommCore does not respond, the container will be routed to this lane. When a CRR response is received, the CommErr lane will be overwritten by the result from normal routing processing. |
| SpcLane | PopUp | Select the lane associated with ScnScanDevice_ScanType types SpcGoodRead1 and SpcGoodRead2. This special configuration device is used to assign a lane on a sorter based on a GoodScan for the scanner selected as SpcGoodRead1. If a scan error occurs, the lane will not be assigned. This is used to pre-assign a lane on a two-sided shoe sorter, causing the shoes to be pulled to the correct side. |
| Lane | Int | Updated by the software to indicate the Outpoint assigned for the last event processed. |
| TooLateUpdatePt | PopUp | Select an update point if the sorter type is a two-sided shoe sorter and the CommCore lane assignment must be ignored when the data arrives too late. The update point should be located just prior to the shoe pull-over (at least one tick prior to the shoe pull-over point). The CRR message will not be processed and the container will not be assigned a destination if the message arrives after the container has reached this update point. Use CommErr to assign an exception handling lane for these containers. The TooLateUpdatePt can also be used to generate a negative divert confirm message. For this use, locate the update point just before the first divert. |
| SendCRRUpdatePt | PopUp | Select an update point here to activate Container Scan Message (CSM) processing. The sorter will send the CSM to CommCore when the scan event occurs, then when the container reaches this update point, the standard CRR message will be sent. This option is used in applications where the WMS system provides the lane assignment at the scan/weigh event, and extra time is required to retrieve the information. Locate the update point at least 500 mSec before the first divert point to allow time for the CRR message reply to be received and processed. The distance/time from the scan point to this update point must be sufficient to allow the CSM to be sent from the AC to CommCore, then from CommCore to the WMS system and back before the CRR is requested. A typical system will require that the scanner be located at least 6 seconds upstream of the first divert. |
| LengthUpdatePt | PopUp | Select an update point here if the container length, calculated from encoder ticks, needs to be captured and stored in the tracking record. The update point must be located at least one tick prior to the scan point. The container must be completely inducted before the scan point is reached. The data is included in the CRR message sLength field (Dat29), which is created at the scan event. |
| TooTallUpdatePt | PopUp | Select an update point here to check the TooTallSignal. If TooTallSignal is ON when the container reaches TooTallUpdatePt, and ContID is “NOLABEL”, the CRR message is sent with sStatus = “ERROR” and sStatusMsg = “NOREAD” (COMM_ERROR_NOREAD). |
| TooTallSignal | PopUp | Select the IO part (probably a photo eye) that will indicate a “too tall” condition. It will be interpreted at TooTallUpdatePt. |
| SecondCRRUpdPt | PopUp | Select an update point here to generate a second CRR message. This may be used to re-verify the destination for a container without the need for a new scanner or scale reading. In addition to any required tracking information, ConfirmScan, ScanQty, Height, Width, Length and Scale are copied to a fresh tracking record prior to generation of the new CRR message. Example 1: A sorter may be configured with a second CRR to determine if a diverter has failed on a sorter where the carton cannot recirculate. The second CRR can be configured to stop the sorter if the carton does not belong on the sorter, or if it is an unknown carton. Note: Set SrtZoneLane_ResetExpected to allow the Outpoint to fire a second time. Example 2: A sorter may be configured with a second CRR to process late responses from the WMS. In this case the CRR is sent before the first lane of the sorter. The second CRR is located past the first lane to allow more time for CommCore to get routing information from the WMS. Note: For Print Apply applications use the “PnA to Sorter Convert” table [PStConvertTable] to configure a second sorter scan event. |
| SecondScanZone | PopUp | Select a scan zone to be used for the second CRR message. |
| SecondChkDiverted | Y/N | Set to Y to decide if the second CRR should be sent based on the container’s divert status. The second CRR will be sent if the Outpoint did NOT fire. Set to N to skip the divert status check and always send the second CRR. Note: SortData_Expected_Lane records the container’s divert status. It is only set if the lane Outpoint has been fired for the container. |
| ToteUpdatePt | PopUp | Select the update point to check for ToteSignal. ToteSignal may be a photoeye that detects a tote, or it may be a photoeye used to detect a tall container. If the ToteSignal is ON while the container passes the PE as it reaches this update point, the ToteFound flag is set in the tracking table. The ToteFound flag will be considered when the CRR is created. If ToteFound is set: - If the ToteID field is empty, the sContainer field in the CRR message will not be changed. If the ToteID field contains a string, the string will be copied into the sContainer field. For example: Set the ToteID field to “TOTE”, and sContainer will be set to “TOTE”. - If the ToteErr field is empty, the sStatus field in the CRR message will not be changed. If the ToteErr field contains a string, the string will be copied into the sStatus field. Set the ToteErr field to “Okay” or “ERROR” to denote a good or bad response, respectively. - If the ToteErrMsg field is empty, the sStatusMsg field in the CRR message will not be changed. If the ToteErrMsg field contains a string, the string will be copied to the sStatusMsg field. For example: Set the ToteErrMsg field to “Tote Found” and sStatusMsg will be set to “Tote Found”. Note: If these strings are left empty, the CRR message will not be altered – default strings will be sent in the CRR message. |
| ToteSignal | PopUp | Select the photoeye signal that will turn ON when a tote is present. The photoeye will trigger a response in the CRR message as described in the ToteUpdatePt described above. |
| ToteUpdErrOnly | Y/N | Set to N to update the CRR message even if a good scan occurs. Set to Y to update the CRR message only if a scan error occurs. An error is indicated to CommCore if the sStatus column is set to “ERROR”. Typical scan errors are “NOSCAN” and “NOREAD”. |
| ToteErr | String | Enter the string that will be loaded into the CRR message sStatus field when ToteSignal is ON at ToteUpdatePt. If empty, the message field is not changed. |
| ToteErrMsg | String | Enter the string that will be loaded into the CRR message sStatusMsg field when ToteSignal is ON at ToteUpdatePt. If empty, the message field is not changed. |
| ToteID | String | Enter the string that will be loaded into the CRR message sContainer field when ToteSignal is ON at ToteUpdatePt. If empty, the message field is not changed. |
| ToteUpdCSMAlso | Y/N | Set to N to update only the CRR message at ToteUpdatePt. Set to Y to update both the CRR and CSM messages at ToteUpdatePt. |
| ConcatScansUpdatePt | PopUp | Select an update point to define where a scan concatenation event should occur. Define up to four individual scans to concatenate in table ScnScanDevice using the ScanTypes “Concat A” through “Concat D”. ScnScanZone_ConcatScans defines the number of scans to concatenate, up to 4. ScnScanZone_Delimiter defines delimiter characters to be inserted between the concatenated elements. Allow enough room between the last scan point and this update point so the longest container will tail out of the scan point before reaching this update point so the NoScan processing will work correctly. NoRead and NoScan processing will be applied to each element of the scan before the concatenation is performed. |
| pToteTrkMnu | Int | Set by the software to indicate a box is over the update point. |
| pToteTrkIndx | Int | Set by the software to indicate a box is over the update point. |
| pToteFound | Int | Set by the software to indicate ToteSignal has turned ON while the box is over the ToteUpdatePt update point. The field is reset when the box is no longer over the update point. |

# The Lane Not Avail Configuration Table

The Lane Not Avail Configuration, or SrtLaneNotAvail table defines how a sorter will assign containers to alternate destinations.  It also can be used to configure a sorter to stop if no destinations are available.

  - Option 1 – Stop or reassign on sorters without recirculation – No update points needed

This option requires only a single entry in table SrtLaneNotAvail for each shifter.  The logic to stop the sorter and to process reassignments is based on the Outpoint locations.  An update point is not required for each divert point.

Note:  Use Option 2 if the stop points or the reassignment points need to be adjusted for each divert point.

This option provides the following features:

- After a destination assignment message (CRR) has been received, the sorter will stop if none of the assigned destinations are available.  The assigned destinations can be found in the sZone1 through sZone10 fields of the CRR response, and in the Z1 through Z10 fields in the tracking record.
Note:  If only one destination is provided, the assignment is made, allowing the container to proceed to the area just prior to the diver point even if that destination is unavailable at the time of assignment.

- If the container is in transit to a destination lane and the lane becomes unavailable the logic will reassign the container to an alternate destination if one is available and the alternates are still physically reachable.  This includes checking for destinations on the same side for a two-sided shoe sorter.  A lane may change status at any time.  The lane may go to a full state or a lane may be disabled via an input signal.  The sorter may be configured to stop if any of these conditions occur.

- The sorter will stop when the container reaches a defined window if no destination is available.  See section 4 for details on how to configure this.  See below for details on how the algorithm works.  Use the ErrConfig setting in table SrtAppControl to select Stop if Not Avail to configure a sorter to stop.  See section 3 for details.  This is only necessary for sorters that do not have recirculation capability.  The motor to stop is configured in table SrtAppControl, column SorterCnvMtr.

Outpoint-1 Search Area

Outpoint-2 Search Area

Outpoint-1

MinReAssignTicks

MinStopTicks

StartTicks

Outpoint-2

MinReAssignTicks

MinStopTicks

StartTicks

If the sorter is composed of multiple shifters, the software will treat the shifters as one large shifter when determining if the container should be reassigned to a new lane, or when determining if the sorter should be stopped.  However, only shifters included in the SrtLaneNotAvail table are searched for containers in transit.  Create a separate entry for each shifter in table SrtLaneNotAvail to include them in the check for containers in transit.

The logic for Option 1 looks for cartons in the window starting at the StartTicks location and ending at the MinStopTicks location.  This window is calculated for each Outpoint.  If the shifter does not have any Outpoints and the StartTicks are set to a large value (number of ticks from the induct point to the last Outpoint, including all shifters), the logic will look for containers in the window defined by the Induct point and the Dump point.

- Set MinStopTicks to a value that is equal to or greater than the number of ticks required for the sorter to drift to a stop.  Use the graphical shifter screen to determine an appropriate value:  Click the Points Info tab to display the positions of the containers on the shifter.  With the sorter running, wait for a container to hit the induct photoeye.  When the induct photoeye turns green, stop the sorter.  Wait until it comes to a complete stop, then use the Boxes Info tab to determine how far “in ticks” the container traveled during the deceleration ramp.  The second column of the Boxes Info tab indicates the current position of the nose of the container on the shifter.  Subtract the position of the induct photoeye (usually 30 ticks – see IN on the Points Info tab) to determine the minimum value for MinStopTicks.

- Set StartTicks to a value that is at least MinStopTicks + 10.  If you want all the sorter lanes to be enabled for reassignment or stopping, set the value to a number that is at least as large as the number of ticks to the last Outpoint location.

- Set MinReAssignTicks to a default value of 10.  This should allow enough time for reassignment before the box shifts into the Outpoint.
For example:  A 300 ft/min belt with 12 ticks per foot = 160 milliseconds for 10 ticks.  This would allow 160 milliseconds to complete the reassignment.  The system should have at least 100 milliseconds to process the reassignment, this allows time for the SERV process to determine the lane re-assignment and complete it before the SORT process diverts the container.

- Note:  For two-sided shoe sorters, each lane must be configured in the srtZoneLane table with TwoSidedShoe = Y, and RightSideDivert must be set as appropriate.  Cartons can only be reassigned to lanes if they are on the same side.

  - Option 2 – Stop or reassign on sorters without recirculation – Update points for each divert

This option requires an update point for each divert point.  An entry must be made in table SrtLaneNotAvail for each Outpoint, with an update point defined in FullUpdate for each one.  The update points allow fine-tuning of the stop points for each lane.

Printer Applicators

Search Area = (FullUpdate + FullUpdateTicks)

Carton and Applicator Pitch

Outpoint-1

FullUpdate

For example:  If a print apply line feeds a shipping sorter, and the shipping sorter does not have the ability to recirculate, the sorter must stop when a lane is full, but the stop point needs to be located where the stop will not cause a print apply applicator to fire during the “stop sequence”.  Each Outpoint needs to be located at a multiple of the print apply pitch.  Each stop point can be individually adjusted using the FullUpdate points.

- Add an update point for each “divert point” on the sorter.  If there are 20 lanes, 20 update points will be required.

- Select the SrtZoneLane record to link the entry to a specific Outpoint.

- Use the FullUpdateTicks field to define a window of ticks to look in for a container.  Set FullUpdateTicks to a value that is equal to or greater than the number of ticks required for the sorter to drift to a stop.  Use the graphical shifter screen to determine an appropriate value:  Click the Points Info tab to display the positions of the containers on the shifter.  With the sorter running, wait for a container to hit the induct photoeye.  When the induct photoeye turns green, stop the sorter.  Wait until it comes to a complete stop, then use the Boxes Info tab to determine how far “in ticks” the container traveled during the deceleration ramp.  The second column of the Boxes Info tab indicates the current position of the nose of the container on the shifter.  Subtract the position of the induct photoeye (usually 30 ticks – see IN on the Points Info tab) to determine the minimum value for FullUpdateTicks.

- Select the update point in the “FullUpdate” field of the appropriate record.  Adjust the update point to stop the conveyor at a location that is a multiple of the carton pitch.  Create a full condition for each lane and verify the print apply line does not fire the tamper while the carton is decelerating.

- Note:  For two-sided shoe sorters, each lane must be configured in the srtZoneLane table with TwoSidedShoe = Y, and RightSideDivert must be set as appropriate.  Cartons can only be reassigned to lanes if they are on the same side.

  - Option 3 – Dynamically reassign on sorters with recirculation

This option requires a sorter that has a path for recirculation.  In order for containers to be dynamically reassigned, multiple potential destinations must be available.

- If a destination is initially available, but the destination becomes full or unavailable while in transit, the container is reassigned to an alternate destination.

- When a reassignment is made, the next available down-stream destination is chosen.  This provides the most chances for a successful divert, as subsequent reassignments can be made at each lane until all possible destinations have been tried.

- With recirculation, even if all destinations are unavailable, the process can begin again on the next pass.

This option is enabled by selecting Reassign and Recirc in the ErrConfig field of the SrtAppControl table.

The SrtLaneNotAvail table must be configured to define the details of how the reassignment should occur.  The first method requires a single entry for each sorter and should be sufficient for most purposes.  The second method allows definition of individual update points for each Outpoint for fine-tuning, if needed.

- Configuration Type 1 – Single Row

Add one row for each sorter.  Each container will be checked as it approaches each Outpoint to determine if it should be reassigned.

  - Select the AppSorter associated with the entry.

  - Select the SrchShifter that matches the shifter associated with the Outpoint.

  - Set MinStopTicks to a value that is equal to or greater than the number of ticks required for the sorter to drift to a stop.  Use the graphical shifter screen to determine an appropriate value:  Click the Points Info tab to display the positions of the containers on the shifter.  With the sorter running, wait for a container to hit the induct photoeye.  When the induct photoeye turns green, stop the sorter.  Wait until it comes to a complete stop, then use the Boxes Info tab to determine how far “in ticks” the container traveled during the deceleration ramp.  The second column of the Boxes Info tab indicates the current position of the nose of the container on the shifter.  Subtract the position of the induct photoeye (usually 30 ticks – see IN on the Points Info tab) to determine the minimum value for MinStopTicks.

  - Set StartTicks to a value that is at least MinStopTicks + 10.  This defines the location prior to the Outpoint where the software will start looking for a container to reassign.  If you want all the sorter lanes to be enabled for reassignment or stopping, set the value to a number that is at least as large as the number of ticks to the last Outpoint location.

  - Set MinReAssignTicks to a default value of 10.  This should allow enough time for reassignment before the box shifts into the Outpoint.
For example:  A 300 ft/min belt with 12 ticks per foot = 160 milliseconds for 10 ticks.  This would allow 160 milliseconds to complete the reassignment.  The system should have at least 100 milliseconds to process the reassignment, this allows time for the SERV process to determine the lane re-assignment and complete it before the SORT process diverts the container.

  - Note:  For two-sided shoe sorters, each lane must be configured in the srtZoneLane table with TwoSidedShoe = Y, and RightSideDivert must be set as appropriate.  Cartons can only be reassigned to lanes if they are on the same side.

- Configuration Type 2 – Multiple Rows

Define individual update points for each Outpoint.  Individual Outpoint Configuration should not be required; however the system supports the configuration if needed.

  - Add an update point for each divert point on the sorter.  If there are 20 Lanes, 20 update points will be required.

  - Select the SrtZoneLane record to link the entry to a specific Outpoint.

  - Use the FullUpdateTicks field to define a window of ticks to look in for a container.  Set FullUpdateTicks to a value that is equal to or greater than the number of ticks required for the sorter to drift to a stop.  Use the graphical shifter screen to determine an appropriate value:  Click the Points Info tab to display the positions of the containers on the shifter.  With the sorter running, wait for a container to hit the induct photoeye.  When the induct photoeye turns green, stop the sorter.  Wait until it comes to a complete stop, then use the Boxes Info tab to determine how far “in ticks” the container traveled during the deceleration ramp.  The second column of the Boxes Info tab indicates the current position of the nose of the container on the shifter.  Subtract the position of the induct photoeye (usually 30 ticks – see IN on the Points Info tab) to determine the minimum value for FullUpdateTicks.

  - Select the update point in the “FullUpdate” field of the appropriate record.  Adjust the update point to the desired location.

  - Note:  For two-sided shoe sorters, each lane must be configured in the srtZoneLane table with TwoSidedShoe = Y, and RightSideDivert must be set as appropriate.  Cartons can only be reassigned to lanes if they are on the same side.

| SrtLaneNotAvail Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this row. |
| Status | String | Indicates the status of the entry and any configuration errors detected. |
| AppSorter | PopUp | Select the sorter associated with this entry. |
| SrchShifter | PopUp | If the sorter motor needs to stop while a container is in transit to a lane, select the shifter where the container will be detected. Also, in table SrtAppControl, ErrConfig must be configured to Stop if Not Avail, and SorterCnvMtr must be set to the motor for the sorter. |
| SortZoneLane | PopUp | If using a FullUpdate update point, link this record to the appropriate destination lane in table SrtZoneLane here. |
| FullUpdate | PopUp | Choose an update point here to define where the sorter stop command will be given if the destination lane is unavailable. This update point can be adjusted in the graphical shifters window. |
| FullUpdateTicks | Int | This column supplies the number of ticks a container may drift during conveyor deceleration. It is important that this value be set accurately. If the container drifts out of the window, the conveyor will restart. See the detailed description of how to determine the correct value earlier in this section. |
| StartTicks | Int | If the SrchShifter is set, also set this field to the starting shifter location where the system will look for containers. Set this column to (MinStopTicks + 10). This integer value is an offset in the upstream direction from the divert point, in ticks. See details earlier in this section. If set to zero, the system will start looking at 30 ticks – the typical induct point location. The parameter is used to allow the system time to get a response from CommCore. Note: When scanner data is married to a container, the system will initially assign the container to the exception handling lane. When CommCore responds, the container is reassigned to the new location. If the position Outpoint - StartTicks is greater than scan update point + time for the CommCore response, but less than the first divert position, the sorter will stop when the exception handling lane is full and wait for the response from CommCore and the lane assignment. |
| MinStopTicks | Int | If SrchShifter is selected, set this field to (Stop distance in ticks + 30). See the full details on how to calculate this value earlier in this section. The value is used to prevent attempts to divert a container when it would drift into the divert point while the conveyor is stopping. Subtract MinStopTicks from the divert point position to define the position where the stop signal will be issued if the lane is unavailable. |
| MinReAssignTicks | Int | This value represents a point upstream of the divert point. It defines a window in which a destination lane will not be considered for reassignment. The default setting of 10 ticks will be set if the value is set to 0. |
| StopInTransit | Y/N | Set this field to Y to stop the sorter when a container is in transit and the lane becomes unavailable. Set it to N to reassign containers without stopping the sorter. The lane will need enough space to allow containers in transit to divert if the lane goes full. |
| pTrnStpMnu | Int | Used by the software to store the tracking table associated with a container that is in transit and is assigned to a lane that is not available. |
| pTrnStpTrk | Int | Used by the software to store the tracking index associated with a container that is in transit and is assigned to a lane that is not available. |
| pIndStpMnu | Int | Used by the software to store the tracking table associated with a container that cannot be assigned a destination because none of the requested zones or lanes are available. |
| pIndStpTrk | Int | Used by the software to store the tracking index associated with a container that cannot be assigned a destination because none of the requested zones or lanes are available. |

# The Bad Gap Configuration Table

The Bad Gap Configuration, or SrtBadGapCnfg table defines how containers with insufficient space between them should be handled.

If the sorter is designed with recirculation capability, this table need not be used.  It can be used to create a tracking record for the Bad Gap containers to capture the scan data and record that a Bad Gap has occurred.  To send a container to “Lane 0” (or recirculation), select a SrtZoneLane table entry that has an Outpoint of N/A (record 0).

Bad Gap containers can be sent to a specific lane.  Note:  The sorter must be physically capable of diverting Bad Gap containers to use this feature!  Pusher-type sorters, for example, must not be configured this way, as they will crush the second container on the return stroke!

| SrtBadGapCnfg Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this entry. |
| Status | String | Indicates the status of the entry. |
| AppSorter | PopUp | Select the sorter associated with this entry. |
| BadGapShifter | PopUp | If Bad Gaps need to be sent to a specific lane select the associated shifter. The sorter must be capable of diverting Bad Gaps to a lane. |
| BadGapLane | PopUp | Select N/A (record 0) if you do not want the sorter to divert Bad Gaps. Select the appropriate lane to configure the sorter to divert Bad Gaps. |
| SendDCMFail | Y/N | Set to N if a DCM fail message is not needed for Bad Gap containers. Set to Y if a DCM fail message is needed for Bad Gap containers. Assign BadGapLane to “Lane 0” (straight, or recirculation). - Configure DCM fail messages in the Outpoints table. A typical DCM fail message is sent if a carton has sent a CRR message to CommCore and the Bad Gap event occurs after the scan event. - This configuration is only needed in cases where a DCM fail message is needed for a Gap Abort, the lane is assigned destination 0, and the scan event occurs further downstream. - The DCM fail message contains the “BADGAP” string and the status is set to “ERROR”. |

# The Round Robin Config Table

The Round Robin Config, or SrtRndRobin table allows lane load balancing for sorters that do not have a scanner and do not determine destinations based on container identity.  Lane availability status, Bad Gap lanes, and recirculation capability are considered when destinations are chosen.

| SrtRndRobin Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this entry. |
| EnableSrt | Y/N | Set to Y to enable the round-robin algorithm for this entry. |
| pStatusMsg | String | Indicates the status of the algorithm. |
| LogLevel | PopUp | Select the log level. Level 2 is a good choice. |
| AppSorter | PopUp | Select the sorter associated with this entry. |
| TrackingTable | PopUp | Select a tracking table, like SrtTrack1, SrtTrack2, etc. |
| Sorter | PopUp | Select the shifter with the induct point for the sorter. |
| pLastLane | Int | Used by the software to store the index of the last lane assigned by the algorithm. |

# Sorter Tracking Tables

The sorter tracking tables are used to hold information about containers on sorters.  The device tracking tables link to the sorter tracking tables and contain scan and scale data associated with containers on sorters.  Five pairs of tables are available:  SrtTrack1 and SrtDevice1 through SrtTrack5 and SrtDevice5.

When a container is inducted onto a sorter, a tracking record is assigned to it.  As scan and scale data is received device tracking records are assigned and linked to the sorter tracking table record.

The tracking and device tracking tables must have enough records to hold data for as many containers as can physically fit onto the sorter with minimum gaps between them all.  When a record needs to be assigned, the oldest record will be reused.  If the number of records is insufficient, bizarre behavior can result, as data in an active record is replaced!  Be sure to calculate the number of records required, and expand the tables, as appropriate.  They are all set to 100 records by default.  Remember that a container may require multiple device tracking records each container – one for each scan and scale reading.

The sorter tracking table also contains status and error information, and the CRR response data, once received.  Message round-trip times are also recorded.

The tracking tables can be inspected to verify proper operation, though since the records get overwritten fairly quickly, it is often best to “take a snapshot” by creating a RUN directory backup for offline examination.

| SrtTrack[1 – 5] Tables |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Automatically populated with the row number, like “R1”. The current record is marked with “====>>>>”. |
| ConfirmScan | String | The container scan sent to CommCore in the sContainer field of the CRR message. |
| ScanQty | Int | The number of scans received for the scan zone. |
| Status | String | Status message that is updated when data is received and processed. |
| ErrorMsg | String | Error message that is updated when data is received and processed. |
| HostZone | String | The zone assigned as the next destination for the container. Taken from the Z* fields, which are from the sZone* fields of the CRR message. Links to a sorter lane via SrtZoneLane_HostZone. |
| CombScan | String | The last combined scan received from the scanner device(s). This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| CombScanSts | PopUp | The status of the CombScan data. This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| CombTote | String | The last combined tote scan received from the scanner device(s). This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| CombToteSts | PopUp | The status of the CombTote data. This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| CombSKU | String | The last combined SKU scan received from the scanner device(s). This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| CombSKUSts | PopUp | The status of the CombSKU data. This field is only used when processing data from multiple scanners that are combined as a single scanner. |
| Z1 – Z10 | String | The data from the CRR message sZone1 thru sZone10 fields. The first available destination will be assigned. |
| ScanZoneID | String | The scan zone associated with this container. It is taken from CRR field sScanner and matched with ScnScanZone_ScanZoneID. |
| ScanZoneIndx | PopUp | The row in table ScnScanZone that matches ScanZoneID. |
| SorterLane | Int | The shifter lane number assigned. Matches LoLane / HiLane in table Outpoints. |
| SrtAppRec | Int | The row number in table SrtAppControl that is associated with this entry. |
| Shifter | Int | The last shifter where a scan, scale, or update point event occurred. It is the row number in table Sorters. |
| Ndx | Int | The row number in table SortData associated with this container. |
| ProcFlag | Int | A value of 1 indicates the scan process has received data and the message process needs to evaluate the record. |
| ProcUpd | Int | A value of 1 indicates the message process needs to update the status for this container. |
| SrtZoneLaneRec | Int | The row number in table SrtZoneLane associated with this tracking record. Indicates the lane selected. |
| CommSent | Int | A non-zero value indicates that a CRR message was sent to request routing for this container. |
| TooLate | Int | A non-zero value indicates the CRR message arrived too late to make the assignment. Particularly important for two-sided shoe sorters, where the shoes must be pulled to the correct side. |
| TooTall | Int | If TooTallSignal and TooTallUpdatePt are defined in table SrtScanBoss, this value will be set. If a container is “too tall” and the ContID is “NOLABEL”, it will be flagged as a NoRead error in the CRR message. The values are: 0 = unset, 1 = not too tall, 2 = too tall |
| SendCRRTrigger | Int | This flag value is set to 1 to generate a CRR message. It is used when SendCRRUpdatePt is defined – when the sorter is configured to use CSM messages. |
| ElapsedTime | Double | The round-trip time for a message to CommCore, in seconds. |
| CommSentSec | Int | The timestamp, in seconds, when the last message was sent to CommCore. |
| CommSentMSec | Int | The millisecond portion of the timestamp when the last message was sent to CommCore. |
| TimeStamp | Time | The timestamp when the record was initialized – usually at the time of the scan event. |
| TampStatus | PopUp | Used for Print and Apply applications. |
| PaProcFlag | Int | Used for Print and Apply applications. |
| PaProcUpd | Int | Used for Print and Apply applications. |
| ExptRF | String | Used for Print and Apply applications. |
| ExptSKU | String | Used for Print and Apply applications. |
| ExptLPN | String | unused |
| VerRF | PopUp | Used for Print and Apply applications. |
| VerSKU | PopUp | Used for Print and Apply applications. |
| VerLPN | PopUp | unused |
| CurrSpeed | Int | Used for Print and Apply applications. |
| Height | Float | Container height, if available. Used for Print and Apply applications. |
| Width | Float | Container width, if available. Used for Print and Apply applications. |
| Length | Float | Container length, if available. Used for Print and Apply applications. |
| TooShort | Int | Used for Print and Apply applications. |
| SendUpdPrt | Int | Used for Print and Apply applications. |
| Scale | String | Container weight, if available. |
| ConfirmTote | String | The last container scan defined as a tote scan. Included in the CRR message as the sToteID value. |
| Param1 – Param5 | String | From the similarly named fields in the CRR message. Used for special scan algorithms and other things. |
| ConfirmShip | String | Used with SrtAppControl_SndShipInCRR. See section 3. |
| ConfirmPack | String | Used with SrtAppControl_SndPackInCRR. See section 3. |
| CombShip | String | Used to manage data for the “ship” scan types. |
| CombPack | String | Used to manage data for the “pack” scan types. |
| CombShipSts | PopUp | Status for the “ship” scan types. |
| CombPackSts | PopUp | Status for the “pack” scan types. |
| pVerifyErr | Int | Used for Print and Apply applications. |
| pPAPassThru | Int | Used for Print and Apply applications. |
| pPNAExempt | Int | Used for Print and Apply applications. |
| pPNAZoneCleared | Int | Used for Print and Apply applications. |
| DCMReq | Int | Flag to track sending of DCM fail messages. |
| SrtSimRecord | Int | Used for sorter simulation. |
| ToteFound | Int | Used to track the state of the SrtScanBoss_ToteSignal. |
| LRMSent | Int | Used for Print and Apply applications. |
| LVMSent | Int | Used for Print and Apply applications. |
| UUID | String | A Universally Unique Identifier string. Used for Print and Apply applications. |

| SrtDevice[1 – 5] Tables |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Automatically populated with the row number, like “R1”. The current record is marked with “====>>>>”. |
| TrkIndx | Int | The row number of the related tracking table record. |
| Scan | String | The string received from the scanner or scale. |
| ScanSts | PopUp | The status of the Scan data. |
| ScanType | PopUp | The scan type for this data. It is taken from the associated ScnScanDevice_ScanType. |
| ScanZone | PopUp | The associated ScnScanZone record. |
| TimeStamp | Time | The time when the record was initialized – usually at the time of the scan event. |

# Message Interface Configuration Example

The message interface between FPC and CommCore is defined through the Machine and message map (MsgMap) tables.

The Machine table defines the connection to the outside world, including the IP address and ports for CommCore (via /etc/hosts entries), and the protocol to interpret the data.  The table below shows the important entries in table Machine for a connection to handle CRR messages on port 9001 and one to handle DCM messages on port 9002.

The Machine_Name strings identify each connection within FPC, but need an alias in the /etc/hosts table to define the IP address of the CommCore server, like this:
  192.168.10.100	CRR1 DCM1
All entries for a given IP address can be made on one line in /etc/hosts.

| Machine Table (partial) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Transmit_Retry | Process | NeedACK |
| CRRMSG | CONNECT_ TCPCLIENT | SrtComm | 9001 | 10 | SERV | 1 |
| CSUDCM | CONNECT_ TCPCLIENT | SrtComm | 9002 | 10 | SERV | 1 |

The MsgMap table makes the connections from the machine interfaces to the tables within FPC where the data is stored.

The Machine_Name field is the connection to the Machine table.

The _Msg_Type fields define which messages the entry will process.  Either the message type or the AC name (like AC1) can be used in these fields.  For this example, the CSU and DCM are handled by the same message table and CommCore port, so the AC name is used.

The Menu_Name field defines the connection to the table that will hold the message data.  These, in turn, are connected to the sorter processing through the fields in table SrtAppControl.  See section 3.

The Low_Cursor and High_Cursor fields define the columns in the message that will be processed.

Stat_Cursor is a reference to the column number of the TxStat column.  Similarly, Key_Cursor refers to column DbKey.  If you set these two to zero, the software will figure out the appropriate values, otherwise, use the MenuColumn (Alt-K from F2-MenuOfMenus) table to find the column number.

Set the Record column to zero to allow all rows in the message table to be used.

| Message Map table (partial) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_ Name | Recv_ Msg_Type | Send_ Msg_Type | Ack_ Msg_Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| CRR Msg | CRRMSG | CRR | CRR | CRR | MsgCR1 | 1 | 44 | 67 | 68 | 0 |
| CSU DCM | CSUDCM | AC1 | AC1 | AC1 | MsgDC1 | 1 | 44 | 67 | 68 | 0 |

# CommCore Interface

Routing information for containers is provided to FPC by CommCore.  CommCore receives that information from the customer’s WMS system.  Divert confirm information flows in the other direction.

CommCore

Oracle
DataBase

Host Messages
WMS Interface

Automation Controllers
CRR, CSU, DCM

CommCore holds information from the customer’s host in an Oracle database for fast retrieval to support FPC and the real-time management of the conveyor system.  CommCore acts as a server for TCP connections from FPC and uses pipe-delimited STX/ETX framed messages of Fortna’s design for communication.  FPC (the client) initiates most messages and expects to receive a reply including the original information plus the response data and a status message.

A normal sequence of events for a container might be:

- Container is scanned, and FPC sends a CRR message

- CommCore determines routing and returns a list of possible destinations

- FPC chooses the first available destination and sends a CSU to indicate the choice.

- CommCore acknowledges the CSU

- The container reaches the destination and is diverted.  FPC sends a DCM

- CommCore acknowledges the DCM

  - Status and Error Messages

Each message includes fields for status and error messages, which can be helpful for debugging.  These fields are included in the tracking record and appear in the log files for the received messages.

CommCore uses the CSU message to update the status of containers.  The chosen destination or an error status will be available through EV10 based on the CSU message content.

Special Case “NoRead”:  A NoRead is reported if the last CSU message did not assign the container to the error lane.  By default, a NoRead is the only way a container can get there.

| Status and Error Messages |  |  |  |
| --- | --- | --- | --- |
| Status | Error | Module | Description |
| “Zone(s) Not Avail” | “No Error” or Message from CC | CRR Message Processing | The sorter is configured to not assign the destination when the lane is “Not Available”. |
| “Not Avail, Stopped” | “No Error” or Message from CC | CRR Message Processing | The sorter is configured to stop when a lane is “Not Available”, and the lane(s) are not available, and the sorter has stopped. |
| “Zone AAA, Lane: NNN” | “No Error” or Message from CC | Lane Assignment Processing | The container has been assigned to the zone and lane indicated. |
| ##### “Sent Error” | “Comm Err” | Scan Data Processing | The container has been assigned to the error lane. CommCore did not send a message or the response was not valid. |
| ##### “Sent Error” | “Scan Err” | Scan Data Processing | The container has been assigned to the error lane. The scan data contained an error. |
| ##### “Sent Error” | “Conflict Err” | Scan Data Processing | The container has been assigned to the error lane. The scan data contained an error. |
| ##### “Error” | “Bad Gap” | Scan Data Processing | The container has a bad gap. In some configurations the container will be assigned to an error lane. |
|  | “No Zone” | CRR Message Processing | The CRR message did not contain any assigned zones (destinations), or the CRR message did not have any valid zones. Note: This status is recorded in the tracking record, but is not sent to CommCore. |

  - FPC Message Tables

Five sets of message tables are provided in FPC for interfacing with CommCore for a variety of purposes.  They are all identical in structure, and could be used interchangeably, but have certain traditional uses based on their names.  Each group has nine numbered copies to allow division of the messages either for clarity or for performance.  The MsgCR tables are used for CRR messages.  The MsgDC tables are for DCM and CSU messages.  The MsgXR and Msg XDC tables may be used for transfer CRR and DCM messages.  The MsgGP (general purpose) tables are used for all other messages.

Message tables are assigned based on the CommCore interfaces available.  For instance, DCM and CSU messages are often assigned to the same MsgDC table because the CommCore interface is designed to accept both messages through a single port.  A separate MsgCR table will be required for each sorter on an AC to achieve sufficient performance.  The table size may need to be increased to hold as many container records as can physically fit on the tracked section, so data is not lost.

Each message table has header columns, a transmit status column, a database key column, and timestamp columns in addition to the (currently) 32 pairs of data columns.  Each numbered data column pair has a “Col” and a “Dat” column.  The “Col” columns will contain the identifier for the data in the “Dat” column.  Each message type may require a different number of data column pairs.  The number of data column pairs actually included in the message is defined in the MsgMap table by the Low_Cursor and High_Cursor fields, which must be adjusted to match the length expected by CommCore for each message type.  Currently, the CRR and DCM messages are expected to have Low_Cursor = 1 and High_Cursor = 44.

Messages generated from (and received into) these tables consist of a STX start character, an optional five-digit sequence number (sequence protocol), a three-character message identifier (CRR, DCM, etc.), the AC name, the data column pairs, and a closing ETX character.

All messages are sent with the expectation of a response.  The response, in many cases, will be an echo of the complete message.  Some responses, like the CRR, will include all of the original identification data plus additional information that the AC can act upon.

The following sections describe data column pairs for each message type.

    - CRR – Container Route Request

The CRR message is used to request and receive destination information for containers at decision points.  The message acknowledgement will contain the routing destination information.  Though many messages may be queued, only one is “in play” at a given moment, so its response must be received before the next one can be sent.

| Container Route Request Message (FPC to CommCore). |
| --- |
| Field | Value or Description |
| CmdHdr | CRR |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 20 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor. Probably ignored by CommCore.) |
| Col2 | sTableName |
| Dat2 | zcContainer (Table name in Oracle) |
| Col3 | sContainer |
| Dat3 | Barcode read (ContainerID or SKU) |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sScanner |
| Dat7 | Scanner ID or decision point from ScnScanZone_ScanZoneID |
| Col8 | sTrackTable |
| Dat8 | Tracking table table ID number |
| Col9 | sTrackIndex |
| Dat9 | Tracking table index or row number |
| Col10 | sScaleData |
| Dat10 | Weight from scale, if any (0 indicates an error) |
| Col11 | ToteID |
| Dat11 | Tote scan (If available) |
| Col12 | sZone1 |
| Dat12 | First destination zone. Corresponds to SrtZoneLane_HostZone. The outbound CRR may contain a comma-separated list of unavailable destinations based on SrtAppControl_SendAvailStatus. |
| Col13 | sZone2 |
| Dat13 | Second destination zone. |
| Col14 | sZone3 |
| Dat14 | Third destination zone. |
| Col15 | sZone4 |
| Dat15 | Fourth destination zone. |
| Col16 | sZone5 |
| Dat16 | Fifth destination zone. |
| Col17 | sZone6 |
| Dat17 | Sixth destination zone. |
| Col18 | sZone7 |
| Dat18 | Seventh destination zone. |
| Col19 | sZone8 |
| Dat19 | Eighth destination zone. |
| Col20 | sZone9 |
| Dat20 | Ninth destination zone. |
| Col21 | sZone10 |
| Dat21 | Tenth destination zone. |
| Col22 | Param1 |
| Dat22 | Parameter 1 value. |
| Col23 | Param2 |
| Dat23 | Parameter 2 value. |
| Col24 | Param3 |
| Dat24 | Parameter 3 value. |
| Col25 | Param4 |
| Dat25 | Parameter 4 value. |
| Col26 | Param5 |
| Dat26 | Parameter 5 value. |
| Col27 | sHeight |
| Dat27 | Height |
| Col28 | sWidth |
| Dat28 | Width |
| Col29 | sLength |
| Dat29 | Length |
| The rest are unused for CRR. |  |

    - CSM – Container Scan Message

The CSM message is used to initiate retrieval of container destination information for eventual retrieval by a CRR when the information must be retrieved by CommCore from a host system.  The message acknowledgement may contain the routing destination information, but probably will not.  If an error lane destination is returned, it will be assigned.  The destination information will be available by the time the CRR request is made, typically 5 or more seconds later.  When the CRR response is received, if an error destination has been set, it will be overwritten.  CSM messages are activated by making an entry in SrtAppControl_CsmMsgTable and defining an update point in SrtScanBoss_SendCRRUpdatePt.  See sections 3 and 5 for details.

The CSM message content is exactly the same as the CRR, except CmdHdr = CSM.

    - CSU – Container Status Update

The CSU message is sent to CommCore after a CRR message has been processed and a destination has been assigned.  If the destination originally chosen becomes unavailable, a new assignment is made and a new CSU is sent to indicate the new choice.

| Container Status Update Message (FPC to CommCore). |
| --- |
| Field | Value or Description |
| CmdHdr | CSU |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 11 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor.) |
| Col2 | sTableName |
| Dat2 | zcContainer (Table name in Oracle) |
| Col3 | sContainer |
| Dat3 | Barcode read (ContainerID or SKU) |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sScanner |
| Dat7 | Scanner ID or decision point from ScnScanZone_ScanZoneID |
| Col8 | sTrackTable |
| Dat8 | Tracking table table ID number |
| Col9 | sTrackIndex |
| Dat9 | Tracking table index or row number |
| Col10 | sStatus |
| Dat10 | Status message. Includes zone/lane chosen. |
| Col11 | sError |
| Dat11 | Error message, if any. |
| The rest are unused for CSU. |  |

    - DCM – Divert Confirm Message

The DCM message is sent when a container is diverted to its destination.

| Divert Confirm Message (FPC to CommCore). |
| --- |
| Field | Value or Description |
| CmdHdr | DCM |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 11 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor.) |
| Col2 | sTableName |
| Dat2 | zcContainer (Table name in Oracle) |
| Col3 | sContainer |
| Dat3 | Barcode read (ContainerID or SKU) |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sScanner |
| Dat7 | Scanner ID or decision point from ScnScanZone_ScanZoneID |
| Col8 | sTrackTable |
| Dat8 | Tracking table table ID number |
| Col9 | sTrackIndex |
| Dat9 | Tracking table index or row number |
| Col10 | sZone1 |
| Dat10 | Divert destination zone. Corresponds to SrtZoneLane_HostZone. |
| Col11 | iDCComplete |
| Dat11 | Divert code: 1 – DCM success 12 – DCM failure |
| The rest are unused for DCM. |  |

    - RHS – Request Host Status

The RHS message can be used to request the status of the host – the client’s WMS system.  This message acts as a heartbeat for the connection to CommCore and also indicates the status of the host’s message queue.

If an error is configured, it must use TrigIO to activate/deactivate the error.  It can also be used to stop the sorter if the host connection is lost.

| Request Host Status Message (FPC to CommCore). |
| --- |
| Field | Value or Description |
| CmdHdr | RHS |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 9 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor. Probably ignored by CommCore.) |
| Col2 | sHostStatus |
| Dat2 | Unused. |
| Col3 | (unused placeholder) |
| Dat3 |  |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sNotUsed |
| Dat7 |  |
| Col8 | sHeartBeatTable |
| Dat8 | Heartbeat table table ID number |
| Col9 | sRecordIndx |
| Dat9 | Heartbeat table index or row number |
| Col10 | sErrTime |
| Dat10 | Time in seconds to wait before a timeout error occurs. |
| Col11 | HostMsg |
| Dat11 | Message to host. |
| The rest are unused for RHS. |  |

# Sorter Simulation

A sorter simulation feature can be used to generate CRR, CSU and DCM messages for the purpose of testing the connection to CommCore.  By adjusting simulated carton generation rates, CommCore throughput can be tested.

The SrtTest protocol may also be used to set up an internal message server.

See document FPC-Sorter-Test-Setup for details and an example.