# Fortna

# Internal

# West Reading, PA

# Sorter Test Setup

Document Version: 1.4

Date: 25-Nov-2020

Revision History

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 05/10/2013 | Niles Martin | Initial Release |
| 1.1 | 12/04/2013 | Barry Conklin | Updated Column Descriptions |
| 1.2 | 12/09/2013 | Barry Conklin | Changed logic to handle auto-generated scans and scans from a table to use the same logic. |
| 1.3 | 19-Jul-2018 | Thurman Kremser | General update for clarification. |
| 1.4 | 25-Nov-2020 | T R Kremser | General update |

Table of Contents

1	Introduction	1

1.1	Document Objective	1

1.2	References	1

1.3	Document Conventions	1

1.3.1	Standard Definitions	1

1.3.2	Acronyms and Abbreviations	1

1.3.3	Requirements Numbering	1

2	Project Overview	2

2.1	Conveyor System Layout	2

2.2	High Level Summary	2

3	User Interface Screens	3

3.1	Batches	3

3.2	SrtSimConfig	4

3.3	SortSimScans[1-5]	5

3.4	TimerTfgScrn	6

3.5	Machine	6

3.6	MsgMap	7

3.7	SendTstCRR	8

3.8	Motor_Chains	9

3.9	Errors	9

3.10	Jam_Zones	9

4	Messaging	10

This page intentionally left blank.

# Introduction

This Internal Design Document contains the software design details for testing Sorter Configuration.

## Document Objective

The purpose of this document is to serve as a vehicle for testing Sorter Configuration without Level II.

The intended audience for this document is predominantly the software development team and the Controls team.

## References

These requirements were gathered through meetings and other communications methods with Client personnel.

Fortna:

Client:

WMS:

## Document Conventions

### Standard Definitions

### Acronyms and Abbreviations

### Requirements Numbering

# Project Overview

This section documents the configuration of a complete Automation Controller test for a Sorter.  For this example, a typical customer configuration will be used.  Version 10.3 or later of the Automation Controller software is required.

## Conveyor System Layout

The Shipping Sorter from a customer site.

## High Level Summary

The sorter simulation is composed of Level II communication simulation and carton simulation.  The carton simulation in this example will include a sorter with an induction scanner, a curve, and the divert lanes.  The simulation will induct boxes at the scanner belt, induct boxes onto the curve and then on to the sorter belt with the divert lanes.

It is assumed the reader understands the configuration of a sorter and this document will focus on the configuration of the simulation.

Since Level II communications are designed to be the same for sorters and transfers, the Level II communication simulation portion can be used for either.

# User Interface Screens

This section provides the detailed configuration of the Automation Controller menus.

## Batches

The Batches menu is located under Configuration Menu…->Standard Controls…->Batches.

The Sorter should already have the “Scan Process”, “Srt Ctrl”, and “Srt Serv” entries configured.

| Column | Description | Simulation |
| --- | --- | --- |
| File_Name | The name of the process to be run. The name must be unique in the Batches menu. In order to run the simulation a new entry needs to be added. | SORT SIM |
| Description | The description of the Batches entry. |  |
| CurrentNum | Not used. | 0 |
| HowOften | Determine how many increments are needed to run the process. This value must be greater than 0 for the process to run. We want this Batch to run on every pass through the SORT Process loop. See “Type”, below. | 1 |
| Increment | Determine the interval for running the process. In this case the “SORT SIM” process needs to run on each pass through the SORT Process. Choose “INVALID”. See “Type”, below. | INVALID |
| Type | Choose the method for determining when to run the Batch. Choose “Count” to run the Batch every “HowOften” passes through the loop. We want this Batch to run on every pass. The “Elapsed” and “Absolute” choices would run the Batch based on clock time. | Count |
| CurrentStat | This field is used by the program to keep track of when the Batch was last run. If the HowOften field is greater than 1 you may see this field incrementing. | Calculated |
| WasStat | Used by the software for Types “Elapsed” and “Absolute”. | Calculated |
| Process | The Automation Controller Process that will run the Batch. Choose SORT for this one. | SORT |
| InternalProc | Batch functions that are run from the GUI and SERV Processes. |  |
| InternalProcCtrl | Batch functions that are run from the CTRL and SORT Processes. | SrtSim SORT |
| Machine | Pop-up. The name of the Machine the batch should be run on. | Name of your Machine. |
| All Machines | Determines if the Batch is run on all machines. | N |
| DoFile | Execute the name in “File_Name” as an external program. | N |

The “SrtSim SORT” Batch will create simulated containers on the shifter based on the parameters found in the SrtSimConfig table.  This function turns the Induct ON and OFF using FIRE and UNFIRE. The Induct should be configured with an Output Memory Bit owned by the SORT Process.

## SrtSimConfig

The SrtSimConfig table is used to define how simulated cartons should be created on the sorter.  It is located under Configuration Menu…->Sorter Utilities…->Cont Sim Config or Configuration Menu…->Simulation Utilities…->Cont Sim Config.  Note that there are several different methods for simulating scans.  This example uses the “Sim Scan Menus” which allows an arbitrary set of scans to be input at any scan point location.

| Column | Description | Value |
| --- | --- | --- |
| Name | The name of the simulation configuration. The name must be unique in the table. Do not use row 0. | See table below |
| EnableSrt | Enables the Sorter Simulation. Set this last. | Y |
| Status | A status column for the simulation. | Calculated |
| AppSorter | The AppSorter Row for the Sorter being simulated. | PTS_SORTER |
| NumContSrt | The number of containers to induct into the sorter simulation. | 100 |
| CurContSrt | The number of containers that have been inducted on to the simulated sorter. | Initially set to 0, the software will update this as cartons are inducted. |
| SimShifter | The Shifter on which to induct the simulated containers. You can match the AppSorter to the Shifter by looking at the SrtZoneLane and Sorter Outpoints menu. | See table below |
| CreateSorterTmr | To run a shifter for simulation, a Sorter Timer is required. To create the timer if it does not already exist, and to allow the simulation software to enable and disable the timer, set the push button “ON”. If the button is turned “OFF” the simulator will not change the sorter encoder type when the simulator is “Enabled”. The “Encoder Type” in the Sorters table is set to a “Timer Type” encoder (1) when the simulator is enabled with this field set to “ON”. The “Encoder Type” is set back to a “Real Encoder” (0) when “EnableSrt” for the simulator is turned “OFF”. | ON |
| ShifterTimerHalt | If the SimShifter uses a timer-based encoder, setting this to “ON” will halt the simulated conveyor in the event of an error. An error may occur if a lane is not available, or if a HeartBeat Error has occurred. | ON |
| NumCharScan | The number of characters to use in the simulated scan. Maximum should be 20. This should be set to at least the sum of characters in NumContSrt, PreFixString, and PostFixString. The scan value will be the PreFixString, the CurContSrt, and the PostFixString concatenated together. Do not use if you are using the Sim Scan Menus. | 0 |
| ScanDevice | The scan device for the sorter. | See table below |
| ScaleDevice | The scale device for the sorter if there is one. |  |
| ScaleString | This will be the value passed in the CRR sScaleData field. Do not use if you are using the Sim Scan Menus. |  |
| PreFixString | A string to add to the start of the container number. Do not use if you are using the Sim Scan Menus. |  |
| PostFixString | A string to add to the end of the container number. Do not use if you are using the Sim Scan Menus. |  |
| InPoint | Pop-up. The IO of the sorter’s induction photo eye. | See table below |
| InductDumpPt | Inducts boxes from the previous simulated shifter. Set to “N” on the first shifter and “Y” on each additional shifter you want to pass the containers to. As the container hits the dump point on the prior shifter, the carton will be inducted onto this shifter. | See table below |
| Length | Defines the length of each inducted box in ticks. | See table below |
| Gap | Defines the distance in ticks between the end of the first box the and the start of the next box. | See table below |
| SimCrrCsu | Determines if the simulator simulates DCM and CSU messages. Only used if the shifter will not be running with a timer encoder. Set to “Y”, if the simulator needs to generate a CSU and DCM. It will do so as soon as the CRR response is received. | N |
| ScanSimMenu | If simulated scans are to be provided by one of the “Sim Scan Menus” tables (SortSimScans1 -5), select the message table in this column. The selected table will contain a list of the scans to be generated. The “CartonID” column is used for PnA simulation. Only configure this for the shifter that has a scanner. | See table below |
| SimScanFile | If a file will be used to enter the values in the “Sim Scan Menus”, enter the file location and the filename into the column. The table in the “Sim Scan Menus” can be entered manually. Only configure this for the shifter that has a scanner. | See table below |
| LoadFile | Causes the file listed in the SimScanFile entry to be loaded. The button is reset after the file has loaded. |  |
| WrapScanList | If the button is turned “ON”, the system will wrap around to the first record once all scans in the table have been sent. If the button is turned “OFF” the simulator will stop simulating scans when the end of the list is reached. This will result in “NOSCAN” events if the “CurrContSrt” is set to continue creating simulated cartons. | See table below |
| ResetSimulation | Resets the simulation. |  |
| pCurrLength | The remaining length in ticks for the currently inducting box. | Calculated |
| pCurrGap | The remaining length in ticks until the next box inducts. | Calculated |
| pCurrSimRecord | The next Sim record that will be extracted from the “Sim Scan Menus” tables. The column is updated after each scan is simulated. | Calculated |

Menu Configuration

| Name | AppSorter | NumContSrt | SimShifter | ShifterTimerHalt |
| --- | --- | --- | --- | --- |
| PTSSCAN | PTS_SORTER | 100 | PTS_SCAN | Checked |
| PTSCURVE | PTS_SORTER | 0 | PTS_CURVE | Checked |
| PTSSORTER | PTS_SORTER | 0 | PTS_SORTER | Checked |

| Name | ScanDevice | ScaleDevice | ScaleString | InPoint | InductDumpPt | Length |
| --- | --- | --- | --- | --- | --- | --- |
| PTSSCAN | SCNPTS | N/A |  | Inpoint 1-0 | N | 5 |
| PTSCURVE | N/A | N/A |  | Inpoint 2-0 | Y |  |
| PTSSORTER | N/A | N/A |  | Inpoint 3-0 | Y |  |

| Name | Gap | ScanSimPoint | ScanSimMenu | SimScanFile | WrapScanList |
| --- | --- | --- | --- | --- | --- |
| PTSSCAN | 10 | Update 1-1 | SortSimScans1 |  | Check if no Level2 |
| PTSCURVE |  | Invalid | None |  | Unchecked |
| PTSSORTER |  | Invalid | None |  | Unchecked |

## SortSimScans[1-5]

Five SortSimScans tables are provided for simulating scanners.  The tables provide a list of scans to be generated for each inducted container.  To start the simulation at the first row, click on the “Reset Simulation” button. The simulation software will wrap if WrapScanList is selected in the SrtSimConfig table.  For the sorter simulation, use the Scan column.

| Column | Description |
| --- | --- |
| CartonID | Used in Print and Apply simulations. |
| Scan | Used in Sorter simulations. |
| Scale | Used in Scale simulations. |

## TimerTfgScrn

To run a shifter for simulation, a Sorter Timer is required. To create the timer, if it does not already exist, and to allow the simulation software to enable and disable the timer, set the push button “CreateSorterTmr” to “ON” in table SrtSimConfig. If the button is set to “OFF” the simulator will not change the sorter encoder type when the simulator is “Enabled”.

For each Sorter entry below, update the Preset time with the following parameters using the Timers table.  The timer may be updated by clicking with the right mouse button from the Sorters Menu. The simulator will change the “Encoder Type” column to 1, when the simulator is enabled, and if the “CreateSorterTmr” button is turned “ON”.

| Sorter | TimerName | Preset | Owner |
| --- | --- | --- | --- |
| PTS_SCAN | PtsScanSimEnc_CAPPTS | 00:0.018 | CAPPTS |
| PTS_CURVE | PtsCurveSimEnc_CAPPTS | 00:0.018 | CAPPTS |
| PTS_SORTER | PtsSortSimEnc_CAPPTS | 00:0.018 | CAPPTS |

## Machine

To simulate the CRR messaging from Level II, the SrtTest or SrtTestSeq protocol will be used.  The protocols use their own separate Machine table entries for communication.  The Machine table is located under Configuration Menu…->Communication Utilities…->Machine.

| Column | Description |
| --- | --- |
| Machine_Name | A unique name for this connection. The Machine_Name is used to look up the IP address of CLIENT connections so it must also have an entry in the /etc/hosts file. If the Machine_Name is not found in /etc/hosts, the process will hang for 11 seconds for each entry while it attempts to look up the name by DNS, which is intentionally not configured. SERVER connections do not need an entry in the /etc/hosts file. |
| Status | The status of the connection. |
| Error | An error message about the connection status. “error EADDRNOTAVAIL” – Check if the TCP/IP port number is valid. “unknown host” – Check the /etc/hosts file for the Machine_Name. This error will cause 11 second delays in the process. |
| Machine_Pid | An id used for the connection internally. -1 indicates there is no connection. Setting this entry to -1 will force a reconnection attempt if the connection is already established. |
| Connection_Type | The method used to communicate. CONNECT_TCPCLIENT – The Automation controller acts as a TCP/IP client and connects to a server with an IP address defined through the Machine_Name. (Requires an /etc/hosts entry) TCPSERV_LISTEN – The Automation controller acts as a TCP/IP server. This entry requires a second Machine table entry for a TCPSERV_RDWR to exchange data. TCPSERV_RDWR – The entry that exchanges data after the TCPSERV_LISTEN entry makes the connection. The MsgMap entry will point to this Connection_Type when the Automation Controller acts as a server. |
| Communication_Protocol | The Communication_Protocol defines the functionally over the connection. SrtComm – Used to provide routing for Sorters and Transfers. SrtCommSeq – Used to provide routing for Sorter and Transfers. Contains a sequence number to help recover from missing messages. SrtTest – Used for testing SrtComm connections. SrtTestSeq – Used for testing SrtCommSeq connections. |
| Machine_Port | The port for the connection to use. For TCP/IP connections, the Machine_Port will be a number. For ActiveMQ connections the Machine_Port will be a “/queue/…” or “/topic/…” entry. For testing the SrtComm and SrtCommSeq protocols use the same Machine_Port number (of your choosing) in the TCPSERV_LISTEN, TCPSERV_RDWR and CONNECT_TCPCLIENT entries. Values in the 9000 range should be a good choice. Choose a unique port number value for each client/server pair. |
| Process | The process used for communication. For this purpose, choose SERV. |
| LogLevel | Used to debug communication issues. Set to 3 for development. |

The CRR_PTS entry is the existing entry to test.

| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Process |
| --- | --- | --- | --- | --- |
| CRR_PTS | CONNECT_TCPCLIENT | SrtComm | 9001 | SERV |
| CRR_TEST_LS | TCPSERV_LISTEN | SrtTest | 9001 | SERV |
| CRR_TEST_RW | TCPSERV_RDWR | SrtTest | 9001 | SERV |

After configuring new Machine menu entries, the Automation Controller software will need to be restarted.

## MsgMap

The MsgMap table connects the Machine table configuration with the tables used to store and send the data.  The MsgMap table is located under Configuration Menu…->Communication Utilities…->Message Map.

The following table describes the MsgMap entry needed for the SERVER (Level II simulation) side of the messaging.  The usual entry for the CLIENT is also required.

Note that no entry will appear in the MsgTST table (SERVER side) as the simulation is run.  Look in the message table used for the CLIENT side of the connection for the message data.

| Column | Description | Value |
| --- | --- | --- |
| Message_Name | A unique name in this table for the configuration. | CRR_TEST_RW |
| Machine_Name | The pop-up to the Machine table row for communication. | CRT_TEST_RW |
| Recv_Msg_Type | A filter on the communication to stop unexpected messages or to separate them into multiple tables. Use ALL for no filtering on machine name or message type. | ALL |
| Send_Msg_Type | A filter on the communication to stop unexpected messages or to separate them into multiple tables. Use ALL for no filtering on machine name or message type. | ALL |
| Ack_Msg_Type | A filter on the communication to stop unexpected messages or to separate them into multiple tables. Use ALL for no filtering on machine name or message type. | ALL |
| Menu_Name | The name of the table to get data from or put data to. Do not use the same table in more than one MsgMap entry. | MsgTst |
| Low_Cursor | Defines the first column of the message data. | 1 |
| High_Cursor | Defines the last column of the message data. Adjust this value to control the number of message fields to be sent to level II. | 66 |
| Stat_Cursor | The column number of the TxStat field, per the definition of the table named in Menu_Name. New versions of the Automation Controller software auto-configure this field when restarted. Auto-configuration doesn’t work for the ScnScanZone and ScnScanDevice tables. | 67 |
| Key_Cursor | The column number of the DbKey field, per the definition of the table named in Menu_Name. New versions of the Automation Controller software auto-configure this field when restarted. Auto-configuration doesn’t work for the ScnScanZone and ScnScanDevice tables. | 68 |

## SendTstCRR

The SendTstCRR table provides the routing information for the CRR response.  The SendTstCRR table is located under Configuration Menu…->Simulation Utilities…-> Config CRR Test Message.  The SendTstCRR table has two methods for configuring test messages.  Rows 1 to 5 are for simulating single messages.  Row 1 simulates a CRR message, row 2 simulates an RHS message, row 3 simulates an EHA message, row 4 simulates an error message, and row 5 simulates an ESM message.  The remaining rows can be used to simulate a pattern of CRR messages.  This example will use rows 6 and greater.

| Column | Description | Value |
| --- | --- | --- |
| Name | The Name column is used to match the sScanner in the CRR message to a test record. The sScanner name can be looked up in the ScnScanZone table ScanZoneID field. | SCNPTS |
| Status | Not used? |  |
| Z[1-10] | The zone to route the carton to. The code will route the container to the first Zone that is available. The Zones can be looked up in the HostZone field of the SrtZoneLane table. | See table below |
| Param[1-5] | Parameters that can be passed to a Transfer |  |
| Last | Marks the last test record used. | Calculated. |

| Column # | Name | Z1 |
| --- | --- | --- |
| 6 | SCNPTS | PTS01 |
| 7 | SCNPTS | PTS14 |
| 8 | SCNPTS | PTS02 |
| 9 | SCNPTS | PTS13 |
| 10 | SCNPTS | PTS03 |
| 11 | SCNPTS | PTS12 |
| 12 | SCNPTS | PTS04 |
| 13 | SCNPTS | PTS11 |
| 14 | SCNPTS | PTS05 |
| 15 | SCNPTS | PTS10 |
| 16 | SCNPTS | PTS06 |
| 17 | SCNPTS | PTS09 |
| 18 | SCNPTS | PTS07 |
| 19 | SCNPTS | PTS08 |

## Motor_Chains

To simulate the motor startup, the ForceAux column needs to be set to ‘Y’.  Note: You can set or clear all the ForceAux entries that have a Heater_Bit defined by running the TurnForceAuxFlagOn or TurnForceAuxFlagOff procedures from the “Security Menu” using Alt-K.

## Errors

In order to simulate starting the system, look at the errors by Hitting F10 then 0.  Clear each error.

## Jam_Zones

In order to enable starting a zone, the Conveyor Part in the Enable Bit must be forced on.

# Messaging

TBD