| # FortnaPlus Control Sorter Configuration Checklist |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 20-Jan-2021 |  |

Revision; Date; Author; Revision Summary. 1.0; 03-Apr-2013; B C Conklin; New document. 1.1; 20-Jan-2021; T R Kremser; Updates and general refresh.

Table of Contents

1.	Sorter Configuration Checklist	4

2.	Process Flow – Scan Event	5

3.	Process Flow – Message Receive Event	6

4.	Batches Table	7

5.	SrtAppControl Table	7

6.	ScnScanZone Table	7

7.	ScnScanDevice Table	8

8.	SrtScanBoss Table	8

9.	Machine Table – Scanner Connections	8

10.	MsgMap Table – Scanner Connections	10

11.	Machine Table – CommCore Connections	10

12.	MsgMap Table – CommCore Connections	11

13.	SrtZoneLane Table	12

14.	Negative Divert Confirm Configuration	12

15.	TCP/IP Scanner Loop-back Testing	13

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Sorter Configuration Checklist

This document is intended as a memory-jogger to help you make sure all important configuration items are covered when setting up a new sorter.  It may also be helpful for troubleshooting and existing sorter.

Refer to the FPC-Sorter-Control-Module document for details on any of the items.

# Process Flow – Scan Event

The sequence of events when a scan is received.

Scan Event

Machine table connection and protocol
(TCP/IP or serial)

MsgMap links to ScanDevice – sets “Recvd” status

ScanDevice processing stores scan in tracking tables, sets “ProcFlag”

Sorter processing (SrtScanBoss table links a scan zone to a sorter), monitors the tracking table “ProcFlag”

Sorter processing builds a CRR message and stores it in the message table selected in SrtAppControl

Message processing monitors MsgMap and when a record with a TxStat of “New” is found (TxStat is a column in the message table), the message is sent. The message is sent to the “Host” name found in the first column of the Machine table (/etc/hosts maps it to an IP).

# Process Flow – Message Receive Event

The sequence of events when a message is received from CommCore.

Message Receive Event

When an incoming message is received, FPC searches table MsgMap for an entry that matches on the Machine table Machine_Name and the message header.  The message header contains the AC name and the message type.  In some protocols “ALL” can be used in MsgMap to process any incoming messages for the machine.

The scan zone (ScanZoneID) and the destination are extracted from the message and copied to the Z1 through Z10 columns of the tracking record.  The tracking table assigned to the sorter is found in the ScnScanZone table.

The Destination Lane table (SrtZoneLane) HostZone column is searched for a match to the destination found in the tracking table.  The first available destination is assigned.

# Batches Table

In table Batches (Configuration Menu | Standard Controls | Batches), ensure that the batch jobs for processing scans (“ScnApp CTRL”), and for sorter management (“SrtApp CTRL” and “SrtApp SERV”) are active.  Do not use “Do Scans” – this is superseded by “ScnApp CTRL”.

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | How Often | Increment | Type | Process | Internal Proc | InternalProc Ctrl | Machine | All Machines | Do File |
| Scan Process | 1 | Invalid | Count | CTRL |  | ScnApp CTRL | AC2 | N | N |
| Srt Ctrl | 1 | Invalid | Count | CTRL |  | SrtApp CTRL (CPN) | AC2 | N | N |
| Srt Serv | 1 | Invalid | Count | SERV | SrtApp SERV (CPN) |  | AC2 | N | N |

# SrtAppControl Table

In table SrtAppControl (Configuration Menu | Sorter Utilities | App Sorter Configuration), define the message tables that will be used for the sorter.  Five sets of message tables are available.  Use the first set for the first sorter on the AC, and the others if more than one sorter is defined on the AC.  Once this is configured correctly, CRR messages will appear in the table defined in CrrMsgTable (MsgCR1 in the example).  CsmMsgTable is only configured if a CSM message is needed.

| SrtAppControl Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | CrrMsg Table | DcmMsg Table | CsuMsg Table | CsmMsg Table | Control Machine |
| Ship Sorter | MsgCR1 | MsgDC1 | MsgDC1 | MsgGP1 | <This AC> |

# ScnScanZone Table

In table ScnScanZone (Configuration Menu | Scanner Utilities |Scan Zone Config), define the decision point (ScanZoneID column) used in the CRR message to CommCore, thereby establishing the cross-reference between the name given to the scan zone on each system.  The tracking table defines where scanner data is stored.  There are also five sets of tracking tables, for use if multiple sorters are configured on one AC.

| ScnScanZone Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Delimiter | ScanZoneID | Tracking Table | DeviceTrk Table | Printer TrkTable | CTRL Process | Scan Algorithm |
| ShipScnZn |  | Ship | SrtTrack1 | SrtDevice1 | None | <blank> | N/A |

# ScnScanDevice Table

In table ScnScanDevice (Configuration Menu | Scanner Utilities | Scan Device Config), define the configuration parameters for each scanner and scale and the connection to the scanner update point.  Group the scanners and scales into a scan zone.  Define the NoRead and ReadConflict strings.

When configured correctly, the scanned string will appear in the RcvData column.

| ScnScanDevice Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | Field | UpdatePt | NoReadString | ReadConflictStr |
| ShipScanner | ShipScnZn | Scan | N/A | Ship Scan | ???????? | ******** |

# SrtScanBoss Table

In table SrtScanBoss (Configuration Menu | Sorter Utilities | Scan Exceptions Configuration), link the sorter to the scan zone.

Verify the scan zone is properly linked to the sorter by checking ProcFlag in the Tracking Table.  ProcFlag is set by the scan processing module and is reset by the sorter processing module.

| SrtScanBoss Table |  |  |
| --- | --- | --- |
| Name | AppSorter | ScanZone |
| Ship Scan | Ship Sorter | ShipScnZn |

# Machine Table – Scanner Connections

The Machine table is used to define connections to external devices.  This section describes how to connect scanner and scale devices.

In table Machine (Configuration Menu | Communication Utilities | Machine), define the source of the scan data.  Scanners may be configured to use a serial port or a TCP interface.

Note:  TCP scanners required an entry in the /etc/hosts file that matches the Machine_Name string and relates it to the scanner’s IP address.

Note:  The protocols STXETX (for TCP) and Alphanum (for serial) both use the AsciiMnu table to filter the received scanner data.  You can alter the set of allowed characters by enabling or disabling them in the StxEtxAllow and AlphaNumAllow columns, respectively.

Note:  When rows are added or removed from the Machine table, FPC must be restarted for the changes to take effect.  In older versions, be sure to remove the “circular buffer” files (cb_*.cbf), found in RUN, before you restart.  Newer versions don’t use the cbf files, so it is always safe to remove them.

TCP scanner Machine table configuration:

| Machine Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry Time | Process | Trig_Size |
| LPNScan | CONNECT_ TCPCLIENT | STXETX | 9200 | 0 | CTRL | 0 |
| VerifyScan | CONNECT_ TCPCLIENT | STXETX | 9200 | 0 | CTRL | 0 |

| Machine Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trig_ InterChar | Trig_ Timeout | Trig_ MsgEndChar | Log Level | Close Delay(ms) | Open Timeout(ms) |
| 0 | 0 | ETX | Level 1 | 0 | 0 |
| 0 | 0 | ETX | Level 1 | 0 | 0 |

Serial scanner Machine table configuration:

| Machine Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry Time | Process | Trig_Size |
| LPNScan | CONNECT_ SERIAL | ALPHANUM | /dev/ser-rckt1 | 0 | CTRL | 2048 |
| VerifyScan | CONNECT_ SERIAL | ALPHANUM | /dev/ser-rckt2 | 0 | CTRL | 2048 |

| Machine Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trig_ InterChar | Trig_ Timeout | Trig_ MsgEndChar | Log Level | Close Delay(ms) | Open Timeout(ms) |
| 10 | 0 | Ctrl J (LF) | Level 1 | 0 | 0 |
| 10 | 0 | Ctrl J (LF) | Level 1 | 0 | 0 |

# MsgMap Table – Scanner Connections

In table MsgMap (Configuration Menu | Communication Utilities | Message Map), define where the scanner data will be placed after the message has been received.  Received data is usually delivered to the ScnScanDevice table.  The Record column in MsgMap must be set to the row number in ScnScanDevice that will receive the data.

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine Name | Recv | Send | Ack | Menu | Low | High | Stat | Key | Record |
| LPN Scan | LPNScan | ALL |  |  | ScnScanDevice | 2 | 2 | 3 |  | 1 |

# Machine Table – CommCore Connections

The Machine table is used to define connections to external devices.  This section describes how to connect FPC to CommCore to handle routing information.

In table Machine (Configuration Menu | Communication Utilities | Machine), define the connections for routing request, divert confirm, and other CommCore messages using TCP/IP.

Multiple connections are used, distinguished by Machine_Port, to achieve the required performance.  Each application may be different, but a typical configuration will use a separate port for the CRR messages and may combine CSU and DCM messages on another port.  Each record with a Connection_Type of CONNECT_TCPCLIENT in table Machine will require an entry in the /etc/hosts file matching Machine_Name to link the entry to CommCore’s IP address.

Note:  You can verify that the /etc/hosts configuration is correct by using the following command from the command line:
  ping <Machine_Name>

Note:  When rows are added or removed from the Machine table, FPC must be restarted for the changes to take effect.  In older versions, be sure to remove the “circular buffer” files (cb_*.cbf), found in RUN, before you restart.  Newer versions don’t use the cbf files, so it is always safe to remove them.

| Machine Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry Time | Process | Trig_ Timeout |
| SRTCRR | CONNECT_ TCPCLIENT | SrtCommSeq | 9210 | 10 | SERV | 0 |
| SRTDCM | CONNECT_ TCPCLIENT | SrtCommSeq | 9211 | 10 | SERV | 0 |

| Machine Table (Part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NeedACK | Log Level | UserName | Password | Open Timeout(ms) |
| 1 | Level 1 |  |  | 0 |
| 1 | Level 1 |  |  | 0 |

# MsgMap Table – CommCore Connections

In table MsgMap (Configuration Menu | Communication Utilities | Message Map), define the message table that will be associated with the Machine_Name and Machine_Port from table Machine.

Note:  The value in MsgMap_Stat corresponds to the TxStat column position in the message table.  The value in MsgMap_Key corresponds to the DbKey column position in the message table.  If the TxStat column in the message table does not change from the value “New”, MsgMap is not configured correctly.  Use F2 – Menu of Menus to find the message table, then Alt-K to open the table definition.  Find the row number for TxStat and DbKey and use them in MsgMap for Stat and Key.

Note:  If the TxStat column in the message table does not change from the value “Sending”, the AC is not connected to CommCore.  Double-check with the ping test, as described above, and make sure Machine_NeedAck = 1.

If all is well, the TxStat column in the message table will change to “Ack” as each message transaction is completed.

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine Name | Recv | Send | Ack | Menu | Low | High | Stat | Key | Record |
| CRR | SRTCRR | ALL | ALL | ALL | MsgCR1 | 1 | 60 | 67 | 68 | 0 |
| DCM | SRTDCM | ALL | ALL | ALL | MsgDC1 | 1 | 60 | 67 | 68 | 0 |

# SrtZoneLane Table

In table SrtZoneLane (Configuration Menu | Sorter Utilities | Destination Lane Configuration), define the lanes associated with a sorter and the agreed-upon name used with CommCore.  The column HostZone is used to define the logical destinations (divert points) used by CommCore.  An entry is needed for each logical destination.  The same physical lane may have multiple logical assignments.  A logical assignment does not have to be a physical lane.  For example:  The destination “STR” could be assigned to physical lane 0, causing the carton to go straight (to not divert).

| SrtZoneLane Table |  |  |  |
| --- | --- | --- | --- |
| Name | Enabled | AppSorter | HostZone |
| Lane 1 | Y | Ship Sorter | SHIP1 |
| Lane 2 | Y | Ship Sorter | SHIP2 |
| Lane 3 | Y | Ship Sorter | SHIP3 |

# Negative Divert Confirm Configuration

A normal divert confirm message (DCM) is generated when a container is diverted from the sorter.  The signal used to initiate the DCM can be the fire of the divert mechanism itself or a photoeye input triggered as the container exits onto the lane.

Some systems may require a DCM message to indicate that the carton did NOT successfully divert, as well.  This is particularly important for SKU-based systems where the containers do not have unique identities but may also be needed in LPN systems for more complete data collection purposes.

The DCM message has a column labeled iDCComplete, which contains the divert success/fail code.  A value of 1 indicates a successful divert.  For sorters, a value of 11 or 12 indicates failure.

There are two options:

- Set Verify_DCM_FAIL in the Outpoints table.  Negative divert confirms will be generated if the lane is not available (full or not enabled), if a BadGap occurs, or if a container is lost between shifters.

- Set an update point in SrtScanBoss_TooLateUpdatePt.  Negative divert confirms will be generated if the CRR message is received after the container passes the update point.

# TCP/IP Scanner Loop-back Testing

It is possible to use FPC to simulate a TCP/IP Scanner for testing.  This might be helpful if the connection to the actual scanner is not yet available and you wish to perform an end-to-end test of the configuration.

A server-style connection in table Machine is used to stand in for the scanner.  An extra entry in table MsgMap connects the “scanner” to the MsgTST table, which is then used as the source of the fake scans.

We will build on the example machine connection for LPNScan given in section 9, above.

Create two additional entries in table Machine to define the fake scanner server-style connection:

| Machine Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry Time | Process | Trig_ Timeout |
| LPN_ LISTEN | TCPSERV_ LISTEN | STXETX | 9200 | 10 | SERV | 0 |
| LPN_ RDWR | TCPSERV_ RDWR | STXETX | 9200 | 10 | SERV | 0 |

| Machine Table (Part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NeedACK | Log Level | UserName | Password | Open Timeout(ms) |
| 0 | Level 1 |  |  | 0 |
| 0 | Level 1 |  |  | 0 |

The Machine_Port values must match the value set for the CONNECT_TCPCLIENT entry (the LPNScan entry in our example).

Set the /etc/hosts entry for the LPNScan machine to point to the AC’s own IP address.  (Only CONNECT_TCPCLIENT connections need /etc/hosts entries).

Add a MsgMap entry to link the LPN_RDWR machine to the MsgTST table:

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine Name | Recv | Send | Ack | Menu | Low | High | Stat | Key | Record |
| LPN_ TEST | LPN_ RDWR | ALL | ALL | ALL | MsgTST | 1 | 1 | 67 | 68 | 0 |

Open the MsgTST table and enter the desired scan value (e.g., “123456”) into the CmdHdr column.  Enter a non-zero number (1 will work) into the DbKey column.

The “scan” should now be sent and processed.  The TxStat column will change to “Sent”.

To re-send the “scan”, change TxStat to “New”.