# FortnaPlus Control StarTech Serial Port Configuration

Document Revision: 1.1

Date: 10-Jan-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 03-Nov-2022 | T R Kremser | New document |
| 1.1 | 10-Jan-2023 | T R Kremser | Clarification on finding FTDI devices |

Table of Contents

Glossary of Terms	4

1.	Purpose	5

2.	Determine Device Serial Number	5

3.	Add Custom USB Configuration QNX	7

3.1.	The USB “char” file	7

3.2.	The USB “enum-usb.conf” file	8

4.	Using the New Configuration	9

5.	Pinouts and Additional Testing	10

6.	Mapping Devices That Are Already Mounted	11

7.	Checking Your Work	12

8.	Preserve Custom Configuration	12

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Purpose

Due to supply constraints on Beckhoff CX2500-0030 (RS232) and CX2500-0031 (RS422/RS485) modules, we have been forced to find another supplier.  StartTech ICUSB422 RS422/RS485 USB-to-Serial devices have been vetted and can be used in place of or in addition to the Beckhoff parts.

This document explains additional configuration needed within QNX when using more than one ICUSB422 device on an AC.

By default, if multiple ICUSB422 devices are added, they will be randomly assigned to serial device designations at boot time.  Since each serial connection will be to a distinct scanner or scale, we require the same logical serial port device to be assigned to each physical device every time the AC is restarted.

Each physical device has a unique USB serial number assigned to it at the factory.  We can use that, along with some extra configuration in QNX to ensure that a particular physical device is always assigned to the same logical device.

# Determine Device Serial Number

Use the “usb -vvv” command at a QNX command prompt to find the serial number of each ICUSB422 device.  Label each device with its serial number and the QNX serial device you wish to assign to it.

The Vendor (ven or vid), Product (dev or did) and Serial Number (ser) will be used to uniquely identify each ICUSB422 device.

- Open a command terminal, either from within the Photon environment (or from a Phindows connection), or via a PuTTY terminal connection.

- First, we must exclude any FTDI devices already present on the system.  Beckhoff uses these devices, too, as part of the touchscreen monitor USB features.  Run “usb -vvv”.  Look for Vendor (ven or vid) = 0x0403 and Product (dev or did) = 0x6001.  Find the serial numbers for all devices that match the vid/did combination and make a note of them.  See the example output below.

- Attach one ICUSB422 device to a USB port.  (Note that you can do this using a virtual machine.  Just make sure the USB device is assigned to the VM, not the host).

- Re-run “usb -vvv” and search the output for the new FTDI device.  It can usually be found in the last section in the output, since it was the last device added.  Record its serial number along with the QNX serial device that you want to assign to it (like /dev/serusb2).  Note that the Beckhoff touchscreen may consume /dev/serusb1.

- Remove the physical device and repeat for each of the others.  Do them one by one so you can tell them apart.  Label each physical device with its Serial Number and intended QNX device.

- Remove the last device when done.

Device Address             : 3

Upstream Host Controller   : 0

Upstream Device Address    : 2

Upstream Port              : 1

Upstream Port Speed        : Full

Vendor                     : 0x0403 (FTDI)

Product                    : 0x6001 (FT232R USB UART)

Device Release             : r6.00

USB Spec Release           : v2.00

Serial Number              : AB0NDWX8

Class                      : 0x00 (Independent per interface)

Max PacketSize0            : 8

Languages                  : 0x0409 (English)

Current Frame              : 846 (1024 bytes)

Configurations             : 1

Configuration            : 1

Attributes             : 0xa0 (Bus-powered, Remote-wakeup)

Max Power              : 90 mA

Interfaces             : 1

Interface            : 0 / 0 (FT232R USB UART)

Class              : 0xff (Vendor-specific)

Subclass           : 0xff

Protocol           : 0xff

Endpoints          : Control + 2

Endpoint         : 0

Attributes     : Control

Max Packet Size: 8

Endpoint         : 1

Attributes     : Bulk/IN

Max Packet Size: 64

Endpoint         : 2

Attributes     : Bulk/OUT

Max Packet Size: 64

USB 1 (EHCI) v1.10, v1.01 DDK, v1.01 HCD

Control, Interrupt, Bulk(SG), Isoch(Stream), High speed

# Add Custom USB Configuration QNX

NOTE:  The files referenced in this section can be found in StarTechUSBMods.tar.gz on the ftp4 server in ftpdata/tkremser/Tools.

Expand the StarTechUSBMods.tar.gz file in /SortPlus/RUN.  This will create a new directory, StarTech, with default copies of the files described here.  You will need to modify the enum-usb.conf file, and both files will need to be copied to their “active” locations.  If you later modify the files in their “active” locations, be sure to copy them back to /SortPlus/RUN/StarTech so they will be backed up when using dobackups or the Create Backup option within FPC.

  - The USB “char” file

QNX is aware of the ICUSB422 device, along with a few other character-mode devices.  A default configuration is found in one of the files that we will modify:  /etc/system/enum/devices/usb/char.  By default, it will recognize these devices and assign them, in the order detected, to /dev/serusb1, /dev/serusb2, etc.  To assign specific devices, we move the configuration for this device out of the generic section and create individual entries for each port device, tagged with a parameter name, as follows.  We add the generic entry for the FTDI 8U232AM device back in at the end for any that do not have a serial number defined, because Beckhoff uses one of these devices internally for the USB ports on the touchscreen monitor.  This is the entire modified file:

#USB-to-serial dongles

device(usb,ven=0557,dev=2008)   # ATEN_232A/GUC_232A

device(usb,ven=2478,dev=2008)   # TRIPP LITE U2009-000-R

device(usb,ven=9710,dev=7720)   # MOSCHIP 7720

device(usb,ven=1199,dev=0120)   # Sierra AirCard U595

device(usb,ven=0681,dev=0040)   # Siemens HC25

device(usb,ven=1bc7,dev=1003)   # Telit UC864E

device(usb,ven=067b,dev=2303)   # Prolific

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno)" )

#Specific instances of StarTech.com ICUSB422 USB to RS422 Serial Adapters

device(usb,ven=0403,dev=6001,usr_spec_id=StarTechSer1)  # FTDI 8U232AM

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno),unit=1" )

device(usb,ven=0403,dev=6001,usr_spec_id=StarTechSer2)  # FTDI 8U232AM

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno),unit=2" )

device(usb,ven=0403,dev=6001,usr_spec_id=StarTechSer3)  # FTDI 8U232AM

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno),unit=3" )

device(usb,ven=0403,dev=6001,usr_spec_id=StarTechSer4)  # FTDI 8U232AM

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno),unit=4" )

#Generic instance of FTDI 8U232AM, since Beckhoff uses that chip, too!

#Put it last so the specific ones get assigned first

device(usb,ven=0403,dev=6001)   # FTDI 8U232AM

start(devc-serusb, "-d vid=0x$(ven),did=0x$(dev),busno=0x$(busno),devno=0x$(devno)" )

The section for StarTech defines four specific instances of ven=0403, dev=6001, each with a different usr_spec_id parameter.  The first uses parameter StarTechSer1, which we will use as an example.  The parameter is declared in the enum-usb.conf file, as described in the next section.  StarTechSer1 will cause device driver devc-serusb to start with unit=1, which will cause it to create /dev/serusb1.

NOTE:  As long as you do not have more than four devices, this file does not need any additional modifications.  More devices could be defined if needed, following the pattern.

Copy this file to its active location with the following command:

cp char /etc/system/enum/devices/usb/char

  - The USB “enum-usb.conf” file

To activate a unique configuration by serial number, we must detect the serial number and use it to set the parameters that will tell the “char” file in the previous section what to do.  This is done using the /etc/enum-usb.conf file.  The modified version found in StarTechUSBMods.tar.gz has template entries defined to match the four StarTechSerx parameters expected in the “char” file.  They are commented-out by default.  This is the entire modified file:

# IPod Devices

#

#Device vid=05AC,did=12*

#

##Check for the presence of an Audio class interface first. If the Audio class

##interface is present, use that configuration. If it is not present, select

##the configuration with a UMASS class interface.

#

#       Config class=01

#               Set usr_spec_id=AppleIpod

#               Set inc_usr_spec_id=/fs/ipod

#       Config class=08

#Check for a StarTech.com USB to RS422 Serial device with the given serial number

#Device vid=0403,did=6001,ser=XXXXXXXX

#        Set usr_spec_id=StarTechSer1

#Device vid=0403,did=6001,ser=XXXXXXXX

#        Set usr_spec_id=StarTechSer2

#Device vid=0403,did=6001,ser=XXXXXXXX

#        Set usr_spec_id=StarTechSer3

#Device vid=0403,did=6001,ser=XXXXXXXX

#       Set usr_spec_id=StarTechSer4

Lines that begin with “#” are comments and have no effect.  The first portion, referring to IPod Devices, was the default file.  I have added the section for StarTech devices.  Each device will require a pair of lines beginning with Device and Set.  Remove the comment characters for each device that you wish to use, then replace XXXXXXXX with the serial number you found in section 2.  The pair of lines you pick here also assigns the QNX device designation:  StarTechSer1 will connect the device to /dev/serusb1, StarTechSer2 to /dev/serusb2, etc.

Choose the logical device name (like /dev/serusb1) that you want.  Add it to the label on the physical device that already has its serial number from section 2.

Copy the modified enum-usb.conf file to its active location with this command:

cp enum-usb.conf /etc/enum-usb.conf

# Using the New Configuration

The configuration file changes take effect immediately, so the next device that is plugged in to a USB port will use them.

It doesn’t matter which USB port you use, or the order that you add them – the configuration will assign each device to its port as you have specified.

Connect each physical device, and use “ls /dev/ser*” after each one to verify that the expected device file appears.

NOTE:  If you don’t get what you expect, double check your serial numbers!  Based on a small sample, it appears they use “1” (one) and “I” (capital ‘eye’), as well as “0” (zero) and “O” (capital ‘oh’), and possibly other similar-looking characters, so look closely!

# Pinouts and Additional Testing

You can use qtalk and a loop-back connector, or a null-modem style cable to test the ports.

Here is the pin assignment table from the StarTech documentation:

| DB9 Pin Assignment |  |  |
| --- | --- | --- |
| DB9 Pin | RS-422 | RS-485 |
| 1 | TxD- | Data- |
| 2 | TxD+ | Data+ |
| 3 | RxD+ | NC |
| 4 | RxD- | NC |
| 5 | GND | GND |
| 6 | NC | NC |
| 7 | NC | NC |
| 8 | NC | NC |
| 9 | NC | NC |

To create a loopback test connector, you will need a female DB9 connector.  Connect pin 1 (TxD-) to pin 4 (RxD-), and pin 2 (TxD+) to pin 3 (RxD+).

To create a null-modem cable:

| RS-422 Null-Modem Cable |  |  |  |  |
| --- | --- | --- | --- | --- |
| DB9 Pin | Signal |  | Signal | DB9 Pin |
| 1 | TxD- |  | RxD- | 4 |
| 2 | TxD+ |  | RxD+ | 3 |
| 3 | RxD+ |  | TxD+ | 2 |
| 4 | RxD- |  | TxD- | 1 |
| 5 | GND |  | GND | 5 |

Start qtalk, referencing the port device you wish to use:

qtalk -m /dev/serusb1

With the loopback plug, you should see characters you type echoed to the terminal.

Open two instances of qtalk in two separate terminal windows to use the null-modem cable.  You will see the characters you type in one qtalk window appear in the other qtalk window, and vice-versa.

To exit qtalk, use Alt-A, then q to quit.

# Mapping Devices That Are Already Mounted

It is possible to figure out the serial number to device file relationship if you have mounted all the ICUSB422 devices already.

Use “ls /proc/mount/dev/serusb*” to find the process IDs of the active devices.  The file names are composed of five numbers separated by commas.  The second number is the process ID (pid).  You will see something like this:

# ls /proc/mount/dev/serusb*

/proc/mount/dev/serusb1:

0,52064315,1,0,0

/proc/mount/dev/serusb2:

0,51765299,1,0,0

Use “ps -ef | grep devc-serusb” to see the command line, with parameters, used to create each device file.

# ps -ef | grep devc-serusb

0          51765299      77837    0 Nov02 ?        00:00:00 devc-serusb -d vid=0x0403,did=0x6001,busno=0x00,devno=0x03,unit=2

0          52064315      77837    0 Nov02 ?        00:00:00 devc-serusb -d vid=0x0403,did=0x6001,busno=0x00,devno=0x04

0          52162621   39501869    0 10:55 ?        00:00:00 grep devc-serusb

NOTE:  For this example, pid=51765299 was mounted with a specific serial number and pid=52064315 was mounted using the generic entry.  Notice the unit=2 parameter is present for the one mounted using its serial number, but not for the generic one.

Use “usb -vvv”, as before, to find the serial number for each device.  “devno” in the ps output corresponds to “Device Address” in the usb command output.

Once you have the /dev/serusb to serial number cross-reference, you can use it to complete the enum-usb.conf file and make the configuration permanent.

# Checking Your Work

As noted in the previous section, the “ps -ef | grep devc-serusb” command can help you distinguish between USB devices that have been mounted by serial number and those that have used the generic mount.  Those that show “unit=” in the output have been mounted based on their specific serial number, as defined in /etc/enum-usb.conf.  If “unit=” is missing, they were mounted using the generic entry, and could re-mount on a different device after a reboot.

# Preserve Custom Configuration

The QNX configuration files /etc/system/enum/devices/usb/char and /etc/enum-usb.conf are not captured to RUN/SystemFiles when you use the dobackups command or the Create Backup option within FPC (not yet, at least – I’ll fix that eventually).  In section 3, we expanded the StarTechUSBMods.tar.gz file in /SortPlus/RUN to create a StarTech directory.  We modified the default files there before copying them to their “active” locations.  If you have made modifications to the active files, be sure to copy them back to /SortPlus/RUN/StarTech so the changes will be captured when backups are created.

If the AC is ever replaced, these files must be copied to their “active” locations again.

NOTE:  If the ICUSB422 devices are ever replaced, the procedures described in this document must be followed again to learn the serial number of the new devices and to update the /etc/enum-usb.conf file accordingly.