# FortnaPlus Control Transfer Configuration Example and Checklist

Document Revision: 1.2

Date: 30-Nov-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.1 | 02-Jan-2015 | B C Conklin | Added transfer example and updates |
| 1.2 | 30-Nov-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	5

1.	Example Parts and Names	6

2.	The Configuration	7

2.1.	The Batches Table	7

2.2.	The ScnScanZone Table	8

2.3.	The ScnScanDevice Table	8

2.4.	The XfRouteBoss Table	8

2.5.	The XfrBoss Table	9

2.6.	The XfrTree Table	11

2.7.	The XfRouteTable Table	11

2.8.	The XfrConfig Table	12

2.9.	The XfrCheckSet Table	12

2.10.	The XfrGoSet Table	13

2.11.	Left-Reverse Route State 1	13

2.12.	Left-Reverse Route State 2	15

2.13.	Left-Reverse Route State 3	16

2.14.	The Scanner Interface	17

2.15.	The CommCore Interface	19

3.	Transfer Testing and Debugging	23

4.	Scanner Physical Configuration	25

5.	Accumulation Conveyor Configuration	26

6.	Process Flow	27

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

CommCore – The Warehouse Control System, also known as “Level 2”.  Provides destination information for containers and collects sortation results.  CommCore connects the client’s host business system to FPC.

FPC – FortnaPlus Control, also known as “Level 1”.

UPS – Uninterruptible Power Supply

# Example Parts and Names

In this example, we will configure a pick module transfer, called Pick1.

The transfer has three routes.  We will examine the left-reverse route.

We will name the various parts as shown below:

- The transfer routes:

  - Pick1-L-R – The Left-Reverse Route

  - Pick1-R-R – The Right-Reverse Route

  - Pick1-Str – The Straight route

- Level 2 Communications:

  - CRR is sent through Port 9310

  - DCM is sent through Port 9311

- Transfer I/O

  - EZPE100_P – The presence photoeye input

  - SSV_EZPE100_P1 – The release solenoid output

  - PE101_J – The Jam photoeye input

  - SSV_101_UP – The output that raises the bed

  - SSV_101_LEFT – The output that runs the bed motor to the left

  - SSV_101_RIGHT – The output that runs the bed motor to the right

  - PRS101_UP – The bed up proximity switch

  - PRS101_DN – The bed down proximity switch

  - PE102_F – The full photoeye input for the left reverse route

  - PE103_F – The full photoeye input for the right reverse route

  - PE104_F – The full photoeye input for the straight route

  - M102 – The output that runs the left reverse motor

  - M103 – The output that runs the right reverse motor

  - M104 – The output that runs the motor for the straight route

The transfer is laid out as shown in the following diagram.  Containers enter from the left, are scanned, then can move either straight through, or to the left or right, then back.

Presence Photoeye

EZPE100_P

Scanner

Box

M101

M104

M102

M103

Jam Photoeye

PE101_J

Full Photoeye

PE104_F

Full Photoeye

PE102_F

Full Photoeye

PE103_F

M101

# The Configuration

This section describes the content of each table required to create a configuration to match the diagram in section 1.

# The Batches Table

In table Batches (Configuration Menu | Standard Controls | Batches), activate the transfer module and processing for scans as shown in the table, below.  You can use the StartXfr button in the Configure Transfers screen (Configuration Menu | Transfer Utilities | Configure Transfers) to automatically activate the existing Batches entries for the transfer module, or just populate the Machine column.

NOTE: Use “ScnApp CTRL”.  “DO SCANS” is obsolete.

| Example Batches Table |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| File_Name | How Often | Increment | Type | Process | Internal Proc | InternalProc Ctrl | Machine | Do File |
| Scan Process | 1 | Invalid | Count | CTRL |  | ScnApp CTRL | CP2 | N |
| Xfr CTRL | 1 | Invalid | Count | CTRL |  | XfrApp CTRL (CPN) | CP2 | N |
| Xfr SERV | 1 | Invalid | Count | SERV | XfrApp SERV (CPN) |  | CP2 | N |
| Xfr Process | 1 | Invalid | Count | CTRL |  | XfrProcess (CTRL) | CP2 | N |

# The ScnScanZone Table

The scanner for the transfer requires a scan zone to be defined in the ScnScanZone table (Configuration Menu | Scanner Utilities | Scan Zone Config).  The ScanZoneID field is used to define the link to the “Decision Point” in CommCore.  This table also declares the tracking tables that will be used for the scan zone.

| Example ScnScanZone Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Delimiter | ScanZoneID | TrackingTable | DeviceTrkTable | PrinterTrkTable |
| Pick1Zone |  | Pick1 | XfrTrack | XfrDevice | None |

# The ScnScanDevice Table

The ScnScanDevice table defines how the scanner will behave and creates a place for the scan data received through the Machine table to be stored.  The MsgMap table defines the link from Machine to ScnScanDevice.

Leave UpdatePt at “Invalid” for transfers.

Set the NoReadString and ReadConflictStr values to match the strings the scanner will return.

| Example ScnScanDevice Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | Field | UpdatePt | NoReadString | ReadConflictStr |
| Pick1Scan | Pick1Zone | Scan | N/A | Invalid | ???????? | ******** |

In old versions, these fields, which are no longer present in the table, also had to be defined here: TrnNoScnTmr – A timer used to detect NoScan events. Now handled by XfRouteBoss_NoScanTmr. PresenceEye – The photoeye that activated TrnNoScanTmr. ConvRunIO – The IO part that would stop the TrnNoScanTmr if the line stopped for a system stop event. The timers are now controlled based on XfrBoss_ProductINPUT.

# The XfRouteBoss Table

The XfRouteBoss table (Configuration Menu | Transfer Utilities | Transfer Scan Processing) links the scan zone to the transfer and defines the message tables that will handle the messages to CommCore.

Note: In ConfigXfgScrn, the AddRouteConfig button creates a XfRouteBoss record.  Select an existing XfRouteBoss record using the down-arrow by XfrRouteConfig.

| Example XfRouteBoss Entries |  |  |
| --- | --- | --- |
| Field | Example Value | Description |
| Name | Pick1 | Enter a unique name for this transfer. |
| ScanZone | Pick1Zone | Choose the scan zone. |
| CrrMsgTable | MsgXR1 | The table for CRR messages. |
| CsmMsgTable | None | Only needed if CSM messages are required. |
| DcmMsgTable | MsgXDC1 | The table for DCM messages. |
| CsuMsgTable | MsgXDC1 | The table for CSU messages. OK to share with DCM |
| CommErr | N/A | Choose a route to use if CommCore does not respond. Set NoRouteTmr to define how long to wait. |
| NoRouteTmr | N/A | How long to wait before declaring the error defined in CommErr. Old versions used MaxWaitTime for this purpose. MaxWaitTime is no longer used. |
| NoCartonTmr | N/A | How long to wait for the presence photoeye to be blocked after a scan is received. |
| CrQueDlyTmr | N/A | This timer is used to detect when a container is physically removed from a transfer. A “container removed” event occurs if the presence photoeye detects a container, but it goes OFF before the container is released by the software |
| DontResetQue | N | A “No Carton” event will occur if a scan is received but the presence photoeye is not activated before NoCartonTmr expires |
| SystemRun | PickStart | Choose a conveyor IO part that will hold NoRouteTmr if the system stops. |
| WaitPresence | Y | Wait for the presence photoeye before the queued move is activated in XfrBoss. |
| RunNewXfr | Y | Use the “New Transfer” module. |

# The XfrBoss Table

The XfrBoss table (Configuration Menu | Transfer Utilities | Transfer Boss) defines a set of paths that a container can take as it moves through a transfer.

Note: In ConfigXfgScrn, the AddXfr button creates a XfrBoss record.  Select an existing XfrBoss record using the down-arrow by SelectedXfr.

| Example XfrBoss Entries |  |  |
| --- | --- | --- |
| Field | Example Value | Description |
| Name | Pick1 | Enter a unique name |
| LogLevel | Level 1 | Control transfer-related logging |
| MaxWaitTM | tm_CP2_X01MaxWait | Set this timer to the worst-case time needed to complete the slowest route. |
| ProductINPUT | EZPE100_P | Choose the presence photoeye. |
| ProductSENSE | Is On | Choose the state that indicates a container is present. |
| ReleaseOUTPUT | SSV_EZPE100_P1 | Choose the motor or solenoid device (SSV_EZPE100_P1) that will be used to stop the container to wait for traffic or to wait for routing from CommCore. This device is usually started in motor chains and is disabled by the transfer when the photoeye transitions ON. The transfer logic handles this automatically. The hardware will usually require an input (output from FPC) that must be enabled to allow the container to feed onto the transfer. If a second container arrives, the transfer will automatically disable ReleaseOUTPUT to hold it. This allows containers to feed into the transfer, automatically stop, wait for traffic or routing information, then continue through the transfer when ready. |
| ReleaseAction | Disable | Choose the way ReleaseOUTPUT will be stopped. Alternatively, this could be set to “Fire”, but then XfrCfgOutput_OutputAction must also be set to “Fire” to feed containers onto the transfer. |
| EnableMaxWait | INVALID | Choose a part here to suspend MaxWaitTM while the system is stopped to prevent spurious transfer timeouts. NOTE: If the transfer must be suspended in a specific state, use XfrConfig_EnabledINPUT instead. |
| EnableMxSense | N/A | Choose the state of EnableMaxWait that will indicate when the system is stopped. |
| Machine | CP2 | Choose the AC that controls this entry. |

# The XfrTree Table

Table XfrTree (Configuration Menu | Transfer Utilities | Transfer Tree) defines all the possible routes a container can take through a transfer.

Note: In ConfigXfgScrn, the AddTree button creates a XfrTree record.  Select an existing XfrTree record using the down-arrow by SelectedTree.

| Example XfrTree Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Boss | Dest | DoConfirm | AvailINPUT | AvailSENSE |
| Pick1-L-R | Pick1 | Pick1-L-R | Y | N/A | N/A |
| Pick1-R-R | Pick1 | Pick1-R-R | Y | N/A | N/A |
| Pick1-Str | Pick1 | Pick1-Str | Y | N/A | N/A |

# The XfRouteTable Table

Table XfrRouteTable (Configuration Menu | Transfer Utilities | Transfer Destination Config) contains entries that define the routes for each XfRouteBoss record (or scanner – or decision point in CommCore lingo).

The HostRouteStr field defines the agreed-upon name that links the FPC configuration to the logical destination, or “divert point” used by CommCore.  An entry is required for each divert point defined in CommCore.  The same physical route may have multiple logical assignments.  The same logical destination string may be used for multiple transfers.  For example, the destination “STR” could be assigned to send the container straight on more than one transfer.

Note: In ConfigXfgScrn, the AddRoute button creates a XfRouteTable record.  Select an existing XfRouteTable record using the down-arrow by XfrRoute.

| Example XfRouteTable Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Enabled | BossRecord | HostRouteStr | Route | NewRoute |
| Pick1 L-R | Y | Pick1 | Pick1-R-F | N/A | Pick1-R-F |
| Pick1-R-R | Y | Pick1 | Pick1-R-R | N/A | Pick1-R-R |
| Pick1-Str | Y | Pick1 | Pick1-Str | N/A | Pick1-Str |

# The XfrConfig Table

The XfrConfig table (Configuration Menu | Transfer Utilities | Transfer Config) defines a sequence of states, inputs, outputs, input timers and delay timers used to control a transfer through a series of steps.  XfrConfig entries are linked to a specific transfer through XfrGoSet records, and traffic is controlled through the XfrCheckSet records.

XfrConfig also defines the current state of a route in pRun, pStatus and pState.

EnableINPUT and EnabledSENSE can be used to stop and hold a transfer in a specific state.

Note: In ConfigXfgScrn, the AddConfig button creates a XfrConfig record.  Select an existing XfrConfig record using the down-arrow by SelectedConfig.

| Example XfrConfig Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | pState | EnabledINPUT | EnabledSENSE | EnabledDelay |
| Pick1-L-R | Idle | INVALID | N/A | N/A |
| Pick1-R-R | Idle | INVALID | N/A | N/A |
| Pick1-Str | Idle | INVALID | N/A | N/A |

# The XfrCheckSet Table

The XfrCheckSet table (Configuration Menu | Transfer Utilities | Transfer Check Set Recs) defines routes from table XfrConfig that must be “Idle” before the route defined in XfrTree can be started.  All the XfrCheckSet records associated with a XfrTree must be “Idle” before the XfrTree route can run.  In complex applications, the columns MyGroup and CheckGroup may be used to create a list of transfers that are checked for an “Idle” condition.

Note: In ConfigXfgScrn, the AddCheckSet button creates a XfrCheckSet record.  Select an existing XfrCheckSet record using the down-arrow by CheckSet.

| Example XfrCheckSet Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Tree | XfrConfig | MyGroup | CheckGroup |
| Pick1-L-R | Pick1-L-R | Pick1-L-R | N/A | N/A |
| Pick1-R-R | Pick1-L-R | Pick1-R-R | N/A | N/A |
| Pick1-Str | Pick1-Str | Pick1-Str | N/A | N/A |

# The XfrGoSet Table

The XfrGoSet table (Configuration Menu | Transfer Utilities | Transfer Go Set Recs) lists configuration sequences that will be executed for the selected tree.  Each tree may be linked to any number of XfrConfig records by adding entries in this table.  Execution will not begin until all the XfrCfgInput and XfrCfgInTimer entries are ready for State1.

A typical transfer requires only one XfrConfig record or route to run for a XfrTree and starts from the “Idle” state.

Note: In ConfigXfgScrn, the AddGoSet button creates a XfrGoSet record.  Select an existing XfrGoSet record using the down-arrow by GoSet.

| Example XfrGoSet Table |  |  |  |
| --- | --- | --- | --- |
| Name | Tree | XfrConfig | InitialState |
| Pick1-L-R | Pick1-L-R | Pick1-L-R | Idle |
| Pick1-R-R | Pick1-L-R | Pick1-R-R | Idle |
| Pick1-Str | Pick1-Str | Pick1-Str | Idle |

# Left-Reverse Route State 1

In State1, we move the container onto the transfer bed.  When the presence photoeye goes OFF, we will wait 0.25 seconds before advancing to State2.

Before the transfer can move the container onto the transfer bed, the bed must be in the down position and the jam photoeye must be OFF.  The input signals needed to enter State1 are defined in table XfrCfgInput:

| Example XfrCfgInput Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredInput | InputSense |
| Check Bed Down | C01 | State1 | PRS101_DN | Is On |
| Check Jam Clear | C01 | State1 | PE101_J | Is Off |

A full-clear timer is used to make sure the transfer bed is clear before releasing a container onto it.  The input timer is defined in table XfrCfgInTimer.  It must be “Done” (Is On) to enter State1:

| Example XfrCfgInTimer Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredTimer | TimerSense |
| Check Not Full | C01 | State1 | tmfc_PE102_F | Is On |

The output that will be activated for State1 is defined in table XfrCfgOutput:

| Example XfrCfgOutput Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | Output | Output Action | State1 Action | State2 Action | State3 Action |
| Release Solenoid | C01 | SSV_EZPE100_P1 | Enable | Activate | N/A | N/A |

Note that the transfer module software automatically disables XfrBoss_ReleaseOUTPUT if the XfrBoss_ProductINPUT transitions ON, even if the output is being enabled here.

A delay timer is used to hold the transfer in this state before moving to State2 so the container can move completely onto the transfer bed.  It is defined in table XfrCfgDlyTimer:

| Example XfrCfgDlyTimer Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | Timer | TimerAction |
| Delay State1 | C01 | State1 | tmCnf01State1Delay | Run Timer |

The timer Preset_Value is set to 0.25 seconds in table timemenu.

Notes:

In ConfigXfgScrn, the AddInput button creates a XfrCfgInput record.  Select an existing XfrCfgInput record using the down-arrow by Inputs.

In ConfigXfgScrn, the AddInTimer button creates a XfrCfgInTimer record.  Select an existing XfrCfgInTimer record using the down-arrow by InTimers.

In ConfigXfgScrn, the AddOutput button creates a XfrCfgOutput record.  Select an existing XfrCfgOutput record using the down-arrow by Outputs.

In ConfigXfgScrn, the AddDelayTimer button creates a XfrCfgDlyTimer record.  Select an existing XfrCfgDlyTimer record using the down-arrow by DelayTimers.

# Left-Reverse Route State 2

In State2, we raise the transfer bed and move the container to the left onto the side spur.  Once the container has cleared the transfer bed, as indicated by the jam photoeye going OFF, we continue to run the bed to the left for 0.5 seconds.

In ConfigXfgScrn, advance to State2 by clicking the NextState button.  The state currently displayed appears in SelectedState.

Before the transfer can raise the bed, it must wait for the presence photoeye to turn OFF.  The input signal needed to enter State2 is defined in table XfrCfgInput:

| Example XfrCfgInput Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredInput | InputSense |
| Presence PE Clear | C01 | State2 | EZPE100_P_J | Is Off |

Note that the transfer module software automatically disables XfrBoss_ReleaseOUTPUT if the XfrBoss_ProductINPUT transitions ON, even if the output is being enabled here.

The outputs that will be activated for State2 are defined in table XfrCfgOutput:

| Example XfrCfgOutput Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | Output | Output Action | State1 Action | State2 Action | State3 Action |
| Release Solenoid | C01 | SSV_EZPE100_P1 | Enable | Activate | N/A | N/A |
| Raise Bed | C01 | SSV_101_UP | Fire | N/A | Activate | DeActivate |
| Move Left | C01 | SSV_101_LEFT | Fire | N/A | Activate | DeActivate |

Note that we have already defined the outputs to be deactivated when we reach State3.

A delay timer is used to hold the transfer in State2 before moving to State3 so the container can move fully onto the side spur after the container clears the transfer bed.  It is defined in table XfrCfgDlyTimer:

| Example XfrCfgDlyTimer Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | Timer | TimerAction |
| Delay State2 | C01 | State2 | tmCnf01State2Delay | Run Timer |

The timer Preset_Value is set to 0.5 seconds in table timemenu.

Notes:

In ConfigXfgScrn, the AddInput button creates a XfrCfgInput record.  Select an existing XfrCfgInput record using the down-arrow by Inputs.

In ConfigXfgScrn, the AddOutput button creates a XfrCfgOutput record.  Select an existing XfrCfgOutput record using the down-arrow by Outputs.

In ConfigXfgScrn, the AddDelayTimer button creates a XfrCfgDlyTimer record.  Select an existing XfrCfgDlyTimer record using the down-arrow by DelayTimers.

# Left-Reverse Route State 3

In State3, we deactivate the outputs from State2 to stop moving left and allow the bed to return to the lowered position.  We fire a memory bit for 0.04 seconds which triggers a Fullline entry to run the side spur for 6.0 seconds.

In ConfigXfgScrn, advance to State3 by clicking the NextState button.  The state currently displayed appears in SelectedState.

The bed must be down and the jam photoeye must be clear.  The input signals needed to enter State3 are defined in table XfrCfgInput:

| Example XfrCfgInput Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredInput | InputSense |
| Check Bed Down | C01 | State3 | PRS101_DN | Is On |
| Check Jam Clear | C01 | State3 | PE101_J | Is Off |

The outputs that will be activated or deactivated for State3 are defined in table XfrCfgOutput:

| Example XfrCfgOutput Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | Output | Output Action | State1 Action | State2 Action | State3 Action |
| Release Solenoid | C01 | SSV_EZPE100_P1 | Enable | Activate | N/A | N/A |
| Raise Bed | C01 | SSV_101_UP | Fire | N/A | Activate | DeActivate |
| Move Left | C01 | SSV_101_LEFT | Fire | N/A | Activate | DeActivate |
| MEM_Run_Spur | C01 | MemSpur102 | Fire | N/A | N/A | Activate |

A delay timer is used to hold the transfer in State3 so the container can move along the side spur.  It is defined in table XfrCfgDlyTimer:

| Example XfrCfgDlyTimer Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | Timer | TimerAction |
| Delay State3 | C01 | State3 | tmCnf01State3Delay | Run Timer |

The timer Preset_Value is set to 0.04 seconds in table timemenu to strobe the memory bit.  When it turns ON the side spur will run, pushing the container away from the transfer.

An entry in table Fullline defines a full/clear timer that will run the side spur for 6.0 seconds, triggered by the memory bit:

| Example Fullline Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Desc | Sensor_Name | Timer_Name | Timer_ Preset | Clr_Timer_Name | Clr_Timer_ Preset | FullClear Output | FullClear ACTION |
| Spur102 | MEMSpur102 | tmMEMSpur102 | 0.04 | tmfcMEMSpur102 | 6.00 | M102 | FIRE |

The Fullline entry is triggered when the transfer completes.  The transfer only requires about 0.04 seconds to trigger the event.  The spur conveyor will continue to run for 6.0 seconds.  The transfer is free to run an additional container while the spur runs.

Notes:

In ConfigXfgScrn, the AddInput button creates a XfrCfgInput record.  Select an existing XfrCfgInput record using the down-arrow by Inputs.

In ConfigXfgScrn, the AddOutput button creates a XfrCfgOutput record.  Select an existing XfrCfgOutput record using the down-arrow by Outputs.

In ConfigXfgScrn, the AddDelayTimer button creates a XfrCfgDlyTimer record.  Select an existing XfrCfgDlyTimer record using the down-arrow by DelayTimers.

# The Scanner Interface

The interface to the scanner requires configuration in the Machine and MsgMap tables.

The Machine table (Configuration Menu | Communication Utilities | Machine) defines how the connection is to be made.

| Machine Table |  |
| --- | --- |
| Field | Description |
| Machine_Name | Sets the name for this connection within FPC. It will appear in log files and requires a matching entry in /etc/hosts to link the name to the IP address of the scanner. |
| Connection_Type | Defines the connection method. The most common types are CONNECT_SERIAL for serial scanners and CONNECT_TCPCLIENT for EtherNet scanners. |
| Communication_Protocol | The dialect to “speak” over the connection. Scanners almost always use the AlphaNum protocol. |
| Machine_Port | Defines the port for this connection. EtherNet scanners will use a port number, serial scanners use a QNX serial device identifier like /dev/ser1. |
| Transmit_Retry | FPC does not transmit to scanners, so the value here does not matter. Still, it is best to set it to 10 seconds. |
| Process | Choose the CTRL process. |
| Trig_MsgEndChar | Choose the character that the scanner is defined to send to indicate the end of the message, usually CTRL-C (ETX) |
| LogLevel | Set this to “Level 1” to see the message content in the logs. Higher values generate more detail. |

Example values for an EtherNet scanner:

| Machine Table Example |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_ Type | Communication_ Protocol | Machine_ Port | Transmit_ Retry | Process | Trig_ MsgEndChar | Log Level |
| Pick1Scan | CONNECT_ TCPCLIENT | AlphaNum | 9200 | 10 | CTRL |  | Level 1 |

The MsgMap table (Configuration Menu | Communication Utilities | Message Map) defines where the scan data will be stored and connects to the Machine entry.

| MsgMap Table |  |
| --- | --- |
| Field | Description |
| Message_Name | Set the name for this map entry within FPC. May appear in log files. |
| Machine_Name | Choose the related Machine entry here. |
| Recv_Msg_Type | Define the message type to process for received messages. Use ALL to match any message received by the scanner. |
| Send_Msg_Type | Not applicable for scanner entries. |
| Ack_Msg_Type | Not applicable for scanner entries. |
| Menu_Name | Choose the table where the scan data is to be stored. |
| Low_Cursor | Enter 2, the column number of the RcvData column in the ScnScanDevice table. |
| High_Cursor | Enter 2, the column number of the RcvData column in the ScnScanDevice table. |
| Stat_Cursor | Enter 3, the column number of the RxStat column in the ScnScanDevice table. |
| Key_Cursor | Not applicable for scanner connections. Leave at 0. |
| Record | This value defines the row number in the ScnScanDevice table that you will use to receive the scan. |

Example values:

| MsgMap Table Example |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_ Name | Recv_Msg_ Type | Send_Msg_ Type | Ack_Msg_ Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| Pick1Scan | Pick1Scan | ALL |  |  | ScnScanDevice | 2 | 2 | 3 | 0 | 1 |

The /etc/hosts file links the Machine Machine_Name to an IP address.

Add a line to /etc/hosts with the IP address of the scanner, followed by the Machine_Name name.  For example:

192.168.1.50		Pick1Scan

Note:  Use the phlip command in QNX to verify “Look in local ‘hosts’ file first” is checked.  You can also inspect /etc/net.cfg for a line in the [global] section that says, “lookup file bind”, which is how the setting is recorded.

# The CommCore Interface

The interface to CommCore requires configuration in the Machine, MsgMap and XfRouteBoss tables, and an entry in the /etc/hosts file.

The Machine table (Configuration Menu | Communication Utilities | Machine) defines how the connection is to be made.

| Machine Table |  |
| --- | --- |
| Field | Description |
| Machine_Name | Sets the name for this connection within FPC. It will appear in log files and requires a matching entry in /etc/hosts to link the name to the IP address of the CommCore server. Multiple names may be associated with the same server in this way. See below. |
| Connection_Type | Defines the connection method. Always CONNECT_TCPCLIENT for CommCore connections. |
| Communication_Protocol | The dialect to “speak” over the connection. It will be SrtCommSeq (“the sequence protocol”), or SrtComm (without the sequence numbers). The CommCore configuration will determine which one to use. |
| Machine_Port | Defines the port number for this connection. The port number is determined by the CommCore configuration. |
| Transmit_Retry | Usually set to 10 seconds. May be set automatically. Do not set to 0, as this will cause the messages to be re-sent before a reply can be generated. |
| Process | Choose the SERV process. |
| NeedAck | Set to 1 for most message types. May be set automatically. This enforces sequential processing so only one message is “in play” at any moment. |
| LogLevel | Set this to “Level 1” to see the message content in the logs. Higher values generate more detail. |

Example values:

| Machine Table Example |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_ Type | Communication_ Protocol | Machine_ Port | Transmit_ Retry | Process | Need ACK | Log Level |
| XfrCRR | CONNECT_ TCPCLIENT | SrtCommSeq | 9310 | 10 | SERV | 1 | Level 1 |
| CSUDCM | CONNECT_ TCPCLIENT | SrtCommSeq | 9311 | 10 | SERV | 1 | Level 1 |

The CSU and DCM messages often share one port, as shown in the example.  This is determined by the CommCore configuration.

The MsgMap table (Configuration Menu | Communication Utilities | Message Map) defines the message tables and connects them to the Machine entry.

| MsgMap Table |  |
| --- | --- |
| Field | Description |
| Message_Name | Set the name for this map entry within FPC. May appear in log files. |
| Machine_Name | Choose the related Machine entry here. |
| Recv_Msg_Type | Define the message type to process for received messages. You may use the message name, like CRR, or the AC name, like AC1, or ALL to match any message received by the Machine connection. |
| Send_Msg_Type | Define the message type to process for messages being sent. |
| Ack_Msg_Type | Define the message type to process for acknowledgement messages. |
| Menu_Name | Choose the table where the message data is to be stored. |
| Low_Cursor | Enter the column number of the first column in the Menu_Name table to process, usually 1. You can find the appropriate value using F2-MenuOfMenus to find the table and then open the table definition using Alt-K. The row number in table MenuColumn is the column number in the table. |
| High_Cursor | Enter the column number of the last column in the Menu_Name table to process. Use the column number of the last “Dat” column unless the CommCore configuration defines another value. |
| Stat_Cursor | Enter the column number of the TxStat column in the Menu_Name table. This value will be auto-populated if you leave it at 0. |
| Key_Cursor | Enter the column number of the DbKey column in the Menu_Name table. This value will be auto-populated if you leave it at 0. |
| Record | Set to 0. This value defines the row number in the table. It is used for the scanner configurations. For CommCore connections, all rows are used, so leave the value at 0. |

Example values:

| MsgMap Table Example |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_ Name | Recv_Msg_ Type | Send_Msg_ Type | Ack_Msg_ Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| XfrCRR | XfrCRR | ALL | ALL | ALL | MsgXR1 | 1 | 66 | 67 | 68 | 0 |
| CSUDCM | CSUDCM | ALL | ALL | ALL | MsgXDC1 | 1 | 66 | 67 | 68 | 0 |

The XfRouteBoss table (Configuration Menu | Transfer Utilities | Transfer Scan Processing) links the message tables and the scan zone to the transfer.

| XfRouteBoss Table |  |
| --- | --- |
| Field | Description |
| Name | Enter a descriptive name. |
| LogLevel | Choose a log level to determine the level of detail written to the log files by the transfer logic. |
| ScanZone | Choose the scan zone for this transfer. |
| CrrMsgTable | Define the table that will store the CRR message data. |
| DcmMsgTable | Define the table that will store the DCM message data. |
| CsuMsgTable | Define the table that will store the CSU message data. |
| SystemRun | Choose the IO part that indicates the transfer is ready to run. |
| RunNewXfr | Set to Y to use the “new” transfer configuration tables. |

Example values:

| XfRouteBoss Table Example |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | LogLevel | ScanZone | CrrMsg Table | DcmMsg Table | CsuMsg Table | SystemRun | RunNewXfr |
| Pick1 | Level 1 | Pick1 Scan | MsgXR1 | MsgXDC1 | MsgXDC1 | PickStart | Y |

If the configuration is correct, the CRR message will appear in the MsgXR1 table.

Multiple transfers may be assigned to the same message tables; however, each transfer requires at least 100 msec to process a message.  If multiple transfers need routing at the same time, the responses will be delayed.

The /etc/hosts file links the Machine Machine_Name to an IP address.

Add a line to /etc/hosts with the IP address of the CommCore server, followed by each of the Machine_Name names, separating each with a space or tab.  For example:

192.168.1.100		XfrCRR CSUDCM

Note:  Use the phlip command in QNX to verify “Look in local ‘hosts’ file first” is checked.  You can also inspect /etc/net.cfg for a line in the [global] section that says, “lookup file bind”, which is how the setting is recorded.

Note:  You can also use the “Comm Config Wizard” (Configuration Menu | Wizard Utilities| Comm Config Wizard) to complete the FPC portion of the configuration covered in this section, but it is probably best to do it “manually”, as described above.

# Transfer Testing and Debugging

Open the ConfigXfgScrn screen (Configuration Menu | Transfer Utilities | Configure Transfers – all the way at the bottom).

- The screen displays the current state of a transfer and indicates if the transfer is waiting for an input or timer.

- Select the scan zone.  All the other popup selections will be filled in automatically.

- Right-click the down-arrow icon to open tables to the first record associated with the selected transfer.  You can use this as a quick way to open the tables referenced in the rest of this section.

When a scan event occurs, scan data is pushed into the table and row defined in MsgMap.  If data is not received check the following:

- Check the Batches table to be sure the entry for “Scan Process”, a.k.a. “ScnApp CTRL” is active.  Verify that Machine is set to this machine and Process is set to “CTRL”.

- Check the Machine table to be sure the entry for the scanner shows a connection in Machine_Pid (not -1).  The Status column should indicate a connection to the scanner.  Verify the Connection_Type is appropriate.  Communication_Protocol is usually “AlphaNum” for scanners.  Check the Machine_Port value is appropriate.  Make sure “CTRL” is selected in Process.  LogLevel = “Level 2” is a good choice so you can see scanner activity in the CTRL log.

- For serial connections, use this command to check the port settings (Substitute the appropriate Machine_Port device): stty < /dev/ser-rckt1

- Check the CTRL log for messages:

- Verify that the /etc/hosts file defines the cross-reference between the Machine_Name in table Machine and the IP address of the scanner.  Use the “ping” command with the IP address to verify the scanner is on the network.  Use the “ping” command with the name from /etc/hosts to verify it is defined correctly.  Make sure the /etc/hosts file has a carriage return at the end of the last line!  Make sure the default lines for localhost are present at the beginning of /etc/hosts:

127.1	localhost.localdomain localhost

::1		localhost.localdomain localhost

- You can also use the following command to verify a CommCore connection:

telnet <Machine_Name> <Machine_Port>

- Verify the MsgMap table entry connects the scanner connection in Machine to the correct row and column in table ScnScanDevice.  The scans should appear in the RcvData field.

- If you see data in ScnScanDevice_RcvData, check for messages in the CTRL log that may help diagnose problems.  Normally, the log message will indicate that the scan data was processed.  If no log messages are found, choose a higher LogLevel value in table ScnScanZone.

- If the scan data is processed correctly, the ScnScanDevice_ParsedData field will be populated.  The parsing process removes unwanted characters to prepare the scan for use.

- If there are multiple scanners, repeat this process for each one.

- The scan processing software stores scan information in the tracking tables: XfrTrack and XfrDevice.  One XfrTrack record will be used for each container at a given transfer.  At least one XfrDevice record will be used for each container at a given transfer – if more than one scan device is used, there will be an XfrDevice record for each scan device.  If multiple scan devices are assigned to the same scan zone, all scans must be received before the transfer module will process the data.

- ScnScanZone_pTrnTrkRecord points to the current tracking record.  Once all scans are received, ScnScanZone_pTrnLastTrk will point to the tracking record.

When a container arrives and triggers the input photoeye defined in XfrBoss_ProductINPUT, and XfRouteBoss_pTrkMenuQue does not have a move queued, the tracking record number created by the newly-arrived scan is copied to XfRouteBoss_pTrkMenuQue and XfRouteBoss_pTrkIndxQue, initiating transfer processing.

- Check ProcFlag in the tracking table (XfrTrack).  It is set when the scan is processed and reset by the transfer processing logic.  If ProcFlag does not get set, the scan zone might not be linked to the transfer properly.  If ProcFlag stays set and the transfer does not run, check the transfer entries in table Batches.  If the configuration is correct, a CRR message will be added to the message table defined in XfRouteBoss_CrrMsgTable (MsgXR1).

Once a CRR message has been queued, if the connection to CommCore has been established, the message is sent to request routing.

- Check the TxStat field in the message table.  If the Machine and MsgMap tables are configured properly, TxStat will start at “New”, and advance to “Circ”, then to “Sending”, “Sent”, and to “Ack” once a response has been received.

- If TxStat stays at “New”, check Machine and MsgMap.  Repeat the ping tests described earlier.  Verify that MsgMap_Key_Cursor is set correctly by using F2-MenuofMenus to find the message table (MsgXR1), then open the table definition with Alt-K.  Use the row number for the DbKey column in MsgMap_Key_Cursor.

- If TxStat stays at “Sending”, CommCore is not responding.  Check the SERV log for error messages.  Check Machine_Port.  Check MsgMap_Ack_Msg_Type.  Verify Machine_NeedAck = 1.

- If TxStat changes to “Ack” – success!  CommCore has responded to the routing request.  The received routing information from the CRR response is stored in the tracking table (XfrTrack).  The Z1 – Z10 fields will contain the received destinations.

- If no destinations were received, check the Status and ErrorMsg fields.

Once routing information has been received, it must be linked to a transfer route.

- Verify the routing strings received in the Z1 – Z10 fields have matching entries in XfRouteTable_HostRouteStr.  Often, the actual strings implemented in CommCore are not the same as initially documented.  Just adjust the HostRouteStr values to match what is actually sent.

Make sure the transfer routes are enabled and available:

- Verify XfRouteTable_Enabled is selected for all desired routes.

- Verify XfrTree_AvailInput matches the state defined in XfrTree_AvailSENSE.

- Verify XfrTree_DoConfirm = Y so divert confirm messages can be generated.

Check the transfer logic.

- In table XfRouteBoss, pRouteTreeQue, pTrkMenuQue and pTrkIndxQue are set by the transfer software when a container is being processed.  Check that one or more of these are set while a container is in the transfer.  Verify that they clear when the container is removed from the transfer.

Verify transfer physical behavior:

- XfrBoss_ReleaseOUTPUT should stop the container to prevent an out-of-sync condition.  Disable the route by de-selecting XfRouteTable_Enabled.  Place a container on the conveyor prior to the transfer and let it move in.  Verify that it stops in the transfer.  If it does not stop and wait, check XfrBoss_ReleaseOUTPUT, which should be OFF (disabled) to hold the container.  A common problem in actual use that may need to be addressed is containers getting pushed through by pressure from upstream containers.  This is undesirable, as it will lead to an out-of-sync condition.

- Remove the container from the transfer bed.  Verify the queued move is automatically cleared by checking that the XfRouteBoss fields pRouteTreeQue, pTrkMenuQue and pTrkIndxQue reset to zero (after the XfRouteBoss_MaxWaitTime timer expires – it is usually set to a very short interval like 0.05 seconds).  Restore XfRouteTable_Enabled to re-enable the routes.

NOTE:  Older versions of FPC write “circular buffer” files in /SortPlus/RUN and will read them in at startup.  If tables are copied from one AC to another, or if Machine table entries have been added or changed, data in the circular buffer files can cause strange behavior.  Exit FPC, remove the circular buffer files with “rm /SortPlus/RUN/cb_*”, then restart FPC.  FPC V10.83.0 and newer do not save or load circular buffer files, but circular buffers are still used internally.  You can run “Flush Machines” from Administration Menu | Set Security Levels, using Alt-K, to clear the circular buffers in a running copy of FPC of any version.

# Scanner Physical Configuration

The scanner and scanner trigger photoeye must be located so the scan has been delivered when the presence photoeye is activated, even if the longest container is not aligned with the conveyor.

The scanner must be configured to transmit the scan data when the container tails out of the scanner trigger photoeye.

Scanner PE

Presence PE

Scanner PE

Presence PE

# Accumulation Conveyor Configuration

Accumulation conveyor must be configured in singulation mode (each container must move out of the downstream zone before the next container upstream is released), not in slug mode (containers in all zones can move simultaneously).

# Process Flow

Scan Event

Machine table
Connection_Type & Communication_Protocol
(TCP/IP or Serial)

MsgMap table links to ScnScanDevice – sets “Recvd” Status

ScnScanDevice processing

stores data in tracking tables, sets ProcFlag

Transfer Scan Processing links ScnScanZone to a transfer and monitors the tracking table ProcFlag

Transfer processing builds a CRR message and stores it in the message table selected in XfRouteBoss.

Message processing monitors the message table (via MsgMap) for records with TxStat = “New”.  Messages are sent to the “host name” found in the first column of the Machine table.  An entry in /etc/hosts maps the host name to an IP address.

Message Receive Event

Incoming messages are linked from Machine through MsgMap to the message table and tracking record.

Message processing extracts the ScanZoneID, tracking index and destinations from the message. The destinations are copied to the Z1 through Z10 fields of the tracking record.

Transfer processing searches the destinations in Z1 – Z10 of the tracking record for matching XfRouteBoss entries.  If one of the routes can be processed, the transfer will run.