# FortnaPlus Control Transfer Test Examples

Document Revision: 1.1

Date: 20-Oct-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 23-Feb-2009 | B C Conklin | New document |
| 1.1 | 20-Oct-2022 | T R Kremser | Complete revamp. |

Table of Contents

Glossary of Terms	5

1.	New Transfer Test Plan	6

1.1.	Inputs	6

1.2.	Outputs	6

1.3.	Timers	6

2.	Testing a Transfer – Initial Configuration	7

3.	Test – XfrBoss Options – Logging Transfer Events	9

4.	Test – XfrBoss Options – Product Input and Release Output	10

5.	Test – XfrBoss Options – MaxWaitTM	11

6.	Test – XfrBoss Options – Reset	12

7.	Example Table Contents	13

7.1.	Batches Configuration	13

7.2.	XfrBoss Configuration	14

7.3.	XfrTree Configuration	15

7.4.	XfrConfig Configuration	15

7.5.	XfrCheckSet Configuration	15

7.6.	XfrGoSet Configuration	15

7.7.	XfrCfgInput Configuration	16

7.8.	XfrCfgInTimer Configuration	16

7.9.	XfrCfgDelayTmr Configuration	16

7.10.	XfrCfgOutput Configuration	16

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# New Transfer Test Plan

The examples in this document will use the following names for inputs, outputs, and timers:

Create the conveyor parts using Configuration Menu | Wizard Utilities | Create Conveyor Part.  Click the Add16MemoryParts button.  Open the Conveyor table by right-clicking the down-arrow next to the ConveyorPart field.  The new parts appear at the end of the list, before the END marker, and are named like this:  MEM0200-00 through MEM0200-15.  Rename parts to match the parts in the following Inputs and Outputs sections.

## Inputs

- ProdPres01 – XfrBoss_ProductINPUT

- EnableMaxWait01 – XfrBoss_EnableMaxWait

- TreeAvail01 – XfrTree_AvailINPUT

- ConfigEnable01 – XfrConfig_EnabledINPUT

- XfrInput01 – XfrCfgInput_RequiredInput

- XfrInput02 – XfrCfgInput_RequiredInput

## Outputs

- ProdRelease01 – XfrBoss_ReleaseOUTPUT

- XfrOutput01 – XfrCfgOutput_Output

## Timers

Create timers from the following list (the ones that are not labeled as auto-generated).  Use Configuration Menu | Wizard Utilities | Create Timer.  Use the TimerName field to name the timer.  Use the AddNewRetentive button to create the timer.

The AC name used in this document is CP2.  If you use another name, substitute that name for CP2 in the auto-generated timers.

- tmRight_F – timemenu_Timer_Name

- tmCnf01State1Delay_CP2 – timemenu_Timer_Name (auto-generated)

- tm_CP2_X01MaxWait – timemenu_Timer_Name (auto-generated)

- tm_CP2_Cnf01EnableDelay – timemenu_Timer_Name (auto-generated)

# Testing a Transfer – Initial Configuration

In this section, we will create a test configuration.  Many of the items are automatically generated.  You can refer to section 7, Example Table Contents, to see the end result in each table.

- The transfer modules are not active by default.  See section Error! Reference source not found. to activate them in the Batches table.

- Use “XfrClearMenus” (Administration Menu | Set Security Levels) to clear all the New Transfers tables.  Run it with Alt-K.

- Open the transfer configuration screen, ConfigXfgScrn (Configuration Menu | Transfer Utilities | Configure Transfers – at the bottom of the list).

- Click on AddXfr to add an XfrBoss entry.

- Verify the new XfrBoss record, “X01”, is selected in ConfigXfgScrn_SelectedXfr.

- Open the XfrBoss table by right-clicking the down arrow next to SelectedXfr.

- Verify that XfrBoss_Name = “X01” is found in row 1, and MaxWaitTM = “tm_CP2_X01MaxWait” (auto-generated).

- Add ProductINPUT = “ProdPres01”.  Select “Is On” in ProductSENSE.

- Add ReleaseOUTPUT = “ProdRelease01”.  Select “Fire” in ReleaseAction.

- Add EnableMaxWait = “EnableMaxWait01”.  Select “Is On” in EnableMxSense.

- Select Machine = “CP2” (or whatever your machine is called).

- Back in ConfigXfgScrn, also select “ProdPres01” in PrsPE to make it easier to see and manipulate its state.

- In ConfigXfgScrn, click on AddTree to add an XfrTree entry.

- Verify the new XfrTree record “Tr01-X01” is selected in ConfigXfgScrn_SelectedTree.

- Open the XfrTree table by right-clicking the down arrow next to SelectedTree.

- Verify that XfrTree_Name = “Tr01-X01” is found in row 1, Boss = “X01”, and Dest = “D01”.

- Set DoConfirm = “Y”, AvailINPUT = “TreeAvail01”, and AvailSENSE = “Is On”.

- Back in ConfigXfgScrn, click on AddConfig to add an XfrConfig entry.

- Verify the new XfrConfig record “C01-X01” is selected in ConfigXfgScrn_SelectedConfig.

- Open the XfrConfig table by right-clicking the down arrow next to SelectedConfig.

- Verify that XfrConfig_Name = “C01-X01” is found in row 1.

- Add EnabledINPUT = “ConfigEnable01”, and EnabledSENSE = “Is On”.

- Back in ConfigXfgScrn, click on AddEnableDelayTM to add a delay timer.

- Open XfrConfig again and verify that EnabledDELAY = “tm_CP2_Cnf01EnableDelay” (automatically added).

- Back in ConfigXfgScrn, click on AddGoSet to add an entry to the XfrGoSet table.

- Verify the new XfrGoSet record “Go01-C01-Tr01-X01” is selected in ConfigXfgScrn_GoSet.

- Open the XfrGoSet table by right-clicking the down arrow next to GoSet.

- Verify that XfrGoSet_Name = “Go01-C01-Tr01-X01” is found in row 1, with Tree = “Tr01-X01”, XfrConfig = “C01-X01”, and InitialState = “Idle”.

- Back in ConfigXfgScrn, click on AddCheckSet to add an entry in the XfrCheckSet table.

- Verify the new XfrCheckSet record “Ck01-C01-Tr01-X01” is selected in ConfigXfgScrn_CheckSet.

- Open the XfrCheckSet table by right-clicking the down arrow next to CheckSet.

- Verify that XfrCheckSet_Name = “Ck01-C01-Tr01-X01” is found in row 1, with Tree = “Tr01-X01”, and XfrConfig = “C01-X01”.

- Back in ConfigXfgScrn, select “State1” in SelectedState.

- Click on AddInput to add an entry to the XfrCfgInput table.

- Verify the new XfrCfgInput record “Inp01-C01-State1” is selected in ConfigXfgScrn_Inputs.

- Open the XfrCfgInput table by right-clicking the down arrow next to Inputs.

- Verify that XfrCfgInput_Name = “Inp01-C01-State1” is found in row 1, with XfrConfig = “C01-X01” and State = “State1”.

- Add RequiredInput = “XfrInput01”.  Select InputSense = “Is On”.

- Back in ConfigXfgScrn, select “State2” in SelectedState.

- Click on AddInput to add another entry to the XfrCfgInput table.

- Verify the new XfrCfgInput record “Inp02-C01-State2” is selected in ConfigXfgScrn_Inputs.

- Open the XfrCfgInputs table by right-clicking the down arrow next to Inputs.

- Verify that XfrCfgInputs_Name = “Inp02-C01-State2” is found in row 2, with XfrConfig = “C01-X01” and State = “State2”.

- Add RequiredInput = “XfrInput02”.  Select InputSense = “Is On”.

- Back in ConfigXfgScrn, click on AddOutput to add an entry in the XfrCfgOutput table.

- Verify the new XfrCfgOutput record “Out01-C01” is selected in ConfigXfgScrn_Outputs.

- Open the XfrCfgOutputs table by right-clicking the down arrow next to Outputs.

- Verify that XfrCfgOutputs_Name = “Out01-C01” is found in row 1, with XfrConfig = “C01-X01” and DoneAction = “DeActivate”.

- Add Output = “ProdRelease01”.  Select OutputAction = “Fire” and State1Action = “Activate”.

- Back in ConfigXfgScrn, select “State1” in SelectedState, then click on AddInTimer to add an entry in the XfrCfgInTimer table.

- Verify the new XfrCfgInTimer record “InT01-C01-State1” is selected in ConfigXfgScrn_InTimers.

- Open the XfrCfgInTimer table by right-clicking the down arrow next to InTimers.

- Verify that XfrCfgInTimers_Name = “InT01-C01-State1” is found in row 1, with XfrConfig = “C01-X01” and State = “State1”.

- Add RequiredTimer = “tmRight_F”.  Select TimerSense = “Is Done”.

- Back in ConfigXfgScrn, click on AddDelayTimer to add an entry in the XfrCfgDelayTmr table.

- Verify the new XfrCfgDelayTmr record “Dly01-C01-State1” is selected in ConfigXfgScrn_DelayTimers.

- Open the XfrCfgDelayTmr table by right-clicking the down arrow next to DelayTimers.

- Verify that XfrCfgDelayTmr_Name = “Dly01-C01-State1” is found in row 1, with XfrConfig = “C01-X01”, State = “State1”, Timer = “tmCnf01State1Delay_CP2”, and TimerAction = “Run Timer”.  Open the timer and set the Preset_Value = 300 seconds (5 minutes).

Now that we have our basic transfer configuration, we can proceed to test it.

# Test – XfrBoss Options – Logging Transfer Events

In this section, we will verify that transfer events are written to the log files.  Note that leaving the LogLevel set at Level 5 is a good way to fill the disk!  It is very helpful for diagnosing configuration issues, however.

Use ConfigXfgScrn (Configuration Menu | Transfer Utilities | Configure Transfers – at the bottom of the list) as a way to see important states, and to quickly open the underlying tables to inspect configuration details.  Below, in parenthesis after each table name, is the name of the field in ConfigXfgScrn that links to that table.

- Use the View IO screen to make sure all the IO in word 200 is OFF and not forced.  ProdRelease01 may or may not be set ON by the software at this point.

- Use table timemenu (Configuration Menu | Standard Controls | Timers) to make sure all the added timers are in the Done = N state.

- In XfrBoss (SelectedXfr), select X01.  Open the table and set LogLevel to “Level 5”.  MaxWaitTM will be prevented from resetting the transfer because EnableMaxWait01 is OFF.

- Force ProdPres01 ON.  You can do this from ConfigXfgScrn or from the View IO screen.  If ProdRelease01 was ON it will now turn OFF.

- Click the StartXfr button.

- Verify that the transfer waits for the XfrTree (SelectedTree) TreeAvail01 input.
XfrState will indicate “Search” and
XfrStatus will indicate the transfer is waiting for the input with:
“Waiting for Avail Input, Tree: TreeAvail01”.  The CTRL log will show ProdPres01 has transitioned ON and ProtRelease01 has transitioned OFF, and X01 has entered SEARCH mode.

- Force TreeAvail01 ON in XfrTree (SelectedTree).

- Verify the CTRL log shows the “TreeAvail01” went ON.

- Verify CfgState = Idle and CfgStatus = “Waiting for Enable: ConfigEnable01, Is On”.

- Force EnabledINPUT = ConfigEnable01 ON in XfrConfig (SelectedConfig).

- Verify CfgState = Idle and CfgStatus = “Waiting for XfrInput01, Is On”.

- Force RequiredInput = XfrInput01 ON in XfrCfgInput (Inputs).

- Verify CfgState = Idle and CfgStatus = “Waiting for In Timer: tmRight_F”.

- Set the timer to the Done state by opening XfrCfgInTimer (InTimers), and right-clicking the down arrow by tmRight_F in column RequiredTimer, then set Done = Y.

- Verify CfgState = State1, CfgStatus = “Waiting for XfrInput02, Is On”, XfrState = Run and XfrStatus = “Waiting for Config to Finish”.  The CTRL log will record that the tree and config have started.

- Verify XfrCfgOutput (Outputs) Output = ProdRelease01 had come ON.

- Force RequiredInput = XfrInput02 ON in XfrCfgInput (Inputs).  The CTRL log will reflect this.

- Verify CfgState = State1, CfgStatus = ”Waiting for Delay Timer: tmCnf01State1Delay_CP2”.

- Open DelayTimers, then open the timer.  Note that the Accumulated time is increasing as the timer runs, and XfrCfgOutput (Outputs) Output = ProdRelease01 is still ON.  Set Done = Y, make sure the timer shows as GREEN.

- The CTRL log will record the timer transition to DONE and show the completion of all transfer states.  ProdRelease01 transitions OFF.

- Verify CfgState, CfgStatus and XfrState = Idle, and XfrStatus = “Idle – Waiting for Routing”.

- Force ProdPres01 OFF to indicate the container has moved.  XfrCfgOutput (Outputs) Output = ProdRelease01 turns ON.  The CTRL log will record these transitions.

# Test – XfrBoss Options – Product Input and Release Output

Here we will test the logic associated with the induct and release photoeyes.

- Use the View IO screen to make sure all the IO in word 200 is OFF and not forced.  ProdRelease01 will be set ON by the software at this point.

- Use table timemenu (Configuration Menu | Standard Controls | Timers) to make sure all the added timers are in the Done = N state.  Set tmCnf01State1Delay_CP2 Preset_Value = 60 seconds

- Verify XfrBoss (SelectedXfr) = X01.

- Verify XfrConfig (SelectedConfig) = “C01-X01”.

- Verify “State1” is selected in SelectedState.

- Click on AddOutput to add an entry in table XfrCfgOutput (Outputs).

- Verify XfrCfgOutput (Outputs) Name = “Out02-C01” in row 2, with XfrConfig = “C01-X01” and DoneAction = “DeActivate”.

- Add XfrCfgOutput (Outputs) Output = “XfrOutput01” with OutputAction = “Fire” and State1Action = “Activate”.

- Verify PrsPE = ProdPres01 is OFF.

- Force XfrCfgInput (Inputs) RequiredInput = XfrInput01 ON.

- Verify XfrCfgInput (Inputs) RequiredInput = XfrInput02 is OFF.

- Verify XfrCfgOutput (Outputs) Output = ProdRelease01 is ON.

- With the transfer Idle, Force PrsPE = ProdPres01 ON.  Verify that ProdRelease01 goes OFF.  Force ProdPres01 OFF.  Verify that ProdRelease01 turns ON again.

- Force ProdPres01 ON again.

- Force ON XfrTree (SelectedTree) AvailINPUT = TreeAvail01 and XfrConfig (SelectedConfig) EnabledINPUT = ConfigEnable01.  XfrCfgInput (Inputs) RequiredInput = XfrInput01 is already ON.

- Set tmRight_F to Done = Y.

- Click on the “StartXfr” button.  Verify CfgState = “State1” and CfgStatus = “Waiting for XfrInput02, Is On”.  Verify ProdRelease01 and XfrOutput01 are both ON.

- With the transfer in State1, force ProdPres01 OFF, then ON.  Verify ProdRelease01 goes OFF when ProdPres01 goes ON.  This tests a second container entering the transfer.

- With ProdPres01 ON, force XfrInput02 ON.  Set XfrConfig (SelectedConfig) EnabledDELAY = tmCP2_Cnf01EnableDelay to Done = Y.  (Make sure it turns Green!).

- Wait for XfrCfgDelayTmr (DelayTimers) Timer = tmCnf01State1Delay_CP2 completes (or set Done = Y).

- Verify XfrCfgOutput (Outputs) Output = XfrOutput01 goes OFF, and the transfer returns to Idle.

- Force ProdPres01 OFF and observe that ProdPresence01 turns ON again.

# Test – XfrBoss Options – MaxWaitTM

Test the operation of the XfrBoss_MaxWaitTM timer.

- Use the View IO screen to make sure all the IO in word 200 is OFF and not forced.  ProdRelease01 will be set ON by the software at this point.

- Use table timemenu (Configuration Menu | Standard Controls | Timers) to make sure all the added timers are in the Done = N state.

- Verify XfrBoss (SelectedXfr) = X01.

- Verify XfrConfig (SelectedConfig) = “C01-X01”.

- Verify “State1” is selected in SelectedState.

- Force TreeAvail01 and ConfigEnable01 ON.  Make sure EnableMaxWait01 is OFF.

- Verify XfrCfgTimer (InTimers) RequiredTimer = tmRight_F is not Done (simulates a full condition).

- Force ProdPres01 ON.

- Click on StartXfr.

- Verify XfrState = “Search” and CfgState = “Idle”.

- Verify timer XfrBoss (SelectedXfr) MaxWaitTM = tm_CP2_X01MaxWait is not timing.  The timer will hold while waiting for the transfer to become available (Input timers and Inputs must be in the selected state to advance the transfer from the Search state).

- Force EnableMaxWait01 ON.

- Verify tm_CP2_X01MaxWait starts timing.

- Force XfrCfgInput (Inputs) RequiredInput = XfrInput01 ON.

- Verify CfgStatus = “Waiting for In Timer: tmRight_F”.

- Set Done=Y for tmRight_F.

- Verify CfgState advances to “State1” and CfgStatus = “Waiting for XfrInput02, Is On”.

- Verify the transfer resets when tm_CP2_X01MaxWait times out.

- Click on StartXfr.

- Force EnableMaxWait01 OFF.

- Verify tm_CP2_X01MaxWait stops timing.

- Force EnableMaxWait01 ON.

- Verify tm_CP2_X01MaxWait starts timing.

- Verify the transfer resets when tm_CP2_X01MaxWait times out.

# Test – XfrBoss Options – Reset

Test the transfer reset button, ResetXfr.

- Use the View IO screen to make sure all the IO in word 200 is OFF and not forced.  ProdRelease01 will be set ON by the software at this point.

- Use table timemenu (Configuration Menu | Standard Controls | Timers) to make sure all the added timers are in the Done = N state.

- Verify XfrBoss (SelectedXfr) = X01.

- Verify XfrConfig (SelectedConfig) = “C01-X01”.

- Verify “State1” is selected in SelectedState.

- Force TreeAvail01 and ConfigEnable01 ON.  Make sure EnableMaxWait01 is OFF.

- Verify XfrCfgTimer (InTimers) RequiredTimer = tmRight_F is not Done (simulates a full condition).

- Force ProdPres01 ON.

- Click on StartXfr.

- Verify XfrState = “Search” and CfgState = “Idle”.

- Click ResetXfr.

- Verify the transfer resets.

- Force XfrCfgInput (Inputs) RequiredInput = XfrInput01 ON.

- Verify XfrCfgInput (Inputs) RequiredInput = XfrInput02 is OFF.

- Set Done=Y for tmRight_F.

- Click StartXfr.

- Verify the XfrState = “Run” and CfgState = “State1”.

- Click ResetXfr.

- Verify the transfer resets.

# Example Table Contents

The following sections show the contents of the tables after completing the configuration in the previous sections.

## Batches Configuration

The transfer processing modules are not active by default.  Activate them in table Batches.  The rows shown in the following table should already be present – just set AllMachines = Y or select your machine name in the Machine column.  If there are no rows defined for the InternalProc and InternalProcCtrl items shown, add them.

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| File_Name | HowOften | Increment | Type | Process | InternalProc | InternalProcCtrl | Machine | AllMachines | DoFile |
| Xfr Serv | 1 | INVALID | Count | SERV | XfrApp SERV (CPN) |  | CP2 | N | N |
| Xfr Ctrl | 1 | INVALID | Count | CTRL |  | XfrApp CTRL (CPN) | CP2 | N | N |
| Xfr Run | 1 | INVALID | Count | CTRL |  | XfrProcess (CTRL) | CP2 | N | N |

## XfrBoss Configuration

| XfrBoss Table, part 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Reset | LogLevel | MaxWaitTM | ProductINPUT | ProductSENSE | ReleaseOUTPUT | ReleaseAction |
| X01 | N | Level 5 | tm_CP2_X01MaxWait | ProdPres01 | Is On | ProdRelease01 | Fire |

| XfrBoss Table, part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | EnableMaxWait | EnableMxSense | NoScanner | Machine |
| X01 | EnableMaxWait01 | Is On | N | CP2 |

## XfrTree Configuration

| XfrTree Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Boss | Dest | DoConfirm | AvailINPUT | AvailSENSE |
| Tr01-X01 | X01 | D01 | Y | TreeAvail01 | Is On |

## XfrConfig Configuration

| XfrConfig Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | pRun | EnabledINPUT | EnabledSENSE | EnabledDelay | pCurrentBoss |
| C01-X01 | N | ConfigEnable01 | Is On | tm_CP2_Cnf01EnableDelay | N/A |

## XfrCheckSet Configuration

| XfrCheckSet Table |  |  |
| --- | --- | --- |
| Name | Tree | XfrConfig |
| Ck01-C01-Tr01-X01 | Tr01-X01 | C01-X01 |

## XfrGoSet Configuration

| XfrGoSet Table |  |  |  |
| --- | --- | --- | --- |
| Name | Tree | XfrConfig | InitialState |
| Go01-C01-Tr01-X01 | Tr01-X01 | C01-X01 | Idle |

## XfrCfgInput Configuration

| XfrCfgInput Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredInput | InputSense |
| Inp01-C01-State1 | C01-X01 | State1 | XfrInput01 | Is On |
| Inp02-C01-State2 | C01-X01 | State2 | XfrInput02 | Is On |

## XfrCfgInTimer Configuration

| XfrCfgInTimer Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | RequiredTimer | TimerSense |
| InT01-C01-State | C01-X01 | State1 | tmRight_F | Is Done |

## XfrCfgDelayTmr Configuration

| XfrCfgDelayTmr Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | Timer | TimerAction |
| Dly01-C01-State1 | C01-X01 | State1 | tmCnf01State1Delay_CP2 | Run Timer |

## XfrCfgOutput Configuration

| XfrCfgOutput Table, part 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | Output | OutputAction | DisableAction | IdleAction |
| Out01-C01 | C01-X01 | ProdRelease01 | Fire | N/A | N/A |
| Out02-C01 | C01-X01 | XfrOutput01 | Fire | N/A | N/A |

| XfrCfgOutput Table, part 2 |  |  |  |
| --- | --- | --- | --- |
| Name | State1Action | State2Action – State20Action | DoneAction |
| Out01-C01 | Activate | N/A | DeActivate |
| Out02-C01 | N/A | N/A | DeActivate |