# Sorter-Scanner Design Example – Scanner to Divert Point

Bar Code

Scanner

Induct Photoeye

Scanner Photoeye

Tail Out Point

Nose of the Longest Box from Scanner Tail Out

Location of first divert

Example calculation for a system where the routing is handled by the Fortna WCS.

- Induct Photoeye – The Induct Phototeye is typically located (.75 * Max Carton Length) from the beginning of the Belt.

- Scanner Tail Out Point - is located at (A combination of the Length of the Scan Tunnel and the Length of the Longest Carton).

- As a rule we need 1 second of travel time from the time we receive the scan until our first divert point.

  - Note: for 2 sided shoe sorters the 1st divert is the shoe pull over.

- Example:

  - Longest Carton 36 inches (3 feet).

  - For this example the carton tails out of the scanner at 1 box length.

  - At 240 fpm we need a minimum of (240/60 = 4 feet) from the nose of the carton or tote until the 1st divert.

  - Total length = ( 3 ft + 3 ft + 4 ft) = 10 ft

# Sorter-Scanner Design Example – Scanner/Scale/CSM/CRR

Bar Code

Scanner

Induct Photoeye

Scanner/Scale Photoeye

Tail Out Point

Nose of the Longest Box from Scanner Tail Out

Location of Send CSM

Location of first divert

Scale

CSM Response Time

CRR Response Time

Location of Send CRR

Example system where the routing requires transactions from the WMS, after scale data is received.

- Induct Photoeye – The Induct Phototeye is typically located (.75 * Max Carton Length) from the beginning of the Belt.

- Scanner/Scale Tail Out Point - is located at (A combination of the Length of the Scan/Scale and the Length of the Longest Carton).

- As a rule we need at least 1 second of travel time from the time we receive the scan/scale data until the CRR Send Point. However this depends on the response time of the Level 2 and WMS Systems. In some cases this can be 10 to 15 seconds. Using 2 seconds for the following example (1 sec for Fortna WCS and 1 second for the WMS).

  - Note: for 2 sided shoe sorters the 1st divert is the shoe pull over.

- In systems where a CSM is needed, the Send CRR Point is located approximately 1 second prior to the first divert point.

- Example: 300 ft/min (5 feet/sec) conveyor, 36 inch (3 ft) maximum carton

  - Induct PE would be located at 36 * (3/4) = 27 inches (2.25 feet)

  - Scan Zone or Tunnel  and the scale = 5 feet

  - Longest carton = 3.0 feet

  - Required response for CSM 2 seconds = 2 * 5 = 10 feet

  - CRR Processing time = 1 sec * 5 = 5 feet

  - Total length of conveyor = 2.25 + 5 + 3 + 10 + 5 = 25.25 feet