# Revision History

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.1 | 1/14/2015 | Barry Conklin | Added Changes for Rev 10 |
| 1.2 | 10-Jun-2020 | T R Kremser | General update and refresh |

# Analog Card Module

The Omega Analog Card module is used to configure and send data to an Omega PCI-DDA08/12 I/O Card. The card has 8 analog output channels and can be configured for 8 voltage ranges.

The card gets its input data from the SpdControl table. The SpdControl table provides a link to other modules (for example: ServoGapper, ZipperMerge, PrintApply).

The AnalogOutput table and the SpdControl tables work together through the Analog Card module to convert and scale the digitized voltage values to appropriate binary patterns for each analog channel, and to SpdControl “Percent” values for use within FortnaPlus.  A 12-bit conversion is used.

Set the “MaxSpeed” Column in the SpdControl table to “0000111111111111” (12 bits). The speed will be scaled for the 12-bit digital-to-analog output.

# Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc. It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document. Fortna, Inc. reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Analog Card Configuration

The Batches table must be configured to include the Analog Card (CTRL) process.

Install the Omega PCI-DDA08/12 Card. The Automation Controller will find the card and will populate the Analog Card with the correct Port Automatically (if the AnalogOutput table is configured).

Use the QNX command-line command “pci –v” to show the data for all of the PCI interfaces. Look for an interface similar to the following:

Vendor ID      = 1307h, 		(Omega Card)

Device ID      = 22h,

PCI index      = 0h

Class Code     = ffffffh Unknown (9939h)

Revision ID    = 2h

Bus number     = 2

Device number  = 10

Function num   = 0

Status Reg     = 280h

Command Reg    = 103h

Header type    = 0h Single-function

BIST           = 0h Build-in-self-test not supported

Latency Timer  = 0h

Cache Line Size= 4h un-cacheable

Base Address   = MEM@feaff800h,32bit length 128 IO@d880h length 128 bytes

IO@dff0 length 8 bytes IO@df80h,32bit length 32 bytes

Subsystem Vendor ID = 1307h

Subsystem ID        = 22h

Max Lat        = 0ns

Min Gnt        = 0ns

PCI Int Pin    = 0

PCI Int Pin    = NC

Interrupt line = no connection

The I/O address (df80) is required for the AnalogCard Software. The software is written for a board with 32 bytes of I/O registers.

# Analog Card Configuration Table

| AnalogOutputs Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | Char | Description of the Analog Card. |
| Enable | Int | Flag used to Enable the Analog Card control function, set the field to Y to enable this function. Set the field to “N” and then set it to “Y” to re-initialize the card when changing the voltage range. |
| pStatus | Char | A message showing the module’s status. |
| pError | Char | A message showing errors detected by the software. |
| PCIDeviceIndex | Int | If more than one card is added to the PC, the PCIDeviceIndex identifies each card. - First Card – 0 - Second Card – 1 |
| VoltRange | Int | Voltage Range 0 – BiPolar 2.5V 1 – BiPolar 2.5V 2 – BiPolar 5V 3 – BiPolar 10V 4 – UniPolar 2.5V 5 – UniPolar 2.5V 6 – UniPolar 5V 7 – UniPolar 10V |
| SpdRec0 – SpdRec7 | Int | Popup selection that indicates the SpdControl record associated with the Analog Channel. The converted digital speed found in the SpdControl_pBinOut column is copied to the Analog Channel. The MaxSpeed column in the SpdControl table must be set to 12 bits on (4096 decimal) to properly scale the speed. |
| ErrorSig | Int | Popup to Conveyor for a part to activate if an error occurs. Will FIRE the output |
| ErrorOutput | Int | Popup to Conveyor for a part to activate if an error occurs. Uses ErrorACTION and ErrorBIT. |
| ErrorACTION | Int | Popup to IOAction |
| ErrorBIT | Int | Choose a bit to disable when an error occurs. |
| pAnalogPort | Int | Analog control Port Address, updated by the software, when the board is initialized. |
| pInit | Int | Indicates the channel has been initialized (set by the software). |
| pChan0 - pChan7 | Int | Analog output value read from the SpdControl table, “pBinOut” Column. Values between 0 and 4095 represent a voltage defined by the VoltRange selection. Example: 4095 would equal 10V if UniPolar 10V is selected. |

# Example Configuration

The following example shows the Batches table entries for both the Analog Card and the Motor Speed Control.

| Example: Batches table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | How Often | Increment | Type | Process | Internal Proc | Internal Proc Ctrl | Machine | All Machines | Do File |
| Analog Card | 1 | Invalid | Count | CTRL |  | Spd CTRL (CPN) | CP2 | N | N |
| Mtr Speed Control | 1 | Invalid | Count | CTRL |  |  | CP2 | N | N |
| Mtr Speed Control | 1 | Invalid | Count | CTRL |  |  | CP2 | N | N |

The AnalogOutputs table is configured with one analog output, and a voltage range (0 – 10 volts). A memory bit is set for the ErrorSig that is configured in Errors to indicate if the card has not initialized. The SpdControl record is selected “Servo Gapper”.

| Example: AnalogOutputs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable | PCI Device Index | Volt Range | SpdRec0 | ErrorSig | Error Output | Error Action | Error Bit |
| Card 1 | Y | 0 | UniPolar 10.0 V | ServoGapper | Analog Card Error | INVALID | N/A | 0 |

The SpdControl table is configured for the ServoGapper, since the I/O is handled by the Omega Analog Output software, it is not mapped in Configio. The MaxSpeed is set to 12 bits, to match the Analog Card scaling.

The Off Percent will be updated by the ServoGapper module. The “MaintIO” column may be set to a memory bit “Mem Test Card”. When the bit is “ON” the “On Percent” Speed will be selected. This should produce 5 volts on Channel 0 of the Analog Card.

| Example: SpdControl |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable Flag | Maint IO | IO Owner | Off Percent | On Percent | IO Addr Out | IO Addr In | Max Speed |
| ServoGapper | Y | Mem Test Card | CTRL | 0 | 50 | 0 | 0 | 111111111111 |

The following example shows the configuration for the ServoGapper. The configuration requires two Shifters. Shifter1 is typically a Brake Meter Belt that is used to create the gap by slowing down. Belt 2 runs at a constant speed and pulls cartons away from Belt 1. Belt 2 may not be an actual belt and may be a shifter with timer based encoder if an actual encoder is not available.

The column “UpdatePt1” selects an update point located at the “End of Belt1”. The column “InductPt2” selects an Induct Point on Belt 2. When the column “CopyShift2” is set to “Y”, the box is copied to “Shifter2”. Shifter 2 must be configured with a Memory Bit in the “Induct Points” table for Shifter 2.

The ServoGapper Software Updates the SpdControl table “ServoGapper” record and the “OffPercent” Column with the calculated speed.

To Verify the ServoGapper is working properly, manually induct a carton onto Shifter1, shift the carton 1 tick past the induct point. Manually induct a carton onto shifter 1. As the carton moves to within 30 ticks of the “End of Belt” Update Point, the speed will start to decrease from 100% in the ServoGapper “Speed3” Column. The same value should appear in the SpdControl “OffPercent” Column. The value will be scaled to value between 0 and 4095 and is shown in the AnalogOutputs Column “pChan0”. The voltage can be measured on Channel 0, of the Omega Analog Output.

| Example: ServoGapper (Part 1) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable Flag | Upd Timer Flag | Copy Shift2 | Log Level | Update Pt1 | Induct Pt2 | Machine | Process |
| Ship Gapper | Y | Y | Y | Level 1 | EndofBelt1 | ServoGap2 | CP2 | SORT |

| Example: ServoGapper (Part 2) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min Ticks | Min Stop | Factor | RunIO | Min Analog | Max Analog | MinMed Spd | MaxMed Spd | Factor MedSpd |
| 30 | 1 | 1.00 | Always ON | 0.00 | 100.00 | 0.00 | 0.00 | 1.00 |

| Example: ServoGapper (Part 3) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MaxSpd Error | Spd Control | AccPer Scan | Enc2 Factor |  |  |  |  |  |
| 0.00 | Servo Gapper | 100.00 | 1.0 |  |  |  |  |  |