# Beacon Configuration

Contents

1.0	Beacon Configuration	1

2.0	Revision History	1

3.0	Beacon Table	3

4.0	Beacon Trigger Table	4

5.0	Beacon Pattern Table	5

6.0	Beacon Message Counter	6

7.0	Beacon Messaging	8

7.1	Setting up Beacon Communications	8

8.0	Beacon Simulation Table	10

8.1	Setting up a Simulation	10

8.2	Running the Simulation	12

8.3	Notes	13

9.0	End	13

# Revision History

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.3 | 12/04/2013 | Barry Conklin | Added setting up communications section |
| 1.4 | 11/25/2014 | Barry Conklin | Added Msg Counter to the logic |
| 1.5 | 15-Jun-2020 | T R Kremser | General update and refresh |

The tables on the Beacon Utilities menu are provided for setting up beacons.  Beacons can be set up to flash in various patterns or to be on continuously.

Selecting the Beacons menu displays a list of options:

“Beacon Table” – Holds one record to define each beacon.

“Beacon Trigger Table” – Used to configure tasks for beacons.

“Beacon Pattern Table” – Used to define the beacon blink patterns.

“Beacon Sim Config” table – Used to configure bacon simulations.

# Beacon Table  [BeaconInfo]

The table “Beacon Table” is used to define the I/O and default pattern for a beacon.

| BeaconInfo Table |  |  |  |
| --- | --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a descriptive name for each beacon. |
| HostBcnName | String | Enter the agreed-upon name for this beacon to match the name used by the Level II System to activate or deactivate the beacon. |
| BeaconOutput | Popup | Select a Conveyor IO part that will turn the beacon ON or OFF. |
| DefaultPattern | Popup | Select a default pattern for the beacon from the BeaconPattern table. |
| pCurPattern | Popup | The software will update this column with the beacon pattern currently selected. |
| pCurPriority | Int | The software will update this column with the priority of the currently selected pattern. |

# Beacon Trigger Table  [BeaconTrigger]

The table “Beacon Trigger Table” is used to define the patterns available for each beacon for different trigger events.  Each event may be configured with a different beacon blink pattern, if desired.

| BeaconTrigger Table |  |  |  |
| --- | --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a descriptive name for this beacon trigger. |
| HostBcnState | String | Enter the agreed-upon name for this beacon state to match the name used by the Level II System to activate or deactivate the beacon. |
| BeaconRecord | Popup | Select a specific beacon from the BeaconInfo table for this state. |
| Pattern | Popup | Select a specific pattern from the BeaconPattern table for this state. |
| Priority | Int | Enter a value to determine which state to use if more than one state is active at the same time for a particular beacon. The BeaconTrigger entry with the lowest priority will be activated. |
| TriggerInput | Popup | Select a Conveyor IO Part that will trigger this state. |
| InvertInput | Y/N | Enter N if the beacon state is triggered from the ON condition of the TriggerTimer. Enter Y if the beacon state is triggered from the OFF condition of the TriggerTimer. |
| TriggerTimer | Popup | Select a timer that will be used to trigger this state. Each time the timer times out the next element of the pattern is activated. |
| InvertTimer | Y/N | Enter N if the beacon state is to be triggered from the Done condition of the timer. Enter Y if the beacon state is to be triggered from the “Not Done” condition of the timer. |
| TriggerFlag | Y/N | Set by the software to activate this state. |
| InvertFlag | Y/N | Enter N if the beacon state is triggered from the ON condition of TriggerFlag. Enter Y if the beacon state is triggered from the OFF condition of TriggerFlag. |
| TriggerCounter | Popup | Select a CounterTrigger record that will increment, decrement, or reset a counter when a beacon message (BCN) is received. The CounterTrigger record must be configured with the appropriate IncDecQty and the Type must be one of the following: - “Reset BCN MSG” - “Count BCN MSG” |
| pOnFlag | Y/N | Set by the software to indicate the present state of this trigger |

# Beacon Pattern Table  [BeaconPattern]

The table “Beacon Pattern Table” may be customized as needed for each project.  Enter a 1 to turn the beacon ON.  Enter a 0 to turn the beacon OFF.  Select the desired pattern and trigger signal in the BeaconTrigger table.

| BeaconPattern Table |  |  |  |
| --- | --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a description for this pattern. |
| PatternString | String | Enter 1 to turn the beacon ON, or 0 to turn the beacon OFF. Example: 11111111110000000000 for a flash pattern of 50% ON and 50% OFF. |
| pCount | Int | Used internally to track which element of the pattern is active. |
| pOnFlag | Y/N | Used internally to indicate if the current pattern element evaluated to ON or OFF |
| MSecPerChar | Double | Set the time, in milliseconds, to dwell on each element of the pattern. Note that a value of 0.00 will activate the “old way”, which does the timing by counting passes through the code. The pattern speeds will be dependent on processor performance. Use the “new way”. |
| pLastSec | Int | Used internally to track elapsed time. |
| pLastNSec | Int | Used internally to track elapsed time. |

# Beacon Message Counter

The Level II system may send BCN messages to the Level I controller to indicate that an event has occurred.  The beacon module will keep track of the received BCN messages using the CounterInfo table (Configuration Menu… -> Standard Controls… -> Counter Configuration… -> Counter Info):
  Example:  When an empty tote has been used from the empty-tote area.

Logic in the Automation Controller can be used to trigger a control sequence when the CounterInfo pAccumCnt value reaches the MaxAccumCnt value.
  Example:  Move a stack of three totes into the empty tote area when the count is greater than or equal to 3.

Logic in the Automation Controller can be used to decrement the CounterInfo pAccumCnt when a control sequence is completed
  Example:  When a stack of three totes has moved into the empty tote area, decrement the CounterInfo pAccumCnt by 3.

Example Configuration:

- Configure a CounterInfo entry

  - Name - 		Cnt Empty Totes

- Configure a CounterTrigger entry

  - Name - 		Tote Count

  - Counter - 		Cnt Empty Totes

  - Type -			Count BCN MSG

  - IncDecQty - 		1

  - PresetCnt -		0

  - ResetCnt - 		0

  - TriggerIO - 		INVALID

  - ACTION - 		N/A

  - BIT - 			0

- Configure a CounterTrigger entry

  - Name - 		Tote Decrement

  - Counter - 		Cnt Empty Totes

  - Type -			Count TRANS OFF

  - IncDecQty - 		3

  - PresetCnt -		0

  - ResetCnt - 		0

  - TriggerIO - 		Memory Bit – FeedComplete

  - ACTION - 		N/A

  - BIT - 			0

- Configure a CounterTrigger entry

  - Name - 		Feed Tote Stack

  - Counter - 		Cnt Empty Totes

  - Type -			Preset >= Accum

  - IncDecQty - 		0

  - PresetCnt -		3

  - ResetCnt - 		0

  - TriggerIO - 		Memory Bit – ToteFeed

  - ACTION - 		Fire

  - BIT - 			0

- Configure a BeaconInfo entry

  - Name - 		Count Empty Totes

  - HostBcnName – 	BCN01

  - BeaconOutput – 	Memory Bit - BCN01

  - DefaultPatern – 	N/A

- Configure a BeaconTrigger – Level 2 Interface

  - Name - 		BCN01 INC

  - HostBcnState - 	INC

  - BeaconRecord - 	Count Empty Totes

  - Pattern - 		Off

  - Priority - 		0

  - TriggerInput-		INVALID

  - InvertInput-		N

  - TriggerTimer-	N/A

  - InvertTimer-		N

  - InvertFlag-		N

  - TriggerCounter - 	Tote Count

- The control sequence needed to move a stack of three totes is completed in a transfer.  The first state of the transfer checks for ToteFeed, if the memory bit is ON the transfer will run.  The last state of the transfer turns on the FeedComplete output for 100 milliseconds.  This decrements the count, preventing the Transfer from feeding another stack of three totes.

# Beacon Messaging

The Level II system typically sends BCN messages to the Level I controller to turn the TriggerFlag ON in the BeaconTrigger table.  The TriggerFlag activates the BeaconTrigger table entry by setting the flag to Y.  The level II system acts as the client, and level I as the server for these connections.

If the BCN message matches the BeaconInfo HostBcnName string:

- Any record in the BeaconTrigger table that matches the HostBcnName and matches the HostBcnState will result in the TriggerFlag being set to Y for that record.

- Any record in the BeaconTrigger table that matches the HostBcnName and does not match the HostBcnState will cause the TriggerFlag to be set to N for that record.

If the BeaconTrigger record is configured to turn the beacon ON with a string of “On”, a message received with “On” will turn on the beacon by setting the TriggerFlag to Y.  Note that any text could be used to indicate an ON condition!  Also, the strings are case-sensitive!

If the Beacon message does not have the string of “On” (for example “Off” is sent), all TriggerFlag values for the beacon found in the BeaconTrigger table will be set to N.  In other words, an entry of “Off” is not required in the BeaconTrigger table to turn a beacon off.

Example Message 1:  BCN|CC1|sHostBcnName|Beacon101|sHostBcnState|On

Example Message 2:  BCN|CC1|sHostBcnName|Beacon101|sHostBcnState|Off

Note: the string “CC1” is not required and can be a single blank character.  The strings “BCN”, “Beacon101” and “On” are required strings that must match the strings found in the MsgMap, BeaconInfo and BeaconTrigger tables.

## Setting up Beacon Communications

Setting up BCN messaging requires the following configuration steps:

- Add two entries to the Machine table (Configuration Menu… -> Communication Utilities… -> Machine):

  - BCN Listen machine “BCN_LS”

    - Connection Type = TCPSERV_LISTEN

    - Communication Protocol = SrtCommSeq

    - Machine Port = 9001 (agreed-upon port)

    - Transmit Retry = 10

    - Process = SERV

    - NeedACK = 0

  - BCN Read/Write machine “BCN_RW”

    - Connection Type = TCPSERV_RDWR

    - Communication Protocol = SrtCommSeq

    - Machine Port = 9001 (agreed-upon port)

    - Transmit Retry = 10

    - Process = SERV

    - NeedACK = 0

- Add one entry to the Message Map table (Configuration Menu… -> Communication Utilities… -> Message Map):

  - BCNRecv

    - Machine name = “BCN_RW”

    - Menu Name = MsgGP2

    - Recv_Msg_Type = ALL

    - Send_Msg_Type = ALL

    - Ack_Msg_Type = ALL

    - Low Cursor = 1

    - High Cursor = 54 (adjust if needed)

    - Stat Cursor = 67 (will auto-fill if left blank)

    - Key Cursor = 68 (will auto-fill if left blank)

# Beacon Simulation Table  [BeaconSimConfig]

The Beacon Control Message (BCN) from Level II may be simulated using the “Beacon Sim Config” table.

| BeaconSimConfig Table |  |  |  |
| --- | --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a description for the simulation record. |
| Enabled | Radio | Enables or disables processing of the simulation record. |
| Beacon | Popup | Select a record from the BeaconInfo table. |
| MsgMenuName | Popup | Select the desired messaging table. |
| BeaconState | String | Enter the state string to transmit to the target beacon in the BCN message. |
| Trigger | Radio | Indicates a request to trigger a BCN message. You can activate this field manually to send a message on demand (Click Apply). |

## Setting up a Simulation

Setting up a BCN simulation requires the following configuration steps:

- Determine the IP address of the PC.  From command line type: ifconfig

  - Add the IP address to the file /etc/hosts with the logical name BCN_CL

  - Example: 192.168.10.210 BCN_CL

- Add three entries to the Machine table (Configuration Menu… -> Communication Utilities… -> Machine):

  - BCN Listen machine BCN_LS

    - Connection Type = TCPSERV_LISTEN

    - Communication Protocol = SrtComm

    - Machine Port = 9001 (must match the port set for BCN_CL)

    - Transmit Retry = 10

    - Process = SERV

    - NeedACK = 1

  - BCN Read/Write machine BCN_RW

    - Connection Type = TCPSERV_RDWR

    - Communication Protocol = SrtComm

    - Machine Port = 9001 (must match the port set for BCN_CL)

    - Transmit Retry = 10

    - Process = SERV

    - NeedACK = 1

  - BCN Client machine BCN_CL

    - Connection Type = CONNECT_TCPCLIENT

    - Communication Protocol = SrtTest

    - Machine Port = 9001 (must match the port set for BCN_LS and BCN_RW)

    - Transmit Retry = 10

    - Process = SERV

    - NeedACK = 0

- Add two entries to the Message Map table (Configuration Menu… -> Communication Utilities… -> Message Map):

  - BCNSimSend

    - Machine Name = “BCN_CL”

    - Menu Name = MsgGP1

    - Low Cursor = 1

    - High Cursor = 54

    - Stat Cursor = 67 (will auto-fill if left blank)

    - Key Cursor = 68 (will auto-fill if left blank)

  - BCNSimRecv

    - Machine name = “BCN_RW”

    - Menu Name = MsgGP2

    - Low Cursor = 1

    - High Cursor = 54

    - Stat Cursor = 67 (will auto-fill if left blank)

    - Key Cursor = 68 (will auto-fill if left blank)

- Add entries to the BeaconSimConfig table (Configuration Menu… -> Beacon Utilities… -> Beacon Sim Config):

For this demonstration, two simulation records will be created:

  - BeaconSim1. Start with record 1 (Do not use record 0)

    - Enabled = Y

    - Beacon = select desired beacon configuration

    - MsgMenuName = MsgGP1

    - BeaconState = “On”

    - Trigger = OFF

  - BeaconSim2. Use Record 2

    - Enabled = Y

    - Beacon = select desired beacon configuration

    - MsgMenuName = MsgGP1

    - BeaconState = “On”

    - Trigger = OFF

## Running the Simulation

- Turn Beacon processing on in the Batches table (Configuration Menu… -> Standard Controls… -> Batches)

  - File Name = “Beacons”

  - How Often = 1

  - Increment = INVALID

  - Type = Count

  - Process = CTRL

  - InternalProcCtrl = BeaconProcess

  - Machine = test machine name

- Open the BeaconInfo table (Configuration Menu… -> Beacon Utilities… -> Beacon Table)

  - Supply a HostBcnName string for each beacon configuration you wish to be able to control.  For this demonstration, choose two configurations and supply the following name strings:

    - “PalletSim01”

    - “PalletSim02”

- Open the BeaconTrigger table (Configuration Menu… -> Beacon Utilities… -> Beacon Trigger Table)

    - Supply a HostBcnState string for each BeaconTrigger configuration you wish to be able to control.  For this demonstration, choose two configurations and set the HostBcnState columns to “On” (this string must match the BeaconState string supplied in the BeaconSimConfig table configured above).

- Open the BeaconInfo table for monitoring.

- Open the BeaconSimConfig table.

- Set the BeaconState column for BeaconSim1 to “On”.

- Set the Trigger column for BeaconSim1 to ON.

- Click Apply and verify that the desired beacon has been turned on.

- Set the BeaconState column for BeaconSim1 to “Off”.

- Set the Trigger column for BeaconSim1 to ON.

- Click Apply and verify that the desired beacon has been turned off.

- Set the BeaconState column for BeaconSim2 to “On”.

- Set the Trigger column for BeaconSim2 to ON.

- Click Apply and verify that the desired beacon has been turned on.

- Set the BeaconState column for BeaconSim2 to “Off”.

- Set the Trigger column for BeaconSim2 to ON.

- Click Apply and verify that the desired beacon has been turned off.

## Notes

The BeaconState column of the BeaconSimConfig table must match the string for the HostBcnState column of the respective BeaconTrigger table record.

Although this simulation used the string “On” to trigger beacon functionality, the string can have any value you choose.

# End