| # FortnaPlus Control Chain Stretch Module |  |
| --- | --- |
|  | Chain Stretch |
| Document Revision: 1.1 |  |

Date: 17 June 2020

Revision; Date; Author; Revision Summary. 1.0; 13-Nov-2013; B Conklin; New document. 1.1; 17-Jun-2020; T R Kremser; Updates and general refresh.

Table of Contents

1.0	Chain Stretch Module	4

2.0	Chain Stretch Processing Logic	6

3.0	Chain Stretch Control Logic Description	7

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Chain Stretch Module

The Chain Stretch module is designed to monitor chain stretch sensors for Spiralveyor spiral conveyors.  Table ChainStretch is used to define the configuration and monitor the status of spiral chains.

Drive Shaft

Idler Shaft

Drive Sensor

Idler Sensor

DriveSignal

IdlerSignal

Pulses are indicating proper operation (no stretch)

DriveSignal and IdlerSignal (not on at the same time)

DriveSignal

IdlerSignal

Pulses are indicating a chain stretch error

DriveSignal and IdlerSignal (are on at the same time)

DriveSignal – Input used to indicate that the drive shaft is turning.

IdlerSignal – Input signal used to indicate that the idler shaft is turning.  When the IdlerSignal is ON the DriveSignal should be OFF.  If the IdlerSignal and the DriveSignal are ON at the same time, the chain is too loose.

# Chain Stretch Processing Logic

The DriveSignal and the IdlerSignal are each expected to generate only one Transition-ON (TRANSON) signal per cycle.  If more than one DriveSignal TRANSON is detected before the IdlerSignal is detected, the chain status (pStatus) is set to “Chain Broken”.

If the DriveSignal or the IdlerSignal is not detected in N seconds (IntervalTimer) while the motor is running, the chain status (pStatus) is set to “Chain Broken”.  A sensor failure would trigger this condition, as well.

The Chain Stretch module will stop the conveyor by performing the StopAction selected in the ChainStretch table whenever the pStatus is set to “Chain Broken”.  If appropriate, MtrBit will be used to carry out the StopAction chosen on the motor defined in ConvMotor.

If the conveyor has stopped due to a fault, the fault may be reset by clicking on the menu option “Reset Stretch Error”, found on the “Reset Utilities…” menu.

The Chain Stretch module will not start processing until the StartTimer has completed.  This helps avoid false alarms as the spiral accelerates to operating speed.  The StartTimer must be set to as low a value as practical.

The Chain Stretch module must be activated in table Batches using “Stretch CTRL (CPN)”.  If the “SORT” process is used to control the Chain Stretch module, the “TightenChainErr” and the “BrokenChainErr” memory bits must be owned by the “SORT” process.

Row 0 in table ChainStretch is not used.  Start with row 1.

# Chain Stretch Control Logic Description

| ChainStretch table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a descriptive name for each specific Chain Stretch sensor. |
| Enabled | Radio | Used to activate processing for the sensor. |
| pStatus | String | Status messages generated by the module. |
| StopIfLoose | Y/N | Set to Y to stop the conveyor when the chain is loose. If N, the conveyor will stop only if a chain break is detected. |
| DriveSignal | Popup | Choose an input signal that indicates the drive shaft is turning. |
| IdlerSignal | Popup | Choose an input signal that indicates the Idler Shaft is turning. When the IdlerSignal is ON the DriveSignal should be OFF. If both signals are ON at the same time, the chain is too loose. The DriveSignal and the IdlerSignal are each expected to generate only one Transition-ON (TRANSON) signal per cycle. If more than one DriveSignal TRANSON is detected before the IdlerSignal is detected, the chain status (pStatus) is set to “Chain Broken”. If the DriveSignal or the IdlerSignal is not detected in N seconds (IntervalTimer) while the motor is running, the chain status (pStatus) is set to “Chain Broken”. A sensor failure would trigger this condition, as well. |
| ControlMachine | Popup | Choose the Machine that controls this spiral. |
| StartTimer | Popup | Choose a timer to control how long to wait when the spiral is started before beginning to monitor for chain stretch. Set the time to allow the spiral to reach full speed. |
| IntervalTimer | Popup | Choose a timer to monitor the IdlerSignal. Set the timer to a period of at least the interval between IdlerSignal pulses when at speed. The IntervalTimer is reset after each DriveSignal/IdlerSignal sequence. If the timer expires before it is reset, a “Chain Broken” error will be raised. |
| TightenChainErr | PopUp | Choose a memory bit that will be set ON if a “Chain Loose” condition is detected. It can be used to raise an Error. |
| BrokenChainErr | PopUp | Choose a memory bit that will be set ON if a “Chain Broken” condition is detected. It can be used to raise an Error. |
| ConvMotor | PopUp | Choose the output part for the motor that drives the spiral. It will be used to stop the motor if the chain is broken (or loose if StopIfLoose is set). |
| StopAction | PopUp | Choose the action required to run the conveyor under normal conditions. The control logic will use the inverse action when it needs to stop the conveyor. If the motor is not to be stopped by this module, select N/A. |
| StopBit | Int | Enter the bit to use if the Enable or Disable action is chosen for StopAction. |
| pDrivePulse | Int | Indicates a rising edge was received by the Drive Signal. |
| pIdlerPulse | Int | Indicates a rising edge was received by the Idler Signal. |
| pStopLoose | Int | Indicates the Stop should be activated for a loose chain. |
| pStopBroken | Int | Indicates the Stop should be activated for a broken chain. |