| # FortnaPlus Control Counter Configuration |  |
| --- | --- |
|  | Counter Configuration |
| Document Revision: 1.1 |  |
| Date: 18 June 2020 |  |

Revision; Date; Author; Revision Summary. 1.0; 02-Dec-2014; B Conklin; New document. 1.1; 18-Jun-2020; T R Kremser; Updates and general refresh.

Table of Contents

1.0	Counter Configuration	4

2.0	The CounterInfo Table	5

3.0	The CounterTrigger Table	6

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Counter Configuration

Counters are used to count events and take action when conditions are met.  The Counter module provides the ability to configure a counter.  Features include triggers based on incrementing events, reset events, and control events.  Outputs can be fired in association with specific count values.

Be sure to activate an entry in the Batches table to run the Counter module.  Use Counters (CTRL) in the InternalProcCtrl column, and the CTRL process to run this module.

Counters may also be linked to Beacon messages through the BeaconTrigger table using the TriggerCounter column.

The tables used to configure Counters can be found under Configuration Menu -> Standard Controls -> Counter Configuration.  It contains two tables:

Counter Info [CounterInfo] – Holds one record for each counter.

Counter Trigger [CounterTrigger] – Holds one or more record for each Counter.  A Counter may have multiple inputs and outputs that are used to trigger events.

For example:  A merge lane may be configured to hold 5 cartons.  A Counter can be defined to keep track of the cartons inducted into the lane and reset the count when the lane is discharged.  The infeed I/O can be disabled when a preset count of 5 is reached, preventing any further inducts.  An output can be turned ON to enable the merge to feed when the count reaches 5.

# The CounterInfo Table

The CounterInfo table defines the Counter and its overall attributes.

| CounterInfo table |  |  |
| --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a descriptive name for the Counter. |
| Enable | Y/N | Set this to Y to enable the Counter. Set to N to disable the I/O, stop counting and turn OFF any outputs configured in CounterTrigger_TriggerIO. |
| EnableIO | PopUp | Choose a part from the Conveyor table. If the selected part is ON, the counts will be updated, and the outputs will be activated. If the selected part is OFF, the counter will stop counting, and the outputs will be turned OFF. |
| LogLevel | PopUp | Choose the desired Log Level for the Counter. Level 1 will report triggering events. |
| MinAccumCnt | Int | Enter the minimum allowed value for the accumulated count in pAccumCnt. CheckMinMaxAccum must be set to Y to activate the minimum and maximum check. |
| MaxAccumCnt | Int | Enter the maximum allowed value for the accumulated count in pAccumCnt. CheckMinMaxAccumn must be set to Y to activate the minimum and maximum check. |
| CheckMinMax Accum | Y/N | Set this to Y to activate the check of minimum and maximum accumulated values. The pAccumCnt values will not be allowed outside of the range set by MinAccumCnt and MaxAccumCnt. If set to N, the pAccumCnt value will have no limits applied to it. |
| pAccumCnt | Int | Set by the module. Current accumulated count for the Counter. |

# The CounterTrigger Table

The CounterTrigger table is used to define the events that increment a Counter, the events that reset a Counter, and the actions to take when the counter value in CounterInfo_pAccumCnt is in various states relative to the PresetCnt.  The following is a detailed list of the Trigger Types.

Inputs (State read from TriggerIO):

- Reset TRANS ON:  The Counter will be set to the PresetCnt from this record when the input in TriggerIO transitions ON.

- Reset TRANS OFF:  The Counter will be set to the PresetCnt from this record when the input in TriggerIO transitions OFF.

- Reset BCN MSG:  The Counter will be set to the PresetCnt from this record when a BCN message is received.  A BeaconTrigger_TriggerCounter field must be linked to this record.

- Count BCN MSG: The Counter will be incremented or decremented by the amount found in the IncDecQty column when a BCN message is received.  A BeaconTrigger_TriggerCounter field must be linked to this record.  An example is found in the Beacons document.

- Count TRANS ON:  The Counter will be incremented or decremented by the amount found in the IncDecQty column when the input in TriggerIO transitions ON.

- Count TRANS OFF:  The Counter will be incremented or decremented by the amount found in the IncDecQty column when the input in TriggerIO transitions OFF.

Outputs (Written to TriggerIO):

- Preset < Accum:  The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is less than the accumulated count.

- Preset <= Accum:  The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is less than or equal to the accumulated count.

- Preset == Accum:  The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is equal to the accumulated count.

- Preset > Accum:  The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is greater than the accumulated count.

- Preset >= Accum:  The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is greater than or equal to the accumulated count.

- Preset != Accum: The output selected in the TriggerIO column will be activated based on the selected ACTION when the PresetCnt is not equal to the accumulated count.

| CounterTrigger table |  |  |
| --- | --- | --- |
| Column | Type | Value or Description |
| Name | String | Enter a descriptive name. |
| pStatus | String | Status message updated by the module to indicate any configuration issues. |
| Counter | PopUp | Choose the CounterInfo record to associate with this trigger record. Multiple CounterTrigger records can be linked to a single CounterInfo record. |
| Type | PopUp | Choose a trigger type. The trigger types are discussed in more detail above. Inputs: 1 – Reset TRANS ON 2 – Reset TRANS OFF 3 – Reset BCN MSG 4 – Count BCN MSG 5 – Count TRANS ON 6 – Count TRANS OFF Outputs: 7 – Preset < Accum 8 – Preset <= Accum 9 – Preset == Accum 10 – Preset > Accum 11 – Preset >= Accum 12 – Preset != Accum |
| IncDecQty | Int | If Type is set to one of the following, the value entered here will be added to the accumulated count in CounterInfo_pAccumCnt when the event occurs. The value may be a positive or negative value. 4 - Count BCN MSG 5 – Count TRANS ON 6 – Count TRANS OFF |
| PresetCnt | Int | If Type is set to one of the following, this value will be compared to the accumulated count in CounterInfo_pAccumCnt, and used to control the state of the selected output in the TriggerIO column. 7 - Preset < Accum 8 – Preset <= Accum 9 – Preset == Accum 10 – Preset > Accum 11 – Preset >= Accum 12 – Preset != Accum |
| ResetCnt | Int | If Type is set to one of the following, this value will be used to set the count value in CounterInfo_pAccumCnt when the event occurs. The value may be a positive or negative value. 1 – Reset TRANS ON 2 – Reset TRANS OFF 3 – Reset BCN MSG |
| TriggerIO | PopUp | Choose a part from the Conveyor table. These will be used either as an input that will be used to trigger a Counter event or an output that will be activated, based on the Type chosen for the row. |
| ACTION | PopUp | Select the action that will be used with TriggerIO for the following output Types. If Enable or Disable is chosen, set a value in the BIT column also. 7 - Preset < Accum 8 – Preset <= Accum 9 – Preset == Accum 10 – Preset > Accum 11 – Preset >= Accum 12 – Preset != Accum |
| BIT | Int | Select the BIT that is associated with the Enable or Disable ACTION. |