| # FortnaPlus Control Disable Bits |  |
| --- | --- |
|  | Document Title |
| Document Revision: 1.2 |  |
| Date: 16 July 2020 |  |

Revision; Date; Author; Revision Summary. 1.0; Unknown; T R Kremser; New document. 1.1; 14-Jul-2020; T R Kremser; Updates and general refresh. 1.2; 16-Jul-2020; T R Kremser; Add ForceOn / Unforce information.

Table of Contents

Disable Bits Explained	4

Assigned Disable Bits	4

Configurable Disable Bits	5

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Disable Bits Explained

Many FortnaPlus Control (FPC) modules make use of the concept of “disable bits”.  Disable bits are a way to have up to 16 independent ways to turn off a particular IO point.  Each IO point has a “Disable I/O” field in table Conveyor.  If any one of the sixteen bits are turned ON, the part will be set to an OFF state, and would be referred to as “disabled”.  This is distinct from the part’s ON or OFF status as the result of a FIRE.  A part can be ON as the result of a FIRE, but disabled as the result of any disable bit being set high.  For example, the motor that drives a section of conveyor might be ON but not running as the result of the Fullline module, which may disable the motor when a full condition is detected.

Some FPC modules lay claim to particular disable bits and are not configurable.  Others allow the disable bit to be explicitly chosen.

Note that there is nothing within the FPC code that prevents the use of the same disable bit by more than one module, but configuring it that way is an error!  Doing so will produce unpredictable results, as one module might turn the disable bit on and the other turn it off again.  This document is intended to help you choose independent disable bits and avoid that situation.

# Assigned Disable Bits

The following table shows the disable bits that are not configurable, and the typical use of a few that are configurable, but are commonly used for a particular purpose.

Note that in all modern versions of FPC, all of the “single digit” disable bits are safe to use!

| Disable Bit | Typical Use |
| --- | --- |
| 0, 1, 2 | Often chosen for triggers |
| 5, 6, 7 | Old-style scanner data processing using table scanstat (not used in V7.5 and later) 5 – Maximum_NoReads *, 6 – Maximum_NoScans *, 7 – Maximum_Conflicts * |
| 10 | Used by Liftgates and Merges |
| 12 | Used by Transfers [old (Xfer) and new (Xfr)] |
| 14 | Used by the Inch and Store module (also uses user-defined bits) |
| 15 | Used by the Fulls module |

* Disables in table scanstat are activated if scanstat_InductStopIO is set.

# Configurable Disable Bits

Many modules that use disable bits allow the bit to be chosen.  They tend to roughly follow a pattern of three columns for OUTPUT, ACTION, and BIT.  The OUTPUT column defines the IO point to act upon, the ACTION column defines the type of action to take, and the BIT column is used to define the disable bit if the ACTION is Disable or Enable.  Note that the Enable option acts on the Disable_I_O bit, but in the opposite (inverted) sense, so Not Enabled = Disabled.

The following list shows all the BIT columns in all tables:

ABS_EIP_ADAPTERS_ErrorBIT

AccumZone_ErrorBit

AccumZone_ZoneRunBit

AnalogOutputs_ErrorBIT

CapacityGapCtrl_StopBIT

CartonCounter_ZoneRunBit

ChainStretch_MtrBit

CombinedJamZones_JamEnableBIT

CounterTrigger_BIT

EnergyZone_StopBit

Errors_WcsBIT

Fulljam_DisableBit

Fullline_FullClearBIT

GapCfg_StopBIT

HSSawLane_AccumBit

HSSawLane_ErrMtrBit

HSSawLane_FastBit

HSSawLane_SlowBit

HSSawLane_SlowOrFastBit

IOCard_ErrorBIT

InchStore_DumpBit

InchStore_SlowBit

KTX_RIO_ADAPTERS_ErrorBIT

MissingPin_EnableBIT

MissingPin_StopBIT

NewGapper_StopBIT

PMIOTable_IOSignalBIT

PowerSupply_ErrorBIT

PrintApplyLine_DataBIT

PrintApplyLine_MtrBIT

PrintApplyLine_ScanBIT

PrintApplyLine_SpeedBIT

PrintApplyLine_VerifyBIT

PrinterBoss_PrinterErrBit

PrinterErrConfig_ErrorBIT

PrinterUpdPnts_ErrorBIT

RateCount_RateHighBIT

SawLane_DisableBit

ScnZoneErrIO_ErrorBIT

ScnDeviceErrIO_ErrorBIT

SpdControl_AuxBit

SpdControl_NetCTRLBit

SpdControl_NetREFBit

SpdControl_ResetBit

SpdControl_StartBit

SpdControl_StopBit

SrtAppControl_DataBIT

SrtAppControl_MtrBIT

SrtAppControl_NotAvailBIT

SrtAppControl_ScanBIT

SrtHrtBeat_CommErrBIT

SrtHrtBeat_HostErrBIT

Trigrset_Rvalue (for triggers that enable or disable bits)

XfrErrConfig_ErrorBIT

XfrParamConfig_ErrorBIT

XfRouteBoss_DataBIT

XfRouteBoss_MtrBIT

XfRouteBoss_ScanErrBIT

XfRouteTable_ErrorBIT

ZipperLane_AccumBit

ZipperLane_StuckBit

ZprGapperZone_ZoneBit

# ForceOn and Unforce

Similar in concept to the “disable bit”, is the “enable bit”.  The disable bit information is stored in table Conveyor, column “Disable I/O” (or Conveyor_Disable_I_O in the program).  The enable bit information is stored in table Conveyor, column “Overide I/O” (or Conveyor_Overide_I_O in the program.  No, that’s not a typo.).  If ForceOn or Unforce is chosen for the ACTION, the bit number chosen in the BIT column will be set or cleared.  The Overide_I_O setting will determine the state of the IO.  These forces are not the same as the forces performed through the View IO window.  Those are controlled by bits set in table FORTNADT.  ForceOn and Unforce should perhaps have been more accurately named Override and CancelOverride.  They act in the opposite sense of Disable and Enable.  If any bit in the Overide_I_O word is on, the part will be set on.  If no bit in the Overide_I_O word is on, the part will return to its “normal” state.  Note that Not Unforced = Forced, just as Not Enabled = Disabled.  Of all these choices, Disable is by far the most often used.