# FortnaPlus Control EV10 Connections

Document Revision: 1.2

Date: 01-Mar-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 16-Sep-2016 | T R Kremser | New document |
| 1.1 | 14-Jul-2020 | T R Kremser | Update and general refresh |
| 1.2 | 01-Mar-2023 | T R Kremser | Consolidate information on the screen capture feature |

Table of Contents

Glossary of Terms	4

1.	Purpose	5

2.	EV10 Interface Overview	5

3.	WCS Connection Details	6

4.	REST Interface Connection Details	6

5.	Hosts File Details	7

6.	Machine Table Details	7

7.	WCSEvents Table	8

8.	MsgWCS Table	9

9.	Errsever Table	9

10.	WCSLINKPARTS Flag	10

11.	Dashboard Configuration Details	10

11.1.	Screen Capture Configuration Details	12

11.2.	Dashboard Troubleshooting	14

12.	Troubleshooting	14

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Purpose

The EV10 interface was designed to serve as a single user interface for all our Fortna WCS products.  It displays status information, and historical reports, and can also be used to alter certain settings that previously required direct access to the underlying software products on individual systems, such as CommCore, the PandA engine, and FortnaPlus.

Data contributed by FortnaPlus Control (FPC) includes current errors, error history, the status of the running conveyor in both textual and pictorial forms, and conveyor performance information like throughput and scanner read rates.

This document describes the interface between FPC and EV10 and how to configure it.  It also covers an older product known as the “Dashboard”, that served a similar purpose.

# EV10 Interface Overview

There are two distinct sets of connections between FortnaPlus and EV10.

The first set, the WCS connections, deliver data as events occur (“push” style) and have a very limited capability to receive incoming requests.  The WCS connections use the ActiveMQ message queue as an intermediary to handle the messages.

The second set of messages use a RESTful (web style) connection.  The REST interface is described in full elsewhere, but in this context, it is used to create a remote interactive interface for two FPC modules:  The Start/Stop Window, and the PandA Operator’s Interface.  These interfaces have the same features that are available directly from the FPC console.

This diagram shows the message flow for the two interfaces:

Automation Controller

ActiveMQ Server

Topic/Queue

Topic/Queue

Stomp
Messages

EV10
or
Web Browser client

WCS Connections

REST Server: 9123

TCP
Messages

# WCS Connection Details

All ACs must be configured with the following three connections.  The entries will be automatically generated in the Machine table, but they will not be activated.  An /etc/hosts entry must be defined for the three connection names to the appropriate ActiveMQ server, and the Machine_Process must be filled in to activate the connections.  See section 7 Machine Table Details, below.

WCS_OUTBOUND – All messages that originate on each AC are sent out through this interface as they occur.  That includes Events and Errors (Alarms).  A message is generated for each error by the AC that owns it when the error occurs, and when the error clears.  Event messages include start / stop information.

WCS_IN_TOPIC – The AC listens for the ALARMSTATE command on this interface.  The EV10 system may request this information at any time, but usually does so only when that system is first started.  Each AC will receive a copy of each message issued through the WCS_IN_TOPIC interface.  Each AC will respond through the WCS_OUTBOUND interface with the state of all the Errors (Alarms) that it owns.

WCS_IN_QUEUE – The AC listens for the ZONECONFIG and ZONESTATE commands on this interface.  The EV10 system may request this information at any time, but usually does so only when that system is first started.  Only one AC will receive each message issued through the WCS_IN_QUEUE interface.  That AC responds with information on behalf of all the ACs.  ZONECONFIG causes the AC to emit the definition of the Start/Stop and Jam zones.  ZONESTATE causes the AC to emit the current state of each Start/Stop and Jam zone.  All responses are sent through the WCS_OUTBOUND interface.

# REST Interface Connection Details

The REST interface connection is defined on one AC to handle all the requests from EV10 on behalf of all the Automation Controllers.

This interface creates a specialized web-server-style connection that can deliver information about the running ACs on-demand and can accept certain commands to perform actions on the ACs.  EV10 uses only a small portion of the REST interface capabilities.  See the FPC-REST-API document for complete details on the API.

Since the web-style requests can put large momentary demands on the AC, use the SRV1 process for this connection to avoid interfering with the time-sensitive CommCore messages that use the SERV process.  See section 7 Machine Table Details, below.

# Hosts File Details

The file /etc/hosts connects the FPC Machine table entries for the WCS connections to EV10 by defining the IP addresses associated with the machine names.  The REST interface, being a server-style connection, needs no /etc/hosts entry, but the IP address of the AC chosen for that connection will be needed for the EV10 configuration, as EV10 will be the client.

Example /etc/hosts entry:

192.168.10.140	WCS_OUTBOUND WCS_IN_TOPIC WCS_IN_QUEUE

# Machine Table Details

The three WCS and two REST Machine table entries will be auto generated (as well as the corresponding entry in table MsgMap) but will be inactive.

Set the Process column to SRV1 to activate each line.  Verify that the LogLevel column is set to “Level 2” to have the messages written to the log for easier debugging.  Transmit_Retry should be set automatically to 10 seconds.  For WCS_OUTBOUND, Machine_Port will be set automatically to the appropriate ActiveMQ destination as needed.  Set up the others as shown in the following table:

| Machine_Name | Connection _Type | Communication _Protocol | Machine_Port | Process | LogLevel |
| --- | --- | --- | --- | --- | --- |
| WCS_OUTBOUND | WCS_TALK | WCS | Set by program | SRV1 | Level 2 |
| WCS_IN_TOPIC | WCS_LISTEN | WCS | /topic/ WCSREQUESTTOPIC | SRV1 | Level 2 |
| WCS_IN_QUEUE | WCS_LISTEN | WCS | /queue/ WCSREQUESTQUEUE | SRV1 | Level 2 |
| REST Listen | TCPSERV _LISTEN | HOST_REST | 9123 | SRV1 | Level 2 |
| REST RdWr | TCPSERV _RDWR | HOST_REST | 9123 | SRV1 | Level 2 |

Be sure to re-start FPC after adding Machine table entries!

# WCSEvents Table

FPC can emit messages for many different types of events.  The list is defined in table WCSEvents.  The WCSEnable column can be used to control which types of events will send messages.  This table also has fields to control a similar feature for generating NMS messages, which are described elsewhere.

| WCSEvents Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| EventName | String | A unique name for the event |
| WCSEnable | Y/N | Flag used to enable or disable the processing of the event. Set the field to Y to enable sending this event. |
| WCSDestination | String | The ActiveMQ topic that will receive this event. These strings are copied to Machine_Machine_Port as the messages are sent. |
| WCSCategory | String | The ActiveMQ message category associated with the event. These strings define how EV10 will handle the messages. |
| WCSService | String | The ActiveMQ message service for the event. These strings define how EV10 will handle the messages. |
| WCSMachProc | Y/N | If set to Y, the Machine_Process is added to the string defining the Machine_Machine_Name. |
| WCSLastTime | DateTime | Timestamp indicating the last time an event of this type was sent. |
| NMSEnable | Y/N | Events are processed and sent to the OpenNMS interface when this column is set to Y. Note: Currently, only the ERROREVENT events can be sent to the OpenNMS Interface. |
| NMSLastTime | DateTime | Timestamp indicating the last time an event of this type was sent to the OpenNMS Interface. |

# MsgWCS Table

When messages are built to send to EV10, they are put into the MsgWCS table.

| MsgWCS Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| MsgText | Blob | The text of the message. There is room for a string up to 2749 characters in length. |
| Destination | String | The ActiveMQ topic for the message. |
| TxStat | PopUp | Message status. 0 – New (Message has not been sent) 1 – Circ (Message is waiting to be sent) 2 – Built (Message has been built) 3 – Sending (Message is in the process of being sent) 4 – Sent (Message has been sent) 5 – Ack (Not used with the ActiveMQ interface) 6 – Nack (Not used with the ActiveMQ interface) |
| DbKey | Int | Indicates the send sequence. |
| DbTime | Timestamp | The time when the message was added, in human-readable format. |
| iDbTime | Int | The time when the message was added, in an internal time stamp integer format. |

# Errsever Table

The Errsever table translates the internal FPC error severity code number to WCS severity strings.  It is also used to control whether an Alarm message is sent to EV10.

The wcsSeverityStates table defines the text strings that EV10 will recognize.  They are Normal, Warning, Major and Critical.  The value NONE, if set in FPC, will prevent a message from being sent for that severity level.

The severity level “FULL WARNING” is set, by default, with a wcsSeverity of NONE so that full errors are not sent.  Most customers do not want full errors to be treated as Alarms, but if they do, the wcsSeverity value can be changed to Warning.

| Errsever Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Severity_Level | String | Text of the error messages, as selected in the Severity column of table Errors for FPC errors |
| Foreground | PopUp | Foreground Color used to display errors in the FPC F10–0 Screen |
| Background | PopUp | Background Color used to display errors in the FPC F10–0 Screen |
| wcsSeverity | PopUp | The severity level as required by EV10. The text comes from table wcsSeverityStates. A setting of NONE will prevent errors of this type from generating Alarm messages for EV10. Change the default of NONE for the “FULL WARNING” row to Warning to send full errors to EV10. |

# WCSLINKPARTS Flag

An optional feature may be activated by setting Integer Flag #99 WCSLINKPARTS = 1.  This causes an additional field to be included in the event messages.  The field consists of a comma-separated list of all parts associated with the item in question.  The flag is off (0) by default.

NOTE:  The EV10 system must be configured to expect this additional field!

# Dashboard Configuration Details

With EV10, the conveyor image is defined from the mechanical design drawing CAD files, and the part status colors are set in a way similar to the way it is done in FPC, with the status of each part evaluated individually and drawn in the appropriate color.  A three-dimensional rendering of the conveyor system is thus available to the user, with colors representing the current operational state of each part.

The older “Dashboard” product used the same WCS Connections as with EV10, but also sent a snapshot image of the conveyor display from one AC periodically to use as a “wallpaper” image on the Dashboard interface.  This method certainly had its problems, starting with the image always being out of date – it was typically refreshed not more than once every 30 seconds.  The image was sent through the ActiveMQ WCS_OUTBOUND connection to a special queue.  Because of its size, it caused additional problems in transmission.  There were often permissions problems when writing it back to a file on the other end.  It also required numerous gyrations in FPC and QNX to create and manipulate the image prior to it being sent.  Since there may still be a few systems out there using this method, I give the details of how to configure it and debug it here.

The “snapshot” of the GUI screen is taken using a batch job called “Print Main Screen”, which is a long-standing FPC feature.  It, unfortunately, honors the current display zoom level, so normal user activity can alter the captured image and render it unsuitable.  The Batches entry for this was configured to run from the GUI process every 30 seconds.

The captured image was in a QNX proprietary format (QNX 6.5 writes it as a bmp, at least, saving one step).  It was manipulated by the shell script SendConvImage (or ConvPhs2Png in even older versions) to convert the image to a png file and send it to ActiveMQ at /topic/WCS.IMAGE.CONVEYOR using the WCS_OUTBOUND connection (ConvPhs2Png used scp and ssh to deliver and manipulate the image file for even more security configuration fun).  SendConvImage was run as a batch job on the same AC.  It also was run every 30 seconds by the GUI process.

To overcome the issues with the screen capture being influenced by the zoom level, an additional batch job was added.  Called “Go View 0”, this batch job is also configured on the same AC to be run by the GUI process every five minutes.  That way, only at most 10 images will be generated at the “wrong” zoom level before it resets, and the user of the AC won’t get too annoyed when the zoom level changes.  At least that’s the theory.  It also required that a PhView “View 0” be created using a “Map” to define the area and zoom level of the “proper” image to be sent to the Dashboard.  Map entries were created by selecting an area of the GUI screen and saving the mapped area as a new “Map”.  The “Part info” box has a check box that enables map selection and creation.

To define A “Map” (PhMap table record), left-click on any part to get the “Part Info” window, check the “Enable map selection” box and click Save.  Then, click and drag to define the area to be included in the map.  Click “Yes” to save the map.  If the box indicates it will be saved as “Map #0”, repeat the process, since we cannot use PhMap record number 0 for this purpose.  Once a Map has been successfully created, Left-click a part again, un-check the “Enable map selection” box and click Save to turn the map selection feature off again.

Once you have a Map entry defined, open table PhView and select the newly-defined Map in the “Map” column for record zero to associate the new Map with “View 0”.  Set X, Y and Z to zero to display the entire map, since they refer to the mapped area, not the entire canvas once a map is defined.

Select View 0 and verify that it displays the desired area.

# Screen Capture Configuration Details

The Screen Capture feature is configured in table Batches to run on one Automation Controller.  The batch job runs the “PrintMainScreen” function that captures an image of the main screen.

| Batches Entries |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| File_Name | HowOften | Increment | Type | Process | InternalProc | Machine | DoFile |
| Print Main Screen | 30 | Sec | Elapsed | GUI | Print Main Screen | This AC |  |
| SendConvImage | 30 | Sec | Elapsed | GUI |  | This AC | Y |
| Go View 0 | 5 | Min | Elapsed | GUI | Go View 0 | This AC |  |

The “PrintMainScreen” function uses the parameters found in the ScreenCapture table to define how to capture the screen image to a file for delivery to the Dashboard.  Configure row 1 as defined in the ScreenCapture table, below.  Row 0 is not active.

The QNX shell script SendConvImage resides in the RUN directory.  It prepares the screen image file and sends it to the Dashboard through the ActiveMQ interface.  The script converts the image file “fpcscreen” (in a QNX proprietary image format) to a PNG file.  SendConvImage uses the stompputfile script found in /usr/local/bin.

The Dashboard web application receives the file and stores the image on the server.

The ScreenCapture table defines the portion of the main screen that will be captured, how often, where the file will be written, and if existing files should be overwritten.  Always use only row 1 of the table.  Row 0 is not active.

| ScreenCapture Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a descriptive name. The default is ‘Screen Capture’. |
| Enable | Y/N | Use this field to manually enable or disable screen capture. This can be handy to stop the screen capture temporarily without losing the configuration. N = No screen capture is performed. Y = The configuration is active. Make sure the Batches entry is active also. |
| OverWrite | Y/N | Use this field to determine if the output file should be overwritten or not. The SendConvImage script is usually used to do something with the screen capture file. N = If the image file exists, it will not be replaced with the latest capture. It is best to use this so the file is not replaced while the SendConvImage script is working with it. SendConvImage will remove it when it is done. Y = The latest capture will write the output file if it does not exist or overwrite it if it does. |
| UpdateInterval | Int | Set the interval between screen capture events, in seconds. It is best to set this to an interval of at least 30 seconds. Set the Batches_HowOften interval for SendConvImage to the same value. |
| StartPageX | Int | Set the X-Y coordinates of the upper-left (StartPage) and lower-right (EndPage) corners of the rectangular area to be captured, in pixels. The origin (0, 0) is at the upper left of the main screen. X increases toward the right. Y increases toward the bottom. When using PhMap, set the StartPage values to (0, 0) as they refer to the map coordinates, not the entire conveyor display. A good starting point for the EndPage coordinates might be (1200, 1200), but some experimentation may be needed to get an acceptable image. |
| StartPageY | Int |  |
| EndPageX | Int |  |
| EndPageY | Int |  |
| FileName | String | Set the file name for the output file. The default is ‘fpcscreen’, which is written in the RUN directory. The SendConvImage script expects the file to have this name and be in this location. You can specify a path, if you wish, following the normal UNIX path rules, so if you start with ‘/’ it will be an absolute path. If you omit the leading ‘/’, the path is relative to RUN. QNX 6.3 produces the file in ‘phs’ format – a proprietary QNX format. QNX 6.4 and 6.5 produces a ‘bmp’ file. The SendConvImage script uses the OS version to determine if it should convert the file to bmp or use it as-is. |
| pError | Int | The screen capture module may write error code numbers here to help diagnose issues: 1000 – Enable = N 1001 – No FileName given 1002 – System error creating device 1003 – FileName exists and OverWrite = N 1004 – System error when opening the screen capture device 1005 – System error when starting the screen capture device 1006 – The main screen widget is not defined 1007 – Can’t find the directory 1008 – Can’t rename FileName from ‘*.tmp’ to the original name 1009 – Can’t change the access mode of FileName 1010 thru 1041 – Screen print page configuration errors |
| pTimeStamp | TimeStamp | Set by the software. The timestamp of the last screen capture |
| pCounter | Int | A running count of screen captures taken. Rolls over at 9,999. |

# Dashboard Troubleshooting

In addition to the troubleshooting in section 12 Troubleshooting check these dashboard-specific items if the conveyor image does not update correctly:

- Verify the batch jobs are creating the files:

ls /SortPlus/RUN/fpcimage.latest.png

- Try running each Batches entry manually and checking for errors:

Turn off batches “Print Main Screen” and “SendConvImage” by removing the Process entries.  Navigate to Administrator Utilities | Set Security Levels and use Alt-K to run “Print Main Screen” manually.  Verify the file /SortPlus/RUN/fpcscreen is created.

Run the SendConvImage manually.  At the command line, run the command /SortPlus/RUN/SendConvImage.  Verify the file /SortPlus/RUN/fpcimage.latest.png is created.

Edit the SendConvImage script and find the LOGGING line near the top.  Turn on logging by setting LOGGING=Y.  Log information will be written to /tmp/SendConvImage.log.  Be sure to turn logging off again once the issue is resolved.

# Troubleshooting

- Verify the interfaces are defined and connecting.  Check the Status and Error columns of the Machine table.  If errors are found, check the SRV1 log for messages associated with WCS_OUTBOUND, WCS_IN_TOPIC, and WCS_IN_QUEUE.

  - The Machine table – Machine_Pid column is set to -1 if a connection has not been established.  Setting this column to -1 manually will reset the connection.

  - The log file may show that a Username and Password are required for the ActiveMQ interface.  If so, they can be added to the Machine table.  If ActiveMQ is configured as it should be, no Username or Password will be needed.

    - Check access to the IP:  Ping WCS_OUTBOUND, WCS_IN_TOPIC, and WCS_IN_QUEUE.  Correct errors in /etc/hosts, if needed.

- Verify Event Messages are being sent.

  - Open the MsgWCS table and verify that new entries are being added.  The cursor will be positioned on the latest row when the table is opened.  The DbTime column contains a date/time stamp.

  - Verify the TxStat column is set to “Sent”.  If the column is not sent to “Sent”, the problem will most likely be found in the Machine or MsgMap tables, or the /etc/hosts file entry for the WCS_OUTBOUND connection.

- ActiveMQ Debugging.  Additional information for debugging ActiveMQ connections may be found in the Remote Authorization document.

| Blank Table |  |  |
| --- | --- | --- |
| Field | Type | Description |