| # FortnaPlus Control End of Wave Module |  |
| --- | --- |
| Document Revision: 1.5 |  |
| Date: 22-Jan-2021 |  |

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 14-Oct-2011 | T R Kremser | Document created |
| 1.1 | 20-Oct-2011 | T R Kremser | Add SawLane_EndOfWave_Error feature |
| 1.2 | 21-Jan-2013 | T R Kremser | Add “Release All” feature |
| 1.3 | 06-Oct-2014 | T R Kremser | Add security on Enable, Disable, Set EndOfWave buttons. This was added with revision 22280, in association with Feature #12569 |
| 1.4 | 27-Jan-2015 | B C Conklin | Added pushbuttons, lights and Aatomatic release features |
| 1.5 | 22-Jan-2021 | T R Kremser | Refresh and general update |

Table of Contents

1.	Introduction	4

2.	Configuration	4

3.	Interface	6

4.	Security	9

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Introduction

The FortnaPlus Control (FPC) End-of-Wave (EOW) module adds functionality to the existing Sawtooth Merge module.  It is intended to be used with tote transport systems where special EOW totes are inserted in the flow to physically mark the division between product waves on the conveyor system.  EOW totes are used in conjunction with additional photoeyes mounted on each of the sawtooth lanes.  The EOW totes are constructed with the addition of reflector material positioned so it will interact with the EOW photoeyes.  The EOW photoeyes are mounted without a reflector and configured in an inverted fashion, so they will signal the presence only of the EOW totes, but not regular totes.

# Configuration

The added functionality is configured using the SawMerge and SawLane tables.

SawMerge Table

(Configuration Menu | Merge Utilities | Saw Tooth Conf |SawMerge Table)

- EOW_RelAllPB – Popup to select the pushbutton that can be used to release a wave.  The logic will only release the wave if all lanes have detected an “End of Wave” tote.

- EOW_FndAllLt – Popup to select the beacon light that will turn ON when all lanes have detected an “End of Wave” tote.

- EOW_AutoRel – Set this to N if the wave will be released manually using the Sawtooth Merge Control Screen (Operator Menu | Saw Merge Control), or by the pushbutton selected in EOW_RelAllPB.  Set to Y if the wave will be released automatically when the EOW tote has been detected at all lanes.

SawLane Table

(Configuration Menu | Merge Utilities | Saw Tooth Conf |SawLane Table)

- EndOfWave_PE – Popup to the Conveyor table.  Select the EOW photoeye for each lane.

- EOW_PB – Popup to select a pushbutton that will put the lane into an “End of Wave” state.  This will manually stop the lane as though an EOW tote has been detected.

- EOW_PB_Toggle – Set to Y to allow the EOW_PB pushbutton to “undo” the “End of Wave” state with a second push of the button.  It will also cancel the “End of Wave” state resulting from the presence of an EOW tote.  If set to N, the end-of-wave state for the lane can only be canceled by disabling and re-enabling the lane, or by releasing the wave.  Note that when a lane is re-enabled, it will release!

- EOW_EnablePB – Popup to select a pushbutton that will toggle the Enable/Disable state of the lane.  When disabled, the lane will stop feeding into the merge.

- EOW_Light – Popup to select a beacon light that will turn ON when the lane is in an “End of Wave” state.

- EOW_EnableLt – Popup to select a beacon light that will indicate if the lane is Enabled.  If the Lane is in an Enabled state it can feed into the merge.

- EOW_LaneNotReq – Set to N if an EOW tote must be detected to allow the lane to release.  Set to Y if an EOW tote is not required for this lane.

- EndOfWave_Error – Popup to the Errors table.  Once an entry is made for each EndOfWave_PE, this field will automatically be populated with Errors table entries when FPC is re-started.  These Errors entries will be created for this purpose as warnings to indicate when a sawtooth lane is in either the Disabled or EndOfWave state.

Three additional columns are controlled by the software:

- pEOW_State holds a number representing the current state of the EOW logic.

- pEOW_Status shows text describing the current state or error messages that should aid in correcting configuration problems.  This information is presented to the operator as text in the Sawtooth Merge Control Screen, described below.

- pEOW_Latch is used by the software to track whether an EOW tote has been detected.

# Interface

The Sawtooth Merge Control Screen is the graphical interface that the operator uses to control the release of waves, correct errors, or override automated behavior.  It can be accessed in two places:

- Operator Menu | Saw Merge Control

- Configuration Menu | Merge Utilities | Saw Tooth Conf | Saw Merge Control

The Sawtooth Merge Control Screen

The fields of the Sawtooth Merge Control Screen are described as follows:

- Sawtooth Selection Box:  In the upper-right corner of the screen, a pull-down selection box allows selection of the Sawtooth to be displayed.  The names shown are those defined in the SawMerge table.

- RELEASE WAVE button:  In the upper-left corner of the screen, a button used to release all lanes on the currently-displayed sawtooth simultaneously.  When all configured lanes are in an “End of Wave” state, the RELEASE WAVE button will become active, signified by its background color changing to green.  Clicking the button while in this state will cause all the configured lanes to release.  The status field for each lane will indicate “Releasing” until the reflector on the EOW tote clears the EndOfWave_PE photoeye, then the status will return to Enabled.  Access to this button is controlled by security settings.  See section 4, Security, below.

- RELEASE ALL button:  At the top-center of the screen, a button used to release all lanes on all sawtooths (sawteeth?) simultaneously.  When all configured lanes on all sawtooths are in an “End of Wave” state, the RELEASE ALL button will become active, signified by its background color changing to green.  Clicking the button while in this state will cause all the configured lanes on all sawtooths to release.  The status field for each lane will indicate “Releasing” until the reflector on the EOW tote clears the EndOfWave_PE photoeye, then the status will return to Enabled.  This will occur on all sawtooths, including those that are not currently displayed.  Access to this button is controlled by security settings.  See section 4, Security, below.

- Saw Lane Name:  In the tabular portion of the screen, the leftmost column will show the names of the lanes in the selected sawtooth, as defined in table SawLane.  This is a read-only field.  The foreground and background colors of the text will change to emphasize the current state of that lane.  See the table, below, for the meaning of the colors.

- Saw Lane Status:  In the tabular portion of the screen, the second column shows a status message explaining the current state of the lane or an error message to aid in diagnosing configuration errors.  See the table, below, for the meaning of the colors.

- Lane Enable:  In the tabular portion of the screen, the third column is an Enable button for each lane.  Clicking the button will cause the lane to transition to the Enabled state, even if an EOW tote is present.  This can be used to release an EOW tote that has been placed on the conveyor erroneously.  The Lane Status field will momentarily show “Enabling” until the EOW tote moves clear of the EOW photoeye.  Access to this button is controlled by security settings.  See section 4, Security, below.

- Lane Disable:  In the tabular portion of the screen, the fourth column is a Disable button for each lane.  Clicking the button will cause the lane to be disabled.  The lane will not run.  The “RELEASE WAVE” button will not turn green if any defined lanes are in the Disabled state.  Access to this button is controlled by security settings.  See section 4, Security, below.

- Lane Set EndOfWave:  In the tabular portion of the screen, the rightmost column is a button labeled “Set EndOfWave” for each lane.  Clicking the button will cause the lane to be set immediately to the end of wave state, regardless of the state of the EndOfWave_PE photoeye.  Once in that state, it will stay until the wave is released, or Enable or Disable is selected.  This may be used to manually force an end-of-wave state on a lane even if no EOW tote is present.  This will cause the lane to stop feeding when the next container arrives at the presence eye.  Access to this button is controlled by security settings.  See section 4, Security, below.

- Help button:  The Help button in the lower-left corner of the screen will open a window providing a document describing the use of the screen.

- Message Window:  The text box in the bottom-middle of the screen will show messages about the operation of the EOW feature.

  - “Wave Released” – when “RELEASE WAVE” has been pressed.

  - “Waiting for End of Wave” – if all active lanes are not in an EOW state when “RELEASE WAVE” is pressed.

- Cancel button:  The Cancel button in the lower-right corner of the screen will close the window.  The window close control (X) at the upper right corner of the window border has the same effect.

States, Colors and Messages State Number; State Name; Color; Messages. 0; EOW_NONE; Darker Grey on Bright White; “No EndOfWave PE” “No Lane Name”. 1; EOW_ENABLED; Green on Bright White; “Enabled”. 2; EOW_SETENABLE; Green on White; “Enabling”. 3; EOW_DISABLED; Red on White; “Disabled”. 4; EOW_ENDOFWAVE; Black on Yellow; “End of Wave”. 5; EOW_SETEOW; Black on Yellow; “Set End of Wave”. 6; EOW_RELEASE; Black on Yellow; “Releasing”. Detailed meaning of the messages: “No EndOfWave PE”: Configuration error message set when SawLane field EndOfWave_PE has no part selected. “No Lane Name”: Configuration error message set when a SawLane row is associated with a Sawtooth (from table SawMerge), but no name has been set for the lane. “Enabled”: Set upon initialization at program startup, and after the EOW tote clears the EndOfWave_PE photoeye after the lane has finished releasing. Also, after the EOW tote clears the photoeye when the Enable button has been pressed. “Enabling”: Set when the Enable button for a lane has been pressed but the EOW photoeye is still blocked. The state will transition to Enabled once the EOW tote has cleared the photoeye. “Disabled”: Set when the Disable button for the lane is pressed. Causes the lane to stop. “End of Wave”: Set when the EOW tote is detected, or after the “Set End of Wave” button is pressed. Note that this state will persist, even if the end-of-wave tote is removed from the photoeye. The lane can be forced from this state using the Enable or Disable buttons. Causes the lane to stop. “Set End of Wave”: Set when the “Set End of Wave” button for the lane is pressed. The state will transition to “End of Wave”. “Releasing”: Set when the RELEASE WAVE button is pressed while all lanes are in the EndOfWave state, but the End-of-Wave tote is still blocking the photoeye. Will transition to Enabled once the End-of-Wave tote clears the photoeye.

# Security

There are five independent security controls associated with the Sawtooth Merge Control Screen.

Security access control is configured in table procnames (Administration Menu | Set Security Levels)

- Access to the entire screen is controlled via "SawMergeCtrl"

- Access to the "RELEASE WAVE" and "RELEASE ALL" buttons at the top of the screen is controlled via "SawMergeCtrl Security".

Access to each of the three sets of buttons associated with the defined lanes is controlled via:

- "SawMergeEnable Security" controls access to all the "Enable" buttons.

- "SawMergeDisable Security" controls access to all the "Disable" buttons.

- "SawMergeSet EOW Security" controls access to all the "Set EndOfWave" buttons.

The default Security_Level value of zero allows access to all these items.

To restrict access, set a non-zero value by double-clicking the Security_Level field for the item of interest.  Check the boxes for the roles that should be allowed access.  If there is any intersection of the roles set for the item to be accessed and the user's roles, the item will be accessible.  (For comparison, you can view each user’s security roles settings in table password (Administration Menu | Set User Passwords) by double-clicking the LEVEL field).

If an item is not accessible, it will appear as "ghosted", and the user will not be able to click on it.

The "RELEASE WAVE" and "RELEASE ALL" button's color reveals the wave's status – turning green when the wave is ready to be released, regardless of the user's ability to click the button.

The "Enable", "Disable", and "Set EndOfWave" buttons will be green, red, and yellow, respectively, if they are accessible, and grey otherwise.