| # FortnaPlus Control Energy Management |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 15-Jan-2021 |  |

Revision; Date; Author; Revision Summary. 1.0; 10-Jun-2001; B C Conklin; New document. 2.0; 27-Mar-2015; B C Conklin; Added ZoneRUN column. 2.1; 15-Jan-2021; T R Kremser; Updates and general refresh.

Table of Contents

1.	The Energy Management Module	4

2.	Batches Configuration	4

3.	Table EnergyZone	5

4.	Table EnergyTrigger	6

5.	Energy Menu Commands	6

6.	Example 1 – Conveyor Segment	7

7.	Example 2 – Sorter Spur	7

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# The Energy Management Module

The energy management module allows zones (sections of conveyor) to go to sleep to save power when no product has been seen for a period of time.

Energy management events can be triggered by other software modules or by inputs from hardware devices.  The configuration is defined using two tables, EnergyZone and EnergyTrigger, which are found in the Energy Menu, along with related control actions:

The Energy Menu can be found under Configuration Menu -> Energy Menu.

# Batches Configuration

The energy management module must be activated from the Batches table:

| File_Name | Description | HowOften | Increment |
| --- | --- | --- | --- |
| Energy Management | Wake Zones | 1 | INVALID |

| Type | Process | InternalProc | InternalProcCtrl |
| --- | --- | --- | --- |
| Count | CTRL |  | EnergyProcess |

| Machine | All Machines | DoFile |
| --- | --- | --- |
| <AC Name> | N | N |

Leave the other columns at their default values.

# Table EnergyZone

The EnergyZone table defines motors (typically) that are controlled based on one or more input sensors and a timer.  The input sensors are defined in table EnergyTrigger, described in section 4.  If none of the associated input sensors change state before the timer expires, the motor will be stopped in a sleep state.  When any of the associated input sensors change state while the motor is stopped, the motor will be restarted.  If a motor defined here is part of a motor chain, the motor chain will honor the motor’s energy management state and stop or start upstream sections accordingly.

| EnergyZone Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for this energy management zone. |
| pMessage | String | Updated by the software to indicate errors or events associated with the entry. Indicates the current state of the zone. - Zone Running - Zone Asleep - Zone Waking Up - Zone Stopped (ZoneRUN signal is OFF) |
| ZoneRUN | PopUp | Select the signal that will be used to allow the zone to run. This prevents the energy zone from executing a start sequence (activating the horn) when it is not running. If the signal is OFF the zone will be put into a sleep state. Typically, this will stop the zone. The energy zone timer (GoTimer) will be reset when the ZoneRUN signal transitions ON. |
| GoTimer | PopUp | Select a timer that defines the wait time before stopping the zone. The timer resets when the associated sensors defined in the EnergyTrigger table change state. |
| pGoFlag | Int | This internal flag indicates that an item has been detected. GoTimer will be reset. |
| StopZoneOutput | PopUp | Select the signal that will be used to stop the zone. Note: Make sure that at least one sensor for an energy zone is outside the sections that will be stopped so system activity can restart the zone. |
| StopAction | PopUp | Select the desired action to stop the zone for the sleep state. |
| StopBit | Int | If a StopAction of Enable/Disable is selected, choose the bit to use. |
| pStopFlag | Int | This internal flag indicates the zone is stopped in a sleep state. |
| HornRecord | PopUp | If desired, select the record from table horns that will be activated when the zone is waking up. |
| pStatus | Int | Internal flag set by the software to indicate the state of the zone. - Zone is awake - Zone is asleep - Zone is waking up (firing the warning horn) |

# Table EnergyTrigger

The EnergyTrigger table defines input signals (inputs to the energy management module) that will be monitored to determine when to stop or start an energy zone.  Multiple inputs may be defined to control a single zone.  The inputs may take the form of a IO signal from a photoeye or other device, or they may be defined as a sorter tracking event.  Sorter tracking events are defined using tracked positions from the Outpoints table and destination information of cartons as they pass those points.

| EnergyTrigger Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for the energy management trigger. |
| ZoneRecord | PopUp | Select the record from the EnergyZone table that will be activated when the trigger event occurs. |
| TriggerInput | PopUp | Select the input sensor that will detect activity. Note: Make sure that at least one sensor for an energy zone is outside the sections that will be stopped so system activity can restart the zone. |
| EventType | PopUp | Select the signal type that will reset the sleep timer. - ON IO – If the signal is ON the timer will be reset. - OFF IO – If the signal is OFF the timer will be reset. - Trans ON – When the signal transitions to the ON state the timer will be reset. - Trans OFF – When the signal transitions to the OFF state the timer will be reset. - Divert – When a carton reaches the point on the sorter defined by DivertWindow and DivertOutpoint the timer will be reset. |
| DivertWindow | Int | Enter the number of ticks upstream of the divert point defined in DivertOutpoint where the timer will be reset. EventType must be set to Divert. |
| DivertOutpoint | PopUp | Select the outpoint that is associated with this energy zone for a sorter. Typically, this is the outpoint used to divert to the spur. EventType must be set to Divert. |
| pLastState | Int | Used to track internally the last transition of TriggerInput. |

# Energy Menu Commands

Hide Internal Columns and Show Internal Columns act on the “p” columns in tables EnergyZone and EnergyTrigger.  Columns with names that begin with a lower-case p contain data used internally by the module.  Hiding them does not interfere with their function, but eliminates visual clutter while working on the configuration.  You may wish to show those columns to aid in debugging.

Note:  Running the Hide Internal Columns or Show Internal Columns commands will cause all open tables to close!

Wake All Zones starts all defined zones.  It does this by setting the EnergyZone_pGoFlag for all records linked to entries in EnergyTrigger.

# Example 1 – Conveyor Segment

A motor, M101, controlling a section of conveyor, will be stopped if a photoeye sensor in the upstream section has not detected an item in the last 60 seconds.

You will need a motor part:  M101, a sensor part: PE100, and a timer: tmEM101.

Open the EnergyZone table.  Choose the first blank row, starting with row 1.  Enter a descriptive name in the Name column.  Select tmEM101 in the Go Timer column.  Select M101 in the Stop Zone Output column.  Choose Disable in StopAction and select an appropriate StopBit (0 is probably fine).  Select an appropriate entry in HornRecord.

Set the timer Preset_Value to 60 seconds in the timemenu table.

Open the EnergyTrigger table.  Choose the first blank row, starting with row 1.  Enter a descriptive name in Name column.  Select the row from EnergyZone created above in the ZoneRecord column.  Select PE100 in the TriggerInput column.  Set EventType to Trans ON.

M101 will continue to run as long as PE100 is triggered at least once every 60 seconds.

M101 will stop when PE100 has not been triggered for 60 seconds.

M101 will restart when PE100 is triggered.  The horn will sound before the motor starts.

# Example 2 – Sorter Spur

A motor, M201, controlling a spur lane on a sorter, will be stopped if nothing has been diverted to that lane in the last 60 seconds.

You will need a motor part:  M201, the lane divert outpoint:  LANE201, and a timer: tmEM201.

Open the EnergyZone table.  Choose the first blank row, starting with row 1.  Enter a descriptive name in the Name column.  Select tmEM201 in the Go Timer column.  Select M201 in the Stop Zone Output column.  Choose Disable in StopAction and select an appropriate StopBit (0 is probably fine).  Do not define an entry in HornRecord.

Set the timer Preset_Value to 60 seconds in the timemenu table.

Open the EnergyTrigger table.  Choose the first blank row, starting with row 1.  Enter a descriptive name in Name column.  Select the row from EnergyZone created above in the ZoneRecord column.  Leave the TriggerInput column at INVALID.  Set EventType to Divert.  Set DivertWindow to 12 ticks.  Set DivertOutpoint to LANE201 from the Outpoints table.

M201 will continue to run normally as long as a carton is diverted to the spur at least once every 60 seconds.

M201 will stop when no cartons destined for LANE201 have passed the outpoint for 60 seconds (actually, the DivertWindow point, 12 ticks prior to the outpoint).

M201 will restart when a carton destined for the lane reaches the point 12 ticks prior to the LANE201 outpoint.  No horn will sound.