# FortnaPlus Control Errors Configuration

Document Revision: 1.2

Date: 25-Aug-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 06-Jul-2012 | B C Conklin | New document |
| 1.1 | 22-Jul-2015 | B C Conklin |  |
| 1.2 | 25-Aug-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	Errors Configuration in FortnaPlus	5

2.	Errors Table	5

3.	RESYNC EJF	8

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

IO – Input / Output.  A reference to electrical interface hardware used to control and receive signals from conveyor devices.

UPS – Uninterruptible Power Supply

# Errors Configuration in FortnaPlus

Errors definitions in FortnaPlus are intended to be common across all ACs at a site, so, when properly configured, the Errors table (and the underlying Errors.asc file) will be identical on all ACs.

The status of the defined errors is shared across the network, meaning that when the AC that owns the error declares it to be active, that status is shared with the other ACs to present a cohesive picture of the state of the system, no matter which AC you consult.

As time goes by, modifications to the system can lead to divergent Errors table definitions, but usually the AC that owns a newly-added feature will have its own Errors defined correctly.

The errors defined in table Errors are most visible through the F10-0 <Current Errors>, F10-1 <Error Status Log> and F10-2 <Error Summary> screens, but they also result in records sent to the Error_Log table in MySQL and messages displayed through the EV10 interface.

Many modules will generate error definitions automatically.  For example, you will find EStop_Error has links to auto-generated Errors records.  The records are created with names that mention the associated parts, but you can always edit the name to make it more descriptive or more meaningful and relevant for the customer.

# Errors Table

In FPC, errors can be declared by many of the modules, but the definition of all errors is made in table Errors.

| Blank Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ErrorName | String | A unique name for this error. Many modules will automatically generate records in this table and will follow naming conventions that help these names be unique and relevant. You can alter the name to make it more meaningful to the customer, though you should try to preserve references to part names or anything that may help with troubleshooting. Use the Description field to create a longer name, if needed. Any changes you make here will appear wherever this record is referenced. |
| Status | Int | Set by the software to indicate the current state of this error. 0 = This error is not occurring currently 1 = This error is active. See SetStatus. |
| Owner | PopUp | Indicates the AC that controls the IO responsible for triggering this error. |
| Description | String | Used to describe the error in more detail. Like ErrorName, this field is auto-generated with some descriptive text based on the device involved and the error purpose, like “PE904_F PTS2 LANE IS FULL”. This is the string that will be displayed on the F10-0 <CURRENT ERRORS> screen. You can alter the string to make the message more relevant for the customer. |
| LinkPart | PopUp | Choose the part that will define the relevant spot on the conveyor display. From the F10-0 <CURRENT ERRORS> screen, Alt-L (l=Locate) will re-center the display on this part to help the customer locate the problem. You must choose these associations manually. |
| TrigIO | PopUp | Many modules reference these records directly and will trigger errors through that connection. This field can be used for errors that must be triggered directly from an IO signal. The signal could be a “real” input or a memory bit. These must be configured manually. WARNING: Do not select a part here if this record is referenced by a module! A common mistake is to define an Estop part here. The Estop module has a link to the record, so no entry is needed here. If one is defined, one clear and one set message will be sent to EV10 for each pass through the CTRL process while the error is occurring! The display in FPC will appear to be correct, however. |
| InvertIO | Y/N | Set the “sense” of the TrigIO signal: N = The error is declared when TrigIO in ON. Y = The error is declared when TrigIO is OFF |
| ScrnIO | PopUp | Choose an IO signal (usually a memory bit) that will be set ON when this error is active. The Description string for this error is also used to set the IO signal’s Motor field. This could be used to have the error Description appear as the text of an on-screen button. If multiple Errors records reference the same IO signal, the Description from the error that has been occurring the longest will be used. To use this feature: Create a memory bit. Set its Process to GUI. Set its part Type to TITLE or TEXT. (TITLE will appear on all zoom levels in the same font size, TEXT will appear only on the zoom levels defined in table TextFontZoom but a different font size can be selected for each zoom level). Set Default Colors to something appropriate, like one with Colorset On = “Conv Flash” and Colorset Off = “Estops”. You may need to create what you need. |
| Suggestion | PopUp | Choose an item from the list. It will be presented to the user when the error occurs. If you can’t find something suitable in the list, you can add items. Double right click the field to open table ErrSugt for editing. Do not modify existing items, as they are used by automatically generated errors. |
| Severity | PopUp | Choose an item from the list. It determines the color scheme for the status bar on the main screen while the error is occurring. This is a popup to table Errsever. |
| Trigger | PopUp | Not used. |
| SetStatus | Int | Set by the software to indicate the current state of this error. You can also use this field to manually set or clear an error. If the error is active, the software will immediately restore it to the state determined by the configuration. If the error is no longer attached to an active configuration, it will stay in the state you have manually chosen. This can be helpful for diagnostic purposes and can help you find error definitions that are no longer in use. Note that the Status field shows the current state of the error and will not accept input. 0 = Clear error 1 = Set error |
| Changed | Int | Used by the software in conjunction with SetStatus. |
| Frequency | Int | Set by the software. Measures the rate of occurrence of this error. Used to determine if this error should be marked as “unstable”. |
| Idle | Int | Set by the software. When an error occurs or clears, a 30 second countdown timer will begin. Used to determine if this error should be marked as “unstable”. |
| TurnedOnTIme | Int | Set by the software. A UNIX timestamp integer that indicates the last time this error occurred. It is set to 0 when the error is cleared. |
| Alarm_Category | PopUp | Choose an alarm category from the list. These are used by the EV10 Web UI. |
| WcsOutput | PopUp | If needed, choose an IO output or memory bit. When this error is active, the output will be set according to WcsACTION. |
| WcsACTION | PopUp | Choose the action to take on WcsOutput when this error changes state. |
| WcsBIT | Int | If WcsACTION is Enable or Disable, choose the bit to use. |

# RESYNC EJF

Each AC sends the state of errors, jams, fulls, and fulljams that it owns to all the other ACs as their states change.  If a network interruption should occur, or if an AC is offline, it may fail to receive the change.  The owning AC will have the correct state, but one or more of the others may not agree.  If the item in question does not change state frequently, it could persist in this state for a long time, which is confusing to the client.

To remedy this situation on a human time scale, use the Batch job “RESYNC EJF”.  It should be set to run on all ACs.  It slowly “walks” the Errors, Jamcheck, Fullline and Fulljam tables, sending out the current state of each item it owns to the other ACs.  It will process one line from each table on each pass, eventually synchronizing them all.  With a typical Batch configuration of one pass every two seconds, it will take (depending on table size) about 15 to 30 minutes to refresh all the records.

A record is automatically inserted in table Batches for “RESYNC EJF”, but it is not active by default.  Activate it by setting Machine or by setting “All Machines” = Y.  [As of V11.38.0, this is set by default]

RESYNC EJF has been improved in V11.48.0 to skip over entries it does not own, skip unused entries at the end of the tables, and to also send the Errors data via the WCS_OUTBOUND connection for EV10 synchronization.  Skipping non-owned and unused entries makes the refresh cycle much shorter and more efficient, so the default Batch rate has been changed from every two seconds to every four seconds.  A record from each table will be sent on every pass (it might not send a message when it skips the unused entries at the end of a table).  If Integer Flag DEBUGMACHINE is >= 2, look for “resync_errors” in the log files to monitor progress.  You will also see the WCS messages in the logs, of course.