| # FortnaPlus Control FastIO Configuration |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 17-Nov-2020 |  |

Revision; Date; Author; Revision Summary. 1.0; 10-Apr-2017; B C Conklin; New document. 1.1; 17-Nov-2020; T R Kremser; Updates and general refresh.

Table of Contents

Section 1	4

Section 2	4

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Purpose

The FastIO feature is used to define a limited set of IO words to be sent to another AC at a faster rate than the default of once per second used for general IO synchronization.

The CTRL and SERV processes are used for this interaction, in much the same way as is done for the general IO Synchronization, but can occur as often as each pass of the SERV process, which is once every 51 milliseconds.

In certain circumstances, it may be more convenient to link two ACs in this way to avoid running wires between the IO panels.  Note that the FastIO feature is less reliable than a hard-wired connection, so it must not be used in any application that would adversely affect operator safety.

# Requirements

The IO words to be shared are as defined in table Configio, column Octal_Word and table Conveyor, column IO_Address_Word.

The smallest unit that can be sent is one IO word.  Contiguous blocks of words can be sent.  Multiple blocks of words can be defined.

The IO words are sent from one specific AC (where the configuration resides) to another specific AC – the words are not broadcast to all other ACs as with the general IO synchronization.

The sending AC must own the word to be sent.

The receiving AC must be connected to the network used for QNet and must be online (Offline = N in table Machine).

# Configuration

All the configuration for the FastIO feature is done on the sending AC, in table FastIO.  The receiving AC handles the incoming messages in the same fashion as the general IO updates, so no additional configuration is required there.

Note that LowWord and HighWord may expect either Octal or Decimal values depending on the OCTAL_MODE setting.

The FastIO feature does not require an entry in table Batches.

| FastIO Table |  |  |
| --- | --- | --- |
| Column | Type | Value or Description |
| SrcMachine | Popup | Select the current machine. |
| DestMachine | Popup | Select the destination machine. |
| LowWord | Octal or Decimal Int | Select the starting word in the block of IO to be sent. If the block of IO includes words that are not owned by the SrcMachine, those words will not be sent. |
| HighWord | Octal or Decimal Int | Select the ending word in the block of IO to be sent. If the block of IO includes words that are not owned by the SrcMachine, those words will not be sent. |
| LoopCount | Int | Enter the number of SERV process iterations between IO updates. The integer flag TIMERSERV defines the interval in milliseconds, which is 51 mSec by default. A value of 1 will send updates every 51 mSec. A value of 2 will send updates every 102 mSec, and so on. |
| pStatus | String | This column will show “Found IO” if the configuration is correct and the IO Block includes words owned by the SrcMachine. If errors are detected in the configuration, the messages shown here should help to find them. |
| pCountUp | Int | The pCountUp column is updated by the software to keep track of the process iterations since the last IO update. Changing values indicate the module is active. |