# FortnaPlus Control Fulls, Jams and Full-jams

Document Revision: 2.0

Date: 27-Jan-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | Unknown | B C Conklin | New document |
| 1.1 | 17-Mar-2015 | B C Conklin | Add Fullline_DontFireResponse column |
| 2.0 | 27-Jan-2023 | T R Kremser | Combine the Fulls, Jams and Fulljam documents. |

Table of Contents

Glossary of Terms	4

1.	Overview	5

2.	Terminology	5

3.	Full Detection	6

3.1.	An Alternate Use for the Full Logic	8

4.	Jam Detection	9

4.1.	Automatically Clearing a Jam Error	10

4.2.	Clearing a Jam Error with a Pushbutton	11

4.3.	Requiring a Privileged User to Clear a Jam Error	11

5.	Full-Jams	11

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Overview

Detection and handling of full and jam conditions are very important to the proper operation of conveyor systems.  They use similar methodologies, so they are both described in this document.  Full-jams are a hybrid of the two that may be appropriate for use in certain circumstances, and are also described here.

Fulls, Jams and Full-jams all use photoeyes to detect the presence of containers, and timers to determine when a condition has occurred that requires the line to stop.  Fulls are expected to resolve on their own, so the line will restart automatically once the photoeye is unblocked.  Jams require operator intervention to restart the line, as they are intended to detect situations that could lead to unsafe conditions or to equipment or product damage if the line continues to run or if it is restarted without corrective action.  Full-jams are a way to detect that operator intervention has occurred and allow the line to restart without need to push a button.  Full-jams are used when the conveyor layout makes it difficult for the operator to access an AC to perform the restart.

Each of these control types is configured using its own table.  The configuration defines the sensors and timers needed to detect the condition, and how the conveyor display will present the condition to the operator, including error messages and screen colors / blink patterns.

# Terminology

Each of these control types requires photoeyes and timers.  We name them based on their primary intended use.

Per our naming convention, photoeye names begin with “PE”, are followed by a number associated with the conveyor section or motor, and then a suffix to indicate their intended use.

“Full eyes” are photoeyes intended for use in detecting full conditions.  Names will end with “_F”.  Example:  PE4127_F.

“Jam eyes” are photoeyes intended for use in detecting jams.  Names will end with “_J”.  Example:  PE230_J.

“Full-jam eyes” are photoeyes intended for use in detecting full-jams.  Names may be the same as for full eyes, or they may include “_JF” or “_FJ” within the name.  Example:  PE106_JF_F.

Note:  One physical photoeye could potentially be used for more than one of these functions.  There is no requirement that the sensors follow the naming conventions – any sensor, or any input signal could potentially be used.

Timer names begin with “tm”.  Timer names for fulls and jams will follow this convention, with the rest of the name being the name of the photoeye.  A “full timer” or “jam timer” triggers the condition.  For fulls, a “clear timer” is also required (see below).  Clear timer names will begin with “tmfc”.  Full-jams expand on this with the full-jam timer name beginning with “tmfj” and the clear timer name beginning with “tmfjc”.

To continue the example, the timers for full eye PE4127_F would be tmPE4127_F for the full timer, and tmfcPE4127_F for the full-clear timer.

Our jam example would use tmPE230_J for the jam timer.

Our full-clear example would use tmPE106_JF_F or tmfjPE106_JF_F for the full-jam timer and tmfjcPE106_JF_F for the full-jam clear timer.

IO input and output devices and conveyor parts are displayed in RED when OFF, or GREEN when ON.  Parts displayed with a yellow background are in a forced state.  Use the Insert and Delete keys to force parts ON or OFF, or to clear a force.

Timers are displayed in RED when not running or when running but not expired, or in GREEN while timing.

# Full Detection

Full detection is configured using table Fullline (Configuration Menu | Standard Controls | Full Detection).  It is used to stop upstream conveyor sections when containers block full eyes for too long.  A full timer is started whenever a full eye is blocked.  If the full eye is unblocked before the timer expires, the conveyor continues to run and the full timer resets.  If the full timer expires while the full eye is still blocked, a full condition is raised, and the conveyor section is stopped.

Downstream activity will eventually clear the blockage and allow the container to move.  When the full eye is unblocked, a second timer, the full-clear timer, begins to run.  Once the full-clear timer expires, the conveyor section restarts without intervention.

Full timers show in RED when not done – when nothing is in front of the full eye, or while the full eye is blocked, and the timer is still running.  If it expires (is done), it will show GREEN, and a full condition is declared.

When the full eye is unblocked, the full-clear timer will turn RED as it starts timing.  When it completes, it will turn GREEN and the conveyor section will start.

The full timer should be set to allow the longest container to pass without expiring while the conveyor is running normally.  A good starting point might be 5 seconds.    The full-clear timer should be set to allow sufficient space to be created downstream before starting the conveyor section again.

| Fullline Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Desc | String | Enter a unique name. |
| Sensor_Name | PopUp | Choose the IO input part that will signal a full condition. Usually, a photoeye. |
| Timer_Name | PopUp | Choose the timer that will measure how long Sensor_Name has been blocked. When it has been blocked for the time defined in Timer_Preset, Response IO and FullClearOutput will be set to their OFF states, considering Invert and FullClearACTION. Usually, the upstream conveyor section will be stopped when this occurs. |
| Timer_Preset | Float | Enter the time for Timer_Name. A full condition will be declared if Sensor_Name is ON for this amount of time. |
| Clr_Timer_Name | PopUp | Choose the timer that will determine how long before Response IO and FullClearOutput are restored to their ON states (considering Invert and FullClearACTION) after Sensor_Name goes OFF while in a full condition. |
| Clr_Timer_Preset | Float | Enter the time for Clr_Timer_Name. This is how long to wait after the full condition clears before restarting the upstream conveyor section. |
| Error_Name | PopUp | Choose an error from table Errors that will be raised when a full condition occurs. It will be visible on the F10-1 screen while it is active. |
| Conveyor_Name | PopUp | Choose a conveyor part. It will be rendered in the appropriate color to match its state. Usually, a “P” part is used for maximum visual impact. The part is normally shown in YELLOW for a full condition. |
| DontFireResponse | Y/N | This determines how Response IO will be treated: N = Use both FIRE / UNFIRE and ENABLE / DISABLE. Y = Use only ENABLE / DISABLE. Do not FIRE / UNFIRE. Note that if Response IO is defined as Conveyor_Type = “MOTOR”, it will use only ENABLE / DISABLE regardless of this setting. |
| Response IO | PopUp | Choose the conveyor IO output that will be deactivated when a full condition occurs. Note that if the selected part is defined as Conveyor_Type = “MOTOR”, it will use only ENABLE / DISABLE, not FIRE / UNFIRE. Fulls use bit 15. |
| Invert | Y/N | Invert the sense of the response when a full condition occurs. N = Response IO is deactivated when a full condition occurs. Y = Response IO is activated when a full condition occurs. See the prior two fields. |
| Go Until | PopUp | Optional. Choose an IO input part that will detect the presence of containers. Using this allows the Response IO motor to run until a container is detected by this input. |
| ReSound Horn | PopUp | Optional. A horn selected here will sound each time before the Response IO motor is restarted. If a horn is not needed, choose “N/A”. “No Horn” can cause problems in some older versions. |
| NoGap | Y/N | Not used. |
| Status | Y/N | Set by the software: N = Not in a full condition. Y = A full condition has been declared for this row. |
| Sent | Int | Set by the software to track when the current state of this row has been sent to the other ACs. |
| FullClearOutput | PopUp | Optional. Choose an IO output part that will be activated while in a not full condition. FullClearACTION defines what activated means. |
| FullClearACTION | PopUp | Choose the action to take on FullClearOutput. |
| FullClearBIT | Int | If FullClearACTION is ENABLE or DISABLE, set the bit to use. |

## An Alternate Use for the Full Logic

The Fullline logic can also be used to move containers downstream on a transfer spur.  In this case, we use a brief pulse generated by the transfer logic to run the spur for a longer period.

When a container is diverted at a transfer, the transfer logic creates a brief pulse that kicks the container off the transfer bed.  We can use that pulse in Fullline_Sensor_Name with a Fullline_Timer_Preset that is short enough to trigger the full logic.  The spur motor is used in Fullline_FullClearOutput field to run the spur, with Fullline_FullClearACTION = UNFIRE.  The output will turn ON while the full-clear timer is timing – the opposite of the normal full behavior.  The Clr_Timer_Preset can then be set to run the spur for as long as required.

If additional containers arrive while the full-clear timer is still running, the timer will be reset, so the spur will continue to run until the full interval has elapsed after the last container.

# Jam Detection

Jam detection photoeyes are located at merge points, divert point, curves, or anywhere where it is likely that containers could interfere with each other or get caught on the conveyor hardware and obstruct the path.

A jam timer is used to detect containers that fail to pass the jam eye in the expected time, in the same fashion as for full detection.  For jams, however, the assumption is intervention will be required to clear the path before the conveyor can be restarted.

When a container blocks the jam eye, the jam timer starts running.  Normally, the container will pass the eye before the jam timer completes.  The jam timer resets when the jam eye is unblocked.  If the jam eye remains blocked until the jam timer expires, a jam error is raised, and the jam zone is stopped.

Note:  The jam timer will only run while the conveyor section is being told to run.  If a container stops in front of a jam eye when the jam zone has been stopped for other reasons, a jam will not occur.

When a jam occurs:

- The line stops.

- The error status bar on the FPC screen turns RED, and a jam error is raised.

- The jam eye flashes RED / WHITE on the FPC main conveyor screen.

- The jam condition must be manually cleared.

- A pushbutton or the Start / Stop screen must be used to restart the conveyor.

Jam detection is configured using table Jamcheck (Configuration Menu | Standard Controls | Jam Detection).

| Jamcheck Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Desc | String | Enter a unique name for this jam. |
| Sensor_Name | PopUp | Choose the IO input part that will detect the jam condition. This is usually a photoeye but could be and IO device. |
| Timer_Name | PopUp | Choose the timer that will activate the jam if it runs to completion. |
| Timer_Preset | Float | Set the time, in seconds, for Timer_Name |
| Error_Name | PopUp | Choose the error, from table Errors, to be raised when a jam is detected. |
| Conveyor_Name | PopUp | Choose the part, from table Conveyor, to be drawn in a jammed state when a jam occurs. Normally, that means flashing RED and WHITE. The jam eye (Sensor_Name) part itself will flash automatically. This field is usually used to select a conveyor section “P” part which makes a much greater visual impact. |
| Zone | PopUp | Choose the jam zone, from table Jamzones, that will stop when a jam occurs. |
| Jam_Owner | PopUp | Choose the AC that will control this jam. |
| Jam_State | Int | Set by the software to track the status of this jam. Used to broadcast the state to the other ACs. |
| Motor Under Jam Eye | PopUp | The motor selected here must be running for Timer_Name to start and run. |
| Sent | Int | Set by the software to track when the status of this jam has been sent to the other ACs. |
| ClearErrorWithJam | Y/N | N = The error will remain active when the Sensor_Name photoeye clears. Y = The error will clear when the Sensor_Name photoeye clears. The conveyor must still be restarted manually. |
| ClearJamSignal | PopUp | Optional. Choose an input signal, perhaps from a pushbutton, that will clear the jam. This provides an alternative to restarting the line from the Start/Stop screen. |
| RequireL2Reset | Y/N | N = Anyone can clear the jam and restart the line. Y = A user with maintenance privilege must clear the jam using: Maintenance Menu ¦ Reset Utilities ¦ Jam Reset L2. All active jams are cleared. |
| L2_Jam_State | Int | Used by the software to track the state of a RequireL2Reset jam. |
| L2_ClearJamSignal | PopUp | Optional. Choose an input signal, perhaps from a key switch, that will clear a RequireL2Reset jam. This will clear any active jams, similar to using the Jam Reset L2 menu item. |

## Automatically Clearing a Jam Error

The standard jam configuration activates Jamcheck_Error_Name and holds the error ON until the operator clears the jam and restarts the conveyor.  By setting Jamcheck_ClearErrorWithJam = Y, the error can be made to clear as soon as Jamcheck_Sensor_Name goes OFF.  The conveyor must still be restarted manually.

## Clearing a Jam Error with a Pushbutton

Once the physical obstruction has been removed, the conveyor must be restarted.  This is normally accomplished using the Start / Stop screen.  You can also configure a pushbutton input in Jamcheck_ClearJamSignal to restart the conveyor.

## Requiring a Privileged User to Clear a Jam Error

Normally, any user can restart the conveyor after a jam has been cleared by using the Start / Stop screen or a pushbutton.  If you want to limit the restart ability to users with maintenance privilege, set Jamcheck_RequireL2Reset = Y.  The jam must be reset from Maintenance Menu | Reset Utilities | Jam Reset L2.

You can also use Jamcheck_L2_ClearJamSignal to define an input, probably from a key switch, that will clear a jam.  The privilege, in this case, resides with the key holder.

Note that “L2” was a customer’s term for the elevated privilege required, not a reference to our “Level 2” software layer.

# Full-Jams

The full-jam feature, like the regular jam feature, stops the conveyor when a jam is detected, and raises a jam error, but will automatically restart the conveyor after the full-jam eye is clear, the clear timer completes and the warning horn sounds.

Note:  The full-jam timer will only run while the conveyor section is being told to run.  If a container stops in front of the full-jam eye when the conveyor has been stopped for other reasons, a jam will not occur.  For full-jams, this is based on the state of Response IO, or of Motor Under Jam Eye, if it is defined.

Note:  The full-jam feature must be activated in table Batches.  Use HowOften = 1, Type = Count, Process = CTRL, InternalProcCtrl = FullJam, Machine = <This AC>.

| Fulljam Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this full-jam. |
| Sensor_Name | PopUp | Choose the part that will detect the full jam condition. This is usually a photoeye but could be any IO device. |
| Enabled | Radio | Use this to manually enable or disable this row. |
| Timer_Name | PopUp | Choose the timer that will activate the full-jam if the timer runs to completion. |
| Timer_Preset | Float | Set the time, in seconds, for Timer_Name. |
| Clr_Timer_Name | PopUp | Choose the timer that will determine when the full-jam condition will clear after the Sensor_Name sensor stops detecting the jammed condition. |
| Clr_Timer_Preset | Float | Set the time, in seconds, for Clr_Timer_Name. Once this timer runs to completion, the full-jam error is cleared. |
| Error_Name | PopUp | Choose the error, from table Errors, to be raised when a full-jam is detected. |
| Conveyor_Name | PopUp | Choose the part, from table Conveyor, to be drawn in a jammed state when a full-jam error occurs. Normally, that means flashing red and white. |
| Motor Under Jam Eye | PopUp | Optional. If an IO part is selected here, the Timer_Name timer will not run unless the part is ON (the motor is running). If nothing is selected here (INVALID), the logic still runs based on Response IO. This allows a full-jam to be configured with two separate motors. |
| Response IO | PopUp | Choose the motor that will be disabled when a full-jam event occurs. This is also used to stop the Timer_Name timer when the conveyor is stopped for other reasons. |
| DisableBit | Int | Enter the disable bit number to use to stop the Response IO motor. |
| Invert | Y/N | N = Response IO is disabled when a full-jam is detected and enabled otherwise. Y = Response IO is enabled when a full-jam is detected and disabled otherwise. |
| ReSound Horn | PopUp | If a horn is selected here (from table horns), it will sound before the Response IO motor is started (or re-started). If a horn is not needed, select “N/A”. The “No Horn” option can cause problems in older versions. |
| Status | Y/N | Set by the software to indicate the status of the full-jam. |
| Owner | PopUp | Choose the AC that will control this full jam. |
| Sent | Int | Set by the software to track when the status of this full-jam has been sent to the other ACs. |
| pClrTmDone | Int | Used internally to determine when it is OK to restart after a full-jam has cleared. |