# FortnaPlus Control Gapping Modules

Document Revision: 1.2

Date: 15-Jul-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.1 | 14-Jan-2015 | B C Conklin | Changes for FPC Version 10 |
| 1.2 | 15-Jul-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	5

1.	Gapping Modules	6

1.1.	Gap Terminology	6

1.2.	New Gapper	6

1.3.	Tick Gapper	6

1.4.	Servo Gapper	6

1.5.	Capacity Gapper	7

2.	New Gapper Details	8

2.1.	Table NewGapper	9

2.2.	Setting the Photoeye Location	11

2.3.	New Gapper State Transition Logic	12

3.	Tick Gapper Details	14

3.1.	GoTo Type Tick Gapper	14

3.2.	Table ZprTickGapper	15

3.3.	Table ZprGapperZone	15

4.	Servo Gapper Details	18

4.1.	Configuring Servo Gapper Induct Points	19

4.2.	Configuring the Servo Gapper Update Point	20

4.3.	Table ServoGapper	20

4.4.	Servo Gapper Example Design	25

5.	Capacity Gapper Details	26

5.1.	Table CapacityIO	27

5.2.	Table CapacityBoss	28

5.3.	Table CapacityGapCtrl	28

5.4.	Zipper Merge Example 1	31

5.5.	Zipper Merge Example 2	33

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

CPM – Containers Per Minute

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

VFD – Variable Frequency Drive

# Gapping Modules

  - Gap Terminology

Box 2

Box 1

Gap

Pitch

Example Box – no trailing shoes added

Trailing Shoes – Associated with a Sorter.  Extends the length of a box.

Trailing Shoes – Used to prevent a box from being cut off as the box changes its location due to shoes changing the position of the box.  Also used to extend the size to compensate for “tracking” errors.

  - New Gapper

The New Gapper module is a replacement for the older GapCfg module.  It has been improved to allow control of gapping belts that may also be controlled by other parts of the system, such as merges.  It will also automatically detect shared discharge conveyors (see the NewGapper_NextConvIO field) and will gap correctly even when a merge is switching between lanes.

  - Tick Gapper

The Tick Gapper module is used to create gap based on encoder ticks rather than time.  Encoder-based gapping is best used if the belt speed varies.  A tick gapper will feed containers to a photoeye and will stop and hold until the number of ticks between the prior container and the current container reaches the defined value.

  - Servo Gapper

The Servo Gapper module is best used in applications where timers are not effective, such as conveyor sections where the speed varies based on other control algorithms or in applications where the gap must be more precise.  This module is designed to control two sections of conveyor where the first section consists of a servo-controlled belt and the second a takeaway belt with an encoder.  Each section of conveyor is linked to a shifter.

  - Capacity Gapper

The Capacity Gapper module is used to regulate the flow of containers based on outgoing capacity.  This module builds on the New Gapper, Tick Gapper or Servo Gapper modules.  It adjusts the gapper parameters for those underlying gappers using real-time input data on the current capacity of the system.

A Capacity Gapper might be a good choice for sorters where the recirculation rate can be monitored or Print and Apply lines where the number of available printers may change.

# New Gapper Details

Use the NewGapper and NewGapperStates tables to configure a New Gapper.

This module does not need to be configured in table Batches.

If the gapper does not have a downstream belt, a system run signal may be selected using the NextConvIO field.  Set DontChkNextShare = Y to allow the gapper to operate without checking the state of other gapper records.

If the gapper is dependent on other lanes (as when it is on one of the infeed lanes of a merge), and if the part selected in NextConvIO is the same for each lane, set DontChkNextShare = N.  The logic will continue to gap unless all other lanes with the same NextConvIO are in one of the following states:

- Idle

- Idle Hold

- Gap Other

- Release Wait

Here is a typical layout:

ProductIO

StopOUTPUT
StopACTION
StopBIT

NextConvIO

MinGapTM
MinPitchTM

  - Table NewGapper

New Gappers are defined using table NewGapper.

| NewGapper Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this gapper. |
| ProductIO | PopUp | Choose the photoeye that will detect the presence of containers on the gapper. |
| NextConvIO | PopUp | Choose the motor AUX for the next conveyor segment downstream of the gap belt. |
| DontChkNextShare | Y/N | Use this to determine if any other records in this table use the same NextConvIO part, which will be the case if this gapper is part of a merge that has a gapper on each infeed lane. N = Check the other records. Y = Do NOT check the other records. |
| GapperMode | PopUp | Choose the operating mode: Standard – Both MinGapTM and MinPitchTM must run to completion to create the required gap. MinPitchTM is stopped and reset while in the Idle, Idle Hold and Gap Other states. Pitch Intervals – Containers are released on pitch intervals. MinPitchTM always runs. |
| MinGapTM | PopUp | Choose a timer to use to measure the gap (tail to nose). |
| MinPitchTM | PopUp | Choose a timer to use to measure the pitch (nose to nose). |
| StopOUTPUT | PopUp | Choose the output for the gap belt motor. |
| StopACTION | PopUp | Choose the method to stop the gap belt motor. Keep in mind that only one module can control any output using Fire/Unfire. Usually Mtrchain will control the motors, so use Disable with a unique StopBIT here. |
| StopBIT | Int | If StopACTION is Enable or Disable, set the bit value here. |
| StopFLAG | Int | Not used. |
| pState | PopUp | Set by the software to show the current state-machine state. The states are defined in table NewGapperStates: Not Used – The logic will set this state if the configuration is incomplete and write “Record Not Configured” to the Debug field. ProductIO, NextConvIO, MinGapTM, MinPitchTM, StopOUTPUT and StopACTION are required. Will write “Newly Configured” to Debug as it goes to Idle once everything is configured. Idle – The gap belt runs while waiting for containers. Idle Hold – Was in Idle state and a container appeared, but the downstream motor is not running. Gap belt stops. Will write “NextConvIO turned off” to Debug as this state is set from Idle. Gap Other – This state will occur if a container arrives and the spacing on other lanes does not allow this lane to release. Will write “ProductIO on; Other Lane Spacing” to Debug as this state is set from Idle. Release – Release a container. Messages in Debug upon state changes: “ProductIO turned on” from Idle, “Other Lane(s) Spaced OK” from Gap Other, “NextConvIO turned on” from Release Hold, “Gap Motor not disabled” from Release Wait. Release Wait – This state will occur from the Release state if the gap motor is disabled by another process. Message in Debug upon state change: “Gap Motor is disabled” from Release. Release Hold – Was in Release state but the downstream motor stopped. Will write “NextConvIO turned off” to Debug as the state is set from Release. Gap – A container is being released and a gap is being created. Messages in Debug upon state changes: “ProductIO turned off” from Release, “NextConvIO turned on” from Gap Hold, “Restarting Timers” from Lost Case. Gap Hold – The gapper belt had to stop while gapping because the downstream conveyor stopped. Message in Debug upon state change: “NextConvIO turned off” from Gap. Stop – A container had to stop because the gap timers had not completed. Messages in Debug upon state changes: “ProductIO turned on” from Gap, “NextConvIO turned on” from Stop Hold. Stop Hold – Was in Stop state but the downstream motor stopped. Will write “NextConvIO turned off” to Debug as the state is set from Stop. Lost Case – The gapper PE went OFF unexpectedly while processing a container. To be safe, the next stop is the Gap state (probably on the way to Idle). Message in Debug upon state change: “Box PE is off” from Stop. Error – This state is set if the logic is unable to find the next state after five attempts. If this state is ever set, the gapper will stop and you will have to recover manually by changing the state here to Idle. This should never occur. |
| Debug | String | Set by the software to show messages useful for debugging. |
| LogLevel | Int | Choose the level of detail for the CTRL log files. Search the log for “NewGapper”. Level 0 = No logging Level 1 = State transition logging. Search for “State Change”. Level 2 = Full detail for code debugging. Search for: “DEBUG IO”, “CASE DRIFT”, “DEBUG TM”. Level 3 = Miscellaneous lane spacing messages. Search for “OtherLane”. |

  - Setting the Photoeye Location

The physical location of the photoeye in relation to the belts is critical for proper gapping performance.

Proper PE Design for good performance

Locate the PE close to the end of the belt.  Containers should stop before touching the next belt

Belt 1

Belt 2

Gapping PE

End of Belt

Containers should stop prior to the end of Belt 1 without touching Belt 2.  The photoeye position and the deceleration of the motor can be adjusted to achieve this.  The idea is to prevent containers from being pulled away by Belt 2 once they stop.

The PE must be near the end of Belt 1 so a container just released for gapping will not be held back as the next container arrives at the PE.

Incorrect PE Location

Belt 1

Belt 2

Gapping PE

The PE is located too far upstream on Belt 1.  The container being gapped is held back by Belt 1.

End of Belt

  - New Gapper State Transition Logic

The following flow chart shows the New Gapper logic:

Gap Motor Not Disabled

ReleaseWait

Stop Motor, Hold Pitch, Reset Gap

Other Spacing Ok

Down Stream Started

IdleHold

Stop Motor, Reset Pitch, Reset Gap

Other Spacing Ok

GapOther

Stop Motor, Reset Pitch, Reset Gap

Box Present

Other Spacing Ok

Other Spacing Not Ok

Down Stream Stopped

Idle

Run Motor, Reset Pitch, Reset Gap

Down Stream Started

ReleaseHold

Stop Motor, Hold Pitch, Reset Gap

Down Stream Started

GapHold

Stop Motor, Hold Pitch, Hold Gap

Down Stream Stopped

This Lane Spacing Ok

Stop

Stop Motor, Count Pitch, Count Gap

Down Stream Stopped

This Lane Spacing Ok

This Lane Spacing Not Ok

Gap

Run Motor, Count Pitch, Count Gap

Box Present

Down Stream Stopped

No Box Present

Gap Motor Disabled

Release

Run Motor, Count Pitch, Reset Gap

Down Stream Started

StopHold

Stop Motor, Hold Pitch, Hold Gap

# Tick Gapper Details

Use a Tick Gapper to control accumulation conveyor prior to a merge.  It can create the required gap between each container even if a single accumulation zone holds multiple containers.  The Tick Gapper module can also use multiple accumulation zones as brake sections.  Each zone is released in sequence, moving upstream, after the downstream zone has completed its cycle.  The gap is created based on encoder ticks, which represent distance, instead of time.  This is best for situations where the belt speeds change.

The Tick Gapper module must be activated in table Batches:

Select Process = CTRL and InternalProcCTRL = “Zpr CTRL (CPN)”
Select Process = SORT and InternalProcCTRL = “Zpr SORT (CPN)”

Last Zone Accumulation

Infeed Lane

Induct Photoeye

Brake Meter Belt

Zone 1 Accumulation

Zone 2 Accumulation

Presence Photoeye

  - GoTo Type Tick Gapper

A Tick Gapper using the “GoTo Zone” configuration (table ZprGapperZone) is useful with merge brake-meter belts.  Containers will advance to the ZprTickGapper_PresenceEye and hold until a gap of the required number of ticks is created.

Induct Photoeye

Brake Meter Belt

Brake Belt

Presence Photoeye

  - Table ZprTickGapper

| ZprTickGapper Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this tick gapper. |
| Enabled | Radio | Use this to manually enable and disable this gapper. |
| pStatus | String | Set by the software to indicate errors or events. |
| ZipperLane | PopUp | Choose the associated merge lane from table ZipperLane. |
| TickSource | PopUp | Choose the associated shifter from table Sorters that will generate the ticks for this gapper. |
| PresenceEye | PopUp | Choose the presence photoeye for the accumulation section |
| AccumMtr | PopUp | Choose the signal that indicates that the accumulation section is running. This is used to control timers in the tick gapper module. |
| StopIfDisabled | Y/N | This is no longer used. |
| pLastEncMeter | Int | Set by the software to detect ticks on the brake-meter belt encoder. |

  - Table ZprGapperZone

Table ZprGapperZone is used to define the individual elements of a tick gapper and how they are to behave.

ZoneType “Zone 1” is the zone closest to the brake-meter belt.  Its ZoneTimer and tick count (pTicksLeft) are reset when its photoeye transitions OFF.

ZoneType “Zone 2” and “Zone 3” are used for zones between “Zone 1” and “Last”, moving upstream (they are identical except for the name).  Their timers and tick counts are reset when their photoeyes transition ON.  If more than four zones are needed, ZoneType “Zone 2” or “Zone 3” can be used more than once for the zones between “Zone 1” and “Last”.

ZoneType “Last” is the most upstream zone.  Its timer and tick count are also reset when its photoeye transitions ON.

Each zone will decrement the tick count and begin timing when the tick count and timer of the prior zone (downstream) have completed and the ZoneIO has been set to OFF.

When the photoeye for “Zone 1” detects the first container, all the other zones will stop.  This allows “Zone 1” to feed the first container.  When the first container clears the “Zone 1” photoeye, “Zone 1” will also stop until the required gap has been created.  Once the TickDelay and (optional) ZoneTimer are satisfied, “Zone 1” will start again.  Once the tick count and timer are satisfied for each of the other zones, they will also be allowed to feed.  This creates a cascading release that will help control push-through from upstream accumulation sections.

| ZprGapperZone Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the zone. |
| pStatus | String | The software will use this field to display error or status messages. |
| GapperLane | PopUp | Choose a row from the ZprTickGapper table to link the accumulator zone to a specific tick gapper row. |
| ZoneType | PopUp | Choose the zone type to define how the zone will be controlled. The zones use the state of the ZprTickGapper_PresenceEye to determine their action. Zone 1 – Use for the first zone. Zone 2, Zone 3 – Use for intermediate zones, if needed. Last – Use for the last zone. GoTo Zone – Use for GoTo-type zones. ZprTickGapper_AccumMtr will run until ZprTickGapper_PresenceEye is blocked. |
| ZoneIO | PopUp | Choose an IO output to control the zone. |
| ReleaseAction | PopUp | Choose an action to control ZoneIO. |
| ZoneBit | Int | Set the bit to use if ReleaseAction is Enable or Disable. |
| TickDelay | Int | Enter the number of ticks from the brake-meter belt encoder to delay before releasing this zone. Zero or positive values are acceptable. |
| ZoneTimer | PopUp | If a timer is chosen for this zone, it will be used to delay release of the zone in addition to ticks defined in TickDelay. |
| DwnStreamMtr | PopUp | Optional. Choose a signal that indicates the downstream belt is running. This is only needed for merges where the next belt is not the zipper speed-control belt. Use only with ZoneType = “GoTo Zone”. Stops ZoneIO when ZprTickGapper_PresenceEye is ON and DwnStreamMtr is OFF. Runs ZoneIO when ZprTickGapper_PresenceEye is OFF and DwnStreamMtr is ON. |
| pTicksLeft | Int | Set by the software to track the remaining ticks while creating a gap. |

# Servo Gapper Details

NOTE:  The Servo Gapper module must be activated in table Batches by selecting “Servo Gapper (SORT)” in the InternalProcCTRL column.  Set Process = SORT.

The Servo Gapper module is designed to control two sections of conveyor where the first section uses a servo-controlled belt, and the second section uses a takeaway belt with an encoder.  Each belt is linked to a shifter.

Belt 1
Servo-Controlled Belt with Encoder

Belt 2
Take Away with Encoder

Induct PE

Infeed

Box 2

MinStop

Belt 3

Exit

End of Belt Update Point

Logical Belt 3

(Ticks from Belt 2)

Box 1

The software monitors the shifter associated with the end-of-belt update point (ServoGapper_UpdatePt1).  When a container arrives, it looks upstream and downstream to determine if the gap is correct.  If the gap needs to be adjusted, a new speed will be calculated for Belt 1 to open the gap.

Speed = Max Speed * (Ticks Left / (Ticks Left + Gap Error)) * Factor

Gap Error = (Desired Gap – Gap between boxes)

Box 2

Desired Gap

Ticks Left on Belt 1 (between Belts)

Box 1

Current Gap

Gap Error

The speed for Belt 1 is controlled by three algorithms:

- If the gap must be increased, the speed is reduced to the calculated speed.

- If the gap must be increased, the speed is limited to MaxSpdWithErr.

- Speed changes may be dampened by adjusting the AccelMilliSec parameter to limit the acceleration / deceleration of the servo-controlled belt.

The module may also be run in digital IO mode if true analog servo control is not an option.  Digital mode is activated by setting ServoGapper_SpdControl = N/A.  If time-based encoders are used, the time base is calculated using ServoGapper_FactorMedSpd.

  - Configuring Servo Gapper Induct Points

Set the induct point location for both the physical belt (Belt 1) and the logical belt (Belt 2) at 30 ticks.  Select the logical belt induct point in ServoGapper_InductPt2.

Use a memory bit for the logical belt induct point.  Set ServoGapper_CopyShift2 = Y.  The software will copy the container image from Belt 1 to Belt 2 as it moves through ServoGapper_UpdatePt1 (the end-of-belt update point for Belt 1).

  - Configuring the Servo Gapper Update Point

Disable the Servo Gapper record by setting ServoGapper_Enable = N.

Put containers on the conveyor prior to the gapper one by one.  They will stop when they reach the update point.

Adjust the position of ServoGapper_UpdatePt1 until they stop at the end of Belt 1.

Test with at least five containers of various sizes and weights.

If the deceleration time is longer than the container, the belt will slow down but it will not stop.  The effective container length can be increased by increasing Inpoints_MinLenTicks until the containers stop consistently.

  - Table ServoGapper

Table ServoGapper is found in:  Configuration Menu | Gapping Utilities | Servo Gapper.

This is version 2 of the Servo Gapper module.  It interfaces with Variable-Frequency Drives (VFDs) through a reference to table SpdControl.  Speed references there are indicated as a percentage of maximum, instead of using analog values.  SpdControl converts the percentages to analog values as needed.

| ServoGapper Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this servo gapper. |
| EnableFlag | Y/N | This entry can be manually enabled or disabled using this field. |
| EnableIO | PopUp | Choose an IO part here to automatically enable or disable this entry. Optional. |
| InvrtEnableIO | Y/N | If a part is selected in EnableIO, determine which state will enable this entry. N = This entry will be enabled when EnableIO is ON Y = This entry will be enabled when EnableIO is OFF |
| UpdTimerFlag | Y/N | For use with timer-based encoders. Determines if the timer will be changed to simulate speed changes for Belt 1. N = The timer is not changed. Y = The time is adjusted to simulate speed changes on Belt 1. |
| CopyShift2 | Y/N | Determine if the data from Shifter 1 should be copied to Shifter 2. N = Data is not copied. A physical induct photoeye is needed for Belt 2. Y = Data is copied, but only while Belt 1 is running. This prevents a container from logically entering Belt 2 if it drifts while stopping. Use a memory bit in place of the induct photoeye for Belt 2. |
| pStatus | String | Set by the software to indicate the current state of this entry. Gapping – A gap is being created as the current containers are not far enough apart. Gap Zero – The gap between the containers is currently 0 ticks. Dist Zero – The distance between the container on Belt 1 and UpdatePt1 is 0 ticks. Gap Okay, Max Speed – The gap between containers is correct. Returning to full speed. Gapping Complete – The gap adjustment has been completed. The container will change to a white color on the graphical shifter screen. MinTicks not defined – Configuration error. The MinTicks field requires a value. No Update Pnt – Configuration error. The UpdatePt1 field requires a value. Err: Machine or Process – Configuration error. The Machine and/or Process fields require a value. |
| LogLevel | PopUp | Choose Level 2 to see gap details when Gap Okay, Max Speed status is set. Search for “ServoGapperProcess” in the SORT logs. |
| UpdatePt1 | PopUp | Choose an update point for the end of Belt 1 (the transition between Belt 1 and Belt 2). This update point is associated with Shifter 1. |
| InductPt2 | PopUp | Choose an induct point for Belt 2. This induct point is associated with Shifter 2. |
| Machine | PopUp | Choose the AC that will control this gapper. |
| Process | PopUp | Choose the SORT process for this gapper. |
| MinTicks | Int | Set the minimum number of ticks for the gap. |
| MinStop | Int | Set the minimum number of ticks before the speed is set to zero if MinTicks is not met. This can be used to adjust the stopping point when containers are too close together. Start with a value of 0 and increase only if needed. |
| Factor | Float | Set the ‘gain’ factor to compensate for various physical parameters of the gapper hardware – the inertia, acceleration, and deceleration behavior. A typical value is 0.8. You probably should not use values greater than 1.0! If VFD parameters are changed, this value will likely need to be adjusted, too. Example 1: The space between containers is 6 inches and the desired gap is 12 inches. If Belt 1 slows down and then stops just as the container reaches the transition between belts, decrease the Factor value. Adjusting in increments of 0.05 should be safe. Example 2: The space between containers is 6 inches and the desired gap is 12 inches. If Belt 1 slows down and the gap is corrected before the transition between belts, increase the Factor value. Adjusting in increments of 0.05 should be safe. Example 3: The space between containers is 6 inches and the desired gap is 12 inches. If the belt slows down and the gap is corrected within two inches of the transition between belts, the Factor value is correct. |
| RunIO | PopUp | Choose an IO signal that indicates the conveyor is running. When the signal is OFF the speed will be set to 0 and StopDigIO will be set ON. If set to INVALID, the gapper will assume the conveyor is always running. |
| FastDigIO | PopUp | Choose a digital output signal to tell the hardware to run at the fast speed. This signal will be set ON if the calculated speed is greater than MaxMedSpd. If the motor does not accept a fast speed input, it is OK to leave this at INVALID. MedDigIO and StopDigIO are set OFF when this is ON. |
| MedDigIO | PopUp | Choose a digital output signal to tell the hardware to run at the medium speed. This signal will be set ON if the calculated speed is greater than or equal to MinMedSpd and less than or equal to MaxMedSpd. If the motor does not accept a medium speed input, it is OK to leave this at INVALID. FastDigIO and StopDigIO are set OFF when this is ON. |
| StopDigIO | PopUp | Choose a digital output signal to tell the hardware to stop. FastDigIO and MedDigIO are set OFF when this is ON. |
| MinAnalog | Float | Set the analog value for the minimum running speed. It is OK to set this to 0.0 if the desired minimum speed is zero. |
| MaxAnalog | Float | Set the analog value for the maximum running speed. If a SpdControl record is selected in MatchRecord, this value will be updated by the software based on the current speed of the matching belt. |
| MinMedSpd | Float | Set the minimum speed at which the MedDigIO output will be activated. If the calculated speed falls below this value, Belt 1 will be stopped (StopDigIO will bet set). Only used for digital control. |
| MaxMedSpd | Float | Set the maximum speed at which the MedDigIO output will be activated. If the calculated speed is greater than this value, Belt 1 will run at the fast speed (FastDigIO will be set). Only used for digital control. |
| FactorMedSpd | Float | Only used for digital control. Set the ‘gain’ factor for use while running at medium speed with UpdTimerFlag set. This value will be used to control the timer-based encoder for Belt 1. Observe the same cautions as described for Factor. |
| MaxSpdWithErr | Float | This value will set an upper limit on the speed for Belt 1 while the gap is less than MinTicks. The “Err” in the name of this field is a reference to deviation from the desired gap. |
| SpdControl | PopUp | For servo (analog) control, choose a record from table SpdControl that will be used to control the Belt 1 motor. The Speed Control module works in terms of percent of maximum motor speed, so the speed values in this table must be expressed in percentages when a record has been selected here. The SpdControl configuration converts the percentages to analog values for the motor. Leave this at N/A for digital control and use the FastDigIO, MedDigIO, and StopDigIO outputs to control the motors. |
| AccelMilliSec | Float | Set the acceleration / deceleration time for Belt 1 in milliseconds. A typical value is 250 mSec. NOTE: Check this value after an upgrade from older versions. It may get set to a value you don’t want based on the old value in AccPerScan, below. |
| Enc2Factor | Float | Set this value to the ratio of the encoder settings between Belt 2 and Belt 1 (the brake-meter belt). Example: The encoder for Belt 1 is set to 2 inches per tick. The encoder for Belt 2 is set to 1 inch per tick. Set Enc2Factor = 0.50. Belt 2 gets twice as many ticks for the same distance traveled, so they must be multiplied by 0.5 to get the correct gap. |
| PitchFlag | Y/N | This setting determines whether the gapper works in Gap mode or Pitch mode. N = A gap of MinTicks is created between containers. Y = A pitch (nose to nose) distance of MinTicks will be created. |
| MatchRecord | PopUp | If you choose a record from table SpdControl here, the servo gapper will follow the speed set in SpdControl_OffTargetPercent. A value must be defined in MaxAnalogMtch. |
| MaxAnalogMtch | Float | If a record has been selected in MatchRecord, a value must be defined here to set the maximum allowed speed as a factor. The value from SpdControl_OffTargetPercent is multiplied by this value to determine the value to be set in MaxAnalog. Start with a value of 1.0 for no adjustment. |
| AdjustForDrift | Y/N | The belts will not stop instantly when containers reach UpdatePt1. N = Do not adjust for drift. Y = Adjust for drift by monitoring ticks that occur after the stop command is issued. |
| MaxAdjustDrift | Int | This value is automatically set to 24 ticks when AdjustForDrift is set to Y. You can change the value to limit the maximum amount of adjustment that can be applied. |
| AccPerScan | Float | Set by the software: AccPerScan = 100 * (SORT process interval) / AccelMilliSec. The standard SORT process interval is 5 mSec. In older versions, enter the maximum change in speed allowed for each scan of the IO, following the formula, above. |
| pTicks1 | Int | Set by the software to the ticks calculated for Belt 1. |
| pTicks2 | Int | Set by the software to the ticks calculated for Belt 2. |
| pSpeed1 | Float | The calculated speed for Belt 1. |
| pSpeed2 | Float | The calculated speed for Belt 1 after the minimum and maximum limits have been applied. |
| pSpeed3 | Float | The calculated speed for Belt 1 after acceleration limits have been applied. |
| pTicksLeft | Int | The number of ticks remaining before the container reaches UpdatePt1. |
| pTicksLogical | Int | The number of gap ticks calculated on the logical belt. |
| pDriftAdjust | Int | The number of drift adjustment ticks applied to the gap. |
| pTicksGap | Int | The total number of gap ticks calculated. |
| pErr | Int | The target or minimum gap minus the total gap, in ticks. |
| pBoxPresent | Int | A flag set by the software to indicate a container is moving through UpdatePt1. |
| pPresetValue | Time | Used with timer-based encoders. The calculated time value for the Belt 1 shifter. |
| pCurrBoxIndx | Int | The index into table SortData (Ndx) for the container currently gapping on the brake-meter belt (Belt 1). |

  - Servo Gapper Example Design

Minimum Container Size:  9 inches.

Maximum Container Size:  36 inches.

Belt 1 Gap Required:  18 inches.

Belt 1 Induct PE Location:  0.75 * Maximum Container Size = 27 inches from the beginning of the belt.

Minimum length of Belt 1:  27 inches + 18 inches = 45 inches.

# Capacity Gapper Details

NOTE:  The Capacity Gapper module requires two entries to be activated in table Batches:  Set “CapacityBoss” and “CapGap CTRL” in the InternalProcCTRL column.  Set Process = CTRL for both.

The Capacity Gapper module works with the other gapper modules defined earlier in this document to add a layer of control.  It will dynamically alter the gapping parameters in New Gapper, Tick Gapper or Servo Gapper configurations based on inputs that you define that indicate the current capacity of the system.  As capacity goes offline, the gap is increased to avoid overloading sorter output lanes or PNA printers or merges.

Gapper

Scanner

Printers

In the above diagram of a PNA line, the gap can be adjusted based on the number of printers that are online.

The RateHighOutput field of the RateCount table could be used to control gap based on the rate of arrival on a recirculation line.

Full eyes on the output lanes of a sorter could be used to adjust the gapping at a merge.

Three tables are used to define the capacity of a system, gather the data, and assign gap or pitch settings.

  - Table CapacityIO

Table CapacityIO (Configuration Menu | Gapping Utilities | Capacity I/O) defines the relationship between IO signals and capacity values and serves as the source of information for the CapacityBoss table.  The Capacity values could be in real units such as containers per minute, or arbitrarily assigned integer values relative to other elements of the system.  If using real units, use the same units for all the related elements on the system.

| CapacityIO Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique descriptive name for this device. Blank rows are not processed. |
| Enabled | Radio | Use this field to manually enable or disable this row. If there is a configuration error, the software will automatically disable the row. |
| Capacity | Int | Set this to the defined capacity of the device, in arbitrary units, when the device is ON. |
| pCapacityOut | Int | Set by the software to the current capacity for this device. When the device is ON, it will be set to Capacity. When OFF, it will be set to zero. |
| pActive | Radio | Set by the software. When ON, the device is active, and the capacity is available. |
| I/O | PopUp | Choose an IO part from table Conveyor that indicates if the device is available. |
| InvertI/O | Y/N | Determine how to interpret the state of the I/O part. N -> ON = Available Y -> OFF = Available |
| I/O_2 | PopUp | Choose a second IO part from table Conveyor that indicates if the device is available. If only one input signal is needed, use I/O and leave this set to INVALID. If both are used, both inputs must be in the Available state (after considering the invert settings) for this line to contribute capacity – the two inputs are combined with a logical AND. |
| InvertI/O_2 | Y/N | Determine how to interpret the state of the I/O_2 part. N -> ON = Available Y -> OFF = Available |
| Machine | PopUp | Choose the AC that will control this entry. |
| pCfgError | Radio | Set by the software to indicate if the configuration is valid or not. |
| pCfgMessage | String | A message set by the software to explain the error status of the configuration. |

  - Table CapacityBoss

Table CapacityBoss (Configuration Menu | Gapping Utilities | Capacity Boss) links the CapacityIO records to table CapacityGapCtrl and adds some error checking.  The records here have a one-to-one relationship with the records in table CapacityIO.

| CapacityBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique descriptive name. Blank rows are not processed. |
| pCapacityOut | Int | The software updates this with the value from the associated pCapacityOut field in the CapacityIO table. |
| CapacitySrc | PopUp | Choose the CapacityIO table here. This will serve as the source of the capacity information. |
| CapInRow | Int | Set this to the row number in the CapacityIO table (from CapacitySrc) to use as input. |
| pIntCol | Int | Set by the software to the column number in table CapacityIO (from CapacitySrc) for column pCapacityOut. |
| Machine | PopUp | Choose the AC that will control this entry. |
| pCfgError | Radio | Set by the software to indicate if the configuration is valid or not. |
| pCfgMessage | String | A message set by the software to explain the error status of the configuration. |

  - Table CapacityGapCtrl

Table CapacityGapCtrl (Configuration Menu | Gapping Utilities | Capacity Gap Ctrl) defines how to gather the capacity information from CapacityIO and CapacityBoss, and the action to take as a result – the adjustments to the gapper parameters of the underlying New Gapper, Tick Gapper, or Servo Gapper associated with the configuration.  Any number of capacity “bins” can be defined to create fine control for different run rates.

| CapacityGapCtrl Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique descriptive name for this configuration. Blank rows are not processed. |
| pActive | Radio | Set by the software. Indicates if this row is active. |
| Enabled | Radio | Use this field to manually enable or disable this row. If there is a configuration error, the software will automatically disable the row. |
| Capacity | Int | Enter a value to define a threshold capacity for this record. This is a unitless arbitrary value that will be compared with the pCapacityOut values collected from the CapacityBoss table and summed into pCapacityOut here. Multiple rows for the same gapper (from NewGapper, ServoGapper or TickGapper) can be defined here with different Capacity values to create as many “bins” as required. A row is required with Capacity = 0 for each gapper. The software will select the highest-value row where Capacity is less than or equal to pCapacityOut. Example: Three rows are defined for a gapper with Capacity of 0, 60 and 120. If pCapacityOut is 59 or less, the Capacity = 0 row is chosen. If pCapacityOut is 60 to 119, the Capacity = 60 row is chosen. If pCapacityOut is 120 or greater, the Capacity = 120 row is chosen. |
| pCapacityOut | Int | Set by the software to the sum of pCapacityOut values from the block of rows in table CapacityBoss as defined by SrcStart and SrcEnd. |
| SrcStart | PopUp | Choose an entry from table CapacityBoss to indicate the start of the contiguous block of rows associated with this capacity gapper configuration. This must be a row number less than or equal to the row selected in SrcEnd. |
| SrcEnd | PopUp | Choose an entry from table CapacityBoss to indicate the end of the contiguous block of rows associated with this capacity gapper configuration. This must be a row number greater than or equal to the row selected in SrcStart. |
| Machine | PopUp | Choose the AC that will control this entry. |
| NewGapper | PopUp | Choose a New Gapper from table NewGapper. If an entry is selected here, make sure ServoGapper and TickGapper are set to N/A (row 0). One of the three must be chosen. |
| ServoGapper | PopUp | Choose a Servo Gapper from table ServoGapper. If an entry is selected here, make sure NewGapper and TickGapper are set to N/A (row 0). One of the three must be chosen. |
| TickGapper | PopUp | Choose a Tick Gapper from table ZprGapperZone. If an entry is selected here, make sure NewGapper and ServoGapper are set to N/A (row 0). One of the three must be chosen. |
| GapTime | Time | Set the desired gap time. For use with New Gappers. This value is expected to increase as Capacity decreases. NOTE: Currently, this only works with New Gappers. In the future, support may be added for other time-based gapper types. NOTE: If this value is set to 0 for a Capacity greater than zero, a configuration error will occur. |
| PitchTime | Time | Set the desired pitch time. For use with New Gappers. This value is expected to increase as Capacity decreases. NOTE: Currently, this only works with New Gappers. In the future, support may be added for other time-based gapper types. NOTE: If this value is set to 0 for a Capacity greater than zero, a configuration error will occur. |
| GapTicks | Float | Set the desired gap size, in ticks. For use with Tick Gappers or Servo Gappers. This value is expected to increase as Capacity decreases. NOTE: Currently, this only works with Tick Gappers and Servo Gappers. In the future, support may be added for other gapper types that work with ticks. NOTE: If this value is set to 0 for a Capacity greater than zero, a configuration error will occur. |
| PitchTicks | Float | Set the desired pitch, in ticks. For use with a Servo Gapper. This value is expected to increase as Capacity decreases. NOTE: Currently, this only works with Servo Gappers. In the future, support may be added for other gapper types that work with ticks. NOTE: If this value is set to 0 for a Capacity greater than zero, a configuration error will occur. |
| StopOUTPUT | PopUp | Choose the output for the gap belt motor. All records for a specific gapper are grouped together. If the capacity for the group is zero, this output is set to OFF, otherwise it will be set ON. |
| StopACTION | PopUp | Choose the method to stop the configured belt. Keep in mind that only one module can control any output using RESP_FIRE/RESP_UNFIRE. Usually Mtrchain will control the motors, so use RESP_DISABLE with a unique StopBIT here. Options that are implemented for this module: RESP_FIRE, RESP_UNFIRE, RESP_ENABLE, RESP_DISABLE, RESP_LATCH, RESP_UNLATCH. |
| StopBIT | Int | If StopACTION is RESP_ENABLE or RESP_DISABLE, set the bit value here. |
| pCfgError | Radio | Set by the software to indicate if the configuration is valid or not. This row will not be used if this flag is set. Conditions that will produce an error include: - Duplicate names in the Name column. - Invalid sources for Capacity. - Both a New Gapper and a Servo Gapper have been selected. |
| pCfgMessage | String | A message set by the software to explain the error status of the configuration. |
| pProcessed | Int | Set by the software to keep track of the rows that have been processed on each pass. |

  - Zipper Merge Example 1

Note:  This example seems contrived to me.  I have tried to flesh it out as much as possible.  There are simpler ways to disable merge input lanes entirely when the recirc lane goes full or the rate goes high.  A better example might be to reduce the rate on (but not shut off) the other infeed lanes when the recirculation rate goes high.  TRK

A zipper merge that manages induction of containers to a sorter loop requires protection against gridlock.  The arrival rate of containers on the recirculation lane must be monitored.  If too many containers are arriving at the merge from the recirculation lane, a Capacity Gapper can be used to give the recirculation lane priority and stop the other lanes.  Once the sorter has had a chance to “catch up”, new containers can be introduced.

In this example, a four-lane zipper merge has a total output capacity of 130 containers per minute (CPM).  Lane 1 is the recirculation lane.  We will configure the merge so the rate for the recirculation lane is 20 CPM in normal operation, and 60 CPM (its maximum rate) when the recirculation rate is high.  We will also increase the rate on the recirculation lane if lanes 2, 3 and 4 are empty.

Infeed Rates:

- Lane 1:  20 CPM (gap of 12 feet, normal condition)

- Lane 1:  60 CPM (gap of 4 feet, high recirc condition)

- Lane 2:  20 CPM (gap of 12 feet)

- Lane 3:  60 CPM (gap of 4 feet)

- Lane 4: 30 CPM (gap of 8 feet)

Capacity Rates:

- Recirc rate below threshold:

  - Lane 1 gap:  12 ft

  - Lane 2 gap: 12 ft

  - Lane 3 gap:  4 ft

  - Lane 4 gap:  8 ft

- Recirc rate above threshold:

  - Lane 1 gap:  4 ft

  - Lane 2 disabled

  - Lane 3 disabled

  - Lane 4 disabled

RateCount:

Configure the RateCount table to activate a memory bit when the recirculation rate gets too high:

| Full Detection – (Part 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Machine | Process | NumBuckets | SecPerBucket | HighWater |
| Recirc Rate | CP1 | CTRL | 30 | 1 | 30 |

| Full Detection – (Part 2) |  |  |  |
| --- | --- | --- | --- |
| LowWater | CountIO | RateHighOutput | RateHighAction |
| 15 | Recirc PE | MemRtHi | Fire |

See the document on Rate Count for additional information.

Capacity I/O:

Define the capacity of each lane in table CapacityIO.  We use memory bit MemRtHi from table RateCount and a memory bit for each lane that indicates containers are present.  They are defined with a Fullline entry to latch the presence of containers in each lane (see the next example for something similar).  Logically, NOT I/O AND I/O_2 define whether the entry is active or not, so if the recirc rate is low and containers are present in a lane, the lane’s entry will be active.

| CapacityIO |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enabled | Capacity | I/O | InvertI/O | I/O_2 | InvertI/O_2 | Machine |
| Lane 2 | Y | 20 | MemRtHi | Y | MemL2P | N | CP1 |
| Lane 3 | Y | 60 | MemRtHi | Y | MemL3P | N | CP1 |
| Lane 4 | Y | 30 | MemRtHi | Y | MemL4P | N | CP1 |

Capacity Boss:

Define the source of each capacity value in table CapacityBoss:

| CapacityBoss |  |  |  |
| --- | --- | --- | --- |
| Name | CapacitySrc | CapInRow | Machine |
| Lane 2 | CapacityIO | 1 | CP1 |
| Lane 3 | CapacityIO | 2 | CP1 |
| Lane 4 | CapacityIO | 3 | CP1 |

Capacity Gap Ctrl:

Define the gap for each lane in table CapacityGapCtrl.  This configuration changes the gap when the recirculation rate goes above the threshold.

For the low recirculation rate entry, Capacity = 70 CPM, which is the merge maximum rate of 130 minus the lane maximum rate of 60.  When this setting is active, Lane 1 will run at 20 CPM with a gap of 12 ft (144 inches).  This setting will also be active if the sum of the Capacity values from table CapacityIO is greater than 70 (in other words, when Lane 3 and one other lane or all three lanes are active).

For the high recirculation rate entry, Capacity = 0 CPM.  This entry will be used in all other cases.  Lane 1 will run at 60 CPM with a gap of to 4 ft (48 inches).  The other lanes will be disabled.

| CapacityGapCtrl – (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Capacity | SrcStart | SrcEnd | Machine | New Gapper | Servo Gapper | Tick Gapper |
| Lane 1 – Normal Rate | 70 | Lane 2 | Lane 4 | CP2 | N/A | N/A | Lane 1 |
| Lane 1 – High Rate | 0 | Lane 2 | Lane 4 | CP2 | N/A | N/A | Lane 1 |

| CapacityGapCtrl – (Part 2) |  |  |  |
| --- | --- | --- | --- |
| GapTime | PitchTime | GapTicks | PitchTicks |
| 0 | 0 | 144 | 144 |
| 0 | 0 | 48 | 48 |

  - Zipper Merge Example 2

Zipper merges run more smoothly when the containers are gapped based on instantaneous activity on the infeed lanes.  In this example, a three-lane zipper merge is configured to adjust the gapping for each lane based on the number of lanes that have detected product in the last ten seconds.  The logic increases the gap if all three lanes are active.  If only one or two lanes are active, the gap is decreased so those individual lanes can feed more rapidly, increasing the overall throughput of the merge while in that condition.

Parameters:

- Total merge output capacity:  120 CPM

- Design Container Size:  24 inches

- Target Gap:  12 inches

- Merge Belt Speed:  360 ft/min

- Highest Infeed Lane Rate:  60 CPM

- Encoder Setting:  1 tick per inch

Infeed Rates:

- Lane 1:  60 CPM (gap of 6 feet at 360 ft/min)

- Lane 2:  30 CPM (gap of 12 feet at 360 ft/min)

- Lane 3:  30 CPM (gap of 12 feet at 360 ft/min)

Capacity Rates:

- No lanes active (0 CPM):

  - Lane 1 gap:  6 ft

  - Lane 2 gap:  6 ft

  - Lane 3 gap:  6 ft

- Lane 2 or Lane 3 active (30 CPM):

  - Lane 1 gap:  6 ft

  - Lane 2 gap:  6 ft

  - Lane 3 gap:  6 ft

- Lane 2 and Lane 3 active (60 CPM):

  - Lane 1 gap:  6 ft

  - Lane 2 gap:  6 ft

  - Lane 3 gap:  6 ft

- Lane 1 and Lane 2, or Lane 1 and Lane 3 (90 CPM):

  - Lane 1 gap:  6 ft

  - Lane 2 gap:  6 ft

  - Lane 3 gap:  6 ft

- All lanes active (120 CPM):

  - Lane 1 gap:  6 ft

  - Lane 2 gap:  12 ft

  - Lane 3 gap:  12 ft

Full Detection:

Configure the Full Detection table (Fullline) to activate a memory bit for 10 seconds when a container is present on each lane:

| Full Detection – (Part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Desc | Sensor_Name | Timer_Name | Timer_Preset | Clr_Timer_Name |
| Lane1 Running | Lane1PE | tmLane1PE | 0.01 | tmfcLane1PE |
| Lane2 Running | Lane2PE | tmLane2PE | 0.01 | tmfcLane2PE |
| Lane3 Running | Lane3PE | tmLane3PE | 0.01 | tmfcLane3PE |

| Full Detection – (Part 2) |  |  |  |
| --- | --- | --- | --- |
| Clr_Timer_Preset | FullClear Output | FullClearACTION | FullClearBIT |
| 10.0 | Lane1Running | UnFire | 0 |
| 10.0 | Lane2Running | UnFire | 0 |
| 10.0 | Lane3Running | UnFire | 0 |

Capacity I/O:

Define the capacity of each lane in table CapacityIO:

| CapacityIO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Enabled | Capacity | I/O | Machine |
| Lane 1 | Y | 60 | Lane1Running | CP2 |
| Lane 2 | Y | 30 | Lane2Running | CP2 |
| Lane 3 | Y | 30 | Lane3Running | CP2 |

Capacity Boss:

Define the source of each capacity value in table CapacityBoss:

| CapacityBoss |  |  |  |
| --- | --- | --- | --- |
| Name | CapacitySrc | CapInRow | Machine |
| Lane 1 | CapacityIO | 1 | CP2 |
| Lane 2 | CapacityIO | 2 | CP2 |
| Lane 3 | CapacityIO | 3 | CP2 |

Capacity Gap Ctrl:

Define the gap for each lane in table CapacityGapCtrl.  This configuration changes the gap based on how many lanes have seen product in the last ten seconds.  Note the “StopMem” memory bit is selected, but it is not used to stop the zipper merge.

| CapacityGapCtrl – (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Capacity | SrcStart | SrcEnd | Machine | New Gapper | Servo Gapper | Tick Gapper |
| Lane 1 - 0 | 0 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane1 |
| Lane 1 - 60 | 60 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane1 |
| Lane 1 - 120 | 120 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane1 |
| Lane 2 - 0 | 0 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane2 |
| Lane 2 - 60 | 60 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane2 |
| Lane 2 - 120 | 120 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane2 |
| Lane 3 - 0 | 0 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane3 |
| Lane 3 - 60 | 60 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane3 |
| Lane 3 - 120 | 120 | Lane 1 | Lane 3 | CP2 | N/A | N/A | Lane3 |

| CapacityGapCtrl – (Part 2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| GapTime | PitchTime | GapTicks | PitchTicks | StopOUTPUT | StopACTION | StopBIT |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 144 | 144 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 72 | 72 | StopMem | Fire | 0 |
| 0 | 0 | 144 | 144 | StopMem | Fire | 0 |