# FortnaPlus Control High-Speed Sawtooth Merge Module

Document Revision: 1.2

Date: 25-Aug-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 24-Mar-2014 | B C Conklin | New document |
| 1.1 | 01-Jul-2022 | T R Kremser | Update and general refresh |
| 1.2 | 25-Aug-2022 | T R Kremser | Incorporate the information from the High Speed Sawtooth Merge Test Plan document. |

Table of Contents

Glossary of Terms	4

1.	High-Speed Sawtooth Merge Concept	5

2.	High-Speed Sawtooth Merge Features	7

2.1.	Reservation Re-use	7

2.2.	Opportunistic Feeding	8

3.	High-Speed Sawtooth Merge Design Concerns	9

4.	Merge Shifter Lane Layout	9

5.	Configuring the Shifter	10

6.	Configuring Lane and Merge Induct Points	11

7.	High-Speed Sawtooth Merge Table Relationships	11

8.	The HSSawMerge Table	13

9.	The HSSawLane Table	14

10.	The HSSawParm Table	19

11.	The HSSawSim Table	21

12.	High-Speed Sawtooth Merge Configuration Example	22

12.1.	Merge and Lane Setup	23

12.2.	Merge Belt Induction Configuration – Feeding Into the End	25

12.3.	Induction Point Adjustments	27

12.4.	Example with Inch-and-Store	28

13.	High-Speed Sawtooth Merge Test Plan	29

13.1.	HSSawMerge Table	29

13.2.	HSSawLane Table	29

13.3.	HSSawParm Table	31

13.4.	Miscellaneous Testing Items	31

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

PE – Photoeye

# High-Speed Sawtooth Merge Concept

When two or more sections of conveyor merge into one, precise timing is required to provide a smooth convergence and to maintain the desired spacing between containers.  The High-Speed Sawtooth Merge module provides support for:

- Multiple overlapping induction points

- Different feed and discharge speeds

- Adjustments for merge induct location based on the lane speeds at the instant of merging

- An unlimited number of reservation sizes based on timers, rate points or IO for controlling the size of induct slugs from each lane

The High-Speed Sawtooth Merge control logic uses one standard shifter linked to a merge belt.  The shifter tracks both reservations and space allocated for actual containers.  The standard shifter logic, in combination with the High-Speed Sawtooth Merge logic, provides coordinated and efficient merging of multiple lanes.

The High Speed Sawtooth Merge can feed each lane at two different speeds.  The slower speed is typically used when the lane is not full, and product is flowing up to the presence PE.  The faster speed is used to discharge a lane when the lane is merging.

Each lane can be configured with parameters that define a reservation size, reservation total, reservation space, slug gap, priority, and speed, and will enable/disable IO based on the selected signals.  The logic will determine which parameters to use based on the state of the selected signals and the order of the table entries.

The lane parameters are stored in a separate table (HSSawParm), where each lane may have one or more records to define its control conditions.

The logic allocates space for each lane based on the “active” parameter set, and the status of the lane.  This provides the ability to change the amount of time allocated for a lane based on additional conditions.

The following is a list of the signals that can make a parameter set “Active”:

- The lane PE is ON (at least one container is present)

  - The “At Least One” parameter set is used

- The full timer is Done or Not Done

  - If Done, the “Done” parameter set is used

  - If Not Done, the “Not Done” parameter set is used

- The priority input signal is ON or OFF

  - If ON, the ON parameter set is used

  - If OFF, the OFF parameter set is used

- The Rate is Low or High

  - If Rate is Low, the Low parameter set is used

  - If Rate is High, the High parameter set is used

The logic will scan the HSSawParm table to determine the parameter set to use.

- ResrvInches - Reservation size required, in inches.  The lane will only feed if a reservation is found of the correct size.  Different parameter sets can have different reservation size requirements.  Smaller reservation sizes are usually required for normal operation than for a full condition.  If the lane changes state and goes full, the lane must wait for a reservation of sufficient size.

- ResrvTotal – The total amount of space required before the lane completes its turn.  If ReservInches is set to 50 inches and ResrvTotal is set to 50 inches, the turn will complete with one reservation of 50 inches.  If ResrvInches is set to 50 inches and ResrvTotal is set to 100, the turn will complete once two reservations of 50 inches are created.  The lane will remain the highest priority lane (waiting for space to insert a reservation) as long as the other lanes are at the same Priority.

- ResrvSpacing – If ReservTotal is larger than ResrvInches, multiple reservations will be created.  ResrvSpacing is the minimum spacing between reservations.

- MaxCartonLen – When a reservation is created, the reservation length is computed as follows:  (ResrvInches + MaxCartonLen + SlugGap).

- SlugGap – Sets the amount of space needed to prevent jams between slugs as they are released from different lanes.

- Priority – Arbitrary units.  A larger value means higher priority.  Set this to the same value for all lanes to allow round-robin operation.  If all the lanes have the same Priority value, a lane gets more time to feed if ResrvInches is set to a larger value (such as when a lane goes full).  During operation, if the Priority value is increased, it will take effect when a new reservation is created – it does not alter reservation space that has already been allocated.  In cases where there is enough space to allocate reservations, changing the Priority value does not affect which lane is allocated space next.

- LaneSpeed - Choose Slow or Fast for this parameter set.

- Enable - Choose Disable or Enable.  If Disable is selected, the lane will be disabled when the parameter set is activated.  If Enable is selected, the state will be set to enabled.

- Override - Select N if the software needs to continue to process additional records for the lane.  Select Y to use the parameters entered and stop processing any other parameters.

If the logic finds that there is space on the merge shifter (the space between the start of the shifter and the lane induction point), and the highest-priority lane needs a reservation, the logic will create a reservation for the lane.

A lane will feed into the reservation space until the reservation space remaining is less than the SlugGap plus the MaxCartonLen.

- If the remaining space in the reservation is less than SlugGap plus MaxCartonLen, but greater than SlugGap, the logic will start looking for a gap on the infeed lane.  When a gap is found, feeding will stop.

- If the remaining reservation space is less than SlugGap, the lane will stop feeding and a jam event will occur.

The next lane to be assigned a reservation is determined by the following logic:

- Find a lane that has not been allocated the required space (space allocated is less than ResrvTotal).

- Find the lane with the highest priority.

- If the Priority value is the same, find the lane with the oldest feed time (the oldest time stamp is not updated until the lane has completed its reservation (until it has been allocated its ResrvTotal).

# High-Speed Sawtooth Merge Features

The High-Speed Sawtooth Merge configuration is more complex than a standard sawtooth merge, adding the ability to run the merge at two speeds.  The merge also includes features to manage the priority and slug size based on a variety of inputs.

  - Reservation Re-use

High-Speed Sawtooth Merge reservations are made available for re-use by other lanes at the largest lane induct point.  As a reservation reaches the largest lane induct point, the software will look at the shift register and if the reservation is allocated to the lane, will change the reservation into available space.

Note: The largest lane induct point is calculated from the lane induct table using the largest value entered.

Lane Induct Point

“Slug Gap” plus “Max Carton Len”

Merge Shift Register Re-Use Example

Area Available for Reservation

Largest Lane Induct Point - At the largest induct point the reservation is converted into unused space.  The space may be used by downstream lanes.

  - Opportunistic Feeding

In this example, Lane 2 is configured to feed individual containers at Slow speed if none of the other lanes have product or if there is unused space at the infeed point.  If Lane 2 is Full, it feeds a 30-foot slug (at the Fast speed).

- Create an entry in the Fullline table for the Full photoeye associated with the lane.

- Create an entry in table HSSawParm to feed individuals:

  - Name = “Lane 2 – Singles”

  - Type = “Timer Dn”

  - Timer:  Select the “Clear” timer for the lane.

  - ReservInches = 2 (minimum value)

  - ResrvTotal = 2 (minimum value)

  - ResrvSpacing = 0

  - MaxCartonLen = 20

  - SlugGap = 18

  - Priority = 0 (all other entries will get Priority = 10)

  - LaneSpeed = “Slow”

- Create an entry in table HSSawParm to feed a slug:

  - Name = “Lane 2 – Full Slug”

  - Type = “Timer Not Dn”

  - Timer:  Select the “Clear” timer for the lane.

  - ReservInches = 360 (30 feet, in inches)

  - ResrvTotal = 360

  - ResrvSpacing = 0

  - MaxCartonLen = 20

  - SlugGap = 18

  - Priority = 10

  - LaneSpeed = “Fast”

The parameter set “Lane 2 – Singles” will be active when the Clear timer is done (the lane is NOT full).  This parameter set has a priority less than all other lanes.  It will only get a reservation if no other lanes need the reservation space, or if opportunistic space is available.

When the lane becomes full, the parameter set “Lane 2 – Full Slug” will become active.  This parameter set changes the priority and will feed a 30-foot slug instead of a single container.

# High-Speed Sawtooth Merge Design Concerns

- To maximize throughput, the configure the merge to feed the maximum size slug (A slug that triggers the Full PE), at the fast speed.

- The throughput is constrained by the slug feed time from each lane.

# Merge Shifter Lane Layout

The diagram, below, shows accumulation, brake-meter, inch and store, and slug belts feeding into a merge.  The shifter is also depicted with the induct point and a reservation (allocated space) for the lane.

Lane Conveyor

Merge Shifter

Induct PE

Brake-Meter Belt

Accumulation Section

Reservations

Merge Slave Section

Merge Conveyor

In-feed Lane

Reflected Induct Point

Induct Photoeye

High Speed Merge Example

Gapping Photoeye

Reflected Induct Point

Inch-Store Photoeye

Reservation

# Configuring the Shifter

Determine the size of the shifter that will be required for the High-Speed Sawtooth Merge:

- Calculate the largest reservation size needed based on the location of the Full photoeyes on each infeed lane.

- Calculate the size of the shifter based on the length of the merge belt.

- Add the largest reservation size, largest container size, and the required gap to the merge belt size to get the total length of the shifter.  Round up – it doesn’t hurt to make it longer.

Configure the shifter in table Sorters based on your calculations.

Note:  Refer to the FPC-Sorter-Control-Module document for more information on configuring, testing, and debugging the shifter.

If the merge uses a timer-based encoder, set HSSawMerge_TimeBasedEncoder = Y.  The software will then automatically start and stop the timer to generate ticks on the sorter based on the state of the input defined in HSSawMerge_MergeRunIO.

# Configuring Lane and Merge Induct Points

Based on the physical position of the induction point and the tick resolution, create the induct point for each of the lanes in the Inpoints table.  Configure the induct point for each merge lane in HSSawLane_LaneInduct.  Choose a memory bit for each lane induct point.  Manually set the memory bits to the OFF state initially.

Define the induct point that represents the beginning of the shift register for the merge.  This point must be located before any input lane induction point.  Select it in HSSawMerge_MergeInduct.  Choose a memory bit for the induct point.  Manually set the memory bit to the OFF state initially, unless the merge has product feeding directly into the entry end of the merge, and it is not configured as a lane.  In that case, use the physical induct PE instead of a memory bit.

# High-Speed Sawtooth Merge Table Relationships

Four tables are used to configure a High-Speed Sawtooth Merge:

      - The HSSawMerge table contains an entry for each merge.

      - The HSSawLane table contains an entry for each merge lane.

      - The HSSawParm table contains the slug size, speed, priority and various lane signals.  Each lane feeding the merge requires at least one entry in this table.

      - The HSSawSim table contains the configuration parameters used to run the merge in simulation mode.

Note: The High-Speed Sawtooth Merge modules must be activated in the Batches table:

InternalProcCtrl:  HSSawtooth (SORT)		Process:  SORT

InternalProcCtrl:  HSSawtooth (CTRL)		Process:  CTRL

HSSawMerge Table

1. Merge 1

HSSawLane Table

Lane 1

Lane 2

Lane 3

Sorter Table

Sorter 1

Induct Points

Lane In 1

Lane In 2

Lane In 3

HS Sawtooth Merge Tables

Lane 1

Lane 2

Lane 3

HSSawParm Table

Lane Induct Point

Slug Gap

Merge Shift Register Example

Area Available for Reservation

MaxCartonLen

As the reservation moves through the Induct Point, the lane will begin the stop-feeding sequence when the space remaining is less than the “Max Carton Len” plus “Slug Gap”.

# The HSSawMerge Table

An entry in the HSSawMerge table defines a High-Speed Sawtooth Merge.

| HSSawMerge Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name to describe this merge. |
| CreateResrv | Y/N | Set to Y to enable creation of reservations on this merge. |
| pStatus | String | The program will display status and error messages here. |
| MergeRunIO | PopUp | Choose an IO part (usually the motor under the merge belt) that indicates whether the merge is running or stopped. |
| MergeInduct | PopUp | Choose the induction point from table Inpoints that defines the beginning of the merge. The Inpoints record defines the link to table Sorters, and table Sorters connects to the encoder in table Encoders. |
| TimeBasedEncoder | Y/N | N = A physical encoder is used. A timer-based encoder is used only for simulation. NOTE: MergeRunIO must be ON for the simulator to run. Y = The merge logic will start or stop the timer-based encoder for the merge belt based on the state of MergeRunIO. |
| LogLevel | PopUp | Choose the desired log level: 0 – Error messages only. 1 – Warnings and error messages. 2 – Warnings, errors and debugging information. |
| ControlMachine | PopUp | Choose the AC that controls this merge. |
| pHighestPriority | Int | Set by the software to the highest priority lane. |
| pCurrentLane | Int | Set by the software to the lane currently reserving space. |
| pStartLocation | Int | Set by the software to the starting location, in ticks, of the reservation currently being assigned. |
| pEndLocation | Int | Set by the software to the ending location, in ticks, of the reservation currently being assigned. |
| pResrvSize | Int | Set by the software to the current size of the reservation being assigned. pEndLocation – pStartLocation |
| pLastEncCnt | Int | Set by the software to record the latest encoder count value processed. |
| pSimOn | Int | Set by the software to indicate if the simulation is running and at least one lane is enabled. |
| pSimEncTmr | Int | Set by the software to indicate that at least one lane has a timer-based encoder enabled for simulation. |

# The HSSawLane Table

The High-Speed Sawtooth Merge Lane table (HSSawLane – Configuration Menu | Merge Utilities | HS Saw Tooth Conf | HS SawLane Table) contains the definition of each infeed lane on the merge.

Entries are required in the SawMerge, LaneInduct, LanePE, LaneSlow, LaneFast and GapError fields.  Others are optional.

| HSSawLane Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name to describe this lane. |
| Enabled | Y/N | Manual override to enable or disable this lane. |
| Opportunistic | Y/N | Set to Y to activate the opportunistic merge logic that allows this lane to use otherwise unused reservation space from other lanes even if this lane does not have the highest priority. The logic will look for unused space starting at the induct point but not overlapping any space needed by the highest priority lane. The current active parameter set defines the space requirements (see section 10 – The HSSawParm Table). NOTE: If the highest priority lane cannot allocate a reservation because there is not enough available space, the reserved area will not be made available for opportunistic feeds! |
| OpportunSingle | Y/N | Set to Y to activate opportunistic merge logic for single containers. |
| SawMerge | PopUp | Assign this lane to a High-Speed Sawtooth Merge by choosing an entry from table HSSawMerge here. |
| pStatus1 | String | Set by the software to show the status of this lane. |
| pStatus2 | String | Set by the software to show errors and warning for this lane. |
| pLaneState | PopUp | Set by the software to show the current state value for this lane. |
| LanePE | PopUp | Choose an input signal for the PE that detects the presence of containers on this lane. |
| FeedCompleteTM | PopUp | Choose a timer that defines how long LanePE must be OFF before the reservation is considered complete. This allows the lane to complete its reservation (and release the space) if it runs out of containers, even if the entire reservation (pTicksFeed) has not been consumed. |
| EnableDelayTM | PopUp | Choose a timer that defines how long to wait after a start or restart (after a jam, for instance) before allowing this lane to begin releasing containers. |
| LaneSlow | PopUp | Choose the output signal that will be activated to make this lane run at the slow speed. The output is deactivated when the lane is stopped. The slow speed is set while waiting for LanePE to turn ON. The lane will stop if containers are present while waiting for a reservation. |
| SlowAction | PopUp | Choose the action to take when LaneSlow is activated. The inverse action is set to deactivate LaneSlow. |
| SlowBit | Int | If SlowAction is Enable or Disable, enter the bit value to use. |
| LaneFast | PopUp | Choose the output signal that will be activated to make this lane run at the fast speed. The output is deactivated when the lane is stopped. The fast speed is set while this lane is discharging, if the parameter set is defined that way. The lane will stop if containers are present while waiting for a reservation. |
| FastAction | PopUp | Choose the action to take when LaneFast is activated. The inverse action is set to deactivate LaneFast. |
| FastBit | Int | If FastAction is Enable or Disable, enter the bit value to use. |
| SlowOrFast | PopUp | Choose the output signal that will be activated to make this lane run at either the slow or fast speed. The output is deactivated when the lane is stopped. This is the output that actually starts and stops the motor. |
| SlowOrFastAction | PopUp | Choose the action to take when SlowOrFast is activated. The inverse action is set to deactivate SlowOrFast. |
| SlowOrFastBit | Int | If SlowOrFastAction is Enable or Disable, enter the bit value to use. |
| AccumFast | PopUp | Choose the output signal that will be activated to make the accumulation section for this lane run at the fast speed. The output is deactivated when the lane no longer needs to run at the fast speed. The accumulation section runs at the fast speed while the lane is discharging, if the parameter set is defined that way. The accumulation section will stop while the lane is waiting for a reservation or if the lane is feeding on the slow speed. |
| AccumAction | PopUp | Choose the action to take when AccumFast is activated. The inverse action is set to deactivate AccumFast. |
| AccumBit | Int | If AccumAction is Enable or Disable, enter the bit value to use. |
| BoxPresentOut | PopUp | Choose a memory bit to use as an output if a container has been detected at this lane. The memory bit state will latch ON once LanePE has been activated. This allows the merge to ‘remember’ that a container was detected, even if it drifts through the PE. This latched state is also used internally by the merge logic. |
| LaneInduct | PopUp | Choose an induction point from table Inpoints. This point defines where the container will stop to wait for a reservation (internally, HSLANE_INDUCT_STOP). Container tracking data is added to the shift register at this point, defining the container position on the merge shifter. The induct point position changes under program control based on the status of the lane. In table Inpoints, choose a memory bit for Induct_I_O_Name and set it initially to an OFF state. |
| GapError | PopUp | Choose a memory bit that will be set ON if this lane does not detect a gap in time to stop the lane normally. The lane will be stopped even without a gap. A gap error occurs if the lane is trying to stop feeding but the LanePE does not go OFF before another reservation or container is detected on the main merge belt from another lane. |
| ResetGapError | PopUp | Choose an input (memory bit OK) to use to reset gap errors. Gap errors latch ON and must be reset to clear. The errors may also be reset using “Maintenance Menu ¦ Reset Utilities ¦ Reset Sawtooth Error”. |
| GapErrMtr | PopUp | Choose an output signal that will be activated when a gap error occurs. |
| ErrMtrAction | PopUp | Choose the action to take when GapErrMtr is activated. The inverse action is set to deactivate GapErrMtr. |
| ErrMtrBit | Int | If ErrMtrAction is Enable or Disable, enter the bit value to use. |
| RunSlowOffset | Int | Enter an adjustment, in ticks, to be applied to the induction point location for containers merging from a slow speed without stopping. |
| StopFastOffset | Int | Enter an adjustment, in ticks, to be applied to the induction point location for containers merging from a stop using the fast speed. |
| RunFastOffset | Int | Enter an adjustment, in ticks, to be applied to the induction point location for containers that merge at the fast speed without stopping. |
| ReAssignLane | Int | Set this to a non-zero value to allow a reservation to be re-assigned to another lane if it is no longer needed for this lane. |
| pResrvReqTicks | Int | Set by the software. HSSawParm_ResrvTotal from the current parameter set is converted into ticks and used to determine when the reservation is complete for this lane. The actual reservation created will contain additional space for the HSSawParm_SlugGap and HSSawParm_MaxCartonLen, however the actual space used in the reservation, on average, will be smaller and it is not added to this value. |
| pResrvTtl | Int | Set by the software. The total accumulated ticks allocated to this lane on the merge shifter in the current reservation cycle. |
| pTicksFeed | Int | Set by the software to track the running total of ticks when this lane is feeding. This value and pResrvTtl are used to determine if this lane requires additional reservation space. It is also used to determine if this lane has completed its requirements in the current reservation cycle. |
| pGapTicks | Int | Set by the software to indicate the number of ticks for the slug gap. |
| pInductLocRel | Int | Set by the software. Records the relative index into the SortBuff table for the induct location when a container is detected prior to assigning a reservation. Once the merge sequence begins, this value is not changed. |
| pInductLoc | Int | Set by the software. Records the actual index into the SortBuff table for the induct location when a container is detected prior to assigning a reservation. Once the merge sequence begins, this value is not changed. |
| pInductLrge | Int | Set by the software. Records the largest induct location value based on the offset values entered. This is the actual index into the SortBuff table. The value + 1 is used to remove the reservation for the lane if it has not been used. |
| pLatchPE | Int | Set by the software. Used to latch the state of the lane presence PE during the Wait PE On and Wait Resrv states. The state of the PE is updated normally in the Feed Slow and Feed Fast states. |
| pParamRecord | Int | Set by the software. The currently selected HSSawParm record for this lane. The selected parameter set changes based on the state of the input signals. |
| pLastRunSec | Int | Set by the software to record the whole seconds portion of the timestamp when the lane has completed the reservation cycle. This value is considered when assigning lane priorities. |
| pLastRunMSec | Int | Set by the software to record the millisecond portion of the timestamp when the lane has completed the reservation cycle. This value is considered when assigning lane priorities. |
| pDelaySpace | Int | Set by the software. Related to the HSSawParm_ResrvSpacing value from the current parameter set. Used to create space between reservations. |
| pGapSpaceFnd | Int | Set by the software. Contains the gap space found when searching the shifter. |
| pPriorState | Int | Set by the software. Used to detect when a lane state changes for logging purposes. |
| pGapErrOn | Int | Set by the software. Used to latch the state of the gap error. |
| pOpporStart | Int | Set by the software. The beginning of space available for the opportunistic calculation. |
| pOpporEnd | Int | Set by the software. The end of space available for the opportunistic calculation. |
| pOppRstStart | Int | Set by the software. The beginning of space available for the opportunistic calculation after checking for restrictions imposed by the other lanes. |
| pOppRstEnd | Int | Set by the software. The end of space available for the opportunistic calculation after checking for restrictions imposed by the other lanes. |
| pOppAvail | Int | Set by the software. The current space available on the shifter for this lane. It is updated after checking for space already allocated to other lanes. |
| pOppNeed | Int | Set by the software. The space needed for the opportunistic requirement. If the Opportunistic option is selected, the space needed will include ResrvInches + MaxCartonLen + SlugGap from the current active parameter set (table HSSawParm). If the OpportunSingle option is selected, the space needed will include only MaxCartonLen + SlugGap. |

# The HSSawParm Table

The High-Speed Sawtooth Merge Parameters table (HSSawParm – Configuration Menu | Merge Utilities | HS Saw Tooth Conf | HS Saw Parameters) contains the parameters needed to control the sawtooth lanes based on the state of the selected signals.  Each lane requires at least one entry in this table (Type = “At Least One”).  Additional entries may be defined as needed for other conditions and requirements.

| HSSawParm Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this record. |
| SawLane | PopUp | Choose the HSSawLane record associated with this record. |
| pStatus | String | The software will display error or status messages here. |
| Type | PopUp | Choose the parameter set type that will determine which conditions to act upon: - At Least One – Choose this to define the default parameters for this SawLane. All other types are optional. This entry will be used if no other parameter sets are found. - Tmr Not Done – Choose this to define the parameters to use when the timer chosen in the Timer field has not completed. - Tmr Done – Choose this to define the parameters to use when the timer chosen in the Timer field has completed. - Input Off – Choose this to define the parameters to use when the signal chosen in InputSignal is OFF. - Input On – Choose this to define the parameters to use when the signal chosen in InputSignal is ON. - Rate Low – Choose this to define the parameters to use when the record selected in the RateRecord field is LOW. - Rate High – Choose this to define the parameters to use when the record selected in the RateRecord field is HIGH. |
| Timer | PopUp | Choose a timer from table timemenu that will be used to determine if the Type “Tmr Done” or “Tmr Not Done” parameter set will be used. |
| InputSignal | PopUp | Choose an IO part from table Conveyor that will be used to determine if the Type “Input On” or “Input Off” parameter set will be used. |
| RateRecord | PopUp | Choose a row from table RateCount that will be used to determine if the Type “Rate Low” or “Rate High” parameter set will be used. |
| ResrvInches | Int | Enter the reservation length, in inches, for this Type on this SawLane. Note: If ResrvInches is set to 0, the lane will not feed into an existing reservation, which may occur if a new parameter set is selected while the lane is feeding, and that parameter set disables the lane. |
| ResrvTotal | Int | Enter the length, in inches, of the total space to be allocated to this SawLane for this turn. This is often set to the same value as ResrvInches to create a single reservation per turn. If it is larger than ResrvInches, multiple reservations may be created for the lane in a single turn. Setting this value larger than ResrvInches creates additional reservation space for opportunistic feeds and can also be used to allow a slow-running lane time to “catch up” between reservations. |
| ResrvSpacing | Int | If ResrvTotal is larger than ResrvInches, multiple reservations can be created for this SawLane. This value defines the space that will be created between those reservations, in inches. |
| MaxCartonLen | Int | Enter the length of the largest container, in inches. This value is added to the reservation to ensure there is enough room to feed the last container in the slug. Note: If MaxCartonLen is set to 0, the SawLane will not feed into an existing reservation, which may occur if a new parameter set is selected while the lane is feeding, and that parameter set disables the lane. |
| SlugGap | Int | Enter a value, in inches, for the gap to be allowed between slugs. This is needed to prevent jams as a new slug is released when switching to a new SawLane. The value is added to the reservation to allow empty space at the end of the slug. Note: If SlugGap is set to 0, the lane will not feed into an existing reservation, which may occur if a new parameter set is selected while the lane is feeding, and that parameter set disables the lane. |
| Priority | Int | Enter a priority value for this parameter set. Larger values are higher priority. For round-robin operation (the usual choice), set all lanes to the same value. With all lanes at the same priority, you can still give a SawLane more time to feed by increasing the value in the ResrvInches field. |
| LaneSpeed | PopUp | Choose the SawLane speed. The options are Slow or Fast. |
| Enable | Y/N | Determine the action to take when this parameter set is activated. N = The SawLane will be disabled when the parameter set is selected. Note that the lane will not feed into an existing reservation if ResrvInches, MaxCartonLen, or SlugGap are 0. Y = The SawLane will be enabled when the parameter set is selected. |
| Override | Y/N | Used to control if additional matching records in this table are to be processed once this record has been reached. The records are processed in order from the top of this table. This might be used if the SawLane must be disabled after processing an entry associated with a full condition. N = Continue processing records for this lane after this entry has been processed. Y = Process this entry, then stop searching for records for this lane. |

# The HSSawSim Table

The High-Speed Sawtooth Merge Simulator table (HSSawSim – Configuration Menu | Merge Utilities | HS Saw Tooth Conf | HS Saw Sim Config) is used to start and stop the simulator, and to define the size of the containers created on each infeed lane.

| HSSawSim Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name. |
| EnableSrt | Y/N | Use this field to enable or disable individual entries. |
| CreateBoxes | Radio | Selecting this field will cause simulated containers to be created on the infeed lane. |
| pStatus | String | Set by the software to indicate errors or status information. |
| SawLane | PopUp | Choose the lane from table HSSawLane to simulate. |
| CreateSorterTmr | Radio | Select this field to create an encoder timer for the sorter associated with this entry. |
| LengthInches | Int | Define the length, in inches, for the simulated containers for this lane. |
| GapInches | Int | Define the length, in inches, of the gap to be created between the simulated containers. |
| pStateForcePE | Int | Set by the software to track the force state of the induct PE on the lane so it can be restored when the simulation is stopped. |
| pCurrLength | Int | Set by the software. The length of the current simulated container as it is being created. |
| pCurrGap | Int | Set by the software. The length of the gap currently being created. |

# High-Speed Sawtooth Merge Configuration Example

This example shows a merge with four infeed lanes.  Lane 1 handles recirculation.  Lanes 2 and 3 will be disabled when a high-rate condition occurs on Lane 1.  Lane 1 does not require any parameter changes when the recirculation rate is high since Lanes 2 and 3 will be disabled.  Lane 4 will run only when it becomes full, or if containers have been waiting at its presence PE for one minute.  This is achieved by using the presence PE as a Full eye.

The “Rate High” entries for Lane 2 and Lane 3 are used to disable the creation of reservations.  A reservation may have already been created when one of the lanes is disabled.  If so, the space may be used by the new parameter set if it has the same or smaller reservation requirements.  If the old reservation space is not large enough, it will not be usable.  New reservation values take effect immediately when a lane changes to a new parameter set.

| HSSawParm Table Entries |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Saw Lane | Type | Resrv Inches | Resrv Total | Resrv Spacing | Slug Gap | Priority | Lane Speed | Enable | Override |
| Lane1 | At Least One | 48 | 48 | 0 | 38 | 1 | Slow | Enable | N |
| Lane1 | Input On-Full PE | 192 | 192 | 0 | 48 | 1 | Fast | Enable | N |
| Lane2 | At Least One | 48 | 48 | 0 | 38 | 1 | Slow | Enable | N |
| Lane2 | Input On-Full PE | 192 | 192 | 0 | 48 | 1 | Fast | Enable | N |
| Lane2 | Rate High | 192 | 192 | 0 | 48 | 1 | N/A | Disable | Y |
| Lane3 | At Least One | 48 | 48 | 0 | 38 | 1 | Slow | Enable | N |
| Lane3 | Input On-Full PE | 192 | 192 | 0 | 48 | 1 | Fast | Enable | N |
| Lane3 | Rate High | 192 | 192 | 0 | 48 | 1 | N/A | Disable | Y |
| Lane4 | Input On-Lane PE | 48 | 48 | 0 | 38 | 1 | Slow | Enable | N |
| Lane4 | Input On-Full PE | 192 | 192 | 0 | 48 | 1 | Fast | Enable | N |

To configure the lane presence PE for Lane 4 as a full PE:  Set Fullline_Sensor_Name to a memory bit.  Select the memory bit in HSSawLane_BoxPresentOut.  The memory bit will latch ON when the lane presence PE activates, picking up containers that have drifted through the PE.  Set the initial state of the memory bit to OFF.

  - Merge and Lane Setup

Configure the High-Speed Sawtooth Merge to accurately merge a container into the reservation.

- In the first setup iteration, the induction point is adjusted for a container merging at a slow speed from a stopped state.  This is the default configuration.

- The lane induction point, and the merge induction point use memory bits that are turned ON and OFF by the software.

  - Make sure the HSSawLane_LaneInduct point has a memory bit selected for each lane.

  - Make sure the HSSawMerge_MergeInduct point is configured with a memory bit.

- The Lane 1 induction point must be located with at least the following space:  30 + “Length of the largest slug in ticks” + “Maximum container length in ticks” + “Slug gap in ticks”.  Example: With an encoder set to 12 ticks per foot, 30 + (12 ft slug * 12 ticks per ft) + (2 ft container * 12 ticks per ft) + (1 ft Gap * 12 ticks per ft) = a tick position of 210 on the graphical shifter screen.

  - Do not change the Lane 1 induction point after the initial adjustment.

  - Reservation space may be adjusted later by shifting all the induction points a fixed amount.  For example:  If an additional 500 ticks of reservation space are needed, add 500 to each lane induction point position.

- Add the Jam PE at the end of the merge to an update point.  The PE will trigger the SORT process to print a shift print to its log file.  The shift print pictograms will help you make adjustments later.

- The induction point defines the StoppedSlow location.

- Adjust the induction point by disabling each lane and setting the speed in the At Least One parameter set to Slow.  Place a container on the lane.  It will advance to the presence PE and stop.  Enable the lane after the container has completely stopped.

  - Change the Jam PE update point position until the nose of the container is consistently shown within two ticks either to left or to the right of the “X” in the center of the shift print.  Repeat with at least ten containers of various sizes and weights, adjusting for the best compromise.

  - Once the Jam PE update point position has been set, do not adjust it again.

- Adjust the lane induction point positions for the remaining lanes one at a time until the shift prints consistently show the nose of the container within two ticks of the center “X”.  Repeat with at least ten containers of various sizes and weights, adjusting for the best compromise.

- On the graphical shifter, the reservation will change color from white to gray to show where the container is located within the reservation.  Unused space will remain white and will be removed as the reservation moves past the update point.

Once the above procedure has been performed on each lane, perform the following:

- RunSlowOffset - The HSSawLane_RunSlowOffset value is determined by enabling each lane, in turn, and setting the speed in the At Least One parameter set to Slow.  The RunSlowOffset value adjusts for containers that do not stop and merge at a Slow speed.  A container placed on the lane will advance to the presence PE and merge.  The shift print is used to calculate the RunSlowOffset.  The difference from the nose of the shift print to the “X”, in ticks, is entered into RunSlowOffset.

- StopFastOffset - The HSSawLane_StopFastOffset value is determined by disabling each lane and setting the speed in the At Least One parameter set to Fast.  The StopFastOffset adjusts for containers that stop, then merge at a Fast speed.  A container placed on the lane will advance to the presence PE and stop.  Enable the lane and the container will merge.  Use the shift print to calculate the StopFastOffset value.  The difference from the nose of the shift print to the “X”, in ticks, is entered into StopFastOffset.

- RunFastOffset - The HSSawLane_RunFastOffset value is determined by enabling each lane, in turn, and setting the speed in the At Least One parameter set to Fast.  The RunFastOffset adjusts for containers that do not stop and merge at a Fast speed.  A container placed on the lane will advance to the presence PE and merge.  The shift print is used to calculate the RunFastOffset value.  The difference from the nose of the shift print to the “X”, in ticks, is entered into RunFastOffset.

  - Merge Belt Induction Configuration – Feeding Into the End

It is possible to configure a High-Speed Sawtooth Merge so product can be fed directly into the end of the merge belt.  Reservations are then created for the normal infeed lanes using the space between those containers.  The merge belt and lane induction points must be configured to represent the actual distance between the merge and lane photoeyes.

- In the first setup iteration, the induction point is adjusted for a container merging at a slow speed from a stopped state.  This is the default configuration.

- The lane induction point, and the merge induction point use memory bits that are turned ON and OFF by the software.

  - Make sure the HSSawLane_LaneInduct point has a memory bit selected for each lane.

  - Select a photoeye that will be used as the induction PE for the HSSawMerge_MergeInduct point.  Install the merge PE as far upstream of the merge as possible to allow as much space as possible for reservations.

- Set the HSSawMerge_MergeInduct point position to 30.

- Move the Lane 1 induction point to 30 + “Distance from the induct point to the lane”.  Example:  With an encoder set to 12 ticks per foot, 30 + (12 ft distance * 12 ticks per ft) = a tick position of 174 on the graphical shifter screen.

  - Do not change the Lane 1 induction point after the initial adjustment.

  - Reservation space cannot be adjusted later since these settings are not abstract – they represent the physical configuration of the merge hardware.  Physical containers will be feeding in through the induction point on the merge.

- Add the Jam PE at the end of the merge to an update point.  The PE will trigger the SORT process to print a shift print to its log file.  The shift print pictograms will help you make adjustments later.

- The induction point defines the StoppedSlow location.

- Adjust the induction point by disabling each lane and setting the speed in the At Least One parameter set to Slow.  Place a container on the lane.  It will advance to the presence PE and stop.  Enable the lane after the container has completely stopped.

  - Change the Jam PE update point until the nose of the container is consistently shown within two ticks either to left or to the right of the “X” in the center of the shift print.  Repeat with at least ten containers of various sizes and weights, adjusting for the best compromise.

  - Once the Jam PE update point position has been set, do not adjust it again.

- Adjust the lane induction point positions for the remaining lanes one at a time until the shift prints consistently show the nose of the container within two ticks of the center “X”.  Repeat with at least ten containers of various sizes and weights, adjusting for the best compromise.

- On the graphical shifter, the reservation will change color from white to gray to show where the container is located within the reservation.  Unused space will remain white and will be removed as the reservation moves past the update point.

Once the above procedure has been performed on each lane, perform the following:

- RunSlowOffset - The HSSawLane_RunSlowOffset value is determined by enabling each lane, in turn, and setting the speed in the At Least One parameter set to Slow.  The RunSlowOffset value adjusts for containers that do not stop and merge at a Slow speed.  A container placed on the lane will advance to the presence PE and merge.  The shift print is used to calculate the RunSlowOffset.  The difference from the nose of the shift print to the “X”, in ticks, is entered into RunSlowOffset.

- StopFastOffset - The HSSawLane_StopFastOffset value is determined by disabling each lane and setting the speed in the At Least One parameter set to Fast.  The StopFastOffset adjusts for containers that stop, then merge at a Fast speed.  A container placed on the lane will advance to the presence PE and stop.  Enable the lane and the container will merge.  Use the shift print to calculate the StopFastOffset value.  The difference from the nose of the shift print to the “X”, in ticks, is entered into StopFastOffset.

- RunFastOffset - The HSSawLane_RunFastOffset value is determined by enabling each lane, in turn, and setting the speed in the At Least One parameter set to Fast.  The RunFastOffset adjusts for containers that do not stop and merge at a Fast speed.  A container placed on the lane will advance to the presence PE and merge.  The shift print is used to calculate the RunFastOffset value.  The difference from the nose of the shift print to the “X”, in ticks, is entered into RunFastOffset.

  - Induction Point Adjustments

The following is an example set of offset parameters for the induct point.

| Induct Adjustment |  |  |
| --- | --- | --- |
| Status | Induct Adjustment | Description |
| StoppedSlow | Location of the Induct Point | Lane is stopped, waiting for a reservation. When it starts, it will run at the slow speed |
| RunSlowOffset | 12 inches | Lane is running. When a container reaches the presence PE it will continue at the slow speed and merge without stopping |
| StoppedFastOffset | 6 inches | Lane is stopped, waiting for a reservation. When it starts, it will run at the fast speed |
| RunFastOffset | 18 inches | Lane is running. When a container reaches the presence PE, it will continue at the high speed and merge without stopping |

  - Example with Inch-and-Store

Note:  See the Inch-and-Store document for a detailed example of an Inch-and-Store feeding a merge.

Configure a merge with an Inch-and-Store belt feeding a slug belt.  The Inch-and-Store belt will build a slug of containers.  If the merge lane is not feeding, the slug will feed from the Inch-and-Store belt onto the slug belt.

The merge will create a reservation for the slug and feed it onto the merge belt.

Slug Belt

Inch+Store Induct PE

Infeed Brake-Meter Belt

Discharge PE

Container Gap

Merge Belt

Merge Lane PE

Inch+Store Belt

IS-Slow and IS-Fast

# High-Speed Sawtooth Merge Test Plan

Use the following information to test a High-Speed Sawtooth Merge configuration.

  - HSSawMerge Table

- CreateResrv

    - When set to N, the creation of reservations will stop.  Once the reservations have cleared, containers will advance to the Lane PE (HSSawLane_LanePE) and the lane will stop.

    - When set to Y, the lane should create reservations and merge.

- MergeRunIO

    - When ON, HSSawLane_EnableDelayTM for each lane should run.  When the timer is Done, the lane will be ready to merge.

    - When OFF, HSSawLane_EnableDelayTM for each lane should reset.  The lane will change to the “Wait PE On” state.  If HSSawLane_LanePE is ON it will move to the “Wait Resrv” State.  The lane will not be enabled to merge until the timer is in a Done state.

- MergeInduct

    - Defines where the reservation assignment area begins.

    - New reservations created by the software will always be located after the MergeInduct point.

- LogLevel – Enable additional log messages, such as change-of-state shift prints by choosing Level 2.

  - HSSawLane Table

- Enabled

- If set to Y, reservations are created when LanePE goes ON.

- If set to N, when the reservations have cleared, cartons will advance to LanePE and the lane will stop.

- Opportunistic

  - If set to N, reservations are only created when the lane has priority.

  - If set to Y, reservations are created if the area at the lane induct is clear, and the space is not reserved by another lane.

- LanePE

  - If ON, and if the reservation is at the induct point and it is of sufficient size, the lane will merge.  The reservation size may change if, for example, the lane goes full.

  - If OFF, the lane will run at the slow speed to advance containers to the LanePE.  If LanePE goes OFF while a reservation is feeding, it will continue to feed for pGapTicks, which are derived from HSSawParm_SlugGap.  If LanePE stays OFF for more than pGapTicks, the remaining reservation space will be made available for downstream use.

- FeedCompleteTM

  - This timer is used to detect when a lane has completed its turn.  It runs when LanePE is OFF, and resets when LanePE is ON.

  - pLatchPE is held ON once a container is detected by LanePE and until the lane has had a turn to run.

  - A lane will complete its turn when pTicksFeed is greater than or equal to the pResrvTtl or when the FeedCompleteTM timer is done.

  - When a reservation is being processed, it will reset when this timer times out and the lane returns to the “Wait PE” or “Wait Reservation” state.  pResrvReqTicks, pResrvTtl, and pTicksFeed are set to 0 at this time.  This forces a lane to complete its turn if the lane has fed all of its cartons.  This only affects lanes that have a HSSawParm_ResrvTotal that is larger than HSSawParm_ResrvInches.

- EnableDelayTM

  - If this timer is Done the lanes can feed and get reservations.

  - If not Done, the lane will stop feeding.  No reservations are created.

- LaneSlow, SlowAction, SlowBit – These define how the motor will be controlled in the slow state.

- LaneFast, FastAction, FastBit – These define how the motor will be controlled in the fast state.

- LaneInduct – Defines the induction point for slow-speed merging – from a stopped lane.

- GapError – This output is turned ON if an error occurs where no gap can be found on the lane, while it is feeding, in time to stop the lane and maintain the HSSawParm_SlugGap between containers.  The output is turned OFF using ResetGapError.

- ResetGapError – Set this to ON to reset GapError.

- RunSlowOffset – Adjusts the induction point for Slow-speed merging, from a running lane.

- StopFastOffset – Adjusts the induction point for Fast-speed merging, from a stopped lane.

- RunFastOffset – Adjusts the induction point for Fast-speed merging, from a running lane.

- ReAssignLane – If defined, declares the lane that should receive unused reservation ticks.  If not defined, unused ticks may be used by any other lane, if possible.

- pInductLocRel – Set by the software to the relative index for the induct point adjusted by the offset.

- pInductLoc – Set by the software to the absolute induction location – the actual location in table SortBuff location in the shifter, adjusted by the offset.

- pInductLrge – Set by the software to the induction location, adjusted by the offset + 1.  This is the location where unused reservation space will be cleared for the lane.

  - HSSawParm Table

- Type – Defines how the selected signals are processed.  See section 10.

- SlugGap – length of the gap added to the reservation.  This value is added to ResrvInches along with the MaxCartonLen to create a reservation.  This value is also added to each container when the HSSawLane_LanePE transitions OFF.  The additional container length is overwritten if HSSawLane_LanePE turns ON.  If the container is the last one in a slug, the additional length will provide the slug gap for the whole slug.

  - Miscellaneous Testing Items

- If HSSawLane_LanePE does not go OFF and the reservation is at its end (past the HSSawParm_SlugGap ticks) and another “Gray” reservation is within HSSawParm_SlugGap ticks, the lane will stop.  HSSawParm_SlugGap ticks are added to the end of the reservation.

  - HSSawLane_GapError is latched ON.

  - When HSSawLane_ResetGapError turns ON, HSSawLane_GapError is reset.

- If HSSawLane_LanePE goes OFF and the merge is running at the Slow speed, or if the merge is running at the Fast speed, the merge shifter will receive additional ticks set to “Gray” equal to the HSSawParm_SlugGap.

- The runtime timestamp in HSSawLane_pLastRunSec and HSSawLane_pLastRunMSec is updated only when the lane feed cycle is complete – after multiple feed events complete a cycle (if HSSawParm_ResrvTotal is larger than HSSawParm_ResrvInches).

- When multiple HSSawParm records are defined, the last active record is used.  (TRK:  I am not sure about this one!)

- Merge lanes will stop:

  - If the merge motor stops

  - If a collision is going to occur

  - If a reservation is not available

  - If a parameter set (HSSawParm) is not available

  - If the speed is not set in the parameter set

  - If the lane is disabled in the parameter set and a reservation is not already assigned.

- Test with multiple merge lanes to make sure they do not interfere with each other.

- Test these items for the opportunistic merge feature:

  - Lane 1 highest priority, but it cannot allocate the reservation

  - Lane 3 space is available