| # FortnaPlus Control Horn Control Module |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 20-Jan-2021 |  |

Revision; Date; Author; Revision Summary. 1.0; 06-Aug-2013; B C Conklin; New document. 1.1; 20-Jan-2021; T R Kremser; Updates and general refresh.

Table of Contents

1.	Horn Control Module	4

2.	The Horns Table	5

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Horn Control Module

The horn control module is used to manage warning horns and beacons.

Horns are often used to warn personnel prior to startup of conveyor segments, for safety.  They can also be used to call attention to error conditions that demand immediate attention.  Beacons are used in a similar fashion for things like full lanes that may not require attention as urgently.  The horn control module could be used to control any output that requires a time-based on-off signal pattern.

For extra flexibility, beacons (or horns) can be configured to blink (sound) in patterns that can indicate different conditions to the operations personnel.

# The Horns Table

All configuration for the horn control module is done in table horns:

| horns Table |  |  |
| --- | --- | --- |
| FIELD | TYPE | DESCRIPTION |
| name | String | Enter a unique name for the horn or beacon. |
| hornio | PopUp | Select the output that activates the horn or beacon. |
| newseq | PopUp | Required. Select a timer to act as the sequence timer. This limits how often a horn/beacon can be re-fired if go is set to 1 (see below), defining a sort of overall ‘envelope’ for a single horn event, including ‘quiet time’ after the time defined in sound, below, if the newseq time is longer than the sound time. |
| sound | PopUp | Required. Select a timer to set the period during which the horn can sound. The horn will sound continuously for the duration of this timer unless ontime and offtime are used. See below. |
| ontime | PopUp | Optional. Both ontime and offtime must be defined to use this feature. The hornio output will be toggled between ON and OFF for the duration of the sound timer (ON first). The ontime timer defines the duration of the ON state. The offtime timer defines the duration of the OFF state. The effect is that of a pulsed horn or a blinking beacon. |
| offtime | PopUp |  |
| go | Int | A non-zero value activates the horn. The go flag is set by the following modules: - Energy Zone – sets it to 1 - Full-Jam Zone – sets it to 2 - Full Zone – set it to 2 - Lift Gate Zone – sets it to 1 - Motor Chains – sets it to 1 if Mtrchain column ResetHorn is set to “N”, or 2 if ResetHorn is set to “Y”. When set to 1, the horn will not start a new “sequence” until the newseq timer times out. When set to 2, a new sequence can begin at any time. |
| isdone | Int | Set by the software to indicate the state of the horn sequence. |

Integer flag #87 – FLAG_TESTHORNS can be set to activate additional logging.  All the horns log messages contain the word “horn”.