| cisco |  |
| --- | --- |
| # FortnaPlus Control IOCard Interfaces |  |
|  | Document Title |
| Document Revision: 1.15 |  |
| Date: 03-October-2022 |  |

Revision; Date; Author; Revision Summary. 1.0; Unknown; B C Conklin; New document. 1.1; 08-Aug-2014; N Martin; Added RTA EtherNet/IP module. 1.2; 21-Aug-2014; B C Conklin; Added a table of contents. 1.6; 10-Nov-2014; N Martin; Updated supported devices and parameter list. 1.7; 19-Feb-2015; B C Conklin; Updated the KTX node and address table. 1.8; 12-Jun-2015; B C Conklin; Added EIP configuration tool. 1.10; 11-May-2018; T R Kremser; Additional clarification on 1794 (Flex) IO parameters. 1.11; 10-Aug-2020; T R Kremser; Updates and general refresh. 1.12; 12-Aug-2020; T R Kremser; Add information about IP aliases. 1.13; 19-Feb-2021; T R Kremser; Fix error in QNX4 Profibus information. 1.14; 06-Sep-2022; T R Kremser; Add detail on OptoMux EtherNet brain boards.. 1.15; 03-Oct-2022; T R Kremser; Add bootpd configuration details.

Table of Contents

1.	Purpose	7

2.	Currently Supported IO Devices	7

2.1.	EtherNet/IP (RTA)	7

2.1.1.	File Locations	7

2.1.2.	XML Format	8

2.1.3.	Network Configuration	8

2.1.4.	EtherNet Switch Configuration	9

2.1.5.	IOCard Table Configuration for RTA EIP	10

2.1.6.	IO Device Configuration	11

2.1.7.	XML Number Format	12

2.1.8.	XML EthernetIP Element	12

2.1.9.	XML Adapter Element	13

2.1.10.	XML Module Element	14

2.1.11.	XML O2T Element	14

2.1.12.	XML T2O Element	14

2.1.13.	XML Key Element	15

2.1.14.	XML Parameters	15

	XML Parameters for Generic Adapter or Module	16

	XML Parameters for 1734-AENT and 1738-AENTR/B Devices	16

	XML Parameters for 1734-IB8 Devices	17

	XML Parameters for 1734-IEC2 Devices	17

	XML Parameters for 1734-OB8E Devices	18

	XML Parameters for PowerFlex70 and PowerFlex700 Devices	18

2.1.15.	XML Query Parameter Request	23

2.1.16.	Supported Modules List	24

2.1.17.	Suggested XML Configurations for Specific Devices	29

	1734-AENT and 1738-AENTR/B	30

	PowerFlex525	30

	PowerFlex70	31

	PowerFlex700	31

	20-COMM-E Module for PowerFlex	31

	Yaskawa SI-EN3 Extended Speed Control	32

	Yaskawa SI-EN3 High-Speed Torque Control	32

	SLC500 1747 with 1746 Modules	33

	Control Logix 1756	34

	Flex 1794	35

	Beckhoff EtherNet/IP Slave (TC-CX20X0 Processor)	38

	Beckhoff EtherNet/IP Slave (BK9105-1000)	39

	Opto22 G4EB2 Pamux	41

	Interroll Accumulation Controller	45

	Wago 750 TGW Wave Sorter Controller	50

	SEW Movifit 32 Word Vertiswitch Controller	53

2.1.18.	The IO Adapter Status Table [KTX_RIO_ADAPTERS]	55

2.1.19.	The EtherNet/IP Configuration Tool [EIPStatusScrn]	61

2.1.20.	Running Multiple Instances of the RTA Driver	70

2.2.	Optomux EtherNet (Opto22)	73

2.2.1.	File Locations	73

2.2.2.	XML Format	74

2.2.3.	Network Configuration	74

2.2.4.	Opto22 Board IP Configuration – Bootp	75

2.2.5.	Device Configuration	77

2.2.6.	XML Number Format	78

2.2.7.	XML Optomux Element	78

2.2.8.	XML Adapter Element	79

2.2.9.	XML Module Element	80

2.2.10.	G4EB2 Brain Board	80

2.2.11.	E1 Brain Board	80

2.2.12.	Importing a CSV File for Optomux EtherNet	81

2.2.13.	CSV File Example for Optomux EtherNet	81

2.3.	Profibus (Molex SST)	82

2.3.1.	Gather Necessary Profibus Items	83

2.3.2.	Install the Profibus Tools on QNX	83

2.3.3.	Configure the Profibus Card	83

2.3.4.	Configure FPC to use the Profibus Card – IOCard Table	84

2.3.5.	IO Adapter Status Table for Profibus [KTX_RIO_ADAPTERS]	86

2.3.6.	Profibus Configio Table	87

2.3.7.	Profibus Troubleshooting	88

2.4.	AC51 Pamux (Opto22)	88

2.4.1.	IOCard Table Configuration for AC51	89

2.5.	DeviceNet (Allen-Bradley)	90

3.	Retired IO Devices	103

3.1.	AC5 PCI (Opto22)	103

3.1.1.	IOCard Table Configuration for AC5	103

3.2.	AC28 Pamux ISA (Opto22)	105

3.3.	Access 120 PCI	106

3.4.	1784-KTX (Allen-Bradley)	109

3.5.	DeviceNet (Interbus S and SST)	126

3.6.	Optomux Serial (Opto22)	131

3.7.	HMS AnyBus Master EtherNet/IP Controller	133

4.	IOCard Watchdog	143

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Purpose

This document describes all the IO interface devices supported by FortnaPlus Control (FPC) and how to configure them.  Many devices have been supported by FPC over its lifetime, but the manufacturers will occasionally discontinue a particular device in favor of a new model.  The physical interfaces for the cards also come and go, so ISA cards were replaced by PCI cards, and now PCI has fallen out of favor.  Section 2 describes the currently supported devices.  Section 3 preserves the information for devices that are no longer in use.

# Currently Supported IO Devices

These IO devices are supported by the current version of FPC and are in active use at client sites.

## EtherNet/IP (RTA)

The FPC rtadrv EtherNet/IP driver is built upon the RTA software stack.  It provides EtherNet/IP capabilities using a standard EtherNet interface adapter.  The rtadrv driver is configured through the FPC IOCard table and an XML file created using the EtherNet/IP configuration tool (wizard) built in to FPC.

### File Locations

The rtadrv driver executable file is found in the RUN directory.

The XML configuration file that the rtadrv driver uses is found in the RUN/FORTNA directory.

The XML file name is generated as follows:
  <Machine Name>-<Configuration Name>-eipcfg.xml
  Example:  CP2-RTA-eipcfg.xml

The <Machine Name> is found on the lower left corner of the main screen on the Automation Controller (AC).  The <Configuration Name> is found in the IOCard table Name column for the row that is used to configure the rtadrv driver.

The rtadrv driver file is included in FPCUPD and FPCRUN tar archive files, so it will receive updates if they are available, along with updates to FPC.

Log files for the rtadrv driver, named rtaeip_log.<day>,  are created under the LOGS directory.

### XML Format

The device configuration file uses an XML format.  XML is a structured text scheme like HTML.  The file always begins with:
  <?xml version="1.0" encoding="UTF-8"?>
to indicate the format of the XML document.

Sections within the file are defined by XML tags, or elements.  Elements start with <element name> and end with </element name>.  If an element does not contain any other elements, a simplified single tag <element name /> can be used.

Each element can have attributes and values in the starting tag after the element name.  Attributes are in the form <element_name attribute=”value”>.  All values must be quoted.

Comments must be enclosed in a special tag, like this:
  <!--This is a comment -->

You should assume that all text is case-sensitive, so Element1 is not the same as element1.

Here is an example showing the form of these tags in an XML file.  The indentation is only for (human) readability:

<Element1 Attribute1=”value1”>
	<Element2 Attribute2=”value2” />
	<Element3 Attribute3a=”value3a” Attribute3b=”value3b”>
		<Element4 Attribute4=”value4” />
	</Element3>
</Element1>

### Network Configuration

The rtadrv driver requires a dedicated network interface (NIC).  That means that the AC will need at least two network interfaces – one for the IO network and one for access to the outside world.

The network interfaces are configured using the QNX Network Configuration tool.  As the root user, open the tool using Configure | Network on the QNX “shelf”, or run phlip from a command-line window.

Use the “Device 1” NIC for the connection to the outside world.  Use the “Device 2” NIC for the IO network connection.  Choose connection type “Manual”.  The IP address for the IO network is set by the Electrical department, but is usually 192.168.1.9 for a system using a single IO NIC.

### EtherNet Switch Configuration

An EtherNet switch is needed to connect the AC’s NIC to the IO adapters.  The current preferred option is a Cisco switch, which is configured and managed by the Infrastructure team, though it may be accessed by a web interface and ssh to aid in diagnosing problems.  Switches from SixNet have been used in many installations and can be accessed for inspection and configuration through a web interface.

We choose to set the IP address for the first switch at 192.168.1.1 and increment the last number for subsequent switches on the same AC, as needed.  Since the IO network on each AC is separate from the ones on other ACs, the same IP addresses are re-used – so each AC will have a switch at 192.168.1.1, and some might also have one at 192.168.1.2, and so on.

The Cisco switches can be accessed to display current connected devices, port speeds and other information using a web browser.  The MAC address table of connected devices is not included in the web interface (for some unknown reason!) but can be found by accessing the switch using ssh.  The command “show mac addr” will display the MAC table.  Other “show” commands, such as “show arp” are also useful.  The Cisco switches are set up with login/password = admin/fortnaSW11.

We have used several different models of SixNet switches with various numbers of ports.  They all are accessible, and fully configurable, via their web interfaces.  We leave the login/password at the default of admin/admin.

Both types of switches may have 100 Mbps and 1,000 Mbps ports.  All IO adapters currently in use operate at 10 Mbps or 100 Mbps, so there is no point in using the 1,000 Mbps switch ports for connection to adapters.  The 1,000 Mbps ports should be reserved for switch-to-switch connections, if needed.

These switches are configured with a gateway of none.  All devices on this network will be addressed by IP address.  No traffic on this network will leave the network.  Configure the ports for auto-negotiate, and flow control = ON.

See the manufacturer’s device manual for additional details on configuring a specific switch model.

Adapters are configured with a gateway of none, or default (192.168.1.1).  Configure the ports for auto-negotiate.  Use a netmask of 255.255.255.0.

### IOCard Table Configuration for RTA EIP

The FPC IOCard table defines the connection between FPC and the RTA driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the RTA driver.

| IOCard Table – RTA EIP |  |
| --- | --- |
| Column | Value or Description |
| Name | The name for this configuration. The name must match the <Config> part of the <Machine>-<Config>-eipcfg.xml file name. |
| Desc | An optional description for this configuration. |
| Type | Choose “RTA EtherNet/IP”. |
| pStatus | Set by the software. Shows the status of the driver. |
| InputMenu | Choose one of the InputData[1-5] tables. Make sure no other IO card entry on this AC uses the same table. |
| OutputMenu | Choose one of the OutputData[1-5] tables. Make sure no other IO card entry on this AC uses the same table. |
| Process | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| MaxInput | Set by the software. NOTE: If the InputData table is not large enough for the defined number of inputs, MaxInput will be set to the size of the table. If this occurs, increase the size of the InputData table. |
| MaxOutput | Set by the software. NOTE: If the OutputData table is not large enough for the defined number of outputs, MaxOutput will be set to the size of the table. If this occurs, increase the size of the OutputData table. |
| LoopCnt | Set by the software. The value will increment when the line is active. |
| Machine | Choose the name of this machine. |
| PCIDeviceIndex | Set this to 0 unless more than one instance of the RTA driver is being used. See section 2.1.20 for more details. |
| LogLevel | “Level 1” is recommended for production. |
| ErrorOutput | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Internal storage used by the RunDelay timer. |
| ABS_IP | Not used by the RTA EtherNet/IP driver. The IP address is set in the *eipcfg.xml file, under element “EthernetIP”, attribute “interfaceip”. See section 2.1.8 for details. |

### IO Device Configuration

The device configuration is specified by the <Machine>-<Config>-eipcfg.xml file found in the RUN/FORTNA directory.  It is best to use the configuration wizard found in FPC on the IO Utilities menu under “Ethernet IP”.  The Electrical department will prepare CSV files to expedite creation of the configuration using this method.  The resulting files are text, however, and can be inspected with any text editor.  Alternatively, a free XML editor called gedit can be found at: http://ftp.gnome.org/pub/GNOME/binaries/win32/gedit/

Here is a brief example of the XML file contents.  It describes a single Flex IO adapter with one 16-point AC input module:

<?xml version=”1.0” encoding=”UTF-8”?>
<EthernetIP name=”CP2” version=”1.0” interfaceip=”192.168.1.9”>
	<Adapter	targetip=”192.168.1.51”
		name=”1794-AENT-1”
		InputAddress=”0”
		OutputAddress=”0”
		<Module name=”1794-AENT-1”
			type=”1794-AENT”
			slot=”0”
			connection=”HEADNODE”>
			<O2T rpi_us=”20000” cnxn_type=”POINT2POINT”/>
			<T2O rpi_us=”20000” cnxn_type=”POINT2POINT”/>
			<Key Compatibility=”false” VendorID=”0”
				DeviceType=”0” ProductCode=”0”
				MajorRevision=”0” MinorRevision=”0”/>
		</Module>
		<Module name=”1794-IA16-2”
			type=”1794-IA16”
			slot=”1”
			connection=”BRIDGED”
		</Module>
	</Adapter>
</EthernetIP>

### XML Number Format

Many of the numeric parameters in the XML file support decimal, octal, or hexadecimal formats.  Decimal number appear without a prefix, like 3458.  Octal numbers are denoted with a leading zero, like 06602.  Hexadecimal numbers are denoted with a leading “0x”, like 0xD82.

### XML EthernetIP Element

The <EthernetIP> element in the XML file indicates the start of the EtherNet/IP configuration.  It has the following attributes:

| EthernetIP Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the entire EtherNet/IP configuration. The name is used in the rtaeip_log.<day> log file and should contain the name of the machine, area, or control panel. |
| version | The version number is to support future changes to the format of the configuration file. The current version “1” should be used. |
| interfaceip | The IP Address of the QNX Ethernet NIC card connected to the I/O panel. The IP Address “192.168.1.9” is recommended. |

### XML Adapter Element

The <Adapter> element in the XML file is contained within the <EthernetIP> element.  It has the following attributes:

| Adapter Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the entire Adapter configuration. The name is used in the rtaeip_log.<day> log file and should contain the name of the device and/or IP address. The name is used in the IO Adapters Status [KTX_RIO_ADAPTERS] table and should be unique. |
| targetip | The IP address of the EtherNet/IP device the driver will connect to. See the sections, below, for each adapter type to determine how to set the device’s IP address. Assigned by the Electrical department. |
| InputAddress | The offset into the table selected in IOCard_InputMenu where the devices on the adapter will start. The offset is in bytes. Always start the InputAddress attribute on an even number. Unused buffer space can be left between Adapter elements. In RSNetWorx, the offset value is the Scanlist Configuration Input Address column multiplied by 2. Decimal, octal, and hexadecimal values are supported. |
| OutputAddress | The offset into the table selected in IOCard_OutputMenu where the devices on the adapter will start. The offset is in bytes. Always start the OutputAddress attribute on an even number. Unused buffer space can be left between Adapter elements. In RSNetWorx the offset value is the Scanlist Configuration Input Address column multiplied by 2. Decimal, octal, and hexadecimal values are supported. |

### XML Module Element

The <Module> element in the XML file is contained within the <Adapter> element.  It has the following attributes:

| Module Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the module configuration. The name attribute is used in the rtaeip_log.<day> log file and should contain the name of the device and/or slot. |
| type | The device type in the slot. Note that slot 0 is always the same as the adapter. The following devices are supported for slot 0: “1783-EMS”, "1734-AENT", “1738-AENTR/B”, “PowerFlex525”, “PowerFlex70”, “PowerFlex700”, “YASKAWA-SI-EN3-ESC”, and “YASKAWA-SI-EN3-HSTC”. The remaining slots support the modules listed in section 2.1.16 |
| slot | The slot number of the device. The adapter is always device 0. The devices should be configured in numeric order. Decimal, octal, and hexadecimal values are supported. |
| connection | The connection type for the device. The adapter in slot 0 is always the “HEADNODE”. Currently, “RACKOPTIMIZED” is the only supported connection type for slots greater than 0. |

### XML O2T Element

The <O2T> element in the XML file is contained within the <Module> element and is only supported for slot 0 “HEADNODE”.  It configures the Origin to Target (output) parameters.  It has the following attributes:

| O2T Element |  |
| --- | --- |
| Attribute | Value |
| rpi_us | The Requested Packet Interval in microseconds. The lower the value the higher the load on the adapter. |
| cnxn_type | The IP connection type. Currently only “POINT2POINT” is supported for O2T connections. |

### XML T2O Element

The <T2O> element in the XML file is contained within the <Module> element and is only supported for slot 0 “HEADNODE”.  It configures the Target to Origin (input) parameters.  It has the following attributes:

| T2O Element |  |
| --- | --- |
| Attribute | Value |
| rpi_us | The Requested Packet Interval in microseconds. The lower the value the higher the load on the adapter. |
| cnxn_type | The IP connection type. “POINT2POINT” and “MULTICAST” are supported for T2O connections. Only 20 “MULTICAST” connections are allowed. Use “POINT2POINT” if more than 20 connections are needed. |

### XML Key Element

The <Key> element in the XML file is contained within the <Module> element and is only supported for slot 0 “HEADNODE”.  The remaining modules in an adapter do not need this.  The Key element prevents connections to older revisions of devices.  It has the following attributes:

| Key Element |  |
| --- | --- |
| Attribute | Value |
| Compatibility | Determines if the version should be restricted. “true” or “false”. “false” is recommended unless a particular version of the hardware is required. If set to “true” the VendorID, DeviceType, and ProductCode must match the hardware exactly. The MajorRevision and MinorRevision of the hardware’s firmware must be greater than or equal to the values in the configuration file. |
| VendorID | The vendor ID of the hardware. Set to “0” if Compatibility is “false”. |
| DeviceType | The device type of the hardware. Set to “0” if Compatibility is “false”. |
| ProductCode | The product code of the hardware. Set to “0” if Compatibility is “false”. |
| MajorRevision | The major revision number of the firmware. Set to “0” if Compatibility is “false”. |
| MinorRevision | The minor revision number of the firmware. Set to “0” if Compatibility is “false”. |

### XML Parameters

The <Parameters> element in the XML file is contained within the <Module> element.  It is used to specify module-specific parameters.  The parameters are sent to the modules during initialization.  Log messages are generated as each parameter is sent, indicating if the parameter was successfully set or not.  The tables in this section describe the parameters supported by each type of module.

Note:  As a shorthand notation, “Parameter[0-8]” indicates nine individual parameters formatted as Parameter0=”Value0”, Parameter1=”Value1”, …

Note:  If multiple parameter entries are needed, add a number to create a unique identity for the XML parser.
Example:
  ClassInstAttrParamMod1=”0xA,1,3,1”  (set attribute 3)
  ClassInstAttrParamMod2=”0xA,1,4,1”  (set attribute 4)

### XML Parameters for Generic Adapter or Module

A generic command allows any parameter to be sent to a module.  Use the following parameter types if the module’s parameter is not found in the tables below.

| Parameter Element -- Generic |  |
| --- | --- |
| Attribute | Value |
| ClassInstAttrParam | Used to send a parameter to a HEADNODE. Example: Set the maximum frequency for a PowerFlex70 drive to 65 Hz. ClassInstAttrParam=”0x93,55,0x9,65” |
| ClassInstAttrParamMod | Used to send a parameter to a module that is connected to a HEADNODE. Uses the same port as the forward open. Example: Set the 1734-OB8E “Reset Mode” for channel 0. The channel number is found in the Instance field (Channel 0 – Instance 1) ClassInstAttrParamMod=”0x9,1,0x76,1” |
| ClassInstAttrPortParam | Used to send a parameter to a module that is connected to a HEADNODE using a specified port. This is used for modules that communicate through a port other than the one used by the forward open command. Example: ClassInstAttrPortParam=”0xA,1,3,1,4” |

### XML Parameters for 1734-AENT and 1738-AENTR/B Devices

The 1734-AENT and 1738-AENTR/B adapters support different alignments for the data transfer.  This alignment can be BYTE, WORD, DOUBLE, or FIXED.  The RTA EtherNet/IP driver only supports WORD.  This format was chosen to maintain compatibility with systems using HMS cards.

| Parameters Elements for 1734-AENT and 1738-AENTR/B |  |
| --- | --- |
| Attribute | Value |
| T2OAlignment | Currently only WORD alignment is supported. EtherNet/IP devices with WORD or DOUBLE data will start at even offsets. The adapter will put a pad byte before the data to align the data. Warning: This can cause the size and offset of data to change just by switching the order of two modules in the rack. |
| T2OSlotSize | Set to 1. This field is currently not used. When FIXED alignment is supported, the field will contain the size of the slot for each module in the Target to Origin (input) direction. |
| O2TAlignment | Currently only WORD alignment is supported. EtherNet/IP devices with WORD or DOUBLE data will start at even offsets. The adapter will put a pad byte before the data to align the data. Warning: This can cause the size and offset of data to change just by switching the order of two modules in the rack. |
| O2TSlotSize | Set to 1. This field is currently not used. When FIXED alignment is supported, the field will contain the size of the slot for each module in the Origin to Target (output) direction. |

### XML Parameters for 1734-IB8 Devices

| Parameters Elements for 1734-IB8 |  |
| --- | --- |
| Attribute | Value |
| Off2OnFilter[0-7] | The 1734-IB8 contains logic to filter OFF to ON transitions shorter than the specified time, in microseconds. Note that use of this feature is not recommended. A full clear timer in FPC can be used instead. |
| On2OffFilter[0-7] | The 1734-IB8 contains logic to filter ON to OFF transitions shorter than the specified time, in microseconds. Note that use of this feature is not recommended. A full clear timer in FPC can be used instead. |

### XML Parameters for 1734-IEC2 Devices

The 1734-IE2C is an analog current input module.  It is often used with a height detection laser sensor in FPC.  The module, by default, uses a range of 4 mA to 20 mA.  The module returns a signed 16-bit value.
Warning:  FPC expects unsigned values, so a signed negative value will appear like a very large positive unsigned value in FPC.

| Parameters Elements for 1734-IE2C |  |
| --- | --- |
| Attribute | Value |
| LowScaling[0-1] | Digital value at 4 mA. Default is 3277. If the measurement falls below 4 mA, a value less than the LowScaling value will be returned. |
| HighScaling[0-1] | Digital value at 20 mA. Default is 16383. Maximum is 32767. If the measurement is greater than 20 mA, a value more than the HighScaling value will be returned. |
| LowAlarm[0-1] | When the measurement is below the LowAlarm value the LowAlarm bit will be set. Default is 3113. |
| HighAlarm[0-1] | When the measurement is above the HighAlarm value the HighAlarm bit will be set. Default is 16547. |
| LowLowAlarm[0-1] | When the measurement is below the LowLowAlarm value the LowLowAlarm bit will be set. Default is 2867. |
| HighHighAlarm[0-1] | When the measurement is above the HighHighAlarm value the HighHighAlarm bit will be set. Default is 16793. |

### XML Parameters for 1734-OB8E Devices

| Parameters Element “1734-OB8E” |  |
| --- | --- |
| Attribute | Value |
| EnableNoLoad[0-7] | The 1734-OB8E module contains logic to detect when an output is either short-circuited or not connected. This parameter controls the fault detection for No Load (not connected). This is useful for unused outputs and can be helpful during debugging by eliminating misleading distractions. By default, unused outputs will blink red, which can be confusing. With this parameter set to true, only short-circuits will blink. false – When no load is detected indicate a channel fault on the module (blink red). This is the default. true – When no load is detected do not indicate a channel fault. |

### XML Parameters for PowerFlex70 and PowerFlex700 Devices

Parameter values for PowerFlex70 and PowerFlex700 devices are provided by the Electrical department and appear on the drawings.  Parameters for these devices and for the 20-COMM-E EtherNet/IP communications module that connects them to the network are all sent through the device connection.  Please consult 20-comm-um010_-en-p.pdf for a detailed description of the 20-COMM-E parameters.  Also see section -1508341504.566.-1508341504..  Consult 20a-um001_-en-p.pdf for a detailed description of the PowerFlex70 and PowerFlex700 parameters.  Each parameter name ends with the parameter number to assist in finding the description in the documentation.  The most commonly used parameters are described in the following table.

| Parameters Elements for PowerFlex70 and PowerFlex700 |  |
| --- | --- |
| Attribute | Value |
| Bootp3 | This is 20-COMM-E parameter number 3. Disabled – Do not use the BootP service to set the IP address. The IP address must be set manually. Enabled – Use a BootP service to set the IP address. |
| EnRateCfg16 | This is 20-COMM-E parameter number 16. This parameter configures the speed and duplex of the EtherNet connection. The PowerFlex device must be power-cycled for this setting to take effect. Autodetect – Auto-negotiate the speed and duplex. 10MbpsFull – 10 Mbps Full Duplex. 10MbpsHalf – 10 Mbps Half Duplex. 100MbpsFull – 100 Mbps Full Duplex. 100MbpsHalf – 100 Mbps Half Duplex |
| CommFltAction21 IdleFltAction22 | CommFltAction21 is 20-COMM-E parameter number 21. CommFltAction21 configures the behavior of the drive when EtherNet/IP communications are disconnected. IdleFltAction22 is 20-COMM-E parameter number 22. IdleFltAction22 configures the behavior of the drive when the connection forces the state to idle. Note: This happens if the AC software crashes and just before exit. The parameter values are the same for CommFltAction21 and IdleFltAction22. Fault – Fault the drive. The drive is faulted and stopped. Stop – Stop the drive. The drive is stopped but not faulted. ZeroData – See 20comm-um010_-en-p.pdf section 3-14. HoldLast – See 20comm-um010_-en-p.pdf section 3-14. SendFltCfg – See 20comm-um010_-en-p.pdf section 3-14. |
| DpiIoCfg23 | DpiIoCfg23 is 20-COMM-E parameter number 23. DpiIoCfg23 configures the IO that is transferred through the adapter. The parameter is an octal, decimal, or hexadecimal number. The PowerFlex device must be power-cycled for this setting to take effect. |
| MSInput35 | MSInput35 is 20-COMM-E parameter number 35. MSInput35 configures the data consumed by the adapter. The parameter is an octal, decimal, or hexadecimal number. The PowerFlex device must be power-cycled for this setting to take effect. |
| MSOutput36 | MSInput36 is 20-COMM-E parameter number 36. MSInput35 configures the data produced by the adapter. The parameter is an octal, decimal, or hexadecimal number. The PowerFlex device must be power-cycled for this setting to take effect. |
| AccessControl54 | AccessControl54 is 20-COMM-E parameter number 54. AccessControl54 configures web and email access for series A adapters with firmware 2.003 or earlier. The parameter is an octal, decimal, or hexadecimal number. The PowerFlex device must be power-cycled for this setting to take effect. |
| AccessControl56 | AccessControl56 is 20-COMM-E parameter number 56. AccessControl56 configures email access for series-B adapters with firmware 3.xxx or later. The parameter is an octal, decimal, or hexadecimal number. The PowerFlex device must be power-cycled for this setting to take effect. |
| MotorNpVolts41 | MotorNpVolts41 is PowerFlex parameter number 41. MotorNpVolts41 is the nameplate voltage rating of the motor. The value is specified as a float. Example: 460 VAC on the name plate is “460.0” in the configuration file. The value accuracy is 0.1 volts. |
| MotorNpFla42 | MotorNpFla42 is PowerFlex parameter number 42. MotorNpFla42 is the nameplate full load amperage rating of the motor. The value is specified as a float. Example: 2.9 amps on the name plate is “2.9” in the configuration file. The value accuracy is 0.1 amps. |
| MotorNpHz43 | MotorNpHz43 is PowerFlex parameter number 43. MotorNpHz43 is the nameplate rated frequency. The value is specified as a float. Example: 60 Hz on the name plate is “60.0” in the configuration file. The value accuracy is 0.1 Hz. |
| MotorNpRpm44 | MotorNpRpm44 is PowerFlex parameter number 44. MotorNpRpm44 is the nameplate rated motor RPM. The value is specified as an integer. Example: 1755 RPM on the name plate is “1755” in the configuration file. The value accuracy is 1 RPM. |
| MotorNpPower45 | MotorNpPower45 is PowerFlex parameter number 45. MotorNpPower45 is the nameplate rated power. The value is specified a float. Example: 1.5 HP on the name plate is “1.50” in the configuration file. The value accuracy is 0.01. Note: The units for MotorNpPower45 are specified in parameter MotorNpPwrUnits46. |
| MotorNpPwrUnits46 | MotorNpPwrUnits46 is PowerFlex parameter number 46. MotorNpPwrUnits46 specifies the units for MotorNpPower45. Horsepower – Parameter MotorNpPower45 is expressed in horsepower. kiloWatts – Parameter MotorNpPower45 is expressed in kilowatts. |
| MaximumFreq55 | MaximumFreq55 is PowerFlex parameter number 55. MaximumFreq55 specifies the highest frequency the drive can output. The value is specified as a float. Example: 60 Hz on the name plate is “60.0” in the configuration file. The value accuracy is 0.1 Hz. |
| Compensation56 | Bit 3 1 Add 8 Bit 2 0 Add 4 Bit 1 1 Add 2 Bit 0 1 Add 1 Compensation56 is PowerFlex parameter number 56. Compensation56 enables or disables compensation for driving the motor. The value is specified in octal, decimal, or hexadecimal. Hexadecimal is the recommended type. It may be necessary to translate the value from binary to hexadecimal. To translate, take each group of 4 bits (nibble) and convert to a single hex digit. Example: Electrial specifies xxxx xx00 0001 1011 (x should be treated as 0). Conversion: 0 to 9 directly converts to hex. 10 decimal = A hex 11 decimal = B hex 12 decimal = C hex 13 decimal = D hex 14 decimal = E hex 15 decimal = F hex Binary 1011 = 11 decimal = 0xB. Binary xxxx xx00 0001 1011 = 0x001B |
| MaximumSpeed82 | MaximumSpeed82 is PowerFlex parameter number 82. MaximumSpeed82 specifies the highest speed the drive can output after scaling. The value is specified as a float. Example: 80 Hz is “80.0” in the configuration file. The value accuracy is 0.1 Hz. |
| SpeedRefASel90 | SpeedRefASel90 is PowerFlex parameter number 90. SpeedRefASel90 selects the source of the speed reference. Values are: AnalogIn1, AnalogIn2, Encoder, MOPLevel, PresetSpd1, PresetSpd2, PresetSpd3, PresetSpd4, PresetSpd5, PresetSpd6, PresetSpd7, DpiPort1, DpiPort2, DpiPort3, DpiPort5, ScaleBlock1, ScaleBlock2, HiResRef. |
| AccelTime1_140 | AccelTime1_140 is PowerFlex parameter number 140. AccelTime1_140 specifies the acceleration time of the drive. The value is specified as a float. Example: 0.1 seconds is “0.1” in the configuration file. The value accuracy is 0.1 seconds. |
| DecelTime1_142 | DecelTime1_142 is PowerFlex parameter number 142. DecelTime1_142 specifies the deceleration time of the drive. The value is specified as a float. Example: 0.1 seconds is “0.1” in the configuration file. The value accuracy is 0.1 seconds. |
| DcBrakeLvl158 | DcBrakeLvl158 is PowerFlex parameter number 158. DcBrakeLvl158 specifies the amount of brake current for the drive. The value is specified as a float. Example: 5.0 amps is “5.0” in the configuration file. The value accuracy is 0.1 amps. |
| DcBrakeTime159 | DcBrakeTime159 is PowerFlex parameter number 159. DcBrakeTime159 specifies brake current injection time for the drive. The value is specified as a float. Example: 0.1 seconds is “0.1” in the configuration file. The value accuracy is 0.1 seconds. |
| BusRegModeA161 | BusRegModeA161 is PowerFlex parameter number 161. BusRegModeA161 selects the active bus regulation mode. Values are: Disabled – Disables active bus regulation. AdjustFreq – Use frequency adjust. DynamicBrak – Use dynamic brake. BothDB1st – Use frequency adjust and dynamic brake but use dynamic brake first. BothFrq1st – Use frequency adjust and dynamic brake but use frequency adjust first. |
| DbResistorType163 | DbResistorType163 is PowerFlex parameter number 163. DbResistorType163 selects if a dynamic brake resistor is used. Values are: InternalRes – Use internal resistor. ExternalRes – Use external resistor. None – No dynamic brake resistor. |
| DirectionMode190 | DirectionMode190 is PowerFlex parameter number 190. DirectionMode190 selects the method for changing the drive’s direction. Values are: Unipolar – Uses drive logic. Bipolar – Uses sign of reference. ReverseDis – Not changeable. |
| ParamAccessLvl196 | ParamAccessLvl196 is PowerFlex parameter number 196. ParamAccessLvl196 selects the parameter display level viewable on the HIM. Values are: Basic – Reduced parameter set. Advanced – Full parameter set. Reserved – Full parameter set and engineering parameters. |
| Language201 | Language201 is PowerFlex parameter number 201. Language201 selects the language on the HIM. Values are: NotSelected, English, Francais, Espanol, Italiano, Deutsch, Portugues, Nederlands |
| DpiPortSelect274 | DpiPortSelect274 is PowerFlex parameter number 274. DpiPortSelect274 selects the port reference values that appears in parameter 275. Values are: NotUsed, DpiPort1, DpiPort2, DpiPort3, Reserved, DpiPort5 |
| DigitalIn1Sel361 DigitalIn2Sel362 DigitalIn3Sel363 | DigitalIn1Sel361 is PowerFlex parameter number 361. DigitalIn2Sel362 is PowerFlex parameter number 362. DigitalIn3Sel363 is PowerFlex parameter 363. Selects the function for the digital input. Values are: NotUsed, Enable, ClearFaults, AuxFault, StopCF, Start, FwdReverse, Run, RunFoward, RunReverse, Jog, JogFoward, JogReverse, StopModeB, BusRegMdB, SpeedSel1, SpeedSel2, SpeedSel3, AutoManual, Local, Acc2AndDec2, Accel2, Decel2, MopInc, MopDec, ExclLink, PiEnable, PiHold, PiReset, PrechargeEn, SpdTrqSel1, SpdTrqSel2, SpdTrqSel3, Jog2, PiInvert, UserSetSel1, UserSetSel2, RunLevel, RunFwdLevel, RunRevLevel, RunWComm, SyncEnable, TraverseEna, ManualAuto |

### XML Query Parameter Request

The <Query> element in the XML file is contained within the <Module> element.  This element provides the ability to read module-specific parameters.  The requested parameters are read during startup, and the results are found in the log file.  It has the following attributes:

The only device type that fully honors this request is the PowerFlex.  Other devices will honor one or more of the following:

| Query Element -- Generic |  |
| --- | --- |
| Attribute | Value |
| ClassInstAll | Read the HEADNODE device’s identity. Example: ClassInstAll=”1,1” |
| ClassInstPortAll | Read a module’s identity through a HEADNODE at a specific port. Example: ClassInstPortAll=”1,1,1” |
| ClassInstAttrParam | Read a parameter from the HEADNODE. Example: Read the maximum frequency for a PowerFlex70 drive: ClassInstAttrParam=”0x93,55,0x9” |
| ClassInstAttrMod | Read a parameter from a module connected through a HEADNODE using the same port as the forward open. Example: Read the 1734-OB8E “reset mode” for channel 0. The channel number is found in the Instance field: ClassInstAttrMod=”0x9,1,0x76” |
| ClassInstAttrPort | Read a parameter from a module connected to a HEADNODE using the selected port. Example: ClassInstAttrPort=”0xA,1,3,1” |

Note:  If multiple parameter entries are needed, add a number to create a unique identity for the XML parser.
Example:
  ClassInstAttrPort1=”0xA,1,3,1”  (read attribute 3)
  ClassInstAttrPort2=”0xA,1,4,1”  (read attribute 4)

### Supported Modules List

The following modules are supported by the FPC rtadrv EtherNet/IP driver.

| Supported Module List (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Part Number | Type | Description |
| 1734-AENT | Point I/O | Adapter (8 Input Banks, 0 Output Banks – Word Aligned) |
| 1734-AENTR | Point I/O | Adapter (8 Input Banks, 0 Output Banks – Word Aligned) |
| 1734-IV8 | Point I/O | 8 Point 24V Input (1 Input Bank, 0 Output Banks) |
| 1734-IB8 | Point I/O | 8 Point 24V Input (1 Input Bank, 0 Output Banks) |
| 1734-OB8 | Point I/O | 8 Point 24V Output (1 Input Bank, 1 Output Bank) |
| 1734-OB8E | Point I/O | 8 Point 24V Output (1 Input Bank, 1 Output Bank) |
| 1734-IE8C | Point I/O | 8 Point Analog Current Input (24 Input Banks, 0 Output Banks) |
| 1734-OE4C | Point I/O | 4 Point Analog Current Output (4 Input Banks, 8 Output Banks) |
| 1734-IA4 | Point I/O | 4 Point 120VAC Input (1 Input Bank, 0 Output Banks) |
| 1734-OA4 | Point I/O | 4 Point 120VAC Output (0 Input Banks, 1 Output Bank) |
| 1734-IE2C | Point I/O | 2 Point current Input (6 Input Banks, 0 Output Banks) |
| 1734-OW4 | Point I/O | 4 Point Relay Output (0 Input Banks, 1 Output Bank) |
| 1734-AENT-LS | Point I/O | Adapter (8 Input Banks, 0 Output Banks – Word Aligned) |
| 1734-OB8E-LS | Point I/O | 8 Point 24V Output (1 Input Bank, 1 Output Bank) |
| 1734-OA4-LS | Point I/O | 4 Point 120VAC Output (0 Input Banks, 1 Output Bank) |
| 1734-OW4-LS | Point I/O | 4 Point Relay Output (0 Input Banks, 1 Output Bank) |
| 1734-OV8 | Point I/O | 8 Point 24VDC Output (1 Input Bank, 1 Output Bank) (The module part name is actually 1734-OV8E) |
| 1738-AENTR/B | Armour Point | Adapter (8 Input Banks, 0 Output Banks – Word Aligned) |
| 1738-IB16DM12 | Armour Point | 16 Point DC Input (3 Input Banks, 0 Output Banks) |
| 1738-OB16EM12 | Armour Point | 16 Point DC Output (1 Input Bank, 2 Output Banks) |
| 1738-OW4M12 | Armour Point | 4 Point Relay Output (0 Input Banks, 1 Output Bank) |
| 1738-IB8M8 | Armour Pont | 8 Point Module Input (1 Input Bank, 0 Output Bank) |
| 1738-IB8M12 | Armour Pont | 8 Point Module Input (1 Input Bank, 0 Output Bank) |
| 1738-IB8M23 | Armour Pont | 8 Point Module Input (1 Input Bank, 0 Output Bank) |
| 1738-OB8EM8 | Armour Pont | 8 Point Module Output (1 Input Bank, 1 Output Bank) |
| 1738-OB8EM12 | Armour Pont | 8 Point Module Output (1 Input Bank, 1 Output Bank) |
| 1794-AENT | Flex | Adapter (4 Input Banks, 0 Output Banks) |
| 1794-IA16 | Flex | 16 Point AC Input (2 Input Banks, 2 Output Banks) |
| 1794-OA8 | Flex | 8 Point AC Output (2 Input Banks, 2 Output Banks) |
| 1794-IB16 | Flex | 16 Point 24VDC Input (2 Input Banks, 2 Output Banks) |
| 1794-IB32 | Flex | 32 Point 24VDC Input (2 Input Banks, 2 Output Banks plus 8 Direct Input Banks, 0 Direct Output Banks) |
| 1794-IB10XOB6 | Flex | 10 Point Input, 6 Point Output 24VDC (2 Input Banks, 2 Output Banks) |
| 1794-OB16P | Flex | 16 Point 24VDC Input (2 Input Banks, 2 Output Banks) |
| 1794-IB8 | Flex | 8 Point 24VDC Input (2 Input Banks, 2 Output Banks) |
| 1794-OB8 | Flex | 8 Point 24VDC Output (2 Input Banks, 2 Output Banks) |
| 1794-OA8I | Flex | 8 Point AC Output (2 Input Banks, 2 Output Banks) |
| 1794-OW8 | Flex | 8 Point Relay Output (2 Input Banks, 2 Output Banks) |
| 1794-OB32P | Flex | 32 Point 24VDC Output (2 Input Banks, 2 Output Banks plus 4 Direct Input Banks, 4 Direct Output Banks) |
| 1794-IE8 | Flex | 8 Point Analog Input, current or voltage (2 Input Banks, 2 Output Banks plus 22 Direct Input Banks, 0 Direct Output Banks) |
| 1794-ID2 | Flex | Frequency input module (2 Input Banks, 2 Output Banks plus 20 Direct Input Banks, 10 Direct Output Banks) |
| 1747-AENTR | SLC | Adapter (0 Direct Input Banks, 0 Direct Output Banks) |
| 1746-IA16 | SLC | 16 Point AC Input (2 Direct Input Banks, 0 Direct Output Banks) |
| 1746-OA16 | SLC | 16 Point AC Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-ITB16 | SLC | 16 Point Isolated Input (2 Direct Input Banks, 0 Direct Output Banks) |
| 1746-OX8 | SLC | 8 Point Relay Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-OA8 | SLC | 8 Point AC Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-OW8 | SLC | 8 Point Relay Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-IB16 | SLC | 16 Point DC Input (2 Direct Input Banks, 0 Direct Output Banks) |
| 1746-OB16 | SLC | 16 Point DC Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-OB16E | SLC | 16 Point DC Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-IB32 | SLC | 32 Point DC Input (4 Direct Input Banks, 0 Direct Output Banks) |
| 1746-OB32 | SLC | 32 Point DC Output (0 Direct Input Banks, 4 Direct Output Banks) |
| 1746-IB8 | SLC | 8 Point DC Input (2 Direct Input Banks, 0 Direct Output Banks) |
| 1746-OW16 | SLC | 16 Point Relay Output (0 Direct Input Banks, 2 Direct Output Banks) |
| 1746-ITV16 | SLC | 16 Point Fast Response Input (2 Direct Input Banks, 0 Direct Output Banks) |
| 1746-SPARE | SLC | Spare Module allows empty slots in the rack (0 Direct Input Banks, 0 Direct Output Banks) |
| 1756-EN2T-04 | Control Net | 4 Slot Rack (36 Input Banks, 20 Output Banks) |
| 1756-EN2T-05 | Control Net | 5 Slot Rack (44 Input Banks, 24 Output Banks) |
| 1756-EN2T-07 | Control Net | 7 Slot Rack (60 Input Banks, 32 Output Banks) |
| 1756-EN2T-10 | Control Net | 10 Slot Rack (84 Input Banks, 44 Output Banks) |
| 1756-EN2T-13 | Control Net | 13 Slot Rack (108 Input Banks, 56 Output Banks) |
| 1756-EN2T-17 | Control Net | 17 Slot Rack (140 Input Banks, 72 Output Banks) |
| 1756-IA16 | Control Net | 16 Point AC Input (8 Input Banks, 4 Output Banks) Change of state filters are hard-coded to 1 ms ON and 9 ms OFF |
| 1756-OA16 | Control Net | 16 Point AC Output (8 Input Banks, 4 Output Banks) |
| 1756-OA8 | Control Net | 8 Point AC Output (8 Input Banks, 4 Output Banks) |
| 1756-OB16E | Control Net | 16 Point DC Output (8 Input Banks, 4 Output Banks) |
| 1756-IB16I | Control Net | 16 Point DC Input (8 Input Banks, 4 Output Banks) Change of state filters are hard-coded to 1 ms ON and 1 ms OFF |
| 1756-IA32 | Control Net | 32 Point DC Input (8 Input Banks, 4 Output Banks) Change of state filters are hard-coded to 1 ms ON and 9 ms OFF |
| 1756-IB32 | Control Net | 32 Point DC Input (8 Input Banks, 4 Output Banks) Change of state filters are hard-coded to 1 ms ON and 1 ms OFF |
| 1756-OB32 | Control Net | 32 Point DC Output (8 Input Banks, 4 Output Banks) |
| 1756-SPARE | Control Net | Spare Module allows empty space in the rack (8 Input Banks, 4 Output Banks) |
| 1783-EMS | Rockwell | AB Switch (48 Input Banks, 0 Output Banks) |
| PowerFlex40 |  | Power Flex 40 (8 Input Banks, 4 Output Banks – Word Aligned) |
| PowerFlex525 | Rockwell | Power Flex VFD (8 Input Banks, 4 Output Banks – Word Aligned) |
| PowerFlex70 | Rockwell | Power Flex 70 (8 Input Banks, 4 Output Banks – Word Aligned) |
| PowerFlex700 | Rockwell | Power Flex 700 (8 Input Banks, 4 Output Banks – Word Aligned) |
| VU4A0004 | Yaskawa | Yaskawa VFD (44 Input Banks, 44 Output Banks – Word Aligned) |
| YASKAWA-SI-EN3-ESC | Yaskawa | Yaskawa VFD (4 Input Banks, 4 Output Banks – Word Aligned) |
| YASKAWA-SI-EN3-HSTC | Yaskawa | Yaskawa VFD (44 Input Banks, 44 Output Banks – Word Aligned) |
| INTERROLL-1101834 | Interroll | Interroll Accumulation Controller – controls four accumulation motors and has four Photoeye inputs (36 Input Banks, 10 Output Banks – Word Aligned) |
| PANTHER-WAGO-750 | Panther PnA | Panther PnA Interface (4 Input Banks, 4 Output Banks – Word Aligned) Config Inst 1 (0x01 Hex) Input Inst 110 (0x6E Hex) Output Inst 111 (0x6F Hex) |
| PANTHER-PREDATOR-9E | Panther PnA | Panther Predator EIP-9E PnA Interface (32 Input Banks, 32 Output Banks – Word Aligned) Config Inst 102 (0x66 Hex) Input Inst 100 (0x64 Hex) Output Inst 101 (0x65 Hex) |
| TC-CX20X0-ADAPTER | Beckhoff | Adapter Interface (4 Input Banks, 4 Output Banks) The Beckhoff controller has 4 bytes of internal status for the Adapter. Config Inst 128 (0x80 Hex) Input Inst 129 (0x81 Hex) Output Inst 130 (0x82 Hex) |
| TC-CX20X0-INPUT | Beckhoff | One Input Card adds 16 bytes of Inputs (16 Input Banks, 0 Output Banks) |
| TC-CX20X0-OUTPUT | Beckhoff | One Output Card adds 16 bytes of Outputs (0 Input Banks, 16 Output Banks) |
| BK9105-ADAPTER | Beckhoff | Adapter Interface (4 Input Banks, 4 Output Banks) The Beckhoff controller has 4 bytes of internal status for the Adapter. Config Inst 100 (0x64 Hex) Input Inst 101 (0x65 Hex) Output Inst 102 (0x66 Hex) |
| BK9105-INPUT-1 | Beckhoff | Input (2 Input Banks, 0 Output Banks) |
| BK9105-OUTPUT-1 | Beckhoff | Output (0 Input Banks, 2 Output Banks) |
| KL1114 | Beckhoff | 4 Point DC Input (4 Input Bits, 0 Output Bits) |
| KL1418 | Beckhoff | 8 Point DC Input (8 Input Bits, 0 Output Bits) |
| KL1704 | Beckhoff | 4 Point DC Input (4 Input Bits, 0 Output Bits) |
| KL2602 | Beckhoff | 2 Point Relay Output (0 Input Bits, 2 Output Bits) |
| KL2622 | Beckhoff | 2 Point Relay Output (0 Input Bits, 2 Output Bits) |
| KL2114 | Beckhoff | 4 Point DC Output (0 Input Bits, 4 Output Bits) |
| KL2184 | Beckhoff | 4 Point DC Neg Output (0 Input Bits, 4 Output Bits) |
| KL2408 | Beckhoff | 8 Point DC Output (0 Input Bits, 8 Output Bits) |
| KL9194 | Beckhoff | Power (0 Input Bits, 0 Output Bits) |
| KL9190 | Beckhoff | Power (0 Input Bits, 0 Output Bits) |
| KL9010 | Beckhoff | Terminator (0 Input Bits, 0 Output Bits) |
| OPTO-G4EB2 | Opto22 | Pamux Adapter (4 Input Banks, 4 Output Banks) The Opto22 G4EB2 Brain Board must be configured with the Ethernet Configuration Tool provided by Opto22. The inputs and outputs must be mapped to Input Instance 100 and Output Instance 101. Config Inst 254 (0x64 Hex) Input Inst 100 (0x65 Hex) Output Inst 101 (0x66 Hex) |
| OPTO-IAC5Q | Opto22 | Quad (4 channel) AC Input G4EB2 defines input size |
| OPTO-IDC5Q | Opto22 | Quad (4 channel) DC Input G4EB2 defines input size |
| OPTO-OAC5Q | Opto22 | Quad (4 channel) AC Output G4EB2 defines output size |
| OPTO-ODC5Q | Opto22 | Quad (4 channel) DC Output G4EB2 defines output size |
| WAGO-750 | TGW Wave Sorter | Wave Sorter Interface (4 Input Banks, 4 Output Banks – Word Aligned) Config Inst 1 (0x01 Hex) Input Inst 110 (0x6E Hex) Output Inst 111 (0x6F Hex) |
| SEW-MOVIFIT-32 | SEW Movifit 32 Word | Vertiswitch Controller Interface (64 Input Banks, 64 Output Banks – Word Aligned) Config Inst 1 (0x01 Hex) Input Inst 170 (0xAA Hex) Output Inst 160 (0xA0 Hex) |
| NT100-RE-DP-1 | Hilscher DP to EIP Coupler 1 Word | Controller Interface (6 Input Banks, 2 Output Banks – Word Aligned) Config Inst 1 (0x01 Hex) Input Inst 100 (0x64 Hex) Output Inst 101 (0x65 Hex) |
| GENERIC-ADAPTER |  | Generic Adapter Interface. For example, a parameter entry can be used to define a Beckhoff controller. Example 1: Beckhoff (Assembly Class = 4, Config Instance = 100, Input Instance = 101, Output Instance = 102) with (4 + 2) = 6 bytes of input, and (4 + 2) = 6 bytes of output: CfgInpOutRdWr=”100,101,102,6,6” Example 2: Opto22 (Assembly Class = 4, Config Instance = 254, Input Instance = 100, Output Instance = 101) with 4 bytes of input, and 4 bytes of output: CfgInpOutRdWr=”254,100,101,4,4” |

### Suggested XML Configurations for Specific Devices

This section gives details on proper configuration of parameters for specific IO devices.  The IO configuration wizard will use the settings shown here automatically.  Once the tool has inserted them in the EIPModules table, they may be modified, if needed.

### 1734-AENT and 1738-AENTR/B

| Adapter 1734-AENT/1738-AENTR/B |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | “1734-AENT” / “1738-AENTR/B” |
| <Module><O2T> | rpi_us | “10000”. In microseconds. This should be set small enough to capture all of the encoder pulses, but large enough to avoid placing an excessive load on the adapter CPU. During testing, check the CPU load on the adapter using the adapter’s web interface. Minimum RPI is 2000 microseconds. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “10000”. In microseconds. This should be set small enough to capture all of the encoder pulses, but large enough to avoid placing an excessive load on the adapter CPU. During testing, check the CPU load on the adapter using the adapter’s web interface. Minimum RPI is 2000 microseconds. |
| <Module><T20> | cnxn_type | “MULTICAST” Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |
| <Module><Parameters> | T2OAlignment | "WORD" |
| <Module><Parameters> | O2TAlignment | "WORD" |

### PowerFlex525

| PowerFlex525 |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | "PowerFlex525" |
| <Module><O2T> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><T20> | cnxn_type | “MULTICAST”. Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |

### PowerFlex70

| PowerFlex70 |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | "PowerFlex70" |
| <Module><O2T> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><T20> | cnxn_type | “MULTICAST”. Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |

### PowerFlex700

| PowerFlex700 |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | "PowerFlex70" |
| <Module><O2T> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “5000” – Minimum RPI is 5000. |
| <Module><T20> | cnxn_type | “MULTICAST”. Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |

### 20-COMM-E Module for PowerFlex

The PowerFlex drives use a 20-COMM-E EtherNet/IP communication module as their interface.  The module uses assembly connections.

Input Banks

- Drive - 8 banks of inputs.

  - 1 word of control status (2 banks)

  - 1 word of speed feedback (2 banks)

  - 1 word (2 banks)

  - 1 word (2 banks)

Output Banks

- 4 banks of outputs.

  - 1 word of control (2 banks)

  - 1 word of speed output (2 banks)

### Yaskawa SI-EN3 Extended Speed Control

| YASKAWA SI-EN3 Extended Speed Control |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | "YASKAWA-SI-EN3-ESC" |
| <Module><O2T> | rpi_us | “6000” – RPI must be a multiple of 2000. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “6000” – RPI must be a multiple of 2000. |
| <Module><T20> | cnxn_type | “MULTICAST”. Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |
| <Module><Parameters> | T2OAlignment | "WORD" |
| <Module><Parameters> | O2TAlignment | "WORD" |

### Yaskawa SI-EN3 High-Speed Torque Control

Use the Extended Speed Control model if possible.  The High-Speed Torque Control model is not recommended unless torque control is needed.

| YASKAWA SI-EN3 High Speed Torque Control |  |  |
| --- | --- | --- |
| Element | Attribute | Value or Description |
| <Module> | connection | “HEADNODE” |
| <Module> | slot | “0” |
| <Module> | type | "YASKAWA-SI-EN3-HSTC" |
| <Module><O2T> | rpi_us | “6000” – RPI must be a multiple of 2000. |
| <Module><O2T> | cnxn_type | “POINT2POINT” |
| <Module><T20> | rpi_us | “6000” – RPI must be a multiple of 2000. |
| <Module><T20> | cnxn_type | “MULTICAST”. Only 20 “MULTICAST” connections are allowed, use “POINT2POINT” if more than 20 connections are needed. |
| <Module><Parameters> | T2OAlignment | "WORD" |
| <Module><Parameters> | O2TAlignment | "WORD" |

### SLC500 1747 with 1746 Modules

SLC 500 on EtherNet/IP uses adapter 1747 as the HEADNODE, and 1746 modules for the IO.  The 1746 modules must be configured as BRIDGED.

The 1747 adapter does not support a connection for itself.  The adapter read and write sizes are 0.  Each module has a direct connection, and the read and write size of the rack is determined by adding up the read and write sizes needed for each module in the rack.

Each module has a module key.  Modules must be included in the configuration or the adapter will not allow the rack to go online.  The module key values can be found in the EDS files.  The configuration wizard will insert the proper module key value.

Input Banks

- Adapter – 0 banks of inputs

- 8- and 16-bit modules – 2 banks of inputs for each input card.

- 32-bit modules – 4 banks of inputs for each input card.

Output Banks

- Adapter – 0 banks of outputs

- 8- and 16-bit modules – 2 banks of outputs for each output card.

  - Set the outputs with 0 = OFF or 1 = ON

- 32-bit modules – 4 banks of outputs for each output card.

Example:  1747-AENT, 1746-IA16, 1746-OA16, 1746-OA8 (4 cards)

- Banks used

  - 1747-AENT – Adapter

    - 0 banks of inputs

    - 0 banks of outputs

  - 1746-IA16 – 16-point input module

    - 2 banks of inputs

    - 0 banks of outputs

  - 1746-OA16 – 16-point output module

    - 0 banks of inputs

    - 2 banks of outputs

  - 1746-OA8 – 8-point output module

    - 0 banks of inputs

    - 2 banks of outputs

  - Total

    - 2 banks inputs

    - 4 banks outputs

### Control Logix 1756

The Control Logix 1756 adapter has a fixed read and write size that is dependent on the rack size.  The adapter has an extension in the part name that defines the rack size.  Note that when determining bank positions, the adapter uses the first slot of the rack.  The first IO card uses the second slot position.  The 1756 modules must be configured as BRIDGED.

Input Banks (For Input AND Output Cards)

- Rack status – 4 banks – These banks contain the card status (1 indicates the card slot is empty, 0 indicates the slot has a card!).  Note:  Bit 0 for the adapter is always 1.

- Adapter – Occupies one slot (the first slot) of the rack.  The inputs are allocated for the slot but are not used.

- Modules – 8 banks of inputs are allocated for each card.

  - 4 banks of IO status (0 – OFF indicates the IO is active)

  - 4 banks of IO inputs (0 = OFF, 1 = ON).  For an input card, these are the inputs.  For an output card, these indicate if the corresponding output is OFF or ON – similar in concept to a Motor_AUX input.

Output Banks (For Input AND Output Cards)

- Rack output block – 4 banks of outputs

- Adapter – Occupies one slot (the first slot) in a rack.  The outputs are allocated for the slot but are not used.

- Modules – 4 banks of outputs for each card

  - For output cards, set to 0 for OFF, or 1 for ON

  - Input cards do not use the 4 banks of outputs, but they are allocated for each card.

Example: 1756-ENBT-7  (7-slot rack)

- Input banks

  - 4 banks of status for the rack

  - (8 banks * 7 slots) = 56 banks of inputs

  - Total = 60 banks of inputs

- Output banks

  - 4 banks of outputs for the rack output block

  - (4 banks * 7 slots) = 28 banks of outputs

  - Total = 32 banks of outputs

### Flex 1794

The Flex 1794 adapter read and write sizes are calculated based on the number of modules included in the rack.  The rack includes the adapter, and up to 8 modules.  The adapter does not consume a “slot” in the rack.  Each module consists of a base and a “card”.  The card and the base have a physical key that must match.  The module also has a logical key value, found in the EDS file, that defines its capabilities.  The IO configuration wizard will insert the proper key value into the XML file for each module type.

Each module must be configured as BRIDGED, and requires two input and two output banks, regardless of type.  Some modules require additional space and a direct connection.  They are allocated space at the end of the rack.  Their “regular” space is still allocated but is not used.

Input Banks

- Adapter – 4 banks.  The adapter uses 4 banks of input (the status does not appear to be updated).

- Modules – 2 banks.  0 = OFF, 1 = ON.  8-bit modules use only the first bank, but the second bank is allocated.

Output Banks

- Adapter – 0 banks

- Modules – 2 banks.  Set 0 for OFF, 1 for ON.  8-bit modules use only the first bank, but the second bank is allocated.

Buffer Space

The RTA driver allocates two extra banks of IO space to match the logic used by the PLC Controller.  These two extra banks are required for each rack.  Be sure to allow for them when allocating space in both the input and output mapping.  The two extra banks will fall after the “normal” space (2 * number of modules), and before the space for the direct connect modules (if any).  See the example at the end of this section.

Direct Connect Modules

Modules that require a direct connection are allocated space at the end of the rack IO space.

- 1794-IB32	32-bit input module

  - Input banks

    - 4 banks of IB32 status

    - 4 banks of IB32 input data

    - Total 8 banks (4 words)

  - Output banks

    - 0 banks of outputs

- 1794-OB32	32-bit output module

  - Input banks

    - 4 banks of IB32 status

    - Total 4 banks (2 words)

  - Output banks

    - 4 banks of outputs

    - Total 4 banks (2 words)

- 1794-IE8	8-channel analog input module

  - Input banks

    - 4 banks (32 bits) of fault status

    - 16 banks (8 words) of analog values

    - 2 banks (16 bits) of status

    - Total 22 (11 words)

  - Output banks

    - 0 banks of outputs

  - Configuration

    - One word of configuration is sent with the forward open command.  Select the module parameter in the RTA XML configuration file.

      - Volts_0_to_10=”Enabled” – sets all 8 channels to 0 to 10 Volts and 0 to 20 mA.

      - MA_4_to_20=”Enabled” – sets all 8 channels to 4 to 20 mA.

      - Volts_10_to_10=”Enabled” – sets all 8 channels to -10 to +10 Volts.

- 1794-ID2	2-channel encoder input module

  - Input banks

    - 4 banks (32 bits) of fault status

    - 16 banks (8 words) of IO state and counter values

    - Total 20 banks (10 words)

  - Output banks

    - 10 banks of outputs (5 words) of encoder module modes and counter control

    - Total 10 banks (5 words)

  - Configuration

    - Zero words of configuration are sent with the Forward Open.

Example: (4 Cards:  1794-IA16, 1794-OA8, 1794-OW8, 1794-IB32)

- Input banks

  - Adapter – 4 banks of adapter status

  - (2 banks * 4 cards) = 8 banks of inputs

  - 2 banks of rack buffer space

  - 8 banks of Inputs for the IB32 card

  - Total 22 banks of Inputs

- Output banks

  - (2 banks * 4 cards) = 8 banks of outputs

  - 2 banks of rack buffer space

  - 0 banks of outputs for the IB32 card

  - Total 10 banks of outputs

4 Input banks

1794-AENT Adapter

0 Output banks

2 Input Banks

All 1794 modules are allocated the same space

2 Output banks

2 Input banks

Buffer space is allocated at the end of the rack

2 Output Banks

N Input banks

If 32-bit cards are included, banks are allocated at the end

N Output banks

Number of banks depends on the qantity of cards, including 32-bit cards

Number of banks depends on the number of 32-bit cards

For Flex 1794, all modules consume outputs.  The start of the outputs needs to be defined for the rack.  In the CSV file example, below, the 1794-IA16 input module shows the rack outputs start at word 0.

### Beckhoff EtherNet/IP Slave (TC-CX20X0 Processor)

The Beckhoff TC-CX20X0 EtherNet/IP interface device supports three module types.

- TC-CX20X0-ADAPTER – connects to the Beckhoff controller with the following parameters.

  - Config instance – 	128 (0x80 Hex)

  - Input instance – 129 (0x81 Hex)

  - Output instance – 130 (0x82 Hex)

  - The adapter interface uses 4 bytes of inputs and 4 bytes of outputs for interface status.

- TC-CX20X0-INPUT – Input card – adds 16 bytes (banks) or 8 words of inputs.

- TC-CX20X0-OUTPUT – Output Card – adds 16 bytes (banks) or 8 words of outputs.

Example:  Beckhoff interface with two Cards, TC-CX20X0-ADAPTER, TC-CX20X0-INPUT, TC—CX20X0-OUTPUT.

- Input Banks

  - Adapter – 4 banks of adapter status

  - TC-CX20X0-INPUT – (16 banks * 1 card) = 16 banks of inputs

  - Total:  20 banks of inputs

- Output Banks

  - Adapter – 4 banks of adapter status

  - TC-CX20X0-OUTPUT – (16 banks * 1 card) = 16 banks of outputs

  - Total:  20 banks of outputs

### Beckhoff EtherNet/IP Slave (BK9105-1000)

The Beckhoff BK9105 EtherNet/IP interface device supports the following module types.  We require the BK9105-1000 variant.  The only difference is the default network is 192.168.1.xxx so the devices will be on the subnet we like to use by default.  The BK9105 is on 172.16.17.xxx by default.  It is possible to change the network on a BK9105 – see below.

BK-9105-ADAPTER – connects to the Beckhoff EtherNet/IP adapter with the following parameters.

- Config instance – 100 (0x64 Hex)

- Input instance – 101 (0x65 Hex)

- Output instance – 102 (0x66 Hex)

- The adapter interface uses 4 bytes of inputs and 4 bytes of outputs for interface status.

- By default, the BK9105-1000 will be set to 100 Mbps, Auto-detect, and Full duplex.  This is what we want, but these can be changed.  See the device manual for details.

- The BK9105-1000 has DIP switch settings that are used to set the IP address of the module.  The subnet will be 192.168.1.xxx by default.

    - Example:  192.168.1.51  (51 decimal = 00110011 binary)

      - Switch 8-off, 7-off,6-on,5-on,4-off,3-off,2-on,1-on

      - Switch 9-off, 10-off (manual addressing)

- Each Beckhoff input and output module uses a portion of a bank depending on the number of bits defined by the module type.  The Beckhoff system always uses full words.  If a portion of a bank is used it will round up to a full word.

Note:  A Beckhoff rack must always have a KL9010 terminator card present.  The logic calculates the required IO when it processes the terminator card.

Example:  Beckhoff Interface with 4 cards:  BK-9105-ADAPTER, KL1114 input card, KL1418 input card, KL2602 output card, KL9010 terminator card.

- Input banks

  - Adapter – 4 banks of adapter status

  - KL1114 – 4 bits of input

  - KL1418 – 8 bits of input

  - KL2602 – 0 bits of input

  - KL9010 – 0 bits of input

  - Total bits – 12 bits/8 = 1.5 banks of input (round up to 2 banks or 1 word).

  - Total: 6 banks of input

- Output banks

  - Adapter – 4 banks of adapter status

  - KL1114 – 0 bits of output

  - KL1418 – 0 bits of output

  - KL2602 – 2 bits of output

  - KL9010 – 0 bits of output

  - Total Bits = 2/8 = 0.25 banks of outputs (round up to 2 banks or 1 word)

  - Total 6 banks Outputs

NOTE:  To change the default network on BK9105 devices from 172.16.17.x to 192.168.1.xxx (to mimic the BK9105-1000 variant), you must use the Beckhoff TCBootP software tool.  See the bk9055_bk9105en.pdf document available from Beckhoff.  The configuration tool installer is found in Beckhoff-bootp-setup.exe.  Efforts to use the QNX bootpd server to do this, in a way similar to that used for Optomux Ethernet, were not successful.

NOTE:  To restore any BK9105 to the factory defaults, connect just a KL9010 end terminal directly to the BK9105.  Set all the DIP switches to the ON position, then apply power.  When complete, the Error LED will light, and the I/O Run and I/O Error LEDs will flash alternately.  Remove power and configure as desired.

### Opto22 G4EB2 Pamux

The Opto22 G4EB2 brain board can be used to convert an Opto22 Pamux IO rack to an EtherNet/IP rack.  The G4EB2 brain board must be configured with the Opto22 EtherNet/IP configuration tool when used in this way.  The tool maps each input or output to a specific bit of the EtherNet/IP input or output banks.  Each G4EB2 requires four banks of input data and four banks of output data, regardless of how many quad modules are installed.

NOTE:  A straight EtherNet option is also available, and is the preferred option.  See section 2.2 for details.

Example: (Opto22 Pamux card with 2 quad input modules and 2 quad output modules.

- Input banks

  - Adapter – 4 banks of adapter inputs

  - IAC5Q – 4 bits of inputs

  - IAC5Q – 4 bits of inputs

  - OAC5Q – 0 bits of inputs

  - OAC5Q – 0 bits of inputs

  - Pad – 24 bits (Pad entry is only used with the Opto22 software)

  - Total bits – 32 bits / 8 = 4 banks of inputs

  - Total:  4 banks of inputs

- Output banks

  - Adapter – 4 banks of adapter outputs

  - IAC5Q – 0 bits of outputs

  - IAC5Q – 0 bits of outputs

  - OAC5Q – 4 bits of outputs

  - OAC5Q – 4 bits of outputs

  - Pad – 24 bits (Pad entry is only used with the Opto22 software)

  - Total bits – 32 bits / 8 = 4 banks of outputs

  - Total:  4 banks of outputs

G4EB2 – Ethernet IP and Watchdog

Input and output quad packs

Input mapping to instance 100

Output mapping to instance 101

Verify results

Opto22 brain board LED indicators

Network Status

Module Status

Brain Status

Activity – Connection 1

Link – Connection 1

Activity – Connection 2

Link – Connection 2

G4EB2 - Brain Board LEDs

### Interroll Accumulation Controller

The Interroll EtherNet/IP accumulation controller is configured using a web browser to enable EtherNet/IP mode.  The web page requires authentication:
    User:  Interroll, Password:  Interroll

The controller has the following IO assigned:

Input banks:  36

Output banks:  10

The screen shot below is from the controller’s web page:

#### Interroll Input Table (36 Banks)

The following table maps the Interroll input IO.

| Interroll Input List (RTA EtherNet/IP) |  |  |  |
| --- | --- | --- | --- |
| Bank | Bit | Value or Description |
| 0 | 0 | Sensor 1 – Status at input high = sensor blocked, input low = sensor not blocked. Logical status and voltage level depend on configuration (NPN/PNP; polarity) |
|  | 1 | Sensor 2 – Status at input "Sensor 2" |
|  | 2 | Sensor 3 – Status at input "Sensor 3" |
|  | 3 | Sensor 4 – Status at input "Sensor 4" |
|  | 4 | Sensor 5 – Aux 1 input. Dependent on configuration. |
|  | 5 | Sensor 6 |
|  | 6 | Sensor 7 |
|  | 7 | Sensor 8 |
| 1 | 0 | Digital IO 1 – Logical status and voltage level depend on configuration (NPN/PNP; polarity) |
|  | 1 | Digital IO IO 2 |
|  | 2 | Digital IO IO 3 |
|  | 3 | Digital IO IO 4 |
|  | 4 | Reserved |
|  | 5 | Reserved |
|  | 6 | Reserved |
|  | 7 | Reserved |
| 2 | 0 | Error1 High = motor error, Low = motor OK |
|  | 1 | Error2 High = motor error, Low = motor OK |
|  | 2 | Error3 High = motor error, Low = motor OK |
|  | 3 | Error4 High = motor error, Low = motor OK |
|  | 4 | Reserved |
|  | 5 | Reserved |
|  | 6 | Reserved |
|  | 7 | Reserved |
| 3 |  | Speed 1 – 0...100 (0 = stop, 100 = max. speed), negative values indicate reversed direction of rotation |
| 4 |  | Speed 2 |
| 5 |  | Speed 3 |
| 6 |  | Speed 4 |
| 7 |  | Reserve byte so that data words are positioned correctly |
| 8,9 |  | Current 1 – Average motor current in mA |
| 10,11 |  | Current 2 |
| 12,13 |  | Current 3 |
| 14,15 |  | Current 4 |
| 16,17 |  | System State – Voltage Motor "Motor Power" voltage in mV |
| 18,19 |  | System State – Voltage Logic "Power Logic+Sensors" voltage in mV |
| 20,21 |  | System State – Temperature in 0.1 °C |
| 22,23,24,25 |  | System Uptime – Operating time in seconds since last start/restart |
| 26 | 0 | Control Input 1 |
|  | 1 | Control Input 2 |
|  | 2 | Control Input 3 |
|  | 3 | Control Input 4 |
|  | 4 | Control Input 5 |
|  | 5 | Control Input 6 |
|  | 6 | Control Input 7 |
|  | 7 | Control Input 8 |
| 27 |  | Decision Byte |
| 28 | 0 | Control Output 1 |
|  | 1 | Control Output 2 |
|  | 2 | Control Output 3 |
|  | 3 | Control Output 4 |
|  | 4 | Control Output 5 |
|  | 5 | Control Output 6 |
|  | 6 | Control Output 7 |
|  | 7 | Control Output 8 |
| 29 | 0 | Handshake Signals – Up In |
|  | 1 | Handshake Signals – Down In |
|  | 2 | Handshake Signals – Left In |
|  | 3 | Handshake Signals – Right In |
|  | 4 | Handshake Signals – Up Out |
|  | 5 | Handshake Signals – Down Out |
|  | 6 | Handshake Signals – Left Out |
|  | 7 | Handshake Signals – Right Out |
| 30 | 0 | Zone State – ZoneBusy1 |
|  | 1 | Zone State – ZoneBusy2 |
|  | 2 | Zone State – ZoneBusy3 |
|  | 3 | Zone State – ZoneBusy4 |
|  | 4 | Reserved |
|  | 5 | Reserved |
|  | 6 | Reserved |
|  | 7 | Reserved |
| 31 |  | Zone State – ZoneError1 |
| 32 |  | Zone State – ZoneError2 |
| 33 |  | Zone State – ZoneError3 |
| 34 |  | Zone State – ZoneError4 |
| 35 | 0 | Global Signals – Start |
|  | 1 | Global Signals – Start Inverse |
|  | 2 | Global Signals – Normal Stop |
|  | 3 | Global Signals – Emergency Stop |
|  | 4 | Global Signals – Alternate Speed |
|  | 5 | Global Signals – Inverse Direction |
|  | 6 | Global Signals – Global Error |
|  | 7 | Reserved |

#### Interroll Output Table (10 Banks)

The following table maps the Interroll output IO.

| Interroll Output List (RTA EtherNet/IP) |  |  |  |
| --- | --- | --- | --- |
| Bank | Bit | Value or Description |
| 0 | 0 | Digital Output 1 – Logical status and voltage level depends on IO configuration (NPN/PNP type; polarity) |
|  | 1 | Digital Output 2 |
|  | 2 | Digital Output 3 |
|  | 3 | Digital Output 4 |
|  | 4 | Reserved |
|  | 5 | Reserved |
|  | 6 | Reserved |
|  | 7 | Reserved |
| 1 |  | Motor Speed 1 – Speed 0...100 (0 = stop, 100 = max. speed), negative values indicate reversed direction of rotation |
| 2 |  | Motor Speed 2 |
| 3 |  | Motor Speed 3 |
| 4 |  | Motor Speed 4 |
| 5 | 0 | Control inputs overwrite – Input 1 |
|  | 1 | Control inputs overwrite – Input 2 |
|  | 2 | Control inputs overwrite – Input 3 |
|  | 3 | Control inputs overwrite – Input 4 |
|  | 4 | Control inputs overwrite – Input 5 |
|  | 5 | Control inputs overwrite – Input 6 |
|  | 6 | Control inputs overwrite – Input 7 |
|  | 7 | Control inputs overwrite – Input 8 |
| 6 |  | Control inputs overwrite – DecisionInput |
| 7 | 0 | Control outputs overwrite – ControlOutput1 |
|  | 1 | Control outputs overwrite – ControlOutput2 |
|  | 2 | Control outputs overwrite – ControlOutput3 |
|  | 3 | Control outputs overwrite – ControlOutput4 |
|  | 4 | Control outputs overwrite – ControlOutput5 |
|  | 5 | Control outputs overwrite – ControlOutput6 |
|  | 6 | Control outputs overwrite – ControlOutput7 |
|  | 7 | Control outputs overwrite – ControlOutput8 |
| 8 | 0 | Handshake Signals – Overwrite UpIn |
|  | 1 | Handshake Signals – Overwrite Down In |
|  | 2 | Handshake Signals – Overwrite Left In |
|  | 3 | Handshake Signals – Overwrite Right In |
|  | 4 | Handshake Signals – Overwrite Up Out |
|  | 5 | Handshake Signals – Overwrite Down Out |
|  | 6 | Handshake Signals – Overwrite Left Out |
|  | 7 | Handshake Signals – Overwrite Right Out |
| 9 | 0 | Global Signals – Start |
|  | 1 | Global Signals – Start Inverse |
|  | 2 | Global Signals – Normal Stop |
|  | 3 | Global Signals – Emergency Stop |
|  | 4 | Global Signals – Alternate Speed |
|  | 5 | Global Signals – Inverse Direction |
|  | 6 | Global Signals – Global Error |
|  | 7 | Reserved |

#### Interroll Parameters

Interroll parameters are defined in the Interroll manual.  All parameters use Class 0x64, Instance 0x01, Attribute 0x4xxx.

For example, the standard accumulation mode is defined by the value 6:
  ClassInstAttrParam = “0x64,1,0x4901,6”

#### Interroll Zone Control – Y Cable and Physical IO

Each Interroll zone controller has four connections that can be configured as input or output.  The Interroll web interface provides a popup selection that can be used to define how the IO will be used.

In the example below, I/O 1 is configured as a start sensor for zone 1.  Sensor 5 is selected for I/O 1.  This defines I/O 1 as a start sensor at an entry zone where there is no MultiControl upstream, which tells Motor 1 to start to advance product to the first downstream sensor Sensor 1.

In the next example, a Y cable and a relay is used to release the last zone (zone 4) as long as Sensor 4 is blocked.  Set I/O 4 to “Handshake In Down” (PNP Positive) and connect the Y cable for I/O 4 with a dry contact between the brown (24v) and black (signal) wires.

#### Interroll Zone Control – EtherNet/IP Interface

Each Interroll multi-controller has ten banks of outputs, which can be used to control the accumulation conveyor.

For example, the “Handshake In Down” internal signal is used to enable the accumulation zone to discharge.  The “Handshake In Down” signal may come from a multi-controller downstream, where each box will stop at the last sensor automatically.  Alternatively, the input signal could come from a physical input as shown in the prior example, or from the EtherNet/IP interface.  To use an EtherNet/IP signal to release the last zone, toggle the handshake signal “Handshake Signals Overwrite DownIn”.  Go to the group of outputs for the Interrol card (192.168….).  Toggle the output (“Output.Data[8].1” : Bank 8 – Bit 1) “Handshake Signals Overwrite DownIn” to release the zone.

### Wago 750 TGW Wave Sorter Controller

The Wago 750 has the following IO assigned to the input and output banks.  The module is configured using a web browser to enable EtherNet/IP mode.

Input banks:  4 banks of input

Output banks:  4 banks of output

Activate EtherNet/IP

The default login information for the web interface:
    User:  admin, Password:  wago

| TGW Inputs (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Field | Size (bits) | Value or Description |
| Data[0].0 | 1 | Divert 1 Divert Trigger Clear PE (Local Input 1) |
| Data[0].1 | 1 | Divert 1 Divert Confirm PE (Local Input 2) |
| Data[0].2 | 1 | Divert 1 Local Arm Divert (Local Input 3) |
| Data[0].3 | 1 | Divert 1 Divert Lane Full PE (Local Input 4) |
| Data[0].4 | 1 | Divert 2 Divert Trigger Clear PE (Local Input 5) |
| Data[0].5 | 1 | Divert 2 Divert Confirm PE (Local Input 6) |
| Data[0].6 | 1 | Divert 2 Local Arm Divert (Local Input 7) |
| Data[0].7 | 1 | Divert 2 Divert Lane Full PE (Local Input 8) |
| Data[0].8 | 1 | Spare |
| Data[0].9 | 1 | Spare |
| Data[0].10 | 1 | Spare |
| Data[0].11 | 1 | Spare |
| Data[0].12 | 1 | Spare |
| Data[0].13 | 1 | Spare |
| Data[0].14 | 1 | Spare |
| Data[0].15 | 1 | Communication Heartbeat |
| Data[1].0 | 1 | Divert 1 Raise Row 1 SOL (Local Output 1) |
| Data[1].1 | 1 | Divert 1 Raise Row 2 SOL (Local Output 2) |
| Data[1].2 | 1 | Divert 1 Raise Row 3 SOL (Local Output 3) |
| Data[1].3 | 1 | Divert 1 Raise Row 4 SOL (Local Output 4) |
| Data[1].4 | 1 | Divert 1 Raise Row 5 SOL (Local Output 5) |
| Data[1].5 | 1 | Divert 1 Raise Row 6 SOL (Local Output 6) |
| Data[1].6 | 1 | Spare Output (Local Output 7) |
| Data[1].7 | 1 | Spare Output (Local Output 8) |
| Data[1].8 | 1 | Divert 2 Raise Row 1 SOL (Local Output 9) |
| Data[1].9 | 1 | Divert 2 Raise Row 2 SOL (Local Output 10) |
| Data[1].10 | 1 | Divert 2 Raise Row 3 SOL (Local Output 11) |
| Data[1].11 | 1 | Divert 2 Raise Row 4 SOL (Local Output 12) |
| Data[1].12 | 1 | Divert 2 Raise Row 5 SOL (Local Output 13) |
| Data[1].13 | 1 | Divert 2 Raise Row 6 SOL (Local Output 14) |
| Data[1].14 | 1 | Spare Output (Local Output 15) |
| Data[1].15 | 1 | Spare Output (Local Output 16) |

| TGW Outputs (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Field | Size (bits) | Value or Description |
| Data[0].0 | 1 | Divert 1 Network Divert Arming Signal |
| Data[0].1 | 1 | Divert 1 Row 1 Manual Raise |
| Data[0].2 | 1 | Divert 1 Row 2 Manual Raise |
| Data[0].3 | 1 | Divert 1 Row 3 Manual Raise |
| Data[0].4 | 1 | Divert 1 Row 4 Manual Raise |
| Data[0].5 | 1 | Divert 1 Row 5 Manual Raise |
| Data[0].6 | 1 | Divert 1 Row 6 Manual Raise |
| Data[0].7 | 1 | Spare |
| Data[0].8 | 1 | Divert 2 Network Divert Arming Signal |
| Data[0].9 | 1 | Divert 1 Row 1 Manual Raise |
| Data[0].10 | 1 | Divert 1 Row 2 Manual Raise |
| Data[0].11 | 1 | Divert 1 Row 3 Manual Raise |
| Data[0].12 | 1 | Divert 1 Row 4 Manual Raise |
| Data[0].13 | 1 | Divert 1 Row 5 Manual Raise |
| Data[0].14 | 1 | Divert 1 Row 6 Manual Raise |
| Data[0].15 | 1 | Communications Heartbeat |
| Data[1] | 16 | Sorter Speed Reference (FPM) |

#### TGW IO Requirements in FPC

Each divert requires one output to arm the diverter.  For example, if the first output word is configured as word 1202 – bit 0, choose that IO in the Outpoints table.  If there are two diverts, the second “arm” output is in bit 8.

Each controller requires an entry in the SpdControl table.  For example, if the second output word is set to word 1203, set IO_Addr_Out to 1203.  If you create bits for the speed in the Conveyor table, you will need to set the matching process.  If you do not create the speed bits, set the process in the SpdControl table to SORT.

If you set the MaxSpeed to “1111101000” (1000 decimal) and set the “OffPercent” to 25.0, the result will be a value sent to the controller of “0011111010” (250 decimal).  The “OnPercent” value can be set to the same value as “OffPercent”.  It is not used unless the selected signal is “ON”

### SEW Movifit 32 Word Vertiswitch Controller

The SEW Movifit 32 has the following IO assigned to the input and output Banks:

Movifit

- Config instance 1 (0x01 hex)

- Input instance 170 (0xAA hex)

- Output instance 160 (0xA0 hex)

IP Address Setting – The IP address is set using “MOVITOOLS-Motion Studio” or the “SEW-EURODRIVE Address Editor”.

Move Command – Turn on bit 0 (No EStop) and one of the position bits.  When the Done bit comes ON the move is complete.

Input Banks:  64 banks (32 Words) of inputs – shown below

Output Banks:  64 banks (32 Words) of outputs – shown below

| Vertiswitch Inputs (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Field | Size (bits) | Value or Description |
| Data[0].0 | 1 | Connected |
| Data[0].1 | 1 | Powered |
| Data[0].2 | 1 | Done – Indicates when a position move is done |
| Data[0].3 | 1 | Active |
| Data[0].4 | 1 | Stopped |
| Data[0].5 | 1 | Estop Activated – High indicates an Estop condition |
| Data[0].6 | 1 | Error – No encoder signal |
| Data[0].7 | 1 | Error – wrong encoder direction |
| Data[0].8 (10 Octal) | 1 | FB Error |
| Data[0].9 (11 Octal) | 1 | Manual Mode override |
| Data[0].10 (12 Octal) | 1 | Heartbeat return |
| Data[0].11 (13 Octal) | 1 | Axis Referenced Signal |
| Data[0].12 (14 Octal) | 1 | Upper Limit Reached |
| Data[0].13 (15 Octal) | 1 | Lower Limit Reached |
| Data[1].0 – 15 | 1 | Requested Axis Mode (only used for troubleshooting) 0 – No Mode 1 – Velocity 2 – Positioning 7 – Homing |
| Data[2].0 – 15 | 1 | Actual Axis Mode (only used for troubleshooting) 0 – No Mode 1 – Velocity 2 – Positioning 7 – Homing |
| Data[3].0 – 15 | 1 | Inverted Fault Status (only used for troubleshooting) |
| Data[4].0 – 15 | 1 | FB Error ID (only used for troubleshooting). Decimal value |
| Data[5].0 – 15 | 1 | Target Incs (only used for troubleshooting). Decimal value between 50 and 600 |
| Data[6].0 – 15 | 1 | Position Incs (only used for troubleshooting). Decimal Value between 50 and 600 |

| Vertiswitch Outputs (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Field | Size (bits) | Value or Description |
| Data[0].0 | 1 | Estop not activated. Must be high for unit to run. |
| Data[0].1 | 1 | Position 1 Requested. Must be high to move to Position 1 |
| Data[0].2 | 1 | Position 2 Requested. Must be high to move to Position 2 |
| Data[0].3 | 1 | Position 3 Requested. Must be high to move to Position 3 |
| Data[0].4 | 1 | Error Reset Requested. Must be high to clear faults. |
| Data[0].5 | 1 | Reference Requested. Must be high to do initial homing. |
| Data[0].6 | 1 | Not used |
| Data[0].7 | 1 | Heartbeat. State is returned as an input for testing. |

### The IO Adapter Status Table [KTX_RIO_ADAPTERS]

The IO Adapter Status table, a.k.a., KTX_RIO_ADAPTERS, is automatically populated with one entry per EtherNet/IP adapter.  The name of the entry is taken from the <Adapter name=”name”> parameter.

| IO Adapter Status Table (RTA EtherNet/IP) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | A unique name for the adapter. Automatically filled in from the adapter name in the eipcfg.xml file. |
| Adapter | Int | Adapter index. Automatically filled in. |
| Active | Y/N | Indicates if the adapter is being monitored. |
| pStatus | String | Text description of the status for each adapter. Updated by the software. - “No Error” –Valid XML file is loaded, adapter is communicating. - “Unable to Connect” – Indicates an issue with connectivity to the rack, such as a disconnected wire, switch, or adapter. - “FO Configuration Error” – A module was missing at startup. These errors can be seen on the adapter itself: - Adapter “Point Bus Status Error” Blinking red – If the light is blinking red, a module is loose or has failed. - Adapter Errors from Browser – The adapter’s web page will show errors indicating a missing or failed module. |
| Config Byte | Hex | Used internally to determine the state of the adapter. 1 – Init, 2 – Good, 3 – Connect Error, 4 – Config Error, 5 – Run Error, 6 – Shutdown, 7 – EM Config Error |
| Fault Byte | Hex | Shows the error value returned by the RTA EtherNet/IP driver. Value depends on Config Byte. |
| IOCard | PopUp | The RTA EtherNet/IP card connected to this device. Select a card from the IOCard list. |
| Rack | Int | Not used for RTA EtherNet/IP. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |

#### EtherNet/IP Adapter Status Bits – Point I/O

Slot Status Bits:  Each Point I/O adapter has a set of input banks that indicate the status of each module in the IO rack.  The Slot Status bits display the connection status for each of the Point I/O modules that use an assembly connection.  Bit 0 is reserved for the adapter.  Each of the other bits (1-63) correspond to a Point I/O module that may be installed in the backplane.

The status bits are defined as:

0 = Module participating with no errors

1 = Module not participating or connection error
(typically, module removed/missing).

#### RTA EtherNet/IP Log Files

The RTA driver creates log files in the /SortPlus/LOGS directory named rtaeip_log.<day>.  The following log segment shows an EtherNet/IP rack that has disconnected and re-connected, indicating a problem with the configuration.  It could be communicating in half-duplex instead of full-duplex, or there may be an issue with the network switch or the adapter.

2015/07/31 00:02:47.801 | 192.168.1.58   1NODE58=> EIPC (101) Client  I/O State "NON-EXIST"

2015/07/31 00:02:47.801 | 192.168.1.58:1 1NODE58-0=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:2 1NODE58-1=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:3 1NODE58-2=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:4 1NODE58-3=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:5 1NODE58-4=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:6 1NODE58-5=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:7 1NODE58-6=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:8 1NODE58-7=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:9 1NODE58-8=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:10 1NODE58-9=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58   1NODE58=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | EXAMPLE_POINTIOSTATE_IO_EXCHANGE: IO_State == 255 disconnect.

2015/07/31 00:02:47.801 | 192.168.1.58   1NODE58=> EIPC (101) Client  I/O State "NON-EXIST"

2015/07/31 00:02:47.801 | 192.168.1.58:1 1NODE58-0=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:2 1NODE58-1=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:3 1NODE58-2=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:4 1NODE58-3=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:5 1NODE58-4=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:6 1NODE58-5=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:7 1NODE58-6=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:8 1NODE58-7=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:9 1NODE58-8=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58:10 1NODE58-9=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:47.801 | 192.168.1.58   1NODE58=> EIPC (101) Client  I/O State "FREE"

2015/07/31 00:02:52.803 | eipc_client_connectToServer [IP c0a8013a]

2015/07/31 00:02:52.803 | eips_usersock_connect 18.

2015/07/31 00:02:52.804 | eipc_client.c 282: UCMM STATE CHANGE

2015/07/31 00:02:52.804 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "REGISTER SESSION"

2015/07/31 00:02:52.804 | eips_usersock_sendTCPData: enter

2015/07/31 00:02:52.804 | eips_usersock_sendTCPData: exit

2015/07/31 00:02:52.806 | eipc_client.c 1043: UCMM STATE CHANGE

2015/07/31 00:02:52.806 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "READY"

2015/07/31 00:02:52.806 | Check 1NODE58 EM for 11

2015/07/31 00:02:52.807 | example_point_fortna: local_server_ioalloc: enter

2015/07/31 00:02:52.807 | eipc_io_openIOConnection: enter

2015/07/31 00:02:52.807 | eips_usersock_sendTCPData: enter

2015/07/31 00:02:52.807 | eips_usersock_sendTCPData: exit

2015/07/31 00:02:52.807 | eipc_io.c 788: UCMM STATE CHANGE

2015/07/31 00:02:52.807 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "BUSY"

2015/07/31 00:02:52.810 | local_procrsp_ucmm: enter

2015/07/31 00:02:52.810 | 192.168.1.58   1NODE58=> EIPC (1) GenStat 0x10ExtStat 0x00ff

2015/07/31 00:02:52.810 | eipc_client.c 415: UCMM STATE CHANGE

2015/07/31 00:02:52.810 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "READY"

2015/07/31 00:02:57.812 | Check 1NODE58 EM for 11

2015/07/31 00:02:57.813 | example_point_fortna: local_server_ioalloc: enter

2015/07/31 00:02:57.813 | eipc_io_openIOConnection: enter

2015/07/31 00:02:57.813 | eips_usersock_sendTCPData: enter

2015/07/31 00:02:57.813 | eips_usersock_sendTCPData: exit

2015/07/31 00:02:57.813 | eipc_io.c 788: UCMM STATE CHANGE

2015/07/31 00:02:57.813 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "BUSY"

2015/07/31 00:02:57.815 | local_procrsp_ucmm: enter

2015/07/31 00:02:57.815 | 192.168.1.58   1NODE58=> EIPC (1) GenStat 0x10ExtStat 0x00ff

2015/07/31 00:02:57.815 | eipc_client.c 415: UCMM STATE CHANGE

2015/07/31 00:02:57.815 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "READY"

2015/07/31 00:03:02.815 | Check 1NODE58 EM for 11

2015/07/31 00:03:02.816 | example_point_fortna: local_server_ioalloc: enter

2015/07/31 00:03:02.816 | eipc_io_openIOConnection: enter

2015/07/31 00:03:02.816 | eips_usersock_sendTCPData: enter

2015/07/31 00:03:02.816 | eips_usersock_sendTCPData: exit

2015/07/31 00:03:02.816 | eipc_io.c 788: UCMM STATE CHANGE

2015/07/31 00:03:02.816 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "BUSY"

2015/07/31 00:03:03.051 | local_procrsp_ucmm: enter

2015/07/31 00:03:03.051 | eips_usersock_joinMulitcast: joined 0xefc0082f

2015/07/31 00:03:03.051 | 192.168.1.58   1NODE58=> EIPC (101) Client  I/O State "CONNECTED"

2015/07/31 00:03:03.051 | eipc_client.c 415: UCMM STATE CHANGE

2015/07/31 00:03:03.051 | 192.168.1.58   1NODE58=> EIPC (1) Client UCMM State "READY"

The next log segment shows an EtherNet/IP rack operating correctly.  If the “H:” (high) values are larger than 2 milliseconds there is a problem communicating with the rack.

2015/07/31 00:03:16.640 | Profile(ms) PointIO Run N:300000 L:0.003587 A:0.004153 H:0.151175 CPS:1498740000

2015/07/31 00:03:16.641 | Profile(ms) PointIO Timer N:60000 L:0.653424 A:0.999362 H:1.931127 CPS:1498740000

2015/07/31 00:03:16.641 | Profile(ms) PointIO Sleep N:60000 L:0.502270 A:0.995236 H:1.927089 CPS:1498740000

#### RTA EtherNet/IP Example XML File

Here is a full example of an RTA driver XML configuration file.  It contains a 1783-EMS (Allen-Bradley Stratix EtherNet Managed Switch) at 192.168.1.1, a 1734-AENT Point IO adapter at 192.168.1.51 with a 1734-IB8 8-point 24V input and a 1734-OB8 8-point 24V output module, and a 1734-AENT Point IO adapter at 192.168.1.52 with another 1734-IB8 and 1734-OB8 and a 1734-IA4 4-point 120V input module and a 1734-OA4 4-point 120V output module.

<?xml version="1.0" encoding="UTF-8"?>

<EthernetIP name="Test Rack" version = "1" interfaceip="192.168.1.9">

<!--The name for the entire configuration.  Suggested names include the name of the machine or panel.-->

<!--Adapter - Contains the configuration for each head node and the device in each slot.-->

<Adapter  targetip="192.168.1.1"

name="1783-EMS"

InputAddress="0"

OutputAddress="0">

<!--Module contains the configuration for each slot in the Adapter.-->

<Module name="1783-EMS_0"

type="1783-EMS"

slot="0"

connection="HEADNODE">

<O2T rpi_us="50000" cnxn_type="POINT2POINT"/>

<T2O rpi_us="50000" cnxn_type="POINT2POINT"/>

<Key Compatibility="false"

VendorID="0"

DeviceType="0"

ProductCode="0"

MajorRevision="0"

MinorRevision="0" />

<!-- T2OAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- O2TAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- BYTE = Data is aligned to 1 byte -->

<!-- WORD = Data is aligned to 2 bytes -->

<!-- DOUBLE = Data is aligned to 4 bytes -->

<!-- FIXED = Each slot has a fixed size. -->

<!-- T2OSlotSize and O2TSlotSize is used if Alignment is FIXED. -->

<!-- T2OSlotSize and O2TSlotSize can range from 1 to 24 bytes. -->

<Parameters T2OAlignment="WORD"

T2OSlotSize="1"

O2TAlignment="WORD"

O2TSlotSize="1" />

</Module>

</Adapter>

<Adapter targetip="192.168.1.51"

name="1734_AENT_51"

InputAddress="48"

OutputAddress="0">

<!-- Module contains the configuration for each slot in the Adapter.-->

<!-- 1734-AENT Input Size = 8, Output Size = 0 -->

<Module name="1734_AENT_0"

type="1734-AENT"

slot="0"

connection="HEADNODE">

<O2T rpi_us="4000" cnxn_type="POINT2POINT"/>

<T2O rpi_us="4000" cnxn_type="MULTICAST"/>

<Key Compatibility="true"

VendorID="1"

DeviceType="12"

ProductCode="196"

MajorRevision="3"

MinorRevision="12" />

<!-- T2OAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- O2TAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- BYTE = Data is aligned to 1 byte -->

<!-- WORD = Data is aligned to 2 bytes -->

<!-- DOUBLE = Data is aligned to 4 bytes -->

<!-- FIXED = Each slot has a fixed size. -->

<!-- T2OSlotSize and O2TSlotSize is used if Alignment is FIXED. -->

<!-- T2OSlotSize and O2TSlotSize can range from 1 to 24 bytes. -->

<Parameters T2OAlignment="WORD"

T2OSlotSize="1"

O2TAlignment="WORD"

O2TSlotSize="1" />

</Module>

<!-- 1734-IB8 Input Size = 1, Output Size = 0 -->

<Module name="1734_IB8_1"

type="1734-IB8"

slot="1"

connection="RACKOPTIMIZED">

</Module>

<!-- 1734-OB8 Input Size = 1, Output Size = 1 -->

<Module name="1734_OB8_2"

type="1734-OB8"

slot="2"

connection="RACKOPTIMIZED">

</Module>

</Adapter>

<Adapter targetip="192.168.1.52"

name="1734_AENT_52"

InputAddress="64"

OutputAddress="8">

<!--Module contains the configuration for each slot in the Adapter-->

<!-- 1734-AENT Input Size = 8, Output Size = 0 -->

<Module name="1734_AENT_0"

type="1734-AENT"

slot="0"

connection="HEADNODE">

<O2T rpi_us="4000" cnxn_type="POINT2POINT"/>

<T2O rpi_us="4000" cnxn_type="MULTICAST"/>

<Key Compatibility="true"

VendorID="1"

DeviceType="12"

ProductCode="196"

MajorRevision="3"

MinorRevision="12" />

<!-- T2OAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- O2TAlignment = BYTE, WORD, DOUBLE, or FIXED -->

<!-- BYTE = Data is aligned to 1 byte -->

<!-- WORD = Data is aligned to 2 bytes -->

<!-- DOUBLE = Data is aligned to 4 bytes -->

<!-- FIXED = Each slot has a fixed size. -->

<!-- T2OSlotSize and O2TSlotSize is used if Alignment is FIXED. -->

<!-- T2OSlotSize and O2TSlotSize can range from 1 to 24 bytes. -->

<Parameters T2OAlignment="WORD"

T2OSlotSize="1"

O2TAlignment="WORD"

O2TSlotSize="1" />

</Module>

<!-- 1734-IB8 Input Size = 1, Output Size = 0 -->

<Module name="1734_IB8_1"

type="1734-IB8"

slot="1"

connection="RACKOPTIMIZED">

</Module>

<!-- 1734-OB8 Input Size = 1, Output Size = 1 -->

<Module name="1734_OB8_2"

type="1734-OB8"

slot="2"

connection="RACKOPTIMIZED">

</Module>

<!-- 1734-IA4 Input Size = 1, Output Size = 0 -->

<Module name="1734_IA4_3"

type="1734-IA4"

slot="3"

connection="RACKOPTIMIZED">

</Module>

<!-- 1734-OA4 Input Size = 0, Output Size = 1 -->

<Module name="1734_OA4_4"

type="1734-OA4"

slot="4"

connection="RACKOPTIMIZED">

</Module>

</Adapter>

</EthernetIP>

### The EtherNet/IP Configuration Tool [EIPStatusScrn]

The EtherNet/IP Configuration Tool in FPC can be used to create the RTA XML configuration file, and to aid in configuring the FPC IO definitions.  It is a “wizard” tool embedded in FPC, but only used during configuration.  Once configuration is complete, the tool is not involved in regular operation.  It has the following features:

      - Build the list of adapters and modules to define the IO rack, or…

      - Import the CSV files generated by the Electrical department.

      - Read an existing XML configuration file then modify the configuration.

      - Verify the integrity of the configuration with the “Check Banks” feature.

      - Update the FPC Configio table based on the rack configuration.

These features are described in detail in the following sections.

The EtherNet/IP Configuration Tool, or EIPStatusScrn, can be found at “Configuration Menu” | “IO Utilities” | “Ethernet IP [EIPStatusScrn]”.

#### Creating a List of Adapters

It is easy to build a rack configuration from scratch with the configuration tool.  First, create the list of adapters.

- Click Clear All to clear the “scratch pad” tables used by the wizard.

- Open the “Adapter List” by right-clicking the down-arrow next to the field.

- Enter a unique Name, Rack and TargetIP for each adapter.

- Enter the starting InputAddress and OutputAddress for each adapter.

- Click Check Banks to update the values in NextAvailInpBank and NextAvailOutBank to assist in determining where to start each adapter.

#### Creating a List of Modules

Next, create a list of modules to match the physical configuration of each rack.

- Open the Module List by right-clicking the down-arrow next to the field.

- Enter a unique Name for each module.

- Select a module Type from the pre-defined list of modules for each module.

- Select the associated adapter in the Adapter field.

- Click Apply.  Rack and all the other columns will be updated automatically.  For most modules, that is all that need be done.  If parameters are needed, they can be added manually in the Param columns.

#### Importing a CSV File

The EtherNet/IP Configuration Tool can read the CSV files provided by the Electrical department.  The CSV file provides all needed data to fill the EIPAdapters and EIPModules tables automatically.

NOTE:  Before reading the CSV file, set Octal Mode using the button on the Create Conveyor Part wizard screen.

NOTE:  If you keep the CSV files in the RUN directory, they will get backed up along with the rest of the configuration.

- Enter the directory and file name of the CSV file to import in the “Import CsvFileNm” field.  The path is relative to the RUN directory or use an absolute path.
Example:  TestCSV4.csv
(equivalent to /SortPlus/RUN/TestCSV4.csv when FPC is installed in the standard spot)

- Click the Read CSV button to import the file.

- View the “Config List” by right-clicking the down-arrow next to the field.  You will see a list of all the imported records from the CSV file.

When the Update EIP button is clicked, an adapter record is created if a module type is found with a MinSlot and MaxSlot of 0.  The adapter record is updated with the InputAddress and OutputAddress of the first input and output bank defined for the adapter (or for the modules assigned to the adapter).  The IP address and RPI value are updated for the adapter if they are defined, as well.  A module record is created for all modules found that have non-zero MinSlot and MaxSlot values using the data in the ModuleType table.

NOTE:  The CSV file must have the exact number of modules in the Type column that are needed for the IO rack.  If a module has multiple banks, leave the Type column blank for the additional banks.

Meaning of the columns in the CSV file:

- Rack – Not used in the import but must be present in the file.

- Card Type – Must match a card type defined in the “Supported Modules” list.

- I or O – Indicates the type of bank being defined. I = input, O = output.

- Fortna Word – The FPC word value associated with the bank, in octal.

- Bit – The bit positions to be used by the bank, in octal, in the form of a range.

- Bank – The bank (byte) position associated with the module in the rack.  See the details for each particular type of IO, as they all use different allocation methods.  If the module requires word alignment, the bank must be an even number.  The values are decimal and refer to the row in the associated InputData or OutputData table.

- IP – The IP address of the adapter or module.  Most module types do not require an IP, as they are referenced through their adapter.

- RPI – Requested Packet Interval, in milliseconds.  Common values are 10 for control applications and 20 for less-critical items.  The Allen-Bradley 1783-EMS switch requires an update only every 500 milliseconds.  These values are determined by the Electrical department.

CSV File Example:

| Rack | Card Type | I or O | Fortna Word | Bit | Bank | IP | RPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP18 | 1734-AENT | I | 2270 | 00 -- 07 | 0 | 192.168.1.51 | 20 |
| CP18 |  | I | 2270 | 10 -- 17 | 1 |  |  |
| CP18 |  | I | 2271 | 00 -- 07 | 2 |  |  |
| CP18 |  | I | 2271 | 10 -- 17 | 3 |  |  |
| CP18 |  | I | 2272 | 00 -- 07 | 4 |  |  |
| CP18 |  | I | 2272 | 10 -- 17 | 5 |  |  |
| CP18 |  | I | 2273 | 00 -- 07 | 6 |  |  |
| CP18 |  | I | 2273 | 10 -- 17 | 7 |  |  |
| CP18 | 1734-IB8 | I | 2200 | 00 -- 07 | 8 |  |  |
| CP18 | 1734-IB8 | I | 2200 | 10 -- 17 | 9 |  |  |
| CP18 | 1734-IB8 | I | 2201 | 00 -- 07 | 10 |  |  |
| CP18 | 1734-IB8 | I | 2201 | 10 -- 17 | 11 |  |  |
| CP18 | 1734-IB8 | I | 2202 | 00 -- 07 | 12 |  |  |
| CP18 | 1734-OB8E | I | RESERVE |  | 13 |  |  |
| CP18 |  | O | 2202 | 10 -- 17 | 0 |  |  |
| CP18 | 1734-OB8E | I | RESERVE |  | 14 |  |  |
| CP18 |  | O | 2203 | 00 -- 07 | 1 |  |  |
| CP18 | 1734-OB8E | I | RESERVE |  | 15 |  |  |
| CP18 |  | O | 2203 | 10 -- 17 | 2 |  |  |
| CP18 | 1734-OB8E | I | RESERVE |  | 16 |  |  |
| CP18 |  | O | 2204 | 00 -07 | 3 |  |  |
| CP18 | 1734-OB8E | I | RESERVE |  | 17 |  |  |
| CP18 |  | O | 2204 | 10 -- 17 | 4 |  |  |
| CP18 | 1734-IA4 | I | 2205 | 00 -- 03 | 18 |  |  |
| CP18 | 1734-IA4 | I | 2205 | 10 -- 13 | 19 |  |  |
| CP18 | 1734-IA4 | I | 2206 | 00 -- 03 | 20 |  |  |
| CP18 | 1734-IA4 | I | 2206 | 10 -- 13 | 21 |  |  |
| CP18 | 1734-IA4 | I | 2207 | 00 -- 03 | 22 |  |  |
| CP18 | 1734-IA4 | I | 2207 | 10 -- 13 | 23 |  |  |
| CP18 | 1734-IA4 | I | 2210 | 00 -- 03 | 24 |  |  |
| CP18 | 1734-IA4 | I | 2210 | 10 -- 13 | 25 |  |  |
| CP18 | 1734-IA4 | I | 2211 | 00 -- 03 | 26 |  |  |
| CP18 | 1734-IA4 | I | 2211 | 10 -- 13 | 27 |  |  |
| CP18 | 1734-IA4 | I | 2212 | 00 -- 03 | 28 |  |  |
| CP18 | 1734-IA4 | I | 2212 | 10 -- 13 | 29 |  |  |
| CP18 | 1734-IA4 | I | 2213 | 00 -- 03 | 30 |  |  |
| CP18 | 1734-IA4 | I | 2213 | 10 -- 13 | 31 |  |  |
| CP18 | 1734-IA4 | I | 2214 | 00 -- 03 | 32 |  |  |
| CP18 | 1734-IA4 | I | 2214 | 10 -- 13 | 33 |  |  |
| CP18 | 1734-OA4 | O | 2215 | 00 -- 03 | 5 |  |  |
| CP18 | 1734-OA4 | O | 2215 | 10 -- 13 | 6 |  |  |
| CP18 | 1734-OA4 | O | 2216 | 00 -- 03 | 7 |  |  |
| CP18 | 1734-OA4 | O | 2216 | 10 -- 13 | 8 |  |  |
| CP18 | 1734-OA4 | O | 2217 | 00 -- 03 | 9 |  |  |
| CP18 | 1734-OA4 | O | 2217 | 10 -- 13 | 10 |  |  |
| CP18 | 1734-OA4 | O | 2220 | 00 -- 03 | 11 |  |  |
| CP18 | 1734-OA4 | O | 2220 | 10 -- 13 | 12 |  |  |
| CP18 | 1734-OA4 | O | 2221 | 00 -- 03 | 13 |  |  |
| CP18 | 1734-OA4 | O | 2221 | 10 -- 13 | 14 |  |  |
| CP18 | 1734-OA4 | O | 2222 | 00 -- 03 | 15 |  |  |
| CP19 | 1734-AENT | I | 2370 | 00 -- 07 | 40 | 192.168.1.52 | 20 |
| CP19 |  | I | 2370 | 10 -- 17 | 41 |  |  |
| CP19 |  | I | 2371 | 00 -- 07 | 42 |  |  |
| CP19 |  | I | 2371 | 10 -- 17 | 43 |  |  |
| CP19 |  | I | 2372 | 00 -- 07 | 44 |  |  |
| CP19 |  | I | 2372 | 10 -- 17 | 45 |  |  |
| CP19 |  | I | 2373 | 00 -- 07 | 46 |  |  |
| CP19 |  | I | 2373 | 10 -- 17 | 47 |  |  |
| CP19 | 1734-IB8 | I | 2300 | 00 -- 07 | 48 |  |  |
| CP19 | 1734-IB8 | I | 2300 | 10 -- 17 | 49 |  |  |
| CP19 | 1734-IB8 | I | 2301 | 00 -- 07 | 50 |  |  |
| CP19 | 1734-IB8 | I | 2301 | 10 -- 17 | 51 |  |  |
| CP19 | 1734-OB8E | I | RESERVE |  | 52 |  |  |
| CP19 |  | O | 2302 | 00 -- 07 | 22 |  |  |
| CP19 | 1734-OB8E | I | RESERVE |  | 53 |  |  |
| CP19 |  | O | 2302 | 10 -- 17 | 23 |  |  |
| CP19 | 1734-OB8E | I | RESERVE |  | 54 |  |  |
| CP19 |  | O | 2303 | 00 -- 07 | 24 |  |  |
| CP19 | 1734-IA4 | I | 2303 | 10 -- 13 | 55 |  |  |
| CP19 | 1734-IA4 | I | 2304 | 00 -- 03 | 56 |  |  |
| CP19 | 1734-IA4 | I | 2304 | 10 -- 13 | 57 |  |  |
| CP19 | 1734-IA4 | I | 2305 | 00-- 03 | 58 |  |  |
| CP19 | 1734-IA4 | I | 2305 | 10 -- 13 | 59 |  |  |
| CP19 | 1734-IA4 | I | 2306 | 00 -- 03 | 60 |  |  |
| CP19 | 1734-IA4 | I | 2306 | 10 -- 13 | 61 |  |  |
| CP19 | 1734-IA4 | I | 2307 | 00 -- 03 | 62 |  |  |
| CP19 | 1734-IA4 | I | 2307 | 10 -- 13 | 63 |  |  |
| CP19 | 1734-IA4 | I | 2310 | 00 -- 03 | 64 |  |  |
| CP19 | 1734-IA4 | I | 2310 | 10 -- 13 | 65 |  |  |
| CP19 | 1734-IA4 | I | 2311 | 00 -- 03 | 66 |  |  |
| CP19 | 1734-OA4 | O | 2311 | 10 -- 13 | 25 |  |  |
| CP19 | 1734-OA4 | O | 2312 | 00 -- 03 | 26 |  |  |
| CP19 | 1734-OA4 | O | 2312 | 10 -- 13 | 27 |  |  |
| CP19 | 1734-OA4 | O | 2313 | 00 -- 03 | 28 |  |  |
| CP19 | 1734-OA4 | O | 2313 | 10 -- 13 | 29 |  |  |
| CP19 | 1734-OA4 | O | 2314 | 00 -- 03 | 30 |  |  |
| CP19 | 1734-OA4 | O | 2314 | 10 -- 13 | 31 |  |  |
| VFD1506 | YASKAWA-SI-EN3-ESC | I | 2500 | 00 -- 07 | 72 | 192.168.1.151 | 12 |
|  |  | I | 2500 | 10 -- 17 | 73 |  |  |
|  |  | I | 2501 | 00 -- 07 | 74 |  |  |
|  |  | I | 2501 | 10 -- 17 | 75 |  |  |
|  |  | O | 2502 | 00 -- 07 | 38 |  |  |
|  |  | O | 2502 | 10 -- 17 | 39 |  |  |
|  |  | O | 2503 | 00 -- 07 | 40 |  |  |
|  |  | O | 2503 | 10 -- 17 | 41 |  |  |

#### Updating the Configio Table

The EtherNet/IP Configuration Tool can update the Configio table with the banks and words read from a CSV file.  Click Update ConfigIO to perform this task.

Each module word found in the Configio table is updated with the matching value from the CSV file based on the word, IO and bit values.  If the module word is not found, it is created.

NOTE:  The CSV file must have all the word and bank values needed for each of the IO cards defined.  If a card uses less than 8 bits (such as a Beckhoff card) the same word and bank can be listed multiple times.  The tool will find it is already defined and will not add a second entry.  You may see warning messages in this case, but you will find the IO is created correctly.

#### Enabling the Configio Records

The EtherNet/IP Configuration Tool can update the Configio table to enable the added records by clicking the Enable ConfigIO button.  This will cause the Interface and Process columns in Configio to be filled in for the related records.

#### Updating the Adapters and Modules

The EtherNet/IP Configuration Tool can update the EIPAdapters and EIPModules tables from the CSV file.  Click the Update EIP button to perform this task and prepare to create the XML configuration file for the RTA driver.  As each adapter and module is added, the required parameters are copied from the ModuleType table.  Parameters (if any) are loaded with default values, which can be modified, if needed.

#### Checking the Banks

The EtherNet/IP Configuration Tool will load the TargetIP, InputAddress and OutputAddress from the CSV file into the EIPAdapters table.  Use the Check Banks button to update the NextAvailInpBank and NextAvailOutBank values for each adapter (rack).  It will also update the InputBank and OutputBank values in the EIPModules table.  If the card type uses less than a full bank, the InputBit and OutputBit values will be updated with the starting bit location for each module.

Often, the Electrical department (or the Controls group) will opt to leave extra space between racks.  In this case, the InputAddress and/or OutputAddress values will need to be increased from the values shown by NextAvailInpBank and NextAvailOutBank.  Future expansion is much easier if the IO mapping is not tightly packed.

Verify the contents of the EIPAdapters and EIPModules tables are correct before proceeding.

#### Create the XML File

The EtherNet/IP Configuration tool can be used to create the XML configuration file used by the RTA driver.

- Enter the directory and file name for the new XML file to be written in the “Export XMLFileNm” field.  The path is relative to the RUN directory or use an absolute path.
Example:  FORTNA/TCP1-RTA1-eipcfg.xml
(equivalent to /SortPlus/RUN/FORTNA/TCP1-RTA1-eipcfg.xml when FPC is installed in the standard spot)

- Verify the adapters and modules are correct.

- Click Create XML to write the new file.

#### Read an XML File

The EtherNet/IP Configuration tool can read a previously-created XML file to use as a starting point for modifications.

- Enter the directory and file name of the XML file to read in the “Import XMLFileNm” field.  The path is relative to the RUN directory or use an absolute path.
Example:  /SortPlus/RUN/FORTNA/TCP1-RTA1-eipcfg.xml
(equivalent to FORTNA/TCP1-RTA1-eipcfg.xml when FPC is installed in the standard spot)

- Click Clear All to clear the “scratch pad” tables used by the wizard.

- Click Read XML to load the file.

- Verify the adapters and modules are correct.

- Make any needed changes.

- Click Create XML to write a new XML file to the location specified in the “Export XmlFileNm” field.

### Running Multiple Instances of the RTA Driver

Multiple instances of the RTA driver are supported but require additional configuration in FPC and QNX.

NOTE:  FPC Version 10.91 or later is required to support this feature.

In the FPC IOCard table, each instance is designated using the PCIDeviceIndex column.  Use 0 for the first instance, 1 for the second, and so on.  The instance number is used to create a distinct log file and a separate shared memory segment between the driver and FPC for the input and output data.  Use separate tables in the InputMenu and OutputMenu columns for the separate instances.

The QNX network configuration must be modified in file /etc/system/enum/include/net.  The required change activates the SO_REUSEPORT option to allow Unicast connections.  This will specify that all Unicast UDP packets are to be delivered to all sockets bound to the port.  This allows multiple RTA instances to connect to the same UDP port (Point2Point connections).

Find the “set” command in the section titled “all” (it will most likely be the only active entry in the file).  Change the “set” command to:
  set(IOPKT_CMD, io-pkt-v4-hc -ptcpip reuseport_unicast)

Once the change has been made, reboot and use “ps -ef | grep pkt” to verify that the option has been set:

Use “sloginfo | grep tcp” to check for errors.  The screen shot, below, shows no tcp errors detected.

Example 1:  One NIC is used.  This is needed because the total number of nodes exceeds 64.  Create two IPs for the same NIC since each iteration of the RTA Ethernet IP driver requires a separate IP Address.  Use the “ifconfig” command to add an alias:

  - Assuming the NIC has an existing IP address of 192.168.1.9 add the following two lines at the end of file /etc/rc.d/rc.local, just before the /usr/photon/bin/Photon lines, to permanently create an alias address at 192.168.1.10:

# Ethernet IP Alias for NIC wm1
ifconfig wm1 inet 192.168.1.10 alias

  - Reboot the machine.

  - Use “ifconfig” to view the assigned alias.

  - Use the command “netstat –r” to verify the routing to each IO rack once the driver is running.

  - Define the IP of the NIC in both of the XML files as the “interfaceip” for the respective connection.

NOTE:  In the QNX Network Configuration tool [phlip], there is an IP Aliases feature under advanced options on the Devices tab.  Do not use it to create IP aliases!  It has a bug.  If you add an alias, then click Done to save it, a line beginning with “alias” will be written to the section for the interface in /etc/net.cfg, and the alias will be created in the running system.  It will persist across reboots, but if the phlip tool is opened again, the alias will not be read from the file, and will be lost when Apply or Done are clicked.  Also, if you click Apply after adding an alias, it is not written to /etc/net.cfg at all, though the alias is created in the running system.  Use ifconfig in the rc.local file instead, as described above.

Example 2:  Two NICs are to be used.  In this example, each instance of the RTA driver is linked to a separate IP address in a separate netmask space.  The same technique can be used to define separate physical networks on separate subnets for the two NICs.

- Create two subnets on the same network in the 192.168.1.XXX space.  Set the subnet mask to 255.255.255.128 for each NIC.  This will route the packets through each NIC.  NIC1: 192.168.1.9, netmask 255.255.255.128 allows traffic from IPs 192.168.1.1 through 192.168.1.126.  NIC2: 192.168.1.130, netmask 255.255.255.128 allows traffic from IPs 192.168.1.128 through 192.168.1.254.

- Define the IP of each NIC in the XML file as the “interfaceip”.

- Use the command “netstat –r” to verify the routing to each IO rack once the driver is running.

## Optomux EtherNet (Opto22)

The Opto22 Optomux EtherNet product line supports several interface options.  They all connect to standard EtherNet Network Interface Card hardware in the Automation Controller.

- Opto Brain Board – G4EB2 (EtherNet for Pamux).  Supports two protocols.

        - Ethernet MMP – The Opto22 EtherNet MMP protocol is supported by the OptomuxEthernet protocol.  The EtherNet MMP protocol automatically configures the brain board, eliminating the need to use a Windows utility to configure the input and output mapping.  This is the preferred method for FortnaPlus.

        - EtherNet/IP protocol – Refer to section 2.1 EtherNet/IP (RTA) for the RTA EtherNet/IP configuration method.  A Windows utility is required to complete the EtherNet/IP configuration on the IO hardware.

      - Opto Brain Board – E1 (EtherNet for Optomux).  Supports the following protocol.

        - EtherNet MMP – The Opto22 EtherNet MMP protocol is supported by the OptomuxEthernet protocol.  The EtherNet MMP protocol automatically configures the brain board, eliminating the need to use a Windows utility to configure the input and output mapping.

The Optomux EtherNet driver is configured in FPC through the IOCard table and an XML file that describes the configuration of each Opto board and its associated input or output modules.

### File Locations

The OptomuxEthernet driver executable file is found in the RUN directory.

The OptomuxEthernet XML configuration file is found in the RUN/FORTNA directory.

The file name is generated as follows:
  <Machine Name>-<Configuration Name>-opmcfg.xml
  Example:  CP2-OPM-opmcfg.xml

The <Machine Name> is found on the lower-left corner of the Automation Controller’s main screen.  The <Configuration Name> is found in the IOCard table “Name” column for the row that is used to configure the OptomuxEthernet driver.

The OptomuxEthernet driver file is included in FPCUPD and FPCRUN tar archive files, so it will receive updates if they are available, along with updates to FortnaPlus.

Log files for the OptomuxEthernet driver, named optomux_log.<day>,  are created under the LOGS directory.

### XML Format

The OptomuxEthernet driver uses XML configuration files, similar to those used by the RTA EtherNet/IP (rtaeip) driver.

### Network Configuration

The OptomuxEthernet driver requires a dedicated network interface (NIC).  That means that the AC will need at least two network interfaces – one for the IO network and one for access to the outside world.

The network interfaces are configured using the QNX Network Configuration tool.  As the root user, open the tool using Configure | Network on the QNX “shelf”, or run phlip from a command-line window.

Use the “Device 1” NIC for the connection to the outside world.  Use the “Device 2” NIC for the IO network connection.  Choose connection type “Manual”.  The IP address for the IO network is set by the Electrical department, but is usually 192.168.1.9 for a system using a single IO NIC.

### Opto22 Board IP Configuration – Bootp

Opto22 Brain boards are shipped with an IP address of 0.0.0.0.  They will also revert to that address after a reset.  The address, subnet mask and other network parameters may be set using the QNX bootp protocol daemon.

Add configuration for the Opto22 brain boards to the file /etc/bootptab to define the relationship between the board’s MAC address and the desired IP address.  The bootptab file will already have a “global.dummy” section:

Global.dummy:\
	:hd=/boot:\
	:hn:bf=null:

Add a line for each brain board to be configured, using this example as a template:

Adapter1:tc=global.dummy:ht=ethernet:ha=00.a0.3d.03.93.a6:sm=255.255.255.0:ip=192.168.1.131:

The MAC address can be found on a label applied across the network connectors, as shown in this photo (00.A0.3D.03.93.A6).  The red button just above the last part of the MAC address is the reset button.  This backplane uses “quad” IO modules, but the same is true for the “single” ones:

Substitute the board’s MAC address and the desired IP address for the text in bold.  Notice the reference to global.dummy, which incorporates the parameters defined in that section.  The MAC address elements must be separated by “.” characters, not “:”!

Make sure the IO network has been wired and the Opto22 brain board is powered on.

If the brain board does not have an IP address assigned, the Status light on the board will flash orange, four times per second.  If the Status light is green, an IP address has already been assigned.  To reset, and allow the IP address to be updated, press and hold the Reset button on the board for one second.

Start the bootpd daemon once the configuration file has been updated:

bootpd -s -dd

You can monitor the bootpd progress in the QNX system log, found in /var/log/syslog.  The following example shows the interaction with a brain board as the bootpd daemons sets its IP address:

Mar 20 10:36:45 nto bootpd[1519663-1]: reading "/etc/bootptab"

Mar 20 10:36:45 nto bootpd[1519663-1]: read 3 entries (1 hosts) from "/etc/bootptab"

Mar 20 10:37:20 nto bootpd[1519663-1]: request from Ethernet address 00:A0:3D:03:93:A6

Mar 20 10:37:20 nto bootpd[1519663-1]: found 192.168.1.131 adapter1

Mar 20 10:37:20 nto bootpd[1519663-1]: trying /boot/null.adapter1

Once the IP address has been set, the Status light will stop flashing orange and turn steady green.  The bootpd daemon can be stopped using Ctrl-C.  The brain board stores and retains the IP address so bootpd is not needed during normal operation, however, since maintenance personnel may push the reset button, it is best to set bootpd to run automatically at startup.

Use the START_BOOTPD variable in /etc/rc.d/rc.local, if it is present, to enable bootpd to start automatically at boot time.  Set the variable to 1 for all ACs that have OptoMux EtherNet IO.  Older version of rc.local do not have START_BOOTPD defined.  In that case, add the following lines near the end of /etc/rc.d/rc.local:

# Start bootpd
if test -f /etc/bootptab; then
  if test -x /usr/sbin/bootpd; then
    /usr/sbin/bootpd -s -dd &
  fi
fi

If a brain board is changed, the new MAC address must still be added to /etc/bootptab, but a support call will be avoided if bootpd is running when an existing brain board is reset!

### Device Configuration

The device configuration is specified by the <Machine>-<Config>-opmcfg.xml file found in the RUN/FORTNA directory.  It is best to use the configuration wizard found in FortnaPlus on the IO Utilities menu under “Opto 22 Ethernet”, which is similar to the one used for EtherNet/IP.  The Electrical department will prepare CSV files to expedite creation of the configuration using this method.  The resulting files are text, however, and can be inspected with any text editor.  Alternatively, a free XML editor called gedit can be found at: http://ftp.gnome.org/pub/GNOME/binaries/win32/gedit/

Here is a brief example of the xml file contents for a single brain board with one quad AC input module:

<?xml version=”1.0” encoding=”UTF-8”?>
<Optomux	name=”CP2”
	Version=”1”
	interfaceip=”192.168.1.9”>
	<Adapter	targetip=”192.168.1.51”
		name=”Adapter1”
		nodeid=”1”
		InputAddress=”0”
		OutputAddress=”0”
		connection=”UDP-MMP
		type=”ADAPTER-G4”
		rpi_us=”10000”>
		<Module name=”IAC5Q-1”
			type=”IAC5Q”
			slot=”1”>
		</Module>
	</Adapter>
</Optomux>

### XML Number Format

Many of the numeric parameters in the XML file support decimal, octal, or hexadecimal formats.  Decimal number appear without a prefix, like 3458.  Octal numbers are denoted with a leading zero, like 04567.  Hexadecimal numbers are denoted with a leading “0x”, like 0xcf5.

### XML Optomux Element

The <Optomux> element in the XML file indicates the start of the Optomux Ethernet configuration.  It has the following attributes:

| Optomux Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the entire Optomux configuration. The name is used in the optomux_log.<day> log file and should contain the name of the machine, area, or control panel. |
| version | The version number is to support future changes to the format of the configuration file. The current version “1” should be used. |
| interfaceip | The IP Address of the QNX Ethernet NIC card connected to the I/O panel. The IP Address “192.168.1.9” is recommended. |

### XML Adapter Element

The <Adapter> element in the XML file is contained within the <Optomux> element.  It has the following attributes:

| Adapter Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the entire Adapter configuration. The name is used in the optomux_log.<day> log file and should contain the name of the device and/or IP address. The name is used in the IO Adapter table and should be unique. |
| targetip | The IP address of the Opto brain board the driver will connect to. See section 2.2.4. Assigned by the Electrical department. |
| nodeid | The node number. Used with the Optomux protocol. Not used with the MMP Protocol. |
| InputAddress | The offset into the table selected in IOCard_InputMenu where the devices on the adapter will start. The offset is in bytes. Always start the InputAddress attribute on an even number. Unused buffer space can be left between Adapter elements. In RSNetWorx, the offset value is the Scanlist Configuration Input Address column multiplied by 2. Decimal, octal, and hexadecimal values are supported. |
| OutputAddress | The offset into the table selected in IOCard_OutputMenu where the devices on the adapter will start. The offset is in bytes. Always start the OutputAddress attribute on an even number. Unused buffer space can be left between Adapter elements. In RSNetWorx the offset value is the Scanlist Configuration Input Address column multiplied by 2. Decimal, octal, and hexadecimal values are supported. |
| connection | The connection type: - UDP_MMP – Opto MMP protocol communicates with all Opto brain boards. |
| type | Module Types: - ADAPTER-G4 – G4 brain board - ADAPTER-E1 – E1 brain board |
| rpi_us | The read time for inputs and the write time for outputs, in microseconds. (typically set to 10000 – 10 milliseconds) |

### XML Module Element

The <Module> element in the XML file is contained within the <Adapter> element.  It has the following attributes:

| Module Element |  |
| --- | --- |
| Attribute | Value |
| name | A name for the module configuration. The name attribute is used in the optomux_log.<day> log file and should contain the name of the device and/or slot. |
| type | The device type in the slot. Module Types: - G4IAC5 – Single AC Input - G4IDC5 – Single DC Input - G4IDC5K – Single DC Input - G4OAC5 – Single AC Output - G4ODC5 – Single DC Output - IAC5Q – Quad AC Input - IDC5Q – Quad DC Input - OAC5Q – Quad AC Output - ODC5Q – Quad DC Output |
| slot | The slot number of the device. The first module is always device 1. The devices should be configured in numeric order. Decimal, octal, and hexadecimal values are supported. |

### G4EB2 Brain Board

The G4EB2 brain board adapter has a fixed read and write size.

Input Banks:  Four banks of input from the modules.  0 = OFF, 1 = ON.

Output Banks:  Four banks of output to the modules.  0 sets the output OFF, 1 sets the output ON.

### E1 Brain Board

The E1 brain board adapter has a fixed read and write size.

Input Banks:  Two banks of input from the modules.  0 = OFF, 1 = ON.

Output Banks:  Two banks of output to the modules.  0 sets the output OFF, 1 sets the output ON.

### Importing a CSV File for Optomux EtherNet

The Opto22 Ethernet configuration tool can read a CSV file.  The file is a comma-separated list of values, where each line in the file provides individual bank, adapter, and module records.

NOTE:  Set “Octal Mode” in the Create Conveyor Part wizard before reading the CSV file.

### CSV File Example for Optomux EtherNet

The following example is based on two racks using quad modules.  Each G4EB2 adapter requires four input banks and four output banks, regardless of the actual number of installed input or output modules.

| Rack, | Card Type, | I or O, | Fortna Word, | Bit, | Bank, | IP, | RPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 | ADAPTER-G4 |  |  |  |  | 192.168.1.51 | 15 |
| CP1 | IAC5 | I | 200 | 00 -- 03 | 0 |  |  |
| CP1 | OAC5 | O | 200 | 04 -- 07 | 0 |  |  |
| CP1 | IDC5 | I | 200 | 10 -- 13 | 1 |  |  |
| CP1 | ODC5 | O | 200 | 14 -- 17 | 1 |  |  |
| CP1 | IAC5 | I | 201 | 00 -- 03 | 2 |  |  |
| CP1 | OAC5 | O | 201 | 04 -- 07 | 2 |  |  |
| CP1 | IDC5 | I | 201 | 10 -- 13 | 3 |  |  |
| CP1 | ODC5 | O | 201 | 14 -- 17 | 3 |  |  |
| CP2 | ADAPTER-G4 |  |  |  |  | 192.168.1.52 | 15 |
| CP2 | IAC5 | I | 202 | 00 -- 03 | 4 |  |  |
| CP2 | OAC5 | O | 202 | 04 -- 07 | 4 |  |  |
| CP2 | IDC5 | I | 202 | 10 -- 13 | 5 |  |  |
| CP2 | ODC5 | O | 202 | 14 -- 17 | 5 |  |  |
| CP2 | IAC5 | I | 203 | 00 -- 03 | 6 |  |  |
| CP2 | OAC5 | O | 203 | 04 -- 07 | 6 |  |  |
| CP2 | IDC5 | I | 203 | 10 -- 13 | 7 |  |  |
| CP2 | ODC5 | O | 203 | 14 -- 17 | 7 |  |  |

The next example is based on a single rack using single modules.  Each G4EB2 adapter requires four input banks and four output banks, regardless of the actual number of installed input or output modules.

| Rack,; Card Type,; I or O,; Fortna Word,; Bit,; Bank,; IP,; RPI. CP2; ADAPTER-G4; 192.168.1.51; 10. CP2; G4IDC5; I; 100; 00 -- 00; 0. CP2; G4IDC5K; I; 100; 01 -- 01; 0. CP2; G4IDC5K; I; 100; 02 -- 02; 0. CP2; G4IDC5K; I; 100; 03 -- 03; 0. CP2; G4IDC5K; I; 100; 04 -- 04; 0. CP2; G4IDC5K; I; 100; 05 -- 05; 0. CP2; G4IDC5K; I; 100; 06 -- 06; 0. CP2; G4OD5C; O; 100; 07 -- 07; 0. CP2; G4OD5C; O; 100; 10 -- 10; 1. CP2; G4OD5C; O; 100; 11 -- 11; 1. CP2; G4ODC5R; O; 100; 12 -- 12; 1. CP2; G4ODC5R; O; 100; 13 -- 13; 1. CP2; G4ODC5R; O; 100; 14 -- 14; 1. CP2; G4ODC5R; O; 100; 15 -- 15; 1. CP2; G4ODC5R; O; 100; 16 -- 16; 1. CP2; X4ODC5R; O; 100; 17 -- 17; 1. CP2; G4IAC5; I; 101; 00 -- 00; 2. CP2; G4IAC5; I; 101; 01 -- 01; 2. CP2; G4IAC5; I; 101; 02 -- 02; 2. CP2; G4IAC5; I; 101; 03 -- 03; 2. CP2; G4IAC5; I; 101; 04 -- 04; 2. CP2; G4IAC5; I; 101; 05 -- 05; 2. CP2; G4IAC5; I; 101; 06 -- 06; 2. CP2; G4IAC5; I; 101; 07 -- 07; 2. CP2; G4IAC5; I; 101; 10 -- 10; 3. CP2; G4IAC5; I; 101; 11 -- 11; 3. CP2; G4IAC5; I; 101; 12 -- 12; 3. CP2; G4OAC5; O; 101; 13 -- 13; 3. CP2; G4OAC5; O; 101; 14 -- 14; 3. CP2; G4OAC5; O; 101; 15 -- 15; 3. CP2; G4OAC5; O; 101; 16 -- 16; 3. CP2; G4OAC5A; O; 101; 17 -- 17; 3. |  |  |  |  |  |  | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Profibus (Molex SST)

The Molex SST Profibus PCI card supports the Profibus protocol.  It has proven to be a fast, reliable, and durable IO interface when used with FPC.  The Electrical department uses a Windows-based software tool to generate a configuration file that describes the IO rack to be controlled.  This “bss” file is then loaded into the card’s non-volatile memory using a configuration tool in QNX.  The card interacts with FPC through shared memory, which is mapped through the input and output data tables and the Configio table.  This section describes how to perform these tasks and how to configure FPC to use Profibus IO.

### Gather Necessary Profibus Items

You will need the “bss” rack configuration file from the Electrical department, and the archive of Profibus tools for use in QNX.  The Profibus tools can be found on the ftp4 server in file:
  ftpdata/JobsArchive/Rel11/Utilities/ProfibusStartup.tar.gz

Put the files on the AC in the /tmp folder.

### Install the Profibus Tools on QNX

Install the Profibus tools within the FPC RUN directory structure so they will be included in backups.  Put a copy of the bss file with them for the same reason.  The preferred spot to put them is the RUN directory, but you may find them in RUN/FORTNA on some jobs.  When the tools archive is expanded, it creates a directory called ProfibusStartup that contains four files.

cd /SortPlus/RUN
tar zxvf /tmp/ProfibusStartup.tar.gz
cd ProfibusStartup

Now add the bss file into the new directory:

cp /tmp/<file from electrical>.bss .

Note the space and “dot” at the end of the previous line, meaning “copy to here”.

You should now have five files in the new directory, and you are ready to configure the Profibus card.

### Configure the Profibus Card

Make sure FPC is NOT running while configuring the Profibus card.

The file README_ProfibusStartup.txt explains use of the ProfibusStartup program, similar to this section.  ProfibusStartup is the program itself.  You can also run the program with no parameters to see a help message.  The pfb3.ss3 file is the “Profibus Personality” which will be loaded to the card first to teach it how to “speak” Profibus.  This file is provided by the manufacturer.  The last file, PciProfibus.bss, is a placeholder “empty” configuration.  The program will look for a file by this name and load it to the card.  It is best to copy the bss file from Electrical over top of this file immediately before each use and preserve the original with a descriptive name to limit the confusion.

Issue these commands, in this order, to configure the card:

First, reset the card:

./ProfibusStartup Reset

Next, boot the card to load the Profibus personality from the pfb3.ss3 file:

./ProfibusStartup Boot

Next, load the configuration from the PciProfibus.bss file:

./ProfibusStartup LoadBSS

Check the result:

./ProfibusStartup Status

(This command can be run to check the card status at any time)

Note:  If more than one Profibus card is to be used, you must add Cardn to each statement above, where n is 0, 1, 2.  Each card must be configured separately.

Note:  FPC will need to load the pfb3.ss3 file each time it initializes the Profibus card, so a copy must be present in RUN.  Copy it to there are this time:

cp pfb3.ss3 ..

Note the space and “dot dot” at the end of the line, meaning “copy to the directory above”.

### Configure FPC to use the Profibus Card – IOCard Table

An entry must be made in the IOCard table to connect FPC to the Profibus card.    The following table shows all the relevant fields.  If a field is omitted, the Profibus card does not use it.

| IOCard Table – Profibus |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select Profibus SST. |
| BasePort | Hex | Set automatically in QNX6. The base port is used to identify the dual-port memory area for the card. |
| ResetPort | Hex | Not used in QNX6. |
| pStatus | String | Updated by the driver with the card status, taken from table Errors. |
| InputMenu | PopUp | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | PopUp | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Hex | Set by the driver. Running state = 2. Set to 0 on reset, 1 during initialization if the card is defined. |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Hex | Internal Address assigned by the operating system for the shared memory area. |
| Error | Hex | Error code returned from the IO card or the driver process. |
| InputPtr | Int | Internal index that points to the InputMenu selected. |
| OutputPtr | Int | Internal index that points to the OutputMenu selected. |
| MaxInput | Int | Set by the driver. Maximum number of bytes read from the card based on the size of the input table. |
| MaxOutput | Int | Set by the driver. Maximum number of bytes written to the card based on the size of the output table. |
| IntrpPort | Hex | Set automatically in QNX6. |
| CardState | Int | Set by the driver. The current state of the driver state machine. |
| IntrrAddr | Hex | Set by the driver. The Internal Address used by the card and assigned by the operating system for the second shared memory area. |
| TimeLastCmd | Int | Set by the driver. The time of the last command sent to the card. |
| LoopCnt | Int | Set by the driver. Indication of activity. Rolls to 0 after 9999. |
| Machine | PopUp | Choose this machine. |
| PCIDeviceIndex | Int | If more than one of a given card type is present, use this to select a particular card. Zero represents the first card. |
| NodeIndex | Int | Set by the driver. Tracks the last adapter or node updated in diagnostics. |
| LogLevel | PopUp | Controls log output. A higher number means more logging. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Int | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Int | Internal storage used by the RunDelay timer. |

Note that FPC in QNX6 will automatically detect the address of the Profibus card.  Older versions required it to be set manually.  In QNX4, the command “show_pci -v” was used to find the card, which has a vendor ID of 133dh and a Device ID of 33h.  The relevant information is found on the Base Address line, with BasePort taken from the “length 262144” element, and ResetPort taken from the “length 32” element.  This information can be found in QNX6 using the “pci -vv” command.

Example:  BasePort=feb80000, ResetPort=febff800
Base Address   = MEM@febffc00h,32bit length 128 MEM@feb80000h,32bit length 262144 MEM@febff800h,32bit length 32

### IO Adapter Status Table for Profibus [KTX_RIO_ADAPTERS]

The IO Adapter Status table, a.k.a., KTX_RIO_ADAPTERS, is automatically populated with one entry per Profibus adapter.

| IO Adapter Status Table (Profibus) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | A unique name for the adapter. |
| Adapter | Int | Adapter index. Automatically filled in. |
| Active | Y/N | Indicates if the adapter is being monitored. |
| pStatus | String | Text description of the status for each adapter. Updated by the software. The Fault Byte contains an error code if the Profibus card returns an error. A text description of that error will appear here. |
| Fault Byte | Hex | Contains the value found in the Profibus node status register. The value of the register is converted to a text message that is displayed in the pStatus field. |
| IOCard | PopUp | The Profibus card connected to this device. Select a card from the IOCard list. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| InpStatusIndx | Int | If InpStatusIndx and StatusMask are set to a non-zero value, the software will read the status from the specified input byte. Two bytes are read – the byte selected in InpStatusIndx and InpStatusIndx + 1. The InpStatusIndx is a numeric index into the table selected in the IOCard_InputMenu field |
| StatusMask | Binary | Select the bits that will be checked for an error. The result of filtering the InpStatusIndx status bits with the StatusMask are stored in the pCurrStatus field. |
| pCurrStatus | Binary | The result generated from data retrieved from InpStatusIndx and StatusMask. |

### Profibus Configio Table

Profibus inputs and outputs are mapped into FPC by adding entries to the Configio table.  Each entry can map up to 8 bits.

| Configio Table – Profibus |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Desc | String | Enter a unique description for the record. |
| Bank | Int | Address of the input or output byte to read or write (Address is defined by the Profibus utility). For example, if an input module is mapped to “Byte 0” of the Profibus interface card, set the Bank to 0 to map the inputs to the FortnaDT IO table. To map inputs and outputs to the same Low byte of the FortnaDT table, configure the IO addresses in Profibus so the input and output address for the data will use the same byte address. Enter the address into the Bank field. |
| Octal_Word | Int or Oct | FortnaDT table index where the input byte will be read from or the output byte written to. |
| LoHi | PopUp | Select either the Low or High byte (FortnaDT table) associated with the Profibus device. |
| In_Out | Binary | Each bit can be set to a 0 (0 indicates input), or a 1 (1 indicates output). If the bit is set to 0 the data will be read from the input table. If the bit is set to 1 the data will be written to the output table. |
| I_O_Type | PopUp | Select digital for the IO type. |
| Interface | PopUp | Select the record in table IOCard for the Interface. |
| Granularity | Int | The monitoring granularity. A value of 0 or 1 causes monitoring to occur on every pass through the critical loop. |
| Status | PopUp | Displays the IO status. |
| Process | PopUp | Select SORT – the process that will monitor the card for messages and transfer input and output data. |

### Profibus Troubleshooting

If communication cannot be established with all devices, check the termination switches on the connectors between devices and make sure they are set to OFF.  Only the final terminator should be set to ON.

## AC51 Pamux (Opto22)

The Opto22 AC51 Pamux card is available as a PCI card.  Choose IOCard_Type Pamux ac51.  The card requires an IOCard_BasePort address.  The driver will find the BasePort address automatically in QNX6.  You can look it up manually, if you like, using the “pci -vv” command.  Look for Vendor ID = 148ah, Device ID = ac51h and find the Base Address line:

Base Address   = MEM@fdefe000h,32bit length 128 MEM@fdefd000h,32bit length 16

In the example above, the IOCard_BasePort would be fdefd000.

The B4 brain board is connected to the AC51 card through a ribbon cable.  The B4 card is typically connected to a 32-point IO rack.  It has two LED indicators and 8 jumpers.  The left LED indicates if the card is selected (running).  The right LED indicates the state of the watchdog.  The first four jumpers are used to define the rack address.  In the table, below, a 0 represents ‘jumper removed’, and a 1 represents ‘jumper installed’.  They form a four-bit binary value from 0 to 15:

| Jumpers 4 thru 1 | Address (Rack number) |
| --- | --- |
| 0000 | 0 |
| 0001 | 1 |
| 0010 | 2 |
| … | … |
| 1111 | 15 |

Jumpers 5 through 8 must all be installed for proper watchdog operation.

Each PCI card can access up to 64 8-bit banks (or 512 points) of IO, by connecting to as many as 16 B4 racks, each with up to 32 points.

Note:  The ISA bus version of AC51 has been discontinued.

### IOCard Table Configuration for AC51

The FPC IOCard table defines the connection between FPC and the AC51 driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the AC51 driver.

| IOCard Table – Pamux AC51 |  |
| --- | --- |
| Column | Value or Description |
| Name | Enter a unique name for this configuration. |
| Desc | Enter a description for this configuration (optional). |
| Type | Choose “Pamux ac51”. |
| BasePort | Set automatically in QNX6. |
| ResetPort | Not used by the Pamux ac51 driver. |
| pStatus | Not used by the Pamux ac51 driver. |
| InputMenu | Not used by the Pamux ac51 driver. |
| OutputMenu | Not used by the Pamux ac51 driver. |
| Status | Set by the driver. 0 – Card is not active. 1 – Card initialization routine completed successfully. |
| Process | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Not used by the Pamux ac51 driver. |
| Error | Error code returned from the IO card or the driver process. 1001 – Board initialization error. Board type is not 1 – 3. 1002 – Machine does not match. |
| Machine | Choose the name of this machine. |
| InitStatus | Set by the driver. 1 indicates the card is present and initialized. |
| PCIDeviceIndex | Set this to 0 unless more than one instance of the ac51 driver is being used. Increment for each additional card. |
| PCI | Not used in QNX6. Set to Y for PCI card, N for ISA card in QNX4. |
| LogLevel | Not used by the Pamux ac51 driver! |
| ErrorOutput | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Internal storage used by the RunDelay timer. |

## DeviceNet (Allen-Bradley)

The Allen-Bradley DeviceNet PCI card is still supported.  See section 3.5 for information on retired DeviceNet cards.

FPC will automatically detect the PCI BasePort and IntrpPort addresses for the Allen-Bradley DeviceNet card.  You can look them up manually using the “pci -vv” command, however, if you wish.  The Allen-Bradley DeviceNet card is identified by Vendor ID=12a0h and Device ID=6h.  The addresses are taken from the “Base Address” line in the pci command output.  In the example, below, BasePort=feaf0000 and IntrpPort=feafe000.  The addresses are associated with the length 32768 and length 4096 entries, respectively:

Base Address   = MEM@feaffc00h,32bit length 128 IO@dc00h length 128 bytes MEM@feaf0000h,32bit length 32768 MEM@feafe000h,32bit length 4096 IO@dff4h length 4 bytes

There are two versions of the A-B card using VID=120a, DID=6 (for some unfathomable reason!).  They can be distinguished by the number of addresses reported by the pci command.  The original cards report four addresses, but some (newer?) cards report only two.  The driver detects the type of card and sets the addresses accordingly.

#### DeviceNet Configuration Software

The DeviceNet IO system uses a complex communication protocol and intelligent devices.  The protocol allows each device (if capable) to communicate through several different methods:

- As standard IO polled devices

- Using messages generated by change-of-state (high performance)

- Using bit stream messages

- Using explicit messages

DeviceNet configuration software from Allen-Bradley provides configuration tools that can configure each device (creating the EDS files) and configure the PC card.

Use the Allen-Bradley software to:

- Configure each DeviceNet node

- Configure the MAC ID

- Configure expansion modules

- Configure inputs and outputs

- Export the EDS file

#### A-B DeviceNet Configuration

Apply power to the network and connect the Automation Controller to the DeviceNet network.  Do not connect the A-B DeviceNet laptop configuration PC until after the DeviceNet card is connected and running in Config Mode 1.  The A-B DeviceNet software will set the baud rate of the DeviceNet network if it is connected first and prevent the DeviceNet card from appearing in the A-B software’s diagram.

The FPC DeviceNet driver requires two addresses.  The addresses are used by the driver to link to the card’s shared RAM memory (BasePort) and to the interrupt memory segment (IntrpPort).  When the driver starts, it reads the serial number from the card and stores it in the IOCard table SerialNumber field.

Note:  Verify a valid serial number has been read from the card.  If the serial number is not valid the card addresses are not correct!  You can use the “IO Card Init” function (found on the “IO Utilities” menu) to test this.  When the function is run, the lights on the card will go through their reset sequence if the connection has been established.

During the configuration process, the A-B card is started in two steps.  The first step is to start the driver with IOCard_ConfigMode = 1.  This will send a parameter command and a start command to the card and hold the card in the start state.  The A-B card must be held in the start state to allow configuration access by the A-B DeviceNet configuration software.

Parameter Command:  MacID = 0, Baud = 500

Once the card and the devices on the network have been configured to talk to each other, the card can be put into a RUN state by setting IOCard_ConfigMode = 0.

Note:  If FPC detects an error, the error code is displayed in the IOCard_Error field.  The error is also logged to the FPC logs.  The relevant messages can be found by searching the log files using “grep DeviceNetAB *.log”.

#### A-B DeviceNet Configuration Diagram

FPC is configured for use with DeviceNet by entries in several tables that are related as shown in the following diagram.

IOCard table

Defines the card and the IO interface tables

Input Data table

Real-time input data

Output Data table

Real-time output data

Configio table

Links input and output data to FortnaDT

Conveyor table

Defines the individual bits of the FortnaDT IO words.

DeviceNet card

Installed in PC

FortnaDT table

All Real-time IO for the system

#### A-B DeviceNet IOCard Table Configuration

The FPC IOCard table defines the connection between FPC and the DeviceNet driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the DeviceNet driver.

| IOCard Table – DeviceNet |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select DeviceNetAB from the card type options. |
| BasePort | Hex | The base port. Set automatically. |
| ResetPort | Hex | The reset port. Not used. |
| pStatus | String | Updated by the driver with the card status, taken from table Errors. |
| InputMenu | PopUp | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | PopUp | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Hex | Set by the driver. Valid values are 0 – 3. Reset state is 0, all others indicate a running state. |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Hex | Internal Address assigned by the operating system for the shared memory area. |
| Error | Hex | Error code returned from the IO card or the driver process. |
| InputPtr | Int | Internal index that points to the InputMenu selected. |
| OutputPtr | Int | Internal index that points to the OutputMenu selected. |
| MaxInput | Int | Set by the driver. Maximum number of bytes read from the card based on the size of the input table. |
| MaxOutput | Int | Set by the driver. Maximum number of bytes written to the card based on the size of the output table. |
| CurrMsg | Int | Set by the driver. Current explicit message number. |
| IntrpPort | Hex | Interrupt port. Set automatically. |
| CardState | Int | Set by the driver. The current state of the driver state machine. |
| IntrrAddr | Hex | Set by the driver. The Internal Address used by the card and assigned by the operating system for the second shared memory area. |
| ConfigMode | Int | Use this field to put the card in PROGRAM mode. A value of 0 selects RUN mode, the normal state. A value of 1 sets PROGRAM mode. The card must be in PROGRAM mode to configure the devices in the rack. This is done using an external program such as RSNetWorx. A value of 2 sets verbose logging mode. Note: ConfigMode=2 will fill the disk rather quickly! |
| TimeLastCmd | Int | Set by the driver. The time of the last command sent to the card. |
| SerialNumber | Hex | Set by the driver. The value is read from the card. |
| NumMsgs | Int | Set by the driver. The number of messages in the queue for the card. |
| Baud | Hex | The communication baud rate. 0=125K, 1=250K, 2=500K |
| LoopCnt | Int | Set by the driver. Indication of activity. Rolls to 0 after 9999. |
| InputCnt | Int | Set by the driver. Shows the inputs received. |
| Machine | PopUp | Choose this machine. |
| PCIDeviceIndex | Int | If more than one of a given card type is present, use this to select a particular card. Zero represents the first card. |
| NodeIndex | Int | Set by the driver. Tracks the last adapter or node updated in diagnostics. |
| LogLevel | PopUp | Controls log output. A higher number means more logging. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Int | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Int | Internal storage used by the RunDelay timer. |

#### A-B DeviceNet IO Adapter Status Table [KTX_RIO_ADAPTER]

The IO Adapter Status table, a.k.a., KTX_RIO_ADAPTERS, is automatically populated with one entry per DeviceNet adapter.

| IO Adapter Status Table (DeviceNet) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | A unique name for the adapter. |
| Adapter | Int | Adapter index. Automatically filled in. |
| Active | Y/N | Indicates if the adapter is being monitored. |
| pStatus | String | Text description of the status for each adapter. Updated by the software. |
| Config Byte | Hex | Not used for DeviceNet. |
| Fault Byte | Hex | Shows the error value returned from the card for the adapter. |
| IOCard | PopUp | The DeviceNet card connected to this device. Select a card from the IOCard list. |
| Rack | Int | Not used for DeviceNet. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| InpStatusIndx | Int | The InpStatusIndx is a numeric index into the table selected in the IOCard_InputMenu field. |
| StatusMask | Int | Select the bits that will be checked for an error. The result of filtering the InpStatusIndx status bits with the StatusMask are stored in the pCurrStatus field. |
| pCurrStatus | Int | The result generated from data retrieved from InpStatusIndx and StatusMask. |

The following table shows DeviceNet adapter status values and their meaning.

| DeviceNetSH and DeviceNetAB Adapter Errors (From IO Adapter Status) |  |
| --- | --- |
| Bit | Description |
| 00 | Card in slot position 0 has an error |
| 01 | Card in slot position 1 has an error |
| 02 | Card in slot position 2 has an error |
| 03 | Card in slot position 3 has an error |
| 04 | Card in slot position 4 has an error |
| 05 | Card in slot position 5 has an error |
| 06 | Card in slot position 6 has an error |
| 07 | Card in slot position 7 has an error |
| 10 | Node address switch setting has been changed |
| 11 | IO state: 0 – IDLE, 1 – RUN |
| 12 - 17 | Bits are not used |

#### A-B DeviceNet Explicit Message Definition [MessageList]

In addition to the IO data, the MessageList table may be used to define explicit messages.  The driver will process explicit (PCP Interbus) messages based on the definitions found there.  When the driver is started, it scans the MessageList table.  If errors are detected, the Status field will be set to zero and the error will be shown in the SndError field.  The DeviceNet initialization routine will verify the messages and initialize the pointers even if the Active field is set to N.  Messages may be activated while in RUN mode.

NOTE:  Set the MessageList Active field to N while making changes to prevent incompletely defined messages from being sent.  The card must be restarted using the “IO Card Init” function, or by restarting FPC.

Explicit messaging can be used to read device information and configure devices.  Each device understands a distinct set of messages and parameters.

Configuration consistency value:
	MAC ID:  2  Node number (depends on the device)
	Class:  1
	Instance:  1
	Attribute:  9  CCV attribute
	Service: 0x0E  (14 decimal)  Get (read) attribute
	Service: 0x10  (16 decimal)  Put (write) attribute

Parameter message for DeviceNet:
	MAC ID:  2  Node number (depends on device)
	Class:  0x0F  (15 decimal)  Parameter data
	Instance:  1 – n  Parameter to read or write
	Attribute:  1  Parameter value  (meaning depends on parameter)
	Service:  0x0E  (14 decimal)  Get (read) attribute
	Service:  0x10  (16 decimal)  Put (write) attribute

Note:  When configuring a DeviceNet message list, a table may be required for the receive message or the send message.  If a send message table is defined, it will contain the message to transmit to the device.  If a receive message table is defined, it will contain the message received.  The A-B DeviceNet driver allows these tables to be defined with string, integer, or unsigned byte data.

| IO Message List (DeviceNet) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the message. |
| Active | Y/N | Flag used to enable or disable this message. |
| Status | Int | Status value returned from the driver indicating if this message is being processed. Zero indicates the message is not being processed. |
| pStatus | String | An error message set by the driver if the message is not being processed. |
| SndError | Hex | Send error code returned from the driver, card, or device. |
| SndFault | Hex | Fault code returned from the driver, card, or device. |
| RcvError | Hex | Receive error code returned from the card or device. |
| IOCard | PopUp | Select a DeviceNet card from the IOCard table. |
| IPAddr | String | Not used by DeviceNet. |
| MacID | Int | Enter the MAC address of the node for DeviceNet applications (0-63). Enter an address of (2 – 63) for Interbus applications. |
| Service | Int | DeviceNetAB: For the AB software set the field to 14 dec to read, and 16 dec to write data to the device. DeviceNetSH and Interbus: Enter the device service (1-255). Since the Synergetics software uses the GetSetParameter function, the field must be set to 1 to read and 2 to write data to a device. |
| Class | Int | DeviceNetAB: Enter the device data class (1 – 65535). DeviceNetSH: Enter the device data class (1 – 255). Interbus: Not used |
| Instance | Int | DeviceNet: Enter the device instance (0 – 65535). Interbus: Enter the device object index (0 – 65535). |
| Attribute | Int | DeviceNet: Enter the device attribute (1 – 255). Interbus: Enter the object sub-index (0 – 255). |
| AssemblyInst | Int | Not used by DeviceNet. |
| SndBytes | Int | Enter the number of bytes to send. |
| SndMenu | PopUp | Select a FPC table to load send data from. If SndBytes is 0, the selection is not used. |
| SndSelc | Int | Not used by DeviceNet. |
| SndColumn | Int | Enter a FPC table column. If SndBytes is 0, the selection is not used. |
| SndRow | Int | Enter a FPC table row. If SndBytes is 0, the selection is not used. |
| RcvBytes | Int | Enter the Number of bytes to receive. The actual number of bytes received may not match the number entered. The driver will truncate or null-fill based on the actual number of bytes returned by the device. |
| RcvActual | Int | Actual number of bytes received. Set by the driver. |
| RcvMenu | PopUp | Select a FPC table where received data is to be saved. If RcvBytes is 0, the selection is not used. |
| RcvSelc | Int | Not used by DeviceNet. |
| RcvColumn | Int | Enter a FPC table column. If RcvBytes is 0, the selection is not used. |
| RcvRow | Int | Enter a FPC table row. If RcvBytes is 0, the selection is not used. |
| HowOften | Int | Enter the number of units of Increment to determine how often messages are sent. No messages will be sent if this is set to 0. |
| Increment | PopUp | Enter the units for the message send interval (hour, min, sec, etc.) |
| Type | PopUp | Enter the type of increment (absolute, elapsed, count, one shot) |
| Waiting | Int | Used by the driver to store the time of the last message sent. Clears when a message is received. |
| Pending | Int | Indicates if a message needs to be sent. |
| CurrentStat | Int | Used by the driver to manage messages. |
| WasStat | Int | Used by the driver to manage messages. |
| SndPtr | Int | Used by the driver to store the Menu/Column Pointer |
| RcvPtr | Int | Used by the driver to store the Menu/Column Pointer |
| SndMsgCtr | Int | Used by the driver to track messages sent |
| RcvMsgCtr | Int | Used by the driver to track messages received |
| SndMnuDataType | Int | Used by the driver to track the type of data in SndColumn. |
| RcvMnuDataType | Int | Used by the driver to track the type of data in RcvColumn. |
| TNS | Int | Not used by DeviceNet. |

#### A-B DeviceNet FortnaPlus Errors

The following error codes may appear in the IOCard_Errors field.

These errors are generated by the FPC DeviceNet driver:

| DeviceNet Errors Detected from FortnaPlus |  |
| --- | --- |
| Hex Value | Description |
| 1001 | Shared memory open error |
| 1002 | Shared memory MMAP error |
| 1005 | Not 8K of dual port memory |
| 1006 | Read / write shared memory test failed |
| 1007 | Watch dog test failed |
| 1008 | Board initialization error: no ready bit |
| 1009 | Row out of range |
| 1010 | Table not valid |
| 1011 | Table not defined |
| 1012 | Invalid column |
| 1013 | Send parameter command failed |
| 1014 | Table error (Pointer not found) |
| 1015 | Table type not valid |
| 1016 | Incorrect data type for column |
| 1018 | Watch dog is not set. Disabling the network |
| 1019 | Start command failed |
| 1020 | Mailbox is full |
| 1021 | Status is not equal to 1 |
| 1022 | Active is not set to 1 or Y |
| 1023 | The time increment is 0. Must be 1 or greater. |
| 1024 | Max messages have been exceeded. Message must wait. |
| 1025 | IOCard address error (BasePort) |
| 1026 | IOCard address error (IntrpPort) |
| 1027 | IO message timed out |
| 1028 | IOCard error: Ack and Req are not equal (Reboot to clear) |
| 1029 | IOCard error: SerialNumber does match previous value |
| 1100 | Card status is not 1 (message error) |
| 1101 | IOCard: Too many messages in process. Message must wait. |

These errors are generated by the A-B DeviceNet card:

| DeviceNetAB Errors Returned from the Card |  |
| --- | --- |
| Hex Value | Description |
| 2XXX | Network status errors returned from the card (card error) |
| 2004 | Node Not Responding (card error) |
| 200C | Response data too large (card error) |
| 2046 | Module failed duplicate node address check. |
| 2047 | Illegal data in scan list table |
| 2048 | Slave device stopped communicating |
| 2049 | Device key parameters do not match scan list table entry |
| 204A | Data overrun detected on port |
| 204B | Scan list does not match the devices found, or no scan list is active |
| 204C | No direct network traffic detected for module |
| 204D | Data size returned does not match scan list entry |
| 204E | Slave device in scan list table does not exist |
| 204F | Module has failed to transmit a message |
| 2050 | Module is in IDLE mode (normal during startup before going to run mode) |
| 2051 | Module is in FAULT mode |
| 2052 | Error detected in sequence of fragmented IO messages from device |
| 2053 | Slave device is returning error responses when module attempts to communicate with it |
| 2054 | Module is initializing the DeviceNet channel |
| 2055 | Run-time data size returned from slave device is not the size expected |
| 2056 | Device is producing idle state data while the scanner is in RUN mode |
| 2057 | Available for allocation |
| 2058 | Shared master has not allocated required choices |
| 2059 | Auto Device Replacement (ADR) error. Slave device responded with an error to the initialization data sent to it by the scanner, or the configuration table in the scanner’s flash memory is not valid for a slave node |
| 205A | User has disabled communication port. Reconfigure your module |
| 205B | Bus-off condition detected on comm port |
| 205C | No network power detected on comm port |
| 205F | Application FLASH update in progress |
| 2060 | Port is in test mode |
| 2060 | Module halted by user command |
| 2060 | Unrecoverable firmware failure |
| 2061 | Unrecoverable hardware failure |
| 4XXX | Network status errors returned from the device (device error) |
| 4100 | The explicit message response indicated an error; however, the explicit message response did not contain the error code. |

These errors are generated by the A-B DeviceNet devices:

| DeviceNetAB Errors Returned from the Device |  |
| --- | --- |
| Hex Value | Description |
| 4002 | Resource unavailable |
| 4008 | Service unavailable |
| 4009 | Invalid attribute |
| 400B | Already in request mode |
| 400C | Object state conflict |
| 400E | Attribute not settable |
| 400F | Permission check failed |
| 4010 | State conflict, device state prohibits execution |
| 4011 | Reply too long |
| 4013 | Not enough data received |
| 4014 | Attribute not supported |
| 4015 | Too much data received |
| 4016 | Object does not exist |
| 4018 | No Stored Attribute |
| 4019 | Store operation failure |
| 401F | Previous Set or Get still active |
| 4020 | Invalid Parameter |
| 40D0 – 40FF | Reserved for Class and Service Errors |

#### A-B DeviceNet Configio Table

The DeviceNet inputs and outputs are mapped into FPC by adding entries to the Configio table.  Each entry can map up to 8 bits.

| Configio Table – DeviceNet |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Desc | String | Enter a unique description for the record. |
| Bank | Int | Address of the input or output byte to read or write (Address is defined by the A-B configuration software). For example, if an input module is mapped to “Byte 0” of the DeviceNet interface card, set the Bank to 0 to map the inputs to the FortnaDT IO table. To map inputs and outputs to the same Low byte of the FortnaDT table, configure the IO addresses so the input and output address for the data will use the same byte address. |
| Octal_Word | Int or Oct | FortnaDT table index where the input byte will be read from or the output byte written to. |
| LoHi | PopUp | Select either the Low or High byte (FortnaDT table) associated with the device. |
| In_Out | Binary | Each bit can be set to a 0 (0 indicates input), or a 1 (1 indicates output). If the bit is set to 0 the data will be read from the input table. If the bit is set to 1 the data will be written to the output table. |
| I_O_Type | PopUp | Select digital for the IO type. |
| Interface | PopUp | Select the record in table IOCard for the Interface. |
| Granularity | Int | The monitoring granularity. A value of 0 or 1 causes monitoring to occur on every pass through the critical loop. |
| Status | PopUp | Displays the IO status. |
| Process | PopUp | Select SORT – the process that will monitor the card for messages and transfer input and output data. |

#### A-B DeviceNet Input Dual-Port RAM Table

When configuring a DeviceNet card, a table must be selected for storing the dual-port RAM input image.  The table must be defined with a single column of unsigned byte data.  The driver will copy the dual-port RAM image from the card based on the handshake mode selected (using the A-B configuration software).  The driver copies the data in block mode.  Set the card to buffered host-controlled mode so the data will not change while the copy is in progress.

#### A-B DeviceNet Output Dual-Port RAM Table

When configuring a DeviceNet card, a table must be selected for storing the dual-port RAM output image.  The table must be defined with a single column of unsigned byte data.  The driver will copy the dual-port RAM image from the card based on the handshake mode selected (using the A-B configuration software).  The driver copies the data in block mode.  Set the card to buffered host-controlled mode so the data will not change while the copy is in progress.

# Retired IO Devices

These IO devices are either no longer available or are not supported by the current version of FortnaPlus.  The configuration information is preserved here for posterity.

### AC5 PCI (Opto22)

AC5 IO is not supported in QNX6.

The Opto22 AC5 card is available as a PCI card.  Choose IOCard_Type AC5.  The card requires an IOCard_BasePort address. Look it up manually using the “show_pci -v” command.  Look for Vendor ID = 148ah, Device ID = ac05h and find the Base Address line:

Base Address MEM@f5400000h Prefetchable,32bit length 4194304  MEM@f53ff000 Prefetchable, length 4096,

In the example above, the IOCard_BasePort would be f5400000.

The two 24-bit IO boards are connected to the AC5 card through a ribbon cable.

Each card can access up to 6 8-bit banks (or 48 points) of IO, by connecting to as many as 2 racks, each with up to 24 points.

Note:  The ISA bus version of AC5 has been discontinued.

Note:  The AC5 card does not have a watchdog timer.  The IO will remain in its last state if FortnaPlus is shut down, or if the connection to the IO card is lost.

Note:  If changes are made to the IOCard configuration, FortnaPlus must be restarted or the Re-init function executed before they will take effect.

### IOCard Table Configuration for AC5

The FPC IOCard table defines the connection between FPC and the AC5 driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the AC5 driver.

| IOCard Table – AC5 |  |
| --- | --- |
| Column | Value or Description |
| Name | Enter a unique name for this configuration. |
| Desc | Enter a description for this configuration (optional). |
| Type | Choose “AC5 PCI”. |
| BasePort | Use show_pci -v to find the address, as described above. |
| ResetPort | Not used by the AC5 driver. |
| pStatus | Not used by the AC5 driver. |
| InputMenu | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Set by the driver. 0 – Card is not active. 1 – Card initialization routine completed successfully 2 – Card has been configured with input and output bits 3 – Card has successfully read the IO (normal running state). |
| Process | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Internal Address assigned by the operating system for the shared memory area. |
| Error | Error code returned from the IO card or the driver process. See the table below. |
| InputPtr | Internal index that points to the InputMenu selected. |
| OutputPtr | Internal index that points to the OutputMenu selected. |
| MaxInput | Set by the driver. Maximum number of bytes read from the card based on the size of the input table. |
| MaxOutput | Set by the driver. Maximum number of bytes written to the card based on the size of the output table. |
| Machine | Choose the name of this machine. |

| AC5 Error Codes |  |
| --- | --- |
| 1001 | Board initialization error, or memory open error |
| 1002 | Memory link error |
| 1010 | A table has not been selected for the input or output IO image |
| 1011 | Table not defined |
| 1012 | Invalid column |
| 1013 | Not used |
| 1014 | Table error (Pointer not found) |
| 1015 | Table type not valid |
| 1016 | Incorrect data type for column |
| 1025 | Base port 0 |
| 2002 | Machine does not match |

### AC28 Pamux ISA (Opto22)

AC28 IO is not supported in QNX6 and is no longer available, but AC51 can be used in its place.  See section 2.4.

The Opto22 AC28 Pamux card is available as a PCI card.  Choose IOCard_Type Pamux.  The card requires a IOCard_BasePort address and a IOCard_ResetPort address. Look the addresses up manually using the “show_pci -v” command in QNX4.  Look for Vendor ID = 148ah, Device ID = ac28h and find the Base Address line:

Base Address IO@d800h length 128  IO@de00 length 256, IO@dd00h length 256 bytes

In the example above, the IOCard_BasePort would be de00 and the IOCard_ResetPort would be d800.

The B4 brain board is connected to the AC28 card through a ribbon cable.  The B4 card is typically connected to a 32-point IO rack.  It has two LED indicators and 8 jumpers.  The left LED indicates if the card is selected (running).  The right LED indicates the state of the watchdog.  The first four jumpers are used to define the rack address.  In the table, below, a zero represents ‘jumper removed’, and a one represents ‘jumper installed’.  They form a four-bit binary value from 0 to 15:

| Jumpers 4 thru 1 | Address (Rack number) |
| --- | --- |
| 0000 | 0 |
| 0001 | 1 |
| 0010 | 2 |
| … | … |
| 1111 | 15 |

Jumpers 5 through 8 must all be installed for proper watchdog operation.

Each card can access up to 64 8-bit banks (or 512 points) of IO, by connecting to as many as 16 B4 racks, each with up to 32 points.

Note:  The ISA bus version of AC28 has been discontinued.

### Access 120 PCI

Access 120 IO is not supported in QNX6.

The Access 120 IO card is a PCI card.  The card is selected in IOCard_Type with AC120 PCI.  The card requires a IOCard_BasePort address.

The card has several jumpers that are used to configure the IO buffers and external interrupts:

| Jumper | Setting | Comment |  |  |
| --- | --- | --- | --- | --- |
| BEN/TST | Set to the BEN | The BEN position permanently enables the IO buffers. |  |  |
| IEN0 | Set to EN0 | Interrupts are not used by FortnaPlus and the jumper may be left in the default setting. |  |  |
| IEN1 | Set to EN1 | Interrupts are not used by FortnaPlus and the jumper may be left in the default setting. |  |  |
| IEN2 | Set to EN2 | Interrupts are not used by FortnaPlus and the jumper may be left in the default setting. |  |  |
| IEN3 | Set to EN3 | Interrupts are not used by FortnaPlus and the jumper may be left in the default setting. |  |  |
| IEN4 | Set to EN4 | Interrupts are not used by FortnaPlus and the jumper may be left in the default setting. |  |  |

After installing the PCIDIO-120 card, run the QNX command:

show_pci -v

Look for Vendor ID = 494fh, Device ID = c78h in the output, and the Base Address line:

Base Address = MEM@feaff800h, 32bit length 128 IO@d880 length 128 bytes IO@df80 length 32 bytes

The IOCard_BasePort address for the above example would be feaff800.

The five 24-bit IO boards are connected to the AC120 card through five ribbon cables.

Each PCI card can access up to 14 banks of IO or can connect to five racks of IO.  Each IO rack can have up to 24 IO points.  A rack of IO has three banks associated with it.

Example:  Card 0:

Rack 0 – Banks 0 – 2
Rack 1 – Banks 3 – 5
Rack 2 – Banks 6 – 8
Rack 3 – Banks 9 – 11
Rack 4 – Banks 12 – 14

Each rack has three banks and they must be configured according to the following constraints:

Bank 0 – All 8 inputs or all 8 outputs
Bank 1 – All 8 inputs or all 8 outputs
Bank 2 – Lower four bits, all inputs or all outputs
Bank 2 – Upper four bits, all inputs or all outputs

If an incorrect bit arrangement is defined in Configio_In_Out, the AC120 driver will set IOCard_Status to “Bad Parameters” when the card is initialized.

Note:  The AC120 card does not have a watchdog timer!  The IO will remain in its prior state if FortnaPlus is shut down.

Note:  If changes are made to the IOCard configuration, FortnaPlus must be restarted or the Re-Init function executed before they will take effect.

#### AC120 IOCard Table Configuration

The FPC IOCard table defines the connection between FPC and the AC120 driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the AC120 driver.

| IOCard Table – AC120 |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select AC120 PCI from the card type options. |
| BasePort | Hex | The base port from the show_pci command. See section 3.3. |
| ResetPort | Hex | The reset port. Not used. |
| pStatus | String | Not used by the AC120 driver. |
| InputMenu | PopUp | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | PopUp | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Hex | Set by the driver. Valid values are 0 – 2. If the card initializes and communicates with the rack, the status is updated. 0 – Card is not active. 1 – Card init routine completed successfully. 2 – Card has been configured with the input and output bits. 3 – Card has successfully read the IO (normal running state). |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Hex | Internal Address assigned by the operating system for the shared memory area. |
| Error | Hex | Error code returned from the IO card or the driver process. See the error table, below. |
| InputPtr | Int | Internal index that points to the InputMenu selected. |
| OutputPtr | Int | Internal index that points to the OutputMenu selected. |
| MaxInput | Int | Set by the driver. Maximum number of bytes read from the card based on the size of the input table and the data read from the card. |
| MaxOutput | Int | Set by the driver. Maximum number of bytes written to the card based on the size of the output table and the data written to the card. |

The following table shows Error codes that may appear in IOCard_Error for the AC120:

| AC120 Error Codes |  |
| --- | --- |
| 1001 | Board init error, memory open error |
| 1002 | Memory link error |
| 1010 | A table has not been selected for the input or output IO image |
| 1011 | Table not defined |
| 1012 | Invalid column |
| 1013 | Not used |
| 1014 | Table error (Pointer not found) |
| 1015 | Table Type not valid |
| 1016 | Incorrect data type for column |
| 1025 | Base port 0 |
| 2002 | Machine does not match |

### 1784-KTX (Allen-Bradley)

1784 KTX and 1784 KTXD are supported by FPC in QNX6, but the cards have reached end-of-life and are no longer available.

The Allen-Bradley 1784 KTX (single) and 1784 KTXD (dual) cards may be configured in either DataHighway or Remote IO communications mode.

In DataHighway mode, the card is used to send and receive messages using the Allen-Bradley DataHighway communications protocol.  The card would typically be connected to an Allen-Bradley PLC processor when used in this way.

In Remote IO (RIO) mode, the card is used to communicate with A-B IO racks that have a RIO adapter.  The RIO communications mode allows FPC to control the IO.

The following files, provided by A-B, must be present in the RUN directory and are used by the driver:
	clrram.bin
	loadpcl.bin
	Scanner.bin

Each KTX card has two shared memory areas:  an input memory area, and an output memory area.  The driver reads the inputs from the table defined in IOCard_InputMenu and writes the outputs to the table defined in IOCard_OutputMenu for each update cycle.  The update cycle time varies and depends on the number of IO racks used in the application.

FPC will automatically detect the PCI BasePort address for the KTX cards.  You can look it up manually using the “pci -vv” command (show_pci -v in QNX4), however, if you wish.  The 1784 KTX (single) card is identified by Vendor ID=12a0h and Device ID=8h.  The 1784 KTXD (dual) version of the card is identified by Vendor ID=12a0h and Device ID=ah and has the equivalent of two single cards built into a single physical card with two KTX connectors.  The addresses are taken from the “Base Address” line in the pci command output.

In this example, for a single (8h) card, BasePort=feafe000.  The address we need is associated with the length 4096 entry:

Base Address   = IO@d880h length 128  MEM@feafe000h, 32bit length 4096 IO@dff4h length 4 bytes

In this example, for a dual (ah) card, BasePort=feaff000 for the first ‘card’ and BasePort=feafe000 for the second ‘card’.  The addresses we need are associated with the length 4096 entries:

Base Address   = IO@dc00h length 128  MEM@feaff000h, 32bit length 4096 MEM@feafe000h, 32bit length 4096 IO@dff4h length 4 bytes

The first ‘card’ is associated with PCIDeviceIndex 0 and SlotIndex 1, whereas the second ‘card’ is associated with PCIDeviceIndex 0 and SlotIndex 2.  These values are important for associating the correct physical device with the configuration within FPC.

Note:  In QNX4 versions, PCIDeviceIndex and SlotIndex, which are used to automatically find the card’s addresses, are not present and not needed because you must manually determine the appropriate addresses.

#### KTX Configuration for RIO

The A-B KTX card requires only a single address for operation.  The address is used by the driver to link to the shared RAM memory (BasePort) of the card.  When the driver is initialized in RIO mode, it reads the number of IO racks from the card and if the quantity is greater than the number required, will update the number of racks detected in the IOCard_RacksReq field.

Note:  Verify the correct number of racks have been read from the card.  If the number of racks detected is not greater than or equal to the number in the IOCard_RacksReq field, the card will not start.

Note:  If the driver detects an error, the error number is displayed in the IOCard_Error field.  Error messages are also written to the FPC log files and may be displayed by issuing this command:
	grep KTX *.log

#### KTX Configuration for DataHighway

If the KTX card is configured for DataHighway mode, the card will send and receive DataHighway messages.  The system sends:

0x0F – cmd, 0xA2 – FNC.  For a protected PLC read.
0x0F – cmd, 0xAA – FNC.  For a protected PLC write.

The messages are built using four-element addresses:

File – Defined in the message field (Class)
Type – Defined in the message field (Instance)
Element – Defined in the message field (Attribute)
Sub-Element – Defined in the message field (Service)

The message destination (A-B node number):

Dest Node – Defined in the message field (MacID)
KTX Node – 0x3D (75 Octal)

#### KTX Messaging for RIO Block Transfer

If the card is configured for Remote IO, it can send and receive block transfer messages.  Block transfer messaging is provided by the system to read and write device (or control card) information as well as provide the ability to configure devices (or cards).  Each device has its own unique set of messages.

The messages are built using four-element addresses:

File – Defined in the message field (Class)
Type – Defined in the message field (Instance)
Element – Defined in the message field (Attribute)
Sub-Element – Defined in the message field (Service)

The message destination (A-B adapter / rack number):

Dest Node – Defined in the message field (MacID)

#### KTX RIO Adapters and Racks

KTX RIO cards can be configured in two different ways during initialization by setting the IOCard_ConfigMode.  See section 3.4.7 for details.

If the KTX RIO card is initialized in the Autoconfiguration mode, FPC will determine the number of attached adapters and will build a scan list based on the racks found.  The scan list is then uploaded from the KTX card and is stored in FPC table KTX_RIO_ADAPTERS.  The table will contain a list of the adapters found and the logical mapping of the racks.  IOCard_RackNum will contain the logical number assigned to the rack by the card.  The table, below, indicates the Bank or address used in FPC table Configio to access the cards in a specific logical rack.

If the KTX RIO card is initialized in the Static Configure mode, FPC will read the list of adapters from the KTX_RIO_ADAPTERS table and then download it to the KTX card.  The Static Configure mode must be used if the IO adapters are not powered on at the time the KTX card is initialized.

The easiest way to configure the KTX_RIO_ADAPTERS table is to power up all adapters and racks and initialize the IO with Autoconfiguration mode set.  Once the system is up, change to the Static Configure mode to ‘lock in’ the configuration.

Note:  A maximum of 64 KTX adapters can be configured.

Note:  In large systems, the IO adapter may need to be placed in the scan list multiple times to increase the IO update rate for the adapter.

Adapter Number = (Rack * 4) + (Module Group/2)

| AB Adapters/Racks |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adapter | Rack | Module Group | Bank/Inputs | Bank/Outputs |
| 0 | 0 | 0 | 0 (decimal) | 0 (decimal) |
| 1 | 0 | 2 | 4 (decimal) | 4 (decimal) |
| 2 | 0 | 4 | 8 (decimal) | 8 (decimal) |
| 3 | 0 | 6 | 12 (decimal) | 12 (decimal) |
| 4 | 1 | 0 | 16 (decimal) | 16 (decimal) |
| 5 | 1 | 2 | 20 (decimal) | 20 (decimal) |
| 6 | 1 | 4 | 24 (decimal) | 24 (decimal) |
| 7 | 1 | 6 | 28 (decimal) | 28 (decimal) |
| 8 | 2 | 0 | 32 (decimal) | 32 (decimal) |
| … | … | … | … | … |
| 12 | 3 | 0 | 48 (decimal) | 48 (decimal) |
| 16 | 4 | 0 | 64 (decimal) | 64 (decimal) |

Note:  One physical rack may be more than one logical adapter or logical rack.  It depends on the type of addressing used (1/2-slot, 1-slot, 2-slot, or complementary, etc.).

A-B Scanner Switches:

| Switch | Meaning |
| --- | --- |
| 1 | 1 – 8 all ON |
| 2 | 1 – OFF. (230.4K) 2 – ON. 3 – OFF. Non-comp IO 4 – OFF. All slots 5 – OFF. Link response unrestricted 6 – OFF. |

Note:  The switch setting may need to be changed as required for the project.  See the A-B manuals for correct settings.  Incorrect baud rate settings may not show as errors on the KTX card.

Note:  KTX to RIO requires the proper termination resistors on each end.  See the installation manuals for details.

#### KTX RIO Adapter Diagnostics

This section describes techniques and tools to evaluate and correct problems with KTX configurations.

If IOCard_ConfigMode is set to update the diagnostics, the KTX_RAM table will display the status of each individual adapter, starting at 0x80 (hex).  It contains a list of possible adapters for each rack.  Each adapter may have a partial rack, in quarters, up to a full rack.  Each rack may have up to four adapters, one for each ¼ rack segment.  Every 12 bytes of data (3 for each adapter and 4 possible adapters per rack) defines the adapters.  For example, if rack 0 has only one adapter, and it starts at module group 0, the table addresses 0x80, 0x81, and 0x82 will contain info for the rack. The remaining 9 addresses 0x83 – 0x9B will not contain any adapter data, and rack 1 will be defined in address 0x9C, 0x9D and 0x9E.

Note:  The KTX_RAM table must have at least 130 rows defined.

Note:  If changes are made to the IOCard configuration, FortnaPlus must be shut down and restarted or the Re-Init function executed.  Re-Init will not reset memory addresses, so if they are changed, FortnaPlus must be shut down.

Note:  The terminating resistors supplied with the KTX card must be installed or start-up initialization and communication errors may occur.  If the RIO Module has a blinking green light, the termination resistor is required.

Note:  If the green light on the A-B RIO module is not on or flashing, try switching the blue and clear wires on the terminal blocks.

A-B Rack DIP Switches:

| Switch | Meaning |
| --- | --- |
| 1 | OFF – Outputs are de-energized on fault |
| 2 | ON |
| 3 | OFF – Non-comp IO |
| 4 | OFF |
| 5 and 6 | Rack Addressing 5 OFF, 6 ON: ½-slot addressing 5 ON, 6 OFF: 1-slot addressing 5 OFF, 6 OFF: 2-slot addressing 5 ON, 6 ON: Not allowed |
| 7 | OFF |
| 8 | OFF |

#### KTX RIO Configuration Diagram

FPC is configured for use with KTX by entries in several tables that are related as shown in the following diagram.

IOCard table

Defines the card and the IO interface tables

Input Data table

Real-time input data

Output Data table

Real-time output data

Configio table

Links input and output data to FortnaDT

Conveyor table

Defines the individual bits of the FortnaDT IO words.

KTX card

Installed in PC

FortnaDT table

All Real-time IO for the system

#### KTX RIO IOCard Table Configuration

The FPC IOCard table defines the connection between FPC and the KTX driver.  You will find table IOCard under Configuration Menu -> IO Utilities -> IO Card Config.  Any IOCard table columns not shown below are not relevant for the KTX driver.

| IOCard Table – KTX |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select KTX RIO, KTX DH+, KTX RIO Dual or KTX DH+ Dual from the card type options. |
| BasePort | Hex | The base port. Set automatically. |
| ResetPort | Hex | The reset port. Not used. |
| pStatus | String | Updated by the driver with the card status, taken from table Errors. |
| InputMenu | PopUp | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | PopUp | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Hex | Set by the driver. Valid values are 0 – 2. If the card initializes and communicates with the rack, the status is updated. 0 – Card is not active. 1 – Card init routine completed successfully. 2 – Card is communicating with the IO rack(s). |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Hex | Internal Address assigned by the operating system for the shared memory area. |
| Error | Hex | Error code returned from the IO card or the driver process. |
| InputPtr | Int | Internal index that points to the InputMenu selected. |
| OutputPtr | Int | Internal index that points to the OutputMenu selected. |
| MaxInput | Int | Set by the driver. Maximum number of bytes read from the card based on the size of the input table. |
| MaxOutput | Int | Set by the driver. Maximum number of bytes written to the card based on the size of the output table. |
| CurrMsg | Int | Set by the driver. The message number of the message currently being processed. |
| IntrpPort | Hex | Interrupt port. Not used by the KTX driver. |
| CardState | Int | Set by the driver. The current state of the driver state machine. |
| IntrrAddr | Hex | Not used by the KTX driver. |
| ConfigMode | Int | Configuration mode. Use this field to set the card’s operating mode. 0 – Autoconfigure is set. The card will always autoconfigure at init. 1 – Autoconfigure, then copy the dual-port RAM image to the KTX_RAM table. 2 – Set the scan list to the list found in table KTX_RIO_ADAPTERS. 3 – Set the scan list to the list found in table KTX_RIO_ADAPTERS, then copy the dual-port RAM image to the KTX_RAM table. When ConfigMode is set to update the KTX_RAM table, that table will contain the status of each individual adapter starting at 0x80. See section 3.4.5 for details. |
| TimeLastCmd | Int | Set by the driver. The time of the last command sent to the card. |
| SerialNumber | Hex | Not used by the KTX driver. |
| NumMsgs | Int | Set by the driver. The number of messages in the queue for the card. |
| Baud | Hex | The communication baud rate. 0xFE – (254 decimal) 230.4K 0xFD – (253 decimal) 115.2K 0xFC – (252 decimal) 57.6K Defaults to 0xFE. |
| LoopCnt | Int | Set by the driver. Indication of activity. Rolls to 0 after 9999. |
| InputCnt | Int | Set by the driver. Shows the inputs received. |
| Machine | PopUp | Choose this machine. |
| ReqRacks | Int | Set by the driver to the number of racks found during auto-configuration. The driver verifies the value before putting the card on line. Reset this to 0 if you change the number of racks during system testing. |
| CardNode | Int | The node address assigned to the KTX card. This is used for DataHighway applications and will be set to the default value of 77 (0x93F) if not set. |
| TNS | Int | Set by the driver. The transaction number used for a message protocol. |
| InitStatus | Int | Initial error code returned from the KTX card. |
| LinkState | Int | Link state returned from the KTX card. |
| StatusByte | Int | Status byte returned from the KTX card. |
| ErrorByte | Int | Error byte returned from the KTX card. |
| LEDStatus | Int | The state of the card LEDs returned from the KTX card. |
| WDCnt | Int | Watchdog counter. |
| BlockCnt | Int | Block count used during file downloads. |
| ReturnState | Int | Last return state. Used for card communications. |
| RtnStatus | Int | Last return status. Used for card communications. |
| PCIDeviceIndex | Int | If more than one of a given card type is present, use this to select a particular card. Zero represents the first card. |
| NodeIndex | Int | Set by the driver. Tracks the last adapter or node updated in diagnostics. |
| SlotIndex | Int | For KTX single cards, set this to 0. For KTXD dual cards, set the first one to 0 and the second to 1. |
| LogLevel | PopUp | Controls log output. A higher number means more logging. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Int | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Int | Internal storage used by the RunDelay timer. |

#### KTX RIO IO Adapter Status Table [KTX_RIO_ADAPTER]

The IO Adapter Status table, a.k.a., KTX_RIO_ADAPTERS, is automatically populated with one entry per KTX adapter.

| IO Adapter Status Table (KTX) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | A unique name for the adapter. |
| Adapter | Int | Adapter index. Automatically filled in. |
| Active | Y/N | Indicates if the adapter is being monitored. |
| pStatus | String | Text description of the status for each adapter. Updated by the software. |
| Config Byte | Hex | Adapter configuration byte: Bit 0 – In scan. The adapter is in the current scan list. Bit 1 – Exists. The adapter has replied to scanner commands. Bit 2 – Known. Bits 3, 4 – Size: 00 – ¼ rack, 01 – ½ rack, 10 – ¾ rack, 11 – full rack. Bit 5 – Node adapter. When set, this is a node adapter device. Bits 6, 7 – Undefined. |
| Fault Byte | Hex | Adapter fault byte: Bits 0, 1, 2, 3 – Fault group number. Bit 4 – In fault group. Bit 5 – Adapter is on line. 1 = adapter on line. 0 = adapter not responding. Bit 6 – Adapter group faulted. Bit 7 – Undefined. |
| Retry Count | Hex | Indicates the number of attempts to communicate with the adapter. |
| IOCard | PopUp | The KTX card connected to this device. Select a card from the IOCard list. |
| Rack | Int | The rack associated with this adapter. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| InpStatusIndx | Int | Not used by KTX. |
| StatusMask | Int | Not used by KTX. |
| pCurrStatus | Int | Not used by KTX. |

#### KTX Block Transfer / DataHighway Messages [MessageList]

In addition to the IO data, the MessageList table may be used to define block transfer or DataHighway messages.  When the driver is started, it scans the MessageList table.  If errors are detected, the Status field will be set to zero and the error will be shown in the SndError field.  The KTX initialization routine will verify the messages and initialize the pointers even if the Active field is set to N.  Messages may be activated while in RUN mode.

NOTE:  Set the MessageList Active field to N while making changes to prevent incompletely defined messages from being sent.  The card must be restarted using the “IO Card Init” function, or by restarting FPC.

Note:  When configuring a KTX message list, a table may be required for the receive message or the send message.  If a send message table is defined, it will contain the message to transmit to the device.  If a receive message table is defined, it will contain the message received.  The A-B KTX driver allows these tables to be defined with string, integer, or unsigned byte data.

Message data is read from or written to FPC tables as defined in the MessageList entry.  The columns containing the data may be defined as bytes, strings, or integers.  The number of bytes read or written is defined by the SndBytes and RcvBytes columns.  Received data is stored as two bytes per integer.

Example block transfer read:
Read 3 words from an analog output module in Rack 0, Slot 7, using single-slot addressing
	Active:	Y
	IOCard:	KTX Card
	MacID:	0 (Rack 0)
	Class:		12 (The logical slot = 2 * (Physical slot – 1))
	RcvBytes:	6 (3 words)
	RcvMenu:	BlkInputData
	RcvColumn:	1
	RcvRow:	0
	HowOften:	1
	Increment:	Invalid
	Type:		OneShot (Type 4)
	CurrentStatus:	0 (Reset to 0 to send a new block transfer)

| IO Message List (KTX) |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the message. |
| Active | Y/N | Flag used to enable or disable this message. |
| Status | Int | Status value returned from the driver indicating if this message is being processed. Zero indicates the message is not being processed. |
| pStatus | String | An error message set by the driver if the message is not being processed. |
| SndError | Hex | Send error code returned from the driver, card, or device. |
| SndFault | Hex | Fault code returned from the driver, card, or device. |
| RcvError | Hex | Receive error code returned from the card or device. |
| IOCard | PopUp | Select a KTX card from the IOCard table. |
| IPAddr | String | Not used by KTX. |
| MacID | Int | RIO: Enter the rack number. DataHighway: Enter the node number. |
| Service | Int | RIO: Not used. DataHighway: Enter the sub-element number. |
| Class | Int | RIO: Enter the card number (1 – 16). DataHighway: Enter the file number (1 – 999). |
| Instance | Int | RIO: Not used. DataHighway: Enter the file type: 133 (0x85) – bit; 134 (0x86) – timer; 135 (0x87) – counter; 136 (0x88) – control; 137 (0x89) -- integer. |
| Attribute | Int | RIO: Not used. DataHighway: Enter the element number. |
| AssemblyInst | Int | Not used by KTX. |
| SndBytes | Int | Enter the number of bytes to send. Set 0 if this is a read command. |
| SndMenu | PopUp | Select a FPC table to load send data from. If SndBytes is 0, the selection is not used. |
| SndSelc | Int | Not used by KTX. |
| SndColumn | Int | Enter a FPC table column. If SndBytes is 0, the selection is not used. |
| SndRow | Int | Enter a FPC table row. If SndBytes is 0, the selection is not used. |
| RcvBytes | Int | Enter the Number of bytes to receive. The actual number of bytes received may not match the number entered. The driver will truncate or null-fill based on the actual number of bytes returned by the device. |
| RcvActual | Int | Actual number of bytes received. Set by the driver. |
| RcvMenu | PopUp | Select a FPC table where received data is to be saved. If RcvBytes is 0, the selection is not used. |
| RcvSelc | Int | Not used by KTX. |
| RcvColumn | Int | Enter a FPC table column. If RcvBytes is 0, the selection is not used. |
| RcvRow | Int | Enter a FPC table row. If RcvBytes is 0, the selection is not used. |
| HowOften | Int | Enter the number of units of Increment to determine how often messages are sent. No messages will be sent if this is set to 0. |
| Increment | PopUp | Enter the units for the message send interval (hour, min, sec, etc.) |
| Type | PopUp | Enter the type of increment (absolute, elapsed, count, one shot) |
| Waiting | Int | Used by the driver to store the time of the last message sent. Clears when a message is received. |
| Pending | Int | Indicates if a message needs to be sent. |
| CurrentStat | Int | Used by the driver to manage messages. |
| WasStat | Int | Used by the driver to manage messages. |
| SndPtr | Int | Used by the driver to store the Menu/Column Pointer |
| RcvPtr | Int | Used by the driver to store the Menu/Column Pointer |
| SndMsgCtr | Int | Used by the driver to track messages sent |
| RcvMsgCtr | Int | Used by the driver to track messages received |
| SndMnuDataType | Int | Used by the driver to track the type of data in SndColumn. |
| RcvMnuDataType | Int | Used by the driver to track the type of data in RcvColumn. |
| TNS | Int | Used by the driver to track the message. |

#### KTX RIO FPC Errors

The following error codes may appear in the IOCard_Errors field.

These errors are generated by the FPC KTX driver:

| DataHighway and RIO Errors Detected from FortnaPlus |  |
| --- | --- |
| Hex Value | Description |
| 1001 | File clrram.bin was not found in the RUN directory (from Allen-Bradley). Writes file 0x1001.txt. “Err: KTX Clear” |
| 1002 | File loadpcl.bin was not found in the RUN directory (from Allen-Bradley). Writes file 0x1002.txt. “Err: Missing loadpcl.bin” |
| 1003 | File scanner.bin was not found in the RUN directory (from Allen-Bradley). Writes file 0x1003.txt. “Err: Missing Scanner.bin” |
| 1004 | Timeout waiting for file to load. To recover, reboot the computer, check the PCI addresses. Writes file 0x1004.txt. “Err: KTX Handshake no response” |
| 1005 | Setting card to program mode, card busy error. Writes 0x1005.txt. “Err: Program mode not set” |
| 1006 | Setting card to program mode, card did not send confirmation. Writes 0x1006.txt. “Err: Run mode not set” |
| 1007 | Setting card to autoconfigure mode, card busy error. Writes 0x1007.txt. “Err: Autoconfigure mode not set” |
| 1008 | Confirming card to autoconfigure mode, card returned an error. Writes 0x1008.txt. “Err: Autoconfigure mode not set” |
| 1009 | Get scan list error. The KTX_RIO_ADAPTER table may be too small or the card detected an error (check the SORT message log). Writes 0x1009.txt. “Err: Scan List upload failed” |
| 100A | Setting the card to RUN mode, card busy error. Writes 0x100A.txt. “Err: RUN mode not set” |
| 100B | Setting the card to RUN mode, card did not send a confirmation. Writes 0x100B.txt. “Err: RUN mode not set” |
| 100C | Setting the card to enable time out error. Writes 0x100C.txt. “Err: KTX did not Enable” |
| 100D | Wait for the Ack response message to clear error. Writes 0x100D.txt. “Err: KTX did not respond” |
| 100E | Waiting for the KTX to clear the prior host command time out. Writes 0x100E.txt. “Err: KTX did not respond” |
| 100F | Set scan list host busy error. Writes 0x100F.txt. “Err: Scan did not download” |
| 1010 – 1016 | A table has not been selected for the input or output IO image. “Err: Input Menu not defined” or “Err: Output Menu not defined” |
| 1011 | Menu not defined |
| 1012 | Invalid column |
| 1013 | Not used |
| 1014 | Table error – column does not exist (Pointer not found) |
| 1015 | Table type not valid (type is 4 – DB) |
| 1016 | Incorrect data type for column (not UNSIGNED_CHAR) |
| 1020 | Set scan list, card did not send a confirmation. Writes 0x1020.txt. “Err: Scan did not download” |
| 1021 | Base address has not been defined (get address from show_pci) |
| 1022 | Error setting the card to enable mode |

The following errors may be returned from the KTX card:

| KTX Errors Returned from the Card |  |
| --- | --- |
| Hex Value | Description |
| 2XXX | Status errors returned from the card (board error). Refer to the KTX Remote IO and KTX Data Highway manuals for further information on error codes. |
| 2001 | In Progress |
| 2002 | Invalid link address. “Err: Unable to attach pci” “Err: mmap_device_memory MAP_FAILED” “Err: Can’t PCI_INIT_ALL” “Err: Can’t pci_attach_device” “Err: mmap_device_memory MAP_FAILED” |
| 2003 | Binary Protocol Mismatch |
| 2004 | Invalid Baud Rate |
| 2005 | Unauthorized Protocol |
| 2006 | Failed Program CRC |
| 200F | Scanner Not Program |
| 2010 | Scan List Too Long |
| 2011 | Set Scan List in Progress |
| 2012 | Autoconfiguring in Progress |
| 2013 | Scan List causes FDG Orphan |
| 2014 | SC Unknown Command |
| 2015 | SC Bad Request |
| 2016 | SC Bad Param |
| 2017 | SC cannot change mode during set scan list |
| 2018 | SC cannot change mode during autoconfig |
| 2019 | Scanner already autoconfiguring |
| 201A | Bad Command Data Length |
| 201B | BT Bad Address |
| 201C | BT Bad address not in scan list |
| 201D | BT Bad Data Length |
| 201E | Too many requests for the module |
| 201F | BT Queue Full |
| 2020 | Address not in scan list |
| 2021 | Cannot set FG while autoconfiguring |
| 2022 | Cannot set FG while setup scan list |
| 2023 | Bad CRC |
| 2024 | BT Request Timeout |
| 2025 | Unsolicited BT Request |
| 2026 | User module request type mismatch |
| 2027 | Used module length mismatch |
| 2028 | Host supplied too little data |
| 2029 | BT Checksum error |
| 202A | Adapter Address Error |
| 202B | Scanner Address Error |
| 202C | Illegal reply cmd |
| 202D | Buffer overflow error |
| 202E | Timeout error |
| 202F | BT Blow off |
| 2030 | IO Mask Data Mismatch |
| 2031 | Invalid adapter address |
| 2032 | Module in Fault Group Requested BT |
| 2033 | Adapter size overlap |
| 2034 | Bad BTW Reply |
| 2035 | Error more than 32 unique devices |
| 2036 | Bad RAM |
| 2081 | Adapter config success |
| 2082 | Too many adapters online |
| 2096 | Initialization timed out |
| 2097 | Confirmation timed out |
| 2098 | Trans Num Mismatch |
| 2099 | Adapter Non-existent |
| 209A | Unknown IO Table |

The following errors are associated with KTX DataHighway and the MessageList table:

| KTX DataHighway Error Messages (MessageList Errors) |  |
| --- | --- |
| Hex Value | Description |
| 2XXX | Status and extended status message errors returned from the status byte of DataHighway messages. SndError will typically contain the following error code. If the message has an extended status code the code is loaded into the RcvError column. |
| 2001 | Not used |
| 2002 | Cannot guarantee delivery |
| 2003 | Duplicate token holder detected |
| 2004 | Local port is disconnected |
| 2005 | Application layer timed out waiting for a response |
| 2006 | Duplicate node detected |
| 2007 | Station is offline |
| 2008 | Hardware fault |
| 2010 | Illegal command or format |
| 2020 | Host has a problem and will not communicate |
| 2030 | Remote node host is missing, disconnected, or shut down |
| 2040 | Host could not complete function due to hardware fault |
| 2050 | Addressing problem or memory protect rungs |
| 2060 | Function disallowed due to command protection selection |
| 2070 | Processor is in program mode |
| 2080 | Compatibility mode file missing or communication zone problem |
| 2090 | Remote node cannot buffer command |
| 20A0 | Not used |
| 20B0 | Remote Node problem due to download |
| 20C0 | Cannot execute command due to active IPBs |
| 20D0 | Not used |
| 20E0 | Not used |
| 20F0 | Extended Status has additional error information |

| KTX Message Errors Detected from FortnaPlus |  |
| --- | --- |
| Hex Value | Description |
| 1010 | Table not valid |
| 1011 | Table not defined |
| 1012 | Invalid column |
| 1013 | Send parameter command failed |
| 1014 | Table error (pointer not found) |
| 1015 | Table type not valid |
| 1016 | Incorrect data type for column |
| 1121 | Status is not == 1 |
| 1122 | Active is not set to 1 or Y |
| 1123 | The time increment is 0, (must be 1 or larger) |
| 1127 | IO message time out |
| 1128 | Invalid time increment |
| 1100 | Card status is not 1, (message error) |
| 1101 | Card has too many messages in process, message must wait |

#### KTX RIO Configio Table

The KTX inputs and outputs are mapped into FPC by adding entries to the Configio table.  Each entry can map up to 8 bits.

| Configio Table – KTX |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Desc | String | Enter a unique description for the record. |
| Bank | Int | Address of the input or output byte to read or write (Address is defined by the logical adapter or rack and the card or slot in the rack). For example, if a 16-bit input module is located in logical rack 1, slot 1, the first input card would be mapped to bytes 16 and 17 of the input card’s shared memory. Set the Bank to 16 decimal (20 octal) to map the first 8 inputs to the FortnaDT IO table. |
| Octal_Word | Int or Oct | FortnaDT table index where the input byte will be read from or the output byte written to. |
| LoHi | PopUp | Select either the Low or High byte (FortnaDT table) associated with the device. |
| In_Out | Binary | Each bit can be set to a 0 (0 indicates input), or a 1 (1 indicates output). If the bit is set to 0 the data will be read from the input table. If the bit is set to 1 the data will be written to the output table. |
| I_O_Type | PopUp | Select digital for the IO type. |
| Interface | PopUp | Select the record in table IOCard for the Interface. |
| Granularity | Int | The monitoring granularity. A value of 0 or 1 causes monitoring to occur on every pass through the critical loop. |
| Status | PopUp | Displays the IO status. |
| Process | PopUp | Select SORT – the process that will monitor the card for messages and transfer input and output data. |

#### KTX RIO Input Dual-Port RAM Table

When configuring a KTX card, a table must be selected for storing the dual-port RAM input image.  The table must be defined with a single column of unsigned byte data.  The driver will copy the dual-port RAM image from the card.  The driver copies the data in block mode.

#### KTX RIO Output Dual-Port RAM Table

When configuring a KTX card, a table must be selected for storing the dual-port RAM output image.  The table must be defined with a single column of unsigned byte data.  The driver will copy the dual-port RAM image from the card.  The driver copies the data in block mode.

### DeviceNet (Interbus S and SST)

Interbus-S and Interbus-SST Synergetics Hilsher DeviceNet are not available in FPC for QNX6.

This section describes the Interbus-S and Interbus-SST Synergetics Hilsher DeviceNet IO cards and their use in QNX4.  See section 2.5 for additional details on DeviceNet configuration and operation.

After installing the card, use the “show_pci -v” command and look for Vendor ID=10b5h, Device ID=9050h.  Both the Synergetics Hilsher DeviceNet and Synergetics Hilsher Interbus cards use the same VID and DID, for some reason, but you can tell them apart by the Base Address values.

The S-H DeviceNet card has a length of 8192, which is the shared memory length the driver is expecting for card type DeviceNet Hilsher.  In this example, BasePort=feafc000:
Base Address   = MEM@feaff800h,32bit length 128 IO@d880h length 128 bytes MEM@feafc000h,32bit length 8192

The S-H Interbus card has a length of 2048, which is the shared memory length the driver is expecting for card type Interbus.  In this example, BasePort=feaff000:
Base Address   = MEM@feaff800h,32bit length 128 IO@d880h length 128 bytes MEM@feaff000h,32bit length 2048

The Allen-Bradley DeviceNet card  (VID=12a0h, DID=6h) has a BasePort length of 32768, and also uses IntrpPort with a length of 4096.  The driver is written for a BasePort length of 8192.  The card type is AB Device Net.  In this example, BasePort=feaf0000 and IntrpPort=feafe000:
Base Address   = MEM@feaffc00h,32bit length 128 IO@dc00h length 128 bytes MEM@feaf0000h,32bit length 32768 MEM@feafe000h,32bit length 4096 IO@dff4h length 4 bytes

#### DeviceNet Configuration Software

The DeviceNet IO system uses a complex communication protocol and intelligent devices.  The protocol allows each device (if capable) to communicate through several different methods:

- As standard IO polled devices

- Using messages generated by change-of-state (high performance)

- Using bit stream messages

- Using explicit messages

The SYCON software (for the PC card) and Device software (such as Allen-Bradley) provide configuration tools that can configure each device (creating the EDS files) and configure the PC card.  Use the Device software (for example Allen-Bradley DeviceNet Configuration software) to configure the device.  Export the EDS file from the software and import the EDS file into the SYCON system for the Synergetics Hilscher Card.  As each device is defined on the SYCON system, download the software to the PC card.

Use the Allen-Bradley software to:

- Configure each DeviceNet node

- Configure the MAC ID

- Configure expansion modules

- Configure inputs and outputs

- Export the EDS file

#### Interbus Configuration with the SYCON Utility

The Interbus network and master (PCI card) is configured with the SYCON utility provided with the card.  The utility runs under Windows and can be connected through a serial port using a DB9-to-DB9 crossover cable.  The SYCON software provides a graphical representation of the configuration in addition to diagnostic monitoring tools.

Use SYCON to:

- Configure and debug the Interbus system. The Interbus system starts with only one node connected.  Each device is individually connected and configured by setting the MAC address.

- The devices are configured as required (need the EDS file for each device.  Interbus EDS Files may be created using the SYCON utility.)

- Configure the master.  Select handshaking mode 3, buffered host controlled.

- The SYCON software provides the ability to scan the network and determine the number of bytes read and written to each device.  Configuration of the master includes defining how the input and output words are mapped to the dual-port memory addresses.

- Download the configuration to the master.

- Start communications.

- Use diagnostics to debug the configuration.

FortnaPlus provides utilities and software for defining how the input and output data is transferred from the dual-port memory of the PC card for use in controlling the overall system.

The Interbus IO system uses a complex communication protocol and intelligent devices.  The protocol communicates with each device through a serial message protocol.  The protocol supports the following:

- Standard IO devices

- Devices with Peripherals Communication Protocol (PCP) messaging capability

PCP message for Interbus:

MAC ID:  2 – 63  Node number (depends on the device)
	Class:  0x0  Not used
	Instance:  0 – 65535  Object index, parameter to read / write
	Attribute:  0 – 255  Object subindex
	Service:  0x01  Get (read) the attribute;  0x02  Put (write) the attribute

#### DeviceNet Configuration with the SYCON Utility

The DeviceNet network and master (PCI card) is configured with the SYCON utility provided with the card.  The utility runs under Windows and can be connected through a serial port using a DB9-to-DB9 cross-over cable. The SYCON software provides a graphical representation of the configuration in addition to diagnostic monitoring tools.

Use SYCON to:

- Configure and debug the DeviceNet system. The DeviceNet system starts with only one node connected.  Each device is individually connected and configured by setting the MAC address.

- The devices are configured as required (need the EDS file for each device.  EDS Files are provided by the manufacturer or odva.org)

- Configure the master.  Select handshaking mode 3, buffered host controlled.

- The SYCON software provides the ability to scan the network and determine the number of bytes read and written to each device.  Configuration of the master includes defining how the input and output words are mapped to the dual-port memory addresses.

- Download the configuration to the master.

- Start communications.

- Use diagnostics to debug the configuration.

FortnaPlus provides utilities and software for defining how the input and output data is transferred from the dual-port memory of the PC card for use in controlling the overall system.

#### DeviceNet FortnaPlus Errors

The following error codes may appear in the IOCard_Errors field.

See section 2.5.7 for errors returned by the FortnaPlus DeviceNet driver.

These errors are generated by the Interbus DeviceNet card:

| Interbuss Errors Returned from the Card |  |
| --- | --- |
| Hex Value | Description |
| 4041 | Connection could not be opened (card error) |
| 4043 | Too many parallel services on one CR (card error) |
| 4081 | Error in application of remote partner |
| 4082 | Unknown function in requested message |
| 4083 | PCP communication not initialized for this device |
| 4087 | Local state does not allow it to send |
| 4097 | Invalid communications reference |
| 409B | Invalid Interbus PCP service |

These errors are generated by the Hilsher DeviceNet card:

| DeviceNetSH Errors Returned from the Card |  |
| --- | --- |
| Hex Value | Description |
| 0x02 | Resource unavailable (device error) |
| 0x08 | Service unavailable (device error) |
| 0x09 | Invalid attribute (device error) |
| 0x0B | Already in request mode (device error) |
| 0x0C | Object state conflict (device error) |
| 0x0E | Attribute not settable (device error) |
| 0x0F | Permission check failed (device error) |
| 0x10 | State conflict, device state prohibits execution (device error) |
| 0x11 | No response from device (device error) |
| 0x13 | Not enough data received (device error) |
| 0x14 | Attribute not supported (device error) |
| 0x15 | Too much data received (device error) |
| 0x16 | Object does not exist (device error) |
| 0x17 | Reply data too large, internal buffer too small (device error) |
| 0x30 | Device not configured (card error) |
| 0x32 | Format error in response from device (card error) |
| 0x36 | Previous Set or Get still active (card error) |
| 0x37 | Mac-ID out of range (card error) |
| 0x38 | Sequence error in fragmented response (card error) |
| 0xC8 | Device not configured (card error) |

### Optomux Serial (Opto22)

This device is not available in FPC for QNX6.

This section describes the Optomux Serial driver and its use in QNX4.

Optomux Serial protocol (RS422/RS485) communicates with Opto22 IO.  The communication with devices on an Optomux network is handled by a special program, “optomux”.  This program was originally written in 1994 and was updated in 2004 to allow multiple networks on the same node.

The optomux driver is started from a start script just prior to loading FortnaPlus.  The operation of the driver is configured using command-line parameters.  To see a list of the parameters, type “use optomux”:

$ use optomux

$Id: //depot/1/SortPlus/LOCALBIN/PROGRAMS/Optomux/opto.c#3 $

optomux - OPTOMUX DRIVER (a small part of the FORTNA SORTPLUS (c) package)

optomux [options] [&]

Config:       Description:                                 Default:

-b <baud>     Changes serial port to <baud>                no change

-d <device>   Device connected to OPTOMUX network          /dev/ser3

-n <name>     Register <name> with nameloc utility         /fortna/optomux

-p <prio>     Set priority to <prio>                       same as parent

-r <retry>    Set retry time when board offline to <retry> 10 seconds

-t <val>      Poll OPTOMUX network every <val> msecs       1000 msecs

-T <val>      Delay after read for <val> msecs             0 msecs

-w <val>      Watchdog between driver and controls         5 seconds

-W <type>     Board watchdog select                        type 1

Debug:        Description:                                 Default:

-c <console>  Console for serial debug messages            same as parent

-s <console>  Fork a status process w/output to <console>  no status

-h            Writes heartbeat, <CR>, to device            no heartbeat

-v            Verbose                                      not verbose

-V            Very verbose                                 not verbose

-k            Display tx, rx, and checksum errors          none

Board watchdog types:

0                            disabled

1             10 seconds     all outputs OFF

2              1 minute      all outputs OFF

3             10 minutes     all outputs OFF

4                            disabled

5             10 seconds     output 0 ON; all others OFF

6              1 minute      output 0 ON; all others OFF

7             10 minutes     output 0 ON; all others OFF

Description:

This program is used to provide message services for access to an OPTOMUX

network.  For a description of services, please see the header file,

optomux.h, in the FORTNA SORTPLUS (c) source code.

Here is an example command line to start the driver:

$ optomux –p26 –d /dev/ser2 –s /dev/con9 –t250 &

$

This would start the optomux driver at priority 26, communicating with a network connected to /dev/ser2.  It would poll the network at 250 millisecond intervals.  In addition, it would display status information on console 9

To check the status of the optomux driver, use “sin –P optomux”.  This will show two or three processes.  The optomux driver forks into two processes when starting.  One process talks to the serial network, and the other talks to the FortnaPlus program.  If the “-s” option is used, a third process is forked to display status information on the desired console.

The optomux driver registers a “global name” that FortnaPlus uses to find the correct optomux process.  By default, the global name is “/fortna/optomux”.  This can be configured by using the “-n” option.  For example, specifying “-n omux1” would register “/fortna/omux1” as a global name.  This name is used in the OmuxName field of the IOCard table.

#### Optomux Serial – IOCard Table Configuration

Each Optomux Serial connection must be defined in table IOCard.

| IOCard Table – Optomux Serial |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select Optomux from the card type options. |
| Status | Hex | Set by the program to 0 or 1 to indicate the status of the connection to the optomux driver. If Status is set to 0 manually, FPC will reconnect to the driver. |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| TimeLastCmd | Int | Set by the driver. The time of the last command sent to the card. Used to track the last reconnection time so that FPC won’t try to reconnect constantly. |
| OmuxName | String | Enter the global name of the driver to connect to. This is “/fortna/optomux” by default but may be set to match the name chosen with the “-n” parameter when the driver was started. |
| OmuxPid | Int | Set by the program. Stores the process ID of the optomux driver that is currently connected. If no driver is connected, the value will be -1. |

### HMS AnyBus Master EtherNet/IP Controller

The HMS AnyBus PCI card was our first EtherNet/IP device in FPC.  It was a dedicated Network Interface Card hardware device with embedded firmware to allow it to “speak” EtherNet/IP.  As it was a special-purpose device, and not a generic NIC, we could treat it much like the other hardware IO interface cards that were already supported at that time, such as Profibus or KTX.  The EtherNet/IP configuration for the attached racks was handled by the Electrical department.  They used a MS-Windows-based program to load the configuration into the HMS card’s non-volatile memory.  Visibility to the web interfaces on the attached adapters required an additional standard NIC to be added and placed on the IO network.  A HMS Slave card was also supported, though it was only used for one customer (Sterling Jewelers), and has since been retired.  The rest of this section describes the HMS Master controller card.

The HMS AnyBus EtherNet/IP Master controller card is installed in an available PCI slot.

FPC will automatically detect the PCI address locations when the IOCard entry is initialized, setting the IOCard_BasePort and IntrpPort values.  There is no need to clear these values when replacing a card.

If multiple HMS cards are installed, you must manually set the IOCard_PCIDeviceIndex value to distinguish between the cards.  If one card is installed, the default value of 0 is used.  The second card would be 1, and so on.

#### Controlling Automatic Configuration Updates with HMS AnyBus Cards

The HMS Master card is configured by the Electrical department using an external (Windows-based) tool that writes the rack configuration to the card’s non-volatile memory.  While the configuration cannot be altered from FPC or QNX, the configuration can be extracted from the card and stored in a file on the AC.  The HMS driver will do this and store the files in RUN/FORTNA, with a name ending with -scanner.cfg.  These files are then backed up along with the rest of the FPC configuration using the normal tools, like dobackups.  The HMS driver will use the -scanner.cfg files to configure the HMS card if it is replaced without need to use the external tool.

The column ABS_UpdateCfg in table IOCard allows control of the automatic configuration feature.  If activated, it will compare the previously-captured configuration file with the current configuration and over-write the card’s configuration if a difference is detected.

This feature is intended to make replacement of a defective card easy in the field.  During initial configuration, however, this feature can inadvertently over-write newly-updated card configurations if the old captured configuration file is not removed.  To alleviate this problem, ABS_UpdateCfg is set to N (the default) to defeat the automatic update of the card's configuration.  It should be kept in this state until the card has been configured appropriately.  Once the card's configuration is correct, remove any *-scanner.cfg files in RUN/FORTNA directory and set ABS_UpdateCfg to Y.  Re-initialize the card, or re-start FortnaPlus, and the card's configuration will be captured.

As long as ABS_UpdateCfg is set to Y, if a difference is detected between the card's configuration and the saved configuration, the saved configuration will be used to overwrite the configuration on the card.

The configuration files are named <MACHINE>-<IOCard_Name>-scanner.cfg, where <MACHINE> is the host name of the QNX machine, and <IOCard_Name> is the value from the Name field in table IOCard.  A file named NEW-<MACHINE>-<IOCard_Name>-scanner.cfg may also be present.  This is the most recently captured configuration from the card and is only used to make the comparison.  The NEW- file will be over-written each time FPC is started or the IO card is re-initialized

#### Diagnostic Information for HMS AnyBus

Table ABS_CARD_INFO, accessible from the IO Utilities menu under “AnyBus Card Info”, contains information about the running EtherNet/IP cards, such as the current IP address and other network addresses, and the host and domain name, configuration file names and other internal variable values as read back from the card.  The Name from the IOCard table is reproduced here, and the record numbers in this table will match those in the IOCard table.

The IOCardResp table feature is available for the EtherNet/IP driver.  This table shows counts of total times through the loop (TtlSort), total inputs received (TtlInputs), and counts in columns Iter0 through Iter10 of how many passes elapsed between receipt of information from the card, histogram-style.  This information is helpful in evaluating the performance of the card.

#### Accessing the Web Interfaces of EIP Devices with HMS AnyBus

We always configure the EtherNet/IP devices on a separate network to keep their traffic separate from the general-purpose network traffic.  We also configure a second standard network card in the automation controller to be on the EtherNet/IP network when using HMS AnyBus cards.  Most of the devices on the EtherNet/IP network have a web interface that may be used to inspect or change settings or view diagnostic information, including the HMS card.  To access these devices with a browser from a remote connection, it is necessary to establish secure-shell tunnels.  Also, the scan list configuration of the HMS AnyBus EtherNet/IP Master (controller) card cannot be configured with the browser included with QNX, so a more modern browser must be used, with the aid of ssh tunnels, to change these settings.

In cases where the automation controller is only accessible through one of our Linux servers, it is necessary to use two tunnels to reach the EtherNet/IP network.  For this example, we will build the tunnels starting at the local end and work toward the EtherNet/IP network.  Starting with a VSphere virtual machine, use PuTTY to establish a tunnel on an unused local port to an unused port on the Linux server.  For this example, we will use port 4000 at the “near” end (on the VSphere machine), and port 4001 on the Linux server to connect to port 80 on the EtherNet/IP network on the Automation Controller.  The port numbers are arbitrary, but be sure to run commcore-connection-status and choose a port that is not already in use!  Different values have been chosen here for clarity of illustration.

The first tunnel is established in PuTTY using Connection | SSH | Tunnels.  Enter the local port (4000) in the “Source port” box, and “localhost:4001” in the “Destination” box.  (In ssh command-line notation, that would be:
  -L4000:localhost:4001).  This connects port 4000 on the VSphere machine to port 4001 on the Linux server.

The next tunnel connects port 4001 on the Linux server to port 80 on the desired device on the EtherNet/IP network.  It must be established at the command line on the Linux server while connecting to the AC.  The command line for this example is:
    ssh root@AC11 –L4001:192.168.3.10:80
where AC11 is the DNS name or IP address of the general-purpose network of the AC, and 192.168.3.10 is the IP address of the device on the EtherNet/IP network that we wish to access.

With these tunnels established, a web browser on the VSphere machine may be used to access the web interface of 192.168.3.10 by using this address:

http://localhost:4000

To access a different device on the EtherNet/IP network, the second tunnel must be re-defined to point to that device.  Additional tunnels may be established on other unused ports if it is necessary to access multiple devices simultaneously.

If the AC may be accessed directly from the VSphere VM, the first tunnel is not needed.  PuTTY may be used to establish the tunnel directly to the AC, in the same fashion as the second tunnel in our example.

Note that in PuTTY, tunnels may be re-established without breaking the connection to a remote machine.  This is done by right-clicking the title bar of the PuTTY window of an established connection, and selecting “Change Settings…”.  In the “PuTTY Reconfiguration” window, choose “Connection | SSH | Tunnels”.  If you wish to remove and re-configure an existing tunnel, find it in the “Forwarded ports” box, click it, then click “Remove”.  It will be placed back into the “Source port” and “Destination” boxes, where you may edit or replace it.  To establish a new tunnel, be sure to click “Add” before clicking “Apply”.  The new tunnel will be usable immediately without having to log out and log in again.

#### Manually Deleting EtherNet/IP Configuration for HMS AnyBus EtherNet/IP

If the status LED on the HMS card is red, it may be caused by an incorrect card configuration.  The card may not be able to communicate with the IO racks, because the configuration for a new card has not been loaded properly.

In some cases, the card configuration must be deleted and reloaded using the card’s web interface.  Use tunnels and the browser as described in the prior section to select “Scan List Config”, then select the “Delete Targets” function from the browser screen.  Once all the racks have been deleted the card should load the configuration properly after restarting FPC.  A green light on the HMS card indicates it is talking to the configured racks.

To verify the IO network and HMS cards are detected in the correct slot and connected to the correct network, flag or force IO and verify the expected devices are changing state.

#### FTP Access and Special Settings for HMS AnyBus EtherNet/IP

HMS cards, once initialized by FPC, may be accessed via ftp through their network ports.  The various configuration files may be retrieved for inspection in this fashion.  Certain items may be updated this way, as well.  For instance, the ABS_AutoNeg field in table IOCard controls the network speed and duplex settings.  If set to N, the speed will be set to 100 Mbps and the duplex to FULL – the most commonly used settings, and the default.  If Y is chosen, the card will attempt to determine the appropriate settings based on the apparent setting of the remote end of its connection.  If a different pre-determined setting is required, a configuration file named “etn.cfg” may be created and placed on the card, using ftp, in the /cfg directory.  If Y is chosen for ABS_AutoNeg and this configuration file is present, the settings it contains will be used.  The file format is as follows:

[AutoNeg]
  OFF

  [Speed]
  100

  [Duplex]
  FULL

Choose appropriate values, as desired.  Using the optional /cfg/etn.cfg file allows explicit selection of parameters other than 100 Mbps, FULL.  Auto-negotiation will not occur if this file is present on the card.  It is not present by default.

#### Firmware Updates for HMS AnyBus EtherNet/IP

HMS AnyBus may provide firmware updates for the card from time to time.  To update the firmware, the card must be operating and accessible via ftp.  The update will be in the form of a file ending with hms.  The update is done simply by using ftp to place the update file in the top-level directory of the card (where the directories cfg and ram are visible).  Be sure to set binary mode when transferring the hms update files.  Once the file has been transferred, re-initialize the card.  It will take some time for the card to digest the update file.  Wait at least two full minutes for the update to complete, then re-initialize the card again.  Use the web interface to verify that the expected firmware revision is active.

#### IOCard Table Entries for HMS AnyBus EtherNet/IP

Configuration of the IOCard table differs for each IO card type, and the HMS card has more to configure there than most.  The following table shows all the relevant fields.  If a field is omitted, the HMS card does not use it.

| IOCard Table – HMS Master Card |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each card. |
| Desc | String | Enter a general description of the card. |
| Type | PopUp | Select “AnyBus EtherNet/IP” from the card type options. |
| BasePort | Hex | The base port. Set automatically. |
| ResetPort | Hex | The reset port. Not used. |
| pStatus | String | Updated by the driver with the card status, taken from table Errors. |
| InputMenu | PopUp | Choose the input data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred from the dual-port memory of the card. Choose from InputData[1-5]. |
| OutputMenu | PopUp | Choose the output data table. The number of rows in the table is used by the driver to determine the number of bytes of data transferred to the dual-port memory of the card. Choose from OutputData[1-5]. |
| Status | Hex | Set by the driver. Running state = 2. Set to 0 on reset, 1 during initialization if the card is defined. |
| Process | PopUp | Choose the SORT process. This defines the process that will monitor the IO card for messages and transfer input and output data to the board. Note: The process that transfers the data from the input and output tables to the FortnaDT table is defined in table Configio. |
| IntAddr | Hex | Internal Address assigned by the operating system for the shared memory area. |
| Error | Hex | Error code returned from the IO card or the driver process. |
| InputPtr | Int | Internal index that points to the InputMenu selected. |
| OutputPtr | Int | Internal index that points to the OutputMenu selected. |
| MaxInput | Int | Set by the driver. Maximum number of bytes read from the card based on the size of the input table. |
| MaxOutput | Int | Set by the driver. Maximum number of bytes written to the card based on the size of the output table. |
| CurrMsg | Int | Set by the driver. Current message number. |
| IntrpPort | Hex | Interrupt port. Set automatically. |
| CardState | Int | Set by the driver. The current state of the driver state machine. |
| IntrrAddr | Hex | Set by the driver. The Internal Address used by the card and assigned by the operating system for the second shared memory area. |
| ConfigMode | Int | Use this field to put the card in IDLE (or PROGRAM) mode. A value of zero selects RUN mode, the normal state. Any other value selects IDLE mode. The card must be in IDLE mode to program the scan list. This is done using an external program such as RSNetWorx or by using the card’s web interface. |
| TimeLastCmd | Int | Set by the driver. The time of the last command sent to the card. |
| SerialNumber | Hex | Set by the driver. The value is read from the card. |
| LoopCnt | Int | Set by the driver. Indication of activity. Rolls to 0 after 9999. |
| InputCnt | Int | Set by the driver. Shows the inputs received. |
| Machine | PopUp | Choose this machine. |
| TNS | Int | Set by the driver. This is the serial number of the last message sent to the card. |
| LinkState | Hex | Set by the driver. Indicates the state of the card. 1 = online, 0 = offline. |
| StatusByte | Hex | Set by the driver. The current value of the card’s handshake register. |
| WDCnt | Hex | Set by the driver. Watchdog counter value. |
| BlockCnt | Hex | Set by the driver. WCI register value – internal value for FortnaPlus-to-card handshaking. |
| PCIDeviceIndex | Int | If more than one of a given card type is present, use this to select a particular card. Zero represents the first card. |
| NodeIndex | Int | Set by the driver. Tracks the last adapter or node updated in diagnostics. |
| TypeIndex | Int | Set by the driver. 0x85 = Master (Controller) card type, 0x83 = Slave card type. |
| LogLevel | PopUp | Controls log output. A higher number means more logging. |
| ErrorOutput | PopUp | Optionally select a part to indicate when an error occurs. |
| ErrorACTION | PopUp | Choose the action to take on ErrorOutput when an error occurs. |
| ErrorBIT | Int | Enter the bit to use if an ErrorACTION of Enable or Disable is chosen. |
| RunDelay | Int | Set the time, in seconds, for the driver to wait before activating the inputs and outputs at startup. Avoids processing spurious inputs or setting outputs inappropriately during startup. |
| pRunDelayTmr | Int | Internal storage used by the RunDelay timer. |
| ABS_IP | String | IP address for the HMS card. Typically set to 192.168.1.10 |
| ABS_Mask | String | Network mask for the card. Typically set to 255.255.255.0 |
| ABS_Gateway | String | Gateway for the card. Typically set to 192.168.1.1 |
| ABS_DNS1 | String | First Domain Name Server address for the card. Typically set to 192.168.1.1 (We don’t need DNS, but the driver must have a value) |
| ABS_DNS2 | String | Second Domain Name Server address for the card. Typically set to 192.168.1.1 (We don’t need DNS, but the driver must have a value) |
| ABS_HostName | String | A host name for the card. Typically set to “FortnaPlus”, or some variant if multiple cards are present. A value is needed by the driver, but the particular value does not matter for proper operation of the card. |
| ABS_Domain | String | The internet domain name for the card. Typically set to “fortna.net”. The value is not important but must be set. |
| ABS_UCCM_TIMEOUT | Int | Default is 5000 msec. Ensures that the card will wait sufficiently long before re-sending an asynchronous message. Needed for proper start-up. |
| ABS_AutoNeg | Y/N | Controls setting of network speed parameters. The default, N, sets 100 MBps and Full Duplex. Setting Y allows the card to auto-negotiate with the switch for these parameters. See section 3.7.5 for full details |
| ABS_UpdateCfg | Y/N | Controls when and how configuration information is pushed to the card. Defaults to N. See section 3.7.1 for full details. When set to N, will NOT alter the configuration information on the card. If the configuration stored in the file does not match the card, a log message is written. If set to Y, the configuration file is used to update the card. The configuration files, “*-scanner.cfg”, are found in RUN/FORTNA. The one that starts with NEW is the file just retrieved from the card with the last initialization. The one without NEW is treated as the reference configuration and will be written to the card if this parameter is set to Y. Once initial configuration is set as desired, this parameter should be set to Y. Once set to Y, if the card ever needs to be changed, the configuration will automatically be updated when the card is initialized. If the rack configuration is changed, the stored configuration file must be removed so a new one may be created with the updated information. |

#### Error Codes for HMS AnyBus EtherNet/IP

The following error codes may be generated by the HMS driver.

| HMS AnyBus EtherNet/IP Error Codes |  |  |
| --- | --- | --- |
| IOCard_Error | IOCard_pStatus | Notes |
| 0x1010 | EIP_InitBoard: Input or output table error. | Invalid table |
| 0x1011 |  | Table has no data (no pointer) |
| 0x1012 |  | Table does not have enough columns |
| 0x1013 |  | Not used |
| 0x1014 |  | Table column does not exist |
| 0x1015 |  | Table type is DB – not allowed |
| 0x1016 |  | Unexpected column type (not UNSIGNED_CHAR) |
| 0x2100 | Err: Unable to attach pci |  |
| 0x2101 | Err: MAP_FAILED BasePort |  |
| 0x2102 | Err: MAP_FAILED IntrpPort |  |
| 0x2013 | Err: can’t PCI_INIT_ALL |  |
| 0x2014 | Err: can’t pci_attach_device |  |
| 0x3000 |  | InitState – no error message |
| 0x3001 |  | HWReset1 state – no error |
| 0x3002 |  | HWReset2 state – no error |
| 0x3003 |  | HWReset3 state – no error |
| 0x3004 | Err: HdwInit Failed |  |
| 0x3005 |  | PrepInit state – no error |
| 0x3006 | Err: StartInitSnd Failed |  |
| 0x3007 | Err: StartInitAck Failed |  |
| 0x3008 | Err: Invalid FieldBus Type Err: SetIPConfig Failed |  |
| 0x3009 | Err: Invalid FieldBus Type Err: SetIPConfigAck Failed |  |
| 0x300A | Err: SetSpdConfig Failed |  |
| 0x300B | Err: SetSpdConfigAck Failed |  |
| 0x300C | Err: SetScanlistCfgCtrl Failed |  |
| 0x300D | Err: SetScanlistCfgCtrlAck Failed |  |
| 0x300E | Err: SetHostDomain Failed |  |
| 0x300F | Err: SetHostDomainAck Failed |  |
| 0x3010 | Err: SetDnsServers Failed |  |
| 0x3011 | Err: SetDnsServersAck Failed |  |
| 0x3012 | Err: SetUCMMTimeout Failed |  |
| 0x3013 | Err: SetUCMMTimeoutAck Failed |  |
| 0x3014 | Err: SetMinCL1OTTmt Failed |  |
| 0x3015 | Err: SetMinCL1OTTmtAck Failed |  |
| 0x3016 | Err: AnybusInitSnd Failed |  |
| 0x3017 | Err: AnybusInitAck Failed |  |
| 0x3018 | Err: EndInitSnd Failed |  |
| 0x3019 | Err: EndInitAck Failed |  |
| 0x301A | Err: GetIPConfig Failed |  |
| 0x301B | Err: GetIPConfigAck Failed |  |
| 0x301C | Err: GetHostDomain Failed Err: Invalid Fieldbus Type |  |
| 0x301D | Err: GetHostDomainAck Failed |  |
| 0x301E | Err: GetDnsServers Failed Err: Invalid Fieldbus Type |  |
| 0x301F | Err: GetDnsServersAck Failed |  |
| 0x3020 | Err: GetUCMMTimeout Failed |  |
| 0x3021 | Err: GetUCMMTimeoutAck Failed |  |
| 0x3022 | Err: GetMACAddr Failed |  |
| 0x3023 | Err: GetMACAddrAck Failed |  |
| 0x3024 | Err: OpenRCfgFile Failed |  |
| 0x3025 | Err: OpenRCfgFileAck Failed |  |
| 0x3026 | Err: ReadCfgFile Failed |  |
| 0x3027 | Err: ReadCfgFileAck Failed |  |
| 0x3028 | Err: CloseRCfgFile Failed |  |
| 0x3029 | Err: CloseRCfgFileAck Failed |  |
| 0x302A | Err: OpenWCfgFile Failed |  |
| 0x302B | Err: OpenWCfgFileAck Failed |  |
| 0x302C | Err: WriteCfgFile Failed |  |
| 0x302D | Err: WriteCfgFileAck Failed |  |
| 0x302E | Err: CloseWCfgFile Failed |  |
| 0x302F | Err: CloseWCfgFileAck Failed |  |
| 0x3030 | Err: SetScanMode Failed Err: Invalid Fieldbus Type |  |
| 0x3031 | Err: SetScanModeAck Failed |  |
| 0x3032 |  | IDLE mode – no error message |
| 0x3033 |  | RUN mode – no error message |
| 0x3034 | Err: SWResetSnd Failed |  |
| 0x3035 | Err: SetScanMode Failed |  |
| 0x3036 | Err: SetScanMode Failed |  |

#### Banks in the Input and Output Tables for HMS AnyBus EtherNet/IP

Bank 0 – Reserved for the adapter
Bank 1 – Reserved for the adapter
Bank 2 – High bits for the first module
Bank 3 – Low bits for the first module

# IOCard Watchdog

NOTE: This feature is disabled by default.

The IOCard software checks the CTRL_RUNNING flag (Integer Flags #151 - flagmenu) to determine if the CTRL process is running.  The flag is updated by the SORT process when the first event of the CTRL process Watchdog Reset command is received.  If the CTRL process does not update the CTRL_RESET_WATCHDOG flag (Integer Flags #148) in the allotted time (Integer Flags #145 – CTRL_WATCHDOG_SEC) the CTRL_FAILED flag will be set and the CTRL_RUNNING flag will be reset by the SORT process.

If the Process is not running, the IOCard interfaces will stop updating the IO.

The default value for the CTRL_INIT_DELAY_SEC flag (Integer Flags #144) is 30 seconds.  It can be set to a larger value to give the CTRL process more time to start, if needed.

If the watchdog fails, the Error “CTRL PROCESS WATCHDOG” will be activated.  The error can be reset using the “IO Card Init” function from the IO Utilities menu, which will also reset the CTRL_FAILED flag (Integer Flags #150).

Set CTRL_WATCHDOG_SEC to 0 to disable this feature, which is the default.