# FortnaPlus Control Inch and Store Module

Document Revision: 1.5

Date: 27-Jan-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 16-Dec-2013 | B C Conklin | New document |
| 1.3 | 02-Jan-2014 | B C Conklin | Add two new columns for motor control |
| 1.4 | 24-Jan-2014 | B C Conklin | Add LastStoreDumpDly timer |
| 1.5 | 15-Feb-2006 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	Overview	5

2.	Software	5

3.	Required Batches Entry	5

4.	Inch and Store Module	5

5.	The InchStore Table	7

6.	Inch and Store Example	10

7.	Example Configuration	11

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Overview

The FortnaPlus Control (FPC) system consists of hardware and software that work together to automatically transport containers via conveyors, sorters, and transfers to their designated areas throughout the distribution center.

The WMS/Host system provides the information on container destinations.  FPC provides the means, both logical and physical, to get the container to its destination as efficiently as possible.

# Software

FortnaPlus machine control software runs on QNX, a real-time operating system.  It is used to control the conveyor hardware.  User interface capabilities include:

View status of conveyor system.

Start and stop the conveyor system.

View errors that stop the system, such as jams, e-stops, and full conditions.

View scanner statistics.

# Required Batches Entry

The Inch and Store module must be activated in table Batches:

InternalProcCtrl = “InchStore   (CTRL)”

# Inch and Store Module

The Inch and Store module uses a specific configuration of conveyor belts and photoeyes to allow accumulation of containers either until the belt is full or a pre-set amount of time has passed.  The accumulated containers are then released as a “slug” before the process begins again.  There is also provision for release under software control, or via a manual override.

Inch and Store lanes are often used in conjunction with merge lanes to build slugs for efficient merge operation.

This document describes the function of the Inch and Store module in detail.

The Inch and Store module is configured through a single table, InchStore, in FPC.  This table can be found under Configuration Menu | Standard Controls | Inch and Store.

The physical conveyor configuration required to use this module is shown in the following diagram.  Three segments of conveyor are required.  The middle segment is the Inch and Store belt.  The upstream segment is controlled by the Inch and Store module to regulate the incoming cartons.  The downstream segment must provide a control signal to the Inch and Store module to stop the other two segments if the downstream segment becomes full or is otherwise unable to accept a slug of containers from the Inch and Store belt.

Inch and Store Belt

Infeed PE Location

Infeed Belt

Discharge PE Location

Carton Gap

Longest Carton

Exit Belt

In addition to the conveyor segments, two photoeyes are required.  The infeed photoeye detects containers entering the Inch and Store belt from the infeed belt and is used to control the motor driving both the infeed belt and the Inch and Store belt.  The discharge photoeye is located near the end of the Inch and Store belt.  It is positioned so the longest container will fit on the belt after the photoeye.  Room is required at the end of the Inch and Store belt after the discharge photoeye because the belt will continue to advance when a container is detected at the discharge photoeye until the infeed photoeye turns OFF.  The most downstream container (the first container in the slug) must not advance onto the exit belt until the discharge even begins.

Provision is also made for input signals that are used to discharge any accumulated containers upon command, and to override the Inch and Store behavior, allowing the belts to operate as “normal” conveyor, if needed.  Partial slugs can also be automatically released after a period of inactivity.

The state diagram, below, shows the conditions required to move between the various states as cartons arrive, and are discharged.

Inch and Store State Machine

Idle

Store

Gap

Gap and Hold

Discharge

Run

InFeed PE ON

InFeed PE OFF

Discharge PE ON

InFeed PE ON

Discharge PE ON

Discharge PE ON

Active Mode OFF

Active Mode ON

Gap Complete

# The InchStore Table

Use the InchStore table to configure an Inch and Store conveyor, as described here:

| InchStore Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the Inch and Store section. |
| Machine | PopUp | Choose the controlling machine. |
| ResetNow | Button | Click this button to start an Inch and Store discharge sequence. This has the same effect as turning DumpInput ON. |
| pState | PopUp | Set by the software to indicate the state of the Inch and Store module. |
| pStatusMsg | String | Status messages may appear here. |
| InFeedPE | PopUp | Choose the photoeye input at the beginning of the Inch and Store belt. This photoeye detects containers as they arrive. It will also cause the infeed belt to stop if new containers arrive while the Inch and Store belt is discharging. It is also used to create gap as containers arrive on the Inch and Store belt. |
| INSMotor | PopUp | Choose the motor output for the Inch and Store belt from the Conveyor table. Disable bit 14 is used to control the motor. |
| SlowOutput | PopUp | Choose an output that will cause the Inch and Store motor to run at a slow speed while containers are being stored. This is only needed in applications where the Inch and Store speed is different than the discharge speed. If the motor is controlled with a single output, select it in INSMotor instead. |
| SlowAction | PopUp | Choose the action type, such as FIRE or ENABLE that is needed to activate SlowOutput. |
| SlowBit | Int | If SlowAction is ENABLE or DISABLE, enter the bit to use. |
| DumpOutput | PopUp | Choose an output that will cause the Inch and Store motor to run at a fast speed while the belt is discharging. This is only needed in applications where the Inch and Store speed is different than the discharge speed. If the motor is controlled with a single output, select it in INSMotor instead. |
| DumpAction | PopUp | Choose the action type, such as FIRE or ENABLE that is needed to activate DumpOutput. |
| DumpBit | Int | If DumpAction is ENABLE or DISABLE, enter the bit to use. |
| FeedTimer | PopUp | Choose a timer that will be used to create a gap between containers as they arrive on the Inch and Store belt. |
| DischargePE | PopUp | Choose the photoeye input at the end of the Inch and Store belt. When the first stored container reaches this photoeye, it will signal the need to enter the discharge state. Locate the photoeye so the longest container will fit on the belt after the photoeye is blocked. When this signal comes ON, the belt will continue to advance until InFeedPE turns OFF, so there must be room for the entire lead container on the belt. If the Inch and Store slug is to be fed onto a downstream belt, verify it will fit on that belt in its entirety so the discharge cycle can complete, and a new slug can be built without waiting for downstream to clear. |
| FeedOutput | PopUp | Select the motor or solenoid that will feed product onto the Inch and Store belt. It will be disabled using bit 14 to create gaps as the Inch and Store belt is being filled and to hold arriving containers while the slug is being discharged. |
| DumpInput | PopUp | Choose an input signal that will initiate a discharge manually. The Inch and Store belt will be dumped when the signal turns ON. This would most likely be a pushbutton an operator could use to discharge the belt before it is full. Choose NEVERON if this is not needed. |
| ResetTimer | PopUp | Optional. Choose a timer that will initiate a discharge event periodically, even if the Inch and Store belt is not full. As long as at least one container has been stored, the belt will release when the timer expires. See ResetFromFirst. This is meant to ensure partial slugs of containers are cleared from the Inch and Store area after a period of inactivity. |
| ResetFromFirst | Button | For use with ResetTimer. If this button is OFF, ResetTimer will be restarted as each container is inducted. The last container to arrive will determine when the automatic release occurs. If this button is ON, ResetTimer will start when the first container arrives after a discharge cycle. The first container to arrive will determine when the automatic release occurs. |
| Enable | PopUp | Choose an input signal to control the Inch and Store logic. When this signal is OFF, the infeed belt and the Inch and Store belt will stop, and DumpOutput will not activate. The timers will be suspended. If the downstream conveyor is controlled by a merge, use DischargeMtrEnabled instead, which will allow the Inch and Store logic to build a slug, but prevent it from discharging. |
| EnableDischarge | PopUp | Choose an input signal, if needed, to control when a discharge cycle will be allowed to start. When OFF, this signal prevents initiation of a discharge cycle. If a discharge cycle has begun and this signal turns ON, the discharge will complete. This could be used to prevent a discharge when a downstream merge goes into recirculation mode, holding additional product until the sorter has cleared the backlog. |
| DischargeMtrEnabled | PopUp | Choose an input signal, if needed, to control when a discharge cycle will be stopped. This is similar to EnableDischarge, but it only prevents a discharge event, allowing slug building to continue even when this signal turns OFF during a slug building cycle. Slug building continues even if the downstream discharge conveyor is stopped. NOTE: If this signal turns OFF during a discharge cycle, the Inch and Store belt must stop! |
| DumpTimer | PopUp | Choose a timer that will set the duration of a discharge event. If new containers arrive at InFeedPE during a discharge cycle, the infeed belt will stop and wait for the discharge to complete before allowing new containers to be inducted. |
| LastStoreDmpDly | PopUp | Choose a timer that will delay the discharge event. When the Inch and Store belt is full (as indicated by InFeedPE turning OFF after DischargePE has turned ON), this timer will start. Once this timer completes, the discharge cycle will begin. |
| Mode | PopUp | Choose an input signal to control bypass mode. The state of the input determines the behavior: OFF = Inch and Store mode is disabled. The belts run as regular conveyor with no special behavior. ON = Inch and Store mode is active. The input could be a manual switch or any other input signal. Choose ALWAYSON of this feature is not needed. |
| LogLevel | PopUp | Choose a logging level. Currently, the only Inch and Store specific logging is at “Level 2”. |
| pDschrgeLtch | Int | Set by the software to track discharge events. 1 = discharging. |
| pAtLeastOne | Int | Set by the software to track if any containers are in process. A value of 1 indicates one or more containers are present on the Inch and Store belt. Used by the ResetTimer feature. |
| pDschrgeStarted | Int | Set by the software to track when a discharge event is occurring. |

# Inch and Store Example

A merge is configured with an Inch and Store belt feeding a slug belt.  Inch and Store will build a slug of cartons.  If the merge lane is not feeding, the slug will feed from the Inch and Store belt onto the slug belt once a complete slug has been built.

The merge will create a reservation for the slug and feed it into the merge when its turn arrives.

Software Configuration

- The Inch and Store belt is configured with two speeds:

  - IS-Slow – Slow speed to build a slug.

  - IS-Fast – Fast speed to discharge a slug.

- The slug belt is configured with two speeds in the HSSawLane table:

- SL-Slow – Slow speed used to transfer the slug from the Inch and Store belt to the slug belt.

- SL-Fast – Fast speed used to feed the slug into the merge.

The Inch and Store belt is interlocked with the merge lane through the motor speed control columns.  Inch and Store is configured with the DischargeMtrEnabled column set to SL-Slow.  This signal only comes ON when the merge lane is running at the slow speed.  The merge lane only runs at the slow speed when it is not discharging, and it does not have a container present.

In this configuration, it will feed the Inch and Store slug up to the merge lane presence PE.  DumpTimer is set to a value that is slightly less than the time needed for the slug to reach the merge lane presence PE.  The merge logic takes over once the Inch and Store discharge completes and moves the slug up to the merge lane presence PE.

When testing the configuration, verify the slug feeds from the Inch and Store onto the merge lane belt and the Inch and Store status transitions from “Discharging” to “Idle” prior to the merge lane presence photoeye.

Slug Belt

SL-Slow

Inch/Store Induct PE

PE120_IND

Infeed Brake-Meter M119

Discharge PE

PE120_DIS

Carton Gap

Merge Belt

Merge Lane PE

Inch and Store Belt

IS-Slow and IS-Fast

# Example Configuration

| Batches Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | HowOften | Increment | Type | Process | InternalProc | InternalProcCtrl | Machine |
| IS1 | 1 | Invalid | Count | CTRL |  | InchStore (CTRL) | CP1 |

| InchStore Table – Part 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine | InFeedPE | INSMotor | Slow Output | Slow Action | Slow Bit | Dump Output | Dump Action | Dump Bit |
| IS1 | CP1 | PE120_IND | INVALID | IS-Slow | FIRE | 0 | IS-Fast | FIRE | 0 |

| InchStore Table – Part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FeedTimer | DischargePE | Feed Output | Dump Input | Reset Timer | Enable | Enable Discharge |
| tim_InsFeed | PE120_DIS | M119 | N/A | N/A | MemSysRUN | INVALID |

| InchStore Table – Part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| DischargeMtrEnabled | DumpTimer | LastStoreDumpDly | Mode | LogLevel |
| SL-Slow | tm_InsDump | N/A | ALWAYSON | Level 2 |