# FortnaPlus Control Communications Configuration – Machine and MsgMap

Document Revision: 1.1

Date: 22-Feb-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | unknown | B C Conklin | New document |
| 1.1 | 22-Feb-2003 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	FPC Communications Configuration	5

2.	Machine Table	5

3.	Connection Types	9

4.	Protocols	10

5.	Message Map	11

6.	Network Connectivity	13

6.1.	Troubleshooting	13

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

WCS – Warehouse Control System

# FPC Communications Configuration

The tables described in this document are used to configure connections to external devices.

# Machine Table

The Machine table (Configuration Menu | Communication Utilities | Machine) defines connections to other systems and devices.  These often include other FPC ACs, scanners, scales, and CommCore.

In an installation with multiple automation controllers, each AC appears in every other AC’s Machine table.  FPC uses these connections to communicate the status of IO parts between ACs.

NOTE: If you add or remove rows in table Machine, you must restart FPC, since the initialization functions are run only at startup.

| Machine Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Machine_Name | String | Enter a unique name for the connection. For TCP connections, this name must also appear in /etc/hosts to link this entry to an IP address. |
| Status | String | Status messages may appear here. |
| Error | String | Error messages may appear here. |
| Machine_Index | Int | Set by the software to manage the connection |
| Machine_Pid | Int | When a connection has been established, the process ID value (a large integer) will appear here. To force reset of a connection, enter -1 (minus one) here. If the connection is stable, the value will not change. If the connection is not stable, you will see -1 appear momentarily, then a new large integer as FPC attempts to re-establish the connection. Consult the log file associated with Process for diagnostic information. |
| Connection_Type | PopUp | Choose a connection type. Popular choices include: CONNECT_UDP – AC-to-AC connections. CONNECT_QNX – AC to AC connections with GNS. CONNECT_SERIAL – Scanners and Scales. CONNECT_TCPCLIENT – Scanners and CommCore connections. AMQ_TALK, AMQ_LISTEN – Active MQ connections. See section 3 |
| Communication_ Protocol | PopUp | Choose a communication protocol. Popular options include: HOST_SORTPLUS – AC-to-AC connections. AlphaNum – Scanners and Scales. SrtComm and SrtCommSeq – CommCore connections. See section 4 |
| Machine_Port | String | Enter the port number, device path, queue, or topic name, or QNX global name to use for this connection. TCP – A port number, like 9200. Serial – A device path, like /dev/ser-rckt1 AMQ – A topic or queue path, like: /topic/WCSEVENT.FPC.FP.MISC QNX – A global name, like AC1_P2 UDP – A port number, like 9200 |
| Transmit_Retry | Int | Set the time, in seconds, between message transmission attempts. The message will be re-sent when the time elapses if it has not been acknowledged. |
| Offline | Y/N | Set by the software. Y indicates data has been lost due to unacknowledged messages. |
| Process | PopUp | Choose the process that will manage this connection. Check the log files for the process selected here for messages and errors related to this connection. |
| UseDB | Y/N | N = Track messages in memory-based tables using circular buffers. This is the usual choice. Y = Use the MySQL database for message tracking. |
| IgnoreDupes | Y/N | See Id and LastId in the MsgMap table. N = If a duplicate message is received, it will be processed and will replace the prior copy. Y = Ignore duplicate copies of messages. |
| NoCircbuff | Y/N | N = Create circular buffers, as normal. Required for non-database messages. Y = Do not create circular buffer space. You can use this to conserve memory if UseDB = Y. |
| CircSize | Int | Defines the size of the circular buffer used to transmit messages. The default size is 1200 messages. This should really be set to match the number of rows in the associated message table, but is seldom adjusted in practice. Be aware that if the connection is broken, but new data is still being generated, you can have many circular buffer pointers to the same record in the message table. The data in the message table is over-written, but the circular buffer pointer remains, so when the connection is restored, the same message data can be re-sent multiple times – once for each circular buffer pointer. Message tables have 100 rows by default. |
| Trig_Size | Int | Use this field to define a fixed message length that will trigger message processing. This can be used in conjunction with Trig_InterChar and Trig_MsgEndChar. |
| Tring_InterChar | Int | For serial connections. Incoming messages are processed after this amount of time elapses between characters, once the first character has been received. The value is in milliseconds. The default value is 100 milliseconds. The allowed range is 100 to 1000 milliseconds. This can be used in conjunction with Trig_Size and Trig_MsgEndChar. |
| Trig_Timeout | Int | For ActiveMQ connections, this value is in microseconds. If set to 0, the software uses 50 microseconds internally. If the logs show the software is processing incomplete messages, increase this value to 1000 (1 millisecond). Search for “amq_iohandler” in the logs. For CONNECT_QNX connections, this value is set internally to twice the Process cycle time. It will cause the connection to reset when send errors occur. For serial connections, this field is obsolete in all QNX6 versions. It was available in QNX4 versions where it was used to initiate message processing based on the total time between the first and last characters received. |
| Trig_MsgEndChar | PopUp | For use with serial protocols. Choose a character from table AsciiMnu to indicate the end of a message. The default is ETX (a.k.a., Ctrl-C or 0x03). Ctrl-J (LineFeed or 0x10) is also a popular choice. When received, this character is interpreted as the end of a complete message and will initiate processing of that message. Most often used with scanners and scales. This can be used in conjunction with Trig_Size, and Trig_InterChar. |
| Log File | PopUp | Seldom used. Choose the process log file to use for the machine log. See LogLevel instead, which defines logging for the Process, where the protocol will write messages. |
| Log Length | Int | Seldom used. Sets the number of characters of each message to write to the machine log file defined in Log File. See LogLevel instead, which defines logging for the Process, where the protocol will write messages. |
| NumCbItems | Int | not used |
| LastMsgAttempts | Int | not used |
| MaxAttempts | Int | Set this value to define the number of send attempts before the message is removed from the queue and set to a status of NACK. A value of 0 means no limit. It is best to define a reasonable value here so a failed message will be discarded and allow newer messages to be sent. |
| StripNulls | Int | not used |
| TimerId | Int | not used |
| RetryCount | Int | not used |
| MaxRetries | Int | not used |
| FortnaMachine | Y/N | Set by the software when the HOST_SORTPLUS protocol is selected. |
| NeedACK | Int | 0 = No acknowledgement message is needed. 1 = An acknowledgement message is required. Many protocols set this value automatically. Note that NeedACK = 1 has the side-effect of preventing a new message from being sent until the previous message has been acknowledged, which is often, but not always, required for proper operation. |
| LogLevel | PopUp | Choose the desired log detail level. Level 1 is most useful, as it writes message contents along with information about connection errors. Level 0 will write only connection error information to the logs. Higher levels may include more detail that is useful for diagnostic purposes but can generate very large log files. |
| Username | String | For ActiveMQ connection types. Enter the username needed to establish the connection. If ActiveMQ is configured properly, no Username or Password should be needed. |
| Password | String | For ActiveMQ connection types. Enter the password needed to establish the connection. Caution: The password is stored in clear text in the Machine.asc file. If ActiveMQ is configured properly, no Username or Password should be needed. |
| CloseDelay(ms) | Int | For the CLOSE_CLIENT or CONNECTC_TCPCLIENT connection type. This defines the time to wait, in milliseconds, before closing the connection. If set to 0, the software uses 500 mSec internally. |
| OpenTimeout(ms) | Int | For ActiveMQ connection types. Set the maximum time, in milliseconds, to wait for a new connection to open before declaring a connection failure. If set to 0, the software uses 10 mSec internally. If the logs indicate “stomp_connect failed”, try increasing this to 1000 mSec (1 Sec). |
| DontSetNeedAck | Y/N | N – NeedACK may be set by the protocol as needed. For example, SrtCommSeq will automatically set NeedACK = 1 Y – Disable automatic setting of NeedACK by the protocol. Use this to override the default setting of NeedACK. |
| pCBuff | Blob | A 2048-character buffer string used internally for circular buffer messages. Note: Blob fields, such as this one, are not saved to the asc files, as they may be quite large, and to protect against characters that are not allowed in asc files. |
| pCBuffLen | Int | Set by the software. The length of the current string stored in pCBuff. |
| pCBMaxLen | Int | Set by the software. The maximum allowed length of the pCBuff string (2048). |
| pCTimeout | Int | Set by the software. Used for serial connections to determine if the Trig_InterChar time has been exceeded. |
| pCharSet | Blob | Set by the software. The current list of characters allowed for messages on this connection. There is room for up to 288 characters. Note: Blob fields are not saved to the asc files, as they may be quite large, and to protect against characters that are not allowed in asc files, so this string is rebuilt in memory from AsciiMnu each time the machine connection is initialized. |
| pSendSequence | Int | Used by the software to track the most recent message sequence number for sequence protocol messages originated by this AC. |
| pRecvSequence | Int | Used by the software to track the most recent message sequence number received for sequence protocol messages. |

# Connection Types

The Connection Types table (table Connect) is accessed from the Machine table using the down-arrows next to the Connection_Type fields.  It lists the connection types supported by FPC for data exchange.  It also shows the functions used to initialize, and maintain the connections, and to send and receive messages for the connection.

| Connection_Type | Description |
| --- | --- |
| N/A | The default ‘OFF’ entry. Must appear as the ‘zeroth’ row in table Connect. Its INIT, MAINTAIN, SEND and RECEIVE functions must all be set to N/A. |
| CONNECT_QNX | For AC-to-AC connections in conjunction with GNS. |
| CONNECT_UDP | For AC-to-AC connection with UDP. This is the preferred method. |
| CONNECT_SERIAL | For connections with serial ports. Works with built-in serial ports (/dev/ser1), RocketPort cards (/dev/ser-rckt1), etc. Usually used with scales and older scanners |
| CONNECT_DISK | For ‘connections’ that send messages via the hard disk. Probably obsolete. |
| CONNECT_MODEM | For modem connections. Definitely obsolete. |
| CONNECT_TCPCLIENT | For TCP/IP sockets connections where FPC connects as client to a server. Used for CommCore connections, and newer scanners. |
| TCPSERV_LISTEN | Creates a TCP/IP listener. For use with TCPSERV_RDWR to establish a server connection point. |
| TCPSERV_RDWR | For TCP/IP connection where FPC will act as a server. RDWR stands for Reader/Writer. Requires a TCPSERV_LISTEN connection, as well, to listen for client connection attempts. |
| CLOSE_UDP or CONNECTC_UDP | Like CONNECT_UDP, but the connection is closed after each message exchange. |
| CLOSE_CLIENT or CONNECTC_TCPCLIENT | Like CONNECT_TCPCLIENT, but the connection is closed after each transmission. This is often used for connections to PNA printers. The close tells the printer the document is complete so it will print it. A new connection is opened each time a document is ready to be sent. |
| CLOSE_LISTEN or TCPSERVC_LISTEN | Like TCPSERV_LISTEN, but the connection is broken after each message exchange. |
| CLOSE_RDWR or TCPSERVC_RDWR | Like TCPSERV_RDWR, but the connection is broken after each message exchange. |
| AMQ_TALK | For connections to an ActiveMQ message broker where the traffic is only from the AC to the message queue. |
| AMQ_LISTEN | For connections to an ActiveMQ message broker where the traffic is only from the message queue to the AC. |
| CONNECT_AMQ | For connections to an ActiveMQ message broker where the traffic will flow in both directions. |
| WCS_TALK | A special version of AMQ_TALK for connections to EV10 (or ‘The Dashboard’). |
| WCS_LISTEN | A special version of AMQ_LISTEN for connection to EV10 (or ‘The Dashboard’) |

# Protocols

The Protocol Types table (table Protocol) is accessed from the Machine table using the down-arrows next to the Communication_Protocol fields.  It lists the messaging protocols supported by FPC for data exchange.  It also shows the functions used to initialize and maintain the protocols, and to build and parse messages.

| Protocol_Type | Description |
| --- | --- |
| N/A | The default ‘OFF’ entry. Must appear as the ‘zeroth’ row in table Protocol. Its INIT, MAINTAIN, BUILD and PARSE functions must all be set to N/A. |
| HOST_STD | For server-style connections with WMS systems using a sequence number format. Requires an acknowledgement message. |
| HOST_ECHO | For server-style connections with WMS systems that require the entire message to be used as the acknowledgement. Set MsgMap_AckLen = 0 to use the entire message in the acknowledgement. If it is set to something greater than 0, this protocol acts just like HOST_ACK. |
| HOST_ACK | For server-style connections with WMS systems that require an acknowledgement. MsgMap_AckLen plus the length of the string in MsgMap_Ack_Msg_Type are used to define how much of the message to compare. |
| HOST_RAW | For server-style connections with devices that require no message header information, such as scanners or scales. No acknowledgement message is needed. |
| HOST_SORTPLUS | For connections between FPC ACs. Used to keep the conveyor display, start/stop state, errors, and IO status in synchronization. |
| HOST_ALPHA | A very special-purpose host-style protocol for a long-forgotten purpose. Builds messages beginning with 11 Ctrl-A characters, and ‘>’, then a 2 character “signid” then Ctrl-B and ‘ee’, then the message contents, and Ctrl-D. This has not been used in a very long time. |
| AlphaNum | For use with devices that require no message header information and expect no acknowledgement message, such as scanners and scales. AsciiMnu_AlphaNumAllow is used to define how to filter the messages, discarding any characters that are not selected there. Messages must end with CR and/or LF to be processed. |
| HOST_WEB | This was an early attempt at a generic web interface. It is not well documented. See HOST_REST instead. |
| SrtComm | Sorter communications protocol. Used to interface with Commcore for sorter and transfer messages. The messages are wrapped with STX and ETX characters and use the pipe character as a delimiter between fields. See the FPC-Sorter-Control-Module document. |
| SrtTest | Mimics CommCore for testing the SrtComm protocol. |
| PaComm | Print and Apply message protocol. This is the original PandA protocol, using pipe-delimited messages. |
| PaLblImg | Print and Apply label image protocol. Used with LMI messages to handle the (sometimes very large) label files that will be sent to the printers. Often, they are in ZPL format. |
| PaTest | Mimics the PandA engine for testing the PaComm protocol. |
| WebSeq | Another early attempt at a generic web interface. It is not well documented. See HOST_REST instead. |
| WebTestSeq | For use in testing the WebSeq protocol. |
| RemAuth | A special protocol for use with the Remote Authorization module. Uses ActiveMQ connections. See the FPC-RemoteAuthorization document for details. |
| WCS | A special protocol used to communicate WCS events from FPC to ActiveMQ for the ‘Dashboard’ and ‘EV10’ interfaces. |
| STXETX | This protocol uses messages that are wrapped with STX and ETX characters, like the SrtComm and SrtCommSeq protocols, but without defined message content. |
| HOST_REST | A RESTful web server interface used primarily by EV10 to issue commands like START/STOP. It also can be used as a full, generic interface to access any FPC table. See the FPC-REST-API document. |
| SrtCommSeq | Like SrtComm, plus a transaction sequence number. This is the current preferred protocol for CommCore interfaces. See the FPC-Sorter-Control-Module document. |
| SrtTestSeq | Mimics CommCore for testing the SrtCommSeq protocol. |
| SrtFPLite | The FortnaPlus Lite message protocol, used on O’Reilly sites, where FPC serves as its own specialized ‘CommCore’. See the FPC-FPLite-Configuration and FPC-OReilly-QNX6-Config documents. |
| PandA | JSON-based message protocol for Print and Apply, like that used for FRC. |

# Message Map

Table MsgMap (Configuration Menu | Communication Utilities | Message Map) controls the flow of messages to and from FPC.  Data for outbound messages is taken from the table defined in Menu_Name.  Data received in inbound messages is parsed into a tracking table or one of the scan tables.

| MsgMap Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Message_Name | String | Enter a unique name for this message map. |
| Machine_Name | PopUp | Choose the Machine table entry to act upon. |
| Recv_Msg_Type | String | Incoming messages with headers that match this string will be processed using this record. This string is compared with both the CmdHdr (like “CRR”) and Header (like “AC1”) fields in the message. The special string “ALL” will cause all messages received on this connection to be processed. |
| Send_Msg_Type | String | Outgoing messages may use this field to start the message. This string is compared with both the CmdHdr (like “CRR”) and Header (like “AC1”) fields in the message. The special string “ALL” will cause all messages sent on this connection to be processed. |
| Ack_Msg_Type | String | Incoming acknowledgement messages with headers that match this string will be processed using this record. This string is compared with both the CmdHdr (like “CRR”) and Header (like “AC1”) fields in the message. The special string “ALL” will cause all messages received on this connection to be processed. |
| Menu_Name | PopUp | Choose the table that will serve as the source of data for messages to send. Examples: MsgCR1 for CommCore messages, ScnScanDevice for scanner connections. |
| Low_Cursor | Int | Defines the first column in the Menu_Name table to include in the message (outbound) or to receive data (inbound). The first column displayed in a table is column 1. You can use F2-MenuOfMenus and Alt-K to open the table column definitions (MenuColumn). The row number in that table is the column number needed here. |
| High_Cursor | Int | Defines the last column in the Menu_Name table to include in the message (outbound) or to receive data (inbound). Find the appropriate value as described in Low_Cursor. |
| Stat_Cursor | Int | Defines the column number in the Menu_Name table of the TxStat or RxStat column – a popup to table SendStat or RecvStat. This is needed to record the send/receive status of the messages. This field will automatically be populated at startup if you leave it at 0, or find the appropriate value manually as described for Low_Cursor. |
| Last_Record | Int | Set by the software to the row number in the Menu_Name table of the last row used. |
| Low_Head | Int | Used by the HOST_STD protocol when parsing the message header. Defines the first header column to parse. Seldom required. |
| High_Head | Int | Used by the HOST_STD protocol when parsing the message header. Defines the last header column to parse. Seldom required. |
| Key_Cursor | Int | Defines the column number in the Menu_Name table of the DbKey column. This field will automatically be populated at startup if you leave it at 0, or find the appropriate value manually as described for Low_Cursor. |
| Id | Int | Defines the column number in the Menu_Name table of the column that the Machine_IgnoreDupes feature will compare with LastId when finding duplicates. Seldom used. |
| LastId | Int | Defines the column number in the Menu_Name table to use to store the last value for use with the Machine_IgnoreDupes feature. Compared with Id. Seldom used. |
| Record | Int | For use with scanner and scale connections. Defines the row number in the Menu_Name table that will receive the incoming data. This, in combination with Low_Cursor and High_Cursor set to the same column limits the input data to a single field. |
| Delimiter | PopUp | For blob data type fields, choose a delimiter character from table AsciiMnu to define the end of the data of interest within the field. |
| FieldLength | Int | For blob data type fields, define the length of the binary data object. |
| AckLen | Int | Use this to set the length of the acknowledgement header for the HOST_ACK and HOST_ECHO protocols. |
| CheckFile | Y/N | For use with the CONNECT_DISK connection type. Y = Use the string in Recv_Msg_Type as the name of a file that will be read as the source of incoming messages. Can be used with files delivered by ftp, nfs, or samba. |
| pMsgType | Int | Set by the software to the internal code for AC-to-AC messages for the HOST_SORTPLUS protocol. |

# Network Connectivity

See the FPC-System-Checklist document for details on how to set up network connectivity.  Use the phlip command to configure the IP address and network interface for the AC.  Choose a name for the AC that reflects its function.  We usually try to include an abbreviated version of the customer and site in the name, like ORPHXAC1.  Note that these settings are written to /etc/net.cfg.

For TCP connections, the Machine_Name field in table Machine is used to reference the /etc/hosts file, which defines a cross-reference from that name to the machine’s IP address.

## Troubleshooting

Use the ifconfig command to reveal the network configuration and its status.  The nicinfo command and the ‘netstat -an’ command are also helpful.  See FPC-System-Checklist for additional details and more helpful commands.

Use the showfp command to display the FPC processes running on all ACs.  If GNS is in use, you will also see the global names.