# FortnaPlus Control Merge Modules

Document Revision: 1.5

Date: 10-May-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.1 | 17-May-2013 | B C Conklin | Updates with table of contents |
| 1.3 | 02-Jan-2014 | B C Conklin | Updated simulation |
| 1.4 | 27-Jan-2015 | B C Conklin | Added End-of-Wave buttons and lights |
| 1.5 | 16-Jun-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	5

1.	Merge Control	6

2.	Simple Merge	6

2.1.	SimpleMerge Table	6

3.	Multiple Input Lane Merge (New Merge)	8

3.1.	MergeBoss Table	8

3.2.	MergeInputs Table	9

3.3.	MergeRoute Table	12

3.4.	MergeNotBusy Table	13

3.5.	MergeRunOutputs Table	14

3.6.	MergeStopOutputs Table	15

3.7.	Three-to-One Merge Example	16

3.8.	Three-to-Two Merge Example	17

3.9.	Two-to-Two Merge Example	18

4.	Sawtooth Merge	20

4.1.	Reservation Configuration Options	22

4.2.	Configuring the Shifter	23

4.3.	Induct Lane Configuration	24

4.4.	Induct Lane Tuning	25

4.5.	Tuning Between Lanes	26

4.6.	SawMerge Table	27

4.7.	SawLane Table	29

4.8.	Saw Merge End-Of-Wave Feature	32

5.	High-Speed Sawtooth Merge	32

6.	Zipper Merge	33

6.1.	Zipper Merge Theory and Examples	33

6.2.	Zipper Merge Design Constraints – Gapping Requirements	34

6.2.1.	Servo-Gapper Example Design – 3-to-1 Merge	35

6.3.	Standard Zipper Merge	36

6.3.1.	Example:  Standard with Brake-Meter Belt	37

6.3.2.	Example:  Standard with Scale	39

6.4.	Sawtooth Zipper Merge	40

6.4.1.	Control Logic	42

6.4.2.	Reservation Logic	43

6.4.3.	Enhanced Throughput Logic	44

6.4.4.	Example With Scale	45

6.4.5.	Example Without Scale	45

6.5.	Zipper Merge Tables	46

6.5.1.	ZipperMerge Table	46

6.5.2.	ZipperLane Table	48

6.5.3.	Zipper Speed Control Settings	55

6.5.4.	ServoGapper Table	56

6.5.5.	ZprSimConfig Table	57

6.5.6.	ZipperErrLog Table	58

6.5.7.	ZprIOAction Table	58

6.5.8.	ZprStatusMsgs Table	58

6.6.	Zipper Merge Operation and Tuning	58

6.6.1.	Setup for Standard Logic	60

6.6.2.	Setup for Sawtooth Logic	64

6.6.3.	Verify Reservation Size and Gap	64

6.6.4.	Verify Brake-Meter Configuration	65

6.6.5.	Verify the Merge Shifter Configuration	66

6.6.6.	Recovering after a Photoeye Repair	67

6.7.	Zipper Merge Tick Gapper	67

6.7.1.	Example Accumulation Tick Gapper with Three Zones	68

6.7.2.	Example Accumulation Tick Gapper with One Zone, GoTo Type	69

6.7.3.	ZprTickGapper Table	71

6.7.4.	ZprGapperZone Table	72

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

PE – Photo Eye.  A device used to detect the presence of a container using a beam of light.

SSV – Solenoid Valve.  An output used to activate an actuator.

CPN – A generic term for an AC.  We once called “Automation Controllers” “Control Panels”, hence the “CP”, and the “N” is as in:  “The n-th one”.  Yeah, so now you know

# Merge Control

When two or more sections of conveyor merge into one, precise timing is required to provide a smooth convergence and to maintain the desired spacing between containers.  FortnaPlus supports several merge variants to match hardware configurations for various application requirements.

- Simple Merge

The Simple Merge module is used to control a single merge lane entering a main line.  Both the main line and the merge lane run at a fixed speed.  Each requires a presence PE.  The main line must have sufficient space between containers to allow containers to flow from the merge lane.  Containers are allowed to flow from the merge lane when:

- The main line is running

- The main line presence signal is OFF (Nothing is present on the main line)

- The merge lane presence signal is ON (A container is present on the merge lane)

- The FullClear timer for the main line presence signal has timed out, indicating space is available to accept a container from the merge lane.

Main Line

Merging Lane

Lane Presence PE

Main Line Presence PE

Length of the longest container or tote

Note:  The main line presence PE must be located far enough upstream to allow the largest possible container to feed from the merge lane.

  - SimpleMerge Table

For Simple Merge merges, all configuration is done using a single table:  SimpleMerge (Configuration Menu | Merge Utilities | Simple Merge).

| SimpleMerge Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Define a unique name for the merge. |
| MainLineRun | PopUp | Choose the run signal for the main line. |
| ClearTimer | PopUp | Choose the retentive Full-Clear timer that defines the interval to hold the feed lane once the MainLinePresence sensor goes off. The timer will be created automatically at startup if the Name and LanePresence fields are populated. |
| MainLinePresence | PopUp | Choose the signal that indicates a container is present on the main line. When this signal is ON, and until ClearTimer has expired, the merge lane will hold. This is the PE across the main line. |
| LanePresence | PopUp | Choose the signal that indicates the merge lane has a container present. This is the PE across the merge lane. |
| LaneRun | PopUp | Choose the signal that will be disabled when the main line has a container present. This is the motor for the merge lane. |
| DontFireLaneRun | Y/N | N = The LaneRun output will be fired as well as enabled when started and unfired and disabled when stopped. Y = The LaneRun output will be enabled / disabled only. Use this when the motor will be controlled by motor chains as a GoTo. |
| Machine | PopUp | Choose the AC that will control this entry. |
| Error | Int | Interpret the code number show in this field as follows: 0 – No error detected 1 – Not controlled by this Machine 2 – ClearTimer is not defined 3 – LaneRun is not defined 4 – MainLinePresence is not defined 5 – LanePresence is not defined |
| pRun | Int | Set by the software. 0 = LaneRun is being set to OFF. 1 = LaneRun is being set to ON. Only meaningful if Error = 0 |
| NoGap | Y/N | Determines if containers on the merge lane are allowed to advance to the LanePresence PE or not when the main line is running and has containers present. N = Containers may proceed to the LanePresence PE even if the MainLinePresence PE is blocked or ClearTimer has not timed out. The merge lane will stop when a container arrives at the LanePresence PE and wait for the main line to clear. There must be a gap between containers on the merge lane for this to work effectively. Y = The merge lane may run only when the main line is clear. This is useful if there is no gap between the containers on the merge lane. |
| LaneGoto | Y/N | N = The merge lane will stop when the main line stops. Y = The LanePresence PE will act as a GoTo eye, even if the main line is stopped. This allows the containers on the merge lane to advance to the LanePresence PE regardless of the state of the main line. |

- Multiple Input Lane Merge (New Merge)

The New Merge module can be used with a variety of hardware configurations, such as Two-to-One, Three-to-One, or Three-to-Two layouts.  Configuration tables are used to define the input points, routes, and output signals required.  Each possible route from an input lane to an output lane is explicitly defined.  “Not Busy” routes must be defined.

  - MergeBoss Table

Each New Merge needs a MergeBoss entry (Configuration Menu | Merge Utilities | New Merges | Merge Boss).

| MergeBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name to describe the merge |
| Process | PopUp | Choose the CTRL process to activate this merge |
| Owner | PopUp | Choose the AC that owns this merge |
| CurrInput | Int | Set by the software. Indicates the next input lane that will feed. The input lane must have all the required signals and it must be the highest priority lane (lowest numerical value) to be assigned a route. See MergeInputs_Index. |
| NumInputs | Int | Set by the software at startup. Indicates the number of input lanes associated with this merge. |
| OperableInput | PopUp | Choose the signal that indicates the merge is able to run. This is usually the motor for the merge belt. If the signal is OFF the merge software will disable the MergeInputs_ReleaseIO and hold its timers. |
| Valid | Y/N | Set by the software – only at startup. N indicates input lane configuration errors have been detected. The pStatus field indicates the specific error detected. Y indicates no error has been detected. |
| SwitchDelayTimer | PopUp | Optional. Choose a timer and set SwitchDelayTimerPreset to a non-zero value to limit the rate at which the merge can switch to another input-to-output configuration. This is useful for merge hardware with a large “cost” associated with changing state, such as large plow arms. The merge will hold in the “Waiting Switch” state until the timer expires. |
| SwitchDelayTimerPreset | Float | Preset value for use with SwitchDelayTimer. Enter a time value, in seconds. |
| IsSwitchDelay | Int | Set by the software. Tracks the state of the SwitchDelayTimer. |
| pStatus | String | Used by the software to show the state of the merge logic. |

  - MergeInputs Table

Each input line requires an entry in table MergeInputs (Configuration Menu | Merge Utilities | New Merges | Merge Inputs).

| MergeInputs Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name to describe the input line for this merge |
| MergeBoss | PopUp | Choose the MergeBoss entry associated with this input line |
| Valid | Y/N | Set by the software at startup if Presense, ReleaseIO, ClrTimerName, RunTimerName, FullTimerName and LnClrTimerName and all associated timer preset values are properly defined. The merge will not function until this field displays Y. If N, check the MergeBoss_pStatus column and the CTRL log for error messages. |
| MyTurn | PopUp | Set by the software. Indicates the status of this input lane. - Stopped – The lane is stopped. - Waiting Route – The lane is waiting for a route to become available. Usually, an input lane will be in this state while waiting for another container to complete its route, but it could also be due to a needed route or lane being disabled. - Waiting Release – The lane is waiting for the timer in RlsDelayTimerName to complete. - Running – The lane is running. - Waiting Switch – The lane has completed its run time. Waiting for MergeBoss_SwitchDelayTimer to complete. - Waiting Gap – The lane has completed its run time and switch time, and is waiting for a gap. - Waiting Clear Tm – The lane has completed its run time and is waiting for the ClrTimerName timer to complete before the next lane can start. |
| Index | Int | Set by the software to indicate the input lane number, a value used internally to identify this lane. |
| CalcPriority | Int | Set by the software to determine which lane will feed next. The lane with the lowest numerical value (highest priority) will feed next. The value results from a bitwise AND of the following: 100000 – Route is disabled 010000 – Input lane is disabled 001000 – Item is not present 000100 – Input lane is NOT full 000010 – Set based on the value in the Priority field 000001 – Round-robin priority |
| MergeRoute | PopUp | The route selected by the software. A reference to the MergeRoute table. |
| IsClear | Int | Set by the software to indicate the state of the ClrTimerName timer. |
| RanEnough | Int | Set by the software to indicate the state of the RunTimerName timer. |
| Presense | PopUp | [sic] Choose the presence PE for this input lane. The presence PE is located at the end of the input lane closest to the merge belt. |
| LaneReadyInput1 LaneReadyInput2 | PopUp | These input signals are used to control the ReleaseIO signal. This feature was added in V10.94.0 for use with a vertical switch device. The signal selected for LaneReadyInput1 would be the Position1 SSV for the switch, while LaneReadyInput2 would be the Move Done input. |
| ReleaseIO | PopUp | Choose the output signal that will control the infeed lane. |
| DontFireRelease | Y/N | Determines how ReleaseIO is handled. N = ReleaseIO will be both FIRED and ENABLED to activate and UNFIRED and DISABLED to deactivate. Y = ReleaseIO will use only ENABLE and DISABLE. This setting allows other software modules to start and stop ReleaseIO. |
| Priority | Int | Set all input lanes to zero to give them all the highest (and equal) priority. Values from 1 to 9 will decrease the lane’s priority relative to the others. A lane with a lower priority will only get a chance to run if it goes full or if all the higher-priority lanes are clear. |
| FullClearTimerName | PopUp | Choose a timer to manage critical full conditions on the input lane. If a lane is in a full state, its priority increases. A full lane will not give up its route, except to another full lane. Once the full state clears, normal priorities will resume and other lanes of equal or higher priority can request the route. |
| ClrTimerName | PopUp | Choose the clear timer for this lane. This timer sets the amount of time required for the merge area to clear to avoid collisions and maintain gapping. A timer will be created automatically during startup if one has not been selected. |
| ClrTimerPreset | Float | Enter the time, in seconds, associated with ClrTimerName. |
| RunTimerName | PopUp | Choose the run timer for this lane. This timer determines how long a lane will run once it has a route assigned to it. A timer will be created automatically during startup if one has not been selected. |
| RunTimerPreset | Float | Enter the time, in seconds, associated with RunTimerName. |
| LnClrTimerName | PopUp | Choose the lane clear timer for this lane. This timer determines how long the presence PE must remain unblocked to declare that the lane is clear. Once clear, the lane will stop feeding and a new lane can be selected. A timer will be created automatically during startup if one has not been selected. |
| LnClrTimerPreset | Float | Enter the time, in seconds, associated with LnClrTimerName. |
| RlsDelayTimerName | PopUp | Optional. Choose a timer here (and set a non-zero RlsDelayTimerPreset) to add a delay after the records in table MergeRunOutputs are processed and before containers are released onto the merge. This is intended to be used when the physical mechanism requires time to re-position for a different merge configuration, such as a merge using a plow arm. |
| RlsDelayTimerPreset | Float | Enter the time, in seconds, associated with RlsDelayTimerName. |
| LaneLatch | Int | Set by the software to filter the presence signal. While the merge is running, LaneLatch follows the presence PE state. If a container is detected while the merge is stopping, LaneLatch will remain ON so the merge remembers the container is present when the merge is restarted. Also see LatchOffTimer. |
| InputDisabled | PopUp | If needed, choose a signal to disable the lane. If the lane is disabled while it is feeding it will enter the “Waiting Gap” state and will lose its turn. Use InvertDisable to determine if ON or OFF will indicate a disabled state. |
| InvertDisable | Y/N | See InputDisabled. N = The lane is disabled when InputDisabled is ON Y = The lane is disabled when InputDisabled is OFF |
| LnClear | Int | Set by the software to indicate the LnClearTimerName timer has completed. |
| LatchOffTimer | PopUp | Optional. Choose a timer to hold the LaneLatch flag on longer. This was added to prevent high-speed merges from feeding both input lanes at nearly the same time, which can occur when the ClrTimerPreset is set very short. |
| RunOffTimerFlag | Int | Set by the software to indicate the need to run the LatchOffTimer when the lane gets a turn. If set, the LaneLatch flag will be delayed when the lane gets its next turn. |
| PitchTimerName | PopUp | Optional. Choose a timer to delay the start of the ClrTimerName timer. The timer is started when the lane gets a turn, and a container is detected by the lane presence PE. If the timer completes before the container has cleared the presence PE, there is no effect. If the container clears the presence PE before the timer has completed, the container length is effectively extended until the timer completes. This feature was added for high-speed merge applications, where containers could be interleaved, to force them to a set pitch. |
| PitchFlag | Int | Toggled by the software on the leading edge of a container. Note that the flag will not change state if a container passes while the merge is stopping. |
| RouteDisable | Int | Set by the software to indicate the lane is disabled because it is part of an active route. (MergeRoute_Busy is set, and MergeRoute_OutDisabled is OFF) whose output lane (as defined in MergeRoute_MergeOutputs) is disabled. |
| pLastRunSec | Int | Set by the software to the elapsed whole seconds while the route is running |
| pLastRunMSec | Int | Set by the software to the elapsed fraction of a second, in milliseconds while the route is running. |

  - MergeRoute Table

A MergeRoute entry must be added for each possible route.  A route is defined as a path from an input lane to an output lane.  (Configuration Menu | Merge Utilities | New Merges | Merge Route)

| MergeRoute Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name to describe the route |
| MergeInputs | PopUp | Choose the MergeInputs entry associated with this route. |
| MergeOutputs | PopUp | This is only needed in applications where there are multiple outputs for a merge. In a merge with a single output, MergeBoss_OperableInput is used to stop MergeInputs_ReleaseIO. Choose an IO part that indicates the ON/OFF state of the output lane. If the route is running (RouteBusy = 1) and MergeOutputs goes OFF, MergeInputs_ReleaseIO will be disabled, and the route will stop and wait until the disabled state clears. If MergeOutputs is OFF when a route is being selected, the route will not be chosen. |
| RouteBusy | Int | Set by the software to indicate the status of the route. A non-zero value indicates the route is currently selected to run. |
| RouteRequested | Int | Set by the software to indicate that this route is currently running but is being requested by another input lane. The value is the row number in MergeInputs that is requesting the route. |
| Priority | Int | Set by the software to indicate the highest priority found for this route when searching MergeNotBusy for an available route. |
| OutDisabled | PopUp | If needed, choose a signal to disable this route. If the route is disabled while it is feeding it will enter the “Waiting Gap” state and will lose its turn. Use InvertOutDisable to determine if ON or OFF will indicate a disabled state. |
| InvertOutDisable | Y/N | See OutDisabled. N = The lane is disabled when OutDisabled is ON Y = The lane is disabled when OutDisabled is OFF |

  - MergeNotBusy Table

A MergeNotBusy entry must be added for each route that must not be busy when selecting a route to run.  (Configuration Menu | Merge Utilities | New Merges | Merge Not Busy)

| MergeNotBusy Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a descriptive name for each line |
| MergeRoute | PopUp | Choose the route. The same route may appear on multiple lines if more than one route must not be busy for this route to run. |
| NotBusyRoute | PopUp | Choose the route that must not be busy for the route in MergeRoute to run. To determine the appropriate entries for this table, draw a matrix with each MergeRoute entry shown on both axes. You need and entry in MergeNotBusy for each point in the matrix, except the “identity” (diagonal) items. See the examples later in this section, and the diagram, below for a 3 to 1 merge. If there are routes that can run simultaneously, they can be removed from the matrix. |

Example 3 to 1 merge “not busy” routes:

|  | A | B | C |
| --- | --- | --- | --- |
| A |  |  |  |
| B |  |  |  |
| C |  |  |  |

The routes marked with  need an entry in table MergeNotBusy: A + B, A + C, B + A, B + C, C + A, C + B.

  - MergeRunOutputs Table

Create an entry in the MergeRunOutputs table for each route that requires additional output signals to operate.  The action is performed on these output signals when the route changes state from “Waiting Route” to “Running”.  If MergeInputs_RlsDelayTimerName is defined, the action is performed when the route changes state from “Waiting Route” to “Waiting Release”.  The route will change state to “Running” once the timer completes.  (Configuration Menu | Merge Utilities | New Merges | Merge Run Outputs)

| MergeRunOutputs Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a descriptive name for this item. |
| MergeRoute | PopUp | Choose the merge route associated with this item. |
| Output | PopUp | Choose the output that will be turned ON or OFF |
| Action | PopUp | Choose the action to take: - LatchIO – Latch the Output signal (turn it ON) - UnLatchIO – Unlatch Output (turn it OFF) - DisableLatch – Disable Output - DisableUnLatch – Enable Output (remove the Disable) |

  - MergeStopOutputs Table

Create an entry in the MergeStopOutputs table for each merge route that requires additional outputs to be changed when the merge route is stopped.  The action is performed on these output signals when the route changes state from “Waiting Clear Tm” to “Stopped”.  (Configuration Menu | Merge Utilities | New Merges | Merge Stop Outputs)

| MergeStopOutputs Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a descriptive name for this item. |
| MergeRoute | PopUp | Choose the merge route associated with this item. |
| Output | PopUp | Choose the output that will be turned ON or OFF |
| Action | PopUp | Choose the action to take: - LatchIO – Latch the Output signal (turn it ON) - UnLatchIO – Unlatch Output (turn it OFF) - DisableLatch – Disable Output - DisableUnLatch – Enable Output (remove the Disable) |

  - Three-to-One Merge Example

A common merge configuration.

Lane 1

Lane 2

Lane 3

Route 1

Route 2

Route 3

MergeBoss: One entry for the merge

Merge with three inputs and one output

Output 1

MergeInputs: Three entries, one for each input.

MergeRoute: Three entries:

- Lane 1 to Output 1

- Lane 2 to Output 1

- Lane 3 to Output 1

MergeNotBusy: Six entries:

Route 1 	– Not Route 2

- Not Route 3

Route 2 	– Not Route 1

- Not Route 3

Route 3 	– Not Route 1

- Not Route 2

MergeRunOutputs:

Not Used – Entries are only defined in the ReleaseIO and OperableInput fields.

  - Three-to-Two Merge Example

In this example, with three input and two output lanes, the center input lane can feed either output lane, but input lanes one and three are restricted so they can run only if input lane two is not running.

Merge with three inputs and two outputs

MergeBoss: One entry for the merge

Route 1

Route 2

Route 3

Route 4

Output 1

MergeRoute: Four entries:

- Lane 1 to Output 1

- Lane 2 to Output 1

- Lane 2 to Output 2

- Lane 3 to Output 2

MergeInputs: Three entries, One for each input.

MergeNotBusy: Four entries:

- Route 1 – Not Route 2

- Route 2 – Not Route 1

- Route 3 – Not Route 4

- Route 4 – Not Route 3

MergeRunOutputs:

- Route 1 – Divert Right OFF

- Route 2 – Divert Right ON

- Route 3 – Divert Left ON

- Route 4 – Divert Left OFF

Output 2

Lane 1

Lane 2

Lane 3

  - Two-to-Two Merge Example

This example shows a possible configuration for a two-to-two “plow arm” divert.  The diverter arm is large and heavy, so its motion must be limited to not more than once every 30 seconds.  The arm can assume three positions.  In the middle position, the merge acts as two independent straight-through lines.  The other two positions create a two-to-one merge to one of the output lanes.

Multiple Fullline table entries are used to provide different time intervals.

- Output 1 or Output 2 is full (1 second timeout) – the merge needs to switch to the other output lane.

- Output 1 or Output 2 is full (2 second timeout) – the merge needs to stop and hold (switch did not occur)

Triggers are used to enable / disable the routes:

- If Output 1 is not full and Output 2 is not full – move arm to middle position

- If Output 2 is full and Output 1 is not full – move arm to right position

- If Output 1 is full and Output 2 is not full – move arm to left position

Lane 1 and Lane 2 – Diverter Middle

Lane 1 and Lane 2 – Diverter Right

Route 1

Route 2

Output 1

Output 2

Output 1

Output 2

Lane 1 and Lane 2 – Diverter Left

Output 1

Output 2

Route 3

Route 4

Route 5

Route 6

Lane 1

Lane 2

Lane 1

Lane 2

Lane 1

Lane 2

MergeNotBusy: Twenty eight entries:

Route 1 	– Not Route 3

- Not Route 4

- Not Route 5

- Not Route 6

Route 2 	– Not Route 3

- Not Route 4

- Not Route 5

- Not Route 6

Route 3 	– Not Route 1

- Not Route 2

- Not Route 4

- Not Route 5

- Not Route 6

Route 4 	– Not Route 1

- Not Route 2

- Not Route 3

- Not Route 5

- Not Route 6

Route 5 	– Not Route 1

- Not Route 2

- Not Route 3

- Not Route 4

- Not Route 6

Route 6 	– Not Route 1

- Not Route 2

- Not Route 3

- Not Route 4

- Not Route 5

MergeRunOutputs:

Route 1 	– Divert Arm Left - UnLatch

- Divert Arm Middle - Latch

- Divert Arm Right - UnLatch

Route 2 	– Divert Arm Left - UnLatch

- Divert Arm Middle - Latch

- Divert Arm Right - UnLatch

Route 3 	– Divert Arm Left - UnLatch

- Divert Arm Middle - UnLatch

- Divert Arm Right - Latch

Route 4 	– Divert Arm Left - UnLatch

- Divert Arm Middle - UnLatch

- Divert Arm Right - Latch

Route 5 	– Divert Arm Left - Latch

- Divert Arm Middle - UnLatch

- Divert Arm Right - UnLatch

Route 6 	– Divert Arm Left - Latch

- Divert Arm Middle - UnLatch

- Divert Arm Right - UnLatch

MergeBoss: One entry for the merge

MergeInputs: Two entries, one for each input.

MergeRoute: Six entries:

Lane 1 to Output 1 – Diverter Middle

Lane 2 to Output 2 – Diverter Middle

Lane 1 to Output 1 – Diverter Right

Lane 2 to Output 1 – Diverter Right

Lane 1 to Output 2 – Diverter Left

Lane 2 to Output 2 – Diverter Left

- Sawtooth Merge

The Sawtooth Merge module is designed to support a hardware configuration that uses an encoder and belts to accurately track containers.  The more sophisticated hardware allows more accurate merging using advanced feed algorithms and can handle more merge lanes with high efficiency and high throughput.

The Sawtooth Merge module control logic uses the standard tracking functions (shifter / sorter) along with Sawtooth-Merge-specific logic to provide coordinated and efficient merging for multiple input lanes.

A reservation system is used to allocate time on the merge for containers coming into each input lane.  Reservations can be created and adjusted based on input signals, timers, or rate counters.

The Sawtooth Merge module can fill in unused space with individual containers from available lanes – it does not need to wait to release an entire slug.

Note:  The Sawtooth Merge module must be activated in table Batches.  InternalProcCtrl:  Sawtooth Process, Process:  CTRL.

Saw Merge Menu

Merge 1

Saw Lane Menu

Lane 1

Lane 2

Lane 3

Lane 4

Sorter Configurations Menu

Sorter 1

Induct Points

Lane In 1

Lane In 2

Lane In 3

Lane In 4

Update Points

Approach Update 1
Collision Update 1
Approach Update 2
Collision Update 2
Approach Update 3
Collision Update 3
Approach Update 4
Collision Update 4

Saw Tooth Merge Tables

  - Reservation Configuration Options

The Sawtooth Merge reservation model is based on time slices, which translate into space on (or distance along) the main merge belt that runs at a constant speed.  Each lane has two choices for the duration of its time slice.  There are several options on how to choose the current time slice value, controlled by SawLane_ReserveWhen.  See section 4.7 SawLane Table for details.

The input lane with the oldest update time is allocated first.  Each lane is allocated until all the time slices are consumed.  In the example below, each lane is configured with a different time slice size for illustrative purposes.  Time allocated to each lane in actual use will depend on presence of containers available to merge.  Unused space for the releasing lane will be filled by downstream lanes, if possible.

Lane 1
time slice = 10 sec

Lane 2
time slice = 20 sec

Lane 3
time slice = 30 sec)

Shifter

10

20

30

Allocation Algorithm

(2 iterations)

10

20

30

Note that if the lane that has the current time slice has a gap between containers, downstream lanes may take the opportunity to fill that gap.  This does not cause the current lane to lose its time slice.  See section 4.3 Induct Lane Configuration for more.

  - Configuring the Shifter

Refer to the FPC-Shifter-And-Sorter-Configuration document for details on shifter configuration.

When determining the size for the merge shifter, consider the length of the main merge belt, in ticks, plus additional length to place the logical SawMerge_ReserveIN point prior to the SawLane_ApproachUP point for the most upstream lane, plus some additional space.  It doesn’t hurt to make it longer!

It is useful to think of the critical points on the input lanes as “reflected” onto the main merge belt.  Think of a ruler representing the main merge belt and another ruler representing each input lane.  The end of each lane’s ruler is attached to the main belt at the lane’s SawLane_LaneIN point.  Pivot (reflect) the lane’s ruler about the LaneIN point until it lies along the main belt’s ruler, then transfer the ApproachUP and SawLane_CollisionUP points to the main merge ruler.  The ReserveIN point must be upstream of all the other points.  The zero point of the scale of our main merge belt ruler represents a physical point, but nothing physical exists at that point.  We arbitrarily set the position of the ReserveIN point at 100 ticks on the scale, though there is nothing physical at that point, either.  We use the junction point between the most upstream input lane and the main merge belt as our physical reference point for calculating all other points.  Tick positions for all the other points are set using the merge belt’s ruler, whether they lie along the main merge belt or are “reflected” from the input lanes to that ruler.

Note:  Use of an encoder to track the motion of the merge belt is preferable, but if a timer is used instead of an encoder, the Sawtooth Merge software will automatically start and stop it based on the state of SawMerge_MotorIO.

Note:  The lane presence PE (SawLane_PhotoEyeIO) is expected to change state between each container during normal operation.  This means the PE must be arranged to reliably “see between” each container.

  - Induct Lane Configuration

Each sawtooth lane requires an induct point (SawLane_LaneIN), an approach point (SawLane_ApproachUP), and a collision point (SawLane_CollisionUP) to be defined.  They are used to check for available space, detect when to stop feeding a lane, and to create tracking information for containers as they enter the main merge belt.

Each SawLane_LaneIN point and the SawMerge_ReserveIN point require a memory bit.

Set the ReserveIN point to tick position 100.  Set the first lane’s LaneIN point to 150.  These choices should allow sufficient room to ensure the first lane’s ApproachUP and CollisionUP points fall after the ReserveIN point.  The other points will be adjusted relative to these, which should not be changed.

Measure Input Lane Positions

Measure these points for downstream lanes

First Lane – Reference point,

150 ticks.

To set the initial position of the other SawLane_LaneIN points, measure from the acute angle between the lane and the main merge belt of the first (most upstream) lane to the similar points for each of the other lanes.  Convert the distances to ticks and add the results to 150 – the value for the first lane.  These values will be adjusted later, but they should be reasonably close to the correct value.

Add the Jam PE for the most downstream lane or another PE across the main merge belt downstream of the last lane to an update point.  This will cause a “shift print” to be generated by the SORT process.  We will use these “shift print” pictograms from the SORT log to fine-tune the positions of the various configuration points for each lane.  The “graphical shifter” is also an important tool for observing the operation of the merge and for making adjustments.

  - Induct Lane Tuning

Shifter (Tracking System)

ReserveIN

ApproachUP

CollisionUP

LaneIN

Must be clear to continue to merge

Must be clear for each container to start the merge sequence

Single Input Lane Shift Register Example

When containers are fed onto the merge from a lane, their weight and size, the lane speed and other factors affect when they arrive at the induct point.

First, we will tune the location of the last Jam PE relative to the first lane.  Run containers through the merge from lane 1 and adjust the update point for the last Jam PE (don’t move the lane 1 LaneIN position!) until the shift print consistently shows the nose of the container within two ticks (left or right) of the “X”.  Verify that the placement is consistent over the full range of container sizes and weights – use at least 10 different containers.  Once set, the position of the last Jam PE will not be changed again.

Next, perform similar tests with each of the other lanes, one lane at a time, but adjust the position of the LaneIN update points until the shift prints show the nose of the container within 2 ticks of the “X”.  Use the full range of container sizes.

  - Tuning Between Lanes

Fine tuning with actual containers will be needed to optimize the approach and collision points to generate an acceptable gap for all containers – between individual containers from the same lane and when switching from lane to lane.

The distance from the ApproachUP point to the LaneIN point defines the space needed before any container can merge.  Set this distance, in ticks, to the length of the longest container plus the required gap (typical gaps are 12 to 16 inches).

The minimum required distance from the ApproachUP point to the CollisionUP point is length of the longest container, which allows the container to feed before an approaching container on the merge belt reaches the collision point.

The distance from the CollisionUP point to the LaneIN point defines where the lane will stop feeding to prevent a collision.  This helps protect the merge if containers are being pushed through without a gap or if the lane presence PE is not detecting the end of the containers.  Set the CollisionUP point so a container can merge if the lane motor stops when the container is partially fed and the container is pulled out by the main merge belt – usually, somewhere between a few ticks prior to the LaneIN point and the size of the gap.

- If a lane continues to feed for too long, move the ApproachUP point upstream, but not past the prior lane’s LaneIN point.  The ApproachUP point adjusts the space between an approaching container on the main merge belt and the container fed from this lane.

- If a lane feeds too soon after a container passes on the main merge belt, increase Inpoints_Trailing_Edge_Pad for the lane.  This helps prevent collisions and jams by logically extending the length of each container as it is inducted.

Note:  At the ApproachUP point, the reservation is converted into unused space (on the graphical shifter, the large gray container representing the reservation changes to blue space).  The space may be used by the current lane if a container is present and there is sufficient space, or it may be passed on to a downstream lane.  As containers are inducted, they are shown in white.

  - SawMerge Table

Create an entry for each sawtooth controlled by this AC.

Note:  See the separate FPC_EndOfWaveModule document for details on use of that optional feature for use with Sawtooth Merges.

| SawMerge Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Create a unique name for this sawtooth merge |
| pStatus | String | The software will use this field to display status messages |
| MotorIO | PopUp | Choose the IO part that will indicate if the merge is running or stopped. Usually the motor under the main merge belt. |
| ReserveIN | PopUp | Choose the Inpoint that defines the beginning of the merge. |
| LogMe | Int | Used to control logging of special Sawtooth Merge messages. 0 – Log only errors. 1 – Log warnings and errors 2 – Log warnings, errors and debugging information |
| LaneEnableDelayTM | PopUp | Optional. Choose a timer to hold all lanes while the sawtooth belt is starting. |
| EOW_RelAllPB | PopUp | Choose a pushbutton that can be used to release all lanes. The button will only be effective if all lanes are in an end-of-wave state. |
| EOW_FndAllLt | PopUp | Choose a light that will turn ON when all lanes are in an end-of-wave state. |
| EOW_AutoRel | Y/N | Defines the behavior when all lanes reach the end-of-wave state. N = The wave must be released manually via the End-Of-Wave operator’s screen or the EOW_RelAllPB. Y = The wave will automatically release when the last lane reaches an end-of-wave state. |
| pSliceSeconds | Float | Set by the software with the time, in seconds, for the current reservation. |
| pSliceStartTime | Float | Set by the software to the timestamp at the start of the current reservation cycle. |
| pTotalTimeSlice | Int | Set by the software to the sum of the time slice values for all input lanes in the current reservation cycle. |
| pTotalSlicesRemaining | Int | Set by the software to the total time remaining in the current reservation cycle. |
| pCurrentLane | Int | Set by the software to the currently selected input lane. |

  - SawLane Table

Define an entry for each lane of the sawtooth.

| SawLane Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Create a unique name for each sawtooth merge lane |
| pStatus | String | The software will use this field to display status messages |
| SawMerge | PopUp | Choose the Sawtooth Merge that this lane belongs to. |
| ApproachUP | PopUp | Choose an entry from the Updates table to represent the approach point for this lane. The distance between the induct point (LaneIN) and the approach point must be at least the length of the longest container plus the gap between containers. That much space must be clear before a container can be inducted. |
| CollisionUP | PopUp | Choose an entry from the Updates table to represent the collision point for this lane. Typically set to a few ticks prior to the induct point, this point indicates when to stop feeding to avoid a collision with containers from another lane. This helps protect against push-through containers, gap issues, induct PE problems, and containers that are partially fed when the lane motor stops. |
| LaneIN | PopUp | Choose an entry from the Inpoints table to represent the induct point for this lane. This point defines the end of the approach and collision logical zones on the input lane and is the point at which the container tracking begins on the main merge belt shifter. The location of the induct point is critical, as it defines the position of the container on the shifter so downstream lanes can merge around it. |
| LaneNdx | Int | Enter a unique integer number to identify each lane. Number them in the direction of flow. The number is used in the shifter to identify the lane for space allocation and subsequent merging. |
| PhotoEyeIO | PopUp | Choose the presence PE at the end of the input lane. |
| DisableIO | PopUp | Choose the output signal to disable to stop this lane. |
| DisableBit | Int | Enter the bit to use for the disable of DisableIO. |
| pLastRunTime | Float | Set by the software. The timestamp of the last time this lane was allocated time on the merge. |
| pState | PopUp | Set by the software to show the current state of this lane. IDLE – The normal state of the lane when no containers are present. The software will set all lanes to this state at startup. HOLDING –Configured conditions are not satisfied to allow the lane to run. Containers are present. WAITING – Waiting for open space on the merge shifter between the approach and induct points. Containers are present. RELEASING – The lane is currently discharging. HANGING – The lane was feeding and an approaching container on the merge belt reached the collision point, or the presence PE detected a container, but then went OFF unexpectedly, indicating it has slipped out onto the merge belt even though the lane was not being told to feed. ERROR – An error has occurred in configuration or operation. Check the pStatus field in this table and in SawMerge. Are the approach, collision and/or induct points overlapping? INVALID – If manually set to INVALID, the lane will not run. Set the state to IDLE to reactivate the lane. |
| TimeSlice | Int | Set this to 1. To adjust the allocation for each lane, see SliceSeconds and ReserveSeconds. |
| pSlicesRemaining | Int | Set by the software. This is a vestige left over from an older algorithm scheme, related to TimeSlice. Basically unused. |
| AllowedToRun | Y/N | A flag used to disable or enable the lane. Other application-specific code may control this flag. Set this to Y if nothing else is configured to control it. When set to N while feeding, the lane will stop feeding the next time the lane presence PE transitions OFF. |
| SliceSeconds | Float | The “time slice”, in seconds, for this lane. See ReserveWhen. |
| ReserveWhen | PopUp | ReserveWhen allows a choice between two “time slice” values based on a variety of conditions. Default – Standard reservations. The SliceSeconds value is used. ReserveFlag=Y – The ReserveSeconds value is used when ReserveFLAG is Y. The SliceSeconds value is used otherwise. ReserveFlag=N – The ReserveSeconds value is used when ReserveFLAG is N. The SliceSeconds value is used otherwise. ReserveTM=green – The ReserveSeconds value is used when ReserveTM is ON (green). The SliceSeconds value is used otherwise. ReserveTM=red – The ReserveSeconds value is used when ReserveTM is OFF (red). The SliceSeconds value is used otherwise. ReserveRATE=high – The ReserveSeconds value is used when ReserveRATE is high. The SliceSeconds value is used otherwise. ReserveRATE=low – The ReserveSeconds value is used when ReserveRATE is low. The SliceSeconds value is used otherwise. |
| ReserveSeconds | Float | The “time slice”, in seconds, for this lane when the condition defined in ReserveWhen is met. This value is not used if ReserveWhen = Default. |
| ReserveFLAG | Y/N | Used with the ReserveWhen ReserveFlag options. The state of this value will determine the time slice value to use. Set this field to Y or N using a trigger or other programmatic means. |
| ReserveTM | PopUp | Used with the ReserveWhen ReserveTM options. The completion state of the timer chosen here will determine the time slice value to use. |
| ReserveRATE | PopUp | Used with the ReserveWhen ReserveRATE options. Choose a record from the RateCount table. The measured rate will determine the time slice value to use. |
| MaxSlicesInTransit | Int | Limits the number of reservations that can be “in transit” for a given lane. This is useful on very long merges. A value of zero defeats this feature (no limit). |
| AllowedInput | PopUp | Optional. Choose an IO signal or memory bit to enable / disable the lane and stop feeding reservations. This is often used to halt introduction of new containers if the recirc lane is full. |
| AllowedInvert | Y/N | Invert the sense of the AllowedInput signal. N = An OFF condition will disable the lane. Y = An ON condition will disable the lane. |
| AllowedRatePop | PopUp | Optional. Choose an entry from table RateCount. The lane will be disabled if the rate goes above the HighWater value. |
| pEnabled | Int | Set by the software to indicate when the lane is allowed to feed. |
| EndOfWave_PE | PopUp | Optional. Choose a signal that will indicate an end-of-wave tote has been detected. See the FPC-EndOfWaveModule document for full details on the End-Of-Wave feature. |
| EOW_PB | PopUp | Optional. Choose a pushbutton signal that will manually put the lane into an end-of-wave state, as though an end-of-wave tote had been detected. This will stop the lane. |
| EOW_PB_Toggle | Y/N | Controls the behavior of the EOW_PB device. N = Repeated presses of the EOW_PB pushbutton will NOT toggle the end-of-wave state. The end-of-wave state will latch ON with the first press of the pushbutton. Y = Repeated presses of the EOW_PB will toggle the end-of-wave state, allowing an “undo”. |
| EOW_EnablePB | PopUp | Optional. Choose a pushbutton signal that will manually toggle the enable state of the lane. When disabled, the lane will stop feeding into the merge. |
| EOW_Light | PopUp | Optional. Choose a signal light device that will illuminate when the lane is in an end-of-wave state. |
| EOW_EnableLt | PopUp | Optional. Choose a signal light device that will illuminate when the lane is enabled. |
| EOW_LaneNotReq | Y/N | Optional, for use with the End-Of-Wave module N = The end-of-wave tote must be detected to allow this lane to release. Y = The end-of-wave tote is not required for this lane. |
| EndOfWave_Error | PopUp | Optional. Once the EndOfWave_PE field has been populated for each lane, restart FPC. This field will be automatically populated with an error to warn when a sawtooth lane is in either a disabled or end-of-wave state. The error description (Errors_Description) will be “SawLane <Lane> is Disabled or EOW”, where <Lane> is taken from SawLane_Name. |
| pEOW_State | Int | Set by the software. The value is the internal state number of the End-Of-Wave logic. |
| pEOW_Status | String | Set by the software. A string describing the state of the EndOfWave_PE and lane. This message is displayed on the End-Of-Wave operator’s screen. |
| pEOW_Latch | Int | Set by the software. Used to indicate whether an end-of-wave tote has been detected. |

  - Saw Merge End-Of-Wave Feature

The End-Of-Wave feature is an optional component that expands upon the Sawtooth Merge module’s capabilities.  Special totes with a reflector are used to hold each merge lane until all the containers associated with a wave have been gathered.  The wave can then be manually or automatically released.  An operator’s screen shows the status and allows override control.

See the separate FPC-EndOfWaveModule document for details.

- High-Speed Sawtooth Merge

The High-Speed Sawtooth Merge module requires another increment in hardware complexity over the Sawtooth Merge module, with the payoff of an increase in throughput.

The High-Speed Sawtooth Merge module is often used in combination with the Inch and Store module.  The Inch and Store module builds a slug of containers on each input lane with the optimum gap needed for the merge, then passes that pre-gapped slug on to a separate belt to feed into the merge.  If so designed, a new slug can be built while the previous slug is being released.  The Inch and Store module and the High-Speed Sawtooth Merge module each are described in separate documents.

- Zipper Merge

The Zipper Merge module requires an additional increment to the hardware sophistication to produce higher throughput and precise gapping.

The zipper merge control logic uses the standard shifter / sorter features along with zipper-merge-specific features to produce coordinated and efficient merge action for multiple merge lanes.  Multiple allocation algorithm options allow selections for balanced throughput across all input lanes, or to prioritize specific input lanes in a desired pattern.  Space is allocated on the merge based on the individual container sizes.

  - Zipper Merge Theory and Examples

A zipper merge runs more smoothly and at a higher throughput when used in conjunction with a Capacity Gapper.  A gapper is used on each input lane to control the gap between containers based on instantaneous activity.  The logic controls the gap based on the number of active lanes at any given moment.  If only one or two lanes are active, the gap is decreased to increase throughput.  See an example in the gapping module document.

Two distinct Zipper Merge configurations are possible:  Standard Zippering Merge and Sawtooth Zippering Merge.  Both require that the infeed lanes be designed with the ability to gap containers, measure the length of each container, and control the feed rate of the containers into the merge.  The hardware requirements are the same for both methods.  Both can support a variety of merge configurations, such as 3-to-1 and sawtooth configurations.

The Standard Zippering Merge allocates space on the merge based on detected containers.  If a lane has no containers, a reservation is not created.  The throughput of the merge is constrained by the pre-merge gapping configuration.  Lanes are given turns on a first-come, first-served basis, though a full lane will take priority.  The throughput of an individual lane is not configurable.  If all lanes have containers, each lane gets the same number of turns.  Throughput is controlled by adjusting the gap prior to the meter belt.  The Capacity Gapper module can be used to adjust the gap based on the number of lanes currently feeding.

The Sawtooth Zippering Merge allocates space on the merge based on “Reservation Turns”.  The turns can be adjusted to allocate pre-defined feed rates from each lane.  The feed rates may be adjusted when lanes go full.

  - Zipper Merge Design Constraints – Gapping Requirements

The gapping section prior to the variable-speed belt must provide reliable container gapping.  Improper gapping will cause the variable-speed belt to stop and start between each container.  For smooth zipper merging, design the gapping section as follows:

Applications where reliable gapping requires a Brake-Meter Belt

Variable-Speed Section with encoder

Brake-Meter Belt, Low-Speed Section

Accumulation Section

In-feed Lane

Induct Photoeye

Brake-Meter Belt, High-Speed Section

Gapping Photoeye

Variable-Speed Section with encoder

Accumulation Section

In-feed Lane

Induct Photoeye

Applications where reliable gapping is possible without a Brake-Meter Belt

Gapping Photoeye

    - Servo-Gapper Example Design – 3-to-1 Merge

- Minimum Container Length:  9 inches

- Maximum Container Length:  36 inches

- Design Container Length:  20 inches (1.66 feet)

- Merge Output Gap Required:  12 inches (1 foot)

- Merge Output Rate:  120 containers per minute

- Calculated Belt Speed:  120 * (1.66 + 1) = 319.2 feet per minute.

- Round up to 320 feet per minute or 64 inches per second.

- Merge Input Rate:  Each lane must handle up to ½ the Output Rate.  120 / 2 = 60 containers per minute (1 container per second).

- 64 / 1 = 64 inches per container for the container plus gap.

- Gap on each input lane = 64 inches – 20 inches = 44 inches

- Use the Servo Gapper module on the Brake-Meter belt to properly gap containers onto the zippering belt.  The gapping belt must be able to change speed as needed to produce the required gap.

- Zippering Belt Induct Photoeye:  36 * 0.75 = 27 inches from the beginning of the belt.

- Zippering Belt Length:  27 + 36 + 44 = 107 inches (8.92 feet).  Round up to 10 feet.

  - Standard Zipper Merge

The Standard Zippering merge module creates a reservation on the merge shifter for each container detected at an infeed lane presence eye.  The location of the reservation is determined by the spacing requirements for the pre-merge and merge conveyor segments and the speed ratio between the two segments.  The software controls the speed of the brake-meter belt to move each container into its target location.

The container image on the merge shifter is based on the size of the container, as measured by the infeed lane presence eye, and the “encoder factor”, which is based on the ratio of the encoder settings for the infeed lane and the main merge shifter.

If a pre-merge belt is not required (ZipperLane_PreMergeInduct is set to N/A), the software will run the merge without any pre-merge gapping requirements.  The speed is controlled by several limiting parameters:  ZipperLane_MaxPercent sets the maximum allowed speed
ZipperLane_MinPercent sets the minimum allowed speed
ZipperLane_TargetPercent sets the optimum speed for merging
ZipperLane_AccelMilliSec defines the time required for the VFD to reach full speed from a full stop.
ZipperLane_DecelMilliSec defines the time required for the VFD to stop from full speed.

- Each lane is checked for the first container in need of a reservation by starting at the ZipperLane_MergeUpdate point and working backwards to the ZipperMerge_MergeInduct point.  The first container found in each infeed lane that has not been assigned a reservation will be part of the set of candidate containers for reservation assignment.

- If the merge shifter has space for the container, the reservation will be created.  The target location will be updated with the reservation’s location.

- The target location is calculated by first determining the pre-merge maximum speed using the gap requirements for the pre-merge section and whether pre-merge inducts or scales are configured.  The pre-merge maximum speed, the encoder factors and the speed factors are used to create a minimum target.  Starting at the minimum target, the first available space is located for the container.  The tail of the last container found in the merge shifter (the tail includes both the container and gap requirements) is used as the target location.  The speed is then calculated to hit the next available tick.

- The container image is copied into the merge shifter based on the encoder factor.  The container size is adjusted based on the ratio of the encoder settings.

- If a container is found on an infeed lane and the container cannot be assigned a target location (the merge does not have available space), the speed of the brake-meter belt will be set to the ZipperLane_NoReservPercent value.  If a container on an infeed lane has been assigned a target location, the speed of the lane will be calculated based on the speed differential.

    - Example:  Standard with Brake-Meter Belt

The standard zippering merge is composed of an accumulation section, a brake-meter belt, and a variable-speed section on each infeed lane, plus a shared merging section.

This configuration provides a smooth speed transition when merging multiple lanes to create a single, continuous merged stream.  By using the tick gapper module on the brake-meter belts, most of the required gap is created there, leaving only fine-tuning to the target gap on the variable-speed belt.  This helps reduce the speed changes needed as the containers are introduced to the merge.

The accumulation section gathers containers prior to the brake-meter belt.  The brake-meter belt holds back containers (pushed by the accumulation section), creates a small gap by means of a speed change (for photoeye sensing), and creates the rough gap needed for the merge by releasing the containers onto the variable-speed belt when needed.  The variable-speed belt makes fine adjustments in speed and gap to position the containers at the target location for introduction to the main merge belt.

Induct Photoeye

Variable Speed Section

Infeed Lanes

Accumulation Section

Slave to Variable-Speed Section

Tick Gapper Section

Update Point

Induct Point

Wait Point

Lane 1 Shifter

Container speeding up

Container slowing down

Merge Shifter

Merge Shifter

Merge Shifter

Lane 2 Shifter

Wait Point

Induct Point

Target Location

Update Point

- The Standard Zippering Merge creates reservations only for actual individual containers detected at the infeed lanes.  Each container created on the merge shifter is defined using the following:

- The spacing needed for the pre-merge section (may include pitch control for a scale, etc.).

- Total size allocated = Measured container length + ZipperLane_ResrvGapInches.  ZipperLane_ResrvGapInches is used instead of ZipperLane_MergeGapInches to create additional space for the merging container (especially useful if the brake-meter is slow in responding to speed changes).  ZipperLane_MergeGapInches is used as the container is merged.

- The location of the container is based on the pre-merge gap and the gap required by the merge.  The location and gap are also dependent on whether the lane is running in pitch or gap spacing mode.

- ZipperLane_MergeMinLenInches is used to set the minimum space allocated on the merge for a container.

- The throughput of the merge is limited by the speed of the individual lanes and the number of lanes that have product available.  For a 3-to-1 merge where the infeed lane speeds are 1/3 of the output speed:

  - If only one lane has product, the merge will be limited to 1/3 of the merge throughput.

  - If two lanes have product, the merge will be limited to 2/3 of the merge throughput.

  - If all three lanes have product, the merge can run at full capacity.

- Containers are released from the accumulation section only when the slave to the variable-speed section is running at target speed.  This prevents pressure from the accumulation section when the slave to the variable-speed section is running slower during gapping.

- The slave to the variable-speed section runs slower than the variable-speed section, creating a gap prior to the induct photoeye.

- Containers will move into the induct photoeye, creating a container image on the lane shifter.

    - Example:  Standard with Scale

Like the prior example, but with a scale in place of the variable-speed section.  The scale section serves the same purpose as the variable-speed section.  The scale requires pitch to be controlled instead of gap to ensure only one container passes over the scale at a time to obtain a valid weight reading (ZipperLane_PreMergeGapType = Pitch).  The same comments apply as in the prior example.

Scale

Variable

Speed

Section

Slave to Variable

Speed

Section

Accumulation

Section

In-feed Lanes

Induct Photoeye

Update Point

Update Point

Scale Shifter

Scale Shifter

Lane 1 Shifter

Merge Shifter

Wait Point

Induct Point

Lane 2 Shifter

Wait Point

Induct Point

Target Location

Container speeding up

Container slowing down

  - Sawtooth Zipper Merge

The Sawtooth Zippering merge module creates a reservation on the merge shifter for each infeed lane.  The software controls the variable-speed section to coordinate movement of each container into its target location, which is constrained by the gapping requirements for the pre-merge section and by the reservation in the merge section.

When a reservation is consumed on an infeed lane, the container tracking data is copied to the merge shifter as the container reaches the ZipperLane_MergeUpdate point.  The status of the reservation is set to “Assigned Lane” at that point.

The container image on the merge shifter is based on the size of the container, as measured by the infeed lane presence eye, and the “encoder factor”, which is based on the ratio of the encoder settings for the infeed lane and the main merge shifter.

The speed is controlled by several limiting parameters:  ZipperLane_MaxPercent sets the maximum allowed speed
ZipperLane_MinPercent sets the minimum allowed speed
ZipperLane_TargetPercent sets the optimum speed for merging
ZipperLane_AccelMilliSec defines the time required for the VFD to reach full speed from a full stop.
ZipperLane_DecelMilliSec defines the time required for the VFD to stop from full speed.

In the examples below, the input lane is composed of an accumulation section, a brake-meter belt, a variable-speed section, and an optional scale, all feeding to the main merge belt.

The accumulation section gathers containers prior to the brake-meter belt.

The brake-meter belt holds back containers that are pushed from the accumulation section.  It also pulls a gap, by means of belt speed differences, to ensure the induct photoeye can detect individual containers.  The brake-meter belt is also stopped and started to create the appropriate gap on the variable-speed section.

The variable-speed section makes remaining speed changes needed to create the desired pitch and to ‘slot’ the containers into the reservation target window.  The target is determined by the gap/pitch requirements for both the pre-merge and merge sections.

Reservations are generated on the merge belt:

- Standard Configuration:  Reservations are only created if a container has been measured by the photoeye.  The reservation length will be the measured container length plus the ZipperLane_ResrvGapInches value.

- Sawtooth Configuration:  Reservation length will be ZipperLane_ResrvLenInches plus ZipperLane_ResrvGapInches.

The maximum throughput of the merge is determined by the configuration choices and by the belt speeds.  For a Standard Configuration using ZipperMerge_GapType = Gap, the maximum throughput will vary with the inverse of the container length.

For a Standard Configuration based on ZipperMerge_GapType = Pitch, or for a Sawtooth Configuration, the maximum throughput will be set by the (constant) configuration values mentioned above.

Containers must wait for their assigned reservations before they can pass on to the main merge belt.

The overall throughput of the merge is limited by the availability of containers and their distribution across the input lanes.

Containers are released from the accumulation section only when the slave to the variable-speed section is running at the target speed.  This prevents pressure from the accumulation section from forcing containers out of the desired positions.

A logical image of the container is created on the lane shifter as the container moves through the induct photoeye.

    - Control Logic

- Calculated Values for a Gap (only used if a pre-merge shifter is defined):

  - ZipperLane_pMeterLocation – The location of the container on the variable-speed belt (usually a brake-meter belt).  The current location is calculated by taking the update location minus the nose location of the container.

  - ZipperLane_pGapTicksPreMerge – Calculated from ZipperLane_PreGapInches and converted into ticks associated with the pre-merge belt.

  - ZipperLane_pPreMergeCurrGap – Calculated by converting the current location to a value in ticks, (adjusted by the pre-merge belt encoder ratio) and adding the distance in ticks from the induction point (on the pre-merge belt) to the first container.

  - ZipperLane_pPreMergeError – The error calculated from the difference between the gap required and the current gap.

  - ZipperLane_pPreMergeMaxSpeed – The pre-merge maximum speed is calculated using the current location and the pre-merge error.  If the gap is larger than needed, this will indicate the maximum speed allowed to prevent loss of the correct gap.  If the gap is smaller than needed, this will indicate the speed needed to create the correct gap.

- Calculated Values for Merging:

  - ZipperLane_pMergeLocation – The location of the container on the variable-speed belt (usually, a brake-meter belt).  The current location is calculated by taking the update location minus the nose location of the container.

  - ZipperLane_pSpeed1 – If the speed of the brake-meter belt is less than the target speed, the target location is adjusted to correct for errors caused by the belt running slower than the target speed.

  - ZipperLane_pMergeTargetLoc – Calculated by taking the reflected update location times the encoder ratio and the speed ratio.  The location is then adjusted by the adding in the speed adjustment.  The result is then used to define the window where a reservation may be created.  If a reservation already exists in the defined window, the reservation is used as the target location.  If a reservation does not exist in the target area, one will be created when there is space available.

  - ZipperLane_pMergeError – Calculated by subtracting the current location from the target location.  If the target location is less than the current location, the merge error will be negative, and the speed may be greater than the target speed but will be less than ZipperLane_pPreMergeMaxSpeed.

  - ZipperLane_pStopTicks – Defines a window of ticks prior to the update point.  When the container is in this window and the merge error is larger than ZipperLane_MaxErrorTicks, the speed will be limited to the speed needed to stop at the update point.  If the merge error is less than or equal to ZipperLane_MaxErrorTicks, the speed will be updated to the target speed (adjusted for any error).

  - ZipperLane_pSpeed3 – Calculated by dividing the current position by the (“Current Position” + “Error”) and multiplying by the target speed.  If the brake-meter belt speed is greater than the maximum speed or less than the minimum speed, the speed will be set to the limiting value.  In addition, the speed will be limited by the acceleration and deceleration parameters.

- If the prior state was “Target Speed”, “Adjusting Speed” or “N/A”, the software will search the lane shifter to determine the new state of the infeed lane as follows:

  - If no containers are found between the induct point and the update Point, the lane state is set to “Target Speed”.

  - If the nose of a container is found between the induct point and the ZIpperLane_pStopTicks point, calculate the current position, current error, and the target point error.  The state is set to “Adjusting Speed”.

  - If the nose of a container is found in the ZIpperLane_pStopTicks window before the update point, calculate the current position, current error, and the target point error.  If the error is greater than zero, the speed is adjusted.  If the container is in the ZIpperLane_pStopTicks window and the error is greater than ZipperLane_MaxErrorTicks, the speed will be reduced to the calculated speed needed to stop at the update point.

  - If the nose of the container has reached the update point, and the error is less than or equal to ZipperLane_MaxErrorTicks, the container will merge.  The state is set to “Target Speed”.

- If the prior state was “Stopped”, the software will search the lane shifter and will determine a new state of the infeed lane as follows:

  - If no containers are found between the induct point and the update point, set the lane state to “Target Speed”.

  - Calculate the current position, current error, and the target point error.

    - If the nose of the container has reached the update point and the error is greater than ZipperLane_MaxErrorTicks, set the state to “Stopped”.

    - If the error is less than ZipperLane_MaxErrorTicks, set the state to “Target speed”.

    - Reservation Logic

The zippering sawtooth reservation logic is based on a “turns” system.  Each lane is assigned a number of turns, defined by ZipperLane_Turns (or ZipperLane_FullTurns if the lane is full), for each cycle of the algorithm.  All the turns for the merge are summed to create a cycle total.  A lane is given a turn after the number of ZipperLane_SkipTurns is satisfied.  See the full definitions for the parameters in the ZipperMerge and ZipperLane table descriptions, section 6.5.1 and 6.5.2.

Example:  The following parameters will feed two containers from Lane 1, one container from Lane 2, and one container from Lane 3.  Setting SkipTurns = 1 prevents two consecutive containers from feeding from Lane 1.

- Lane 1:  Turns = 2, SkipTurns = 1

- Lane 2:  Turns = 1, SkipTurns = 1

- Lane 3:  Turns = 1, SkipTurns = 1

The next lane to get a reservation is determined by finding the lane that has the largest number of reservations required (though the skip count must also be 0).

If ZipperMerge_pCurrentCycle is less than ZipperMerge_pTotalTurns and none of the lanes can run, a new cycle is started.

If ZipperMerge_pCurrentCycle is equal to or greater than ZipperMerge_pTotalTurns a new cycle is started.

    - Enhanced Throughput Logic

The software increases overall throughput by taking advantage of unused space if a container will fit.  For instance, if there is a gap between containers arriving at an upstream infeed lane, and a container is waiting at a downstream lane, the downstream container will feed into the space in the upstream lane’s reservation.

A similar use of available space will occur in the Sawtooth configuration if a container that is shorter than the maximum arrives a little late (the reservation has already begun to move past the reflected update point).  It can still feed using the remainder of the reservation if it fits.

Similarly, if two small containers arrive that, with gap, will fit into the space required for the largest container, they can both feed.  This is true even if the two small containers arrive on two different infeed lanes – the first will use the beginning portion of the reservation, and the second will fill in the remainder.

This enhanced logic can be controlled, to some degree, with the ZipperLane_ReAssignLane feature.  Setting this value to the index number (within table ZipperLane) of a specific lane causes unused reservation space to be assigned to that lane.  If set to zero, the space will be available for any lane to use.  Any unused ticks are freed for use at the reflected update location.

    - Example With Scale

A scale is inserted between the variable-speed section and the merge slave section.  Scales usually require use of ZipperMerge_GapType = Pitch.

Lane Shifter

Pre-Merge Shifter (Scale)

Merge Shifter

Wait Point

Induct Point

Reflected Update Point

Scale

Variable Speed Section

Brake-Meter Belt

Accumulation Section

Reservations

Merge Slave Section

Merge Conveyor

Infeed Lane

Reflected Update Point

Induct Photoeye

Update Point

Sawtooth Zippering Merge Example - Scale in front of the Merge

Three shifters are required.

Gapping Photoeye

    - Example Without Scale

The variable-speed section connects directly to the merge slave section.

Lane Shifter

Merge Shifter

Wait Point

Induct Point

Variable Speed Section

Brake-Meter Belt

Accumulation Section

Reservations

Merge Slave Section

Merge Conveyor

Infeed Lane

Reflected Update Point

Induct Photoeye

Update Point

Sawtooth Zippering Merge Example - No Scale

Update Point

Gapping Photoeye

Reflected Update Point

Two shifters are required.

  - Zipper Merge Tables

This section describes the tables required to configure a zipper merge.

    - ZipperMerge Table

Each zipper merge needs a ZipperMerge entry (Configuration Menu | Merge Utilities | Zipper Menu | Zipper Merge).

| ZipperMerge Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this merge. |
| CreateResrvFlag | Y/N | Enable or disable the creation of reservations. During setup and testing, you can set this to N to hold containers on all infeed lanes while you adjust the update points for the brake-meter belts. Set to Y for normal operation. |
| pStatus | String | The program will display errors or events here. |
| MergeType | PopUp | Choose the merge type: Standard – Choose this if the application does not require control of individual lane feed rates. A reservation is created for each container as it arrives. Containers are fed on a first-come-first-served basis. Can be used for 3-to-1 or sawtooth physical configurations. Sawtooth – Choose this if the application requires control of each lane by creating reservations for each infeed lane. |
| LogLevel | PopUp | Control the amount of information written to the log files. N/A – Only errors are logged Level 2 – All messages are generated Level 5 – At initialization, a table of speed adjustments is written to the logs |
| MergeInduct | PopUp | Choose an induct point for the main merge shifter. The area from the induct point to the reflected lane update point is used to create reservations. |
| MergeMtr | PopUp | NOT USED. In older versions, this was used to choose a signal that would start and stop the merge. |
| MergeMtrAction | PopUp | NOT USED |
| MtrBit | Int | NOT USED |
| ControlMachine | PopUp | Choose the machine that controls this merge. |
| MergeSpdFtPerMin | Float | Measure the main merge belt speed and enter the value here. It must be accurate – use a tachometer! The software uses this speed along with the speed of each lane to calculate the target location for the container. It also uses the information to control the brake-meter belt to position the container as it is merged. |
| ErrorSignal | PopUp | NOT USED. In earlier versions, this was used to choose an error input signal that would stop the merge. |
| MergeTicksPerInch | Float | Enter the ticks-per-inch value for the merge encoder. |
| GapType | PopUp | Choose the gap type for the merge. The merge gap and the pre-merge gap may be two different types. Gap – The spacing is adjusted to create a gap of a specific size between containers. Pitch – The spacing is adjusted to create a specific distance from the leading edge of one container to the leading edge of the next. The pre-merge gapper often requires Pitch type, whereas the merge gapper often requires Gap type. If a scale is used, Pitch type is usually needed. |
| MergeRunIO | PopUp | Choose an IO part that shows when the merge belt is running or stopped – usually, the motor for the merge belt. |
| LaneMergeDlyTM | PopUp | Choose a timer to delay release of the infeed lanes when the merge belt starts (when the MergeRunIO signal comes on) to allow the belt to get up to speed before merging begins. |
| MergeJamUpdate | PopUp | Choose the update point associated with the merge jam photoeye. The ZipperErrLog table will be updated with tick position error information that is useful while configuring the merge. The update point will also cause shift print images to be written to the SORT log file when the container reaches the photoeye. |
| pLastEncCnt | Int | Set by the software. The last encoder increment read from the shifter. |
| pTotalTurns | Int | Set by the software. The total number of turns calculated for the merge for the current complete cycle of the infeed lanes. Recalculated on each cycle to account for full lanes. |
| pCurrentCycle | Int | Set by the software. The turns consumed so far this cycle. The cycle is complete when this value counts up to pTotalTurns. |
| pCurrResrvLane | Int | Set by the software. The lane that is currently generating a reservation. |
| pCurrResrvLen | Int | Set by the software. The length remaining to be generated in the current reservation. |
| pLastLaneAssgn | Int | Set by the software. The lane number of the last lane assigned. |
| pMergeJamState | Int | Set by the software. The state of the jam photoeye referenced in the MergeJamUpdate update point. |

    - ZipperLane Table

| ZipperLane Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for each merge lane. |
| Enabled | Radio | Use this to manually enable or disable a lane. |
| EnableIO | PopUp | Choose an IO part that will enable or disable the lane under program control. Works with InvrtEnableIO to change the sense of the signal. |
| InvrtEnableIO | Y/N | Works with EnableIO. N = The lane is enabled when EnableIO is ON Y = The lane is enabled when EnableIO is OFF |
| pStatus1 | String | The software will report lane errors or events in this field. |
| pStatus2 | String | The software will report lane errors or events in this field. |
| pAccumState | PopUp | Set by the software to show the current state of the lane accumulator: Holding – Containers are being held for gapping. Feeding – Containers are being released onto the brake-meter belt. |
| pLaneState | PopUp | Set by the software to show the current control state of the lane: Stopped – The lane is stopped waiting for available space. Adjusting Speed – The lane speed is being changed to try to place the container in the target location on the merge belt. Target Speed – The lane speed has reached the target speed for merging, or to wait for the next container. This field is not updated if the lane motor is stopped or if the lane is disabled. |
| ZipperMerge | PopUp | Choose the merge that this lane is a part of from the ZipperMerge table. |
| Turns | Int | If ZipperMerge_MergeType = Sawtooth, set the number of turns to allocate to this lane for each throughput cycle. |
| SkipTurns | Int | If ZipperMerge_MergeType = Sawtooth, set the number of turns to skip between Turns before assigning this lane again. |
| FullTurns | Int | If ZipperMerge_MergeType = Sawtooth, set the number of turns to allocate (instead of Turns) if the lane is full. FullClearTimer must be defined. |
| MeterTicksPerInch | Float | The setting, in ticks per inch, for the brake-meter shifter encoder. |
| PreTicksPerInch | Float | The setting, in ticks per inch, for the pre-merge shifter encoder. |
| ResrvLenInches | Float | If ZipperMerge_MergeType = Sawtooth, the length of the longest container, in inches, for this lane. |
| ResrvGapInches | Float | The length of the reservation gap or pitch, in inches. This value is combined with the length of the container to allocate the target space for a container on the merge and is typically set larger than MergeGapInches to allow for the response time of the belts. The gap should be large enough to accommodate the longest container even when it is skewed. The spacing will be calculated based on the ZipperMerge_MergeType setting. This value is used for both Standard and Sawtooth types. The extra gap is especially useful if the brake-meter belt is slow to respond to speed changes, and it is difficult to hit the target space allocated on the main merge shifter. |
| MergeGapInches | Float | The length of the gap or pitch, in inches. This value is combined with the actual container length to determine the space required on the main merge shifter as the container merges. The container will not merge if the space on the main merge shifter is not large enough to accommodate the container plus this value. |
| MergeMinLenInches | Float | If the container’s measured length, in inches, is less than this value, this value will be used in its place. |
| PreMergeGapType | PopUp | Choose the gap type to be used for the pre-merge (or scale) section on this lane: Gap – Consider the space between containers. (The gap between containers will be made constant) Pitch – Consider the distance from leading edge of one container to the next container. (The container + gap will be made constant) |
| PreGapInches | Float | Gap or pitch size, in inches, (depending on the PreMergeGapType setting) for the pre-merge section. |
| StopInches | Float | The distance, in inches, prior to the MergeUpdate point where deceleration will begin when the lane needs to stop and wait for a gap. |
| AccelMilliSec | Float | Set the time, in milliseconds, required for the VFD to increase the belt speed from zero to full speed. This value should match the VFD setting. Default is 250 mSec. |
| DecelMilliSec | Float | Set the time, in milliseconds, required for the VFD to decrease the belt speed from full speed to zero. This value should match the VFD setting. Default is 250 mSec. |
| MinPercent | Float | Minimum speed for the brake-meter belt, in percent. Set this to a value that will not stall the belt when fully loaded. |
| TargetPercent | Float | The speed, in percent, for the brake-meter belt that matches the speed of the pre-merge (or scale) section. The brake-meter belt reaches this speed as the container transitions to the pre-merge section at the BrakeMeterUpdate point. This value must be less than or equal to MaxPercent. |
| MaxPercent | Float | Maximum speed for the brake-meter belt, in percent. Usually, 100%. The VFD should be configured so the speed is close to 100% when matching the main merge belt speed. |
| NoResrvPercent | Float | The speed, in percent, for the lane to use when it does not have a reservation. For merges where the lanes are close together, the throughput may be increased by quickly moving containers to the BrakeMeterUpdate point, so set this to 100% initially. When at the update point, containers may be close enough to take advantage of unused reservation space from upstream lanes. |
| MeterSpdFtPerMin | Float | Enter the measured speed of the lane when running at MaxPercent. The speed must be accurate as it is used in conjunction with ZipperMerge_MergeSpdFtPerMin to calculate the merge point. Measure it with a tachometer when the lane is empty! |
| BrakeMeterMtr | PopUp | Choose the signal that indicates if the brake-meter belt is stopped or running. If this signal is selected and it is OFF, the speed for the lane will be set to zero and reservations will not be created for the lane and containers will not merge (logically). NOTE: If you disable the lane when the speed is set to zero, you will create a situation where the lane cannot run – when the input is OFF, the speed is set to zero <-> when the speed is set to zero, the lane is disabled |
| AccumIO | PopUp | Choose the signal that will start and stop the accumulator infeed. This signal will be turned ON if pSpeed3 indicates the brake-meter belt is running at the target speed. This signal will be turned OFF if the brake-meter belt slows down. |
| StopAction | PopUp | Choose the action to take on AccumIO. |
| AccumBit | Int | Set the bit to use if StopAction is Enable or Disable. |
| InductStuckErr | PopUp | Choose the signal (probably a memory bit) to be activated when the induct PE (PreMergeInduct) fails to change state. The induct PE is located on the brake-meter belt. This error will be triggered if a continuous logical container is generated that extends from the induct PE to the BrakeMeterUpdate point (end of belt). The error will clear when the induct PE signal goes OFF. |
| StuckAction | PopUp | Choose the action to take on InductStuckErr. |
| StuckBit | Int | Set the bit to use if StuckAction is Enable or Disable. |
| FullClearTimer | PopUp | Choose a full timer for the lane. When using the Sawtooth logic, if the lane goes full, FullTurns is used instead of Turns to allocate reservations. When using the Standard logic, the full condition determines the priority for assignment, which will only have an effect if the available space can accommodate two containers or less. |
| LaneMergeDlyTM | PopUp | Choose a timer to hold the lane for the defined time after a jam or estop. The timer action is keyed to the state of ZipperMerge_MergeRunIO. |
| BrakeMeterUpdate | PopUp | Choose an update point that defines the update location for the brake-meter belt. This sets the logical point at which the brake-meter belt ends, and should correspond with the physical end of the belt. |
| PreMergeInduct | PopUp | Choose an induct point for the pre-merge shifter. The pre-merge shifter is used by the software to determine the gap or pitch for the container when a scale is used on the infeed. This shifter requires an encoder that indicates the speed of the scale. Do not use a real photoeye for this shifter, as the software inducts containers onto the shifter automatically – use a memory bit. This is optional for the Standard Zipper Merge configuration. |
| MergeUpdate | PopUp | Choose an update point to define the reflected induct location on the merge shifter. When creating the merge shifter, allow enough room so this point for the first lane can be located at twice the distance from the brake-meter induct point to the BrakeMeterUpdate point. This allows space for multiple reservations in transit for the lane in Sawtooth mode. For Standard mode, the distance between the merge induct point and the merge update point (all lanes will typically be close together) is usually set to a value large enough to allow a reservation for each lane to be created when a container is detected, however, to give priority to a full lane the distance should be limited to the size of the largest container. |
| SpdControl | PopUp | Choose an entry from table SpdControl that defines how the brake-meter belt speed is controlled. |
| UseSpdFeedback | Y/N | Y = Use the speed returned from the VFD. An entry is required in table SpdControl with a valid SpdControl_IO_Addr_In. N= Use the speed calculated from the configured AccelMilliSec and DecelMilliSec values. |
| Gain | Float | Set a multiplier to be applied to the calculated speed. Typical values are 0.8 to 0.9 for additional damping, if needed. Use caution if setting this value larger than 1.0. |
| SpdFactor | Float | Set a multiplier for the speed error calculation. Example: If containers are hitting the reservation in the shift print properly when the speed is 0% or 100%, but not for 20%, set SpdFactor = 0.9. This will reduce the speed used in the error calculation and increase the error correction distance. |
| MaxErrorTicks | Int | Define the maximum allowable error, in ticks. When a container reaches the update point, if the calculated error exceeds this value the lane stops to wait for additional space. |
| SlowLenAdjTicks | Int | Set the number of ticks to add to the merge target location when the lane is running slowly or is stopped. This helps compensate for sluggish brake-meter speed adjustments. |
| AdjustForDrift | Y/N | Drift correction is used to adjust the target merge point based on the distance, in ticks, the container moves past the BrakeMeterUpdate point when the container has been stopped. N = Disable drift correction. Y = Enable drift correction. NOTE: The merge must be re-timed if this setting is changed! |
| MaxAdjustDrift | Int | Set this value, in ticks, to the limit for drift correction adjustments. If a container drifts past the BrakeMeterUpdate point by more than this amount, the correction will be limited to this value. The default is 24 ticks. |
| ReAssignLane | Int | Used to give unused reservation space to a specific lane. Set to the index number of the lane to use for reservation reassignment. If set to 0, the reservation space is available to the first lane that can use it. |
| AccelRate | Float | Set by the software in the current version. See AccelMilliSec. In older versions, set this to 100 * (SORT process cycle time in milliseconds) / (desired acceleration time in milliseconds) |
| DecelRate | Float | Set by the software in the current version. See DecelMilliSec. In older versions, set this to 100 * (SORT process cycle time in milliseconds) / (desired deceleration time in milliseconds) |
| pMeterLocation | Float | The software sets this to the current tick location of the brake-meter belt shifter. |
| pMergeLocation | Float | The software sets this to the current target location on the merge belt, if assigned. |
| pMeterBoxLength | Float | The software sets this to the container length measured by the brake-meter belt. |
| pPreMergeCurrGap | Float | The software sets this to the current gap on the pre-merge. |
| pPreMergeMaxSpeed | Float | The software sets this to the calculated maximum speed for the pre-merge belt based on the gapping requirements. |
| pMergeRsvLenReq | Float | The software sets this to the reservation length required for the container that is merging. |
| pMergeRsvLenFnd | Float | The software sets this to the reservation length found on the merge shifter. |
| pMergeTargetLoc | Float | Set by the software. When searching for a reservation, the software will look upstream of this point. |
| pMergeReservLoc | Float | Set by the software. The location on the merge belt of the first available reservation that can be used by this lane. |
| pMergeMaxSpeed | Float | The software sets this to the calculated maximum speed for the brake-meter belt based on the speed constraints for the pre-merge gap and the reservation. |
| pPreMergeErr | Float | The software sets this to the pre-merge target error for the current container based on the pre-merge gap requirements. |
| pMergeError | Float | The software sets this to the merge target error for the current container based on the assigned reservation and the reflected location on the merge belt. Use this to determine how closely the reservation tracks the logical container in the lane shifter. |
| pSpeed1 | Float | The software sets this to the speed calculated from the error and the distance to the target with the minimum and maximum speeds applied. |
| pSpeed2 | Float | The software sets this to the speed calculated after the motor run state is considered. |
| pSpeed3 | Float | The software sets this to the speed calculated after adjusting for acceleration and deceleration requirements are considered. This is the speed that is actually set. |
| pSpdFdBack | Float | The software sets this to the speed returned from the VFD, as configured in the SpdControl table. |
| pStopTicks | Int | The software sets this to the number of ticks calculated prior to the update point where deceleration would need to begin if the lane must be stopped. |
| pLastMeterIndx | Int | The software sets this to the index in table SortData for the last container processed on the brake-meter shifter. The value is used to copy the container image to the reservation on the merge shifter. |
| pReservLenTicks | Int | The software sets this to the length of the reservation, in ticks. |
| pReservGapTicks | Int | The software sets this to the length of the reservation gap, in ticks. |
| pEncRatioMerge | Float | The software sets this to the calculated encoder ratio for the merge shifter. |
| pEncRatioPreMerge | Float | The software sets this to the calculated encoder ratio for the pre-merge shifter. |
| pSpeedRatio | Float | The software sets this to the calculated ratio of the speeds of the brake-meter and main merge shifters |
| pCurrentTtl | Int | Set by the software. Each time a reservation is assigned to the merge shifter the total number of turns assigned to the lane is updated. The total may change if lane is configured to get additional turns when full. |
| pCurrentTurn | Int | The software sets this to the number of turns this lane has been assigned for the current throughput cycle. |
| pCurrentSkip | Int | The software sets this to the remaining number of turns to skip for this throughput cycle before assigning the next turn for this lane. The value is decremented as turns are skipped. It is preserved across throughput cycles. The lane will not get a turn until all required skips have been consumed. |
| pLastEncPre | Int | The software sets this to the last encoder count read from the pre-merge shifter. |
| pLastEncMeter | Int | The software sets this to the last encoder count read from the brake-meter shifter. |
| pGapTicksPreMerge | Float | The software sets this to the calculated number of ticks needed for the pre-merge gap. |
| pRatioCombined | Float | The software sets this to the combined merge encoder ratio and brake-meter to merge speed ratio. |
| pGapTicksMerge | Int | The software sets this to the calculated number of gap ticks included in the container length as it merges. |
| pMinLenMerge | Int | The software sets this to the minimum length value converted to ticks. |
| pLanePriority | Int | Used for Standard Merge mode. The software sets this to the current priority of the lane. A lower value indicates a higher priority. |
| pCreateRsvTarget | Int | Used for Standard Merge mode. The software sets this to the calculated target starting location where a reservation could be assigned for the lane if the space is not already reserved. |
| pCreateRsvLenReq | Int | Used for Standard Merge mode. The software sets this to the calculated length of the reservation needed. |
| pCreateRsvFndTrg | Int | Used for Standard Merge mode. The software sets this to the calculated target after considering reservation space already allocated to other lanes. |
| pCreateRsvLenFnd | Int | Used for Standard Merge mode. The software sets this to the calculated length of the available unused reservation space found. |
| pSqrRootAdjTicks | Float | The software sets this to the square root of the SlowLenAdjTicks value. The error adjustment is interpolated and squared to create an exponential error adjustment to correct for sluggish acceleration on merge lanes. |
| pSqrRootStopTicks | Float | The software sets this to the square root of pStopTicks. The value is use for linear interpolation to improve the accuracy when stopping the lane. |
| pSqrRootDistTicks | Float | The software sets this to the square root of the available distance when stopping the lane. The value is used for linear interpolation to improve the accuracy when stopping the lane. |
| pLastMergeNdx | Int | The software sets this to the last container Ndx value processed. |
| pLastMergeSpd | Int | The software sets this to the speed of the brake-meter belt for the last container merged. |
| pLastAdjTicks | Int | The software sets this to the adjustment ticks calculated from the speed. |
| pDriftAdjust | Int | The software sets this to the amount of drift from the last container to pass the BrakeMeterUpdate point. |
| pInductPEStuck | Int | The software sets this flag to indicate the lane is in a “stuck” state. The lane must be enabled. See InductStuckErr. |
| pLastInvSpd | Int | Used by the software to calculate lane update point corrections based on SpdFactor and the UseSpdFeedBack setting. |

    - Zipper Speed Control Settings

The speeds of the brake-meter belts are controlled by configuring entries in table SpdControl.

| SpdControl Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | A unique descriptive name for the entry. |
| pError | String | Set by the software to display error messages for this entry. |
| EnableFlag | Y/N | Use this to manually enable or disable this entry. |
| Maint_IO | PopUp | Choose an IO signal that will activate maintenance mode. ON = The motor speed is set from OnPercent. OFF = The motor speed is set from OffPercent. NOTE: The Zipper Merge module alters the value in OffPercent. A signal defined here will override the zipper merge, servo gapper, print apply, or other software modules. Leave this field set to INVALID for use with these modules. |
| IO_Owner | PopUp | Choose the process that will control the IO for this motor. If the bits are configured in the Conveyor table, select the same process here. If they are not defined in the Conveyor table, use the SORT process. |
| OffPercent | Float | Set the motor speed to use, as percent of maximum (0 to 100) when Maint_IO is OFF or undefined. The value will be converted to a bit pattern and sent to the VFD. NOTE: The Zipper Merge module alters this value if ZipperLane_SpdControl is linked to this entry! |
| OnPercent | Float | Set the motor speed to use, as percent of maximum (0 to 100) when Motor_IO is ON. The value will be converted to a bit pattern and sent to the VFD. |
| MatchRecord | Int | To match the speed of another conveyor section, enter the row number of the master section here (the row in this table). The OffPercent value of the master row will be copied to this row. |
| MatchPercent | Float | When using MatchRecord, you can adjust the slave section speed using this entry. You can use this to create a gap between sections. The OffPercent value from the master section will be multiplied by the percent value entered here. Values over 100 are acceptable if you want this section to run faster than the master section. A value of 0.0 defeats this feature. |
| MatchWhenMaintOn | Y/N | Use the Maint_IO input to control speed matching. N = Set the speed to OnPercent when Maint_IO is ON. Y = Use MatchRecord and MatchPercent when Maint_IO is ON. |
| IO_Addr_Out | Int | Enter the output IO address word assigned to the device. NOTE: Make sure the numeric base matches the setting for Configio_Octal_Word or convert accordingly! |
| IO_Addr_In | Int | Enter the input IO address word assigned to the device. NOTE: Make sure the numeric base matches the setting for Configio_Octal_Word or convert accordingly! |
| MaxSpeed | Binary | Set this to the binary pattern that defines the maximum speed value for the VFD. Most VFDs use the lower 15 bits: 0111111111111111. |
| PrintLine | PopUp | If this entry is associated with a print apply line, select it here. |
| MtrStartIO | PopUp | Choose an output signal here, if needed. The signal will be set ON if the speed is greater than zero. |
| StartAction | PopUp | Choose the action to take on MtrStartIO. |
| StartBit | Int | If StartAction is Enable or Disable, define the bit to use. |
| MtrStopIO | PopUp | Choose an output signal here, if needed. The signal will be set OFF if the speed is zero. This is used to disable a motor if it creeps when enabled and set to zero speed. |
| StopAction | PopUp | Choose an action to take on MtrStopIO. |
| StopBit | Int | If StopAction is Enable or Disable, define the bit to use. |
| VFD_NetCTRL | PopUp | Choose a signal, if needed, to activate when EnableFlag = Y. Use this if the VFD requires a signal to put it into network control mode. |
| NetCTRLAction | PopUp | Choose an action to take on VFD_NetCTRL. |
| NetCTRLBit | Int | If NetCTRLAction is Enable or Disable, define the bit to use. |
| VFD_NetREF | PopUp | Choose a signal, if needed, to activate when EnableFlag = Y. Use this if the VFD requires a signal to put it into network control mode. |
| NetREFAction | PopUp | Choose an action to take on VFD_NetREF. |
| NetREFBit | Int | If NetREFAction is Enable or Disable, define the bit to use. |
| VFD_RstTimer | PopUp | Choose a timer to control the pulse duration for the VFD_Reset signal. The VFD reset signal will be set ON when the timer is timing. Select the timer from the Mtrchains table for this motor. This is used to reset a network drive whenever the system is restarted. |
| VFD_Reset | PopUp | Choose a signal, if needed, to activate when VFD_RstTimer is used. This is used in applications where the VFD must be reset each time it is restarted. |
| ResetAction | PopUp | Choose an action to take on VFD_Reset. |
| ResetBit | Int | If ResetAction is Enable or Disable, define the bit to use. |
| VFD_MIN_Prcnt | Float | Used with VFD_Aux. Enter a percent speed value (0 to 100) to define the speed at which VFD_Aux will turn ON. |
| VFD_Aux | PopUp | Choose an IO output that will activate when the speed is at or above VFD_MIN_Prcnt, indicating the motor is running. |
| AuxAction | PopUp | Choose an action to take on VFD_Aux. |
| AuxBit | Int | If AuxAction is Enable or Disable, define the bit to use. |
| pActualPcnt | Int | Set by the software using IO_Addr_In. The binary pattern is converted to a speed percent value that is used with VFD_MIN_Prcnt. |
| pBinOut | Binary | Set by the software to the binary bit pattern sent to the IO_Addr_Out speed control word of the drive. |
| pBinIn | Binary | Set by the software to the binary bit pattern received from the IO_Addr_In speed control word of the drive. |

See the separate document on SpdControl for additional details.

    - ServoGapper Table

See the Gapping Modules document for details on configuration of a servo gapper using the ServoGapper table.  Example configurations are given.

    - ZprSimConfig Table

The ZprSimConfig table can be used to test the Zipper Merge software.  It creates simulated containers on the merge infeed lanes.  Use the EnableSrt field to enable and disable the simulation.

A timer encoder can be created automatically (in Sorters_Encoder_tmName), if it does not exist, using the CreateSorterTmr button.

| ZprSimConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique descriptive name. An entry is required for each merge lane. |
| EnableSrt | Y/N | Enable or disable the simulation for this lane. |
| CreateBoxes | Radio | Use this to enable creation of simulated containers for this lane. |
| Status | String | The software will use this field to display error and status messages. |
| InPoint | PopUp | Choose the induct point associated with this lane. These would be the same induct points used in ZipperLane_PreMergeInduct. |
| CreateSorterTmr | Radio | ON = Enable simulation (EnableSrt must also be Y), create a timer-based encoder (if it doesn’t exist) in Sorters_Encoder_tmName, and activate it by setting Sorters_Encoder_Type = 1. OFF = Disable simulation and the timer encoder. |
| Length | Int | Enter the length, in ticks, for the simulated containers. |
| Gap | Int | Enter the gap size, in ticks, to create between the simulated containers. |
| EncoderGain | Float | Enter a correction factor multiplier to be applied to the timer-based encoder time. If the simulation is running too fast or too slow, this can be used to adjust it. For example, enter 0.9 to reduce the time base by 10 percent and increase the speed of the simulated encoder. A value of 0.0 applies no factor, so it is equivalent to 1.0. |
| pCurrLength | Int | Set by the software to the number of ticks remaining for the container currently being created. |
| pCurrGap | Int | Set by the software to the number of ticks remaining for the gap on the current container. |
| pLastShiftCnt | Int | Set by the software to the last shift count detected for the shifter. |
| pStateForcePE | Int | Used by the software to regulate the action of the simulated induct photoeye. |

    - ZipperErrLog Table

Records and logs zipper merge errors and events.  Data is taken from table ZipperLane.  Log entries are written to the CTRL log for ZipperMerge_LogLevel = “Level 2” and higher.

View the data directly from this table or search the CTRL log for tags “ZprAddErrorLogEntry” and “ZprProcessMergeJamPhotoeye”.

| ZipperErrLog Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Taken from ZipperLane_Name |
| Ndx | Int | Taken from ZipperLane_pLastMergeNdx |
| Spd | Int | Taken from ZipperLane_pLastMergeSpd |
| Err | Float | Taken from ZipperLane_pMergeError |
| Drift | Int | Taken from ZipperLane_pDriftAdjust |
| Adj | Int | Taken from ZipperLane_pLastAdjTicks |
| ShftPrtErr | Int | Error ticks calculated for the event from the shift print. |
| ErrUpdated | Y/N | Indicates if ShftPrtErr has been calculated for this event. |
| LastRowUpdated | Int | Used internally to select the next row in this table to use. |
| TimeStamp | Time | Timestamp of the last update. |

    - ZprIOAction Table

Not used.

    - ZprStatusMsgs Table

Not used.

  - Zipper Merge Operation and Tuning

Two entries are required in table Batches to run the Zipper Merge logic:

- Zpr CTRL (CPN)

- Zpr SORT (CPN)

The Zipper Merge control logic and its associated control parameters will produce coordinated and efficient merging of multiple lanes when properly configured.

When properly configured, the brake-meter belt will maintain a constant speed when a container is feeding across the belt.  Verify the container merges into the front portion of the reservation.

If the speed or gain is not set properly, or the acceleration / deceleration parameters are not set properly or do not agree with the settings on the VFD, the speed of the drive will vary while the container is feeding across the brake-meter belt.

Containers Stop on Brake-Meter Belt

If containers stop before the end of the brake-meter belt before merging, the brake-meter belt is running too fast.  If the speed setting is correct, check the ZipperLane_Gain setting.  Values of 0.99 to 0.90 will help dampen the speed correction calculation.

Containers Miss the Reservation

If containers merge into the end of the reservation or miss the reservation window entirely, the brake-meter belt is running too slow.  If the speed setting is correct, check the ZipperLane_Gain setting.  Values of 1.01 to 1.10 will increase the speed correction value.  Be conservative with gain values greater than 1.0!

Containers Merge Late from a Complete Stop

Occasionally, containers will come to a complete stop while waiting for space on the merge.  The speed, acceleration and deceleration settings may need additional compensation to achieve accurate merging in this situation.

An update point should already be defined using a jam photoeye near the end of the merge belt.  That photoeye will trigger the SORT process to generate a “shift print” of the virtual container images in the log file.  Use the shift prints to adjust as needed.

Allow a container to pass through the merge lane.  Make note of the container position in the shift print image.  Disable the lane and allow a container to move to the end of the belt and stop.  Enable the lane and let the container pass through.  Compare the shift print for that container with the first one.  Count the number of ticks (characters) the container has moved in the shift print.  That value is the ticks required to compensate for the stopped condition.  Enter the value in ZipperLane_SlowLenAdjTicks.  Repeat the test to verify.

Containers Miss the Mark at Medium Speeds

If containers merge properly from a full stop or when passing through at full speed, but not at slow or medium speeds, use ZipperLane_SpdFactor to compensate.  The SpdFactor value is multiplied by the calculated speed to adjust the actual speed set for the motor.

Try SpdFactor values in the range of 0.99 to 0.90 to reduce the speed used in the error calculation and increase the error correction distance.

    - Setup for Standard Logic

Set the following parameters in table ZipperMerge:

  - MergeSpdFtPerMin – Enter the measured speed of the main merge belt, in feet per minute.  A typical value is 360 FPM.

  - MergeTicksPerInch – Set this to match the configuration of the encoder for the main merge belt.  Usually set to 1.

  - GapType – Usually set to Gap unless a scale is used then you probably want Pitch.

Set the following parameters in table ZipperLane:

    - MeterTicksPerInch – Set this to match the configuration for the brake-meter belt encoder.  Usually set to 1.

    - MeterSpdFtPerMin – Enter the measured speed of the lane when running at MaxPercent.  A typical value is 360 FPM.

    - ResrvGapInches – Set to the desired gap, in decimal inches.

    - MergeGapInches – Acts as a minimum gap size value.  The container length plus this value must fit in the reservation for it to merge.  Set to 0 initially.

    - StopInches – The distance required to stop the lane.  Set this in conjunction with the VFD setting to match the actual stopping distance.  An initial value of 0 is acceptable.

    - MaxErrorTicks – Defines the maximum allowable error, in ticks.  Set to 0 initially.

    - SlowLenAdjustTicks – Set to 0 initially.  Can be used to compensate for sluggish brake-meter belts.

    - BrakeMeterUpdate – Choose an update point that defines the logical and physical end of the brake-meter belt.  To set the update point position, measure from the induct photoeye to the end of the belt, convert the distance to ticks and add the induct photoeye tick position (usually 30).

    - MergeUpdate – Choose an update point that defines the reflected induct location from the main merge belt.  Set the position for the first lane at a location that is far enough from the beginning of the merge to allow the largest reservation from each lane.  If the values for the lanes need to be adjusted later, they must all be moved to preserve the relative spacing between them.  Example:  420 (This allows 420 ticks of reservation space.  If there are 4 lanes, that is 105 ticks per lane.)

    - MinPercent – Set this value to the minimum belt speed that will not stall the motor under full load for the brake-meter belt, in percent (The range is 0 to 100%).

Verify the encoder and the speed control configuration are working properly.

    - Open the Encoders table.  Measure the speed of the belt using a tachometer.  Verify the correct speed is displayed in the Calculated FPM column.  Set SampleSeconds to 0.25.

    - Open the SpdControl table.  Set OnPercent to 0.  OffPercent is set by the software and should be at 100.  Select an unused memory bit in the Maint_IO column.  The memory bit will be forced ON and OFF to switch between the OnPercent and OffPercent values.

    - Force the memory bit OFF.  The Calculated FPM speed should go to 0% within 0.25 seconds.

    - Force the memory bit ON.  The Calculated FPM speed should go to 100% within 0.25 seconds.

    - Enter 50 into the OnPercent column.

    - Force the memory bit ON.  The Calculated FPM speed should go to 50% within 0.25 seconds.  For Example:  If the Lane is running at 300 ft/min, the speed should go to 150 ft/min.  If the speed does not switch to 50% of the speed measured in the first step, the encoder or the speed control is not configured properly.

    - If the speed does not change in 0.25 seconds, the drive parameters may not be configured properly.

Disable each lane using ZipperLane_Enabled.  Place containers on the belt prior to the brake-meter belt of different sizes and weights.  Each container should move to the update point position defined in ZipperLane_BrakeMeterUpdate and stop.

  - Adjust the stop point by increasing the value in StopInches until the container stops prior to the end of the belt.  Containers must not drift onto the downstream belt.  Containers must not feed through the update point.  The nose of the container must stop within a few ticks of the update point.

  - Use the graphical shifter to determine the variation in stopping position for the full range of container sizes and weights.  Obtain as complete a sample of containers as possible – at a minimum, use ten containers.  Make note of the position of the end-of-belt update point on the Points Info tab.  The graphical shifter displays the tick location for each container on the Boxes Info tab.  Record the stop positions for each container in your sample and calculate the difference from the end-of-belt point for each one.  If the container stops past the update point, the difference is negative.  Find the minimum and maximum difference from the whole set of containers.  Calculate the difference between those values.  For example, using three containers, if the differences were -1, 1 and 2, the range would be 2 - -1 = 3 ticks.  Set ZipperLane_MaxErrorTicks = 3.

  - Repeat this procedure for each lane.

Re-enable the lanes using ZipperLane_Enabled.  This will allow the containers to merge.

- Place a container on the first lane and determine if it hits the reservation.  The container should advance onto the brake-meter belt and should not stop.  If it stops, the reservation on the merge is not getting to the merge point at the correct time.  Verify the encoder ratio and the speeds of the lane and the merge are correct.

    - If the container does not stop, verify it is merging into the front portion of the reservation.  The reservation will change color from white to gray to show where the container falls in the reservation.  If the container is in the last portion of the reservation, verify the encoder ratio, and the speeds of the lane and the merge are correct.

If you have not done so already, add the jam photoeye at the end of the main merge belt to an update point.  It will trigger the SORT process to print a shift print in the SORT log.  Select the update point in ZipperMerge_MergeJamUpdate to cause the ZipperErrLog table to be updated with the calculated error for each container.  The latest data will appear in row 1.

If the merge includes vertical belts, verify they are running at the correct speed.

    - Set the vertical belt speed to (merge belt speed / cosine of the angle).

    - Usually, containers feeding from lane 2 are not affected by the vertical belts.  Place at least five containers of different sizes, one container at a time, on lane 2.  Verify they feed through the merge without changing speed and verify the shift print always shows the container at the same location, to within +1 tick.  If the shift print shows a larger variation, the encoder or IO rack update speed is not configured properly.

    - Place at least five containers of different sizes, one container at a time, on lane 1.  Verify they feed through the merge without changing speed and verify the shift print always shows the container at the same location, to within +1 tick.  If the shift print shows a larger variation, increase the speed of the vertical belt by 1%.  Adjust the speed until the variation is +1 tick.

    - Repeat the adjustments for each vertical belt.

Place containers on lane 1, one at a time.  Use the shift prints to observe the container positions on the main merge belt.  If adjustments are needed, do not change the lane 1 update point position – use the jam photoeye update point position instead for lane 1 only.  Adjust until the shift prints show the containers consistently within two ticks of the nose.

Once the jam photoeye update point is adjusted for lane 1, do not change it again.

For the remaining lanes, adjust the lane update points until the shift prints show the container consistently within two ticks of the nose.  You can also use the ZipperErrLog table for this purpose.  The containers should not stop once the adjustment is made for each of the lanes.

Place a variety of containers on the belt prior to the brake-meter belt.  Verify the shift prints show them consistently within two ticks of the nose.

Place a variety of containers on the belt prior to the brake-meter belt.  Use the graphical shifter to verify they merge within two ticks of the nose of the reservation.  The reservation will change color from white to gray, to show where the container falls in the reservation.  Stop the system, if needed, to verify this.

Determine how far containers fall back from the desired position when merging from a full stop.  Disable the lane and allow the container to feed up to the stop point on Lane 1.  Enable the lane and allow the container to merge.  Use the shift print to determine how many ticks the container has fallen back.  Repeat this test with containers of different sizes and weights.  Compute an average error from these values.  Set ZipperLane_SlowLenAdjTicks to the calculated value to compensate for the error.  Do this for each lane.

    - Setup for Sawtooth Logic

See the sections, below.

    - Verify Reservation Size and Gap

Zipper Standard Merge Configuration

In standard mode, reservations are created per container as they move onto the brake-meter belt.  Reservations are allocated based on the status of all the lanes that are waiting for a reservation.

ZipperLane_ResrvGapInches is used if it is larger than ZipperLane_MergeGapInches.  In systems where the control of the brake-meter belt speed is sluggish, ZipperLane_ResrvGapInches can be increased to allow the container to lag behind the reservation.  The container will use the end of the reservation instead of the beginning (Note:  If the brake-meter belt lags behind too far, a new reservation will be created).  If the system is missing reservations, they will stay white on the graphical shifter for the merge belt.

The distance between ZipperMerge_MergeInduct and the first lane determines how far ahead reservations can be created.  If priority needs to be provided to a lane (such as for a full lane condition), the distance between ZipperMerge_MergeInduct and the first lane may be decreased to allow room for only one or two reservations.  The lanes will have to wait and compete for the reservation space and the lane with the lowest priority (full condition) will be given the reservation.

Zipper Sawtooth Merge Configuration

Zipper Sawtooth mode is activated by setting ZipperMerge_MergeType = Sawtooth.  In this mode, ZipperMerge_CreateResrvFlag determines the way the reservation will be created:

If ZipperMerge_CreateResrvFlag = Y, ZipperLane_ResrvLenInches (the reservation size) is used to determine the amount of space allocated.  Set the reservation size to at least the length of the longest container.  If the size is set too large, however, throughput may be restricted.  The software always attempts to use any available space that is large enough, whether it is in a reservation or not.

If ZipperMerge_CreateResrvFlag = N, space is allocated for containers when they reach the ZipperLane_MergeUpdate point.  Reservation space will be allocated with priority given to the first lane found in table ZipperLane.  ZipperLane_ResrvLenInches is not used.

In either mode, ZipperLane_ResrvGapInches is used to set the gap for each lane.

    - Verify Brake-Meter Configuration

The software sets the ZipperLane_AccelRate acceleration and ZipperLane_DecelRate deceleration to the values needed based on ZipperLane_AccelMilliSec and ZipperLane_DecelMilliSec, which are the times, in milliseconds, to go from zero to full-speed and vice-versa.  The times should match those set in the VFD.  The containers must stop and start without slipping and must maintain tracking (the logical image on the shifter must not move forward or backward relative to the physical container, as seen via the shift print images).
NOTE:  Older versions require the values to be set here using a value linked to the SORT process iteration time.  (0.5 – 1 second, 1 – 0.5 second, or 2 – 0.25 seconds).

Adjust the ZipperLane_BrakeMeterUpdate point for each lane before adjusting the ZipperLane_MergeUpdate points.  Containers will stop and hold at the ZipperLane_BrakeMeterUpdate point, which should be at the end of the brake-meter belt.  You can test this by using ZipperLane_Enabled to disable the lane.  Containers will advance to the ZipperLane_BrakeMeterUpdate point and stop.  Adjust the point as needed so the container stops at the end of the brake-meter belt.

Enable the Lane.  Place two containers on the belt prior to the brake-meter belt close enough to each other that the induct photoeye cannot see between them.  Adjust ZipperLane_PreGapInches so that the second container stops on the brake-meter belt.  It should stop within two inches of the end of the belt.

If the brake-meter belt does not decelerate and stop accurately, use ZipperLane_StopInches to cause the belt to stop based on the position of the container relative to the ZipperLane_BrakeMeterUpdate point.  Note that if StopInches is set too large, the brake-meter belt will stop excessively.

Adjusting the Belt Speeds

Use a tachometer to measure the speed of the brake-meter belt when it is running at ZipperLane_MaxPercent (this is the normal speed when the lane is empty).  Use the measured value to set ZipperLane_MeterSpdFtPerMin.

Use a tachometer to measure the speed of the main merge belt when it is running empty.  Use the measured value to set ZipperMerge_MergeSpdFtPerMin.

    - Verify the Merge Shifter Configuration

An update point should already be defined using a jam photoeye near the end of the merge belt.  That photoeye will trigger the SORT process to generate a “shift print” of the virtual container images in the log file.  Use the shift prints to adjust as needed.

Start with the lane entering the main merge belt at the most upstream end (Lane 1).  Space the containers about 6 feet apart.  Adjust the update point for the shift print photoeye – NOT the Lane 1 update point!  Adjust until the container is shown consistently with two ticks ahead of the shift print center point.  Once this update point is set, do not change it again.  The shift print update point serves as the reference point for all the merge lanes.

For all the other lanes, adjust the LANE update point until the shift prints consistently show the nose of the container two ticks past the center point.

    - Recovering after a Photoeye Repair

The Zippering Merge will stop merging if the brake-meter belt induct photoeye is blocked or misaligned or if it has failed in an ON state.  This will create an unusually large container on the graphical shifter that will not merge.

After the photoeye has been repaired, the shifter will correct itself if the fake container can fit on the merge.  Most likely, it will be too large to be processed.

Remove the oversized logical container using the graphical shifter.  Use Alt-S to manually shift the shifter until the container image moves off the end of the shifter.  Make sure the induct photoeye is not forced ON when you are finished by verifying the box around “IN  0” is white, not yellow.  If it is yellow, press Alt-I to remove the force.

An alternate method uses the SpdControl table.  This method causes the lane to run physically as well as logically.  Find the appropriate entry in table SpdControl and make sure OnPercent is set to a non-zero value (100 is a good choice if it is not set).  If Maint_IO has a part selected, force it ON using Insert.  If Maint_IO is INVALID, select the ALWAYSON part temporarily.  The motor will run at the OnPercent speed.  Once the logical (and physical) lane is cleared, remove the force.  Restore any other settings that were changed.  The conveyor will return to normal operation.

  - Zipper Merge Tick Gapper

The zipper merge logic also works with the tick gapper, which can be used to control the accumulation conveyor or a brake-meter belt on each infeed lane.

The tick gapper uses several columns in the ZipperMerge and ZipperLane tables to determine the status of the downstream conveyor and if the lane is enabled.

- From ZipperMerge and ZipperLane:  The machine that controls the merge, and delay timers

- From ZipperLane:  The Enable/Disable state of the lane.

    - Example Accumulation Tick Gapper with Three Zones

The logic processes for “Zone 1” and the “Last” zone are special.  The logic for “Zone 2” and “Zone 3” is the same and can be used for any number of intermediate zones.

“Zone 1” is the zone closest to the brake-meter belt.  Its timer and tick count are reset when ZprTickGapper_PresenceEye transitions OFF.

“Zone 2”, “Zone 3” are used for zones upstream of “Zone 1” (they could be used more than once, if needed).  Their timers and tick counts are reset when ZprTickGapper_PresenceEye transitions ON.

The “Last” zone must be the most upstream zone defined.  Its timer and tick count are also reset when ZprTickGapper_PresenceEye transitions ON.

Each zone will decrement its tick count and will begin timing when the prior (downstream) zone’s count has reached zero, its timer has expired and its ZprGapperZone_ZoneIO is OFF.

- When the first container is detected (transition ON of ZprTickGapper_PresenceEye), the logic will hold “Zone 2”, “Zone 3” and the “Last” Zone.  This will allow “Zone 1” to feed the first container and will hold any containers behind it.

- When the first container clears ZprTickGapper_PresenceEye (transition OFF), the logic will hold “Zone 1” and all the others.

- When the timer for “Zone 1” times out, and sufficient ticks have been detected from ZprTickGapper_TickSource, “Zone 1” is re-enabled and can feed.  Each subsequent zone will be allowed to feed in turn after its tick count and timer has been satisfied.  This creates a cascading release that helps control push-through from the accumulation section.

In this example, the tick gapper is used to control an accumulation section composed of multiple segments connected to individual solenoids.  The brake-meter belt is used to adjust the speed as the container is launched onto the main merge belt.  The example shows three tick gapper zones, though any number are possible.  Each tick gapper zone is released in sequence (moving upstream) after the prior zone has completed its cycle.

Last Zone

Accumulation

Infeed Lane

Induct Photoeye

Brake-Meter Belt - Zipper Merge Speed Control Belt

Zone 1

Accumulation

Zone 2

Accumulation

Presence Photoeye

    - Example Accumulation Tick Gapper with One Zone, GoTo Type

The “GoTo Zone” type can be used with a single accumulation section prior to a brake-meter belt.

If ZprTickGapper_PresenceEye is not blocked, the accumulation section (ZprTickGapper_AccumMtr) will run until a container arrives.  If another container arrives before ZprGapperZone_TickDelay ticks have been measured since the previous container, it will wait until a gap of that size has been created.

In “GoTo” mode, the tick count and the timer are reset when ZprTickGapper_PresenceEye transitions OFF.

The ZprTickGapper_StopIfDisabled setting alters the behavior as described in section 6.7.3.

The belt will be enabled when the timer has timed out and the tick count has reached zero.

In this example, a “GoTo Zone” tick gapper is used to control a single accumulation section prior to a brake-meter belt.  The ZprTickGapper_PresenceEye is located before the brake-meter belt.  For this configuration, set ZprTickGapper_StopIfDisabled = Y to stop the accumulation section even if ZprTickGapper_PresenceEye is not blocked, which helps reduce push-through from excessive pressure on the containers entering the accumulation section.

Presence Photoeye

Gapper Section

Brake-Meter Belt -Zipper Merge Speed Control Belt

Gap Presence PE before Brake-Meter

Zone 1

Accumulation

In this example, a “GoTo Zone” tick gapper is used to control a single accumulation section prior to a brake-meter belt, and a variable-speed belt is added prior to the main merge belt.  The ZprTickGapper_PresenceEye is located after the brake-meter belt.  Containers will move to the end of the variable-speed belt and stop to wait at that point, if necessary.  Set ZprTickGapper_StopIfDisabled = N in this case and select the variable-speed belt motor in ZprGapperZone_DwnStreamMtr.  This will allow containers to feed up to the ZipperLane_MergeUpdate point and stop if the lane is empty and disabled.  When a second container arrives, it will advance to ZprTickGapper_PresenceEye.  If the gap is okay, and the variable-speed belt is not stopped, the container will advance onto that belt.  If the variable-speed belt stops, the brake-meter belt will also stop.

Presence Photoeye

Gapper Belt

Brake-Meter Belt

Induct Photoeye

Gap Presence PE after Brake-Meter

Variable-Speed Belt – Zipper Merge Speed Control Belt

ZipperLane_MergeUpdate point. Containers stop and wait here.

    - ZprTickGapper Table

Table ZprTickGapper is used to define items for and infeed lane tick gapper.

| ZprTickGapper Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the lane. |
| Enabled | Radio | Used to manually enable or disable this lane. |
| pStatus | String | The software will use this field to show error and status messages. |
| ZipperLane | PopUp | Choose the lane from table ZipperLane to link with the accumulator section. |
| TickSource | PopUp | Choose a shifter that provides ticks for the tick gapper. The ticks are used to determine the size of the gap. |
| PresenceEye | PopUp | Choose a presence photoeye for the accumulation section. |
| AccumMtr | PopUp | Choose a signal that will indicate if the accumulation section is running. This is used to control timers. |
| StopIfDisabled | Y/N | Use this to help prevent push-through when the zipper lane is stopped. N – The conveyor will not be stopped when the zipper lane is disabled. The container will move on to the speed-control belt up to the PresenceEye. Y – The conveyor will be stopped when the zipper lane is disabled. Containers will only feed when the lane is feeding and when ZprGapperZone_DwnStreamMtr is running (if defined). |
| pLastEncMeter | Int | Set by the software to detect ticks on the brake-meter belt encoder. |

    - ZprGapperZone Table

The ZprGapperZone table is used to configure one or more zones for the tick gapper.

| ZprGapperZone Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the zone. |
| pStatus | String | The software will use this field to display error or status messages. |
| GapperLane | PopUp | Choose a row from the ZprTickGapper table to link the accumulator zone to a specific tick gapper row. |
| ZoneType | PopUp | Choose the zone type to define how the zone will be controlled. The zones use the state of the ZprTickGapper_PresenceEye to determine their action. Zone 1 – Use for the first zone. Zone 2, Zone 3 – Use for intermediate zones, if needed. Last – Use for the last zone. GoTo Zone – Use for GoTo-type zones. ZprTickGapper_AccumMtr will run until ZprTickGapper_PresenceEye is blocked. |
| ZoneIO | PopUp | Choose an IO output to control the zone. |
| ReleaseAction | PopUp | Choose an action to control ZoneIO. |
| ZoneBit | Int | Set the bit to use if ReleaseAction is Enable or Disable. |
| TickDelay | Int | Enter the number of ticks from the brake-meter belt encoder to delay before releasing this zone. Zero or positive values are acceptable. |
| ZoneTimer | PopUp | If a timer is chosen for this zone, it will be used to delay release of the zone. |
| DwnStreamMtr | PopUp | Optional. Choose a signal that indicates the downstream belt is running. This is only needed for merges where the next belt is not the zipper speed-control belt. Use only with ZoneType = “GoTo Zone”. Stops ZoneIO when ZprTickGapper_PresenceEye is ON and DwnStreamMtr is OFF. Runs ZoneIO when ZprTickGapper_PresenceEye is OFF and DwnStreamMtr is ON. |
| pTicksLeft | Int | Set by the software to track the remaining ticks. |