| # FortnaPlus Control Missing Pin Detection Configuration |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 04-Dec-2020 |  |

Revision; Date; Author; Revision Summary. 1.0; 10-Dec-2015; B C Conklin; New document. 1.1; 04-Dec-2020; T R Kremser; Updates and general refresh.

Table of Contents

Section 1	4

Section 2	4

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Purpose

The missing pin detection module is used to help protect shoe sorters from physical damage resulting from shoes (and their pull-over pins) that are out of place.  To prevent costly damage from displaced pins, the module stops the sorter and raises an error when a missing pin is detected.

# Features

- Detects missing pins at any sorter speed.

- Supports use of multiple pin sensors at multiple locations on the sorter.

- Performs a controlled stop of the sorter as soon as a missing pin is detected.

- The FPC Errors module is used to display the location of the sensor that detected the missing pin and the distance the sorter has traveled since the error was detected to help find the trouble spot quickly.

- The module also detects a failed pin proximity sensor.

- Errors can be cleared using an input signal, perhaps from a key switch, or through the software using an access-controlled reset button, typically available only to maintenance personnel.

# Hardware Requirements

Pairs of proximity pin detection sensors are required, with one on each side of the sorter.  Additional pairs of sensors may be placed at intervals along the sorter, if desired, to minimize detection time.  Sensors are required on both sides of the sorter to detect pins regardless of the side the shoes are on.

Detection is done by counting encoder ticks after each pin sensor transitions ON.  If no transition is detected in MaxTicks ticks while the sorter is running, the missing pin error is triggered, and the sorter is stopped.  The sorter will also be stopped if the sensor does not transition off after MaxTicks to detect a failed proximity sensor.

# Configuration – Batches Table

The missing pin module must be activated through the Batches table using these two processes:

- InternalProcCtrl:  Missing Pin (CTRL).  Process:  CTRL

- InternalProcCtrl:  Missing Pin (SORT).  Process:  SORT

Set Machine or All Machines to activate the feature.

# Configuration – MissingPin Table

Table MissingPin contains the configuration information to define the proximity sensors and control signals for each missing pin detection location on the sorter.

| MissingPin Table |  |  |
| --- | --- | --- |
| Column Name | Type | Description |
| Name | String | Enter a unique description. |
| Enable | Y/N | Enter Y to enable this entry, or N to disable. |
| Status | String | The module will update this field. |
| ControlMachine | Popup | The AC that will process this row. |
| Prox1 | Popup | The first proximity sensor input in the pair described by this row. The convention is to assign the left-side sensor here. |
| Prox2 | Popup | The second proximity sensor input in the pair described by this row. The convention is to assign the right-side sensor here. |
| MaxTicks | Int | The maximum number of encoder ticks allowed between pins before an error is declared. Example: If the encoder is set to 12 ticks per foot (1 tick per inch) and the pins are 3 inches apart, set the value to 4 (a pin should always be detected in 4 inches). An error will occur if 4 encoder ticks are detected and at least one of the pin detection proximity sensors has not transitioned ON. |
| Sorter | Popup | Choose the sorter. |
| Encoder | Popup | Choose the Encoder associated with this sorter. |
| ClearErrorSig | Popup | Select an optional input signal to use to clear the error, perhaps a key switch. Or use Maintenance Menu ¦ Reset Utilites ¦ Reset Missing Pin Error |
| SorterRunSig | Popup | Select an input signal that indicates the sorter is running. The software only checks for a stuck sensor when the sorter is running. |
| DontCntRunOff | Y/N | If set to Y, the module will not count encoder pulses when SorterRunSig is OFF. If set to N, the module will perform its checks regardless of the state of SorterRunSig. |
| SpdControl | Popup | If a record is configured in table SpdControl for the speed of the VFD powering this sorter, select the record here. On each encoder tick the software will calculate the current speed (in the pSpdFdBack field) to determine if the sorter is moving fast enough to check for missing pins. |
| SpdMinimum | Float | If the SpdControl field has been populated, enter the minimum speed needed to check for missing pins. A value of 0.0 will allow checking at any speed above a full stop. |
| ErrorSig1 | Popup | Select an output error IO signal that will be activated when a missing pin is detected. Select a memory bit that will be used to activate an error in the Errors table. The signal will be set ON when an error occurs and OFF otherwise. |
| Error1 | Popup | Select an unused row in table errsugt to use when ErrorSig1 is triggered. The string for the error suggestion will be composed using the text in ErrMsg1 and “, N Inches Trvl”, where N is the distance from the sensor to the current position of the sorter belt. This string will be written to errsugt_Error_Suggestion for the row selected. The error suggestion string is only modified on the controlling machine. The Status field in this table is also updated in a similar way, appending “, N Inches”. |
| ErrMsg1 | String | Define the first part of the error suggestion string to be written to errsugt_Error_Suggestion when an error occurs. Choose a descriptive string that is unique for this use, like “CALL-MAINTENANCE”. Note the hyphen that make it distinct from the standard entry. An example of the constructed string: CALL-MAINTENANCE, 10 Inches Trvl |
| ErrorSig2 | Popup | Select an output error signal that will be activated when Prox1 is stuck on. Use a memory bit to activate an error in the Errors table. The signal will be set ON when an error occurs and OFF otherwise. |
| ErrorSig3 | Popup | Select an output error signal that will be activated when Prox2 is stuck on. Use a memory bit to activate an error in the Errors table. The signal will be set ON when an error occurs and OFF otherwise. |
| StopSorterSig | Popup | Select an output signal that will be de-activated when an error occurs. Configure the sorter with Encoders_NoSortOnSpeed set to Y. The sorter will stop firing any diverts as soon as the Encoders_EnableBit signal goes OFF. The Encoders_EnableBit signal should have the motor run signal for the sorter. If the sorter cannot be configured with Encoders_NoSortOnSpeed = Y, see DivertEnableSig, below. |
| StopACTION | Popup | Select the action that will be used for StopSorterSig. |
| StopBIT | Int | If an Enable or Disable action is selected in StopACTION, set the bit to be used. |
| DivertEnableSig | Popup | The part selected in this field is de-activated when an error occurs. Use this if the sorter cannot be configured with Encoders_NoSortOnSpeed = Y. If the sorter is configured with a memory bit here and in Sorters_DivertEnableIO, the sorter will stop diverting when a missing pin error occurs. |
| EnableACTION | Popup | Select the action that will be used for DivertEnableSig. |
| EnableBIT | Int | If an Enable or Disable action is selected in EnableACTION, set the bit to be used. |
| pSpdFdBack | Float | If a SpdControl record has been selected, on each encoder pulse the current speed will be calculated. The result can be seen here as a percentage (0 – 100%) based on SpdControl_MaxSpd and the current speed. |
| pLastEncoder | Int | The previous encoder count read. Used to detect transitions. |
| pCurrCount | Int | The latest encoder count read. This value is reset to 0 during initialization. It is also reset when Prox1 or Prox2 transition ON. If the value reaches or exceeds MaxTicks, an error is raised. |
| pStuckCount1 | Int | The number of encoder ticks since Prox1 turned ON. Resets to 0 when Prox1 is OFF. Only increments while SorterRunSig is ON. If the value reaches or exceeds MaxTicks, an error is raised. |
| pStuckCount2 | Int | The number of encoder ticks since Prox2 turned ON. Resets to 0 when Prox2 is OFF. Only increments while SorterRunSig is ON. If the value reaches or exceeds MaxTicks, an error is raised. |
| pPEState1 | Int | The current state of Prox1. |
| pPEState2 | Int | The current state of Prox2. |
| pLastCount | Int | The number of encoder ticks since the last proximity sensor transition ON signal. |
| pStopTicks | Int | The number of ticks since a broken pin was detected. |
| pMissingPinErr1 | Int | Flag that indicates a pin is missing. Either Prox1 or Prox2 did not change state. |
| pStuck1Err2 | Int | Flag that indicates Prox1 is stuck on. |
| pStuck2Err3 | Int | Flag that indicates Prox2 is stuck on. |

# Algorithm Details

- During initialization, pCurrCount is set to 0.

- When Prox1 or Prox2 transition ON, pCurrCount is reset to 0.

- If the pCurrCount value is greater than or equal to MaxTicks the sorter is stopped by executing the actions for each of the selected signals.  This will stop the sorter, turn on the error and disable diverts.

- pStuckCount1 and pStuckCount2 will only increment if SorterRunSig is ON.

- If the pStuckCount1 or pStuckCount2 values are greater than or equal to MaxTicks the sorter is stopped by executing the actions for each of the selected signals.  This will stop the sorter, turn on the error and disable diverts.

- When Prox1 or Prox2 transition OFF, pStuckCount1 or pStuckCount2, respectively, are reset to 0.

- The errors are cleared by an input signal on MissingPin_ClearErrorSignal (if defined) or from Maintenance Menu | Reset Utilites | Reset Missing Pin Error.