# FortnaPlus Control Motor Speed Control

Document Revision: 1.2

Date: 24-Aug-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.1 | 02-Jan-2015 | B C Conklin | Added revision history |
| 1.2 | 24-Aug-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	Motor Speed Control	5

2.	Details	5

3.	SpdControl Table	5

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

IO – Input / Output.  A reference to electrical interface hardware used to control and receive signals from conveyor devices.

UPS – Uninterruptible Power Supply

# Motor Speed Control

The Motor Speed Control module is used to convert the target speed for a variable-speed motor to a binary bit pattern.

The desired speed is expressed as a percentage of maximum speed.  The module converts that value to a binary pattern that is set in the motor controller’s IO input space (output from FPC).

Various modules within FPC will generate a motor speed in percentage terms.  They can use this module to set the speed of the motor in the way it understands.

This module can also be used with the analog control module.

# Details

Two speed settings can be defined, OffPercent and OnPercent.  The Maint_IO field is used to select the desired speed.

NOTE:  Maint_IO would perhaps have better been named SpeedSelect, or something more descriptive.  It really only defines which speed setting to use!

NOTE:  Modules, such as Zipper Merge, Servo Gapper, and Print Apply will set the value in OffPercent.  When used with those modules, it is best to leave Maint_IO undefined (INVALID) so they can control the speed.  If Maint_IO is used, when it is in an ON state, the value in OnPercent will set the speed, effectively disabling speed control by those other modules.

This module can also be used to create a software-defined gapper by chaining together two speed-control sections of conveyor and defining the ratio of their speeds so the downstream one runs faster.

# SpdControl Table

All configuration is done using the SpdControl table (Configuration Menu | Standard Controls | Motor Speed Control).

| SpdControl Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique descriptive name. |
| pError | String | Set by the software to display error messages for this entry. |
| EnableFlag | Y/N | Use this to manually enable or disable this entry. |
| Maint_IO | PopUp | Choose an IO signal that will select the desired output value. ON = The motor speed is set from OnPercent. OFF = The motor speed is set from OffPercent. NOTE: The Zipper Merge module alters the value in OffPercent. A signal defined here will override the zipper merge, servo gapper, print apply, or other software modules when it is in the ON state. Leave this field set to INVALID for use with these modules. |
| IO_Owner | PopUp | Choose the process that will control the IO for this motor. If the bits are configured in the Conveyor table, select the same process here. If they are not defined in the Conveyor table, use the SORT process. |
| OffPercent | Float | Set the motor speed to use, as percent of maximum (0 to 100) when Maint_IO is OFF or undefined. The value will be converted to a bit pattern and sent to the VFD. NOTE: The Zipper Merge module alters this value if ZipperLane_SpdControl is linked to this entry! |
| OnPercent | Float | Set the motor speed to use, as percent of maximum (0 to 100) when Maint_IO is ON. The value will be converted to a bit pattern and sent to the VFD. |
| MatchRecord | Int | To match the speed of another conveyor section, enter the row number of the master section here (the row in this table). The OffPercent value of the master row will be copied to this row. |
| MatchPercent | Float | When using MatchRecord, you can adjust the slave section speed using this entry. You can use this to create a gap between sections. The OffPercent value from the master section will be multiplied by the percent value entered here. Values over 100 are acceptable if you want this section to run faster than the master section. A value of 0.0 defeats this feature. |
| MatchWhenMaintOn | Y/N | Use the Maint_IO input to control speed matching. N = Set the speed to OnPercent when Maint_IO is ON. Y = Use MatchRecord and MatchPercent when Maint_IO is ON. |
| IO_Addr_Out | Int | Enter the output IO address word assigned to the device. NOTE: Make sure the numeric base matches the setting for Configio_Octal_Word or convert accordingly! |
| IO_Addr_In | Int | Enter the input IO address word assigned to the device. NOTE: Make sure the numeric base matches the setting for Configio_Octal_Word or convert accordingly! |
| MaxSpeed | Binary | Set this to the binary pattern that defines the maximum speed value for the VFD. Most VFDs use the lower 15 bits: 0111111111111111. |
| PrintLine | PopUp | If this entry is associated with a print apply line, select it here. |
| MtrStartIO | PopUp | Choose an output signal here, if needed. The signal will be set ON if the speed is greater than zero. This is used to disable a motor if it creeps when enabled and set to zero speed. |
| StartAction | PopUp | Choose the action to take on MtrStartIO. |
| StartBit | Int | If StartAction is Enable or Disable, define the bit to use. |
| MtrStopIO | PopUp | Choose an output signal here, if needed. The signal will be set ON if the speed is zero. This is used to disable a motor if it creeps when enabled and set to zero speed. |
| StopAction | PopUp | Choose an action to take on MtrStopIO. |
| StopBit | Int | If StopAction is Enable or Disable, define the bit to use. |
| VFD_NetCTRL | PopUp | Choose a signal, if needed, to activate when EnableFlag = Y. Use this if the VFD requires a signal to put it into network control mode. |
| NetCTRLAction | PopUp | Choose an action to take on VFD_NetCTRL. |
| NetCTRLBit | Int | If NetCTRLAction is Enable or Disable, define the bit to use. |
| VFD_NetREF | PopUp | Choose a signal, if needed, to activate when EnableFlag = Y. Use this if the VFD requires a signal to put it into network control mode. |
| NetREFAction | PopUp | Choose an action to take on VFD_NetREF. |
| NetREFBit | Int | If NetREFAction is Enable or Disable, define the bit to use. |
| VFD_RstTimer | PopUp | Choose a timer to control the pulse duration for the VFD_Reset signal. The VFD reset signal will be set ON when the timer is timing. Select the timer from the Mtrchains table for this motor. This is used to reset a network drive whenever the system is restarted. |
| VFD_Reset | PopUp | Choose a signal, if needed, to activate when VFD_RstTimer is used. This is used in applications where the VFD must be reset each time it is restarted. |
| ResetAction | PopUp | Choose an action to take on VFD_Reset. |
| ResetBit | Int | If ResetAction is Enable or Disable, define the bit to use. |
| VFD_MIN_Prcnt | Float | Used with VFD_Aux. Enter a percent speed value (0 to 100) to define the speed at which VFD_Aux will turn ON. |
| VFD_Aux | PopUp | Choose an IO output that will activate when the speed is at or above VFD_MIN_Prcnt, indicating the motor is running. |
| AuxAction | PopUp | Choose an action to take on VFD_Aux. |
| AuxBit | Int | If AuxAction is Enable or Disable, define the bit to use. |
| pActualPcnt | Int | Set by the software using IO_Addr_In. The binary pattern is converted to a speed percent value that is used with VFD_MIN_Prcnt. |
| pBinOut | Binary | Set by the software to the binary bit pattern sent to the IO_Addr_Out speed control word of the drive. |
| pBinIn | Binary | Set by the software to the binary bit pattern received from the IO_Addr_In speed control word of the drive. |