# FortnaPlus Control New Transfers

Document Revision: 2.3

Date: 29-Sep-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.6 | 18-Apr-2013 | B C Conkin | Added XfSimConfig SimCrrCsu notes. |
| 1.7 | 17-Jun-2013 | T R Kremser | Added information on the SrtFPLite protocol. |
| 1.8 | 24-Sep-2013 | B C Conkin | Added description updates for XfRouteBoss. |
| 1.9 | 10-Oct-2013 | B C Conkin | Changes to transfer scan processing. |
| 2.0 | 22-Aug-2018 | T R Kremser | Add the ToteSensor feature in XfRouteBoss. |
| 2.1 | 06-Dec-2018 | T R Kremser | Add info on Pack and Ship scan types. Edits for clarity. |
| 2.2 | 26-Sep-2022 | T R Kremser | Complete rewrite. |
| 2.3 | 29-Sep-2022 | T R Kremser | Incorporate the contents of the Transfer Work Sheet document. |

Table of Contents

Glossary of Terms	5

1.	Transfer Design	6

2.	New Transfers	6

2.1.	Transfer Routes or Destinations	9

2.2.	Transfer Config Sequence	9

2.3.	Transfer States	9

2.4.	Transfer Configuration using XfgScrn	9

2.5.	Example Transfer Layout	17

2.6.	Scan Process – Scan Events	19

2.7.	Transfer Scan Event Processing	19

2.8.	Scan Message Processing	19

2.9.	Route Queue	19

2.10.	Transfer Step 1 (Boss Idle)	19

2.11.	Transfer Step 2 (Boss Search)	20

2.12.	Transfer Step 3 (Boss Wait)	20

2.13.	Transfer Step 4 (Boss Run)	20

2.14.	Transfer Step 5 (Boss Done)	20

3.	Transfer Scan Processing and Messaging	20

4.	XfRouteBoss Table – Transfer Scans and Messaging	21

5.	XfRouteTable Table – Transfer Destination Configuration	27

6.	XfrTrack and XfrDevice Tables – Transfer Tracking	28

7.	MsgXR1 and MsgXDC1 Tables – Transfer Messages	31

8.	XfSimConfig Table – Transfer Simulator Configuration	32

9.	XfrBoss Table	33

10.	XfrTree Table	36

11.	XfrConfig Table	36

12.	XfrCfgInput Table	38

13.	XfrCfgInTimer Table	39

14.	XfrCfgOutput Table	39

15.	XfrCfgDelayTmr Table	40

16.	XfrSndMsg Table – Sending Messages to Devices	40

17.	XfrParamConfig Table – Transfer Parameter Configuration	42

18.	XfrTrig Table – Transfer Triggers	47

19.	XfrErrConfig Table – Transfer Error Configuration	48

20.	XfrCheckSet Table	49

21.	XfrGoSet Table	50

22.	CRR and CSM Message Structure	50

23.	DCM Message Structure	52

24.	Testing and Debugging	53

24.1.	Example Configuration – Transfer Left	55

24.2.	Transfer Configuration Worksheet	56

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Transfer Design

The diagram shows a very basic transfer, with a Left, Right, and Straight destination.  They will often take this, or more elaborate forms, such as an “H” configuration, with Left-Rear, Left-Forward, Right-Rear, Right-Forward and Straight destination.

Presence Photoeye

Scanner Photoeye

Scanner

Container

The scanner is configured to send the scan data when the container “tails out” of the scanner photoeye.  The scanner photoeye must be located far enough upstream of the presence photoeye to allow the largest container to be scanned.  The container must “tail out” of the scanner photoeye before it activates the presence photoeye.

# New Transfers

The “New Transfer” module includes configuration tools, debugging and monitoring tools, and flexible state control software.  The tools can be used to control a large variety of equipment with the focus on transfers.

Note:  The “Old Transfer” module is still supported in “Old Transfer Utilities [TransferUtil]”, but that is documented elsewhere.

Conveyor transfers may have multiple input points and multiple output points.  The transfer control logic is configured through a set of tables that work together.  A wide variety of conveyor configurations can be supported, as transfers are really just a state machine.  In fact, the transfer module could be used for many generic logic tasks that take actions based on inputs.  The following is a description of the menus and tables, logic and configuration methods used to control transfers.

A brief description of the menu contents is listed below.  Details appear later in this document.  You can access the “Transfer Utilities [NewXfrMenu] from the “Configuration Menu”.

- Transfer Scan Processing – Table XfRouteBoss defines the message tables and links a transfer to a scan zone.  There is typically one record for each scan or routing point.

- Transfer Destination Config – Table XFRouteTable defines logical strings and associates them with a destination.

- Transfer Boss – Table XfrBoss is used to define the product input, release output, and the max wait time for a transfer.  There is typically one record for each scan or routing point.

- Transfer Tree – Table XfrTree defines specific routes and destinations a container may be assigned.  Thhe route from each input to each output.

- Transfer Config – Table XfrConfig defines the detailed control elements for each transfer section.  A row may contain all the steps associated with a destination or it may contain a portion of the steps.  Multiple rows may be combined for control of more complex entities.

- Transfer Inputs – Table XfrCfgInput contains a list of inputs and their respective “Sense” (IsOn, IsOff etc.) required to allow the transfer to enter a given state.  For example, a container must be present before entering “State 1”.

- Transfer Outputs – Table XfrCfgOutput contains a list of outputs and the respective “Output Actions” (Fire, Enable, etc.) that are activated for the selected states.  Outputs may be defined to be “Activated” or “Deactivated” for each state of the transfer.  For example, a container is fed onto the transfer bed in “State 1” by firing the output “Zone102Release”.

- Transfer In Timers – Table XfrCfgInTimer contains a list of “In Timers” and the “Timer Sense” (timer done or timing) required to allow the transfer to enter a given state.  For example, a container cannot proceed to the “Right” until the “Full Clear Timer” for the “Right” destination “Is Done”.

- Transfer Delay Timers – Table XfrCfgDelayTmr contains a list of “Delay Timers” and their respective “Timer Actions” that are activated for their respective state.  “Delay Timers” are used to extend a state.  For example, a “Delay Timer” may be set so the transfer will remain in “State 1” for an additional 200 mSec after a container feeds out of the presence photoeye before the transfer moves to “State 2”.

- Transfer Check Set Recs – Table XfrCheckSet defines one or more transfer sections (XfrConfig) that must be “Idle” before a container can proceed along a path defined as an XfrTree.

- Transfer Go Set Recs – Table XfrGoSet defines one or more transfer sections (XfrConfig) that will run for a particular path defined as an XfrTree.  Each transfer section may be started in an arbitrary initial state, however in a typical configuration the initial state is set to “Idle”.

- Transfer Tracking – Table XfrTrack contains tracking records associated with transfers.  One table can usually handle all transfers defined on an AC due to the low-speed and single-container nature of transfers.

- Transfer Device Tracking – Table XfrDevice contains scans associated with a tracking record, and their status information.  Multiple scans may be associated with a single tracking record if more than one scanner is defined for a transfer.

- Transfer Messages – The XfrMsgUtil menu contains all the tables typically used for transfer-related messages.

- Transfer Error Config – Table XfrErrConfig contains a list of “Timeout Errors” and their respective error actions.  Each row in the table is associated with an XfrConfig record and a “State”.  The error processing is executed when a transfer “Cycle Complete” timer completes, and the transfer has not finished processing.

- Transfer DCM Config – Table XfrParamConfig contains a list of actions that will be processed when the transfer enters the selected state.  Each row in the table is linked to a specific state.  Actions include the ability to record the state of a signal in one of the parameter columns in the DCM message.  When a DCM event occurs, the parameters are sent to Level 2 in the DCM Message.  Additional functions include the ability to retrieve a scan, such as a verify scan.

- Transfer DCM Trigger – Table XfrTrig provides the ability to trigger another transfer when the currently-executing transfer reaches the selected state.  A transfer may trigger another transfer.

- Transfer Send Message – Table XfrSndMsg contains a list of “Messages” that are sent to the selected device when the transfer enters the selected state.  Each row in the table is associated with an XfrConfig record and a “State”.  Messages may be sent to serial devices or networked devices.

- Transfer Pallet Maintenance – The XfrPalletMaint special operator control screen for pallet management.  Used at McKesson.

- Transfer PNA Monitor – The XfrPNAMenu transfer print and apply monitor screen, and its configuration table.

- Transfer PnaInput – Table XfrCfgPnAInput is used to define transfer print and apply inputs.

- Transfer PnAOutput – Table XfrCfgPnAOutput is used to define transfer print and apply outputs.

- Transfer Sim Config – Table XfSimConfig is used to configure transfer simulations.

- Transfer Sim Scans – Menu XfrSimScanUtil has tables used to define simulated scan values for the transfer simulator.

- Configure Transfers – The ConfigXfgScrn screen is used to configure, monitor, and manipulate transfers.

## Transfer Routes or Destinations

Each decision point usually requires a scanner.  The transfer module processes the scanner data and determines a route for the container to follow.

Note:  Transfers should be configured so that table XfrBoss contains the “Container Present Input”, and “State 1” contains the “Full Timer(s)”.  The logic will not run the transfer until the “Container Present Input” and the “State 1” inputs and InTimer(s) are ready.

Example 1:  A simple application has two routes:  Destination “D01” routes to Workstation A, and Destination “D02” routes to Workstation B.  Both routes pass through Transfer Xfr101.  The two routes are defined as:

Tree Entry 1:	“D01”
Tree Entry 2:	“D02”

When the scanner and transfer modules receive and process the scanner data, function XfrTransferInduct() will pass the string “D01” or “D02” to the routine.

## Transfer Config Sequence

The transfer module controls a transfer through a series of sequential steps.  The delay timer must complete, and the inputs must be in the defined state, and InTimers must be complete before the next state can start.  The XfrConfig record may contain configured inputs, InTimers, delay timers, and outputs for any of the listed states.

## Transfer States

- Disable – Disabled state.  If the transfer XfrConfig is configured with an EnabledINPUT, the outputs are disabled when the EnabledINPUT is not in the selected state.

- Idle – Idle state.  Does not have entries in XfrCfgInput, XfrCfgInTimer, or XfrCfgDelayTmr.  Outputs may be assigned to Idle, however, this should only be done if the output is used in only one XfrConfig sequence.

- State 1 thru State 10 – These states are available to build the sequences required.

- Done – The last state processed by the transfer.  Outputs should be set to their normal state here.  The wizard automatically selects “DeActivate” for this state.

## Transfer Configuration using XfgScrn

The Configure Transfers screen provides a single view of all the transfer configuration tables.  “Add” buttons are provided that can be used to add a record to each table.

| ConfigXfgScrn Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ScanZone | PopUp | This field contains the selected ScnScanZone record, or it contains the last record added by clicking the AddScanZone button. The ScanZone is the highest-level control record for a transfer. The record defines the decision point, and the tracking tables that will be used for the selected transfer. |
| AddScanZone | Radio | When this button is clicked, an entry is added to the ScnScanZone table. The following fields in the record are updated: - Name – An auto-generated name is added. - ScanZoneID – An auto-generated name used to identify the decision point is added. This must match the CommCore decision point name, so you will likely need to change it. - TrackingTable – Assigns XfrTrack to the scan zone. - DeviceTrkTable – assigns XfrDevice to the scan zone. The remaining fields must be entered manually. Use the right mouse button (click on ScanZone) to open the ScnScanZone table to update the remaining fields. |
| XfrRouteConfig | PopUp | This field contains either the transfer XfRouteBoss record from the popup, or the last record added by clicking the AddRouteConfig button. The XfRouteBoss record contains a link to the scanner, links to message records, and exception processing for the transfer. |
| AddRouteConfig | Radio | When this button is clicked, an entry is added to the XfRouteBoss table. The following fields in the record are updated: - Name – An auto-generated name is added. - RunNewXfr – Set to Y. The remaining fields must be entered manually. Use the right mouse button (click on XfrRouteConfig) to open the XfRouteBoss table to update the remaining fields. |
| SelectedXfr | PopUp | This field contains either the transfer selected from the popup, or the last transfer added by clicking the AddXfr button. |
| AddXfr | Radio | When this button is clicked, an entry is added to the XfrBoss table. The following fields in the record are updated: - Name – An auto-generated name is added. - MaxWaitTM – A max wait timer is added. - Machine – The current Machine is added. The remaining fields must be entered manually. Use the right mouse button (click on SelectedXfr) to open the XfrBoss table to update the remaining fields. |
| SelectedTree | PopUp | This field contains either the XfrTree record selected from the popup, or the last one added by clicking on the AddTree button. |
| AddTree | Radio | When this button is clicked, an entry is added to the XfrTree table. The following fields in the record are updated: - Name – An auto-generated name is added. - Boss – The currently-selected Boss is added. - Dest – An auto-generated destination string is added. - DoConfirm – Set to Y to indicate DCM messages will be generated. The remaining fields must be entered manually. Use the right mouse button (click on SelectedTree) to open the XfrTree table to update the remaining fields. |
| XfrRoute | PopUp | This field contains either the XfRouteTable entry selected from the popup, or the last entry added by clicking on the AddRoute button. |
| AddRoute | Radio | When this button is clicked, an entry is added to XfRouteTable. The following fields in the record are updated: - Name – An auto-generated name is added. - BossRecord – The currently-selected XfRouteBoss entry is added. - HostRouteStr – The host route string is set to the same string found in the XfrTree “Dest” column. The string needs to be updated to match the CommCore routing string for the destination. - NewRoute – Set to the currently-selected XfrTree record. The remaining fields must be entered manually. Use the right mouse button (click on XfrRoute) to open XfRouteTable to update the remaining fields. |
| SelectedConfig | PopUp | This field contains either the transfer XfrConfig record selected from the popup, or the last transfer XfrConfig added by clicking on the AddConfig button. |
| AddConfig | Radio | When this button is clicked, an entry is added to the XfrConfig table. The following fields in the record are updated: - Name – An auto-generated name is added. A transfer config record is assigned to a specific transfer through the XfrGoSet table. The transfer config may be re-used in multiple XfrGoSet entries. |
| NextConfig | Radio | When this button is clicked, the next XfrConfig record is selected. |
| AddEnableDelayTM | Radio | When this button is clicked, a timer entry is added to the EnabledDelay column of the XfrConfig table. The timer is used to add a delay to the enable event. For example, the transfer is disabled when a lift gate is opened. The transfer is re-enabled after a two-second delay when the lift gate is closed. |
| CheckSet | PopUp | This field contains either the transfer CheckSet selected from the popup, or the last transfer CheckSet added by clicking on the AddCheckSet button. |
| AddCheckSet | Radio | When this button is clicked, an entry is added to the XfrCheckSet table. The following fields in the record are updated: - Name – An auto-generated name is added. - Tree – The currently-selected Tree is added. - XfrConfig – The currently-selected XfrConfig is added. Each transfer may be configured with any number of CheckSet entries. For example, a transfer tree must not execute unless the following XfrConfig entries are “Idle”: - C01-X01 - C02-X02 - C03-X03 For each of the three XfrConfig records shown, (repeat three times) select the XfrConfig record, (C01-X01, C02-X02, C03-X03) click on the AddCheckSet button. This will add three XfrCheckSet records linked to the three XfrConfig records. |
| GoSet | PopUp | This field contains either the transfer GoSet selected from the popup, or the last transfer GoSet added by clicking on the AddGoSet button. |
| AddGoSet | Radio | When this button is clicked, an entry is added to the XfrGoSet table. The following fields in the record are updated: - Name – An auto-generated name is added. - Tree – The currently-selected Tree is added. - XfrConfig – The currently-selected XfrConfig is added. - InitialState – Set to “Idle” Each Transfer may be configured with any number of GoSet entries. For example, a transfer tree will execute the following XfrConfig entries: - C01-X01 - C02-X02 - C03-X03 For each of the three XfrConfig records shown, (repeat three times) select the XfrConfig record (C01-X01, C02-X02, C03-X03), click on the AddGoSet button. This will add three XfrGoSet records linked to the three XfrConfig records. |
| SelectedState | PopUp | This field contains either the transfer State selected from the popup, or the last transfer State selected using the NextState or PrevState buttons. The selected State is also set to “State 1” when the table is initialized or if a different XfrConfig record is selected. |
| NextState | Radio | When this button is clicked, the next XfrConfig state is selected. For example, if the SelectedState is set to “State 1” and the NextState button is clicked, the SelectedState will advance to “State 2”. The Inputs, Outputs, InTimers, and DelayTimers associated with “State 2” will be selected. |
| PrevState | Radio | When this button is clicked, the previous XfrConfig state is selected. For example, if the SelectedState is set to “State 2” and the PrevState button is clicked, the SelectedState will change to “State 1”. The Inputs, Outputs, InTimers, and DelayTimers associated with “State 1” will be selected. |
| Inputs | PopUp | This field contains either the transfer Inputs entry selected from the popup, or the last transfer Inputs entry added by clicking on the AddInput button. Also, the selection is updated if the SelectedConfig or SelectedState are changed. |
| AddInput | Radio | When this button is clicked, an input entry is added to the XfrCfgInput table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. The remaining fields must be entered manually. Use the right mouse button (click on Inputs) to open the XfrCfgInput table to update the remaining fields. Each transfer may be configured with any number of input entries for any of the available states. For example, a transfer input “Bed Down – Is On” is required for “State 1”. Transfer input “Bed Raised” is required for “State 2”. The XfrCfgInput entries are created by selecting “State 1”, clicking on AddInput, clicking on NextState, and clicking on AddInput again. This adds two input records associated with the selected XfrConfig – for “State 1” and “State 2”. |
| Outputs | PopUp | This field contains either the transfer Outputs entry selected from the popup, or the last transfer Outputs entry added by clicking on the AddOutput button. Also, the selection is updated if SelectedConfig or SelectedState is changed. Note: The XfrCfgOutput table has columns for each transfer state. Select the “State Action” that will be performed when the transfer is in a specific state. If “N/A” is selected for a “State Action”, the IO will be left in the prior state. DoneAction is automatically set to “DeActivate” when a new record is added. For example, if a transfer “raise solenoid” is activated in “State 2”, select “Activate” for the “State2Action”. If the solenoid is to be deactivated when the transfer is “Done”, no further entries are required (Deactivate is automatically set when the record is added (by clicking AddOutput). |
| AddOutput | Radio | When this button is clicked, an entry is added to the XfrCfgOutput table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - DoneAction – Set to Deactivate. The remaining fields must be entered manually. Use the right mouse button (click on Outputs) to open the XfrCfgOutput table to update the remaining fields Each transfer may be configured with any number of Outputs entries. For Example, a transfer output “Accum Release – UnDisable” is required for “State 1”. Transfer output “Bed Raise - Fire” is required for “State 2”. The XfrCfgOutput entries are created by selecting “State 1”, clicking on AddOutput, clicking on NextState, and clicking on AddOutput. This adds two output records associated with the selected config – for “State 1” and “State 2”. Select the conveyor part for “Accum Release” manually in the Output column. Select Activate manually for “Accum Release” “State1Action”. Select the conveyor part “Bed Raise” manually in the Output column. Select “Activate” for “Bed Raise” State2Action. The conveyor parts are turned OFF by the “DoneAction – DeActivate”, (auto-populated value from the AddOutput pushbutton). |
| InTimers | PopUp | This field contains either the transfer InTimers entry selected from the popup, or the last transfer InTimers entry added by clicking on the AddInTimer button. Also, the selection is updated if the SelectedConfig or SelectedState are changed. |
| AddInTimer | Radio | When this button is clicked, an InTimers entry is added to the XfrCfgInTimer table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. The remaining fields must be entered manually. Use the right mouse button (click on InTimers) to open the XfrCfgInTimer table to update the remaining fields. Each transfer may be configured with any number of XfgCfgInTimer entries for any of the available states. For example, a transfer InTimers entry of “Right FullClear” – “Is Done” is required for “State 1”. The XfrCfgInTimer entries are created by selecting “State 1” and clicking on AddInTimer. This adds one InTimers record associated with the selected XfrConfig, for “State 1”. The InTimers “RightFullClear” and “Is Done” entries must be selected manually. |
| DelayTimers | PopUp | This field contains either the transfer DelayTimers entry selected from the popup, or the last transfer DelayTimers entry added by clicking on the AddDelayTimer button. Also, the selection is updated if SelectedConfig or SelectedState are changed |
| AddDelayTimer | Radio | When this button is clicked, a DelayTimers entry is added to the XfrCfgDelayTmr table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. - Timer – An auto-generated timer is added. - TimerAction – The timer action is set to “RunTimer”. The remaining fields must be entered manually. Use the right mouse button (click on DelayTimers) to open the XfrCfgDelayTmr table to update the remaining fields. Each transfer may be configured with a DelayTimers entry for each State; however, a State may only have one DelayTimer entry. For example, a transfer DelayTimers entry is required for “State 1”. The timer will delay completion of “State 1” by 200 milliseconds (this allows the carton to feed onto the transfer far enough before sending it to the right). The XfrCfgDelayTmr entry is created by selecting “State 1” and clicking on AddDelayTimer. This adds one DelayTimers record associated with the selected XfrConfig – for “State 1”. The DelayTimers entry is created and added to the table with a TimerAction of “RunTimer”. |
| Send Msg | PopUp | This field contains either the transfer Send Msg selected from the popup, or the last transfer Send Msg added by clicking on the AddMsg button. Also, the selection is updated if SelectedConfig or SelectedState are changed. |
| AddMsg | Radio | When this button is clicked, a Send Msg entry is added to the XfrSndMsg table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. - Enable – Set to Y. The remaining fields must be entered manually. Use the right mouse button (click on Send Msg) to open the XfrSndMsg table to update the remaining fields. Each transfer may be configured with any number of Send Msg entries for any of the available states. |
| Param Process | PopUp | This field contains either the transfer Param Process selected from the popup, or the last transfer Param Process added by clicking on the AddParam button. |
| AddParam | Radio | When this button is clicked, an entry is added to the XfrParamConfig table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. - Enable – Set to Y. Each transfer may be configured with any number of “Done Process” entries. For example, a transfer tree (when completed) will copy the parameters from a ‘box closer’ device into parameters 1 through 3. |
| Trig Next | PopUp | This field contains either the transfer Trig Next selected from the popup, or the last transfer Trig Next added by clicking on the AddTrig button. |
| AddTrig | Radio | When this button is clicked, an entry is added to the XfrTrig table. The following fields in the record are updated: - Name – An auto-generated name is added. - CurrXfrTree – The currently-selected tree is added. - Enable – Set to Y. Each transfer may be configured with any number of TrigNext entries. For example, a transfer tree (when completed) will trigger a CRR message to CommCore and will receive a response CRR that will activate a transfer move based on the destination returned. |
| Errors | PopUp | This field contains either the transfer Error selected from the popup, or the last transfer error added by clicking on the AddError button. Also, the selection is updated if SelectedConfig or SelectedState are changed. |
| AddError | Radio | When this button is clicked, a timeout Errors entry is added to the XfrErrConfig table. The following fields in the record are updated: - Name – An auto-generated name is added. - XfrConfig – The currently-selected XfrConfig is added. - State – The currently-selected State is added. - Enable – set to Y. The remaining fields must be entered manually. Use the right mouse button (click on Errors) to open the XfrErrConfig table to update the remaining fields. Each transfer may be configured with any number of Errors entries for any of the available states. |
| XfrState | String | This field is updated with the state name from XfrBoss_pState in the current XfrBoss record. |
| XfrStatus | String | This field is updated with the contents of the Status from the selected XfrBoss record. |
| CfgState | String | This field is updated with the state name from XfrConfig_pState in the current XfrConfig record. |
| CfgStatus | String | This field is updated with the contents of the Status from the selected XfrConfig record. |
| LstDest | String | Shows the last destination processed. Taken from HostZone in the tracking record. |
| LstScan | String | Shows the last scan processed. Taken from ConfirmScan in the tracking record. |
| PrsPE | PopUp | The presence photoeye device for the selected transfer. Taken from XfrBoss_ProductINPUT. |
| LstTrk | PopUp | The tracking record index for the selected transfer. Taken from either XfrBoss_pItemIndex or XfRouteBoss_pTrkIndxQue. |
| StartXfr | Radio | Click this button to start the selected transfer. |
| ResetXfr | Radio | Click this button to reset the selected transfer. |
| Status | String | Status based on the last screen selection. |
| Error | String | Error based on the last screen selection. |
| Comment | String | Comment based on the last screen selection. |

## Example Transfer Layout

The following example shows the relationship between the XfrBoss, XfrTree and XfrConfig tables for a transfer with two input points (two entries in the XfrBoss table) and six paths (six entries in the XfrTree table) and eight transfer sections (eight entries in the XfrConfig table).

XfrBoss Table

1. Input 1

2. Input 2

XfrTree Table

Path A

Path B

Path C

Path D

Path E

Path F

XfrConfig Table

Transfer Section 1

Transfer Section 2

Transfer Section 3

Transfer Section 4

Transfer Section 5

Transfer Section 6

Transfer Section 7

Transfer Section 8

Top-Level Transfer Tables

The conveyor transfer diagram indicates there are six possible routes (A through F).  Each route is broken down into eight transfer sections.  Each route uses different groups of transfer sections:

Route A – Sec 1, Sec 2

Route B – Sec 1, Sec 3, Sec 4

Route C – Sec 1, Sec 3, Sec 5, Sec 6

Route D – Sec 7, Sec 8, Sec 2

Route E – Sec 7, Sec 4

Route F – Sec 7, Sec 5, Sec 6

F		Input Point 2

E

D

Output Point 3

Output Point 1

Input Point 1	Route B

Route C

Route A

Output Point 2

Example Transfer with six Routes

Input Point 1

Sec 6

Sec 5

Sec 8

Sec 3

Sec 2

Sec 7

Sec 1

Sec 4

Output Point 2

Output Point 1

Input Point 2

Output Point 3

Example Transfer with eight Transfer Sections

## Scan Process – Scan Events

The scan processing module processes scan events and stores the scan data in a tracking record.  The tracking table column ProcFlag is used to indicate when a scan event occurs.

## Transfer Scan Event Processing

Once the scan data has been processed, the transfer module detects the scan events by monitoring the ProcFlag column.  The XfRouteBoss table contains a link to the scan zone associated with a scanner.

## Scan Message Processing

When a scan event is processed, a CRR message is sent to CommCore, which will respond with one or more destinations.  Alternatively, simple routing may be performed using the SrtFPLite protocol when the barcode contains the destination.

## Route Queue

When a CRR response is received, the route processing software will determine if the route or routes are available.  If a route is available (not full and all inputs in State 1 are in the required states) the route will be executed.  If the route is not available, the tracking record will be queued for repetitive routing attempts (every iteration of the process).

## Transfer Step 1 (Boss Idle)

A transfer is initiated (once the CommCore response is received and the destination determined) by calling the transfer induct routine with a XfrBoss record, a destination, a tracking table (to record the “verify” event), and a tracking table index.  Once complete, XfrBoss_pState is set to ‘Boss Search’.

NOTE:  The transfer control function “XfrProcess CTRL” must be set up in the Batches table.

## Transfer Step 2 (Boss Search)

If the destination is available, the transfer will place the search for the destination in XfrTree_Dest and then put the transfer into “Search Mode”.

After locating the destination, XfrTree is checked to find the paths that must be “Idle” before the transfer can proceed.  The XfrCheckSet entries assigned to the XfrBoss record are checked in the XfrCheckSet table.  Each transfer section in the XfrCheckSet table must be in a state of XFR_IDLE.  If not, the XfrBoss record will remain in a search state.

If the XfrCheckSet test is successful, the XfrGoSet is checked to see if each XfrCfgInput and XfrCfgInTimer entry is in the ready state.  Since the transfer IO has already been checked, the destination is not full and the container is present (or any inputs and input timers configured in “State 1” are ready), the transfer will begin execution.

If the XfrCfgInput and XfrCfgInTimer entries for “State 1” are ready, the XfrConfig records referenced in XfrGoSet will be started at the state defined.  The initial transfer state is normally set to Idle, but some transfer sections are reused for different routes and may be set to different starting states for each route.

## Transfer Step 3 (Boss Wait)

If the XfrCheckSet and the XfrGoSet tests indicate the transfer is ready, it will start the configuration records found in XfrGoSet (The pRun column is set to Y).  When the XfrConfig moves from “Boss Idle” to any other state, XfrBoss_pState will move from “Boss Wait” to “Boss Run”.

## Transfer Step 4 (Boss Run)

Once the transfer is active (XfrConfig_pRun = Y), the XfrConfig steps through the transfer logic based on the XfrCfgInput and XfrCfgInTimer entries configured for each state.

## Transfer Step 5 (Boss Done)

When the XfrConfig entries are complete (all the XfrConfig_pRun(s) = N in XfrGoSet).  When all the XfrConfig entries are back in an IDLE state, the XfrBoss will move to the DONE state, completing the transfer cycle.  At this point, another transfer cycle may be initiated.

# Transfer Scan Processing and Messaging

The transfer module requires the following entries in the Batches table:

- ScnApp CTRL – Required to process incoming scans.

- Xfr App SERV – Required to run the “SERV” portion of the software.

- Xfr App CTRL – Required to run the “CTRL” portion of the software.

- XfrProcess (CTRL) – Required to run the “CTRL” for transfers.

- Xfr App Sim CTRL – Required to run the “CTRL” simulator software.

The transfer control logic uses the standard transfer control functions (Transfer Boss and Transfer Tree) along with specific routing control logic to provide scanner input, container tracking, scanner error processing, full lane control, and CommCore messaging.

The transfer control module provides the ability to configure and control multiple transfers within one AC or across multiple ACs.

The following sections describe the tables used to configure transfers.

# XfRouteBoss Table – Transfer Scans and Messaging

The XfRouteBoss table defines the link to the scan zone in table ScnScanZone.  Learning the container’s identity is the first step in determining its destination.  The scan processing module is also used to define scan zone error processing.

XfRouteBoss also defines connections to message tables used to request routing information from CommCore.

Each XfRouteBoss entry may have multiple XfRouteTable entries.  XfRouteTable defines an agreed-upon string in HostRouteStr that links destinations sent from CommCore to a transfer route.

Note:  Scan error routing assignments are no longer controlled by the AC.  Errors are processed by CommCore, which will make explicit destination assignments for errors, however XfRouteBoss_CommErr is still used to assign a route destination if CommCore does not respond to a routing request.  Likewise, XfRouteBoss_ScanError can still be used in applications where no CommCore connection is available or required.

Table XfRouteBoss defines links to four timers that are used to detect scan error events: “No Scan”, “No Route”, “No Carton” and “Carton Removed” events.  The timers and other configuration options provide the ability to detect and respond to the four types of exception events:

- No Scan Event (NoScanTmr) – A carton was detected at the transfer, but a scan message was not received.

- No Route Event (NoRouteTmr) – A carton was detected and scanned at the transfer, but no routing data was received for the carton.

- No Carton Event (NoCartonTmr) – Scan data was received, but a carton was not detected.

- Carton Removed Event (ClrQueDlyTmr)– A carton was detected, but the carton was removed.

Note that XfRouteBoss is used for both “Old Style” and “New Style” transfers.

| XfRouteBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer. |
| Desc | String | Optional additional descriptive text. |
| pScanStatus | String | The software will update this to indicate the status of the last message processed. |
| pXfrErrorMsg | String | The software will update this if errors are detected during message processing. |
| LogLevel | PopUp | Choose the level of detail for the log files. Set to Level 1 by default. Error messages only with Level 0. Additional detail on available routes with Level 2. Debug detail with Level 5 – do not leave this set to Level 5, it will fill the disk! |
| ScanZone | PopUp | Choose the scan zone from table ScnScanZone associated with this transfer. |
| EsmMsgTable | PopUp | Choose the table to hold External Scan Message (ESM) data. The ESM is used to trigger a transfer from an external source. For example, a container may be manually scanned directly to CommCore. The scan data is sent to the AC using an ESM. The ESM message contains a “decision point name” that must match the string defined in ScnScanZone_ScanZoneID. To use an ESM to execute a move to a destination, use the ESMRoute field. When an ESM is used to trigger a CRR, do not use ESMRoute (see below). |
| EssMsgTable | PopUp | Choose the table to hold External Scan Status (ESS) message data. An ESS message is sent in response to an ESM if a container is present at the decision point mentioned in the ESM. |
| CrrMsgTable | PopUp | Choose the table to hold Container Routing Request (CRR) data. For transfers, start with MsgXR1. Since transfers have much lower message volume than sorters, they can share one CRR message table. You may opt to use multiple tables for clarity, if desired. To generate a CSM instead of a CRR, use the next field. |
| CsmMsgTable | PopUp | Choose the table to hold Container Status Message (CSM) data. For transfers, start with MsgXR1. This is an alternate message that may be sent instead of a CRR. A CRR is sent to request routing of a container. A CSM is sent to indicate that a scan event has occurred at a decision point, but CommCore might not return routing information in its response. |
| DcmMsgTable | PopUp | Choose the table to hold Divert Confirm Message (DCM) data. For transfers, start with MsgXDC1. Since transfers have much lower message volume than sorters, they can share one DCM message table. You may opt to use multiple tables for clarity, if desired. |
| CsuMsgTable | PopUp | Choose the table to hold Container Status Update (CSU) data. For transfers, start with MsgXDC1. Since transfers have much lower message volume than sorters, they can share one CSU message table. You may opt to use multiple tables for clarity, if desired. |
| SendAvailStatus | Y/N | The status of each destination within this transfer can be sent along with the routing request. Also known as the ‘not available list’, the host strings for each destination that is not available are sent in a comma-separated list in the sZone1 field of the CRR message. In the list of possible valid destinations in the CRR response, CommCore will put the available lanes first and will usually include the unavailable lanes at the end of the list in case they become available. FPC always starts at the beginning of the list and works its way down. N = Do not send the available/full status information Y = Send the list of lanes that are not available. |
| AvailTransfer | Int | Normally, this is set to 0 to report the Available/Full status for this transfer. If the Available/Full status of a different transfer needs to be reported in the CRR message, enter the index of the row in table XfRouteBoss for that transfer in this field. If necessary, you can use this to create a transfer representing a group of zones for the purpose of sending their status to CommCore to assist in routing decisions. This virtual transfer is not used for physical control – only to collect data from a group of zones. |
| Straight | PopUp | Choose the XfRouteTable record associated with the straight route. Normally, CommCore will assign all routes, so this is not needed, but it doesn’t hurt to define it. |
| CommErr | PopUp | Choose the XfRouteTable record to use in the event of a communications error. This is used to route the container when CommCore does not respond with routing within the number of seconds defined by NoRouteTmr. It will also be used if the response does not include a destination. If nothing is chosen here, the container will wait indefinitely for a response. |
| ScanError | PopUp | Choose the XfRouteTable record to use when a scan error occurs. This is only used for “Fortna Lite” applications – with the SrtFPLite protocol. |
| NotDefined | PopUp | Choose the XfRouteTable record to use if the routing string returned in the CRR does not match any entries in XfRouteTable_HostRouteStr. If all expected routes are defined in XfRouteTable, this will never be needed, but it doesn’t hurt to define it. |
| NoData | PopUp | Not Used |
| RouteFull | PopUp | Choose the XfRouteTable record to use if the desired route is full or unavailable. |
| ESMRoute | PopUp | If a route needs to be executed for an ESM, choose it from XfRouteTable. |
| MaxWaitTime | Int | Not Used |
| NoScanTmr | PopUp | This timer is used to detect “No Scan” events, which occur if a container moves into the transfer, activating the “Presence Photoeye”, but the scanner does not send data. - The timer is automatically populated when FPC starts. - The default is 10 seconds. Adjust the time as needed for the case where the container activates the presence PE before the scan data is sent. - If the timer expires, the container will be assigned a scan string of “NOSCAN”, and the ScanError route will be used. |
| NoRouteTmr | PopUp | This timer is used to detect “No Route” events, which occur if a container moves into the transfer, has scan data, but does not receive routing. - The timer is automatically populated when FPC starts. - The default is 10 seconds. - If the timer expires before CommCore responds to the CRR message, a No Route error occurs. This can be due to network issues or internal CommCore issues. - If NoRouteRetry = N, and the CommErr route is set to N/A, the container will wait for routing forever. - If NoRouteRetry = N, and the CommErr route is set to a valid route, the container will be sent there when the timer expires. - If NoRouteRetry = Y, the CRR message will be re-sent each time the timer expires until an answer is received. |
| NoCartonTmr | PopUp | This timer is used to detect “No Carton” events, which occur if a scan event occurs, but a container does not activate the presence PE before the timer expires. - The timer is automatically populated when FPC starts. - The default is set to 10 seconds. Adjust the timer to allow enough time for the container to arrive at the presence PE in all cases after the scan data is received. - “No Carton” event processing is used to clean up a queued transfer route, or to clean up incomplete scan data. |
| ClrQueDlyTmr | PopUp | This timer is used to detect when a container is physically removed from a transfer. A “container removed” event occurs if the presence PE detects a container, but the PE goes OFF before the container is released by the software. - The timer is automatically populated when FPC starts. - The default is set to 0.05 seconds. - The timer must be set to a value that is small enough to detect “Push through” events, which may occur if operators push the containers, or when pressure from upstream containers forces them through. - The timer must be set to a value that is large enough to not clear the tracking information when a “bounce” event occurs, such as if the container triggers the presence PE multiple times by bouncing or rocking as it arrives. - If the timer completes, the tracking record will be removed. |
| DontCheckNoScan | Y/N | N = A NoScan event will be detected if a container moves into the transfer, and activates the presence PE, but the NoScanTmr expires before scan data arrives. This is the normal behavior Y = The NoScan processing logic is deactivated. |
| DontResetQue | Y/N | N = A “No Carton” event will occur if a scan is received but the presence PE is not activated before NoCartonTmr expires. This is the normal behavior. Y = The “No Carton” processing logic is deactivated. |
| DontResetRcv | Y/N | N = Discard scan data if the presence PE does not turn ON before NoCartonTmr expires. This is the normal behavior. Y = The “Reset Receive” processing logic is deactivated. |
| NoRouteRetry | Y/N | N = A “No Route” even will occur if no response is received for the CRR message from CommCore before NoRouteTmr expires. If the CommErr route is defined, the container will be sent there. If the CommErr route is set to “N/A”, the container will wait forever. Y = The CRR message will be re-sent each time NoRouteTmr expires until a response is received. |
| SystemRun | PopUp | Choose an IO signal that will start and stop the transfer in the sense that it will control these timers: NoScanTmr, NoRouteTmr, NoCartonTmr, ClrQueDlyTmr |
| WaitPresence | Y/N | For “old” transfers. For “new” transfers, this is effectively always set to Y. N = Start executing the transfer immediately. Y = Wait for the presence PE to turn ON before executing the transfer move. Always use Y for “old” transfers! |
| XferCnvMtr | PopUp | Choose the motor that will be disabled when a transfer error occurs. Transfer errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| MtrACTION | PopUp | Choose the action to take when a transfer error occurs. |
| MtrBIT | Int | Enter the bit to use if MtrACTION is Enable or Disable. |
| ScanErrSig | PopUp | Choose a memory bit to be activated when a transfer scan error occurs. Transfer scan errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| ScanACTION | PopUp | Choose the action to take when a transfer scan error occurs. |
| ScanErrBIT | Int | Enter the bit to use if ScanACTION is Enable or Disable. |
| DataErrSig | PopUp | Choose a memory bit to be activated when a transfer data error occurs. Transfer data errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| DataACTION | PopUp | Choose the action to take when a transfer data error occurs. |
| DataBIT | Int | Enter the bit to use if DataACTION is Enable or Disable. |
| RunNewXfr | Y/N | Y = Use the “new” transfer logic. N = Use the “old” transfer logic. |
| CRRReadyToSend | PopUp | Choose an IO signal that will control when the CRR message is sent. You can use this (and CRRSendSense) to delay sending the CRR message until the presence PE turns ON, for instance. |
| CRRSendSense | PopUp | Choose the required state needed for the CRRReadyToSend signal. |
| SendOneCRR | Y/N | Use this to keep from sending more than one CRR if multiple scans are received while the transfer is in progress. A scanner may generate multiple scans due to push-through events. If the flag is set to Y, only one scan event will be generated. This is especially important in print-apply applications, where an extra CRR will generate an extra label, causing “out of sync” issues. A setting of Y will prevent that. N = A CRR will be sent for each scan event. Y = Only one CRR message will be generated even if additional scans are received. |
| ToteSensor | PopUp | Choose an input device to indicate a tote has been detected. Selecting something here activates the feature. |
| TotePresentStr | String | The string to be sent in the CRR or CSM message if ToteSensor detected a tote. |
| ToteAbsentStr | String | The string to be sent in the CRR or CSM message if ToteSensor did not detect a tote. |
| pProductState | Y/N | Set by the software to indicate the state of the presence PE. |
| pRouteTreeQue | Int | Used by the software to store a XfrTree index reference while waiting for it to be available for assignment. |
| pTrkMenuQue | PopUp | Used by the software to store a tracking table entry while waiting for a route to become available. |
| pTrkIndxQue | Int | Used by the software to store a tracking table index reference while waiting for a route to become available. |
| pProcFlag | Int | Used by the software to trigger the SERV process when scanner data is received. |
| pXfrError | Int | Used by the software to store the error status of the transfer. |
| pGrtHalfSecond | Int | Updated by the software to indicate the number of responses that have exceeded 0.5 second. |
| pGrtQtrSecond | Int | Updated by the software to indicate the number of responses that have exceeded 0.25 second. |
| pMaxResponse | Float | Updated by the software to indicate the longest response time from CommCore. |
| pNumSamples | Int | Updated by the software to indicate the number of responses processed. |
| pScanError | Int | Internal flag used to set ScanError to the required state. |
| pDataError | Int | Internal flag used to set DataError to the required state. |
| pWrkScanError | Int | Internal flag used to process scan data. |
| pWrkDataError | Int | Internal flag used to process scan data. |
| pLastScan | String | Internal string used to record the last scan received (used with verify processing). |
| pLastScanCnt | Int | Internal flag used to track scan events (used with pLastScan). |
| pLastScanProc | Int | Internal flag indicating if the last scan was processed (used with pLastScan). |
| pParamConfigError | Int | Internal flag used for parameter processing. |
| pTimeoutError | Int | Internal flag used for timeout processing. |
| pESMAck | Int | Internal flag used to trigger the ESS send. |
| pTotePresent | Int | Internal flag used to latch the state of ToteSensor. This is reset when a CRR or CSM message is sent. |
| pLastPack | String | Internal string used to record the last pack scan received (used with verify processing). |
| pLastPackCnt | Int | Internal flag used to track pack scan events (used with pLastPack). |
| pLastPackProc | Int | Internal flag indicating if the last pack scan was processed (used with pLastPack). |

# XfRouteTable Table – Transfer Destination Configuration

The transfer routing table, XfRouteTable, contains entries that define the routes for each XfRouteBoss record (or scanner – or decision point in CommCore lingo).  The HostRouteStr field contains a string that must match the string received from CommCore.

The table also includes fields to configure an ErrorIO for use if a transfer has experienced a timeout associated with a route, or if the transfer has experienced a “Transfer Reset”.  The HRXfrResets field is also updated, for use by the History Module, based on the number of events detected.

A transfer timeout and reset may occur if the CRR routing response is not valid or if the response is not received within the configured time, or if the transfer does not complete the routing sequence in the configured time.  A transfer reset often indicates the transfer is not functioning properly and may require maintenance.

| XfRouteTable Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer route. |
| Enabled | Radio | N = This route is not available for assignment. Y = This route is available for assignment. If the CRR response includes more than one possible route, the available routes will be selected in the order listed in the message. If no available routes are found and the transfer is configured with a XfRouteBoss_RouteFull option, the container will be routed there, otherwise it will wait until a route becomes available. |
| BossRecord | PopUp | Choose the XfRouteBoss entry associated with this transfer. |
| HostRouteStr | String | Enter the agreed-upon string that identifies this transfer route. It will be matched against the “divert point” string returned in the sScanner field of the CRR response. When used with the SrtFPLite protocol, this string must match the destination encoded in the tote barcode. |
| Route | PopUp | For “old” transfers. Choose the XferTree entry associated with the route. This is the route that will be assigned if the route is available. |
| NewRoute | PopUp | For “new” transfers. Choose the XfrTree entry associated with the route. This is the route that will be assigned if the route is available. |
| AutoResetCons | Y/N | Controls the automatic reset of consecutive errors. Y = Automatically reset pConsecErrors each time a container successfully transfers. N = Do not automatically reset the error count. Once MaximumErrors is reached, a manual reset is required. |
| MaximumErrors | Int | Enter the number of consecutive errors that will trigger ErrorIO. Setting this to 0 disables the feature. |
| ErrorIO | PopUp | Choose an IO part to activate when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO when MaximumErrors has been reached. The inverse action will occur when pConsecErrors is less than MaximumErrors. |
| ErrorBIT | Int | Enter the bit to use if ErrorACTION is Enable or Disable. |
| ErrorSignal | PopUp | Choose an IO signal that will be set ON when MaximumErrors is reached. This is usually a memory bit for use in table Errors to activate an error. The signal will be set OFF when pConsecErrors is less than MaximumErrors. |
| ResetCons | Y/N | Set this to Y to manually reset the consecutive error count in pConsecErrors. The software will automatically set the column back to N. |
| ResetConsSignal | PopUp | Choose an IO signal that will trigger a reset of the consecutive error count (in pConsecErrors) when the signal turns ON. |
| HRTotalDiverted | PopUp | Defines the history record (in table HistConfig) associated with total diverts for this route. Usually configured using the History Wizard. |
| HRFullAborts | PopUp | Defines the history record (in table HistConfig) associated with full aborts for this route. Usually configured using the History Wizard. |
| HRXfrResets | PopUp | Defines the history record (in table HistConfig) associated with transfer resets for this route. Usually configured using the History Wizard. |
| DSRRequest | PopUp | Use this to link to a DSR configuration so CommCore can set the enable status of this route. Each DSR message may contain status information on up to ten routes. If more than ten routes are required, the status must be requested using multiple messages. |
| pRouteFull | Int | Set by the software to indicate a route full condition has occurred. |
| pConsecErrors | Int | Set by the software to track the number of consecutive errors that have occurred for the route. |

# XfrTrack and XfrDevice Tables – Transfer Tracking

The Transfer tracking tables XfrTrack and XfrDevice are used to manage information on containers in-process on one or more transfers.  You can inspect the data held there to verify the system is working correctly and to review recent scan events.

A XfrTrack record is created for each container as it arrives at a transfer.

Records are added to XfrDevice when each scan event occurs.  Each XfrDevice record is associated with the XfrTrack record for the container.  If multiple scan devices are defined for the scan zone associated with a transfer, each one will generate a XfrDevice record, all linked to the XfrTrack record.

The default table size for both the XfrTrack and XfrDevice tables is 100 records, but they can be expanded if needed.  If multiple scan devices are used for a single transfer, increase the size of the XfrDevice table to reflect that multiple.

As each scan is received, the scan data is stored in XfrDevice along with status information that indicates if any errors occurred during the scan processing.

The response from CommCore is stored in XfrTrack along with the elapsed time for the message round trip.

Note: XfrTrack and the SrtTrack tables have the same structure so they can be processed by the same functions.  Some of the fields are not applicable for use with transfers.

| XfrTrack Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Set by the software to either the row number, like “R57” or “=====>>>>” to indicate the last record updated. |
| ConfirmScan | String | The container scan string. Sent to CommCore. |
| ScanQty | Int | The number of scans received for the zone. |
| Status | String | Set by the software to show the status as data is received and processed. |
| ErrorMsg | String | Error messages will appear here. |
| HostZone | String | Indicates the zone assigned as the next destination for the container. |
| ConfirmTote | String | If a scan is defined as a tote scan, it will appear here. Sent to CommCore. |
| CombScan | String | If multiple scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombScanSts | PopUp | The status of the CombScan data. |
| CombTote | String | If multiple tote scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombToteSts | PopUp | The status of the CombTote data. |
| CombSKU | String | If multiple SKU scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombSKUSts | PopUp | The status of the CombSKU data. |
| Z1 thru Z10 | String | The CRR response from CommCore may contain up to ten destination options. The AC will choose the first one that is available. |
| ScanZoneID | String | Indicates the scan zone name, taken from ScnScanZone_Name. |
| ScanZoneIndx | Int | The row number in table ScnScanZone for ScanZoneID |
| SorterLane | Int | not applicable for transfers |
| SrtAppRec | Int | not applicable for transfers |
| Shifter | Int | not applicable for transfers |
| Ndx | Int | not applicable for transfers |
| ProcFlag | Int | Set by the software. Indicates scan data has been received and a message must be generated. |
| ProcUpd | Int | Internal flag that indicates this record requires processing. |
| SrtZoneLaneRec | Int | not applicable for transfers |
| CommSent | Int | Used to track if a CRR has been sent when XfRouteBoss_SendOneCRR = Y. |
| TooLate | Int | not applicable for transfers |
| SendCRRTrigger | Int | not applicable for transfers |
| ElapsedTime | Float | Indicates the time, in seconds, it took to send a message to CommCore and to receive the response. |
| CommSentSec | Int | Records the time, in whole seconds, when the message was sent to CommCore. |
| CommSentMSec | Int | Records the millisecond portion of the time beyond CommSentSec when the message was sent to CommCore. |
| TimeStamp | Time | Records the time when the record was initialized (usually at the time of the scan event). |
| TampStatus | PopUp | not applicable for transfers. Used for Print Apply applications. |
| PaProcFlag | Int | not applicable for transfers. Used for Print Apply applications. |
| PaProcUpd | Int | not applicable for transfers. Used for Print Apply applications. |
| ExptRF | String | not applicable for transfers. Used for Print Apply applications. |
| ExptSKU | String | not applicable for transfers. Used for Print Apply applications. |
| ExptLPN | String | not applicable for transfers. Used for Print Apply applications. |
| VerRF | PopUp | not applicable for transfers. Used for Print Apply applications. |
| VerSKU | PopUp | not applicable for transfers. Used for Print Apply applications. |
| VerLPN | PopUp | not applicable for transfers. Used for Print Apply applications. |
| CurrSpeed | Int | not applicable for transfers. Used for Print Apply applications. |
| Height | Float | not applicable for transfers. Used for Print Apply applications. |
| Width | Float | not applicable for transfers. Used for Print Apply applications. |
| Length | Float | not applicable for transfers. Used for Print Apply applications. |
| TooShort | Int | not applicable for transfers. Used for Print Apply applications. |
| SendUpdPrt | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyErr | Int | not applicable for transfers. Used for Print Apply applications. |
| LastTree | String | Indicates the transfer tree assigned in the transfer boss. |
| RouteBossRec | Int | Indicates the XfRouteBoss associated with this tracking record. |
| EmptyConfirm | Int | Not currently used |
| pConfirmTrig | Int | The confirm process uses this flag to indicate the transfer has completed the cycle and a DCM message needs to be sent to CommCore. |
| RouteTableRec | Int | The index in table XfRouteTable associated with this record. |
| Param1 - Param5 | String | Optional parameters may be passed in the CRR and DCM messages. They are stored here. |
| ConfirmShip | String | The container ship scan string. Sent to CommCore. |
| ConfirmPack | String | The container pack scan string. Sent to CommCore. |
| CombShip | String | If multiple ship scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombPack | String | If multiple pack scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombShipSts | PopUp | The status of the CombShip data. |
| CombPackSts | PopUp | The status of the CombPack data. |
| ESMInductTime | String | Time string from CommCore. Used to link to the ESS message response in sInductionTime (Dat5). |
| pTimeOutFlag | Int | Used to track whether the transfer has timed out for this record. |
| DCMStatus | Int | Used to track DCM messages. |
| XfrZoneID | String | For use with transfer print-apply applications. Taken from XfrBoss_XfrZoneID. |
| pVerifyFlagTop | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagLeft | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagRight | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagFront | Int | not applicable for transfers. Used for Print Apply applications. |
| pInitialTrkIndx | Int | Used by the software to store the original tracking index if this tracking record was created as the result of one transfer triggering another transfer. |
| pPNAExempt | Int | not applicable for transfers. Used for Print Apply applications. |
| LRMSent | Int | not applicable for transfers. Used for Print Apply applications. |
| LVMSent | Int | not applicable for transfers. Used for Print Apply applications. |
| UUID | String | not applicable for transfers. Used for Print Apply applications. |

| XfrDevice Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Set by the software to either the row number, like “R23” or “=====>>>>” to indicate the last record updated. |
| TrkIndx | Int | The index of the associated row in table XfrTrack. |
| Scan | String | The string received from the scanner device. |
| ScanSts | PopUp | The status of the Scan data. |
| ScanType | PopUp | The scan device type as defined for this scanner in ScnScanDevice_ScanType. |
| ScanZone | PopUp | The scan zone for this scan device as defined in ScnScanDevice_ScanZone. |
| TimeStamp | Time | Indicates the time when the record was initialized (usually at the time of the scan event). |

# MsgXR1 and MsgXDC1 Tables – Transfer Messages

Table MsgXR1 is the first of nine ‘XR’ tables.  They are intended for use with CRR or CSM messages.  They are structured the same as all the ‘Msg’ tables, like MsgCR1, so any of them could be used.  The convention is to use the ‘XR’ tables for transfers.

Unlike sorters, transfers consume message table records more slowly, so it is safe to use the same message table for many, often all, transfers configured on a single AC.  You can split them up if you like, however, to group them by area or purpose, or whatever make the most sense for the situation.

Tables MsgXDC1 – MsgXDC9 are similar, and are intended for use with DCM messages.

The key features of the ‘Msg’ tables are:
CmdHdr – Contains the message type, like CRR
Header – Contains the AC name
Numbered Col / Dat column pairs – Contain the data in key / value pairs where the pairs are defined for each message type.  There is room for up to 32 pairs, though each message type and even the same message type in different software versions may use different numbers of pairs.  The CommCore instance may also determine the data pairs required.  The first pair, in Col1 / Dat1, is always iQty and the number of column pairs used.
TxStat – Records the send / receive status of the message.
DbKey – A code number used to pair the response with the original message when it is received.
DbTime – A time stamp in YYYYMMDDHHMMSS format
iDbTime – A Unix timestamp integer for the creation time of the message.

See the sections on the message types for details on the contents of these messages.

# XfSimConfig Table – Transfer Simulator Configuration

The Transfer Simulation Configuration table (XfSimConfig)is used to define a set of containers for simulations.

NOTE:  When using a configuration from a real project, use the SimCrrCsu field to create a CSU and a DCM without actually running the transfer.  This provides the ability to process a CRR response and respond with a CSU and DCM without creating a simulation transfer.

Transfers may be configured in a test environment to send carton routing requests to a test CommCore instance to test the system throughput and to test the interactions between the AC and CommCore.

You will need a transfer configuration with at least XfRouteBoss, XfRouteTable, XfrBoss and XfrTree entries.  The transfer should simulate a transfer event including a DCM.

| XfSimConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this simulation configuration. |
| EnableXfr | Y/N | Enable or disable this entry. |
| Status | String | The software will use this field to show status or error information. |
| AppTransfer | PopUp | Choose the XfRouteBoss record associated with this simulation. |
| NumContXfr | Int | Set the highest container number to assign. |
| CurContXfr | Int | The container number for the container currently being processed by the transfer. This number must be less than NumContXfr or the simulator will not induct containers! |
| NumCharScan | Int | The length, in characters, of the container ID string to generate. Must be no greater than 20. |
| PreFixString | String | Optional static string to start each container ID. You can leave this blank if it is not needed. |
| PostFixString | String | Optional static string to end each container ID. You can leave this blank if it is not needed. |
| ScanDevice | PopUp | Choose a scan device from the ScnScanDevice table. It is used to build the CRR message. |
| SimTimer | PopUp | Choose a timer to control the time between scan events. Set the timer Preset_Value as needed to simulate the throughput requirements for the transfer. |
| SimCrrCsu | Y/N | Determine if messages will be processed by the simulator. N = The simulation will not process any messages. Use this if you are connected to a “real” CommCore instance. Y = The simulation will process CRR, CSU and DCM messages received from the transfer for the decision point (ScnScanZone_ScanZoneID XfRouteTable_HostRouteStr). |
| InductEye | PopUp | Choose the transfer induct photoeye, XfrBoss_ProductINPUT. |
| InductCycleTimer | PopUp | Choose a timer to control InductEye. |
| ScanSimMenu | PopUp | Optional. Choose a table to draw the simulation container ID, scan and scale data from. Use XfrSimScans1 – XfrSimScans5. |
| SimScanFile | String | Optional. Define the full path to a file that contains the simulation data. The data will be loaded in to ScanSimMenu. |
| LoadFile | Radio | Click this to load data from SimScanFile. |
| WrapScanList | Radio | If checked, the simulation will re-use the scan list data in ScanSimMenu when it reaches the end of the list. |
| ResetSimulation | Radio | Click this to reset the simulation. The software will reset it and set pCurrSimRecord to 0. |
| pCurrSimRecord | Int | Set by the software to the row index of the record currently in use in table ScanSimMenu. |

# XfrBoss Table

The XfrBoss table defines a set of paths that a container can take as it moves through a transfer.

| XfrBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer. |
| Desc | String | Optional additional descriptive text. |
| pStatus | String | The software will use this to show the state of the transfer. |
| Dest | String | The software will set this to the current destination. You can also set a this manually when testing a transfer. Enter the string from XfrTree_Dest and set pState = ‘Search’. XfrConfig_pRun will change to Y and the transfer will execute. |
| CurrentTree | PopUp | The software will set this to the tree or route currently being executed by the transfer. |
| pState | PopUp | The software will set this to the current state of the transfer. |
| Reset | Y/N | Use this to manually reset the transfer. |
| LogLevel | PopUp | Choose the logging detail level. The wizard sets this to Level 1. Configuration errors are logged at Level 0. State changes and other basic functional details at Level 1. IO transitions and message triggers at Level 2. Configuration issues, verbose state change information and parameter processing details at Level 5. NOTE: Choosing Level 5 is a good way to fill the disk! |
| MaxWaitTM | PopUp | This timer controls how long the transfer will wait to complete its route before it initiates a transfer reset. If this timer times out while the transfer is still executing, XfrBoss_pState and XfrConfig_pState will be set to ‘Idle’. All outputs will be set to their ‘DeActivate’ states. This timer is created automatically using ConfigXfrScrn by clicking AddXfr. |
| ProductINPUT | PopUp | Choose the “product input” or “presence” photoeye. ProductINPUT and ReleaseOUTPUT are controlled automatically by the transfer to function like a “GoTo” photoeye. |
| ProductSENSE | PopUp | Set the “sense” of ProductINPUT to determine whether ON or OFF will initiate the transfer. Usually set to IsOn, which will cause ReleaseOUTPUT to stop when ProductINPUT transitions ON. |
| ReleaseOUTPUT | PopUp | Choose the output signal that will release a container to enter the transfer. ReleaseOUTPUT must be included as an output in “State 1” to allow the container to feed into the transfer. Works with ProductINPUT. |
| ReleaseAction | PopUp | Choose the action to take on ReleaseOUTPUT. |
| EnableMaxWait | PopUp | Choose an IO signal to control the MaxWaitTM timer. Use this if the transfer must be held for an external event. The timer is also suspended when XfrConfig_EnabledINPUT turns OFF, though in some cases it may be necessary to use this option – to detect when a downstream conveyor is stopped, for example. This will prevent a transfer reset. Often used with a system run signal that goes OFF when the line stops for things like E-Stop errors. |
| EnableMxSense | PopUp | Set the “sense” of EnableMaxWait to determine whether ON or OFF will halt the timer. |
| NoScanner | Y/N | To create a transfer that does not need a scanner, use NoScanner = Y and select a NoScannerRoute. The transfer will be triggered by the ProductINPUT photoeye. The route, selected from XfRouteTable, defines a link to XfRouteBoss, which links to a scan zone in ScnScanZone, which then links to a tracking table so a tracking record can be created. XfRouteTable also defines the destination and transfer tree. For example: A container arrives at photoeye “Xfr In PE”. The container needs to be moved forward to a blade stop. The XfrBoss entry is configured with ProductINPUT = “Xfr In PE”. The XfrConfig entry is set up to “Raise Blade Stop” in “State 1”. The XfrCheckSet entry is configured to make sure no other XfrConfig sequences are running before allowing the container to move (provides traffic control). The last step of the sequence is to set an internal bit to “Enable Xfr Right”. |
| Machine | PopUp | Choose the AC that controls this entry. |
| NoScannerRoute | PopUp | For use with NoScanner = Y. Choose the route that will be assigned when ProductINPUT is ON. If this field has no route selected, the software will search for a XfrTree_Dest of “PE”. An entry is also required here if the transfer executes a route that will collect parameter data and send the data with a DCM message. The container ID may come from a downstream scanner. |
| CompleteOneIter | Y/N | This item is used to prevent a race condition in applications where simultaneous moves can be made. N = The transfer requires two passes through the CTRL process to complete. Y = The transfer will be completed in one pass of the CTRL process. |
| XfrResetINPUT | PopUp | Choose an IO signal, such as a pushbutton, to initiate a transfer reset. See XfrResetSENSE. |
| XfrResetSENSE | PopUp | Set the “sense” of XfrResetINPUT to determine if the reset occurs when the signal transitions ON or OFF. |
| OverrideINPUT | PopUp | Choose an IO signal that will prevent the automatic start of the transfer. It also prevents sending of a CRR or CSM. See OverrideSENSE. pManualOverride is set when this signal is detected. |
| OverrideSENSE | PopUp | Set the “sense” of OverrideINPUT to determine if the override occurs when the signal transitions ON or OFF. |
| XfrZoneID | String | This field is used by the transfer print and apply module to define a unique name for the zone that this entry controls. When setting up a transfer print apply, one transfer is used to trigger another transfer to run using ProcessType = “NextXfrTrig” in table XfrTrig, which will create a new set of tracking records that will contain all the information, including this string. |
| pItemMenu | PopUp | Used by the software to store the DCM table. |
| pItemIndex | Int | Used by the software to store the current row in the DCM table. |
| pNoScanState | Y/N | Set by the software to indicate if the container has not received a scan. |
| pProductState | Y/N | Set by the software to indicate the current state of ProductINPUT. |
| pEnableMaxWaitState | Y/N | Set by the software to indicate the current state of EnableMaxWait. |
| pScrnUpdateTmr | Int | Set by the software to indicate the current value of the screen update timer. |
| pSwitchToSearch | Int | Indicates if the transfer is in “Search Mode”. |
| pAtProdInput | Int | Set by the software to detect when multiple containers activate ProductINPUT. ReleaseOUTPUT is activated when a second container arrives. |
| pReleaseTranitions | Int | [sic] Set by the software when multiple containers activate ProductINPUT. ReleaseOUTPUT is activated when a second container arrives. |
| pManualOverride | Int | This flag indicates that OverrideINPUT was activated. |

# XfrTree Table

The XfrTree table holds one record for each route, or full path, that a container may take.

| XfrTree Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this route. |
| Desc | String | Optional additional descriptive text. |
| Boss | PopUp | Choose the XfrBoss entry associated with this tree. |
| Dest | String | Enter a string that uniquely identifies the transfer tree entry. |
| DoConfirm | Y/N | Control generation of divert confirm messages. Y = Trigger a DCM message when the transfer completes. |
| AvailINPUT | PopUp | Choose an IO signal to enable the transfer tree. This signal only prevents the transfer from starting. Once started, the transfer will complete the move even if this input goes OFF. Use XfrConfig_EnabledInput to stop a transfer while it is executing. |
| AvailSENSE | PopUp | Set the “sense” of AvailINPUT to determine if the tree is available when the signal transitions ON or OFF. |
| pAvailIOState | Int | Used internally to track the state of AvailINPUT, considering AvailSENSE. |

# XfrConfig Table

This table defines a sequence of states, inputs, outputs, input timers, and delay timers used to control a transfer through a series of steps.  XfrConfig entries are linked to a specific transfer through XfrGoSet records, and traffic is controlled through the XfrCheckSet records.

| XfrConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the transfer configuration. |
| Desc | String | Optional additional descriptive text. |
| pRun | Y/N | Set by the software to indicate when the XfrBoss is executing a transfer. |
| pStatus | String | Shows the status of the configuration sequence. |
| pState | PopUp | The current state of the configuration. |
| EnabledINPUT | PopUp | Choose an IO signal that will determine if this configuration is enabled or disabled. The disabled state is included in the XfrCfgOutput table, and each output can be configured to be activated or deactivated depending on the state of the signal. Note: When a configuration is disabled, its timers are suspended. |
| EnabledSENSE | PopUp | Set the “sense” of EnabledINPUT to determine if the configuration is available when the signal is ON or OFF. |
| EnabledDELAY | PopUp | Choose a timer. When EnabledINPUT is ready, the timer is started. When the timer completes, the configuration sequence begins execution again. |
| DontDisableIdle | Y/N | When the transfer is disabled (considering EnabledINPUT, EnabledSENSE and EnabledDELAY), this setting determines whether the outputs will be set or disabled while in the IDLE state. N = The transfer outputs are disabled and XfrBoss_MaxWaitTM is reset while in the IDLE state. Y = The transfer outputs are active while in the IDLE state. |
| pCurrentBoss | PopUp | Set by the software to indicate the current XfrBoss record associated with this entry. |
| pStateForRecs | Int | Not used. |
| pEnabledIOState | Int | Set by the software to indicate the current state of EnabledINPUT, considering EnabledSENSE. |
| pFirstInputRec | Int | Set by the software to indicate the first XfrCfgInput record in the chain associated with this configuration (Searches through a linked list). |
| pFirstOutputRec | Int | Set by the software to indicate the first XfrCfgOutput record in the chain associated with this configuration (Searches through a linked list). |
| pFirstInTmrRec | Int | Set by the software to indicate the first XfrCfgInTimer record in the chain associated with this configuration (Searches through a linked list). |
| pFirstDelayTmrRec | Int | Set by the software to indicate the first XfrCfgDelayTmr record in the chain associated with this configuration (Searches through a linked list). |
| pScrnUpdateTmr | Int | Used by the software to determine when to update the pStatus field. The field is refreshed every 3 seconds. |

# XfrCfgInput Table

The XfrCfgInput table defines the inputs associated with an XfrConfig record and with a State for that configuration.  Each input may also be configured with “OR” signals that will be required in order for it to go into an enabled state, as well.

Note:  Inputs configured for “State 1” should not include transition detection unless there is only one input configured for “State 1”.

Note:  Inputs used in the “OR” columns to trigger a transfer to run for simultaneous moves associated with MDR or roller-bed conveyor may result in a move being triggered unexpectedly.  Add a delay to the end of the transfer and turn the output OFF before the delay if the output is used as a trigger for a simultaneous move.

| XfrCfgInput Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer input |
| Desc | String | Optional additional descriptive text. |
| XfrConfig | PopUp | Choose the XfrConfig record associated with this input. |
| State | PopUp | Choose the state associated with this input. |
| RequiredInput | PopUp | Choose the IO signal that must be ready for the XfrConfig record to enter this state. |
| InputSense | PopUp | Set the “sense” of RequiredInput to determine if the input is considered available when the signal is ON or OFF. |
| OrSignal1 – OrSignal7 | PopUp | Choose up to seven “OR” signals that must be ready for the XfrConfig entry to begin this state. |
| OrSense1 – OrSense7 | PopUp | Set the “sense” of the corresponding OrSignal to determine if it should be treated as OR (accept signal ON) or OR NOT (accept signal OFF). |
| pInputState | Int | Used internally to aggregate the state of RequiredInput and InputSense. |
| pOrState1 – pOrState7 | Int | Used internally to aggregate the state of each OrSignal/OrSense pair. |
| pLinkNextRec | Int | Set by the software to indicate the next linked-list record. |
| ClearLatch | Y/N | Use this to determine if the RequiredInput state should be preserved when the input goes OFF. N = Preserve the RequiredInput state when it goes OFF. Y = Reset the RequiredInput state when it goes OFF. |

# XfrCfgInTimer Table

The XfrCfgInTimer table is used to define input timers.  Each entry is linked to a XfrConfig record and also defines a state for that configuration.  Typically, input timers are Full/Clear timers that are in the Done state, which indicates the destination is not full.

| XfrCfgInTimer Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this input timer. |
| Desc | String | Optional additional descriptive text. |
| XfrConfig | PopUp | Choose the record from XfrConfig to associate with this timer. |
| State | PopUp | Choose the XfrState to associate with this timer. |
| RequiredTimer | PopUp | Choose the timer that must be ready before the XfrConfig will enter this State. |
| TimerSense | PopUp | Choose the “sense” of the timer that indicates it is ready. |
| pTimerState | Int | Used by the software to apply TimerSense to RequiredTimer. |
| pLinkNextRec | Int | Set by the software to indicate the next linked-list record. |

# XfrCfgOutput Table

The XfrCfgOutput table is used to define transfer outputs.  Each entry is linked to a XfrConfig record and defines the available states.  Outputs can be configured to “Activate” or “Deactivate” for each state.

| XfrCfgOutput Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer output. |
| Desc | String | Optional additional descriptive text. |
| XfrConfig | PopUp | Choose the record from XfrConfig to associate with this entry. |
| Output | PopUp | Choose the IO output that will be activated, deactivated, or left in its current state for each of the states listed in the State*Action columns |
| OutputAction | PopUp | Choose the action to activate the output. The inverse action will be used to deactivate the output. |
| DisableAction | PopUp | Choose the action to set when in the disabled state. N/A = No action is taken. Activate = Apply the OutputAction. Deactivate = Apply the inverse of OutputAction. |
| IdleAction | PopUp | Choose the action to set when in the Idle state. Note: In most cases this should be set to “N/A” – especially in the case where multiple XfrConfig records control a single output. |
| State1Action – State20Action | PopUp | Choose the action to set when in this state. N/A = No action is taken. Activate = Apply the OutputAction. Deactivate = Apply the inverse of OutputAction. |
| DoneAction | PopUp | Choose the action to set when in the Done state. |
| StateParameter | PopUp | Choose the parameter that will be used to control the output. The output will be activated if the selected parameter is set to “1”. The output will be deactivated if the parameter is “0”. The output will not be changed if the string does not exactly match a “0” or a “1”. |
| pLinkNextRec | Int | Set by the software to indicate the next linked-list record. |
| pOutState | Int | Set by the software to indicate the state of the output after all conditions are applied. |

# XfrCfgDelayTmr Table

The XfrCfgDelayTmr table can be used to define timers that will delay the advance to the next transfer state.  Each XfrConfig record may be associated with one delay timer.

| XfrCfgDelayTmr Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this delay timer. |
| Desc | String | Optional additional descriptive text. |
| XfrConfig | PopUp | Choose the record from XfrConfig to associate with this entry. |
| State | PopUp | Choose the state associated with this delay timer. |
| Timer | PopUp | Choose the timer. |
| TimerAction | PopUp | Choose the state of the timer that indicates it is ready. |
| pLinkNextRec | Int | Set by the software to indicate the next linked list record. |

# XfrSndMsg Table – Sending Messages to Devices

A transfer may need to send one or more messages.  For instance, the MSC HBU PNA systems send serial messages to the AutoSlip machines.  The message is sent when the transfer enters the selected State.

The XfrSndMsg table is used to define the XfrConfig and State where the message is to be sent, and the parameters to include in the message.

| XfrSndMsg Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the message to be sent in the defined configuration and state. |
| Desc | String | Optional additional descriptive text. |
| Enable | Y/N | Enable or disable this entry. |
| pStatus | String | The software will use this field to show status or error information. |
| XfrConfig | PopUp | Choose the record from XfrConfig to associate with this message. |
| State | PopUp | Choose the state associated with this message. |
| Param1 | PopUp | Choose the first parameter for this message. |
| Param2 | PopUp | Choose the second parameter for this message. |
| MsgType | PopUp | Choose the message type: - MSC Serial1 – Send the message with the parameter selected in Param1 from the tracking record. The message is framed with STX (Ctrl-B or 0x02) and ETX (Ctrl-C or 0x03) characters. - MSC Serial2 – Send the message with the parameters selected in Param1 and Param2 from the tracking record. The message is framed with EOT (Ctrl-D or 0x04) and ENQ (Ctrl-E or 0x05) characters. - PA Send Label – Send a print-apply file to the selected Machine. SendLabel defines the row in table LabelTemplates table that points to the file to send. You can create a test entry in the PrintApplyLine table with entries in the Machine and PrinterBoss tables, as follows: - Machine_Connection_Type – CONNECTC_TCPCLIENT or CLOSE_CLIENT - Machine_Communications_Protocol – HOST_RAW - Machine_Machine_Port – 9001. - PrintApplyLine_ControlMachine – Choose this AC. - PrintApplyLine_SndLabelQueue – Choose a label table. - PrintApplyLine_NumPrinters – 1. - PrinterBoss_PrintLine – Choose the PNA line. - PrinterBoss_Enabled – Y. - PrinterBoss_PrinterMachine – Choose the entry for the printer from table Machine, as defined above. |
| Machine | PopUp | Choose the entry from table Machine that defines the connection to the serial device. |
| SendLabel | PopUp | For use with the “PA Send Label” MsgType. Choose the entry from table LabelTemplates that defines the file to be sent |
| CheckParameter | PopUp | Use this to send the message only if a condition is true. Choose the parameter to use for the condition. The message will be sent only if the parameter is a single character “1” (one). |

# XfrParamConfig Table – Transfer Parameter Configuration

A transfer may have up to five parameters that are sent in the CRR or DCM messages.  The XfrParamConfig table is used to define them.  The parameter values are added to the tracking record, which are then added to the CRR or DCM Message when the XfrTree completes.

The table also includes the ability to configure each status event to trigger an error, or to trigger after an arbitrary number of sequential events.

When scan data is received, it is stored in XfRouteBoss_pLastScan.  The parameter processing logic can be configured to check XfRouteBoss_pLastScanCnt and compare it with XfRouteBoss_pLastScanProc to verify new data was received.  If new data has been received, the scan data is loaded into the tracking record.  The parameter processing logic then uses the scan data for the CRR or DCM message or stores it in a selected parameter.  If XfRouteBoss_pLastScanCnt does not indicate a scan event occurred, the CRR or DCM message is loaded with the “NOSCAN” string (found in ScanProcess.h).

Example 1:  The Savoye interface includes the following signals that are sent to CommCore when a carton exits the Savoye/Javaro machine.  The container ID is not known, however, until the carton reaches a scan zone, several zones further downstream:

- Box no lid – ON when the box exiting the Javaro does not have a lid.

- Box no air bag – ON when the box exiting the Javaro does not have an air bag.

- Box no cut – ON when the box exiting the Javaro has not been cut.

- Box no read – ON when the box exiting the Javaro was a NoRead.

- Known Box – ON when a box exiting the Javaro has information about its air bag, lid, cut, and no read status.

Initially the five signals are stored in the tracking record when the carton exits the Savoye/Javaro, then after the carton reaches the scanner, the signals are sent to CommCore.

Each of these signals are sent back to CommCore in the DCM message as Param1 – Param5.  The XfrParamConfig table is configured with five entries where each entry is linked to the same XfrTree row, and each entry is configured with the associated interface bit from the Savoye.

The DCM Message containing the state of the signals is sent to CommCore when the container moves into the zone where the scanner is mounted.  The container ID is copied from XfRouteBoss_pLastScan.

Each of these signals may also be configured to generate errors, if needed.

Example 2:  Again using the MSC PA PNA lines as an example, when a container is scanned at the first scanner on each line, CommCore determines if the container is the correct size for the line.  If the size is incorrect, CommCore sends the “F1_Error” route.

The AC will run the transfer route “F1_Error” which is configured with a ProcessType of “ErrorOnly”.  This ProcessType will only process the configured error and will not process any parameter data.  The XfrParamConfig table is configured to activate an error that will indicate the line has an invalid box size.  The operator is instructed to remove the box and reset the error.

Example 3:  You can use a transfer to count events.  It can be configured to turn an output ON when the count reaches the MaximumErrors value.  The count can be reset using an IO signal configured in the ResetConsSignal field.

Create a transfer that is triggered by a photoeye.  Select the signal that will be counted in XfrBoss_ProductINPUT.  Add an entry for State 1 in the XfrParamProcess table.

Select a ProcessType of “ErrorOnly”.  Select an IO signal in StatusSignal that determines when the input signal should be counted.  This can be the same signal selected in XfrBoss_ProductINPUT (If StatusSignal is not selected, changes in the input signal will always be counted).  Set ErrorSense “Is On”.  Enter a MaximumErrors value of at least 1.

| XfrParamConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this parameter configuration. |
| Desc | String | Optional additional descriptive text. |
| Enable | Y/N | Enable or disable this entry. |
| XfrConfig | PopUp | Choose the record from XfrConfig to associate with this parameter configuration. |
| State | PopUp | Choose the state associated with this parameter configuration. |
| ProcessType | PopUp | Choose the action to take for this parameter configuration: - StoreInTrk – Store in the tracking record. When a transfer reaches the selected state, each row that matches the XfrConfig entry and State will store a parameter in the tracking record. The parameter will be included in the DCM or CRR Message. MsgProcType must also be defined to determine if the contents of MsgSigOff and/or MsgSigOn will be copied into the selected parameter. - ErrorOnly – Only process the error, don’t store or retrieve parameter data. This selection uses ErrorSense to determine if an error has occurred. If the selected state is valid, the error will be activated. - GetLastScan – When a transfer moves into the selected state, XfRouteBoss_pLastScan will be copied to the ConfirmScan field of the tracking record. This function is used to populate a tracking record with a scan that occurs downstream of an event that is captured in a parameter. - GetVerifyScan – When a transfer moves into the selected state, XfRouteBoss_pLastScan will be copied into the parameter selected in the Parameter field. - StringToMenu – For use with the XfrPalletMaint feature. Copies the string in SndMsg to the column and row in table XfrPalletMaint defined by MenuColumn and MenuRow. - PutIdIntoMenu – For use with the XfrPalletMaint feature. Copies the pallet ID from the tracking record ConfirmScan to the column and row in table XfrPalletMaint defined by MenuColumn and MenuRow. - GetIdFromMenu – For use with the XfrPalletMaint feature. Copies the pallet ID from the zone and page defined in MenuRow (1 – 20) and PageIndx (which refer to table XfrPalletMaint) to the ConfirmScan column in the tracking record. MenuColumn must contain a string, but its value is not used! - StartXfr – An entry must be made here using this option to be able to start a transfer from the XfrPalletMaint table. The action will occur when a change is made in the table. - ResetXfr – An entry must be made here using this option to be able to reset a transfer from the XfrPalletMaint table. The action will occur when an interactive change is made in the table. Not very useful - GetLastPack – When a transfer enters the selected state, XfRouteBoss_pLastPack is copied to the parameter selected in field Parameter. Scan types Pack1 – Pack4 can be used to capture a pack scan event. |
| ProcessEnableIO | PopUp | This IO signal may be used to determine if the processing for this record will be completed or skipped. |
| EnableIOSense | PopUp | Set the “sense” of ProcessEnableIO to determine if the record will be processed when the signal is ON or if it is OFF. |
| Parameter | PopUp | Choose the message parameter to associate with this entry. |
| SndMsg | String | Defines the string for the “StringToMenu” ProcessType. |
| MenuName | PopUp | For use with the “StringToMenu”, “PutIdIntoMenu”, and “GetIdFromMenu” ProcessTypes. It is used to validate the value from MenuRow. This looks like an error, but the XfrPalletMaint table must have at least 20 rows to make this work. |
| MenuColumn | String | Used with the “StringToMenu”, “PutIdIntoMenu”, and “GetIdFromMenu” ProcessTypes. |
| MenuRow | Int | Used with the “StringToMenu”, “PutIdIntoMenu”, and “GetIdFromMenu” ProcessTypes. Selects columns Z01 – Z20 in table XfrPalletMaint. |
| PageIndex | Int | Used with the “StringToMenu”, “PutIdIntoMenu”, and “GetIdFromMenu” ProcessTypes. Chooses the record number (or page) in the XfrPalletMaint screen. |
| StatusSignal | PopUp | Choose an optional IO signal. Its state, tempered by StatusSense, determines if MsgSigOn or MsgSigOff will be sent in the DCM message. Defaults to ON if no signal is selected. |
| StatusSense | PopUp | Set the “sense” of StatusSignal that will determine if the record is processed when the signal is ON or if it is OFF. |
| ErrorSense | PopUp | Set the “sense” of StatusSignal regarding when the signal is in an error condition. If StatusSense is “N/A”, the error will not be processed. Earlier versions checked for a valid part. Now, the INVALID, ALWAYSON and ALWAYSOFF parts are not processed. |
| MsgSigOff | String | Define the string that is sent in the DCM message when StatusSignal, as tempered by StatusSense is not active. |
| MsgSigOn | String | Define the string that is sent in the DCM message when StatusSignal, as tempered by StatusSense is active. |
| XfrScanRecord | PopUp | If ProcessType is “GetLastScan” or “GetVerifyScan” set this field to the XfRouteBoss row that holds the container ID. The container ID is stored in XfRouteBoss_pLastScan and is associated with the scan event that occurs when a container is moved into the downstream zone. |
| UpdateHistory | Y/N | Use this to control processing of history, or “Scanner Statistics” data for this row. |
| HRRecord | PopUp | Choose the HistConfig record to be updated for this entry. |
| AutoResetCons | Y/N | Use this to control whether the consecutive error count (in pConsecErrors) is automatically reset when a “good” event occurs. N = Do NOT reset when a “good” event occurs. The count must be reset manually. Y = Reset the count in pConsecErrors to zero when a “good” event occurs. |
| MaximumErrors | Int | Enter the number of consecutive errors that must occur before ErrorIO is activated. A value of 0 deactivates the feature. |
| ErrorIO | PopUp | Choose an IO part to activate when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO when MaximumErrors has been reached. The inverse action will occur when pConsecErrors is less than MaximumErrors. |
| ErrorBIT | Int | Enter the bit to use if ErrorACTION is Enable or Disable. |
| ErrorSignal | PopUp | Choose an IO signal that will be set ON when MaximumErrors is reached. This is usually a memory bit for use in table Errors to activate an error. The signal will be set OFF when pConsecErrors is less than MaximumErrors. |
| ResetCons | Y/N | Set this to Y to manually reset the consecutive error count in pConsecErrors. The software will automatically set the column back to N. |
| ResetConsSignal | PopUp | Choose an IO signal that will trigger a reset of the consecutive error count (in pConsecErrors) when the signal turns ON. |
| ResetSeconds | Int | Enter the number of seconds to wait before automatically resetting the error. This feature is disabled if MaximumErrors = 0. |
| XfrBossToStop | PopUp | Choose the XfRouteBoss record that contains the motor to stop when MaximumErrors is reached. |
| CheckParameter | PopUp | Choose a parameter to use as an override. If StatusSignal, as tempered by StatusSense, is active and the parameter chosen here has a value of 0, the override will occur, resulting in Parameter being set to the value in MsgParamEqZero. If “N/A” is chosen here, no override will occur. |
| MsgParamEqZero | String | Enter the value to use for Parameter if the value of the item chosen in CheckParameter is 0. |
| MsgProcType | PopUp | Choose the processing method associated with this event when using ProcessType = “StoreInTrk”: - Set Msg On-Off – Parameter will receive the string found in MsgSigOff or MsgSigOn. - Set Msg On Only – Parameter will receive the string found in MsgSigOn only. - Set Msg Off Only – Parameter will receive the string found in MsgSigOff only. |
| XfrBossRecord | PopUp | For use with the transfer pallet maintenance feature. When a button on the Transfer Pallet Maintenance screen is clicked, the software searches this table for a record with ProcessType = “StartXfr”. If the column for the button matches the row (Z01 through Z20) and PageIndx matches the row for the button, this field will be used to identify the XfrBoss entry that will be run. |
| XfrTreeRecord | PopUp | For use with the transfer pallet maintenance feature. When a button on the Transfer Pallet Maintenance screen is clicked, the software searches this table for a record with ProcessType = “StartXfr”. If the column for the button matches the row (Z01 through Z20) and PageIndx matches the row for the button, this field will be used to identify the XfrTree entry that will be run. |
| XfrRouteBossRecord | PopUp | For use with the transfer pallet maintenance feature. When a button on the Transfer Pallet Maintenance screen is clicked, the software searches this table for a record with ProcessType = “StartXfr”. If the column for the button matches the row (Z01 through Z20) and PageIndx matches the row for the button, this field will be used to identify the XfRouteBoss entry that will be run. |
| XfrTriggerRoute | PopUp | For use with the transfer pallet maintenance feature. When a button on the Transfer Pallet Maintenance screen is clicked, the software searches this table for a record with ProcessType = “StartXfr”. If the column for the button matches the row (Z01 through Z20) and PageIndx matches the row for the button, this field will be used to identify the XfRouteTable entry that will be run. |
| pConsecErrors | Int | Set by the software to the number of consecutive errors that have occurred associated with this entry. |
| pResetSecsLeft | Int | Set by the software using ResetSeconds. It represents the number of CTRL process iterations left before reset. NOTE: This assumes a CTRL process cycle time of 10 milliseconds! |

# XfrTrig Table – Transfer Triggers

In certain applications, one transfer may need to trigger another transfer, or send a message to CommCore to simulate a scan event.  The XfrTrig table is used to configure for those situations.

A container may be passed from one transfer zone to another.  The first zone receives container data from a scanner.  Subsequent zones receive the data from the prior zone.

| XfrTrig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer trigger configuration. |
| Desc | String | Optional additional descriptive text. |
| Enable | Y/N | Enable or disable this entry. |
| XfrConfig | PopUp | Choose the transfer configuration associated with this trigger. |
| State | PopUp | Choose the state associated with this entry. |
| ProcessType | PopUp | Choose the action that this trigger will perform: - NextXfrTrig – A simulated scan event is used to create a new tracking record with a copy of the data from the old tracking record. If CopyParmData = Y, the parameter data is included. The new transfer object is triggered to run. No message is generated. - XfrCRRTrig – A simulated scan event is used to create a new tracking record with a copy of the data from the old tracking record. If CopyParmData = Y, the parameter data is included. The new transfer object is triggered to run. A CRR message is generated. The CRR response will queue the next transfer move. - XfrCSMTrig – A simulated scan event is used to create a new tracking record with a copy of the data from the old tracking record. If CopyParmData = Y, the parameter data is included. The new transfer object is triggered to run. A CSM is generated. This function is blocked if the XfrBoss_pManualOverride is ON. |
| CrrXfrBoss | PopUp | Choose the transfer object from XfRouteBoss that will be used to populate the CRR message if ProcessType = ‘XfrCRRTrig’. |
| NextXfrRoute | PopUp | Choose the transfer route from XfRouteTable that will run if ProcessType = ‘NextXfrTrig’. |
| CopyParmData | Y/N | Control the handling of parameter data when new tracking records are created: N = Do not copy the parameter data to the new tracking record. Parameters in the new tracking record will each receive a single space character. Y = Copy the parameter data to the new tracking record. |
| PNALine | PopUp | Choose a record from table PrintApplyLine if this entry will be used for a print-apply application. |

# XfrErrConfig Table – Transfer Error Configuration

Each transfer state may have any number of error actions defined.  Error processing occurs when the XfrBoss_MaxWaitTM timer completes but the transfer has not finished processing.

| XfrErrConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this error configuration. |
| Desc | String | Optional additional descriptive text. |
| Enable | Y/N | Enable or disable this entry. |
| XfrConfig | PopUp | Choose the transfer configuration associated with this error. |
| State | PopUp | Choose the state associated with this error. |
| SendDCMOnError | Y/N | N= A DCM message is not created when an error occurs. Y = A DCM message is created when an error occurs. |
| ErrMsg | String | Enter the string to send in the sStatusMsg field (Dat6) of the DCM message when a timeout error occurs. |
| ProcessType | PopUp | Choose the action that this error will perform: - NextXfrTrig – A simulated scan event is used to create a new tracking record with a copy of the data from the old tracking record. If CopyParmData = Y, the parameter data is included. The new transfer object is triggered to run. No message is generated. - XfrCRRTrig – A simulated scan event is used to create a new tracking record with a copy of the data from the old tracking record. If CopyParmData = Y, the parameter data is included. The new transfer object is triggered to run. A CRR message is generated. The CRR response will queue the next transfer move. |
| NextXfrRoute | PopUp | Choose the transfer route from XfRouteTable that will run if ProcessType = ‘NextXfrTrig’. |
| CrrXfrBoss | PopUp | Choose the transfer object from XfRouteBoss that will be used to populate the CRR message if ProcessType = ‘XfrCRRTrig’. |
| CopyParmData | Y/N | Control the handling of parameter data when new tracking records are created: N = Do not copy the parameter data to the new tracking record. Parameters in the new tracking record will each receive a single space character. Y = Copy the parameter data to the new tracking record. |
| Parameter | PopUp | The ErrMsg string will be copied to the tracking record in the parameter chosen here. If a DCM is generated, the string will be included in the selected parameter. |
| UpdateHistory | Y/N | Determine if HRRecord should be updated. |
| HRRecord | PopUp | Choose the HistConfig record to update for this error. |
| AutoResetCons | Y/N | Controls the automatic reset of consecutive errors. N = Do not automatically reset the error count. Once MaximumErrors is reached, a manual reset is required. Y = Automatically reset pConsecErrors each time a transfer runs to completion without timing out. |
| MaximumErrors | Int | Enter the number of consecutive errors that will trigger ErrorIO. Setting this to 0 disables the feature. |
| ErrorIO | PopUp | Choose an IO part to activate when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO when MaximumErrors has been reached. The inverse action will occur when pConsecErrors is less than MaximumErrors. |
| ErrorBIT | Int | Enter the bit to use if ErrorACTION is Enable or Disable. |
| ErrorSignal | PopUp | Choose an IO signal that will be set ON when MaximumErrors is reached. This is usually a memory bit for use in table Errors to activate an error. The signal will be set OFF when pConsecErrors is less than MaximumErrors. |
| ResetCons | Y/N | Set this to Y to manually reset the consecutive error count in pConsecErrors. The software will automatically set the column back to N. |
| ResetConsSignal | PopUp | Choose an IO signal that will trigger a reset of the consecutive error count (in pConsecErrors) when the signal turns ON. |
| ResetSeconds | Int | Enter the number of seconds to wait before automatically resetting the error. This feature is disabled if MaximumErrors = 0. |
| XfrBossToStop | PopUp | Choose the XfRouteBoss record that contains the motor to stop when MaximumErrors is reached. |
| pConsecErrors | Int | Set by the software to track the number of consecutive errors that have occurred. |
| pResetSecsLeft | Int | Set by the software using ResetSeconds. It represents the number of CTRL process iterations left before reset. NOTE: This assumes a CTRL process cycle time of 10 milliseconds! |

# XfrCheckSet Table

The XfrCheckSet table lists configuration sequences that must be “Idle” before the transfer may execute the selected tree.  Each tree may be linked to any number of XfrConfig records.  This is used for traffic control – preventing a sequence from executing while other sequences are still in progress.

| XfrCheckSet Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name. |
| Desc | String | Optional additional descriptive text. |
| Tree | PopUp | Choose the XfrTree record associated with this entry. |
| XfrConfig | PopUp | Choose the transfer configuration associated with this error. |
| MyGroup | PopUp | Optional. Choose an entry from table CheckSetGroups to create a group of check sets. All records with the same MyGroup value will be treated as a group. See CheckGroup. |
| CheckGroup | PopUp | Optional. Choose an entry from table CheckSetGroups. All the records in the group will be checked before the transfer is allowed to run. |

# XfrGoSet Table

The XfrGoSet table lists configuration sequences that will be executed for the selected tree.  Each tree may be linked to any number of XfrConfig records by adding entries in this table.  Execution will not begin until all the XfrCfgInput and XfrCfgInTimer entries are ready for “State 1”.

| XfrGoSet Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name. |
| Desc | String | Optional additional descriptive text. |
| Tree | PopUp | Choose the XfrTree record associated with this entry. |
| XfrConfig | PopUp | Choose the transfer configuration associated with this error. |
| InitialState | PopUp | Choose the initial state for this entry. |

# CRR and CSM Message Structure

The CRR and CSM messages have the same structure.  The CmdHdr field distinguishes between the two.  The Param1 – Param5 fields are used for parametric data for transfers.  The MSC PNA AutoPack lines use these fields for status indicators for the Savoye / Jivaro machines.  CommCore uses the sScanner value as its “Decision Point” name, which determines how the data is processed.

| Container Route Request or Container Status Message (FPC to CommCore) |  |
| --- | --- |
| Field | Description |
| CmdHdr | CRR or CSM |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 20 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor. Probably ignored by CommCore.) |
| Col2 | sTableName |
| Dat2 | zcContainer (Table name in Oracle) |
| Col3 | sContainer |
| Dat3 | Barcode read (ContainerID or SKU) |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sScanner |
| Dat7 | Scanner ID or decision point from ScnScanZone_ScanZoneID |
| Col8 | sTrackTable |
| Dat8 | Tracking table table ID number |
| Col9 | sTrackIndex |
| Dat9 | Tracking table index or row number |
| Col10 | sScaleData |
| Dat10 | Weight from scale, if any (0 indicates an error) |
| Col11 | ToteID |
| Dat11 | Tote scan (If available) |
| Col12 | sZone1 |
| Dat12 | First destination zone. Corresponds to SrtZoneLane_HostZone. The outbound CRR may contain a comma-separated list of unavailable destinations based on SrtAppControl_SendAvailStatus. |
| Col13 | sZone2 |
| Dat13 | Second destination zone. |
| Col14 | sZone3 |
| Dat14 | Third destination zone. |
| Col15 | sZone4 |
| Dat15 | Fourth destination zone. |
| Col16 | sZone5 |
| Dat16 | Fifth destination zone. |
| Col17 | sZone6 |
| Dat17 | Sixth destination zone. |
| Col18 | sZone7 |
| Dat18 | Seventh destination zone. |
| Col19 | sZone8 |
| Dat19 | Eighth destination zone. |
| Col20 | sZone9 |
| Dat20 | Ninth destination zone. |
| Col21 | sZone10 |
| Dat21 | Tenth destination zone. |
| Col22 | Param1 |
| Dat22 | Parameter 1 value. |
| Col23 | Param2 |
| Dat23 | Parameter 2 value. |
| Col24 | Param3 |
| Dat24 | Parameter 3 value. |
| Col25 | Param4 |
| Dat25 | Parameter 4 value. |
| Col26 | Param5 |
| Dat26 | Parameter 5 value. |
| Col27 | sHeight |
| Dat27 | Height |
| Col28 | sWidth |
| Dat28 | Width |
| Col29 | sLength |
| Dat29 | Length |
| The rest are unused for CRR. |  |

# DCM Message Structure

The DCM message has the following structure.  The Param1 – Param5 fields are used for parametric data for transfers.  The MSC PNA AutoPack lines use these fields for status indicators for the Savoye / Jivaro machines.  CommCore uses the sScanner value as its “Decision Point” name, which determines how the data is processed.

| Divert Confirm Message (FPC to CommCore) |  |
| --- | --- |
| Field | Description |
| CmdHdr | DCM |
| Header | AC1 |
| Col1 | iQty |
| Dat1 | 16 (total number of data column pairs, starting with Col1/Dat1. Included fields are based on MsgMap_High_Cursor.) |
| Col2 | sTableName |
| Dat2 | zcContainer (Table name in Oracle) |
| Col3 | sContainer |
| Dat3 | Barcode read (ContainerID or SKU) |
| Col4 | sFunction |
| Dat4 | Read (Function associated with this message) |
| Col5 | sStatus |
| Dat5 | Okay or Error (If Error, sStatusMsg will contain the error string) |
| Col6 | sStatusMsg |
| Dat6 | Error status message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | sScanner |
| Dat7 | Scanner ID or decision point from ScnScanZone_ScanZoneID |
| Col8 | sTrackTable |
| Dat8 | Tracking table table ID number |
| Col9 | sTrackIndex |
| Dat9 | Tracking table index or row number |
| Col10 | sZone1 |
| Dat10 | Divert destination zone. Corresponds to SrtZoneLane_HostZone. |
| Col11 | iDCComplete |
| Dat11 | DCM Divert codes: 1 – Success 4 – From PNA verify 8 – PNA verify failure, but at least one passed 9 – PNA verify failure 10 – PNA verify success 11 – Transfer error 12 – Divert failure |
| Col12 | Param1 |
| Dat12 | Parameter 1 value. |
| Col13 | Param2 |
| Dat13 | Parameter 2 value. |
| Col14 | Param3 |
| Dat14 | Parameter 3 value. |
| Col15 | Param4 |
| Dat15 | Parameter 4 value. |
| Col16 | Param5 |
| Dat16 | Parameter 5 value. |
| The rest are unused for DCM. |  |

# Testing and Debugging

Here are some pointers that may help you configure and debug transfers:

Each XfrConfig record may have up to twenty associated states (State1 – State20), (Idle, Done and DoneDelay are controlled by the software).  Each state may have inputs and/or an input timer associated with the state.

The inputs (XfrCfgInput) assigned to a state must be satisfied to enter a state.
Note:  Inputs configured for State1 should not include transition detection unless there is only one input configured for State1.

The outputs (XfrCfgOutput) are turned ON when in a state.

The delay timer (XfrCfgDelayTmr) runs when the transfer can move to the next state, (When the inputs to the next state are satisfied).

Outputs are NOT turned ON or OFF when the transfer is in the Idle state.  Any output that is controlled by another module (such as a gapper, or full processing) must be turned ON by motor chains or triggers, otherwise it may go OFF and not come back ON when the transfer returns to the Idle state.

In applications (i.e., accumulation) where the solenoid needs to be disabled when a carton moves into the photoeye, select the accumulation solenoid in XfrBoss_ReleaseOUTPUT.  It will be disabled when XfrBoss_ProductINPUT comes ON.  In table XfrCfgOutput, select the solenoid to feed the container from the accumulation.  If XfrBoss_ProductINPUT is not configured, containers will not stop and hold when the destination is full, etc.

To stop a transfer while in cycle, select a signal in XfrConfig_EnabledINPUT.  This will stop the transfer while it’s running.

Note:  Outputs are turned ON and OFF based on the actions selected in the XfrCfgOutput table.  Select an action for each state.  A custom pattern may be created and each state set to:

  - N/A – No change in the IO.

  - Activate – Turn the IO ON.

  - DeActivate – Turn the IO OFF.

The XfrGoSet table is used to define the XfrConfig records that will be executed for each transfer tree.  Each transfer tree defines a specific route or path that a container may follow.

A transfer will not begin execution until all the inputs and all the input timers needed for the first state defined in XfrGoSet_InitialState are ready.  If the InitialState selected is the “Idle” state, all inputs and all input timers for State1 must be ready before the transfer will begin.

Note: Inputs configured for State1 should not include transition detection unless there is only one input configured for State1.

Each transfer tree (XfrTree) entry defines a specific path.

When testing a transfer, XfrBoss_Dest can be set manually.  Enter the string from XfrTree_Dest and set pState = ‘Search’.  XfrConfig_pRun will change to Y and the transfer will execute.

In standard routing applications, XfrBoss_ProductInput is used to run the timer found in XfRouteBoss_MaxWaitTime.  Also, XfrBoss_ReleaseOUTPUT is disabled when XfrBoss_ProductINPUT comes ON.  In table XfrCfgOutput, add a record and select the solenoid and ENABLE it to feed the container from the accumulation.  If XfrBoss_ReleaseOUTPUT is not configured, containers will not stop and hold when the destination is full, etc.

## Example Configuration – Transfer Left

The first XfrConfig moves the container onto the transfer.  The second XfrConfig moves the container left.  Both are in the XfrCheckSet and XfrGoSet tables.

| Transfer Worksheet |  |  |  |  |
| --- | --- | --- | --- | --- |
| XfrConfig | XfrCfgInput | State | XfrCfgDelayTmr | XfrCfgOutput |
| Config 1 | Clear PE – Is Off | State 1 |  | Release Tote |
|  | Tote Present – Trans Off | State 2 |  | Timer Delay |
|  |  |  |  | Raise Bed (Don’t turn OFF) |
| Config 2 | Dest Full - Dn | State 1 |  |  |
|  | Bed Up - On | State 2 |  | Transfer Left - ON |
|  | Right Presence - ON | State 3 |  | Transfer Left - OFF |
|  |  |  |  | Side Feed - ON |
|  |  |  |  | Raise Bed - OFF |
|  | Right Presence – Trans Off | State 4 |  | Side Feed - OFF |

## Transfer Configuration Worksheet

Worksheet for:

Desc:

| Transfer Worksheet |  |  |  |  |
| --- | --- | --- | --- | --- |
| XfrConfig | XfrCfgInput | State | XfrCfgDelayTmr | XfrCfgOutputs |