# FortnaPlus Control Old-Style Transfers

Document Revision: 1.1

Date: 11-Nov-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 12-Sep-2015 | B C Conklin | Add Transfer Example and updates |
| 1.1 | 11-Nov-2022 | T R Kremser | Complete rewrite |

Table of Contents

Glossary of Terms	5

1	Old-Style Transfers	6

2	Old Transfers Configuration	6

2.1	Batches Configuration	6

2.2	Transfer Configuration Steps	6

2.2.1	Define a scanner:	6

2.2.2	Define a message interface:	7

2.2.3	Configure the transfer:	7

2.3	Transfer Configuration Example	7

2.4	Starting a Transfer	10

3	Old Transfers Control Logic	10

3.1	Transfer Routes	10

3.1.1	Example 1	10

3.1.2	Example 2	11

3.1.3	Example 3	11

3.2	Transfer Step 1 (bossIDLE)	13

3.3	Transfer Step 2 (bossSEARCH)	14

3.4	Transfer Step 3 (bossTRANSFER)	14

3.5	Transfer Step 4 (bossDONE)	15

4	Old Transfers Tables	16

4.1	XfRouteBoss Table	16

4.2	XfRouteTable Table	22

4.3	XferBoss Table	23

4.4	XferCfg Table	25

4.4.1	XferStatePop Table	27

4.4.2	XferPattern Table	28

4.5	XferCheckSet Table	29

4.6	XferGoSet Table	29

4.7	XfrTrack Table	30

4.8	XfrDevice Table	32

4.9	Transfer Message Tables	32

4.10	XferTree Table	33

4.11	XfSimConfig Table	34

5	Old Transfers Worksheet Example	35

5.1	XferCfg Table Contents	36

5.2	XFER_IDLE State	36

5.3	PREPARE Section	36

5.4	RELEASE Section	36

5.5	HOLD Section	37

5.6	TRANSFER Section	37

5.7	DONE Section	37

5.8	BACKTOIDLE Section	37

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

CommCore – The software layer used to determine routing and destination decisions, and to record conveyor activity and results.  Also referred to as Level 2.

FPC – FortnaPlus Control.  The machine control layer.  Implemented in C and running on the QNX operating system.  Also referred to as Level 1.

Transfer – A style of conveyor hardware, available in many configurations, that handles one container at a time, or the software used to drive that hardware.  The transfer software is implemented as a state machine, and can be used for many purposes.

UPS – Uninterruptible Power Supply

# Old-Style Transfers

The “New Transfer” module, NewXfrMenu (Configuration Menu | Transfer Utilities) has been around for a long time now, but the older original transfer module is also still supported.  It can be found in TransferUtil (Configuration Menu | Old Transfer Utilities).

Having both the old and new options available has been of great benefit in upgrading old installations from the QNX4 versions of FPC, but it can cause some confusion.  Though there are some exceptions, table names that start with “Xfer” are associated with old transfers, whereas table names that start with “Xfr” are associated with new transfers.  The two top-level tables, XfRouteBoss and XfRouteTable, are shared by both modules, as are the XfrTrack and XfrDevice tables that store information about containers in process.

Typical transfer hardware configurations (the “H” pattern and variants) are supported by both modules.

The general idea and workflow are the same in both cases:

- Both are state machines.

- Both use a scanner to identify containers.

- Both use a connection to CommCore to request a destination for the containers.

- Both use IO signals to control the hardware.

- Both will send a divert confirmation (DCM) message to CommCore upon completion of the route.

# Old Transfers Configuration

## Batches Configuration

Two entries must be made in table Batches to activate the old transfer processing:

File_Name = TransferProcess, HowOften = 1, Type = Count, Process = CTRL, InternalProcCtrl = “TransferProcess”, Machine = <this machine>.

File_Name = XfrProcess, HowOften = 1, Type = Count, Process = CTRL, InternalProcCtrl = “XfrProcess  (CTRL)”, Machine = <this machine>.

## Transfer Configuration Steps

These steps define a template for creating a transfer.  The details will be given later in this document.

### Define a scanner:

Create a scan zone using table ScnScanZone.  The string in ScanZoneID defines the agreed-upon name of the CommCore decision point.  Use TrackingTable = XfrTrack and DeviceTrkTable = XfrDevice.

Create a scan device using table ScnScanDevice.

Create a Machine table entry to define the scan device.

Add a MsgMap table entry to link the ScnScanDevice entry to the Machine entry.

### Define a message interface:

Create Machine table entries to define the connections to CommCore for the CRR, CSU and DCM messages.

Add MsgMap table entries to link the new Machine table entries to message tables.  Use Menu_Name = MsgXR1 for the CRR messages, and MsgXDC1 for the CSU and DCM messages.

### Configure the transfer:

Create an entry in table XferBoss to define the transfer.  This is the main control record for a transfer.  You will need entries in Name, MaxWaitTM, ScanInput, BossMachine, and ScanOutput.  When using a MaxWaitTM timer, set ResetNoSearch = Y to prevent the transfer from going into search mode if an item is present at ScanInput.  The ScanOutput entry is the feed/release into the transfer.  It must be configured in motor chains, which will turn the solenoid or motor ON.  The transfer will disable ScanOutput to hold a container and enable it to release a container.

Create a record in table XfRouteBoss to link the scanner and message interfaces to the transfer.  Choose the scan zone in ScanZone.  Connect the messages using CrrMsgTable, CsuMsgTable and DcmMsgTable.

Create entries in XferCfg.  One entry is required for each path through the transfer.  Define the IO and states needed to control the transfer for each path (route) needed.

Add an XferGoSet entry and link it to the XferCfg record to define how a route will be activated.  A transfer route can be activated with multiple XferCfg records, though typically only one is used.

Add XferCheckSet entries.  These define possible conflicts between routes and prevent use of a route until all conflicting paths are clear.

Add XferTree entries.  These link XferGoSet and XferCheckSet records to a specific XferBoss record.

Add XfRouteTable entries.  These link a CommCore destination (or logical route) to a XferTree and XfRouteBoss.  When a CRR response message containing a destination is received from CommCore, the ScnScanZone and XfRouteTable entries define the correct transfer route to reach the desired destination.

## Transfer Configuration Example

This section shows the steps needed to configure a typical transfer.  Our example will have three possible paths:  Left, Right and Straight.

Configure the Scanner

Use the FPC-Scanner-Control-Configuration document to configure the connection to the scanner device.  Entries will be needed in the ScnScanZone and ScnScanDevice tables.  ScnScanZone_ScanZoneID must be the agreed-upon name of the decision point that CommCore expects.

Use XfrTrack and XfrDevice for the tracking tables.

Configure the CommCore Interface

CommCore will usually expect CRR, CSU and DCM messages for transfers.

Create Machine table entries to define each of the message interfaces.  Use the port values provided by the CommCore team.  Usually, one port is used for the CRR message, and one port is shared by the CSU and DCM messages.

Select message tables for each connection.  Use MsgXR1 for the CRR connection, and MsgXDC1 for the CSU and DCM connection.

Be sure to add the Machine_Machine_Name strings to an entry in the /etc/hosts file.

Configure XfRouteBoss

Choose a name for the route boss in XfRouteBoss_Name.

Link the scan zone to the transfer using XfRouteBoss_ScanZone.

Choose the message tables in XfRouteBoss_CrrMsgTable, XfRouteBoss_CsuMsgTable and XfRouteBoss_DcmMsgTable.

The timers will be automatically created:

XfRouteBoss_NoScanTimer runs when the presence photoeye in XferBoss_ScanInput is ON, and the scanner data has not yet been sent.  If this timer expires, a NOSCAN event will be generated.

While the presence photoeye is ON and after a scan has been received, the XfRouteBoss_NoRouteTmr runs until a response is received from CommCore.  If this timer expires, a NoRoute event will occur.

The XfRouteBoss_ClrQueTmr runs if the presence photoeye turns ON then goes OFF again before the software has released the container.  When this timer expires a “Carton Removed” event occurs.  This is used to discard tracking data and reset the transfer when a container is physically removed so the transfer logic can recover automatically for the next container.

Configure XferBoss

Choose a name for the transfer boss in XferBoss_Name.

Create a maximum wait time timer and select it in XferBoss_MaxWaitTm.  Set an appropriate Preset_Value in table timemenu.

Choose the presence photoeye in XferBoss_ScanInput.

Choose this AC in XferBoss_Boss_Machine.

Choose the transfer feed/release device in XferBoss_ScanOutput.  Note that the feed/release into the transfer must be configured in the Mtrchain table.  The solenoid or motor is turned ON by the motor chain.  The transfer will disable it when in needs to hold a container and enable it to release that container.

Set XferBoss_ResetNoSearch = Y.

Configure XferCfg

Create an entry for each transfer path (or route).  For our example, we will have three entries:  Left, Right and Straight.  For each path, define the following in table XferCfg:

| XferCfg States |  |  |  |
| --- | --- | --- | --- |
| State | Field | When Status Is: | Description |
| Idle | ProductINPUT | ON | Move to the Prepare state. |
| Prepare | PreparedINPUT | In state selected in PreparedSENSE and | Move to the Release state. |
|  | PreparedDELAY | Done |  |
| Release | TransferINPUT | In state selected in TransferSENSE and | Move to the Transfer state. |
|  | TransferDELAY | Done |  |
| Transfer | DoneINPUT | In state selected in DoneSENSE and | Move to the Done state. |
|  | DoneDelay | Done |  |
| Done | FinishINPUT | In state selected in FinishSENSE and | Move to the Idle state. |
|  | FinishDELAY | Done |  |

Configure XferGoSet

Create an entry for each of the three paths defined in table XferCfg.

Give each entry a unique and descriptive name in XferGoSet_Name, perhaps GoLeft, GoRight and GoStraight for our example.

Choose the relevant XferCfg record in XferGoSet_Xfer1 and “xferIdle” in XferGoSet_InitState1.  Repeat for all three paths.

Configure XferCheckSet

Create an entry for each of the three paths defined in table XferCfg.

Give each entry a unique and descriptive name in XferCheckSet_Name, perhaps ChkLeft, ChkRight and ChkStraight for our example.

For each entry, select all the other entries using as many of the XferCheckSet_Xfer1 through XferCheckSet_Xfer5 columns as needed.  For our example, ChkLeft would reference Right and Straight in Xfer1 and Xfer2.  ChkRight would reference Left and Straight.  ChkStraight would reference Left and Right.

Configure XferTree

Create an entry for each of the three paths defined in table XferCfg.

Give each entry a unique and descriptive name in XferTree_Name, perhaps TreeLeft, TreeRight and TreeStraight.

Choose the associated XferBoss record in XferTree_Boss.

In keeping with our Left, Right, Straight example, in XferTree_GoBits, enter “=Left”, “=Right” and “=Straight”, respectively, for the three entries.

In XferTree_MustBeIdle, choose the matching record from XferCheckSet.

In XferTree_XferSet, choose the matching record from XferGoSet.

Set XferTree_DoConfirm = Y

Configure XfRouteTable

Create an entry for each of the three paths defined in table XferCfg.

Give each entry a unique and descriptive name in XfRouteTable_Name, perhaps RteLeft, RteRight and RteStraight.

Choose the associated XfRouteBoss record in XfRouteTable_BossRecord.

Enter the agreed-upon name for this path (or destination in CommCore lingo) in XfRouteTable_HostRouteStr.

Choose the associated XferTree record in XfRouteTable_Route.

## Starting a Transfer

The transfer module checks the following before it starts a transfer:

The route must be enabled by setting XfRouteTable_Enable.

A transfer tree must be selected in XfRouteTable_Route.

XferTree_Prune must equal N for the selected record.

XferTree_EnableIO must be ON (if selected).

The XferCheckSet record selected in XferTree_MustBeIdle must have all selected XferCfg records in the xferIDLE state.

The destination defined in the XferGoSet records must not be full.  Each referenced XferCfg record must have XferClearINPUT = OFF (if selected), DestClearTM = Done, and EnabledINPUT, if selected, must match the state defined by EnabledSENSE.

The desired route must be available, as defined by XferCfg_State = xferIDLE.

If XfRouteBoss_WaitPresence = Y, the IO part selected in XferBoss_ScanINPUT must be ON.

When all these conditions are met, the module will start the transfer by setting XferCfg_Go = Y.

# Old Transfers Control Logic

Once a container has been identified, and routing information has been received, a transfer “tree” is selected and executed to send the container to its destination.

## Transfer Routes

The following examples illustrate how routes are defined and used.

### Example 1

In a simple application with two routes, the XferTree_GoBits entries might be defined as:

Tree1: “=WorkstationA”

Tree2: “=WorkstationB”

When the scanner data has been received and processed, the software calls the transfer_induct_ready() function with the strings “WorkstationA” and “WorkstationB”.  When the “=” symbol is used as the first character of a XferTree_GoBits entry, it indicates that an exact match is required for the rest of the string.

### Example 2

A more complex application has six routes.  The XferTree_GoBits entries might be defined as:

Tree1: “1-----”

Tree2: “-1----”

Tree3: “--1---”

Tree4: “---1--”

Tree5: “----1-“

Tree6: “-----1”

When the scanner data has been received and processed, the software calls the transfer_induct_ready() function to check for busy routes with a string (the “application string”) that might look like:

“1-1--1”

Which indicates the container may be assigned to one of three routes:  Route 1, 3, or 6.  The transfer may be configured to pick the first available route or to wait for the first route to become available.

The dash (“-“) character in XferTree_GoBits strings indicates that the comparison should be skipped for that position.  You can also use the asterisk (“*”) character to indicate that any numeric digit in the application string should match that position.

A similar string can be defined in XferTree_NoGoBits, with “-“ and “*” having the same meanings.  If a match is found, the route will NOT be selected.

The following table shows a few additional examples:

| Application String | GoBits | NoGoBits | Action |
| --- | --- | --- | --- |
| --1-1-3- | ----1--- | -------- | Route match (the 5th route matches exactly, 1 = 1) |
| --1-1-3- | ------1- | -------- | No route match |
| --1-1-3- | ------*- | -------- | Route match (the 7th route matches on the wildcard) |
| --1-1-3- | ----1--- | --1----- | No route match (explicitly not matched on the 3rd route NoGoBit) |

### Example 3

The diagrams in this example attempt to show the relationship between the XferBoss, XferTree and XferCfg tables for a transfer with two input points, six paths, and eight transfer sections.

XferBoss

1. Input 1

2. Input 2

XferTree

Path A

Path B

Path C

Path D

Path E

Path F

XferCfg

Transfer Section 1

Transfer Section 2

Transfer Section 3

Transfer Section 4

Transfer Section 5

Transfer Section 6

Transfer Section 7

Transfer Section 8

Transfer Tables

There are six possible routes (A through F):

F				Input Point 2

E

D

Output Point 3

Output Point 1

Input Point 1	Route B

Route C

Route A

Output Point 2

Example Transfer with Six Routes

Each route is composed of different groups of the eight transfer sections:

Input Point 1

Sec 6

Sec 5

Sec 8

Sec 3

Sec 2

Sec 7

Sec 1

Sec 4

Output Point 2

Output Point 1

Input Point 2

Output Point 3

Example Transfer with Eight Transfer Sections

| Route | Transfer Sections |
| --- | --- |
| Route A | Section 1 => Section 2 |
| Route B | Section 1 => Section 3 => Section 4 |
| Route C | Section 1 => Section 3 => Section 5 => Section 6 |
| Route D | Section 7 => Section 8 => Section 2 |
| Route E | Section 7 => Section 4 |
| Route F | Section 7 => Section 5 => Section 6 |

## Transfer Step 1 (bossIDLE)

Within the application-specific project code, a transfer is initiated from the bossIDLE state once scanner data has been received and the destination has been determined from the response to the CRR message.  The transfer induction function is called with a XferBoss record, a destination (as defined by the “destination bits” which are copied to XferBoss_DestBits), a local table used to note the “verify” (copied to XferBoss_ItemMenu), and the row index in that table (copied to XferBoss_ItemIndex).  The state is set to bossSEARCH in XferBoss_State once the following conditions are met:

- XfRouteTable_Enable = Y

- XfRouteTable_Route must have an XferTree record selected.

- XferTree_Prune = N for the selected record.

- The IO part selected in XferTree_EnableIO must be ON for the selected record

- The XferCheckSet record selected in XferTree_MustBeIdle must have all selected XferCfg records in the xferIDLE state.

- The XferGoSet record selected in XferTree_XferSet requires all selected XferCfg records to have the XferCfg_DestClearTM timer in the Done state (if selected), XferCfg_XferClearINPUT must be OFF (if selected), and XferCfg_EnabledINPUT must match the state specified in XferCfg_EnabledSENSE (if selected).

- XferBoss_State must be bossIDLE.

- If XfRouteBoss_WaitPresence = Y, XferBoss_ScanINPUT must be ON.

## Transfer Step 2 (bossSEARCH)

The transfer software will perform a destination check by comparing XferTree_GoBits with XferBoss_DestBits.

Two modes are supported:

Exact match mode – The first character of the string in XferTree_GoBits must be an equal sign (“=”).  The remaining (arbitrary) characters must match the string in XferBoss_DestBits exactly.  A ‘normal’ induction will set XferBoss_DestBits = “PE”, requiring an XferTree_GoBits entry of “=PE” to match.

NOTE:  Older (QNX4) implementations may have custom logic that sets XferBoss_DestBits directly.  It will also then set the state to bossSEARCH so XferTree will be explored.  This is not done in newer (QNX6) implementations.

Multi-Match mode – Each character in the string represents a segment of the transfer.  The strings are compared character by character.  Valid characters are: numeric digits (0-9), dash (“-“), and asterisk (“*”).  Numeric digits must match exactly.  Dash characters are skipped.  Asterisks in XferTree_GoBits will match any numeric character in the same character position in XferBoss_DestBits.  The test passes on the first match that is found.

Two special keywords are also supported:

ALL – This path is valid for all destinations – it is the default route.  If used, ALL must be the last entry.

NONE – This path is never valid.  It can be used to disable the entry without removing the configuration.

Next, XferTree_NoGoBits and XferBoss_DestBits are compared.  The comparison is the same as for XferTree_GoBits, but a match prevents the path from being selected.  An ALL entry here effectively disables the path entirely.  An entry of NONE is equivalent to no entry.

Next, XferTree_MustBeIdle is checked to find the paths that must be idle before the transfer can proceed.  XferTree_MustBeIdle selects an entry in the XferCheckSet table.  Each transfer section referenced in the XferCheckSet record must be in the xferIDLE state before the container can move.  This behavior can be modified with XferTree_BusyContinue = Y, which will allow the search for a route to continue with the next XferTree record.

Finally, the record in table XferGoSet that has been selected in XferTree_XferSet is checked to verify that all the XferCfg records referenced there have destinations that are not full.  This is done by checking that each XferCfg_DestClearTM timer is Done.  If any timers are not done, the search stops, and the container will not move.  However, if XferTree_FullContinue = Y, the search will continue with the next XferTree record.

Once all these checks pass, XferBoss_State is set to bossTRANSFER.

## Transfer Step 3 (bossTRANSFER)

Transfer execution begins.

XferBoss_CurrentTree is set to the selected XferTree record.

The XferGoSet record from XferTree_XferSet is activated by setting XferCfg_Go = Y for each XferCfg record referenced in the XferGoSet record.

XferCfg_State is also set to the initial state defined in the XferGoSet record for each referenced XferCfg record – usually xferIDLE, but some transfer sections may be reused for different routes, requiring them to be set to different starting states for those routes.

## Transfer Step 4 (bossDONE)

Each XferCfg record sequences through the transfer based on the IO and timers configured for that product path.  As each transfer section completes, XferCfg_Go is set to N. for that section, and XferCfg_State returns to xferIDLE for that section.

When all the transfer sections have returned to the xferIDLE state, XferBoss_State will move to the bossDONE state, completing the transfer.

Once the bossDONE state is reached, XferBoss_State will return to the bossIDLE state.  At this point, another transfer may be initiated.

# Old Transfers Tables

These tables are used to configure Old Transfers.  They can be found in the TransferUtil menu (Configuration Menu | Old Transfer Utilities).

## XfRouteBoss Table

The XfRouteBoss table defines the link to the scan zone in table ScnScanZone.  Learning the container’s identity is the first step in determining its destination.  The scan processing module is also used to define scan zone error processing.

XfRouteBoss also defines connections to message tables used to request routing information from CommCore.

Each XfRouteBoss entry may have multiple XfRouteTable entries.  XfRouteTable defines an agreed-upon string in HostRouteStr that links destinations sent from CommCore to a transfer route.

Note:  Scan error routing assignments are no longer controlled by the AC.  Errors are processed by CommCore, which will make explicit destination assignments for errors, however XfRouteBoss_CommErr is still used to assign a route destination if CommCore does not respond to a routing request.  Likewise, XfRouteBoss_ScanError can still be used in applications where no CommCore connection is available or is not required.

Table XfRouteBoss defines links to four timers that are used to detect scan error events: “No Scan”, “No Route”, “No Carton” and “Carton Removed” events.  The timers and other configuration options provide the ability to detect and respond to the four types of exception events:

- No Scan Event (NoScanTmr) – A container was detected at the transfer, but a scan message was not received.

- No Route Event (NoRouteTmr) – A container was detected and scanned at the transfer, but no routing data was received.

- No Carton Event (NoCartonTmr) – Scan data was received, but a container was not detected.

- Carton Removed Event (ClrQueDlyTmr)– A container was detected, but it was removed.

Note:  XfRouteBoss is used for both “Old Style” and “New Style” transfers.
Set RunNewXfr = N to use old transfers.

| XfRouteBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer. |
| Desc | String | Optional additional descriptive text. |
| pScanStatus | String | The software will update this to indicate the status of the last message processed. |
| pXfrErrorMsg | String | The software will update this if errors are detected during message processing. |
| LogLevel | PopUp | Choose the level of detail for the log files. Set to Level 1 by default. Error messages only with Level 0. Additional detail on available routes with Level 2. Debug detail with Level 5 – do not leave this set to Level 5, it will fill the disk! |
| ScanZone | PopUp | Choose the scan zone from table ScnScanZone associated with this transfer. |
| EsmMsgTable | PopUp | Choose the table to hold External Scan Message (ESM) data. The ESM is used to trigger a transfer from an external source. For example, a container may be manually scanned directly to CommCore. The scan data is sent to the AC using an ESM. The ESM message contains a “decision point name” that must match the string defined in ScnScanZone_ScanZoneID. To use an ESM to execute a move to a destination, use the ESMRoute field. When an ESM is used to trigger a CRR, do not use ESMRoute (see below). |
| EssMsgTable | PopUp | Choose the table to hold External Scan Status (ESS) message data. An ESS message is sent in response to an ESM if a container is present at the decision point mentioned in the ESM. |
| CrrMsgTable | PopUp | Choose the table to hold Container Routing Request (CRR) data. For transfers, start with MsgXR1. Since transfers have much lower message volume than sorters, they can share one CRR message table. You may opt to use multiple tables for clarity, if desired. To generate a CSM instead of a CRR, use the next field. |
| CsmMsgTable | PopUp | Choose the table to hold Container Status Message (CSM) data. For transfers, start with MsgXR1. This is an alternate message that may be sent instead of a CRR. A CRR is sent to request routing of a container. A CSM is sent to indicate that a scan event has occurred at a decision point, but CommCore might not return routing information in its response. |
| DcmMsgTable | PopUp | Choose the table to hold Divert Confirm Message (DCM) data. For transfers, start with MsgXDC1. Since transfers have much lower message volume than sorters, they can share one DCM message table. You may opt to use multiple tables for clarity, if desired. |
| CsuMsgTable | PopUp | Choose the table to hold Container Status Update (CSU) data. For transfers, start with MsgXDC1. Since transfers have much lower message volume than sorters, they can share one CSU message table. You may opt to use multiple tables for clarity, if desired. |
| SendAvailStatus | Y/N | The status of each destination within this transfer can be sent along with the routing request. Also known as the ‘not available list’, the host strings for each destination that is not available are sent in a comma-separated list in the sZone1 field of the CRR message. In the list of possible valid destinations in the CRR response, CommCore will put the available lanes first and will usually include the unavailable lanes at the end of the list in case they become available. FPC always starts at the beginning of the list and works its way down. N = Do not send the available/full status information Y = Send the list of lanes that are not available. |
| AvailTransfer | Int | Normally, this is set to 0 to report the Available/Full status for this transfer. If the Available/Full status of a different transfer needs to be reported in the CRR message, enter the index of the row in table XfRouteBoss for that transfer in this field. If necessary, you can use this to create a transfer representing a group of zones for the purpose of sending their status to CommCore to assist in routing decisions. This virtual transfer is not used for physical control – only to collect data from a group of zones. |
| Straight | PopUp | Choose the XfRouteTable record associated with the straight route. Normally, CommCore will assign all routes, so this is not needed, but it doesn’t hurt to define it. |
| CommErr | PopUp | Choose the XfRouteTable record to use in the event of a communications error. This is used to route the container when CommCore does not respond with routing within the number of seconds defined by NoRouteTmr. It will also be used if the response does not include a destination. If nothing is chosen here, the container will wait indefinitely for a response. |
| ScanError | PopUp | Choose the XfRouteTable record to use when a scan error occurs. This is only used for “Fortna Lite” applications – with the SrtFPLite protocol. |
| NotDefined | PopUp | Choose the XfRouteTable record to use if the routing string returned in the CRR does not match any entries in XfRouteTable_HostRouteStr. If all expected routes are defined in XfRouteTable, this will never be needed, but it doesn’t hurt to define it. |
| NoData | PopUp | Not Used |
| RouteFull | PopUp | Choose the XfRouteTable record to use if the desired route is full or unavailable. |
| ESMRoute | PopUp | If a route needs to be executed for an ESM, choose it from XfRouteTable. |
| MaxWaitTime | Int | Not Used |
| NoScanTmr | PopUp | This timer is used to detect “No Scan” events, which occur if a container moves into the transfer, activating the “Presence Photoeye”, but the scanner does not send data. - The timer is automatically populated when FPC starts. - The default is 10 seconds. Adjust the time as needed for the case where the container activates the presence PE before the scan data is sent. - If the timer expires, the container will be assigned a scan string of “NOSCAN”, and the ScanError route will be used. |
| NoRouteTmr | PopUp | This timer is used to detect “No Route” events, which occur if a container moves into the transfer, has scan data, but does not receive routing. - The timer is automatically populated when FPC starts. - The default is 10 seconds. - If the timer expires before CommCore responds to the CRR message, a No Route error occurs. This can be due to network issues or internal CommCore issues. - If NoRouteRetry = N, and the CommErr route is set to N/A, the container will wait for routing forever. - If NoRouteRetry = N, and the CommErr route is set to a valid route, the container will be sent there when the timer expires. - If NoRouteRetry = Y, the CRR message will be re-sent each time the timer expires until an answer is received. |
| NoCartonTmr | PopUp | This timer is used to detect “No Carton” events, which occur if a scan event occurs, but a container does not activate the presence PE before the timer expires. - The timer is automatically populated when FPC starts. - The default is set to 10 seconds. Adjust the timer to allow enough time for the container to arrive at the presence PE in all cases after the scan data is received. - “No Carton” event processing is used to clean up a queued transfer route, or to clean up incomplete scan data. |
| ClrQueDlyTmr | PopUp | This timer is used to detect when a container is physically removed from a transfer. A “container removed” event occurs if the presence PE detects a container, but the PE goes OFF before the container is released by the software. - The timer is automatically populated when FPC starts. - The default is set to 0.05 seconds. - The timer must be set to a value that is small enough to detect “Push through” events, which may occur if operators push the containers, or when pressure from upstream containers forces them through. - The timer must be set to a value that is large enough to not clear the tracking information when a “bounce” event occurs, such as if the container triggers the presence PE multiple times by bouncing or rocking as it arrives. - If the timer completes, the tracking record will be removed. |
| DontCheckNoScan | Y/N | N = A NoScan event will be detected if a container moves into the transfer and activates the presence PE, but the NoScanTmr expires before scan data arrives. This is the normal behavior Y = The NoScan processing logic is deactivated. |
| DontResetQue | Y/N | N = A “No Carton” event will occur if a scan is received but the presence PE is not activated before NoCartonTmr expires. This is the normal behavior. Y = The “No Carton” processing logic is deactivated. |
| DontResetRcv | Y/N | N = Discard scan data if the presence PE does not turn ON before NoCartonTmr expires. This is the normal behavior. Y = The “Reset Receive” processing logic is deactivated. |
| NoRouteRetry | Y/N | N = A “No Route” even will occur if no response is received for the CRR message from CommCore before NoRouteTmr expires. If the CommErr route is defined, the container will be sent there. If the CommErr route is set to “N/A”, the container will wait forever. Y = The CRR message will be re-sent each time NoRouteTmr expires until a response is received. |
| SystemRun | PopUp | Choose an IO signal that will start and stop the transfer in the sense that it will control these timers: NoScanTmr, NoRouteTmr, NoCartonTmr, ClrQueDlyTmr |
| WaitPresence | Y/N | For “old” transfers. For “new” transfers, this is effectively always set to Y. N = Start executing the transfer immediately. Y = Wait for the presence PE to turn ON before executing the transfer move. Always use Y for “old” transfers! |
| XferCnvMtr | PopUp | Choose the motor that will be disabled when a transfer error occurs. Transfer errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| MtrACTION | PopUp | Choose the action to take when a transfer error occurs. |
| MtrBIT | Int | Enter the bit to use if MtrACTION is Enable or Disable. |
| ScanErrSig | PopUp | Choose a memory bit to be activated when a transfer scan error occurs. Transfer scan errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| ScanACTION | PopUp | Choose the action to take when a transfer scan error occurs. |
| ScanErrBIT | Int | Enter the bit to use if ScanACTION is Enable or Disable. |
| DataErrSig | PopUp | Choose a memory bit to be activated when a transfer data error occurs. Transfer data errors are processed using the configuration in the ScnDeviceErrIO and ScnZoneErrIO tables. Transfers may be halted for consecutive scanner and/or comm errors. |
| DataACTION | PopUp | Choose the action to take when a transfer data error occurs. |
| DataBIT | Int | Enter the bit to use if DataACTION is Enable or Disable. |
| RunNewXfr | Y/N | Y = Use the “new” transfer logic. N = Use the “old” transfer logic. |
| CRRReadyToSend | PopUp | Choose an IO signal that will control when the CRR message is sent. You can use this (and CRRSendSense) to delay sending the CRR message until the presence PE turns ON, for instance. |
| CRRSendSense | PopUp | Choose the required state needed for the CRRReadyToSend signal. |
| SendOneCRR | Y/N | Use this to keep from sending more than one CRR if multiple scans are received while the transfer is in progress. A scanner may generate multiple scans due to push-through events. If the flag is set to Y, only one scan event will be generated. This is especially important in print-apply applications, where an extra CRR will generate an extra label, causing “out of sync” issues. A setting of Y will prevent that. N = A CRR will be sent for each scan event. Y = Only one CRR message will be generated even if additional scans are received. |
| ToteSensor | PopUp | Choose an input device to indicate a tote has been detected. Selecting something here activates the feature. |
| TotePresentStr | String | The string to be sent in the CRR or CSM message if ToteSensor detected a tote. |
| ToteAbsentStr | String | The string to be sent in the CRR or CSM message if ToteSensor did not detect a tote. |
| pProductState | Y/N | Set by the software to indicate the state of the presence PE. |
| pRouteTreeQue | Int | Used by the software to store a XfrTree index reference while waiting for it to be available for assignment. |
| pTrkMenuQue | PopUp | Used by the software to store a tracking table entry while waiting for a route to become available. |
| pTrkIndxQue | Int | Used by the software to store a tracking table index reference while waiting for a route to become available. |
| pProcFlag | Int | Used by the software to trigger the SERV process when scanner data is received. |
| pXfrError | Int | Used by the software to store the error status of the transfer. |
| pGrtHalfSecond | Int | Updated by the software to indicate the number of responses that have exceeded 0.5 second. |
| pGrtQtrSecond | Int | Updated by the software to indicate the number of responses that have exceeded 0.25 second. |
| pMaxResponse | Float | Updated by the software to indicate the longest response time from CommCore. |
| pNumSamples | Int | Updated by the software to indicate the number of responses processed. |
| pScanError | Int | Internal flag used to set ScanError to the required state. |
| pDataError | Int | Internal flag used to set DataError to the required state. |
| pWrkScanError | Int | Internal flag used to process scan data. |
| pWrkDataError | Int | Internal flag used to process scan data. |
| pLastScan | String | Internal string used to record the last scan received (used with verify processing). |
| pLastScanCnt | Int | Internal flag used to track scan events (used with pLastScan). |
| pLastScanProc | Int | Internal flag indicating if the last scan was processed (used with pLastScan). |
| pParamConfigError | Int | Internal flag used for parameter processing. |
| pTimeoutError | Int | Internal flag used for timeout processing. |
| pESMAck | Int | Internal flag used to trigger the ESS send. |
| pTotePresent | Int | Internal flag used to latch the state of ToteSensor. This is reset when a CRR or CSM message is sent. |
| pLastPack | String | Internal string used to record the last pack scan received (used with verify processing). |
| pLastPackCnt | Int | Internal flag used to track pack scan events (used with pLastPack). |
| pLastPackProc | Int | Internal flag indicating if the last pack scan was processed (used with pLastPack). |

## XfRouteTable Table

The transfer routing table, XfRouteTable, is used for both new and old transfer configurations.  Use the Route field to make the link to old transfers.  The NewRoute field would be used for new transfers.

XfRouteTable contains entries that define the routes for each XfRouteBoss record (or scanner – or decision point in CommCore lingo).  The HostRouteStr field contains a string that must match the string received from CommCore.

The table also includes fields to configure an ErrorIO for use if a transfer has experienced a timeout associated with a route, or if the transfer has experienced a “Transfer Reset”.  The HRXfrResets field is also updated, for use by the History Module, based on the number of events detected.

A transfer timeout and reset may occur if the CRR routing response is not valid or if the response is not received within the configured time, or if the transfer does not complete the routing sequence in the configured time.  A transfer reset often indicates the transfer is not functioning properly and may require maintenance.

| XfRouteTable Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this transfer route. |
| Enabled | Radio | N = This route is not available for assignment. Y = This route is available for assignment. If the CRR response includes more than one possible route, the available routes will be selected in the order listed in the message. If no available routes are found and the transfer is configured with a XfRouteBoss_RouteFull option, the container will be routed there, otherwise it will wait until a route becomes available. |
| BossRecord | PopUp | Choose the XfRouteBoss entry associated with this transfer. |
| HostRouteStr | String | Enter the agreed-upon string that identifies this transfer route. It will be matched against the “divert point” string returned in the sScanner field of the CRR response. When used with the SrtFPLite protocol, this string must match the destination encoded in the tote barcode. |
| Route | PopUp | For “old” transfers. Choose the XferTree entry associated with the route. This is the route that will be assigned if the route is available. |
| NewRoute | PopUp | For “new” transfers. Choose the XfrTree entry associated with the route. This is the route that will be assigned if the route is available. |
| AutoResetCons | Y/N | Controls the automatic reset of consecutive errors. Y = Automatically reset pConsecErrors each time a container successfully transfers. N = Do not automatically reset the error count. Once MaximumErrors is reached, a manual reset is required. |
| MaximumErrors | Int | Enter the number of consecutive errors that will trigger ErrorIO. Setting this to 0 disables the feature. |
| ErrorIO | PopUp | Choose an IO part to activate when MaximumErrors is reached. |
| ErrorACTION | PopUp | Choose the action to take on ErrorIO when MaximumErrors has been reached. The inverse action will occur when pConsecErrors is less than MaximumErrors. |
| ErrorBIT | Int | Enter the bit to use if ErrorACTION is Enable or Disable. |
| ErrorSignal | PopUp | Choose an IO signal that will be set ON when MaximumErrors is reached. This is usually a memory bit for use in table Errors to activate an error. The signal will be set OFF when pConsecErrors is less than MaximumErrors. |
| ResetCons | Y/N | Set this to Y to manually reset the consecutive error count in pConsecErrors. The software will automatically set the column back to N. |
| ResetConsSignal | PopUp | Choose an IO signal that will trigger a reset of the consecutive error count (in pConsecErrors) when the signal turns ON. |
| HRTotalDiverted | PopUp | Defines the history record (in table HistConfig) associated with total diverts for this route. Usually configured using the History Wizard. |
| HRFullAborts | PopUp | Defines the history record (in table HistConfig) associated with full aborts for this route. Usually configured using the History Wizard. |
| HRXfrResets | PopUp | Defines the history record (in table HistConfig) associated with transfer resets for this route. Usually configured using the History Wizard. |
| DSRRequest | PopUp | Use this to link to a DSR configuration so CommCore can set the enable status of this route. Each DSR message may contain status information on up to ten routes. If more than ten routes are required, the status must be requested using multiple messages. |
| pRouteFull | Int | Set by the software to indicate a route full condition has occurred. |
| pConsecErrors | Int | Set by the software to track the number of consecutive errors that have occurred for the route. |

## XferBoss Table

XferBoss requires one record for each transfer.

| XferBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the transfer. |
| DestBits | String | Compared with XferTree_GoBits and XferTree_NoGoBits. May contain arbitrarily-named explicit destinations like “STR” or “PK01” that would be matched by “=STR” or “=PK01” in the XferTree strings. In QNX4 implementations, custom code might set this field. For multi-routing configurations, this string may be composed of numeric digits and dash (“-“) characters. Dashes are ignored. See XferTree_GoBits for details on how these fields are compared. In “normal” use, the internal variable destbits (which eventually sets this field) is set to “PE” for induction at the presence photoeye in the call to transfer_induct() within function xferboss_process(). When set in custom code, State must also be set to bossSEARCH so XferTree will be explored. |
| CurrentTree | PopUp | The software selects a record from table XferTree here when it enters the bossTRANSFER state. |
| State | PopUp | Shows the current state from the XferStatePop table. Values are: bossIDLE, bossSEARCH, bossTRANSFER, bossRESET, bossWAIT, bossDONE. |
| Debug | Y/N | Set this to Y to write additional diagnostic messages to the logs. |
| Reset | Y/N | Set this to Y to manually reset the transfer to the bossIDLE state. |
| MaxWaitTM | PopUp | Choose a timer that will define how long to wait before an automatic reset. The reset action depends on the state of the ScanINPUT signal and the current state of the transfer: - ScanINPUT is OFF – reset to bossIDLE state. - ScanINPUT is ON or not defined, State = bossWAIT – reset and search for a new XferTree using the previous DestBits - ScanINPUT is not defined, State is not bossWAIT – reset to bossIDLE state. |
| ScanINPUT | PopUp | Choose a photoeye or other input signal from table Conveyor that will indicate a container is present. When this field is defined, several special actions will occur: When ScanINPUT transitions ON: - If MaxWaitTM is defined, the timer will run. - XferTree is searched for an entry with GoBits = “PE”. - If the transfer is not running, ScanOUTPUT will be disabled if it is defined. When ScanINPUT transitions OFF: - The field defined by ScanMenu, ScanRecvCol and ScanRecvRec is set to 0. |
| ScanMenu | PopUp | Choose a table that will receive the scanner data. ScanMenu, ScanRecvCol and ScanRecvRec together define a specific field in the table. When ScanINPUT goes OFF, the field is set to 0. |
| ScanRecvCol | Int | Choose a column that will receive the scanner data in the table selected in ScanMenu. |
| ScanRecvRec | Int | Choose a row that will receive the scanner data in the table selected in ScanMenu. |
| Boss Machine | PopUp | Choose the AC that will control this transfer (this machine). |
| ItemMenu | PopUp | Set by the software. A parameter used in the code. |
| ItemIndex | Int | Set by the software. A parameter used in the code. |
| NoScanner | Y/N | N = The transfer will use a scanner. Y = The transfer will not require a scanner. |
| pNoScanRoute | Y/N | Use by the software in conjunction with the NoScanner setting. |
| ScanOUTPUT | PopUp | Choose an IO output that will be controlled by the transfer. It will be disabled if ScanINPUT is ON and the transfer is not running. It will be enabled when the transfer is ready to run. Bit 12 is used for the disable. |
| ResetNoSearch | Y/N | Y = prevent the transfer from entering search mode if a container is present at ScanINPUT and MaxWaitTM is defined. |

## XferCfg Table

Table XferCfg is used to define all IO and timers for each transfer section or partial product path.

| XferCfg Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the transfer section or partial path. |
| Go | Y/N | Set by the software. Y = this entry is active. N = inactive. |
| ProductINPUT | PopUp | Choose the IO input that indicates a container is present and ready to transfer. |
| XferDownINPUT | PopUp | Choose the IO input that indicates the transfer mechanism is in the down position. |
| XferClearINPUT | PopUp | Choose the IO input that indicates the container has cleared the transfer. |
| DestClearTM | PopUp | Choose a (full) timer that indicates the destination is clear. |
| ReleaseOUTPUT | PopUp | Choose the IO output that will release the container. |
| ReleaseACTION | PopUp | Choose the action to take on ReleaseOUTPUT. Not present in older versions. |
| DirectionOUTPUT | PopUp | Choose the IO output that will set the direction for the transfer. |
| DirectionACTION | PopUp | Choose the action to take on DirectionOUTPUT. Not present in older versions. |
| TransferINPUT | PopUp | Choose the IO input that indicates the transfer mechanism is in the up position. |
| TransferSENSE | PopUp | Choose the signal state for TransferINPUT that will indicate the transfer is in the up position. |
| TransferDELAY | PopUp | Choose a timer here if a delay is required before TransferOUTPUT is activated. |
| TransferOUTPUT | PopUp | Choose the IO output that will activate the transfer mechanism and raise the bed. |
| TransferACTION | PopUp | Choose the action to take on TransferOUTPUT. Not present in older versions. |
| DoneINPUT | PopUp | Choose the IO input that indicates the container has completed the transfer. |
| DoneSENSE | PopUp | Choose the signal state for DoneINPUT that will indicate the container has completed the transfer. |
| DoneDELAY | PopUp | Choose a timer here if a delay is required before DoneOUTPUT is activated. |
| DoneOUTPUT | PopUp | Choose an IO output that will activate when the transfer is complete. |
| DoneACTION | PopUp | Choose the action to take on DoneOUTPUT. Not present in older versions. |
| PrepareOUTPUT | PopUp | If needed, choose an IO output that would be used to prepare the transfer for arrival of a container. |
| PrepareACTION | PopUp | Choose the action to take on PrepareOUTPUT. Not present in older versions. |
| PreparedINPUT | PopUp | Choose an IO input signal that will indicate that the preparation requested by PrepareOUTPUT has been completed. |
| PreparedSENSE | PopUp | Choose the signal state for PreparedINPUT. |
| PreparedDELAY | PopUp | Choose a timer here if a delay is required for PreparedINPUT. |
| EnabledINPUT | PopUp | Choose an IO input signal that will enable the transfer. |
| EnabledSENSE | PopUp | Choose the signal state for EnabledINPUT. |
| EnabledDELAY | PopUp | Choose a timer here if a delay is required for EnabledINPUT. |
| State | PopUp | Set by the software to the current state of the transfer. |
| Debug | Y/N | Y = Enable additional diagnostic log messages. |
| CurrentBoss | PopUp | Set by the software to the XferBoss record that has enabled this entry. |
| Flex1OUTPUT | PopUp | If needed, choose an additional IO output signal. |
| Flex1ACTION | PopUp | Choose the action to take on Flex1OUTPUT. Not present in older versions. |
| Flex2OUTPUT | PopUp | If needed, choose an additional IO output signal. |
| Flex2ACTION | PopUp | Choose the action to take on Flex2OUTPUT. Not present in older versions. |
| FinishINPUT | PopUp | Choose an IO input signal to indicate the transfer is finished. |
| FinishSENSE | PopUp | Choose the signal state for FinishINPUT. |
| FinishDELAY | PopUp | Choose a timer here if a delay is required for FinishINPUT. |
| PreparePATTERN | PopUp | Choose the control pattern for the PrepareOUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| ReleasePATTERN | PopUp | Choose the control pattern for the ReleaseOUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| DirectionPATTERN | PopUp | Choose the control pattern for the DirectionOUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| TransferPATTERN | PopUp | Choose the control pattern for the TransferOUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| DonePATTERN | PopUp | Choose the control pattern for the DoneOUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| Flex1PATTERN | PopUp | Choose the control pattern for the Flex1OUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| Flex2PATTERN | PopUp | Choose the control pattern for the Flex2OUTPUT signal. The pattern defines whether the signal is turned ON or OFF for each transfer section state. |
| AtProductInput | Int | Set by the software for use with the FIRSTONLY output type from table XferOutTypePop. |
| UseNewEnable | Y/N | N = Use the “old” enable method. Y = Use “new” enable method. The differences between the two methods are subtle, and not exactly clear – TRK. |
| pTransferIOState | Int | Set by the software to track the current transfer state internally. |
| pDoneIOState | Int | Set by the software to track the current transfer state internally. |
| pPreparedIOState | Int | Set by the software to track the current transfer state internally. |
| pEnabledIOState | Int | Set by the software to track the current transfer state internally. |
| pFinishIOState | Int | Set by the software to track the current transfer state internally. |

### XferStatePop Table

| XferStatePop Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| StateName | String | Transfer states. There are two sections. One section is used for transfer states, the other for transfer boss states: xferIDLE – Output states are not changed while in this state. Move to xferPREPARE when XferCfg_Go = Y and XferCfg_ProductINPUT is ON. xferPREPARE – Defined outputs are activated. Move to xferRELEASE when XferCfg_PreparedINPUT matches XferCfg_PreparedSENSE and XferCfg_PreparedDELAY has been completed. xferRELEASE – Defined outputs are activated. Move to xferHOLD when XferCfg_ProductINPUT is OFF unless XferCfg_TransferINPUT matches XferCfg_TransferSENSE and XferCfg_TransferDELAY has been completed, then move to xferTRANSFER. xferHOLD – Defined outputs are activated. Move to xferTRANSFER when XferCfg_TransferINPUT matches XferCfg_TransferSENSE and XferCfg_TransferDELAY has been completed. xferTRANSFER – Defined outputs are activated. Move to xferDONE when XferCfg_DoneINPUT matches XferCfg_DoneSENSE and XferCfg_DoneDELAY has been completed. xferDONE – Defined outputs are activated. Move to xferBACKTOIDLE and set XferCfg_Go = N when XferCfg_FinishINPUT matches XferCfg_FinishSENSE and XferCfg_FinishDELAY has been completed. xferBACKTOIDLE – Defined outputs are activated. Move to xferIDLE and set XferCfg_Go = N and set XferCfg_CurrentBoss to the zeroth record – the undefined choice. xferRESET – Returns to the xferIDLE state. This state may be set if the boss record is reset, which may occur automatically if XferBoss_MaxWaitTM is exceeded, or if the transfer is manually reset. xferDISABLED bossIDLE bossSEARCH – The initial state in XferBoss_State when a transfer is initiated. bossTRANSFER bossRESET bossWAIT bossDONE |

### XferPattern Table

Table XferPattern is used to customize the patterns used to control transfer outputs.  The output control patterns defined here are selected in the PATTERN fields in table XferCfg.  The desired state of the outputs may be defined for each control state of each transfer section.  The actual output signals are defined in table XferCfg – only the states are defined here.

Each of the PATTERN outputs (Prepare, Release, Direction, Transfer, Done, Flex1 and Flex2) can be configured for each control state of each transfer section.  Custom entries may be added as required.  The default entries all begin with “std”.

| XferPattern Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this pattern. |
| IdleState | PopUp | No outputs are altered while in the Idle state. No entry is required. |
| PrepareState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| ReleaseState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| HoldState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| TransferState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| DoneState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| BackToIdleState | PopUp | Choose ACTIVATE, deactivate, or FIRSTONLY. |
| DisabledState | PopUp | No outputs are altered while in the Disabled state. No entry needed. |

The following table shows the default contents of the XferPattern table.

Key: AC = ACTIVATE, de = deactivate, FI = FIRSTONLY.

| XferPattern Defaults |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | Idle | Prepare | Release | Hold | Transfer | Done | BackToIdle | Disabled |
| n/a | -- | -- | -- | -- | -- | -- | -- | -- |
| stdPrepare | -- | AC | de | de | de | de | de | -- |
| stdRelease | -- | de | AC | de | de | de | de | -- |
| stdDirection | -- | AC | AC | AC | AC | de | de | -- |
| stdTransfer | -- | de | de | de | AC | de | de | -- |
| stdDone | -- | de | de | de | de | AC | de | -- |
| stdRelToDone | -- | de | AC | AC | AC | AC | de | -- |
| StdMotorRelease | -- | de | FI | FI | FI | FI | FI | de |

#### XferOutTypePop Table

This table is used with XferPattern.  Each “State” column in XferPattern requires one of the choices listed below.

| XferOutTypePop Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| OutTypeName | String | ACTIVATE – The state is activated. deactivate – The state is deactivated. FIRSTONLY – ACTIVATE for the first container only. |

## XferCheckSet Table

Table XferCheckSet defines transfer paths that must be idle before the transfer sequence can begin.

| XferCheckSet Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this record. |
| Xfer1 | PopUp | Choose up to five transfer sections or partial product paths from table XferCfg that must be idle before the transfer sequence can begin. |
| Xfer2 | PopUp |  |
| Xfer3 | PopUp |  |
| Xfer4 | PopUp |  |
| Xfer5 | PopUp |  |

## XferGoSet Table

Table XferGoSet defines transfer paths that are required for this transfer route

| XferGoSet Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the path. |
| Xfer1 | PopUp | Choose up to five sets of transfer paths (from table XferCfg) with their initial states. |
| InitState1 | PopUp |  |
| Xfer2 | PopUp |  |
| InitState2 | PopUp |  |
| Xfer3 | PopUp |  |
| InitState3 | PopUp |  |
| Xfer4 | PopUp |  |
| InitState4 | PopUp |  |
| Xfer5 | PopUp |  |
| InitState5 | PopUp |  |

## XfrTrack Table

Table XfrTrack contains tracking records associated with transfers.  One table can usually handle all transfers defined on an AC due to the low-speed and single-container nature of transfers.

| XfrTrack Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Set by the software to either the row number, like “R57” or “=====>>>>” to indicate the last record updated. |
| ConfirmScan | String | The container scan string. Sent to CommCore. |
| ScanQty | Int | The number of scans received for the zone. |
| Status | String | Set by the software to show the status as data is received and processed. |
| ErrorMsg | String | Error messages will appear here. |
| HostZone | String | Indicates the zone assigned as the next destination for the container. |
| ConfirmTote | String | If a scan is defined as a tote scan, it will appear here. Sent to CommCore. |
| CombScan | String | If multiple scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombScanSts | PopUp | The status of the CombScan data. |
| CombTote | String | If multiple tote scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombToteSts | PopUp | The status of the CombTote data. |
| CombSKU | String | If multiple SKU scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombSKUSts | PopUp | The status of the CombSKU data. |
| Z1 thru Z10 | String | The CRR response from CommCore may contain up to ten destination options. The AC will choose the first one that is available. |
| ScanZoneID | String | Indicates the scan zone name, taken from ScnScanZone_Name. |
| ScanZoneIndx | Int | The row number in table ScnScanZone for ScanZoneID |
| SorterLane | Int | not applicable for transfers |
| SrtAppRec | Int | not applicable for transfers |
| Shifter | Int | not applicable for transfers |
| Ndx | Int | not applicable for transfers |
| ProcFlag | Int | Set by the software. Indicates scan data has been received and a message must be generated. |
| ProcUpd | Int | Internal flag that indicates this record requires processing. |
| SrtZoneLaneRec | Int | not applicable for transfers |
| CommSent | Int | Used to track if a CRR has been sent when XfRouteBoss_SendOneCRR = Y. |
| TooLate | Int | not applicable for transfers |
| SendCRRTrigger | Int | not applicable for transfers |
| ElapsedTime | Float | Indicates the time, in seconds, it took to send a message to CommCore and to receive the response. |
| CommSentSec | Int | Records the time, in whole seconds, when the message was sent to CommCore. |
| CommSentMSec | Int | Records the millisecond portion of the time beyond CommSentSec when the message was sent to CommCore. |
| TimeStamp | Time | Records the time when the record was initialized (usually at the time of the scan event). |
| TampStatus | PopUp | not applicable for transfers. Used for Print Apply applications. |
| PaProcFlag | Int | not applicable for transfers. Used for Print Apply applications. |
| PaProcUpd | Int | not applicable for transfers. Used for Print Apply applications. |
| ExptRF | String | not applicable for transfers. Used for Print Apply applications. |
| ExptSKU | String | not applicable for transfers. Used for Print Apply applications. |
| ExptLPN | String | not applicable for transfers. Used for Print Apply applications. |
| VerRF | PopUp | not applicable for transfers. Used for Print Apply applications. |
| VerSKU | PopUp | not applicable for transfers. Used for Print Apply applications. |
| VerLPN | PopUp | not applicable for transfers. Used for Print Apply applications. |
| CurrSpeed | Int | not applicable for transfers. Used for Print Apply applications. |
| Height | Float | not applicable for transfers. Used for Print Apply applications. |
| Width | Float | not applicable for transfers. Used for Print Apply applications. |
| Length | Float | not applicable for transfers. Used for Print Apply applications. |
| TooShort | Int | not applicable for transfers. Used for Print Apply applications. |
| SendUpdPrt | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyErr | Int | not applicable for transfers. Used for Print Apply applications. |
| LastTree | String | Indicates the transfer tree assigned in the transfer boss. |
| RouteBossRec | Int | Indicates the XfRouteBoss associated with this tracking record. |
| EmptyConfirm | Int | Not currently used |
| pConfirmTrig | Int | The confirm process uses this flag to indicate the transfer has completed the cycle and a DCM message needs to be sent to CommCore. |
| RouteTableRec | Int | The index in table XfRouteTable associated with this record. |
| Param1 - Param5 | String | Optional parameters may be passed in the CRR and DCM messages. They are stored here. |
| ConfirmShip | String | The container ship scan string. Sent to CommCore. |
| ConfirmPack | String | The container pack scan string. Sent to CommCore. |
| CombShip | String | If multiple ship scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombPack | String | If multiple pack scan strings are combined as though they came from a single scanner, the combined string will appear here. |
| CombShipSts | PopUp | The status of the CombShip data. |
| CombPackSts | PopUp | The status of the CombPack data. |
| ESMInductTime | String | Time string from CommCore. Used to link to the ESS message response in sInductionTime (Dat5). |
| pTimeOutFlag | Int | Used to track whether the transfer has timed out for this record. |
| DCMStatus | Int | Used to track DCM messages. |
| XfrZoneID | String | For use with transfer print-apply applications. Taken from XfrBoss_XfrZoneID. |
| pVerifyFlagTop | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagLeft | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagRight | Int | not applicable for transfers. Used for Print Apply applications. |
| pVerifyFlagFront | Int | not applicable for transfers. Used for Print Apply applications. |
| pInitialTrkIndx | Int | Used by the software to store the original tracking index if this tracking record was created as the result of one transfer triggering another transfer. |
| pPNAExempt | Int | not applicable for transfers. Used for Print Apply applications. |
| LRMSent | Int | not applicable for transfers. Used for Print Apply applications. |
| LVMSent | Int | not applicable for transfers. Used for Print Apply applications. |
| UUID | String | not applicable for transfers. Used for Print Apply applications. |

## XfrDevice Table

Table XfrDevice contains scans associated with a tracking record, and their status information.  Multiple scans may be associated with a single tracking record if more than one scanner is defined for a transfer.

| XfrDevice Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Set by the software to either the row number, like “R23” or “=====>>>>” to indicate the last record updated. |
| TrkIndx | Int | The index of the associated row in table XfrTrack. |
| Scan | String | The string received from the scanner device. |
| ScanSts | PopUp | The status of the Scan data. |
| ScanType | PopUp | The scan device type as defined for this scanner in ScnScanDevice_ScanType. |
| ScanZone | PopUp | The scan zone for this scan device as defined in ScnScanDevice_ScanZone. |
| TimeStamp | Time | Indicates the time when the record was initialized (usually at the time of the scan event). |

## Transfer Message Tables

The XfrMsgUtil submenu contains links to the tables used for transfer messages.  They all have the same structure, but convention dictates that the MsgXR tables be used for CRR messages, while the MsgXDC tables are used for CSU and DCM messages.  Start with the first one in each set.

The ports assigned for CommCore connections will likely determine how many sets of tables you will require, as each port requires a separate table.

Operationally, transfer message volume is far lower than that for sorters, so it is OK to use the same set of tables for more than one transfer, though it is perfectly acceptable to split them across multiple tables.  The message tables normally associated with sorters (the MsgCR and MsgDC sets) are also structured the same way and could be used if you must configure many transfers on a single AC.

The key features of the ‘Msg’ tables are:
CmdHdr – Contains the message type, like CRR
Header – Contains the AC name
Numbered Col / Dat column pairs – Contain the data in key / value pairs where the pairs are defined for each message type.  There is room for up to 32 pairs, though each message type and even the same message type in different software versions may use different numbers of pairs.  The CommCore instance may also determine the data pairs required.  The first pair, in Col1 / Dat1, is always iQty and the number of column pairs used.
TxStat – Records the send / receive status of the message.
DbKey – A code number used to pair the response with the original message when it is received.
DbTime – A time stamp in YYYYMMDDHHMMSS format
iDbTime – A Unix timestamp integer for the creation time of the message.

## XferTree Table

Each record in the XferTree table represents one route, or full path, that a container may take through the transfer.

| XferTree Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the path. |
| Boss | PopUp | Choose the record from table XferBoss that owns this path. |
| GoBits | String | A string of ones, dashes or asterisks that define the destinations that will be accepted for this path. The comparison is made against XferBoss_DestBits. A dash indicates the comparison for the route represented by that character position should be skipped. An asterisk indicates that any numeric digit in DestBits will match in that position. Any string beginning with an equal sign, such as “=Left”, can be used to create an explicit destination. The “=” symbol in the first character indicates that an exact match is required on the remaining text in the string. The word “ALL” can be used to indicate that this path is valid for all destinations and serves as a default route. If “ALL” is used, it must appear as the last entry and is often used to recirculate containers. The word “NONE” can be used to disable this route without removing it from the configuration. |
| NoGoBits | String | A string of ones, dashes or asterisks that define the destinations that will be explicitly not allowed if there is a match. The same character matching rules apply as for GoBits. The word “ALL” can be used to indicate that this route can never be selected (matches all strings). The word “NONE” here is equivalent to no entry – meaning a match in GoBits alone will select the route. |
| MustBeIdle | PopUp | Choose a record from table XferCheckSet that defines the path or paths that must be clear before the transfer can start. |
| BusyContinue | Y/N | N = Wait if this path is busy. Y = Check other possible paths if this path is busy. |
| XferSet | PopUp | Choose the record from table XferGoSet that defines the path. |
| FullContinue | Y/N | N= Wait if this path is busy. Y = Even if one of the required paths is not idle, the remaining paths will be checked. |
| SelfVerifyLane | Int | Not used. |
| DoConfirm | Y/N | Y = Create a divert confirm (DCM) message for this path. |
| Prune | Y/N | Y = Prune the tree. Indicates this branch is not used. Weird. |
| EnableIO | PopUp | Choose an IO signal that will enable or disable this path. If no signal is selected, the path is always enabled. |
| InvertEnableIO | Y/N | N= EnableIO must be ON for this path to be considered. Y = EnableIO must be OFF for this path to be considered. |

## XfSimConfig Table

The Transfer Simulation Configuration table (XfSimConfig)is used to define a set of containers for simulations.

NOTE:  When using a configuration from a real project, use the SimCrrCsu field to create a CSU and a DCM without actually running the transfer.  This provides the ability to process a CRR response and respond with a CSU and DCM without creating a simulation transfer.

Transfers may be configured in a test environment to send carton routing requests to a test CommCore instance to test the system throughput and to test the interactions between the AC and CommCore.

You will need a transfer configuration with at least XfRouteBoss, XfRouteTable, XferBoss and XferTree entries.  The transfer should simulate a transfer event including a DCM.

| XfSimConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this simulation configuration. |
| EnableXfr | Y/N | Enable or disable this entry. |
| Status | String | The software will use this field to show status or error information. |
| AppTransfer | PopUp | Choose the XfRouteBoss record associated with this simulation. |
| NumContXfr | Int | Set the highest container number to assign. |
| CurContXfr | Int | The container number for the container currently being processed by the transfer. This number must be less than NumContXfr or the simulator will not induct containers! |
| NumCharScan | Int | The length, in characters, of the container ID string to generate. Must be no greater than 20. |
| PreFixString | String | Optional static string to start each container ID. You can leave this blank if it is not needed. |
| PostFixString | String | Optional static string to end each container ID. You can leave this blank if it is not needed. |
| ScanDevice | PopUp | Choose a scan device from the ScnScanDevice table. It is used to build the CRR message. |
| SimTimer | PopUp | Choose a timer to control the time between scan events. Set the timer Preset_Value as needed to simulate the throughput requirements for the transfer. |
| SimCrrCsu | Y/N | Determine if messages will be processed by the simulator. N = The simulation will not process any messages. Use this if you are connected to a “real” CommCore instance. Y = The simulation will process CRR, CSU and DCM messages received from the transfer for the decision point (ScnScanZone_ScanZoneID XfRouteTable_HostRouteStr). |
| InductEye | PopUp | Choose the transfer induct photoeye, XfrBoss_ProductINPUT. |
| InductCycleTimer | PopUp | Choose a timer to control InductEye. |
| ScanSimMenu | PopUp | Optional. Choose a table to draw the simulation container ID, scan and scale data from. Use XfrSimScans1 – XfrSimScans5. |
| SimScanFile | String | Optional. Define the full path to a file that contains the simulation data. The data will be loaded in to ScanSimMenu. |
| LoadFile | Radio | Click this to load data from SimScanFile. |
| WrapScanList | Radio | If checked, the simulation will re-use the scan list data in ScanSimMenu when it reaches the end of the list. |
| ResetSimulation | Radio | Click this to reset the simulation. The software will reset it and set pCurrSimRecord to 0. |
| pCurrSimRecord | Int | Set by the software to the row index of the record currently in use in table ScanSimMenu. |

# Old Transfers Worksheet Example

In this example, containers enter from the left and move to the right.  They move to a blade stop, Stop1, at which point they break the beam for Photoeye1.  Once inducted, the container moves into XferSec1 until it breaks the beam for Photoeye2.  Solenoid XfrFwdSol1 activates the rollers, and solenoid RaiseXferSol2 raises the transfer bed.

XfrFwdSol1

Photoeye1

XferSec1

Stop1

RaiseXferSol2

Photoeye2

## XferCfg Table Contents

The following table shows values for our example transfer.

NOTE:  Fields that are not mentioned below are not used in this example or are set by the software.  Leave them at their default “blank” values.

| Example XferCfg Contents |  |  |
| --- | --- | --- |
| Field | Selection | Description |
| Name | TestTransfer1 | A name for our example transfer |
| Go | Y/N | Set by the software |
| ProductINPUT | Photoeye1 | The presence photoeye that will initiate induction |
| XferClearINPUT | Photoeye2 | Indicates a container is present on the transfer bed |
| ReleaseOUTPUT | Stop1 | Release the stop to allow the container onto the transfer |
| DirectionOUTPUT | XfrFwdSol1 | Sets the transfer direction |
| TransferOUTPUT | RaiseXferSol2 | Activates (raises) the transfer mechanism |
| DoneINPUT | Photoeye2 |  |
| DoneSENSE | IS ON |  |
| DoneDELAY | Timer2 |  |
| EnabledINPUT | SystemRun |  |
| EnabledSENSE | IS ON |  |
| State | xferIDLE | Initially set to xferIDLE by the software |
| Debug | Y | Activate additional log messages |
| CurrentBoss |  | Set by the software |
| ReleasePATTERN |  | Release pattern |
| DirectionPATTERN |  | Direction pattern |
| TransferPATTERN |  | Transfer pattern |

## XFER_IDLE State

Outputs:	The output states are not changed while in the xferIDLE state.

When:	XferCfg_Go = Y and XferCfg_ProductINPUT (Photoeye1) is ON

Set:	XferCfg_State = xferPREPARE

## PREPARE Section

Outputs:	The outputs for the xferPREPARE state are activated.

NOTE:  These optional items are not used in this example

When:	XferCfg_PreparedINPUT matches XferCfg_PreparedSENSE
AND
XferCfg_PreparedDELAY is Done (or no timer is selected)

Set:	XferCfg_State = xferRELEASE

## RELEASE Section

Outputs:	The outputs for the xferRELEASE state are activated.

When:	XferCfg_ProductINPUT (Photoeye1) is OFF

Set:	XferCfg_State = xferHOLD

OR (not used in this example)

When:	XferCfg_TransferINPUT matches XferCfg_TransferSENSE
AND
XferCfg_TransferDELAY is Done (or no timer is selected)

Set:	XferCfg_State = xferTRANSFER

## HOLD Section

Outputs:	The outputs for the xferHOLD state are activated.

NOTE:  These optional items are not used in this example

When:	XferCfg_TransferINPUT matches XferCfg_TransferSENSE
AND
XferCfg_TransferDELAY is Done (or no timer is selected)

Set:	XferCfg_State = xferTRANSFER

## TRANSFER Section

Outputs:	The outputs for the xferTRANSFER state are activated.

When:	XferCfg_DoneINPUT (Photoeye2) matches XferCfg_DoneSENSE
AND
XferCfg_DoneDELAY is Done (or no timer is selected)

Set:	XferCfg_State = xferDONE

## DONE Section

Outputs:	The outputs for the xferDONE state are activated.

NOTE:  These optional items are not used in this example

When:	XferCfg_FinishINPUT matches XferCfg_FinishSENSE
AND
XferCfg_FinishDELAY is Done (or no timer is selected)

Set:	XferCfg_Go = N
AND
XferCfg_State = xferBACKTOIDLE

## BACKTOIDLE Section

Outputs:	The outputs for the xferBACKTOIDLE state are activated.

Set:	XferCfg_State = xferIDLE
AND
XferCfg_Go = N
AND
XferCfg_CurrentBoss = N/A (the 0th record)