| # FortnaPlus Control Destination Status Request |  |
| --- | --- |
| Document Revision: 1.1 |  |
| Date: 06-Jan-2021 |  |

Revision; Date; Author; Revision Summary. 1.0; 05-Jul-2012; B C Conklin; New document. 1.1; 06-Jan-2021; T R Kremser; Updates and general refresh.

Table of Contents

1.	Glossary of Terms	4

2.	Destination Status Request Message	4

3.	Table DSRConfig	4

4.	Assigning DSR Configurations	6

4.1.	DSR Messages for Sorters	6

4.2.	DSR Messages for Transfers	6

4.3.	DSR Messages for Print and Apply	6

5.	DSR Message Structure	6

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller.  The hardware hosting FPC.

DSR – Destination Status Request

CommCore – The Level 2 system that provides routing information and accepts information about conveyor system activity.

FPC – FortnaPlus Control.  The machine control software that operates the conveyor hardware.  Also known as Level 1.

# Destination Status Request Message

The DSR message is used in applications where the enable/disable status is needed on the AC to control the conveyor hardware or printers.

A DSR message is sent to CommCore to request the status of one or more destinations.  Each message can contain up to ten destinations.  The status is extracted from the destination enable/disable status in the routing configuration.

DSR messages are sent at regular intervals to poll for the status of each destination.

# Table DSRConfig

The DSRConfig table defines the nature and timing of each DSR message.

| DSRConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this DSR message definition. |
| EnableFlag | Y/N | Select Y to enable this entry. |
| Status | String | Updated by the software to indicate the status of the configured entry. |
| RtnStatus | String | Updated by the software to indicate the status of the response received from CommCore. |
| ControlMachine | PopUp | Select the machine to use for the DSR Messages. |
| MsgMenu | PopUp | Select the message table where the DSR messages will be queued. The MsgGP* tables are recommended. |
| DSRTimeIntrvl | Int | Enter the polling interval in seconds between DSR messages. |
| ConvRunSignal | PopUp | Select the conveyor run signal. |
| DisableDelaySec | Int | Enter the number of seconds to delay activation of the disable signal for Print Apply destinations. When a Print Apply line status is changed to disabled, the line is first set to “disabled for assignment”, then after this interval has elapsed the applicator is disabled. This prevents new assignments while allowing any cartons already assigned to move past the applicators before they are disabled. Disabling an applicator will allow the printers to be serviced without stopping the line. |
| RequestSorterDest | Y/N | Select Y if this entry is associated with Sorter destinations. |
| RequestXfrDest | Y/N | Select Y if this entry is associated with Transfer destinations. |
| RequestPrinterDest | Y/N | Select Y if this entry is associated with Print Apply destinations. |
| OracleTable | String | Enter a string to be sent to CommCore to indicate the Oracle table name associated with this destination. |
| DestNameColumn | String | Enter a string to be sent to CommCore to indicate the Oracle column name associated with this destination. |
| DestStatusColumn | String | Enter a string to be sent to CommCore to indicate the Oracle column name associated with the status of this destination. |
| pLastTime | TimeStamp | Updated by the software. Indicates the last time the module ran. |
| pLastMsgTime | TimeStamp | Updated by the software. Indicates the last time a DSR Message was sent. |
| pQtyRequested | Int | Updated by the software. The number of destinations requested from CommCore. Each message can contain up to 10 destinations. |

# Assigning DSR Configurations

Once DSRConfig records have been created, they can be assigned for use with Sorters, Transfers, or Print and Apply configurations.  Each DSRConfig record can be connected to up to 10 rows in the following tables.

## DSR Messages for Sorters

Sorter configuration table SrtZoneLane (Destination Lane Configuration) column DSRRequest is used to connect destination lanes to a DSR message.

## DSR Messages for Transfers

Transfer configuration table XfRouteTable (Transfer Destination Config) column DSRRequest is used to connect transfer destinations to a DSR message.

## DSR Messages for Print and Apply

Print Apply configuration table PrinterBoss (Printer Configuration) column DSRRequest is used to connect Print and Apply printers to a DSR message.

# DSR Message Structure

The following table describes the content of the DSR message.  It fits the format of other standardized messages between FPC and CommCore, though the meanings of the fields and their content is unique to the DSR message.

When CommCore receives a DSR message, it will construct a response that indicates the enable/disable status of each of the zones listed.

When FPC receives a DSR response message from CommCore, the AC will enable or disable each zone as indicated in the message.  If the message is a disable message for a printer, the printer will first be “disabled for assignment”, then disabled.  This allows the line to clear before the printer is disabled.

| Destination Status Request Message Structure |  |
| --- | --- |
| Field | Value or Description |
| Start Character | <STX> (Start of text character) |
| CmdHdr | “DSR“ (Destination Status Request) |
| Header | “CP1” (The FPC name of this AC) |
| Col1 | “iQty” |
| Dat1 | 20 (Quantity of fields set, starting with Col1) |
| Col2 | “sTableName” |
| Dat2 | String from DSRConfig_OracleTable |
| Col3 | “sDestNameColumn” |
| Dat3 | String from DSRConfig_DestNameColumn |
| Col4 | “sFunction” |
| Dat4 | “Read” |
| Col5 | “sStatus” (Column Identifier) |
| Dat5 | “Okay” or “Error” (If Error the StatusMsg will contain the error string) |
| Col6 | “sStatusMsg” (Column Identifier) |
| Dat6 | Error Message or Status Message (Set if an error occurs or if status is included, OKAY if no error) |
| Col7 | “sDestStatusColumn” |
| Dat7 | String from DSRConfig_DestStatusColumn |
| Col8 | “sDSRTable” |
| Dat8 | Table number of DSRConfig table. |
| Col9 | “sDSRIndex” |
| Dat9 | Row in table DSRConfig associated with this message. |
| Col10 | “sScaleData” |
| Dat10 |  |
| Col11 | “sToteID” |
| Dat11 |  |
| Col12 – Col21 | Destination identifiers from SrtZoneLane_HostZone or XfRouteTable_HostRouteStr or PrinterBoss_HostPrinterName based on the choices described in section 4 |
| Dat12 – Dat21 | Empty in outgoing message. Will contain CommCore response in reply. |
| End Character | <ETX> (End of text character) |