# FortnaPlus Control OpenNMS Configuration

Document Revision: 1.1

Date: 24-Oct-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 09-Sep-2013 | B C Conklin | New document |
| 1.1 | 24-Oct-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	4

1.	NMS Configuration in FPC	5

2.	Communication Interface	5

3.	WCSEvents Configuration	5

4.	NMS Interface Debugging	6

5.	Configuring Procagent for NMS	7

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

EV10 – Web interface and reporting system

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# NMS Configuration in FPC

Error events in FPC can be sent to an NMS server, which can then raise alarms via email notifications or other means.

Be aware that very little configuration effort is required in FPC to activate this feature, but to be useful, a considerable amount of configuration may be required on the NMS server side!  This document does not describe how to configure the NMS server, only how to activate the feature in FPC.

Automation Controller

Open NMS Server

Port 5817

TCP/IP Message

# Communication Interface

Error event messages are sent from the AC to the NMS server using the pre-defined Machine table interface NMS_OUTBOUND, which is NOT active by default.  Activate it by setting Process = SRV1 (or SERV in older versions).

The NMS_OUTBOUND connection uses Connection_Type = CONNECT_TCPCLIENT, Communication_Protocol = WCS, Machine_Port = 5817, Transmit_Retry = 10 and Process = SRV1.  Adjust LogLevel as needed.

Be sure to create an entry in table /etc/hosts to define the IP address of the NMS server and associate it with the NMS_OUTBOUND connection.  Example:
	192.168.10.140	NMS_OUTBOUND

# WCSEvents Configuration

Use the WCSEvents table to control and monitor sending of NMS messages.  The WCSEvents table is primarily used to manage the many types of messages available for use with EV10.  In the context of NMS messages, only the ERROREVENT row is applicable.  Set NMSEnable = Y on the ERROREVENT row to activate the feature.  Use NMSLastTime to see the timestamp of the last NMS message sent.

| WCSEvents Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| EventName | String | Description of the event. These are defined by the program. The ERROREVENT entry is the only one applicable in the context of NMS messages. |
| WCSEnable | Y/N | Flag used to enable or disable the processing of the event. Set the field to Y to enable sending this event. |
| WCSDestination | String | The ActiveMQ topic that will receive this event. These strings are copied to Machine_Machine_Port as the messages are sent. |
| WCSCategory | String | The ActiveMQ message category associated with the event. These strings define how EV10 will handle the messages. |
| WCSService | String | The ActiveMQ message service for the event. These strings define how EV10 will handle the messages. |
| WCSMachProc | Y/N | If set to Y, the Machine_Process is added to the string defining the Machine_Machine_Name. |
| WCSLastTime | String | Timestamp indicating the last time an event of this type was sent. |
| NMSEnable | Y/N | Events are processed and sent using the NMS_OUTBOUND interface when this column is set to Y. Note: Currently, only the ERROREVENT events can be sent to the NMS server. |
| NMSLastTime | String | Timestamp indicating the last time an event of this type was sent to the NMS server. |

# NMS Interface Debugging

- Verify the interface is connecting by checking the Status, Error, and Machine_Pid fields on the NMS_OUTBOUND row in the Machine table:

  - If errors are found, check the SRV1 (or SERV) log for messages associated with NMS_OUTBOUND.

    - The Machine_Pid field will be -1 if a connection has not been established.  A large integer value indicates a connection.  If the connection is not behaving as expected, set the field to -1 to reset the connection.  A new connection will automatically be attempted.

  - If no connection is established, ping the IP address, then ping NMS_OUTBOUND to help determine where the issue may lie.

- Verify NMSEnable = Y in table WCSEvents for the ERROREVENT row.

- Verify messages are being sent:

  - Open the MsgNMS table and verify that new entries are being added.

  - Verify the TxStat fields are being set to “Sent”.

    - If the fields are not being sent to “Sent”, re-check the Machine table.

# Configuring Procagent for NMS

Most sites also use the NMS server to query the status of crucial processes on each AC with the aid of a small server process that runs on the AC called “procagent”.  The NMS server will connect to procagent on each AC periodically, usually once per minute, on port 9999 and request the status of each defined process.  The list of processes is defined in the NMS configuration and should include all the FPC processes: fortna, fpgui, fpsort, fpctrl, fpserv, fpsrv1 and fpsrv2.  The procagent server responds with the up / down status of each process.  NMS will raise an alarm if it finds processes reported as down, or if it is unable to connect to an AC.

The procagent server is started by an entry in the standard startup file, /etc/rc.d/rc.local, which runs automatically at boot time.  You can verify that it is running using “ps -ef | grep procagent”.

The procagent server writes a log file in /var/log named procagent.log.  You will also find procagent.log.<weeknumber>.gz if the logroll cron job is running (as it should be).  Older versions of procagent do not record a timestamp in the log with each access attempt, and do not tolerate spurious input well.  Make sure to update procagent to the latest version, especially if you find it is not writing timestamps to the log file.  You can find the latest version in ftpdata/tkremser/Tools on the ftp4 server, or in the latest fpcupdate650 tool.

NOTE:  You must stop the running instance of procagent to replace it, which may generate NMS alerts!