# FortnaPlus Control Photoeye Status Reporting

Document Revision: 1.1

Date: 10-Jan-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | Unknown | B C Conklin | New document |
| 1.1 | 10-Jan-2023 | T R Kremser | Update and General Refresh |

Table of Contents

Glossary of Terms	4

1.	Photoeye Status Reporting	5

2.	Table PeList	5

3.	Table PeDisplay	6

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Photoeye Status Reporting

The Photoeye Status Reporting module can be used by maintenance to monitor the health of the photoeyes connected to Automation Controllers.  The module monitors photoeye state changes while the system is running.  Configuration is accomplished using these tables:

- PeList –Lists the photoeyes to be monitored by the software.  The table can be found in Configuration Menu | Standard Controls | PhotoEye List.

- PeDisplay – Lists photoeyes that are in the ON state.  Photoeyes that have been blocked for the longest time are listed first.  The table can be found in Configuration Menu | Standard Controls | PhotoEye Display.  Update the table using the “PeReport Copy(GUI)” function.  Add the function to Batches and set it to run at least once per minute.  Use InternalProc = “PeReport Copy(GUI)”, HowOften = 1, Increment = Min, Type = Elapsed, and Process = GUI.

- The function “PeReport Track(CTRL)” must also be configured in Batches.  The function checks for transitions of the photoeyes in the PeList table and updates the timestamps.  Configure the Batches entry to run on each pass of the CTRL process using InternalProcCtrl = “PeReport Track(CTRL)”, HowOften = 1, Type = Count, and Process = CTRL.

Note:  A good starting point for finding critical photoeyes is Jamckeck_Sensor_Name.

# Table PeList

PeList may be manually populated or it may be automatically generated by manually activating the function “PeReport Fill(ANY)” from the “Set Security Levels” table (Administration Menu | Set Security Levels).  Find the function and use Alt-K to run it one time.

| PeList Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this photoeye. Auto-generated entries will use Conveyor_IO_Name for this. |
| ConveyorRec | PopUp | Choose the photoeye to monitor. |
| IntentionallyForced | Y/N | N = The photoeye will be monitored and added to the PeDisplay table when it is ON. Y = The photoeye will never be added to the PeDisplay table. Use this setting to disable an entry. |
| pLastTransitionSec | Int | Set by the software to the whole-second portion of the timestamp of the last transition. |
| pLastTransitionNSec | Int | Set by the software to the fractional-second portion of the timestamp of the last transition, in nanoseconds. |

# Table PeDisplay

PeDisplay is updated by the software each time the Batch job runs to create a list of the photoeyes from PeList that were in a blocked state.  The list is sorted from the longest to the shortest duration.  The entire contents of this table are refreshed each time the Batch job runs.

| PeDisplay Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Description | String | The description of the photoeye. Taken from Conveyor_General_Description. |
| ConveyorRec | PopUp | The photoeye part from table Conveyor. |
| Duration | TimeStamp | The elapsed time that the photoeye has been ON. |
| Location | Radio | Click this to show the location of the part on the conveyor display. |