# FortnaPlus Control Rate Points

Document Revision: 1.1

Date: 26-Oct-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 24-Jul-2013 | B C Conklin | New document |
| 1.1 | 26-Oct-2022 | T R Kremser | Update and general refresh |

Table of Contents

1.	Section	4

2.	Section	4

3.	Section	4

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

UPS – Uninterruptible Power Supply

# Rate Points

Rate points are used to control the flow of containers into merges or sorters based on the number of containers passing a specified point in the system.

Rate points are often used to enable / disable merge lanes to prevent gridlock in a system.

For example, a rate point may be configured to limit the in-feed to a merge when the recirculation line reaches a specific rate.  The SawLane table ReserveRATE column allows you to associate a rate point from table RateCount and define the parameters that will adjust the rate.

NOTE:  The rate control feature must be activated in table Batches by selecting InternalProcCtrl = “Rate Process”.  Use HowOften = 1, Type = Count and Process = CTRL.

# Table RateCount

| RateCount Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this rate calculation configuration |
| Machine | PopUp | Choose the AC that will control this row |
| Process | PopUp | Choose CTRL |
| Pointer | Int | Internal variable used to allocate memory for the rate count process |
| NumBuckets | Int | Set the number of samples that will be averaged when calculating the rate. A larger value smooths the rate and makes it less likely to fluctuate rapidly. A smaller value makes the rate more responsive to changes in arrival rate. 30 is a good starting point. |
| SecPerBucket | Int | The time in seconds during which counts will be assigned to the current bucket before moving on to the next bucket. A longer time can reduce ‘jitter’ but will make the rate less responsive to changes. A good initial value is 1 second. |
| CountNow | Int | Internal variable used to record the ON transitions for the selected IO signal. |
| ResetNow | Int | Enter a non-zero value here to force a reset of the counts in each bucket. |
| CurrentCount | Int | Set by the software to the sum of the counts in all the buckets. |
| RatePerMin | Float | The rate calculated based on the value in CurrentCount and the total time represented by the product of the values in NumBuckets and SecPerBucket. The value is in containers (or ON transitions) per minute. |
| HighWater | Float | Set the RatePerMin value that will trigger a WentHigh transition. When the RatePerMin increases to a value above this value, WentHigh will turn ON and WentLow will turn OFF. |
| LowWater | Float | Set the RatePerMin value that will trigger a WentLow transition. When the RatePerMin decreases to a value below this value, WentLow will turn ON and WentHigh will turn OFF. |
| WentLow | Y/N | A flag set by the software to indicate the rate has dropped below the LowWater value. |
| WentHigh | Y/N | A flag set by the software to indicate the rate has risen above the HighWater value. |
| CountIO | PopUp | Choose the input signal to monitor for the rate calculation. Usually a photoeye across the conveyor segment of interest, such as a recirculation lane. |
| EnableIO | PopUp | Choose an IO signal to control when the rate calculation is active. Usually, the motor run signal for the conveyor segment of interest. This will pause the rate calculation when the line is stopped, preventing the calculated rate from erroneously dropping to zero while the line is not running. |
| RateHighOutput | PopUp | Choose an IO output signal to activate when HighWater is reached or exceeded. See RateHighACTION. |
| RateHighACTION | PopUp | Choose the action to take on RateHighOutput when HighWater is reached. |
| RateHighBIT | Int | Set the bit to use if RateHighAction is Enable or Disable. |
| RateHighError | PopUp | Choose an error configuration from table Errors that will be activated when HighWater is reached. |

# Example

Example:  Stop feeding “Lane 1” when the arrival rate on the recirculation lane rises to 15 containers per minute.  Resume merging when the rate falls to 6 containers per minute.  Configure the following in table RateCount:

- Select a photoeye from the recirculation line in CountIO

- Select the motor that runs the conveyor under the photoeye in EnableIO

- Set NumBuckets = 30

- Set SecPerBucket = 1

- Set HighWater = 15

- Set LowWater = 6

- Test the configuration by forcing CountIO ON and OFF repeatedly.  The CurrentCount will increment each for each ON/OFF cycle.  RatePerMin will show the current calculated rate.

- WentLow and WentHigh will show the current control state, based on RatePerMin