# FortnaPlus Control Print Apply Configuration Checklist

Document Revision: 1.7

Date: 16-Mar-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 |  |  | Initial version |
| 1.1 | 15-Jul-2013 | B C Conklin | Added notes on setting up the “Wait Update” and the “SORT” owner for the “Printer Tamp” and “Printer Error”. |
| 1.2 | 08-Apr-2014 | B C Conklin | Added Examples |
| 1.3 | 02-Aug-2017 | T R Kremser | Updates |
| 1.4 | 12-Jan-2018 | T R Kremser | Clarify HeightInDT description. |
| 1.5 | 26-Jul-2021 | T R Kremser | Update and general refresh |
| 1.6 | 10-Jan-2022 | T R Kremser | Add PStConvertTable UseVerScan feature |
| 1.7 | 16-Mar-2022 | T R Kremser | Add note regarding SrtTrack and SrtDevice table sizes. |

Table of Contents

1.	Abstract	5

2.	Assumptions	5

3.	Print Apply Tables	5

3.1.	Batches Table	5

Example Batches Configuration:	5

3.2.	Machine Table	6

	LPN Scanners	6

	Verify Scanners	6

	Printers	7

	LRM Send	8

	LRM Receive (ActiveMQ only)	8

	LMI Receive	9

	LVM Send	10

Machine Examples	11

3.3.	MsgMap Table	12

	PNA MsgMap Entries	13

MsgMap Examples	14

3.4.	ScnScanZone Table	15

Example ScnScanZone Table	15

3.5.	ScnScanDevice Table	16

Example Scan Device Table	17

3.6.	PrintApplyLine Table	17

Example Print Apply Line Table	19

3.7.	PrinterBoss Table	19

Example Printer Boss Table	20

3.8.	PrinterErrConfig Table	20

Example PrinterErrConfig Table	22

3.9.	PrinterUpdPnts Table	22

Example Sorter Updates Table and PrinterUpdPnts Table	24

3.10.	PrintBelt Table	24

Example PrintBelt Table	24

3.11.	HeightWidth Table	25

Example HeightWidth Table (Photoeye)	26

3.12.	PAScannerBoss Table	26

Example PAScannerBoss Table	27

3.13.	PAPrinterScanAssign Table	27

3.14.	PStConvertTable Table	27

Example ScnScanZone Table	28

Example ScnScanDevice Table	29

Example Printer to Sorter Convert Table	29

Example SrtAppControl Table	29

Example SrtZoneLane (Destination) Table	29

Example SrtScanBoss (Scan Exceptions) Table	29

3.15.	LabelTemplates Table	29

Example LabelTemplates Configuration	31

3.16.	Round-Robin Tables	32

4.	Testing Configurations	32

4.1.	Enabling Test Mode	32

4.2.	TestConfig Table	33

Example Test Line Table	33

4.3.	TestCnfgPrt Table	33

Example TestCfgPrt table	33

5.	Print Apply Status Screen	34

6.	Disable Zebra Printer Disconnect	34

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Abstract

This document provides a summary of the tables and configuration items involved in designing a Print and Apply (PNA) line in FortnaPlus Control (FPC).  Individual configuration parameters will be described briefly, and selection items defined where appropriate.

# Assumptions

This document assumes an understanding of the general operation of a PNA line.  Please refer to the PNA design documentation for detailed descriptions.

This document assumes a sorter has been configured for the PNA line.  For details on configuring a sorter, please refer to the Sorter Configuration Checklist document.

# Print Apply Tables

This section describes tables needed to configure a PNA line.  The first few are not specific to PNA lines but must be configured to provide the framework for the PNA portion.

# Batches Table

The Batches table (Configuration Menu | Standard Controls | Batches) is used to enable/disable FPC features.  To enable the PNA subsystem, enable the following batch jobs.

Enable each batch by selecting the machine name of this AC in the Machine column.

### Example Batches Configuration:

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | How Often | Incre-ment | Type | Process | Internal Proc | Internal Proc Ctrl | Machine | All Machines | Do File |
| Scan Process | 1 | Invalid | Count | CTRL |  | ScnApp CTRL | PASIM1 | N | N |
| PA Sort | 1 | Invalid | Count | SORT |  | PA SORT (CPN) | PASIM1 | N | N |
| PA Ctrl | 1 | Invalid | Count | CTRL |  | PA CTRL (CPN) | PASIM1 | N | N |
| PA Serv | 1 | Invalid | Count | SERV | PA SERV (CPN) |  | PASIM1 | N | N |
| PA GUI | 1 | Invalid | Count | GUI | PA GUI (CPN) |  | PASIM1 | N | N |
| PA Simulation | 1 | Invalid | Count | SORT |  | PNASim SORT | PASIM1 |  |  |

# Machine Table

The Machine table (Configuration Menu | Communication Utilities | Machine) defines the devices that will be communicating with the PNA machine. For a typical PNA line, you will need to create the following machines:

- LPN Scanners

The purpose of the LPN scan is to identify a carton prior to PNA transactions.  Define a Machine table entry for each scanner:

  - Connection Type

    - If connecting via a serial interface, select CONNECT_SERIAL

    - If connection via an Ethernet interface, select CONNECT_TCPCLIENT

  - Communication Protocol

    - Select AlphaNum

  - Machine Port

    - If connecting via a serial interface, enter the serial port

      - /dev/ser-rckt1 (Enter the appropriate port)

    - If connecting via an Ethernet interface, enter the TCP port number

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the CTRL process

  - LogLevel

    - Set to “Level 1”

- Verify Scanners

The purpose of the Verify scan is to determine if the label was successfully applied to the carton and to verify the printed barcode on that label can be read by the scanner.  The scan results are compared to the expected strings found in the LRM response message.  Define a Machine table entry for each verify scanner.

  - Connection Type

    - If connecting via a serial interface, select CONNECT_SERIAL

    - If connecting via an Ethernet interface, select CONNECT_TCPCLIENT

  - Communication Protocol

    - Select AlphaNum

  - Machine Port

    - If connecting via a serial interface, enter the serial port

      - /dev/ser-rckt2 (Enter the appropriate port)

    - If connecting via an Ethernet interface, enter the TCP Port number

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the CTRL process

  - Log Level

    - Set to “Level 1”

- Printers

A PNA line will feature at least one label printer.  Create a Machine table entry for each printer on the line.  The Machine table entry and matching “/etc/hosts” entry create a client-style connection to the printer for sending labels.

  - Connection Type

    - Select CLOSE_CLIENT

  - Communication Protocol

    - Select AlphaNum

  - Machine Port

    - Enter the TCP Port defined for the printer

      - 9100 (usually)

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the SERV process.

- LRM Send

The purpose of the LRM (Label Request Message) Send Machine table entry is to transmit a carton’s data to the Level 2 PNA Engine to request the carton’s labeling requirements.

  - Connection Type

    - If using ActiveMQ, select AMQ_TALK.
Note that the ActiveMQ connections handle traffic one way only, so a corresponding AMQ_RECEIVE connection will be needed, as well, to receive the LRM response.  See below.

    - If using TCP/IP sockets, select CONNECT_TCPCLIENT.
TCP sockets connections handle two-way traffic. No other connection is needed to handle the LRM messages.

  - Communication Protocol

    - Select PAComm for pipe-delimited messages, PandA for JSON messages.

  - Machine Port

    - If using ActiveMQ, enter the name of the queue defined for the LRM send interface:

Pipe-delimited messages:  /queue/WCS.PNA.WCSAS.LRM

JSON messages:  /queue/AC.PNA.LABELREQUEST

    - If using TCP/IP sockets, enter the TCP port number defined for the LRM send interface.

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the SERV process.

- LRM Receive (ActiveMQ only)

The purpose of the LRM Receive Machine table entry is to handle the LRM response from the Level 2 PNA Engine containing a carton’s labeling requirements.

  - Connection Type

    - If using ActiveMQ, select AMQ_LISTEN

    - If using TCP/IP sockets, a separate LRM Receive entry is not needed.

  - Communication Protocol

    - Select PAComm

  - Machine Port

    - If using ActiveMQ, enter the name of the queue defined for the LRM receive interface:  /queue/WCS.PNA.<MACHINE>.LRM

    - If using TCP/IP sockets, a separate LRM Receive entry is not needed

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the SERV process.

- LMI Receive

The purpose of the LMI Receive Machine table entry is to process label files and queue them for transmission to a printer.  If using JSON message format, an LMI connection is not needed – the label images will be returned in the LRM (LabelRequest) response.

  - Connection Type

    - If using ActiveMQ, select AMQ_LISTEN

    - If using TCP/IP sockets, two machine entries need to be made (server-style connection):

      - LRM Listener – connection type TCPSERV_LISTEN

      - LRM Handler – connection type TCPSERV_RDRW

  - Communication Protocol

    - Select PALblImg

  - Machine Port

    - If using ActiveMQ, enter the name of the queue defined for the LMI receive interface:  /queue/WCS.PNA.<MACHINE>.LMI

    - If using TCP/IP sockets, enter the TCP port number defined for the LMI receive interface in both entries

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the SERV process.

- LVM Send

The purpose of the LVM Send Machine table entry is to communicate the results of a carton’s label verification to the Level 2 PNA Engine.

  - Connection Type

    - If using ActiveMQ, select AMQ_TALK

      - If using TCP/IP sockets, select CONNECT_TCPCLIENT

  - Communication Protocol

    - Select PAComm

  - Machine Port

    - If using ActiveMQ, enter the name of the queue defined for the LVM send interface:  /queue/WCS.PNA.WCSAS.LVM

    - If using TCP/IP sockets, enter the TCP port number defined for the LVM send interface

  - Transmit Retry

    - Set to 10 seconds

  - Process

    - Select the SERV process.

Note:	For machines that will be communicating via a network interface, the machine names must be included in the PNA node’s /etc/hosts file along with the respective IP Address.  All ActiveMQ connections (always client), and all TCPCLIENT connections require an /etc/hosts entry.  TCPSERV connections do not require an entry.

Note:	Both the Serial and Ethernet Protocols AlphaNum and STXETX use the table AsciiMnu to filter the received scanner data to remove invalid characters.  Load the latest “AsciiMnu.asc” from the latest release, and make any adjustments, as needed.

Note:	If changes are made to the Machine table, FortnaPlus must be shut down and restarted.  Init functions are only run once during startup.  Circbuff files found in the RUN directory “cb_*.cbf” must be deleted after FortnaPlus is shut down.  Circbuff files are not used or created as of V10.83.0.

### Machine Examples

Example Machine table:  Scanners - Serial

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry | Process | Trig_Size |
| LPNScan | CONNECT_ SERIAL | AlphaNum | /dev/ser-rckt1 | 0 | CTRL | 2048 |
| VerifyScan | CONNECT_ SERIAL | AlphaNum | /dev/ser-rckt2 | 0 | CTRL | 2048 |

Example Machine table: Scanners - Serial (Continued)

| Machine Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trig_ InterChar | Trig_ Timeout | Trig_ MsgEndChar | Log Level | Close Delay(ms) | Open Timeout(ms) |
| 10 | 0 | Ctrl J (LF) | Level1 | 0 | 0 |
| 10 | 0 | Ctrl J (LF) | Level1 | 0 | 0 |

Example Machine table: Scanners - Ethernet

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry | Process | Trig_Size |
| LPNScan | CONNECT_ TCPCLIENT | ALPHANUM | 9200 | 10 | CTRL | 0 |
| VerifyScan | CONNECT_ TCPCLIENT | ALPHANUM | 9200 | 10 | CTRL | 0 |

Example Machine table: Scanners - Ethernet (Continued)

| Machine Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Trig_ InterChar | Trig_ Timeout | Trig_ MsgEndChar | Log Level | Close Delay(ms) | Open Timeout(ms) |
| 0 | 0 | ETX | Level 1 | 0 | 0 |
| 0 | 0 | ETX | Level 1 | 0 | 0 |

Example Machine table: Level II Communications

| Machine Table (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry | Process | Trig_ Timeout | Log Level |
| LRMSend | AMQ_TALK | PaComm | /queue/WCS.PNA. WCSAS.LRM | 10 | SERV | 1000 | Level 1 |
| LRMRecv | AMQ_LISTEN | PaComm | /queue/WCS.PNA. PNA1.LRM | 10 | SERV | 1000 | Level 1 |
| LVMSend | AMQ_TALK | PaComm | /queue/WCS.PNA. WCSAS.LVM | 10 | SERV | 1000 | Level 1 |
| LMIRecv | AMQ_LISTEN | PaLblImg | /queue/WCS.PNA. PNA1.LMI | 10 | SERV | 1000 | Level 1 |

Example Machine table: Level II Communications (Continued)

| Machine Table (Part 2) |  |  |
| --- | --- | --- |
| User Name | Password | OpenTimeout(ms) |
|  |  | 1000 |
|  |  | 1000 |
|  |  | 1000 |
|  |  | 1000 |

Example Machine table: Printer Communications

| Machine Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Name | Connection Type | Communication Protocol | Machine Port | Transmit Retry | Process | Trig_ Timeout | Log Level |
| Panther_P1 | CLOSE_ CLIENT | AlphaNum | 9100 | 10 | SERV | 0 | Level 1 |
| Panther_P2 | CLOSE_ CLIENT | AlphaNum | 9100 | 10 | SERV | 0 | Level 1 |

### Example /etc/hosts entries

192.168.10.100	LPNScan

192.168.10.101	VerifyScan

192.168.10.10	LRMSend LRMRecv LVMSend LMIRecv

192.168.10.102	Panther_P1

192.168.10.103	Panther_P2

(Always leave an empty line at the end)

# MsgMap Table

The MsgMap table (Configuration Menu | Communication Utilities | Message Map) specifies the link between the tables used to store incoming and outgoing messages and each of the communication devices defined in the Machine table.  For a typical PNA line, you will need to create the following entries:

- PNA MsgMap Entries

The data from scanners will usually be copied to the ScnScanDevice table.  However, in some cases the barcode must be parsed into Carton IDs, Height and Width data, in which case the received message is stored in the ScnScanZone table.

  - MsgMap_Record

    - The MsgMap_Record field is a reference to the row number of the ScnScanZone or ScnScanDevice table that will contain the scanner data.

The content of data messages such as LRM and CRR will use message tables such as MsgCR1, MsgDC1, etc.

  - Cursor Fields

    - The Low_Cursor, High_Cursor, Stat_Cursor and Key_Cursor fields must be set accurately for messages to be transmitted and parsed properly.

      - For scan data, where a single string is copied to one data field, set Low_Cursor and High_Cursor to the same value:

        - ScnScanZone – set Low_Cursor and High_Cursor to “4” (the RcvDataA field) and the Stat_Cursor to “5” (the RxStatA field)

        - ScnScanDevice – set Low_Cursor and High_Cursor to “2” (the RcvData field) and the Stat_Cursor to “3” (the RxStat field)

      - In cases of field-delimited messages, such as LRM, CRR, etc, set the Low_Cursor to “1”.  The Stat_Cursor value can be determined by opening the message table using F2:Menu-of-Menus and Alt-K (for example MsgCR1) and paging down to the TxStat Column. The row number (for example 67) will be the value needed for the “Stat_Cursor” Column in the MsgMap table.  The High_Cursor value (for example 66) is the Stat_Cursor value minus 1. The Key_Cursor value is the row number of the DbKey column (for example 68).

Note: All versions of FortnaPlus Control since version 10 automatically set the Stat_Cursor and Key_Cursor values correctly at start-up.

A typical PNA line will contain at least the following MsgMap entries:

- LPN Scan Data – usually uses the ScnScanDevice table.

- Verify Scan Data – usually uses the ScnScanDevice table.

- LRM Send – usually uses a MsgCR1 – MsgCR5 table.

- LRM Response – uses the same table as LRM Send, above.

- LVM – usually uses a MsgDC1 – MsgDC5 table.

Note:  LMI messages do not require a MsgMap entry.  The PNA subsystem handles LMI messages in a unique way which does not require message queuing.

### MsgMap Examples

Example MsgMap Table:  Level 2 Communications

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine Name | Recv | Send | Ack | Menu | Low | High | Stat | Key | Record |
| PNA1 LRM Send | LRMSend | ALL | ALL | ALL | MsgCR1 | 1 | 66 | 67 | 68 | 0 |
| PNA1 LRM Recv | LRMRecv | ALL | ALL | ALL | MsgCR1 | 1 | 66 | 67 | 68 | 0 |
| PNA1 LVM Send | LVMSend | ALL | ALL | ALL | MsgDC1 | 1 | 66 | 67 | 68 | 0 |

Example MsgMap Table:  Scanner Communications

| Message_Map_Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Machine Name | Recv | Send | Ack | Menu | Low | High | Stat | Key | Record |
| LPN Scan | LPNScan | ALL |  |  | ScnScan Device | 2 | 2 | 3 |  | 1 |
| Verify Scan | VerifyScan | ALL |  |  | ScnScan Device | 2 | 2 | 3 |  | 2 |

# ScnScanZone Table

The ScnScanZone table (Configuration Menu | Scanner Utilities | Scan Zone Config) provides a way to associate one or more scanners or scales with a particular point on the line.  Most PNA lines will have two scan zones:

- LPN Scan Zone

- Verify Scan Zone

Each scan zone must be configured with the following parameters:

- ScanZoneID – A unique alphanumeric name.  This name will be inserted into PNA messages LRM and LVM and must be coordinated with the Level II2 PNA engine.

- TrackingTable – Select a tracking table:

  - SrtTrack1 – SrtTrack5

- DeviceTrkTable – Select a device tracking table:

  - SrtDevice1 – SrtTrack5

- PrinterTrkTable – Select a printer tracking table:

  - PrtTrkTable1 – PrtTrkTable5

- PrinterErrTable – Select an error table to capture errors for the line:

  - PrtTrkErrTbl1 – PrtTrkErrTbl5

- CTRLProcess – select “PA (ScanProcess)”. This activates the special processing needed for the Print Apply Scan Processing.

  - PA (ScanProcess)

Usually, each PNA line has a separate AC, so only the first set of tables is needed (SrtTrack1, etc.).  The higher-numbered table sets are only needed if multiple Print Apply Lines are configured on a single AC.

### Example ScnScanZone Table

Note:  The Tracking Table must be the same table for all the Scan Zones on a Print Apply Line.

NOTE:  Versions prior to V11.47.0 have a bug that requires the TrackingTable and DeviceTrkTable pair chosen to be defined with the same number of records for Print and Apply applications.  If DeviceTrkTable is larger than TrackingTable, the symptom is LRM messages will stop being generated when DeviceTrkTable row numbers greater than the last row of TrackingTable are used.  This is not a problem for Sorter applications.

| ScnScanZone (Part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Delimiter | ScanZoneID | TrackingTable | DeviceTrkTable | PrinterTrkTable |
| PA1 LPN Zone |  | PNA1LPN | SrtTrack1 | SrtDevice1 | PrtTrkTable1 |
| PA1 Verify Zone |  | PNA1Verify | SrtTrack1 | SrtDevice1 | PrtTrkTable1 |

| ScnScanZone (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PrinterErrTable | CTRLProcess | ScanAlgorithm |  |  |  |
| PrtTrkErrTbl1 | PA (ScanProcess) | N/A |  |  |  |
| PrtTrkErrTbl1 | PA (ScanProcess) | N/A |  |  |  |

# ScnScanDevice Table

The ScnScanDevice table (Configuration Menu | Scanner Utilities | Scan Device Config) is used to define individual scanner or scale connections for a scan zone.  Each PNA scan zone will contain at least one individual scan device.  Each scan device must be configured with the following parameters:

- ScanZone – Select the scan zone associated with this scan device.

- ScanType – Select the scan type to define how the data will be used.  These are the most common types:

  - LPN Scan – Use this type to read the identity of the carton.

  - Verify Top, Verify Left, Verify Right, Verify Front – Use these types for verification scans.  Choose the one that matches the location of the label, as viewed along the direction of travel.  They map to Top Label, Left Label, Right Label, and Front Label, respectively on the Operator’s screen.
Note:  The labels on the Operator’s screen can be changed to suit the application in the usual way – using the Ctrl-D Designer Pane.

- Field – Use this if scanner data needs to be parsed into multiple fields.  This column provides the ability to pick a specific parameter from a scan message that contains multiple delimited parameters.  The scan message must be stored in the ScnScanZone table, instead of the ScnScanDevice table in this case.  Use the Delimiter field in table ScnScanZone to define how to separate the fields.

- UpdatePt – Select the appropriate shifter update point.  This is the point where LPN Scan or Verify scan results are married to a carton.

Note:  If the scan device is part of a scan zone containing more than one scan device, and the scan device’s data is transmitted to FortnaPlus in one delimited string from a master unit, you need only define one update point for the scan event.  In this situation, the single update point should be selected for each of the scan device records in the scan zone.

- NoReadString – Enter the string that the scanner will send to define a NoRead, usually something similar to “????????”.  If the scan results exactly match this string, the LRM will supply the string “NOREAD” to the PNA engine.

- ReadConflictStr – Enter the string that the scanner will send to define a ReadConflict, usually something similar to “********”.  If the scan results exactly match this string, the LRM will supply the string “READCONFLICT” to the PNA engine

- MinWeight – If a scale is integrated into a scan zone, enter the minimum valid weight in the units that the scale will report.

- MaxWeight – If a scale is integrated into a scan zone, enter the maximum valid weight in the units that the scale will report.

### Example Scan Device Table

| ScnScanDevice – (Part 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | Field | UpdatePt | NoReadString |
| PA1 LPN | PA1 LPN Zone | LPN Scan | N/A | PA1 LPN Scan | ???????? |
| PA1 Verify | PA1 Verify Zone | Verify Right | N/A | PA1 Verify Scan | ???????? |

| ScnScanDevice (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ReadConflictStr | StartChars | ChkLength | MinWeight | MaxWeight |  |
| ****** | 0 | N | 0 | 0 |  |
| ****** | 0 | N | 0 | 0 |  |

The remaining tables describe the configuration specific to PNA lines.  They are found under Configuration Menu -> PrintApply Utilities.

# PrintApplyLine Table

The PrintApplyLine table (Configuration Menu | PrintApply Utilities | Printer Apply Line) defines the high-level PNA line configuration.

- AssignType – This parameter specifies the algorithm to be used in sharing labeling tasks among multiple printers.  Choose from the following options:

  - Static Assign – No round robin algorithm.  Labels will be sent to specific printers based upon their label type as per the LabelTemplates table.

  - Round Robin – Printer tasks are shared among available printers in round-robin fashion, dependent upon label type.

  - Line Round Robin – Printer tasks are shared among available printers in round-robin fashion, regardless of label type.  (TRK:  I can’t imagine when this would be useful.  Use Round Robin instead)

- SendType – This parameter specifies when the label image is sent to the printer.

  - Scan – Lbl Snd:  The label is sent to the printer as soon as it is received from the PNA Engine.  You probably don’t want this option!

  - Upd Pnt – Lbl Snd:  The label is not sent until the carton reaches the “Send Label” update point (see section 3.9).  This is the preferred configuration option.  This forces the label to be sent in carton sequence.

- ControlMachine – Select the PNA AC name.

- NumPrinters – Enter the number of printers installed on the PNA line.

- ScnErrSendLRM – Enable this to send an LRM when a scan error (No Read, No Scan, Read Conflict) occurs.

- TampClrSendLVM – Enable this to send an LVM if the carton’s tamp flag has been cleared.

- LrmMsgTable – Select the message table to be used for transmitting Label Request Messages.  Usually:  MsgCR1 – MsgCR5

- LvmMsgTable – Select the message table to be used for transmitting Label Verify Messages.  Usually:  MsgDC1 – MsgDC5

- RcvLabelTable – Select the table to be used for queuing received labels.  Usually:  RcvLabels1 – RcvLabels5

- SndLabelQueue – Select the table to be used for transmitting received labels to the destination printer.  Usually:  SndLabelQue1 – SndLabelQue5

- Printer Errors – To stop the line when errors occur, the PrinLineCnvMtr and MtrACTION must be selected with the associated motor.

- Printer Wait Points – To stop the line at wait points, the PrintLineSpdCtrl and SpeedACTION must be selected with the associated motor.

### Example Print Apply Line Table

| Print Apply Line Table (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Assign Type | TestMode | Send Type | LogLevel | ControlMachine | NumPrinters |
| PA1 | Round Robin | Test Mode Off | Upd Pnt – Lbl Snd | Level 1 | CP1 | 2 |

| Print Apply Line Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ScanErrSendLRM | TampClrSendLVM | LrmMsgTable | LvmMsgTable | TrmMsgTable | TstMsgTable |
| Y | Y | MsgCR1 | MsgDC1 |  |  |

| Print Apply Line Table (Part 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| RcvLabelTable | SndLabelQueue | PrintLineCnvMtr | MtrACTION | MtrBIT |
| RcvLabels1 | SndLabelQue1 | PA1Mtr | Disable | 5 |

| Print Apply Line Table (Part 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| PrintLineSpdCtrl | SpeedAction | SpeedBit | AutoErrHandling | NoPrintersError |
| PA1Mtr | Disable | 6 | Y | Shutdown Line |

# PrinterBoss Table

The PrinterBoss table (Configuration Menu | PrintApply Utilities | Printer Configuration) defines the configuration for each printer on the PNA line.

Create an entry in the table for each printer installed on the PNA line.

- Select the appropriate PNA line in the PrintLine column.

- Select the respective printer name in the PrinterMachine column.

  - In the PrinterMachine column, select the Machine table entry that sends labels to the label printer (Example:  Panther_P1 – Port: 9100)

- Select the appropriate label applicator type (top or side) in the ApplType column.

- Select the appropriate conveyor parts in the PrinterErrorIn, TamperHomeIn, LabelOnTampIn, CycleCompleteIn, PrinterResetOut, TamperSolOut, and PrinterErrLight columns.

  - The Conveyor Parts for the PrinterResetOut and the TamperSolOut are controlled by the SORT Process (Conveyor Parts table – ProcNum: SORT).  All other parts are controlled by the CTRL Process.

### Example Printer Boss Table

| PrinterBoss Table (Part 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | AvailAssign | Enabled | DontAutoEnable | PrintLine |
| PA1 Printer 1 | ON | ON | OFF | PA1 |
| PA1 Printer 2 | ON | ON | OFF | PA1 |

| PrinterBoss Table (Part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PrinterMachine | ApplType | PrinterErrorIn | TamperHomeIn | LabelOnTampIn |
| Panther_P1 | Top | ??? | ??? | ??? |
| Panther_P2 | Top | ??? | ??? | ??? |

| PrinterBoss Table (Part 3) |  |  |  |
| --- | --- | --- | --- |
| CycleCompleteIn | PrinterResetOut | TamperSolOut | PrinterErrLight |
| ??? | ??? | ??? | ??? |
| ??? | ??? | ??? | ??? |

| PrinterBoss Table (Part 4) |  |  |
| --- | --- | --- |
| PrinterErrACTION | PrinterErrBIT | PrinterBelt |
| Fire | 0 | Belt 1 |
| Fire | 0 | Belt 1 |

# PrinterErrConfig Table

The PrinterErrConfig table (Configuration Menu | PrintApply Utilities | Printer Error Config) defines the error conditions and actions for each printer on the PNA line.

Create an entry in the table for each printer installed on the PNA line.  Select the desired error actions and options for each printer.

- Turn the Enable column ON.  This enables the error action.

- Select the appropriate printer in the Printer column.

- Select N in the UpdateHistory column.

- Select N/A in the HRRecord column.

- Select Y in the AutoResetCons column.  This allow the software to automatically reset after an error if the next carton is processed without an error.

- Set MaximumVerifyErrs to define the maximum number of verification errors that are allowed before the applicator will be taken offline.

- Turn DisableOnMaxVerifyErrs ON to disable a printer if the MaximumVerifyErrs quantity is reached.

- Turn DisableLineOnPrinterErr ON to stop the print apply line when any one of the printers experiences an error.  The error may be caused by an internal error, verify error, or missing label.

- Turn DisableOnLOTARFail ON to disable a printer if a carton reaches the printer and the LOTAR signal is not ON.  The LOTAR signal is defined in the PrinterBoss table, field LabelOnTampIn and indicates that a Label is On the Tamp ApplicatoR.

- If DontClearIfSent is set to Y the “Sent”, “Labeled” or “Tamped” status of a carton will not be cleared with the printer “Clear” button or the line “Clear All” button on the Operator Print Apply Screen.

When set to Y, only cartons that do not have all their labels and cannot be labeled manually (manually feeding them from the applicator) are cleared from the belt after a line stop.

When set to N, the “Sent”, “Labeled” or “Tamped” carton status will be cleared with the printer “Clear” button or the line “Clear All” button on the Operator Print Apply Screen, requiring all in-process cartons (between the induct eye and the verify scanner) to be re-introduced prior to the induct photoeye for re-processing.

- If ClearLabelStatus is set to Y, the label status is set to “Error” with the “Clear” or “Clear All” buttons.  In “Auto Mode” the label status is set to “Error” when a printer goes “Offline”.  Setting the label status to “Error” allows the carton to continue without stopping when it reaches an applicator or when it reaches a verify point.  All cartons with a status of “Error” will be sent to the exception lane.

Set ClearLabelStatus to N to prevent printer errors, the printer “Clear” button, or the line “Clear All” button from setting the carton status to error.

- If DontClearOnVerifyErr is set to Y and DisableOnMaxVerifyErrs is set to Y, the line will stop, however the printer that has the verify error will not have its tracking records cleared.  This allows the line to stop, but the tracking records will not be cleared unless the “Clear All” is clicked. This provides the ability to stop the line on verify errors, but if the label is okay, the operator can click on “Reset All” and “Reset Fault” allowing the line to restart without removing the cartons.

Set DontClearOnVerifyErr to N to set the carton status to “Error” when a verify error occurs, allowing the cartons on the belt to continue without stopping.  The carton is sent to the exception destination.

### Example PrinterErrConfig Table

| PrinterErrConfig Table (Part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Enable | Printer | UpdateHistory | HRRecord |
| PA1 P1_Errors | Y | PA1 Printer 1 | N | N/A |
| PA1 P2_Errors | Y | PA1 Printer 2 | N | N/A |

| PrinterErrConfig Table (Part 2) |  |  |  |
| --- | --- | --- | --- |
| AutoResetCons | MaximumVerifyErrs | DisableOnMaxVerifyErrs | DisableLineOnPrinterErr |
| Y | 3 | Y | N |
| Y | 3 | Y | N |

| PrinterErrConfig Table (Part 3) |  |  |  |
| --- | --- | --- | --- |
| DisableOnLOTARFail | DontClearIfSent | ClearLabelStatus | DontClearVerifyErr |
| Y | N | N | N |
| Y | N | N | N |

| PrinterErrConfig Table (Part 4) |  |  |
| --- | --- | --- |
| ErrorIO | ErrorACTION | ErrorBIT |
| BeaconLine1 | Fire | 0 |
| BeaconLine1 | Fire | 0 |

# PrinterUpdPnts Table

The PrinterUpdPnts table (Configuration Menu | PrintApply Utilities | Update Pnts) connects shifter update points to PNA events.  For each update point:

- Select the PNA update point type in the Type column.

  - Wait Label Feed – This update point type may be selected to hold the container until the PNA Engine responds to the LRM and to wait if the label has NOT been fed onto the tamp head (if the signal is available from the applicator).  This update point is used to hold the container prior to the applicator to allow time for the conveyor to return to full speed before applying a label.

Adjust the “Wait Label Feed” point to a location between the scan point and the “Tamp Small” update point.  The location should be far enough in front of the labeler to allow it to return to full speed before the applicator cycles.  This improves the consistency of label placement.  The PrintBelt and SpdControl tables must be configured, and PrintBelt must be selected in the PrinterBoss table.

To verify the “Wait Label Feed” point is set properly, unplug the Ethernet cable, and make sure the carton stops at least 12 ticks or 0.5 seconds prior to the “Tamp Small” update point.  The box location is found on the graphical shifters window, “Boxes Info” tab (second column).  The “Points Info” tab shows the location of the “Tamp Small” update point.

  - Send Label Update – This update point type may be selected for each printer and is used to both send the label data to the printer and to hold at the designated point to wait for the label data to be received from the PNA Engine.  If PrintApplyLine_SendType is set to “Upd Pnt – Lbl Snd”, this update point type must be defined for each printer.  If the SendType is set to “Scan – Lbl Snd” the send update points are not required.  Adjust this update point to a location between the scan point and the “Wait Label Feed” update point.  The print apply line will stop and wait for the host to define the carton requirements and, if a label is required, the print apply line will stop and wait for the label to be received.  Once the label is received, the label will be sent to the printer and the carton will move on to the “Wait Label Feed” update point.  The Send Label Update point must be adjusted past the scan update point for the longest carton to ensure the conveyor does not stop before the scanner data is received (“NO SCAN” and “NO READ” events do not occur until the carton tails out of the scanner photoeye and the scanner update point).

- Select the PNA line in the PrintLine column.

- Select the respective printer in the Printer column.

- Select the label number in the LabelNum column.

- Select the respective shifter update point in the UpdatePnt column.

### Example Sorter Updates Table and PrinterUpdPnts Table

| Sorter Updates |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Update Name | Sorter | Update Number | Location | Name | Window Size | Description |
| PA1 LPN Scan | PA1 | 0 | 70 | INVALID | 0 | PA1 LPN Scan |
| PA1SendP1 | PA1 | 1 | 90 | INVALID | 0 | PA1SendP1 |
| PA1SendP2 | PA1 | 5 | 100 | INVALID | 0 | PA1SendP2 |
| PA1P1 Wait | PA1 | 2 | 175 | INVALID | 0 | PA1P1 Wait |
| PA1P1-L1 Small | PA1 | 3 | 185 | INVALID | 0 | PA1P1-L1 Small |
| PA1P1-L1 Large | PA1 | 4 | 190 | INVALID | 0 | PA1P1-L1 Large |
| PA1P2 Wait | PA1 | 6 | 200 | INVALID | 0 | PA1P2 Wait |
| PA1P2-L1 Small | PA1 | 7 | 220 | INVALID | 0 | PA1P2-L1 Small |
| PA1P2-L1 Large | PA1 | 8 | 226 | INVALID | 0 | PA1P2-L1 Large |
| PA1 Verify Scan | PA1 | 9 | 348 | INVALID | 0 | PA1 Verify Scan |

| PrinterUpdPnts Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Type | PrintLine | Printer | LabelNum | UpdatePnt |
| PA1SendP1 | Send Label | PA1 | PA1 Printer 1 | Label 1 | PA1SendP1 |
| PA1SendP2 | Send Label | PA1 | PA1 Printer 2 | Label 1 | PA1SendP2 |
| PA1 Wait P1 | Wait Label Feed | PA1 | PA1 Printer 1 | N/A | PA1P1 Wait |
| PA1 P1L1 Small | Tamp (1 Label) Small | PA1 | PA1 Printer 1 | Label 1 | PA1P1-L1 Small |
| PA1 P1L1 Large | Tamp (1 Label) Large | PA1 | PA1 Printer 1 | Label 1 | PA1P1-L1 Large |
| PA1 Wait P2 | Wait Label Feed | PA1 | PA1 Printer 2 | N/A | PA1P2 Wait |
| PA1 P2L1 Small | Tamp (1 Label) Small | PA1 | PA1 Printer 2 | Label 1 | PA1P2-L1 Small |
| PA1 P2L1 Large | Tamp (1 Label) Large | PA1 | PA1 Printer 2 | Label 1 | PA1P2-L1 Large |

# PrintBelt Table

The PrintBelt table (Configuration Menu | PrintApply Utilities | Printer Belt) is used to define the parameters associated with the PNA Belt.  The system uses the parameters to control the speed of the belt when a carton is present that gets a label, when a carton gets two labels, and when a carton needs to hold and wait for messages or labels to be sent to the printer(s).

Add an entry for the PNA line and select the appropriate shifter in the Sorter column.  Set the EnableFlag to Y.  Set the belt speed columns as per the application requirements.

### Example PrintBelt Table

| PrinterBelt |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | EnableFlag | Sorter | SlowSpeed | MaxSpeed | WaitRdySpeed |
| PA1 Belt | Y | PA1 | 0 | 100 | 0 |

# HeightWidth Table

The HeightWidth table (Configuration Menu | PrintApply Utilities | Height Width Config) specifies the carton dimension measurement and validation parameters.  A record must be defined for each PNA line, even if height and width measurement devices are not used!

- Select the appropriate sensor type in the SensorType column.

- Select the PNA line in the PrintLine column.

- If the sensor type is a Photoeye:

  - Select the appropriate conveyor parts and IO for the following fields:

    - BoxDetectIn – not used

    - TooShortIn – not used

    - Height1In – Height 1 PE

    - Height2In – Height 2 PE

- If the sensor type is Ultrasonic (also use this for laser range finders):

  - Enter the IO word that is storing the incoming data in the HeightInDT field.  The value expected here is in decimal notation.  Your value may be in Octal notation, so convert accordingly.

Note:  If your version of FortnaPlus includes the Height Width Calibration wizard (Configuration Menu | PrintApply Utilities | Height Width Calibration) this wizard can be used to simplify the following steps.

- Enter the lowest carton height in the BoxHgtInches1 column.

- If using an Ultrasonic device, enter the raw data value for the lowest carton height in the SensorHgt1 column.

- Enter the highest carton height in the BoxHgtInches2 column.

- If using an Ultrasonic device, enter the raw data value for the highest carton height in the SensorHgt2 column.

- If measuring carton width, use the respective width columns:  BoxWdtInches1, BoxWdtInches2.

- If each carton’s height should be validated to be within the given tolerance, set the ValidateHeight column to Y.

- If each carton’s width should be validated to be within the given tolerance, set the ValidateWidth column to Y.

- Enter the minimum and maximum allowable carton heights in the MinHeightInches and MaxHeightInches columns.

- Enter the minimum and maximum allowable carton widths in the MinWidthInches and MaxWidthInches columns.

Note: the HeightWidth table requires a record even if Height and Width are not used for the Print Apply Line.  The minimum configuration required is:  “Name”, “SensorType”, and “PrintLine”.  An update point is not required.

### Example HeightWidth Table (Photoeye)

| HeightWidth (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Sensor Type | Print Line | Box DetectIn | Too ShortIn | Height1In | Height2In | HeightInDT |
| PA1 HW | Photoeye | PA1 | Invalid | Invalid | PE_Height1 | PE_Height2 | 0 |

| HeightWidth (Part 2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BoxHgt Inches1 | BoxHgt Inches2 | BoxWdt Inches1 | BoxWdt Inches2 | Length TicksPerInch | ScalingFactor |
| 5.00 | 14.00 | 0.00 | 0.00 | 1.00 | 100 |

| HeightWidth (Part 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ValidateHeight | ValidateWidth | MinHeightInches | MaxHeightInches |
| Y | Y | 0.00 | 0.00 |

| HeightWidth (Part 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| MinWidthInches | MaxWidthInches | LPNScanZone | VerifyScanZone |
| 0.00 | 0.00 | N/A | N/A |

# PAScannerBoss Table

The PAScannerBoss table (Configuration Menu | PrintApply Utilities | Scanner Config) associates scanner devices with a PNA line.  Create a new record for each LPN scanner, Verify scanner or scale on the line.

- Select the PNA line in the PrintLine column.

- If the verification process should generate a Label Verify Message (LVM), select “Send LVM Msg” in the SendLVM column for the verify scanner entry.

- Select the appropriate verify type in the VerifyType column.  The choices are “Verify One” and “Verify All”.

- Select the related scan device in the ScanDevice column.

### Example PAScannerBoss Table

| PAScannerBoss |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | PrintLine | SendLVM | VerifyType | ScanDevice |
| PA1 LPN Scan | PA1 | N/A | N/A | PA1 LPN |
| PA1 Verify Scan | PA1 | Send LVM Msg | Verify All | PA1 Verify |
| PA1 Scale | PA1 | N/A | N/A | PA1 Scale |

# PAPrinterScanAssign Table

The PAPrinterScanAssign table (Configuration Menu | PrintApply Utilities | Printer Scan Assign) associates verification scanner devices with printers on the PNA line.  Printers are also assigned to specific label types using this table.  Multiple printers (in load sharing applications) may be assigned to the same verification scanner.

### Example PAPrinterScanAssign Table

| PAPrinterScanAssign |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Printer | LabelNum | PAScanRecord |
| PA1 P1 Verify | PA1 Printer1 | Label 1 | PA1 Verify Scan |
| PA1 P2 Verify | PA1 Printer2 | Label 1 | PA1 Verify Scan |

# PStConvertTable Table

The PStConvertTable table (Configuration Menu | PrintApply Utilities | PnA to Sorter Convert) is used to transition a PNA line to a sorter without the need for a scan device.  Essentially, a fake scan event will be generated to trigger the sorting functionality.  This table is not needed if the post-PNA sorter has a physical scanner.

For each PNA line that feeds a sorter with a fake scanner:

- Create a shifter configuration.  The shifter used to define the sorter will have its own induct photoeye.  The photoeye is important to find the location of the carton.  The carton may shift on the belt when the label is applied.  The data from the PNA line must transition onto the sorter shifter using a Dump Point.

- The sorter is configured with all the associated tables, except for the Machine table entry and MsgMap entry for the scanner.  The PStConvert logic will transition the scan data from the print apply tracking record into the ScnScanDevice table.

- Create a scan zone in the ScnScanZone table.  This zone identifies the scan zone for the PNA engine.  Create the scan zone with the same tracking table as the PNA line.

- Create a scan device in the ScnScanDevice table.  This device will be used to generate the fake scan event.

- Create an update point that will trigger the fake scan event:

  - Note:  The positioning of this update point is important.  When selecting the update point position, you must ensure that the distance between the verify scan point and the PSTConvert update point (as measured in encoder ticks) is greater than the length of the system’s largest carton (as measured in encoder ticks).  Failure to meet this space requirement can result in interference with the label verification process.

- Add a record to the PStConvertTable for each PNA line that needs a fake scanner.

  - Select the fake scan device in the ScanDevice column.

  - Select the fake scan update point in the UpdatePt column

  - If UseVerScan is set to N/A, the fake scan value will be taken from the LPN scan (the tracking table ConfirmScan string).  If a label template is chosen, the fake scan value will be taken from the verification scan (the printer tracking table L1VerifyScan string).  This allows the feature to be used with SKU-based systems where the newly-applied label can be used to uniquely identify the carton.

### Example ScnScanZone Table

| ScnScanZone (Part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Delimiter | ScanZoneID | TrackingTable | DeviceTrkTable | PrinterTrkTable |
| PA1 LPN Zone |  | PNA1LPN | SrtTrack1 | SrtDevice1 | PrtTrkTable1 |
| PA1 Verify Zone |  | PNA1Verify | SrtTrack1 | SrtDevice1 | PrtTrkTable1 |
| Ship |  | Ship | SrtTrack1 | SrtDevice1 | PrtTrkTable1 |

| ScnScanZone (Part 2) |  |
| --- | --- |
| CTRLProcess | ScanAlgorithm |
| PA (ScanProcess) | N/A |
| PA (ScanProcess) | N/A |
| blank | N/A |

Note: Use the same Tracking Table for Print Apply and the Sorter.

Note:  CTRLProcess should be blank for fake scanners.

### Example ScnScanDevice Table

| ScnScanDevice |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | Field | UpdatePt | NoReadString |
| PA1 LPN | PA1 LPN Zone | LPN Scan | Field 1A | PA1 LPN Scan | ???????? |
| PA1 Verify | PA1 Verify Zone | Verify Right | Field 1A | PA1 Verify Scan | ???????? |
| Ship Scan | Ship | Scan | N/A | ShipSorterUpdate | ???????? |

### Example Printer to Sorter Convert Table

| PstConvertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Status | ScanDevice | UpdatePt | UseVerScan |
| PA1 to Ship |  | FakeScanDevice | ShipSorterUpdate | N/A |

### Example SrtAppControl Table

| SrtAppControl |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | CrrMsg Table | DcmMsg Table | CsuMsg Table | Control Machine | SorterCnvMtr | MtrACTION | MtrBit |
| Ship | MsgCR2 | MsgDC2 | None | PNA1 | CnvMtr | Disable | 4 |

### Example SrtZoneLane (Destination) Table

| SrtZoneLane |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enabled | AppSorter | Lane | Host Zone |  |  |  |
| Lane1 | Y | Ship | Lane 1 | Lane 1 |  |  |  |

### Example SrtScanBoss (Scan Exceptions) Table

| SrtScanBoss |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | AppSorter | ScanZone |  |  |  |  |  |
| Ship Scan | Ship | Ship |  |  |  |  |  |

# LabelTemplates Table

The LabelTemplates table (Configuration Menu | PrintApply Utilities | Label Configuration) is used to define the label types to be used for each PNA line and which printer(s) can be assigned those label types.  In some systems, certain label types (such as NoRead and NoData) might be assigned to one specific printer, while others can be sent to any printer.  Additionally, a label number can be specified for systems where more than one label is applied to the same box from a single printer.  This table defines such assignment rules.  Create an entry for each label type, per printer, per label number:

- Select the destination printer in the Printer column.

- Select the label number in the LabelNum column.  The choices are “Label 1” or “Label 2”.

- Select the label type in the LabelType column:

  - Template – A special label that contains static information and placeholders that are replaced with data from the LMI message.  Templates can be used to reduce the size of the LMI message by eliminating the need to re-send static things like company logo images with each label.  The template is sent to the printer when FPC is started, and each time the printer is reset.

  - Standard Label – The most commonly used type.  The PNA Engine sends a complete label image in the LMI each time with all the data filled in.

  - Test Label – A static label image loaded from the file defined in LabelFileName for use when TestMode = “Test Mode ON” in table PrintApplyLine.

  - Host Label – Used to define a host-selectable error label.  The PNA Engine can select the label using the ExtLabelType, so multiple labels of this type may be defined.

  - Clear Printer – Used to define a special label image that will cause the printer to clear its print buffer.  The static label image is loaded from the file defined in LabelFileName.  It is sent to the printer when the operator selects the Clear button.

  - No Read – Optional.  Used to define a special label image that will be applied when a NoRead is received from the container identifier scanner (LPN or SKU).  The static label image is loaded from the file defined in LabelFileName.

  - No Data – Optional.  Used to define a special label image that will be applied when a NoData event occurs.  A NoData event occurs when the LRM response indicates an error.  The static label image is loaded from the file defined in LabelFileName.

- Select load type in the LoadType column:

  - File Load – The label image is stored locally in the file defined by LabelFileName.

  - Host Load – The label image is requested from the PNA Engine when FPC starts or when LoadFile is selected.  All records with the same ExtLabelType will be updated, so it is best to set only one to Host Load, and the others to Host Copy.

  - Host Copy – The label image is sent from the PNA Engine when requested by a Host Load record.  If the ExtLabelType matches, the Host Copy records will be updated also.  For example, if several NOREAD labels are defined, they will all be updated when one is set to Host Load and the rest are set to Host Copy.  This avoids redundant requests.

- If the label is being supplied from an external source, the ExtLabelType column is used to specify the label’s identification string.  This name is an agreed-upon string between FortnaPlus and the PNA Engine.  The strings must be unique within this table, so if multiple print lines are configured in one AC, ExtLabelType must be unique across all of them.  In the JSON message format, ExtLabelType is associated with the applicatorType field.

- If the label is being supplied from the FortnaPlus machine, the LabelFileName specifies the path and label name for the label file.

  - If a relative file path is specified (no leading slash ‘/’), the path is relative to the RUN directory.  It is a good practice to create a special directory under RUN so you can keep all these files together, and they will be backed up with a “Save Data” command.  For example, create directory LABELS under RUN, and place the files there, then a NoRead label might be referred to with: “LABELS/NoRead.zpl”.

  - Files of this type will be loaded automatically when FortnaPlus starts.

  - To force the system to load these on-board files, click the LoadFile button for the desired entry and click Apply.  Check the Status field for file loading results.

### Example LabelTemplates Configuration

| LabelTemplates (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Printer | LabelNum | LabelType |
| PA1 P1 | PA1 Printer1 | Label 1 | Standard Label |
| PA1 P1 Test | PA1 Printer1 | Label 1 | Test Label |
| PA1 P2 | PA1 Printer2 | Label 1 | Standard Label |
| PA1 P2 Test | PA1 Printer2 | Label 1 | Test Label |

| LabelTemplates (part 2) |  |  |  |
| --- | --- | --- | --- |
| LoadType | ExtLabelType | LabelFileName |
| N/A | Ship1 |  |
| File Load |  | [enter path and filename here] |
| N/A | Ship2 |  |
| File Load |  | [enter path and filename here] |

# Round-Robin Tables

Tables RndRobinLast and LineRndRobinLast are used to track the last used printer.  If the feature is to be used, one of the two tables must be populated.

If using the “Line Round Robin” algorithm, open the “Line Rnd Robin Last” table (Configuration Menu | PrintApply Utilities | Rnd Robin | Line Rnd Robin Last).

- Select the PNA Line in the PALine column.

- Select one of the line’s printers in the pLastPRBossIndx column.

If using the “Round Robin” algorithm, open the “Rnd Robin” table (Configuration Menu | PrintApply Utilities | Rnd Robin | Rnd Robin).

- Create an entry for each defined label type by supplying the ExtLabelType string from the LabelTemplates table.

- Select one of the line’s printers in the pLastPRBossIndx column.

### Example RndRobinLast Table

| RndRobinLast |  |  |  |  |
| --- | --- | --- | --- | --- |
| ExtLabelType | pLastPrinter | pLastPRBossIndx |  |
| Ship1 |  | N/A |  |

### Example LineRndRobinLast Table

| LineRndRobinLast |  |  |  |  |
| --- | --- | --- | --- | --- |
| PALine | pLastPrinter | pLastPRBossIndx |  |
| PA1 |  | N/A |  |

# Testing Configurations

The PNA testing subsystem provides a means for testing print and apply functionality without need for the PNA Engine connections.

# Enabling Test Mode

To enable the Print and Apply test mode:

- Open the PrintApplyLine table.

- Select “Test Mode ON” in the TestMode column of the line you wish to test.

- You can also enable test mode from the PNA Operator’s Screen.

Note:  When you have completed testing, remember to set TestMode back to N/A!

# TestConfig Table

The TestConfig table (Configuration Menu | PrintApply Utilities | Test Config Line) provides test data to the testing subsystem.

Create a new entry for each PNA line.

- Select the PNA line to test in the PrintLine column.

- Enter the label type you wish to test in the ExtLabelType1 column.

  - When you wish to test a new label type, modify this entry accordingly.

### Example Test Line Table

| TestConfig |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | PrintLine | VerifyStrSKU | VerifyStrRF | ExtLabelType1 | ExtLabelType2 | RtnStatus Msg |
| PA1 | PA1 |  |  | Ship1 |  |  |

# TestCnfgPrt Table

The TestCnfgPrt table (Configuration Menu | PrintApply Utilities | Test Config Printer) provides test data related to printer assignment and label verification.

Create a new entry for each printer on the PNA line.

- Select one of the line’s printers in the Printer column.

- Supply a label verification string in the VerifyStrL1 column.

### Example TestCfgPrt table

| TestCnfgPrt |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | VerifyStrL1 | VerifyStrL2 |
| L1P1 | L1P1 | 123456 |  |
| L1P2 | L2P2 | 123456 |  |

Note:  If the print apply line is set up for test mode, a row must be entered for each printer or it will not assign a label.  The printer tracking table indicates if a label was assigned.

# Print Apply Status Screen

The PNAStatusDisplayScreen table (Configuration Menu | PrintApply Utilities | PNA Status Data) is used to hold data displayed on the PNA Status Panel.  A row must be selected for each PNA line.

Note:  If a row is not selected in the PNAStatusDisplayScreen table, the screen will not update with printer errors.

### Example PNAStatusDisplayScreen table

| PNAStatusDisplayScreen |  |  |  |
| --- | --- | --- | --- |
| Name | PNALine |  |  |
| PA1 | PA1 |  |  |

Note:  Older Versions – The Print Apply Status Screen updates the “Line Status” from the Encoders table – EnableBit column.  The column must have the correct line run signal selected.

Note:  Newer Versions – The Print Apply Status Screen updates the “Line Status” from the PrintApplyLine table – PrintLineCnvMtr column.  The column must have the correct line run signal selected.

DDS:  pna_system_status_window.  bypass_mode_enabled “Bypass Mode Enabled”.  halt_on_printer_error. “Halt Line on Printer Error”

# Disable Zebra Printer Disconnect

Zebra label printers are configured by default to disconnect automatically after a period of inactivity.  This is not desirable for our purposes.  Set TIMEOUT CHECKING to NO using the printer’s web interface.  See the FPC-PNA-ZebraPrintEngineAutomaticDisconnect document for full detail.