# FortnaPlus Control Print Apply Configuration

Document Revision: 1.7

Date: 19-Oct-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 |  |  | New document |
| 1.1 | 10-May-2012 | T Sollenberger | Add PNA changes for Adcock project |
| 1.2 | 6-Nov-2012 | B C Conklin | Add dimensions to the LVM |
| 1.3 | 25-Jan-2013 | B C Conklin | Update to the latest version |
| 1.4 | 12-Jan-2018 | T R Kremser | Clarify the value expected for HeightInDT |
| 1.5 | 16-Aug-2021 | T R Kremser | General Update and Refresh |
| 1.6 | 10-Jan-2022 | T R Kremser | Add the PStConvertTable_UseVerScan feature |
| 1.7 | 19-Oct-2022 | T R Kremser | Correct errors and omissions. Add information about PandA Communication_Protocol – the JSON message protocol. |

Table of Contents

1.	Purpose	6

2.	Print Apply Design	6

Example Physical Layout:	8

2.1.	Rate Calculation	8

Example Rate Calculation:	9

2.2.	Data Flow – Host Communication	10

2.3.	Label Placement	11

Sources of Placement Error	12

Top Placement	12

Side Placement	13

2.4.	Label Types	13

2.5.	Label Confirmation	14

2.6.	Error Correction / Exception Handling	14

2.7.	Statistics	15

3.	Print Apply Control Logic	15

4.	Print Apply Configuration	18

4.1.	Print Apply Processes	18

4.2.	Table PrintApplyLine	18

4.3.	Table PrinterBoss	23

4.4.	Table PrinterErrConfig	29

4.5.	Other Print Apply Error Detection Options	33

4.6.	Table PrinterUpdPnts	34

4.7.	Table PrintBelt	40

4.8.	Table HeightWidth	41

4.8.1.	Example Configuration:  Photoeye Height Detection	44

4.8.2.	Example Configuration:  Ultrasonic Height Detection	45

4.8.3.	Example Configuration:  Scan Tunnel	45

4.8.4.	Example Configuration:  Adjust Tamp Using Length	45

4.9.	Table PAScannerBoss	46

4.10.	Table PAPrinterScanAssign	48

4.11.	Table PStConvertTable (Downstream Sorter)	49

4.12.	Table TestConfig	50

4.13.	Table TestCnfgPrt	50

4.14.	Table LabelTemplates	51

4.15.	Table PNAStatusDisplayScreen	53

5.	Example Configurations	55

5.1.	Example 1:  Two Printers, Auto-Disable Mode	55

5.2.	Example 2:  Two Printers, Lines Stop on Error	56

5.3.	Example 3:  Two Printers, Auto-Disable, Minimal Clear	56

5.4.	Example 4:  Shipping Label over Contents Label	57

6.	PNA Operators Screen	58

7.	PNA Printer Test Options Screen	60

8.	PNA Operators Screen (Old Version)	61

9.	PNA Message Content Details	63

9.1.	LRM – Label Request Message	63

9.2.	LMI – Label Image Message	65

9.3.	LVM – Label Verify Message	67

9.4.	TRM – Template Request Message	69

9.5.	PRM – Printer Ready Message	70

10.	PNA Dimension Sensor Calibration	72

10.1.	Using Two Photoeyes	72

10.2.	Using One Photoeye	72

10.3.	Using Ultrasonic or Laser Height/Width Sensors	73

11.	PNA Dimension Sensor Update Point Calibration	74

12.	PNA RFID Label Setup	75

13.	Print Apply Speed Control	77

14.	Zebra Printer Configuration	78

15.	Test Mode Configuration	78

16.	JSON Message Protocol	79

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

PNA, PandA – Shorthand for Print and Apply

PNA Engine – The “level II” Fortna server that provides label requirements and verification requirements, and records PNA results.

# Purpose

Print and Apply, or PNA, or PandA refer to the FPC feature for managing label printers and applicators used to attach shipping labels, packing slips or other items to the exterior of containers based on their identity.  The PNA module in FPC communicates with the PNA Engine that provides label information and data on how to verify that the applied labels are correct.

Up to six printers can be controlled in a single PNA instance.  A special PNA operator interface manages control, diagnostics, and error correction.

A shifter is used to track containers through the PNA line.  A series of update points are used to define where various events are expected to happen as the containers are processed.  The Graphical Shifter window is used to adjust and monitor the behavior of containers on the PNA line.  See the FPC-Shifter-And-Sorter-Configuration document for more detail on shifters.

# Print Apply Design

4. Height and Width Scanner

5. Barcode Scanner

14. Print Apply Belt

8. Print Apply Labeler

11. Verification Scanner

1. Skew Section

2. Brake / Meter Belt

3. Tracking Induct Point

6. Scanner Tailout Point

7. Nose of the Longest Box from Scanner Tailout

9. Labeler tamp point

10. Labeler Endpoint, Beginning of Verify Scan

12. Verify Scanner Tailout Point

13. Nose of the Longest Box from Verify Tailout

- Skew Section – Skewed rollers that align the containers with one edge of the belt to ensure accurate label placement.

- Brake / Meter Belt – Used to create appropriate gaps between the containers.

- Tracking Induct Point – The photoeye that measures the length of the incoming container.  The point where tracking begins on the PNA shifter and a tracking record is created to hold the data about the container.  The point is located so that at least three-quarters of the longest container will be on the belt when it breaks the induct photoeye beam.  This ensures the container is moving with the belt and will be accurately tracked.

- Height and Width Scanner – Measures container height and width.

- Barcode Scanner – Reads the LPN or identifying barcode on the container.

- Scanner Tailout Point – The position of the barcode scanner’s trigger photoeye (not a part in FPC).  When the nose of the carton reaches this point, the scanner activates.  When the tail of the container reaches this point, the scan is delivered to FPC.  If no barcode has been detected, a NoRead occurs as the container tail passes this point.

- Nose of the Longest Box from Scanner Tailout – The point where the nose of the longest carton will be as the tail passes the scanner tailout point.

- Print Apply Labeler – The printer and applicator mechanism.  Typically requires 48 inches of space along the belt.  Often mounted on a movable frame that can be retracted for ease of service, it is best to mount it on the side that the containers will be justified to.  Order the configuration that places the tamp mechanism at the downstream end to allow the most time to prepare the label.

- Labeler Tamp Point – The point where the label will be applied.  This point must be located to allow at least four seconds from point 7.

- Labeler Endpoint, Beginning of Verify Scan – This point represents the end of the label applicator and the beginning of the verification section.

- Verification Scanner – Reads the applied label to verify that the correct label was placed on the container.

- Verify Scanner Tailout Point – The position of the verification scanner’s trigger photoeye (not a part in FPC).  The point at which verification scans are delivered to FPC.

- Nose of the Longest Box from Verify Tailout – The point where the nose of the longest carton will be as the tail passes the verification scanner tailout point.

- Print Apply Belt – The conveyor segment, represented by a shifter, that tracks the cartons as they move through the PNA process.

### Example Physical Layout:

Design a PNA line that will have one printer.  The longest carton will be 34 inches.  The tallest carton will be 18 inches.  The required throughput is 30 cartons per minute (2 seconds per carton).  The identifying label is on the front of the carton.

- Because the identifying label is on the front of the carton, the gap between cartons must be set to at least 18 inches (for best scanner read performance, the read angle should not exceed 45 degrees).  Allow additional gap for a safety factor:  26 inches.  That makes the longest carton plus the gap a total of 60 inches.

- The required belt speed is ((60 In / 2 Sec) * (60 Sec / Min)) / (12 In / Ft) = 150 Ft / Min, which is achievable with the standard SORT configuration.

- The induct point location is 34 * 0.75 = 25 In.

This is a basic layout.  More complex layouts can be supported.  Two identical applicators can be placed in series to handle the same label type for increased throughput.  Applicators for different label types, such as a shipping label and a pack slip label, could be applied and verified on each container.  Labels can be placed on the top, front, or side surfaces of the container.  Up to six total printers are supported with up to six different label types, or in any combination of redundant applicators.

## Rate Calculation

The design rate of each PNA line is constrained by several factors:

- The number of printers for each label type.  Redundant printers increase the throughput, subject to the other constraints.

- The size of each label.

- The label feed rate of the printer.

- The time needed to generate the label image.  When applying RFID tags, the write/read cycle time of the RFID encoding device will be a factor.

- The time needed to deliver the label image to the printer.  Modern printers can store multiple label images and can receive the next label image while printing the current one, reducing the impact of this factor.

- The response time of the PNA Engine and/or host system providing the label image and verification data.

- The location on the carton of the identifier barcode.  If the label is on the front of the carton, the gap between containers must be sufficient for the scanner to “see” the label.  Setting the gap to the height of the tallest container produces a 45-degree read angle – the steepest practical angle for reliable reads.

- The label placement accuracy requirement.  To achieve higher accuracy, the belt speed must be reduced, or the encoder resolution must be increased.  With the standard 5 mSec SORT process update time in FPC, the encoder pulse rate limits belt speed to 166 Ft/Min with a 0.5-inch resolution or 83 Ft/Min with a 0.25-inch resolution.  The SORT process update time can be reduced to as low as 2 mSec for increased performance.

### Example Rate Calculation:

Continuing the design of our PNA line with one printer.  The printer has a feed rate of 8 inches per second, printing a 6-inch label.  It will take 0.75 seconds to print the label.  It takes 0.25 seconds for the printer to process the label image.  The label will be applied to the top of the carton.  The cartons will range in height from 10 to 20 inches.  The applicator can travel at up to 24 inches per second (this may need to be reduced to avoid damaging the container or the contents).  The label must be placed with +/- 0.5-inch accuracy.  The longest carton is 36 inches.  The identifying label is on the side of the carton.

- Because the identifying label is on the side of the carton, the gap between containers can be set to 4 inches (a practical minimum safe value).  That makes the longest carton plus the gap a total of 40 inches.

- The printer will take 0.25 + 0.75 = 1.0 second to produce the label, assuming the label image will already be in the printer’s memory before the carton arrives.

- Set the applicator at a height of 22 inches above the belt to allow a safety margin of 2 inches above the tallest carton.

- For a 10-inch-tall carton, the applicator will need to travel a total of 24 inches for the round trip.  That will take 1.0 second.

- The label generation time and the application time consume the entire 2.0 seconds available per carton needed to achieve the 30 cartons per minute requirement.  Allowing a 10% safety factor yields 27 cartons per minute – not quite meeting the required rate.

- At 40 inches per container, that is 1080 In/Min or 90 Ft/Min – so the belt speed is well below the maximum rate supported by the standard SORT process at a 0.5-inch resolution of 166 Ft/Min.

- If a second printer is added, the 2.0 seconds per container printing time is cut in half, resulting in 54 containers per minute (with safety factor), which would result in a belt speed of 180 Ft/Min, so the line would now be constrained by the maximum belt speed to about 49 containers per minute unless the SORT process update time was reduced.  At a 4 mSec update time, both printers could be used at maximum rate as the upper limit would be about 59 containers per minute.

## Data Flow – Host Communication

Each container will require an identifying label to initiate the PNA process.  This is usually a unique LPN, but some systems use a (non-unique) SKU identifier.

The PNA AC sends the identifier to the PNA Engine in an LRM (Label Request Message).  The PNA Engine answers with an LRM response message that contains information on how to verify the applied label(s).  The PNA Engine also sends a LMI (Label Image Message) message that contains the image to be sent to the printer, typically in the form of ZPL commands.  The label image is stored temporarily within the AC until the container arrives at the applicator, when it is sent to the printer.

The PNA Engine may need to request the label image and verification information from the customer’s host system at the time the LRM message is received, or it may have received that information in advance from the customer’s host.  In either case, it is stored in the Oracle database until needed.

Once the label has been applied, the verification scanner reads the label and uses the verification information from the LRM response message to determine success or failure.  The results of the comparison are sent to the PNA Engine in the form of a LVM (Label Verification Message).  Containers that fail verification may be sent to a local or downstream error correction station.  The EC station will interact with the PNA Engine in a similar way to help the operator manually correct the problem and generate a corrected label.

Carton Data (messages from Host)

PNA Engine

Carton Labels (files from Host)

PNA AC

Error Correction Station

Control Network

Various methods are used by our customers to generate and transmit the label image files.  CommCore or the PNA Engine will store the label images and associated label data and verification strings in the Oracle database so they can be accessed when an LRM message arrives.

NOTE:  It is the responsibility of the Host software to generate the label image files with all required printer control commands embedded, such as the printer print speed.

## Label Placement

Labels can be placed on most container surfaces, though the most common are the top or the side.  The containers are justified toward a guide rail on one edge of the conveyor by the skew roller section preceding the PNA belt.  The PNA applicator should be installed on that side.  Consistent label placement is measured relative to the guide rail and the nose or tail of the container for top apply, and relative to the belt surface and the nose or tail of the container for side apply.

Front or even back surface application is possible using special applicators, with placement referenced to the belt surface and guide rail, similar to side-apply.

NOTE:  Labels must not be applied over the identifying barcode to facilitate error correction.

### Sources of Placement Error

Consistent justification of the containers is essential for repeatable label placement.  Poor justification will directly affect top label placement.  Poor justification will result in fore-and-aft placement error for side labels, as the container will move along the belt while the tamper travels the extra distance to reach it.

If the belt speed changes while the tamper is in motion, fore-and-aft label placement error will occur.  If the belt is stopping, the label will be applied forward of the intended position.

### Top Placement

Top of the widest container

Conveyor guide rail (containers are shown justified toward the viewer)

The applicator is also mounted on this side or overhead.

Label

Labels are placed relative to the front edge and the guide rail.

Label

### Side Placement

Label

Labels are placed relative to the front edge and the conveyor surface.

Container

Conveyor

Label

Conveyor

## Label Types

Table LabelTemplates is used to manage definitions of labels for the system.  Column ExtLabelType defines a name for each logical or physical label variant and serves as the agreed-upon name that links the FPC configuration and the PNA Engine configuration.  A label type is associated with the physical label attributes such as label size or label stock color.  It can also be associated with the information the label will contain, such as a shipping label or a packing slip.  The label type is, therefore, linked to the physical printer/applicator, as each type will be printed on certain stock and applied to the container in a certain position and orientation.  Depending on throughput requirements and other design considerations, more than one printer may be designated to handle the same label type, but each label type must have at least one printer/applicator assigned.

If the same label type is assigned to more than one printer, the printers will be used in a round-robin fashion to distribute the load.

Table LabelTemplates can also define special labels that are stored locally on the AC for things such as error labels, template labels, or “reset” labels that send special codes to the printer to clear the print buffer.  Column LabelFileName defines the path to the file on the local system where the label image is stored.  If the connection to the printer is broken then restored, the “reset” label and template labels are re-sent.

NOTE:  In some older systems, the label images are sent directly to the printers from the customer’s host system or from CommCore.  In those implementations, it is necessary to enable and disable the printers and alter the round-robin assignments from those systems.  The FPC AC is configured with a unique label type name for each physical printer, allowing the host to control the printer usage.

See section 4.14 for details on the LabelTemplates table.

## Label Confirmation

Each label type may be configured for verification.  Verification scanners read the applied label and the results are compared with the values designated in the LRM response.  If the comparison fails, the line may be stopped, or the container may be routed to an exception handling lane.

## Error Correction / Exception Handling

If a separate manual PNA workstation is to be used for error correction at an exception handling station, make sure the printer can print on the same label stock and at the same resolution as the automated print apply Labelers.  200 dpi is a typical resolution.  Labels must be rendered for a particular resolution and label size.

The PNA workstation is usually a Linux workstation running special software that can access the PNA Engine and control standalone printers.  It will have a hand-held scanner that can read the container identifier barcode.  It should be in a central area for easy access and to allow the corrected containers to be placed back on the conveyor.

## Statistics

The standard FPC “Scanner Statistics” feature can be used to gather statistics on the read rate of the container identifier scanner and/or the verification scanner(s).

The PNA module can use the scanner information to take action if sequential errors occur.  For example, three NoRead events in a row could cause the line to stop.

# Print Apply Control Logic

Print apply lines can be configured with the following options:

- Template Load Option – The templates for the printer, the error labels for the printer, and the test Labels for the printer may be all configured to load from a local file or from the PNA Engine.

- Request Template – When the FPC AC is initially started it will request templates as defined in the LabelTemplates table.  Template messages (TRM) can also be used to request label templates and error labels for each printer.  See section 9.4.

- Height Width Option – Height and width detection may be configured to use data from several sources (See section 4.8):

  - Photoeye height detection – Photoeyes located at several heights can be used to adjust the tamp point for the applicator.

  - Ultrasonic or laser height or width detection – The tamp point can be interpolated based on the sensor data.

  - Scanner with height and/or width detection – The scanner located prior to the applicator may include a sensor that detects the height and/or width of the carton.  The height and width information is received along with the scan data, and can be used to adjust tamp points.

- Height, Width and Length – The dimensions of each carton are detected and recorded in the tracking record.  The height (top apply) or width (side apply) are used to calculate the tamp point for each label using linear interpolation between the small carton update point and the large carton update point defined for each printer.  The width of a carton is only used in applications where a side label is applied and the distance from the applicator to the carton varies with carton size.

- Static Option or Round Robin Option – Use the static option if a single printer is available for each label type.  If multiple printers are available to apply the same label type, they can be assigned using a round-robin method to distribute the work across all available printers.

  - The round robin option includes the ability to check a carton against a minimum height requirement and assign printers accordingly.  It can also assign a priority to a specific printer (preferred printer).

- LVM Options – A Label Verify Message may be sent to the PNA Engine when each label is verified, or it may be sent after all labels are verified.

- Scan Carton –Scan events trigger a request for carton data.  A Label Request Message (LRM) is sent to the PNA Engine.  The number of labels and their types are returned to the PNA AC.

If the system is configured to use the AC to send the labels to the printer, the PNA Engine will deliver the label data to the AC.  If the system is configured to send the label data directly to the printer, the AC will wait for a host message, or wait for the label presence, before proceeding to the tamp point.

If the AC is used to send the label data to a printer, the AC will determine which printer will be used to apply the label based on the label type.  If the system is configured for round robin, the printer will be selected based on the availability of a printer in a printer group.

In addition to the number of labels and their types, the PNA Engine will also indicate the verify strings for each label.  If a verification field is left blank, the AC will not verify the label.

- Receive LRM – The LRM response includes the label type, which is used to determine the printer that will apply the label to the carton.  If a label type is defined to be stored locally, the AC will load the label from the local file and send it to the printer (For example, a “No Data” label).

- Receive LMI - When a Label Image Message is received, the AC will store the label in the label queue.  The tracking record number is used to identify the carton associated with the label image.  If the “Label Status Flag” is set in the tracking record, the label will be sent to the printer and the “Send Flag” will be set to indicate the label has been sent.  Labels are received and processed using the “PaLblImg” protocol.  The received label is stored in the table selected in the RcvLabelTable column of the PrintApplyLine table.

- Wait Host – For use when the PNA Engine sends the label image directly to the printer.  If the Wait Host update point is defined, the line will stop and hold the carton at the update point until the PNA Engine sends a Printer Ready Message (PRM).  Once received, the AC will set the label status to “Sent” in the tracking record.  The “Sent Status” indicates the label was sent to the printer.  The carton will proceed.

- Wait Label Feed – If this update point is defined, the carton will hold until the PNA Engine sends a response to the LRM.  If the LRM indicates a label is to be applied, the carton will wait until the printer reports a label has advanced on to the applicator.  When the “Label on Tamp” (LOTAR) signal indicates the label is ready, the carton will proceed.

- Send Label – When the carton reaches this update point, the label image is sent to the printer.  If the label image has not been received, the carton will hold until it arrives.  Consult the tracking record for the status of each label.

  - If the “Tamp Flag” is reset by a verify error the label will not be sent to the printer and the carton will proceed.  This prevents the application of a second label in a label-over-label configuration if an error occurs.

- Tamp Small – This update point defines the tamp point for a small carton.

- Tamp Tall – This update point defines the tamp point for a large carton.

- Verify Scan – This update point is used to define the location of a verification scanner.  There are two options:

  - Verify All – All required labels (as defined in the LRM response) are evaluated.  Place the update point after the tailout point of the most downstream scanner.  Scan tunnels may return all the required data in one message.

  - Verify One – Just one label is verified.  This may be needed in label-over-label applications or whenever a specific label must be verified before another label can be requested.

- Test Configuration – Each PNA line can be configured with a test configuration that can be used to verify the scanners, applicators and conveyor controls are working properly.  Switching the line to “Test Mode” allows it to be operated independently of the PNA Engine, so it can be tested by placing any carton on the line.  Test labels are applied, and the labels are verified, however the test data is not sent to the PNA Engine and the test does not affect any waves or orders that are waiting to be processed.

# Print Apply Configuration

  - Print Apply Processes

The Print Apply module requires these additional entries in table Batches:

- PA  SERV (CPN) – Required to run the SERV portion of the software.

- PA  CTRL (CPN) – Required to run the CTRL portion of the software.

- PA  SORT (CPN) – Required to run the SORT portion of the software.

- PA GUI (CPN) – Required to run the GUI portion of the software.

A special scan processing function is also required, but it is selected in the ScnScanZone table, NOT in table Batches:

- PA (ScanProcess) – Required to run the scanner processing software when a PNA scan event occurs.  Select this process in table ScnScanZone, column CTRLProcess.

  - Table PrintApplyLine

The PrintApplyLine table defines the top-level configuration for a PNA Line.  Each line may use multiple printers – either for multiple label types or for redundant capacity, or both.  Multiple PNA lines may be controlled by a single AC (theoretically – this has not been done in practice).

| PrintApplyLine Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the PNA Line. |
| Status | String | Updated by the software to show the status of the line, including error messages or events. |
| AssignType | PopUp | Define how printers will be assigned: Static Assign – One printer is expected to be designated for each label type. Round Robin – If multiple printers are designated for each label type, the work will be divided among them. Line Round Robin – Printer tasks are shared among available printers regardless of label type. (This is not likely to be useful!) |
| TestMode | PopUp | Used to control testing or simulation modes. See section 15 below. Test Mode OFF – When test mode is off, the line will only apply labels sent from the PNA Engine. Test Mode ON – Labels are applied and verified using locally-defined label images. The line does not exchange messages with the PNA Engine. Sim Mode ON – Locally-defined simulation data is used. See the PNA simulation document for details. |
| SendType | PopUp | Note: For all options listed here, the send of the LRM message initiates the process, which is initiated by the scan of the container. Scan – Lbl Snd – Labels are sent to the printer as soon as they are received from the PNA Engine. If TestMode is on, they are sent as soon as the scan is received. Upd Pnt – Lbl Snd – Labels are sent to each printer when the container reaches that printer’s update point. Each printer requires a send update point that must be selected in table PrinterUpdPnts. Use this option to guarantee labels will be sent to each printer in the proper order. This option is required for label-over-label applications. With this method, any labels that are to be applied over another label will not be applied if the underlying label was not applied. Host (PRM) – Lbl Snd – The host sends labels directly to each printer. The PNA module moves the container to the Printer Ready Wait update point. The host sends a PRM message to indicate that the printer is ready. The PNA module checks the label-on-tamp signal before triggering the applicator. Host (No PRM) – Lbl Snd – The host sends labels directly to each printer. The PNA module moves the container to the Label Ready update point and checks the label-on-tamp signal before triggering the applicator. |
| LogLevel | PopUp | Choose the level of detail for log messages: Level 0 – Error messages only Level 1 – Some detail about scans and tamp activity Level 2 – Container size information, operational logic info, additional error messages Level 4 – Applicator details Level 5 – Maximum detail |
| ControlMachine | PopUp | Select the AC that controls this PNA line. |
| NumPrinters | Int | Set the total number of individual printers that are controlled by this PNA Line, whether for different label types or redundant printers for the same label type – count them all. |
| ScanErrSendLRM | Radio | Activate this to send an LRM when a scan error occurs. This should always be activated, as the PNA engine requires the information for proper statistics. The PNA engine can send error labels and keep track of scan errors when this is activated. |
| TampClrSendLVM | Radio | Activate this to send a LVM for each container on the line when the operator clears the line using the “Clear” button to reset after an error. |
| LrmMsgTable | PopUp | Select the message table to use for LRM messages. Start with MsgCR1. |
| LvmMsgTable | PopUp | Select the message table to use for LVM messages. Start with MsgDC1. |
| TrmMsgTable | PopUp | Select the message table to use for TRM messages, if needed. Start with MsgGP1. |
| TstMsgTable | PopUp | Select the message table to use for test messages. Use MsgTst. |
| RcvLabelTable | PopUp | Select the table to use for received label images, via LMI messages. Start with RcvLabels1. |
| SndLabelQueue | PopUp | Select the table to use for sending label images to the printers. Start with SndLabelQue1. |
| PrintLineCnvMtr | PopUp | Choose the output signal that will control the conveyor motor. It will be used to stop the line when a scan, verify or printer error occurs. |
| MtrACTION | PopUp | The action to take on PrintLineCnvMtr when an error occurs: Fire – Turn the signal ON UnFire – Turn the signal OFF Enable – Enable the signal Disable – Disable the signal Latch – Latch the signal ON UnLatch – UnLatch the signal OFF ForceOn – Force the signal ON using Conveyor_Overide_I_O UnForce – Force the signal OFF using Conveyor_Overide_I_O DTForce – Not defined, do not use. * DTUnForce – Not defined, do not use. * DTClrForce – Not defined, do not use. * DTForceClr – Not defined, do not use. * DTUnForceClr – Not defined, do not use. * * -- Would use FORTNADT_Forced_On if defined. |
| MtrBIT | Int | The bit to use for a MtrACTION of Enable or Disable. |
| PrintLineSpdCtrl | PopUp | Select the signal to use to stop the line to wait for events, such as host messages, label-on-tamp signals, etc. Requires an entry in table SpdControl, linked to this record via the PrintLine field and enabled via the EnableFlag field, even though the line might not use servo speed control. When the speed is set to 0 the selected motor is stopped. |
| SpeedACTION | PopUp | The action to take on PrintLineCnvMtr when the speed goes to 0. The choices are the same as defined in MtrACTION, above. |
| SpeedBIT | Int | The bit to use for a SpeedACTION of Enable or Disable. |
| ScanErrLight | PopUp | Select the signal to use when a scan error occurs, as defined in table ScnDeviceErrIO. Requires the ScnDeviceErrIO entries to be linked to this table via the PrintLine field. Alternatively, you may use a memory bit here to activate an Errors entry directly using the TrigIO field. |
| ScanACTION | PopUp | The action to take on ScanErrLight when a scan error occurs. The choices are the same as defined in MtrACTION, above. |
| ScanBIT | Int | The bit to use for a ScanACTION of Enable or Disable. |
| VerifyErrLight | PopUp | Obsolete. Use the PrinterErrConfig table to disable the line on a verification error using the DisableLineOnPrinterErr option or use the “Auto Disable” mode. Select the signal to use when a verification error occurs, as defined in table ScnZoneErrIO for a Bad Read condition. Requires the ScnZoneErrIO entry to be linked to this table via the PrintLine field. Alternatively, you may use a memory bit here to activate an Errors entry directly using the TrigIO field. |
| VerifyACTION | PopUp | The action to take on VerifyErrLight when a scan error occurs. The choices are the same as defined in MtrACTION, above. |
| VerifyBIT | Int | The bit to use for a VerifyACTION of Enable or Disable. |
| DataErrLight | PopUp | An output signal that is turned on when a data error occurs. The ScnZoneErrIO or ScnDeviceErrIO table must be configured to select the appropriate print apply line in PrintLine for each error. You may also use a memory bit here to turn on an error indicator for the line. Example: An entry is made in table ScnDeviceErrIO for ErrorType “No Data” at the LPN scanner. MaximumErrors is set to 2. After two “Unknown container” messages are received from the PNA Engine, the error will be raised, and the line will stop. |
| DataACTION | PopUp | The action to take on DataErrLight when a data error occurs. The choices are the same as defined in MtrACTION, above. |
| DataBIT | Int | The bit to use for a DataACTION of Enable or Disable. |
| ResetLineSignal | PopUp | This is used to reset the print apply line. The transition ON of the signal triggers the same action as “Reset Fault” on the print apply operator’s screen. |
| ClearLineSignal | PopUp | This input signal is used to clear the applicator status for all cartons being tracked on the line and to clear the printer buffers (if configured). The same action occurs as with the “Clear All” function on the operator’s screen. |
| ClearErrLane | PopUp | The outpoint assigned when a “Clear All” is performed. Select the exception handling lane. |
| PassThroughModeEnable | PopUp | Enable or disable pass-through mode for the print apply line. |
| AutoErrHandling | Y/N | When set to Y, printers will be automatically disabled when a print error occurs. The printer is re-enabled when the error condition clears. The “Auto Disable Printers” button on the operator’s screen toggles this setting. Possible printer errors (from PrinterBoss): Printer Error IO signal is ON (PrinterErrorIn) A verification error has occurred A tamp error has occurred (TamperHomeIn) A cycle error has occurred (CycleCompleteIn) A LOTAR error has occurred (LabelOnTampIn) |
| NoPrintersError | PopUp | Choose the action to take when no printer of a given type is available. The options are: “N/A” (take no action) or “Shutdown Line”. |
| HostCommFault | PopUp | Select the SrtHrtBeat record used to check for a host communications fault. When the “HeartBeat – Host Alarm” is triggered, the line will stop, and the Print Apply Line Message will display the error “WCS Communications Lost”. A host communications fault is an error returned from the PNA Engine, it is not an indication that the connection to that system has failed. |
| TampFaultAction | PopUp | The line can be stopped specifically for tamp errors. Set AutoErrHandling = Y and DisableLineOnPrinterErr = N in table PrinterErrConfig. When a tamp fault occurs, the printer will be disabled but the line will NOT stop unless “Shutdown Line” is selected in this field. Possible tamp faults (from PrinterBoss): 1. The applicator is not home (TamperHomeIn) 2. The tamp does not cycle in the time allotted (CycleCompleteIn) |
| ScaleDisable | Int | This is linked to the “Scale Disable” button on the operator’s screen. The inverse of the value set here is passed directly to the PNA Engine through the LRM message sScaleValid field. In some installations, when the scale is disabled in this way, the PNA Engine can substitute a default weight value and allow the line to keep running. The PNA Engine must be configured to understand this extra information. Note that security roles can be adjusted on every field in the operator’s screen using Ctrl-D, so the use of this feature can be controlled at the AC as well. 0 = Scale Enabled. Normal, consecutive error processing will occur for scale-type scan devices. LRM: sScaleValid = 1. 1 = Scale Disabled. Error processing for scale-type devices is suspended. LRM: sScaleValid = 0. |
| PassThruCRR | Int | Not used. |
| pErrorFlag | Int | Used internally to indicate an error has occurred. |
| pResetFault | Int | Used internally for fault reset processing. |
| pMaxResponse | Float | Set internally to the max response time for LRM messages since the last reset. You can enter 0 manually to reset on the fly. |
| pGrtHalfSecond | Int | Number of LRM responses that took longer than 0.5 Sec. |
| pGrtQtrSecond | Int | Number of LRM responses that took longer than 0.25 Sec. |
| pNumSamples | Int | Number of LRM responses in the data set. |
| pTrigError | Int | Used internally to initiate error processing. |
| pErrDesc | String | The last error description received from the PNA Engine. |
| pCurrErrCnt | Int | The current error count. |
| pScanError | Int | Used internally to indicate a scan error. |
| pVerifyError | Int | Used internally to indicate a verification error. |
| pDataError | Int | Used internally to indicate a data error. |
| pWrkScanError | Int | Used internally to process a scan error. |
| pWrkVerifyError | Int | Used internally to process a verification error. |
| pWrkDataError | Int | Used internally to process a data error. |
| pBoxHeightError | Int | Used internally to indicate a height error. |
| pBoxWidthError | Int | Used internally to indicate a width error. |
| pPassThroughMode | Int | Internal flag used to indicate pass-through mode is active. |
| pActivePrinterCount | Int | The current number of active printers. |

  - Table PrinterBoss

The PrinterBoss table is used to define an entry for each printer on the print apply Line.  There are fields for associated control IO, timers, and fault indicators.

| PrinterBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for the printer. |
| AvailAssign | Radio | Enable flag used to make a printer available for assignment. When off, the printer will not be considered for round-robin assignment. |
| Enabled | Radio | Enable flag used to indicate the printer is online and the fault signals should be used to stop the conveyor. If the printer is not enabled, the printer fault signals will not stop the line. This can be manually set to perform service on a printer, such as replenishing labels or ribbon, while the line continues to operate using a redundant printer. |
| DontAutoEnable | Radio | If set to Y the printer will NOT be automatically enabled when printer errors are cleared. If set to N the printer will be automatically enabled when the printer errors are cleared. Pro tip: Select Y to prevent a printer from being put online if it is not working properly. |
| Status | String | Messages that indicate errors or events associated with the printer status will appear here. |
| QueStatus | String | Messages that indicate errors or events associated with the printer queue will appear here. |
| DspStatus | String | Messages that indicate errors or events associated with the label disposition will appear here. |
| PrintLine | Popup | Choose the print apply line associated with this printer. |
| PrinterMachine | Popup | Select the Machine table entry that defines the communication interface for this printer. Printers are usually configured with Connection_Type = CONNECTC_TCPCLIENT, Communication_Protocol = AlphaNum, and Machine_Port = 9100. The CONNECTC_TCPCLIENT (some show CLOSE_CLIENT) connection type causes the connection to the printer to close after each label is sent. The close is often needed as a signal to the printer that the label transmission is complete so it will print and eject the label. |
| ApplType | Popup | Choose the applicator type: Top – Select this if the printer will be applying the label to the top surface of the carton. If a height sensor is available, the height information can be used to adjust the tamp point for more accurate label placement on a variety of box heights. Side – Select this if the printer will be applying the label to the side surface. If a width sensor is available and the label is applied to the side of a carton where the tamp distance will vary, width information can be used to adjust the tamp point in the same way as for variable height cartons with top-apply. |
| VerifyScans | Y/N | Y – The SKU received from the PNA Engine is compared to the ContID from the scanned barcode. N – Use the response from the PNA Engine even if it does not match the container being processed. In some applications the PNA Engine does not respond with matching scan data, such as when the scan data is a compound scan parsed by the PNA Engine. |
| TrailEdge | Y/N | Choose whether the printer should be triggered by the leading (nose) or trailing (tail) edge of the carton. N – Labels will be justified to the nose of the carton. Y – Labels will be justified to the tail of the carton. |
| PrinterErrorIn | Popup | Select the IO input part for the printer error signal. The printer will set this signal when an error has occurred. The AutoErrHandling and PrinterErrConfig_DisableLineOnPrinterErr settings determine if the printer will be disabled, or the entire line will stop when an error occurs: If PrinterErrConfig_DisableLineOnPrinterErr = N, the Line will not stop when an error occurs. If PrinterErrConfig_DisableLineOnPrinterErr = Y, the Line will stop when an error occurs. (PrintApplyLine_PrintLineCnvMtr must be selected to stop the line) If AutoErrHandling = N, the printer will not be disabled. If AutoErrHandling = Y, the printer will be disabled. The line will not stop unless all printers of the same label type are disabled. |
| TamperHomeIn | Popup | Select the IO input part for the tamper home signal. The signal indicates the tamp mechanism is in the home position. |
| LabelOnTampIn | Popup | Select the IO input part for the label-on-tamp (LOTAR) signal. The signal indicates a label has been fed from the printer and is waiting on the tamp head. |
| CycleCompleteIn | Popup | Select the IO input part for the cycle complete signal. The signal indicates the applicator has completed a tamp cycle. |
| PrinterResetOut | Popup | Select the IO output part for the printer reset signal. This is tied to the “Reset Printer” button on the operator’s screen. NOTE: This output must be owned by SORT. |
| TamperSolOut | Popup | Select the IO output part for the printer tamp signal. This is used to fire the tamper when a label is to be applied. NOTE: This output must be owned by SORT. |
| PrinterErrLight | Popup | Select the IO output part used to indicate a printer error has occurred. This is usually connected to a beacon light to provide a visual indication of an error and summon the operator for corrective action. The line will automatically stop by disabling PrintApplyLine_PrintLineCnvMtr if PrinterErrConfig_DisableLineOnPrinterErr = Y. This signal could come on for the following reasons: - Printer Error IO signal is ON (PrinterErrorIn) - Printer has not completed a prior tamp (busy error) - Cycle timer for a “Send Label”, “Wait Label Feed”, or “Wait Host Message” Type in the PrinterUpdPnts table has timed out. - A verification error has occurred - A tamp error has occurred (TamperHomeIn) - A TampCycleTimer or TampRetractTimer in the PrinterBoss table has timed out. - A cycle error has occurred (CycleCompleteIn) - A LOTAR error has occurred (LabelOnTampIn) |
| PrinterErrACTION | Popup | The action to take when a printer error occurs. The action is performed on the signal selected in PrinterErrLight. |
| PrinterErrBIT | Int | The bit to use for a PrinterErrACTION of Enable or Disable. |
| PrintBelt | Popup | Select the record from table PrintBelt for this printer. This is used to adjust the speed of the belt for special circumstances, such as label-over-label configurations. |
| DontCreateTmrs | Y/N | N – Timers will be automatically created for TampPulseTimer, TampCycleTimer, TampRetractTimer and ErrorResetTimer. Y – The timers are not automatically created, and will remain set to N/A. Use this for ink jet printer applications that must fire the ink jet for the length of the carton. |
| TampPulseTimer | Popup | This timer is used to determine the length of the pulse to trigger a tamp cycle. Set the timer to a value that is large enough to create a pulse signal that can be reliably detected by the applicator. 50 mSec is a good value. |
| TampBoxLength | Radio | OFF – The tamp signal will be held ON only for the duration of the TampPulseTimer. ON – The tamp signal will be activated for the entire box length. This is for use with ink jet printers. TampBoxLength is combined with TamperHomeIn and TampCycleTimer to create different behaviors for the Tamp solenoid. - If TampBoxLength is ON and TampHomeIn is not selected, and TampCycleTimer is not selected, TampSolOut will fire for the entire box length (not including trailing shoes). - If TampBoxLength is ON and TampHomeIn is selected, and TampCycleTimer is not selected, TampSolOut will fire until TampHomeIn is toggled OFF and ON. |
| TampBoxMargin | Int | Enter the number of ticks to delay the fire of the tamp signal. This is used with TampBoxLength on ink jet applications to start the printing in from the edge of the box. |
| TampCycleTimer | Popup | This timer is used to put a time limit on the entire print and tamp cycle so tamp cycle errors can be detected. Set the timer to a value that is large enough to allow for the completion of a normal tamp cycle. 2 seconds is a good choice. |
| TampRetractTimer | Popup | This timer is used to detect when the tamper has not returned to its home position. Set the timer to a value that is large enough to allow the tamp mechanism to extend and retract. 2 seconds is a good choice. |
| ErrorResetTimer | Popup | This timer determines the length of the pulse to trigger a reset cycle. Set the timer to a value that is large enough to create a pulse signal that can be reliably detected by the applicator. 50 mSec is a good value. |
| MinLength | Int | The minimum length of a carton, in encoder ticks. This is used to filter out phantom cartons and real cartons that should not be processed because they are too short. 4 ticks is a good value. |
| MinHeight | Float | Set the minimum height carton allowed for this applicator. Example: A side-apply applicator is applying a label that is 6 inches tall, so the carton must be at least 6 inches tall. Set MinHeight = 6.0. The round-robin assignment algorithm will not assign the applicator for a carton that is too short. Used in applications where both a top and side applicator are applying the same label type. |
| PreferredPrinter | Y/N | N – All printers of the same label type in a round-robin configuration will be considered equally for each label. Y – If this printer is included in a round-robin configuration it can be considered first for each label. Example: The side-applicator printer from the previous example needs to be selected if the carton is greater than 6 inches tall, and if the printer is online. MinHeight is set to 6 inches, as before, and PreferredPrinter is set to Y. |
| HostPrinterName | String | An agreed-upon string that serves as the name of this printer for the PNA Engine. This name will be used in DSR messages if the PNA Engine requests the status of the printer. This is only used in applications where the PNA Engine can enable and disable printers. |
| DSRRequest | PopUp | For systems designed to allow the PNA Engine to control the printer status. This field links the printer to the DSR message configuration found in table DSRConfig. Each DSR message may contain status information for up to 10 printers. If there are more than 10 printers on a single line, additional DSR messages must be configured to split them up. Note: The printer status is changed in 2 steps. When the “Disable” message is received from the PNA Engine, the printer is no longer being assigned labels. After the timeout found in the DSR message configuration, the printer “Enable” is turned OFF. Once that occurs, the printer will not stop the line if a printer fault occurs. |
| pPrintFlag | Y/N | Set by the GUI when an operator manually triggers a label to be printed. |
| pTampFlag | Int | Used internally to activate a tamp cycle. |
| pTampMnuIndx | Int | Used internally while processing a tamp request. |
| pTampTrkIndx | Int | Used internally while processing a tamp request. |
| pTampLblNum | Int | Used internally while processing a tamp request. |
| pErrorFlag | Int | Used internally to indicate a printer error has occurred. |
| pResetState | Int | Used internally to indicate a printer reset is in progress. |
| pResetFault | Int | Used internally to process a fault reset. |
| pLastEncCnt | Int | Used internally to track encoder ticks. |
| pPrinterErrLight | Int | Used internally to indicate the state of the printer error light. |
| pDisableDelay | Int | Used internally to delay the disable of the printer in conjunction with DSR messages. |
| pTampFault | Int | Used internally to indicate a tamp fault. |
| LogLOTAR | Radio | Activate this to turn on LOTAR logging. |
| pLOTARState | Int | Used internally to indicate the LOTAR signal state. |
| pLOTARFail | Int | Used internally to indicate the LOTAR has failed. |
| pBoxEndTicks | Int | Used internally to indicate the number of ticks left on the current box. |
| pErrOneShot | Int | Used internally while clearing printer errors. |
| pErrHgtRndRbn | Int | User internally for checking minimum height requirements while selecting a printer from the round-robin pool. |

  - Table PrinterErrConfig

The PrinterErrConfig table is used to define how a printer will behave when errors occur.  In systems with redundant printers, the line may be configured to automatically take printers offline when errors occur.  As long as one printer is available for each label type, the line will continue to run.  This allows the printer error to be corrected while the line continues to operate.

If PrinterErrConfig_Enable = N (manually set), the line may continue to run even if the printer has an active error.

| PrinterErrConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for each printer error record. |
| Desc | String | Optional descriptive text for the record. |
| Enable | Y/N | Y – Enable printer error processing N – Disable printer error processing. The line may continue to run even if the printer has an active error. pConsecVerifyErrs is set to 0. |
| Printer | PopUp | Select the printer associated with this error configuration row. |
| UpdateHistory | Y/N | Y – Update the history record selected in HRRecord. |
| HRRecord | PopUp | Select a history record if data needs to be collected for this error event. |
| AutoResetCons | Y/N | Y – The error count will be reset when a good cycle of the printer occurs. N – An error state will persist until manually reset. |
| MaximumVerifyErrs | Int | Enter the number of consecutive verification errors at which the printer is taken offline. |
| DisableOnMaxVerifyErrs | Y/N | Y – Disable this printer when MaximumVerifyErrs has been reached. N – Do not disable the printer. |
| DisableLineOnPrinterErr | Y/N | Y – Disable the line when this printer has an error. N – If PrintApplyLine_AutoErrHandling = Y and PrintApplyLine_NoPrintersError = “Shutdown Line”, the line will stop when all printers of this label type are disabled. If PrintApplyLine_AutoErrHandling = N the line will not stop! Printer errors: - A verification error while DisableOnMaxVerifyErrs = Y - PrinterBoss_PrinterErrorIn = ON - The printer applicator cycle fails due to an issue with PrinterBoss_TamperHomeIn, PrinterBoss_LabelOnTampIn, or PrinterBoss_CycleCompleteIn. This field is tied to the “Halt Line on Printer Error” button on the operator’s screen (and PrintApplyScrn_Stop_Belt_on_Error in older versions). |
| DisableOnLOTARFail | Y/N | Y – Disable this printer when a LOTAR error occurs (PrinterBoss_LabelOnTampIn) at the applicator fire point. N – Do not disable the printer at the applicator fire point. The printer could still be disabled at the “Wait Label Feed” update point, if configured, and PrintApplyLine_AutoErrHandling = Y. A LOTAR error occurs when the carton reaches the tamp point and the LOTAR signal is not ON. This is more likely to occur if no “Wait Label Feed” update point is defined. It can also be caused by belt drift if the line is stopped as the carton reaches the wait point. |
| DontClearIfSent | Y/N | This field controls the behavior when resetting the line after an error has caused it to stop. Y – Choose this option if cartons are not going to be physically cleared from the belt if they have reached state “Sent”, “Labeled” or “Tamped”. Cartons in state “Sent” or “Labeled” are set to a status of “Tamped” when the “Clear All” button is pressed. The operator must make sure any available labels (check the tamp head, and manually feed any in the printer’s buffer) are manually applied to the correct carton. Only cartons that do not have all their labels and cannot be labeled manually must be removed from the belt. N – All cartons must be physically removed from the line between the induct photoeye and the verification scanner. They can be re-introduced to the line prior to the induct photoeye once the error has been corrected. Be sure to remove any labels that were applied before re-introducing cartons. The “Clear” and “Clear All” buttons will remove all logical cartons from the line. If PrintApplyLine_AutoErrHandling = Y (a.k.a., “Auto Disable Printer(s)” is set on the operator’s screen), the label status is cleared when a printer goes offline, and the label has not been tamped. If the label has been tamped, it will NOT be logically removed. This allows labeled cartons to be verified. Since cartons are logically cleared when a printer fails, this only affects line stops when a dimension error occurs. |
| ClearLabelStatus | Y/N | Y – The label status will be set to “Error” when the “Clear All” button is pushed on the operator’s screen, allowing the carton to continue through the line. This is used in applications where dimensional errors can stop the line. The “Error” status will cause the carton to go to the exception handling destination downstream, or it can be manually removed before the line is restarted. “Clear All” will also cause the system to set the “Exempt” and “Pass Through” status, allowing cartons to proceed through the dimensional check. Since cartons are logically cleared when a printer fails, this only applies to line stops when a dimension error occurs. |
| DontClearVerifyErr | Y/N | Y – If this is set, and DisableOnMaxVerifyErrs = Y the line will stop on a verification error, but the tracking record will not be cleared. This provides the ability to use “Reset All” and “Reset Fault” to recover without removing all the cartons if the label is found to be OK or the error can be corrected without moving the carton. For any other problem, the line must be cleared physically and “Reset All”, “Reset Fault” and “Clear All” used to clear it logically. The removed cartons can be re-introduced before the induct photoeye after the problem is resolved. Note: Clicking “Clear All” causes the tracking record to be cleared. N – The tracking record is cleared when a verification error occurs. This allows the carton to continue to the exception lane. The line does not stop. |
| ErrorIO | PopUp | Select an IO part to be activated when the verify count reaches the MaximumVerifyErrs value. This could be used to light a beacon. |
| ErrorACTION | PopUp | The action to take when a MaximumVerifyErrs error occurs. The action is performed on the signal selected in ErrorIO. |
| ErrorBIT | Int | The bit to use for an ErrorACTION of Enable or Disable. |
| ErrorSignal | PopUp | Select a memory bit that will be activated when this error occurs. This could be used to activate an error from the Errors table. |
| ResetCons | Y/N | Set this to Y manually to reset the consecutive errors count. pConsecVerifyErrs will be set to 0 and ResetCons will change back to N. |
| ResetConsSignal | PopUp | Select an input IO signal if you wish to reset pConsecVerifyErrs using that signal. When the signal comes ON, the error count will be reset to 0. |
| ResetSeconds | Int | Set the number of seconds to wait before resetting pConsecVerifyErrs. If set to 0, it will not be reset based on time. It looks to me like this is mis-named. It is not in seconds, but in number of passes through the CTRL process, which occur every 10 mSec – TRK. |
| pConsecVerifyErrs | Int | Used internally to keep a count of the consecutive verification errors. |
| pResetSecsLeft | Int | Used internally with ResetSeconds to determine the time remaining until pConsecVerifyErrs is reset. |

  - Other Print Apply Error Detection Options

The rest of the FPC error handling capabilities outside of the PNA module are also available for use.  There are many configurable options.  Here are some of the commonly used ones:

- LPN/SKU/Scale/Dimension Errors

  - These errors are processed in the scanner module and in the print apply module.  Each scan device and each type of scan error can be configured with individual errors and statistics and can activate a beacon.

    - To configure the line to stop on a scan error, add an entry for the scanner or scale in table ScnDeviceErrIO (Configuration Menu | Scanner Utilities | Device Err Config).

    - The PNA Engine can also be configured to send a special error label that is placed on the carton indicating the type of error.

  - Dimension Errors are processed in table HeightWidth (Configuration Menu | PrintApply Utilities | Height Width Config).  A range can be defined for each dimension to stop the line for “out of range” conditions.

- Messaging Errors

  - LRM, LVM and PNA Engine communication errors are processed with the SrtHrtBeat table (Configuration Menu | Communication Utilities | COMM HeartBeat), which can monitor the send message tables, and activate an error, or stop the line when a send error occurs.

  - The SrtHrtBeat table can also exchange RHS messages, which can include the status of the WMS and the link to it.  An error can be activated, or the line can be stopped.  Create a link to the SrtHrtBeat RHS message row in PrintApplyLine_HostCommFault to detect host communication errors.  The PNA Engine must be configured to accept these messages.

- Label Errors

  - If the PNA Engine does not send a carton label, the error is detected at the Label Send update point.  The PrinterUpdPnts table is used to configure a timer to determine when an error needs to be triggered.

  - If the PNA Engine does not have information for the carton, or if it does not have a label for the carton, the LRM response message will contain the error description.  The returned error description can be used to trigger an error.  These errors are configured in table ScnZoneErrIO (Configuration Menu | Scanner Utilities | Zone Err Config).  Specific error strings are identified in the ScnZoneErrIO_CommErrorMsg column.  Select the print line in the ScnZoneErrIO_PrintLine column to stop the line.

- Printer Errors

  - If the applicator fails to feed a label due to a label format issue or internal printer issue, the error is detected at the “Wait Label Feed” update point.  The PrinterUpdPnts table can be configured with a timer to determine when an error needs to be triggered.

    - Option 1 – PrintApplyLine_AutoErrHandling = N (a.k.a., “Auto Disable Printer(s)” is OFF).  The line will stop if PrintApplyLine_PrintLineCnvMtr column is configured.

    - Option 2 – PrintApplyLine_AutoErrHandling = Y (a.k.a., “Auto Disable Printer(s)” is ON).  The printer will be disabled but the line will not stop.  To make the line stop, set PrinterErrConfig_DisableLineOnPrinterErr = Y.  Alternatively, if this is the only printer error that will stop the line, you could add the print apply line motor to the PrinterUpdPnts_ErrorSignal column.

  - If the applicator detects an internal error and activates the PrinterBoss_PrinterErrorIn signal, the printer can be disabled, or the line can be stopped.

  - If the applicator detects a tamper error or a cycle error, the printer can be disabled, or the line can be stopped.  PrinterBoss_TamperHomeIn and PrinterBoss_TampRetractTimer are used to detect tamper errors.  PrinterBoss_CycleCompleteIn and PrinterBoss_TampCycleTimer are used to detect cycle errors.

- Verify Errors

  - After applying each label, the carton is scanned and the LRM data is used by the PNA AC to verify the label is correct.  Two types of verification errors can be detected:

    - Mismatch errors – wrong label applied.

    - Scanner errors – the label cannot be read.

  - Table PrinterUpdPnts

The PrinterUpdPnts table defines important points along the print belt where events will occur related to interaction with the PNA Engine, the scanners, and the printers.

| PrinterUpdPnts Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the update point. |
| Status | String | Set by the program to indicate errors or events associated with the update point. |
| Type | PopUp | Height Update – Determines when the height information for a carton will be recorded. The height is used to adjust the tamp point for top-apply labels for improved label placement accuracy. Note: If a Height Update point is not defined, the tamp point will be assigned to the Small update point. See sensor calibration information in section 0 for more detail. The printer tracking record (in tables PrtTrkTable[1-5]) is updated with the tamp index: - TmpIndxG1 – Label tamp point for the first label if only one label is being applied by this printer. - TmpIndxG2 – Label tamp point for the first Label if two labels are being applied by this printer. - TmpIndxG3 – Label tamp point for the second label if two labels are being applied by this printer. |
| Type, cont. | PopUp | Width Update – Determines when the width information for a carton will be recorded. The width is used to adjust the tamp point for side-apply labels when the carton will be justified to the side of the conveyor away from the applicator. Note: If a Width Update point is not defined, the tamp point will be assigned to the Small update point. See sensor calibration information in section 0 for more detail. |
| Type, cont. | PopUp | Send Label – Defines when to send the label image data to the printer. It will also cause the line to stop and wait for the label data to be received from the PNA Engine, if necessary. If PrintApplyLine_SendType = “Upd Pnt – Lbl Snd”, this update point must be defined for each Printer. If PrintApplyLine_SendType = “Scan – Lbl Snd”, Send Label update points are not required, but if they are not defined, the label will be sent to the printer queue immediately upon receipt, which is usually not desirable. Adjust the Send Label point to a location between the scan point and the Wait Label Feed point. The line will stop and wait for the LRM response and LMI message – if a label is required. Once the label is received, it will be sent to the printer and the carton will move on to the Wait Label Feed point. NOTE: The Send Label update point must be adjusted past the scan update point for the longest carton to ensure the conveyor does not stop before the scanner data is received (NoScan and NoRead events do not occur until the carton tails out of the scanner photoeye and the scanner update point). |
| Type, cont. | PopUp | Wait Host Message – This type is used in applications where the label is sent directly to the printer by another system. When the label has been sent, the PNA Engine will send a message to indicate the container may proceed to the tamp point. Adjust the point to a location between the scan point and the Wait Label Feed point. Once the message is received, the carton will move on to the Wait Label Feed point. |
| Type, cont. | PopUp | Wait Label Feed – This type will cause a carton to wait until the label has been fed onto the tamp head, including waiting for the PNA Engine to respond to the LRM message if necessary. An input from the applicator, configured in PrinterBoss_LabelOnTampIn, is required. Adjust the point position to a location between the scan point and the Small tamp point. It should be far enough ahead of the tamp point to allow the belt to return to full speed before the applicator cycles. This helps to achieve the most consistent label placement. The PrintBelt and SpdControl tables must be configured, and the PrintBelt record must be selected in PrinterBoss_PrintBelt. Note: To verify the Wait Label Feed point is set properly, unplug the network cable for the PNA Engine connection, and verify the carton stops at least 12 ticks or 0.5 seconds prior to the Small tamp point. |
| Type, cont. | PopUp | Tamp (1 Label) Small – This type is used to define the tamp location for the smallest carton when only one label is being applied to a container. Smallest means shortest for top-apply, or narrowest for side-apply. For side-apply with the cartons justified to the same side as the applicator, the distance will always be the same regardless of carton width. |
| Type, cont. | PopUp | Tamp (1 Label) Large – This type is used to define the tamp location for the largest carton when only one label is being applied to a container. Largest means tallest for top-apply, or widest for side-apply. For side-apply with the cartons justified to the same side as the applicator, the distance will always be the same regardless of carton width. |
| Type, cont. | PopUp | Tamp (2 Labels) Small – This type is used to define the tamp location for the smallest carton for both the first and second label when two labels are being applied to a container. The LabelNum field will determine which label this point refers to. Each LabelNum will need its own update point of this type. Note: If needed, configure DelayTimer to allow the belt to reach full speed after the second label has been tamped. |
| Type, cont. | PopUp | Tamp (2 Labels) Large – This type is used to define the tamp location for the largest carton for both the first and second label when two labels are being applied to a container. The LabelNum field will determine which label this point refers to. Each LabelNum will need its own update point of this type. |
| Type, cont. | PopUp | Gather Errors – This type is used to capture scanner errors and errors found in the tracking tables (PrtTrkTable[1-5]). They are written to the printer error tracking tables (PrtTrkErrTbl[1-5]). The table is then sorted by TimeStamp to create a chronological list of errors for cartons flowing on the print belt. Select the tables in the ScnScanZone table, PrinterTrkTable and PrinterErrTable fields. |
| Type, cont. | PopUp | PNA Zone Clear – This type is used to define an endpoint on the belt where the tracking records will be cleared. This feature is used in applications where the line is stopped for applicator errors or for length, width, or height errors. The “Clear All” function can be configured to set the carton status to “Exempt” or “Pass Through” to allow the belt to run without halting after erroneous cartons have been removed. Older versions: The identity of the last carton cleared is copied to “Last Clear Point” on PrintApplScrn. |
| Type, cont. | PopUp | PNA Start Zone Clear – This type is used to define a start point on the belt where the tracking records will be cleared. This feature is used in applications where line is stopped for applicator errors or for length, width, or height errors. The “Clear All” function can be configured to set the carton status to “Exempt” or “Pass Through” to allow the belt to run without halting after erroneous cartons have been removed. |
| Type, cont. | PopUp | Adjust Tamp Using Length – This type is used to change the tamp point based on the carton length based on the configuration in the HeightWidth table. The carton length, in ticks, is divided by HeightWidth_LengthTicksPerInch to get a carton length in inches. This value is compared with HeightWidth_ChkLengthForAdjInches. If the carton is longer than the value specified, the tamp point is adjusted by converting HeightWidth_TampAdjInches to ticks and adding that value to the previously-calculated tamp point value, which will cause the label to be applied closer to the nose of the box. Note that the adjustment value will appear in PrinterUpdPnts_Status with the text “Adjusting Tamp” and in the log files with “PrintApplyLengthOffsetTicksUpdatePoint… Offset Ticks”. |
| PrintLine | PopUp | Choose the print apply line associated with the Printer. |
| Printer | PopUp | Choose the printer associated with this update point. |
| LabelNum | PopUp | Choose the label number associated with this update point. If only one label will be applied, choose “Label 1” |
| UpdatePnt | PopUp | Choose the update point from table Updates that defines the location for the event trigger. |
| DelayTimer | PopUp | This is used to define the delay time before the event is completed. Example: When applying two labels, to wait for 0.5 seconds after applying the second label before returning to full speed, set the delay timer to 0.5 seconds. |
| CycleTimer | PopUp | Use this timer to define how long to wait after an error event stops the line before activating a printer error. If no timer is selected here, error events will cause the line to stop and wait for the operator to click “Reset Fault”. If ErrorSignal is defined it will be turned ON. If PrintApplyLine_AutoErrHandling = Y the printer will be disabled. If PrinterErrConfig_DisableLineOnPrinterErr = Y, the line will stop, and the error will be displayed on the operator’s screen. Entries for “Send Label”, “Wait Label Feed”, and “Wait Host Message” Types require an entry here. Entries for all other Types do not stop the conveyor and do not require a CycleTimer. |
| ErrorSignal | PopUp | An IO output signal selected here will be turned ON when the error condition occurs. The output will turn ON when the CycleTimer expires, if defined, and will be turned OFF when the operator clicks “Reset Fault”. |
| ErrorACTION | PopUp | The action to take when an error occurs. The action is performed on the signal selected in ErrorSignal. |
| ErrorBIT | Int | The bit to use for an ErrorACTION of Enable or Disable. |
| CancelTampIfErr | PopUp | Select the action to take when CycleTimer times out. This only affects update points that stop the line. - Select “N/A” to allow the system to continue without changing the label apply status. The printer error will be set. Depending on the printer error settings this may stop the line. - Select “Tamp Only” to cancel the label apply process for one label and assign an exception handling lane, chosen in the CancelErrLane field. The printer error will be set. - Select “Cancel All” to cancel the label apply process for one carton and to cancel the verify process as well. “Cancel All” will cancel the verify process for any additional labels on this carton as well. The printer error will be set. - Selecting “Cancel Label – Enable Printer” will set the status of the label to “Upd Error”. The printer error will not be set. If a “Send Label” update point is defined and the PNA Engine does not respond with a LRM or LMI message in the required time, the carton will continue on without a label, probably to the error correction destination. |
| CancelErrLane | PopUp | Select a row from the Outpoints table to be assigned as the carton destination if an error occurs and the CancelTampIfErr field is set to Y. |
| pProcFlag | Int | Used internally |
| pCurrSpeed1 | Int | Used internally |
| pCurrSpeed2 | Int | Used internally |
| pMnuIndx | Int | Used internally |
| pTrkIndx | Int | Used internally |
| pErrorFlag | Int | Used internally |

  - Table PrintBelt

The PrintBelt table is used to define the behavior of and current state of the conveyor belt for print apply lines.

| PrintBelt Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for the print apply belt |
| Error | String | The software may display error messages to do with the belt here. |
| EnableFlag | Y/N | Each print belt can be enabled or disabled using this field. |
| Sorter | PopUp | Choose the sorter associated with this print apply line. |
| SlowSpeed | Int | For use in applications where a single printer applies two labels to the same carton. The belt will slow to this speed to apply the first label. The belt will return to MaxSpeed after the second label has been applied. This value is expressed as a percentage of the belt’s mechanical full speed. |
| MaxSpeed | Int | Set to 100. For use in applications where a single printer applies two labels to the same carton. The maximum belt speed allowed while a label is being applied, as a percentage of the belt’s full mechanical speed. This value sets an upper logical limit on the belt speed, but other conditions may require the use of SlowSpeed or WaitRdySpeed. This value will also be used when there are no cartons on the belt prior to the dump point. |
| WaitRdySpeed | Int | Set to 0. The belt is set to this speed when the printer is not ready. This value is expressed as a percentage of the belt’s mechanical full speed. If the belt is to be controlled by an input signal, use PrintApplyLine_PrintLineSpdCtrl, and set up a record in table SpdControl that references the print line in SpdControl_PrintLine. This will ensure that the motor is stopped when the speed is set to zero. |
| pCalcSpeed | Int | The current expected belt speed, as a percentage, calculated from SlowSpeed, MaxSpeed and WaitRdySpeed. |
| pTtlCont | Int | The number of containers currently present on the belt. |
| pFinalSpeed | Int | The belt speed that is actually set by the software after considering all containers currently on the belt, their label requirements and the applicator status. Expressed as a percentage of the belt’s full speed. |

  - Table HeightWidth

The HeightWidth table is used to adjust tamp points based on the size of the carton.  The tamp point is determined by linear interpolation of the height or width.  Several measurement methods are supported.  The dimensions of the carton are supplied to the PNA Engine through the sHeight, sWidth and sLength fields of the LRM message and may be used there to determine how the carton will be labeled.  For example, a carton that is not tall enough for a side-apply label may be labeled with top-apply instead.

NOTE:  A record must be created in the HeightWidth table even if height and width sensors are not used.  The minimal configuration requires Name, SensorType and PrintLine.

| HeightWidth Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the row. |
| Status | String | The software will show errors or events associated with this row. |
| SensorType | PopUp | Choose the sensor type for the height or width measurements. - Photoeye – Two photoeyes are used to select a low, medium, or high box height and set the tamp location to “Tamp Small”, halfway between or, “Tamp Large”, respectively. Alternatively, a single height photoeye can also be used, resulting in two possible tamp locations. - Ultrasonic – An ultrasonic or laser sensor value is used to compute a tamp point anywhere between “Tamp Small” and “Tamp Large”. - Scanner Data – Uses data from a scanner that returns dimension information in a compound scan to define the length, width, and height of the carton. The values are used to compute a tamp point anywhere between “Tamp Small” and “Tamp Large”. - Copy Length To Height – The carton length, in ticks, and LengthTicksPerInch are used to compute carton height, in inches, which is then used as a height. MinHeightInches and MaxHeightInches can be used to put bounds on the resulting tamp point, and ScalingFactor can be used to adjust the resulting height. This may be a good choice if hardware is not available for the other options. |
| PrintLine | PopUp | Choose the print and apply line associated with this entry. |
| BoxDetectIn | PopUp | Not currently used. |
| TooShortIn | PopUp | Not currently used. However, if a signal is defined, its state will be recorded in the TooShort field of the tracking record. It was originally intended to detect boxes that are too short to receive a side-apply label. |
| Height1In | PopUp | For use with the “Photoeye” SensorType. Select an input signal to indicate the small end of the height range. Cartons that do not trigger this sensor will use the “Tamp Small” tamp point location. Cartons that trigger this sensor, but not the Height2In sensor will be tamped at a point halfway between “Tamp Small” and “Tamp Large” tamp point locations. |
| Height2In | PopUp | For use with the “Photoeye” SensorType. Select an input signal to indicate the large end of the height range. Cartons that trigger this sensor will use the “Tamp Large” tamp point location. |
| HeightInDT | Int | For use with the “Ultrasonic” (or laser) SensorType. This is the IO address word where the sensor will store the value it detects. The height data value uses a full 16-bit word. The value is expected to be in decimal notation. If your value is in octal, you can easily convert it: Find the word in the “View I/O” screen, then toggle the “Octal Mode” button to see the row numbers in decimal notation. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. Note that the word address in decimal notation is also the row number in table FORTNADT, where the bit values, forces and invert states are stored. |
| BoxHgtInches1 | Float | Enter the box height, in inches, for small cartons. Used for SensorType “Photoeye” and “Ultrasonic”. See section 0. |
| SensorHgt1 | Int | For use with the “Ultrasonic” (or laser) SensorType. Enter the height sensor reading for small cartons. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. |
| BoxHgtInches2 | Float | Enter the box height, in inches, for large cartons. Used for SensorType “Photoeye” and “Ultrasonic”. See section 0. |
| SensorHgt2 | Int | For use with the “Ultrasonic” (or laser) SensorType. Enter the height sensor reading for large cartons. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. |
| WidthInDT | Int | For use with the “Ultrasonic” (or laser) SensorType. This is the IO address word where the sensor will store the value it detects. The width data value uses a full 16-bit word. The value is expected to be in decimal notation. If your value is in octal, you can easily convert it: Find the word in the “View I/O” screen, then toggle the “Octal Mode” button to see the row numbers in decimal notation. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. Note that the word address in decimal notation is also the row number in table FORTNADT, where the bit values, forces and invert states are stored. |
| BoxWdtInches1 | Float | Enter the box width, in inches, for small cartons. Used for SensorType “Photoeye” and “Ultrasonic”. See section 0. |
| SensorWdt1 | Int | For use with the “Ultrasonic” (or laser) SensorType. Enter the width sensor reading for small cartons. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. |
| BoxWdtInches2 | Float | Enter the box width, in inches, for large cartons. Used for SensorType “Photoeye” and “Ultrasonic”. See section 0. |
| SensorWdt2 | Int | For use with the “Ultrasonic” (or laser) SensorType. Enter the width sensor reading for large cartons. You can use the calibration tool (Configuration Menu ¦ Wizard Utilities ¦ PNA Height Calibration) to set this value. See section 0. |
| LengthTicksPerInch | Float | For use with the “Copy Length To Height” SensorType. Enter the ticks-per-inch value for the conveyor belt encoder. Used to compute a height estimate for the carton based on its length. Will default to 2.0 if not set. Also requires MinHeightInches and MaxHeightInches to be set. |
| ScalingFactor | Float | This value is used to scale the height and width values. The values are divided by this factor. |
| ValidateHeight | Y/N | Y – The height data received from the scanner is checked against the range specified by MinHeightInches and MaxHeightInches. A dimension error will be generated if the value is out of range. N – No range check is performed on height. |
| ValidateWidth | Y/N | Y – The width data received from the scanner is checked against the range specified by MinWidthInches and MaxWidthInches. A dimension error will be generated if the value is out of range. N – No range check is performed on width. |
| MinHeightInches | Float | Set the minimum height, in inches, for use with ValidateHeight. Also required for the “Copy Length To Height” SensorType. |
| MaxHeightInches | Float | Set the maximum height, in inches, for use with ValidateHeight. Also required for the “Copy Length To Height” SensorType. |
| MinWidthInches | Float | Set the minimum width, in inches, for use with ValidateWidth. |
| MaxWidthInches | Float | Set the maximum width, in inches, for use with ValidateWidth. |
| LPNScanZone | PopUp | If height and width data are provided by a scanner, the min and max range check requires the carton identity scan zone be selected here. |
| VerifyScanZone | PopUp | If height and width data are provided by a scanner, the min and max range check requires the verification scan zone be selected here. |
| ChkLengthForAdjInches | Int | Used with “Adjust Tamp Using Length” update points, as defined in table PrinterUpdPnts_Type. A tamp point adjustment is made if the carton length is greater than the value entered here, in inches. See section 4.6 for details. |
| TampAdjInches | Int | Used with “Adjust Tamp Using Length” update points, as defined in table PrinterUpdPnts_Type. If a tamp point adjustment is needed, the tamp point is advanced from its normal position by the value entered here, in inches. See section 4.6 for details. |
| pLastHeightInches | Float | The last height processed by the software, converted to inches. |
| pLastSensorHgt | Int | The last height processed by the software in the original binary format received from the height sensor. |
| pLastWidthInches | Float | The last width processed by the software, converted to inches. |
| pLastSensorWdt | Int | The last width processed by the software in the original binary format received from the width sensor. |
| pLastHeightPercent | Int | Used with “Adjust Tamp Using Length” update points. The last height processed by the software, as a percentage of the height between MinHeightInches and MaxHeightInches. |
| pLastLengthInches | Float | The last length processed by the software, converted to Inches. |
| pLastWidthPercent | Int | The last width processed by the software, converted to a percentage. |

    - Example Configuration:  Photoeye Height Detection

  - Choose a SensorType of “Photoeye”.

  - Choose the sensor for the lowest photoeye in Height1In.

  - Choose the sensor for the highest photoeye in Height2In.

  - Enter the height, in inches, for the tallest carton that does not activate the Height1In sensor in BoxHgtInches1.

  - Enter the height, in inches, for the box that is just tall enough to activate the Height2In sensor in BoxHgtInches2.

The calculated height is stored in the pLastHeightInches field and the tracking record Height field.

    - Example Configuration:  Ultrasonic Height Detection

  - Choose a SensorType of “Ultrasonic”.

  - In HeightInDT, enter the IO input word value, in decimal notation, for the height value from the sensor.

  - Enter the height, in inches, for the smallest carton in BoxHgtInches1.

  - Enter the value read from the height sensor when the smallest carton is under the sensor in SensorHgt1.

  - Enter the height, in inches, for the tallest carton in BoxHgtInches2.

  - Enter the value read from the height sensor when the tallest carton is under the sensor in SensorHgt2.

The value read from the sensor is stored in the pLastSensorHgt field.  The calculated height, in inches, is stored in the pLastHeightInches field and the tracking record Height field.

    - Example Configuration:  Scan Tunnel

      - Choose a SensorType of “Scanner Data”.

      - Enter a ScalingFactor value.  If no scaling is needed, enter 1.0.

      - Enter the height, in inches, for the smallest carton in BoxHgtInches1.

      - Enter the height, in inches, for the tallest carton in BoxHgtInches2.

      - Select the scan zone associated with this record in LPNScanZone.

      - If height range checking is needed, set values in MinHeightInches, MaxHeightInches and set ValidateHeight = Y.

The calculated height is stored in field pLastHeightInches and in the tracking record Height field.

    - Example Configuration:  Adjust Tamp Using Length

Induct point

Longest carton tailout point

Smallest carton tamp point

“Adjust Tamp Using Length” update point

“Height Sensor” update point

      - Choose a SensorType of “Adjust Tamp Using Length”.

      - Enter the ticks-per-inch value in LengthTicksPerInch.

      - Enter the minimum carton length, in inches, that should trigger a tamp adjustment in ChkLengthForAdjInches.

      - Enter the length adjustment, in inches, in TampAdjInches.

      - Determine the location of the height sensor update point.

      - Determine the location of the tamp point for the smallest carton.

      - Determine the location where the longest carton tails out of the induct point.

      - Set the position of the “Adjust Tamp Using Length” update point using the graphical shifter.  Use the illustration above as a guide.  The “Adjust Tamp Using Length” point must be downstream of the height sensor point and the longest carton tailout point, and upstream of the tamp point for the shortest carton.

  - Table PAScannerBoss

The PAScannerBoss table is used to define scanners for the print apply line.  Scanners are used to read an LPN or SKU to identify the carton.  Scanners are also used to verify applied RFID tags, SKUs or up to six labels on a single carton.

The normal scan processing configuration tables are used to define the basic function of the scanners:  ScnScanZone, ScnZoneErrIO, ScnScanDevice and ScnDeviceErrIO.  See the FPC-Scanner-Control-Configuration document for details.

Each print apply line will have at least two scan zones – one for identification of the carton (LPN or SKU), and one for verification of applied labels.  For label-over-label configurations, additional scan zones are needed.

A verify scan zone may have multiple scanners that verify specific labels.  Verification scanner error statistics are recorded slightly differently for a print apply line.  Scan errors are recorded as “Bad Read”.  An error will be recorded only if the carton has a label requirement and a verification requirement.  If multiple labels are required for a carton, only the first error found will be counted.  This prevents double-counting for labels that have a dependency on a prior label (i.e., label-over-label applications).

NOTE:  The operator’s screen will indicate a verify error if ScnZoneErrIO_PrintLine or ScnDeviceErrIO_PrintLine is defined.

| PAScannerBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the scanner. |
| ScanStatus | String | The program will display error or event messages here. |
| PrintLine | PopUp | Choose the print apply line associated with this scanner. |
| SendLVM | PopUp | NO LVM Msg (or N/A) – No message will be sent to the host when this label is verified. Used in label-over-label applications. Send LVM Msg – A message will be sent to the host when the label is verified. The message is sent with a status for each label in the scan zone. The status indicates if the label matched or if an error occurred. The LVM sStatus[1-6] fields will contain a code number indicating the verification status 0 – Label has not been verified 1 – Label verified successfully 2 – Label mismatch 3 – Verify scan error 4 – Not verified due to prior verify failure |
| VerifyType | PopUp | N/A – The label will not be verified, even if the verify update point is configured. Verify One – Used in label-over-label applications. Only this label will be checked. If it does not match, a “Bad Read” error will be recorded in the scanner statistics (if configured in ScnZoneErrIO or ScnDeviceErrIO). Downstream labels will not be applied. The SendLVM setting determines if an LVM is sent. Verify All – All the labels in the scan zone will be checked. If all the labels have been received, and if an error occurred on any label, a “Bad Read” error will be recorded (if configured in ScnZoneErrIO or ScnDeviceErrIO). Multiple (or all) scanners in the scan zone may be configured to “Verify All”, as nothing will be processed until all scans are received. The SendLVM setting determines if an LVM is sent. |
| ScanDevice | PopUp | Select the ScnScanDevice record associated with this scanner. In ScnScanDevice, select one of the following ScanTypes: LPN Scan – The scanner data will be recorded as LPN data. SKU Scan – The scanner data will be recorded as SKU data. Scan1, Scan2, Scan3, Scan4 – For use with an LPN scan with multiple scanners. A separate PAScannerBoss record must be created for each scanner. SKU1, SKU2, SKU3, SKU4 – For use with a SKU Scan with multiple scanners. A separate PAScannerBoss record must be created for each scanner. Verify Top, Left, Right, Front – Up to six scanners may be configured using these types. The data for each scan will be compared with the verification strings returned in the LRM message. Vrfy RF, Vrfy SKU – The scanner data will be compared with the verification string included in the LRM message. Scan – This is a special type used to load the sContainer field when a carton is scanned at a verification scanner. The Container ID is tracked from the first scan on a print apply line, however in some cases it must be updated with the scan received at the verify scanner. |
| pLane | Int | Used internally. Indicates the error correction lane assigned if a lane is selected in the ErrorLane field of table ScnZoneErrIO or ScnDeviceErrIO. |

  - Table PAPrinterScanAssign

The PAPrinterScanAssign table is used to link printers and labels to a specific verification scanner.  Multiple printers may be assigned to the same verification scanner in load-sharing configurations by creating multiple records here.

| PAPrinterScanAssign Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique description for the record. |
| Printer | PopUp | Choose the printer. |
| LabelNum | PopUp | Choose the label number. |
| PAScanRecord | PopUp | Choose the PAScannerBoss record associated with this printer and label. |

  - Table PStConvertTable (Downstream Sorter)

The PStConvertTable table makes it possible to transfer cartons logically from a print apply line to a downstream sorter without re-scanning.  The print apply tracking record (in PrtTrkTable[1-5]) is converted to a regular sorter tracking record by copying the scan data into the ScnScanDevice table and triggering a standard sorter scan event.  Use the same PrtTrkTable[1-5] for both the print apply and sorter configurations.  Configure the sorter in the usual way, except no Machine table or MsgMap table entries are required.

This feature originally was only able to pass along the LPN scan and could not be used with SKU-based systems.  Use the UseVerScan column to pass along a verification scan value instead.

Make sure the sorter Batch jobs are active!

| PStConvertTable Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this record. |
| Status | String | The program will display error or event messages here. |
| ScanDevice | PopUp | Choose the scan device that will be used to simulate a scanner. The carton scan data from the printer tracking record will be inserted to simulate a new scan. The standard scanner configuration will trigger the appropriate messages (CRR). |
| UpdatePt | PopUp | Choose the update point associated with ScanDevice. The action will happen when the carton reaches this point. |
| UseVerScan | PopUp | To use the scan value associated with a certain label, select the label definition record here. This can be used to pass the verification scan value along to the downstream sorter instead of the LPN or SKU scan. The value is taken from L1VerifyScan in the associated printer tracking table record. Leave this set to N/A to use the original behavior, which uses the ConfirmScan value (the LPN scan) from the tracking record. |

NOTE:  Prior to version 10.94, scan errors and exceptions processing were handled differently, and required additional configuration, as follows:

The print apply scan logic will convert a NoRead string to NOREAD, ReadConflict to READCONFLICT, NoScan to NOSCAN and BadRead to BADREAD, requiring special configuration in the ScnScanDevice table for these items:

  - NoReadString = NOREAD

  - ReadConflictStr = READCONFLICT

  - ChkLength = Y + QtyChars = 20.  Note:  This can be used to detect other errors, such as NOSCAN and BADREAD, but NOREAD will become BADREAD

  - Table TestConfig

The TestConfig table is used to define how the print apply line will behave while in test mode.  Test mode allows all aspects of the line to be tested without needing a connection to the PNA Engine.

| TestConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the testing record. |
| Status | String | The program will display error or event messages here. |
| PrintLine | PopUp | Choose the print apply line. |
| VerifyStrSKU | String | Enter a string here to compare with the verification string for PAScannerBoss_ScanDevice = “Vrfy SKU”, if needed. |
| VerifyStrRF | String | Enter a string here to compare with the verification string for PAScannerBoss_ScanDevice = “Vrfy RF”, if needed. |
| ExtLabelType1 | String | Use this if the line is configured for “Round Robin”. This string is used to determine the first label type for the round-robin algorithm. |
| ExtLabelType2 | String | Use this if the line is configured for “Round Robin”. This string is used to determine the second label type for the round-robin algorithm. This is needed only in applications where a single printer applies two labels to the same carton. |
| RtnStatusMsg | String | Used to define an error string that is returned with the LRM message. This can be used to verify the string generates the expected response. This string is compared with the string defined in CommErrorMsg in ScnZoneErrIO and ScnDeviceErrIO. If this string is found within the strings defined there, the error will be raised. The PATest protocol must be used for the test connection. It will generate a LRM response mimicking the PNA Engine that does not contain any assigned labels but does include this string. Disable the test labels in table LabelTemplates while using this feature. |

  - Table TestCnfgPrt

The TestCnfgPrt table defines test strings to use for verification for each printer while the line is in test mode.

| TestCnfgPrt Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the testing record. |
| Status | String | The program will display error or event messages here. |
| Printer | PopUp | Choose the printer. |
| VerifyStrL1 | String | Enter a string to be compared against the verification scan for the first label defined for this printer. |
| VerifyStrL2 | String | Enter a string to be compared against the verification scan for the second label defined for this printer. |

  - Table LabelTemplates

The LabelTemplates table is used to define the connection between labels and printers.

| LabelTemplates Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for the label definition. |
| LoadFile | Radio | Click the button to force the system to load the template if it is defined as a “File Load”, “Host Load” or “HostCopy” LoadType. The Status field will show the result. |
| Status | String | The program will display error or event messages here. |
| Printer | PopUp | Choose the printer. |
| LabelNum | PopUp | Choose “Label 1” in most cases. “Label 2” is for a second label to be applied by the same printer – usually used for label-over-label applications. |
| LabelType | PopUp | These label types are available: - Template – A special label that contains static information and placeholders that are replaced with data from the LMI message. Templates can be used to reduce the size of the LMI message by eliminating the need to re-send static things like company logo images with each label. The template is sent to the printer when FPC is started, and each time the printer is reset. - Standard Label – The most commonly used type. The PNA Engine sends a complete label image in the LMI each time with all the data filled in. - Test Label – A static label image loaded from the file defined in LabelFileName for use when TestMode = “Test Mode ON” in table PrintApplyLine. - Host Label – Used to define a host-selectable error label. The PNA Engine can select the label using the ExtLabelType, so multiple labels of this type may be defined. - Clear Printer – Used to define a special label image that will cause the printer to clear its print buffer. The static label image is loaded from the file defined in LabelFileName. It is sent to the printer when the operator selects the Clear button. - No Read – Optional. Used to define a special label image that will be applied when a NoRead is received from the container identifier scanner (LPN or SKU). The static label image is loaded from the file defined in LabelFileName. - No Data – Optional. Used to define a special label image that will be applied when a NoData event occurs. A NoData event occurs when the LRM response indicates an error. The static label image is loaded from the file defined in LabelFileName. |
| LoadType | PopUp | Defines the source of the label: - File Load – The label image is stored locally in the file defined by LabelFileName. - Host Load – The label image is requested from the PNA Engine when FPC starts or when LoadFile is selected. All records with the same ExtLabelType will be updated, so it is best to set only one to Host Load, and the others to Host Copy. - Host Copy – The label image is sent from the PNA Engine when requested by a Host Load record. If the ExtLabelType matches, the Host Copy records will be updated also. For example, if several NOREAD labels are defined, they will all be updated when one is set to Host Load and the rest are set to Host Copy. This avoids redundant requests. |
| ExtLabelType | String | If the label is being supplied from an external source, this field is used to specify the label’s identification string. This name is an agreed-upon string between FPC and the PNA Engine. |
| VerifyStr | String | This is used to define a verification string for NoRead or NoData labels. If those label types do not require verification leave the field blank. |
| LabelFileName | String | Defines the local file name and path for entries with LoadType = “File Load”. The path is relative to the RUN directory, so “LABELS/TestLabel.zpl”, and “/SortPlus/RUN/LABELS/TestLabel.zpl” are equivalent if the full path to the file is “/SortPlus/RUN/LABELS/TestLabel.zpl”. The relative path of the first example is preferable, as the absolute path will only work if the RUN directory is installed in the normal spot! |
| pNumBytes | Int | The calculated length of the label image, in bytes. |
| pTimeStamp | Time | The last time the label image was loaded. |
| pLastTime | Time | The last time a LabelType = “Template” label image was sent to the printer. Used to ensure the image is refreshed if the printer is reset. |
| pLabelImg | Blob | The actual label image data. The default size is 20,000 bytes in FPC. It may contain control characters, though the specification for the ZPL format does not include them. The maximum size allowed by the PNA Engine may be different. |

  - Table PNAStatusDisplayScreen

The PNAStatusDisplayScreen table contains the underlying data for the operator’s screen.

| PNAStatusDisplayScreen Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | Enter a unique name for this display screen. |
| PNA Line | PopUp | Choose the print apply line for this screen. |
| Shifter | PopUp | Not used |
| Line Status | String | Not used |
| Error Message | String | Shows the last error message for the line. |
| Status Message | String | Shows the last status message for the line. |
| Comment | String | Shows the last comment for the line. |
| Induct Scan | String | Shows the last induct scan received for the line. |
| Ship Barcode Expected | String | Ship label verification string from the LVM. |
| Ship Barcode Actual | String | Ship label verification string read from the scanner. |
| Ship Barcode Result | Int | Pass/Fail result for the Ship label. |
| SKU Barcode Expected | String | SKU verification string from the LVM. |
| SKU Barcode Actual | String | SKU verification string read from the scanner. |
| SKU Barcode Result | Int | Pass/Fail result for the SKU. |
| RF Barcode Expected | String | RF tag verification string from the LVM. |
| RF Barcode Actual | String | RF tag verification string read from the scanner. |
| RF Barcode Result | Int | Pass/Fail result for the RF tag. |
| Verify Top Expected | String | Top label verification string from the LVM. |
| Verify Top Actual | String | Top label verification string read from the scanner. |
| Verify Top Result | Int | Pass/Fail result for the Top label. |
| Verify Left Expected | String | Left label verification string from the LVM. |
| Verify Left Actual | String | Left label verification string read from the scanner. |
| Verify Left Result | Int | Pass/Fail result for the Left label. |
| Verify Right Expected | String | Right label verification string from the LVM. |
| Verify Right Actual | String | Right label verification string read from the scanner. |
| Verify Right Result | Int | Pass/Fail result for the Right label. |
| Verify Front Expected | String | Front label verification string from the LVM. |
| Verify Front Actual | String | Front label verification string read from the scanner. |
| Verify Front Result | Int | Pass/Fail result for the Front label. |
| Status Induct | Int | Not used |
| Status Scan | Int | Not used |
| LPN Scan | String | Updated by the software |
| Status LRM Out | Int | Current state of the outgoing LRM message. |
| Status LRM In | Int | Current state of the incoming LRM message. |
| Status LMI1 In | Int | Current state of the LMI for label 1. |
| Status LMI2 In | Int | Current state of the LMI for label 2. |
| Status Label1 Sent | Int | Indicates if label 1 has been sent to the printer. |
| Status Label2 Sent | Int | Indicates if label 2 has been sent to the printer. |
| Status Tamp1 | Int | Not used |
| Status Tamp2 | Int | Not used |
| Status Verify Scan | Int | Current state of the verification scan. |
| Verify Scan | String | The latest verification scan received. |
| Status LVM Sent | Int | Current state of the outgoing LVM message. |

# Example Configurations

Examples for four common configurations are given.

Barcode scanner

Verification scanner

Print and Apply labelers

Print Apply belt

Skew section

Brake/meter belt

Tracking PE induct point

Scanner tailout point

Nose of the longest box from scan tailout

Labeler tamp point

Labeler endpoint, beginning of the scan tunnel

Verify scanner tailout point

Nose of the longest box from verify tailout

Height detection

  - Example 1:  Two Printers, Auto-Disable Mode

The line has two printers printing the same label type.

The line shall continue to run if a printer error occurs.

The line will stop only if both printers fail.

- Set PrintApplyLine_AutoErrHandling = Y

- Set PrinterErrConfig_DisableOnMaxVerifyErrs = Y

- Set PrinterErrConfig_DisableLineOnPrinterErr = N

- Set PrinterErrConfig_DisableOnLOTARFail = Y

- Set PrinterErrConfig_DontClearIfSent = N

- Set PrinterErrConfig_ClearLabelStatus = N

- Set PrinterErrConfig_DontClearVerifyErr = N

Operationally, if the line stops for an error, all cartons with a label already applied but not yet verified and those that have a label in the printer’s queue must be physically removed from the belt as part of the error recovery procedure (DontClearIfSent).  There is no need to use the “Clear All” button on the operator’s screen since the label status is automatically cleared for all involved cartons when the error occurs (ClearLabelStatus).  If an error occurs on one printer the line keeps running.  The cartons associated with that error will have their tracking information cleared automatically so they can stay on the belt and proceed to the error correction destination (DontClearVerifyErr).

  - Example 2:  Two Printers, Lines Stop on Error

The line has two printers printing the same label type.

The line shall stop if a printer error occurs.

- Set PrintApplyLine_AutoErrHandling = N

- Set PrinterErrConfig_DisableOnMaxVerifyErrs = Y

- Set PrinterErrConfig_DisableLineOnPrinterErr = Y

- Set PrinterErrConfig_DisableOnLOTARFail = Y

- Set PrinterErrConfig_DontClearIfSent = N

- Set PrinterErrConfig_ClearLabelStatus = Y

- Set PrinterErrConfig_DontClearVerifyErr = Y

Operationally, a single printer error will cause the line to stop.  All cartons with a label already applied but not yet verified and those that have a label in the printer’s queue must be physically removed from the belt as part of the error recovery procedure (DontClearIfSent).  Tracking data is not cleared, allowing the cartons to be checked at the verification scanner (DontClearVerifyErr).  The label status will not be cleared until the operator clicks “Clear All” (ClearLabelStatus).

  - Example 3:  Two Printers, Auto-Disable, Minimal Clear

The line has two printers printing the same label type.

The line has height, width, and length detection as part of the LPN scanner.  The line will stop for cartons that are out of range.  Valid ranges are defined in the HeightWidth table.

If a dimension error occurs, the line will stop.  The operator will manually correct as many cartons as possible without moving them on the belt.  Any that cannot be corrected must be physically removed.  Cartons with labels (after correction) are left on the belt.

The line will continue to run for printer errors as long as at least one printer is available.

- Set PrintApplyLine_AutoErrHandling = Y

- Set PrinterErrConfig_DisableOnMaxVerifyErrs = Y

- Set PrinterErrConfig_DisableLineOnPrinterErr = N

- Set PrinterErrConfig_DisableOnLOTARFail = Y

- Set PrinterErrConfig_DontClearIfSent = Y

- Set PrinterErrConfig_ClearLabelStatus = N

- Set PrinterErrConfig_DontClearVerifyErr = N

- Define update points in table PrinterUpdPnts for:

  - “PNA Start Zone Clear” at the point labeled “Scanner tailout point” in the diagram above.

  - “PNA Zone Clear” at the point labeled “Labeler endpoint, beginning of the scan tunnel” in the diagram above.

Operationally, if the line stops (for a dimension error) cartons that have had a label applied or that can be manually labeled from a label on the tamp head or in the printer’s queue can remain on the belt (DontClearIfSent).  The label status will be cleared automatically if a printer error occurs – there is no need to use “Clear All” (ClearLabelStatus).  Tracking data is automatically cleared, allowing cartons to proceed to the error correction lane (DontClearVerifyErr).  The “PNA Start Zone Clear” and “PNA Zone Clear” update points frame the portion of the belt that must be cleared.

  - Example 4:  Shipping Label over Contents Label

The line has one contents label printer and one shipping label printer.  Each printer has a verification scanner.  The one for the contents label is located between the two printers.  The one for the shipping label is located after the shipping label printer.

One belt is used for the entire line.  An LPN scanner is used to identify the cartons.

The line has an ultrasonic height sensor.

The line has a local reject handling spur.

- Set PrintApplyLine_AssignType = Static

- Set PrintApplyLine_TestMode = N/A (not in test mode)

- Set PrintApplyLine_SendType = “Upd Pnt – Lbl Send” (the label is sent to the printer when the update point is reached.  This is used so the shipping label will not be sent if an error was detected on the Contents label.

- Set PrintBelt_Sorter = “PNA Sorter”

- Set PrintBelt_SlowSpeed = 0 (percent)

- Set PrintBelt_MaxSpeed = 100 (percent)

- Set PrintBelt_WaitRdySpeed = 0 (percent)

Configure seven update points in table PrinterUpdPnts:

| Example 4, PrinterUpdPnts Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Type | PrintLine | Printer | LabelNum | UpdatePnt |
| PA1 Height (Records carton height) | Height Update | Line 1 | N/A | N/A | Height Upd |
| P1 Send Label (Waits for the contents label, then sends it to the printer) | Send Label | Line 1 | P1 | Label 1 | Send P1 Update |
| P1 Tamp (1 Label Small) (Defines the tamp point for the shortest carton) | Tamp 1 Label Small | Line 1 | P1 | Label 1 | P1 Label Small |
| P1 Tamp (1 Label Large) (Defines the tamp point for the tallest carton) | Tamp 1 Label Large | Line 1 | P1 | Label 1 | P1 Label Large |
| P2 Send Label (Waits for the shipping label, then sends it to the printer. If this point is downstream of the P1 verify point, the shipping label will not be printed if an error was detected with the contents label) | Send Label | Line 1 | P2 | Label 1 | Send P2 Update |
| P2 Tamp (1 Label Small) (Defines the tamp point for the shortest carton) | Tamp 1 Label Small | Line 1 | P2 | Label 1 | P2 Label Small |
| P2 Tamp (1 Label Large) (Defines the tamp point for the tallest carton) | Tamp 1 Label Large | Line 1 | P2 | Label 1 | P2 Label Large |

- For the LPN scanner, set ScnScanDevice_ScanType = “LPN Scan”, PAPrinterScanAssign_Printer = “P1”, PAPrinterScanAssign_LabelNum = “Label 1”, PAScannerBoss_PrintLine = “Line 1”, PAScannerBoss_SendLVM = N/A (no LVM for the induction scanner).

- For the contents label verification scanner, set PAScannerBoss_PrintLine = “Line 1”, PAScannerBoss_SendLVM = N/A, PAScannerBoss_VerifyType = “Verify Scan (One)”.  Verify only this label, do not send the LVM – yet.  If an error occurs here, the shipping label need not be applied or checked.

- For the shipping label verification scanner, set PAScannerBoss_PrintLine = “Line 1”, PAScannerBoss_SendLVM = “Send LVM Message”, PAScannerBoss_VerifyType = “Verify All”.  Verify all labels and send an LVM message.

# PNA Operators Screen

The PNA operator’s screen.  Data is taken from the PNAStatusDisplayScreen table, though not everything in the table is shown on the operator’s screen.

- Print and Apply Line – Select the PNA line to display.

- Reset Fault – Click this button to reset a line fault.

- Print and Apply Line Status – The items in this box apply to the entire line.

        - Line Status – Shows a colorized box indicating if the line is RUNNING or STOPPED.

        - Error Message – The last error message for the line.

        - Status Message – The last status message for the line.

        - Comment – Any additional information related to the status or error for the line.

        - Clear – Click this button to clear line errors.

        - Induct Scan – The last induct scan (LPN or SKU) received.

- Print and Apply Last Results – The items in this box show the latest verification data.

        - Induct Scan Barcode (LPN) Value – The LPN associated with the verification data.

        - Verify Label Types – Verification data is presented for two label types:  Shipping Label and Contents Label.

        - Expected – These fields show the strings from the LVM that are compared to the Actual verification scans.

        - Actual – These fields show the received verification strings that are compared to the Expected strings.

        - Result – The result of the comparison of the Actual and Expected verification data.

        - Clear – Click this button to clear the verification data.

# PNA Printer Test Options Screen

The Printer Test Options button on the operator’s screen opens this screen with more control options and status information for each printer.

- Printer Test Options box – This box has items that can be used to take actions on a printer:

        - Printer – Choose the printer to act upon.

        - Clear Labels – Click this button to send the “Clear Printer” LabelType from the LabelTemplates table to the printer.  If properly configured, it will send commands to the printer to clear any unprinted label images from its queue.

        - Reset Printer – Click this button to send the PrinterResetOut signal (defined in table PrinterBoss) to the printer.

        - Print Test Label – Click this button to send the “Test Label” LabelType from the LabelTemplates table to the printer.  A locally-stored label image file is sent to the printer.

        - Tamp Printer – Click this button to send the PrinterBoss_TamperSolOut signal to fire the tamper.

        - Send Template – Click this button to send the “Template” LabelType from the LabelTemplates table.  If templates are being used, the label image will contain a reference to the template and the data to “fill in the blanks” on the template form.  After a reset, the printer may lose the template.

        - ReSend Label – Click this button to re-send the last label to the printer.

        - Load Label – Enter the file name (and path) for a local label file to load.  You can use the button with the ellipsis dots to open a dialog box to help you find the file.

        - Print Label – Click this button to send the file from the Load Label box to the printer.

- Printer Status box – This box shows status items for the selected printer:

        - Error – Displays the current state of the PrinterBoss_PrinterErrorIn signal.

        - Tamp Home – Displays the current state of the PrinterBoss_TamperHomeIn signal.

        - LOTAR – Displays the current state of the PrinterBoss_LabelOnTampIn signal.

        - SOLENOID – Displays the current state of the PrinterBoss_TamperSolOut signal.

        - CYCLE COMPLETE – Displays the current state of the PrinterBoss_CycleCompleteIn signal.

        - STATUS – Displays the contents of the PrinterBoss_DspStatus field for this printer.

- Exit –Click this button to exit the screen.

# PNA Operators Screen (Old Version)

This older version of the PNA operator’s screen may still be in use at some sites.

The LPN or SKU is displayed as the carton enters the line.

If the line stops for a printer fault or a verification error, it will show in the ErrorMsg field, along with the other available data.

- Scanned LPN – The last LPN scanner message received.

- Scanned SKU – The last SKU scanner message received.

- Scanned RF – The last RF scanner message received.

- Scanned – The last verification scanner message received.

- Expected SKU – When the Scanned SKU field is updated the Expected SKU field is also updated with the verification data from the LRM message.

- Expected RF - When the Scanned RF field is updated the Expected RF field is also updated with the verification data from the LRM message.

- Expected – When the Scanned field is updated the Expected field is also updated with the verification data from the LRM message.

- Printer – Choose a specific printer from the list for the following maintenance functions:

        - Send Template – Click the button to send the template to the printer.

  - Print Test Label – Click the button to print a test label.

  - Tamp Printer – Click the button to tamp a label.

  - Error – This button will be illuminated if the printer has an error.

  - Reset – Click this button to reset a printer error.

  - Clear – Click this button to clear the label data from the belt for the selected printer.

- Status – Indicates the status of the print apply line.

- ErrorMsg – The latest error message for the print apply line, if any.

- Comment – Additional comment line.  Some status and error messages may have additional detail.

      - VerifyError – If lit, indicates the line has stopped due to a verification error.

- Clear All – Click this button to clear the label data from the belt for all printers.

      - Reset Fault – Click this button to reset a fault for the line.  If a printer error is active, the printer will be reset.  If the line has stopped, this will also restart it.

# PNA Message Content Details

Messages exchanged between the PNA AC and the PNA Engine are pipe-delimited text wrapped with a <STX> (Start-of-text, Ctrl-B) and <ETX> (End-of-text, Ctrl-C) character.  A message header field identifies the type of message, and a data header field identifies the data source.  The rest of the fields appear as key/value pairs.

  - LRM – Label Request Message

The LRM is generated by the PNA AC when a carton is scanned at the induction scanner.  The LRM response from the PNA Engine will list required labels and their verification requirements.  If the label queue is to be managed by FPC, the PNA Engine will also generate LMI messages for each required label.

In the following tables, Field shows the name of the field in the message table in FPC, such as MsgCR1.  Content shows literal or example content for the field as it will appear in the message.  Description has some explanation of the meaning of the field.

| Message Content: Label Request Message, Outbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | LRM | Identifies this as a Label Request Message |
| Header | PA1 | Machine_Name from table Machine for this AC. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “iQty” pair is counted as the first pair. |
| Dat1 | 28 | LVM has twenty-eight key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcContainer | Table name. |
| Col3 | “sContainerNum” or “sSKU” | Key name literal. The ConfirmScan string from the tracking record. “sContainerNum” is used for LPN configurations, “sSKU” is used for SKU-based configurations. |
| Dat3 | 0123456789 | The carton identifier string, as read by the induct scanner. |
| Col4 | sRFID | Key name literal. |
| Dat4 |  | The RFID data read from the scanner. |
| Col5 | sScanZone | Key name literal. |
| Dat5 |  | The scan zone identifier, as found in “ScanZoneID” in the tracking table. |
| Col6 | sStatus | Key name literal. |
| Dat6 | Okay | For LRM, as sent from the PNA AC, this will always be “Okay”. |
| Col7 | sStatusMsg | Key name literal. |
| Dat7 |  | For LRM, as sent from the PNA AC, this will always be blank. |
| Col8 | sTrackTable | Key name literal. |
| Dat8 | 337 | The FPC internal table number for the tracking table. |
| Col9 | sControlNum | Key name literal. |
| Dat9 | 17 | The FPC internal row number for the tracking record within sTrackTable. |
| Col10 | sHeight | Key name literal. |
| Dat10 |  | The height value. Numeric: nnn.nn |
| Col11 | sWidth | Key name literal. |
| Dat11 |  | The width value. Numeric: nnn.nn |
| Col12 | sLength | Key name literal. |
| Dat12 |  | The length value. Numeric: nnn.nn |
| Col13 | sVerifyRF | Key name literal. |
| Dat13 |  | Blank in the outbound message. |
| Col14 | sVerifySKU | Key name literal. |
| Dat14 |  | Blank in the outbound message. |
| Col15 – Col26 | sTypeL1 – sTypeL6 | Key name literal for the six possible labels for the carton. |
| Dat15 – Dat26 |  | Blank in the outbound message. |
| Col27 | sScale | Key name literal. |
| Dat27 | 5.2 | Weight of carton. |
| Col28 | sScaleValid | Key name literal. |
| Dat28 | 1 | 0 = Invalid scale reading 1 = Valid scale reading |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

Note:  If Col3 is sent as sSKU, the PNA Engine will assign the SKU to the carton.  The PNA Engine must be configured in such a way as to not assign that carton to multiple PNA lines, as SKUs are not unique!

Note:  If Col3 is sent as sContainer, and an LRM is sent to the PNA Engine for a carton it does not know about, it will create the record in the Oracle zcContainer table.  If this occurs, of course, the PNA Engine will not have labels or verification requirements for the carton.

The PNA Engine responds to an LRM message by echoing back the identifier information, and filling in the label and verification requirements.

| Message Content: Label Request Message, Inbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | LRM | Identifies this as a Label Request Message |
| Header | PA1 | Machine_Name echo from LRM request. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “iQty” pair is counted as the first pair. |
| Dat1 | 28 | LVM has twenty-eight key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcContainer | Table name. |
| Col3 | “sContainerNum” or “sSKU” | Key name literal. The string from the LRM request is echoed. sContainerNum and sSKU are handled differently by the PNA Engine. See the note, above. |
| Dat3 | 0123456789 | The carton identifier string, echoed from the LRM request. |
| Col4 | sRFID | Key name literal. |
| Dat4 |  | The RFID data, echoed from the LRM request. |
| Col5 | sScanZone | Key name literal. |
| Dat5 |  | The scan zone identifier, echoed from the LRM request. |
| Col6 | sStatus | Key name literal. |
| Dat6 | “Okay” or “Error” | “Okay” – sStatusMsg will be blank. “Error” – sStatusMsg will contain a description. |
| Col7 | sStatusMsg | Key name literal. |
| Dat7 |  | See Dat6, above. |
| Col8 | sTrackTable | Key name literal. |
| Dat8 | 337 | The FPC internal table number for the tracking table, echoed back from the LRM request. |
| Col9 | sControlNum | Key name literal. |
| Dat9 | 17 | The FPC internal row number for the tracking record within sTrackTable, echoed back from the LRM request. |
| Col10 | sHeight | Key name literal. |
| Dat10 |  | The height value, echoed back from the LRM request. |
| Col11 | sWidth | Key name literal. |
| Dat11 |  | The width value, echoed back from the LRM request. |
| Col12 | sLength | Key name literal. |
| Dat12 |  | The length value, echoed back from the LRM request. |
| Col13 | sVerifyRF | Key name literal. |
| Dat13 |  | The verification requirement for a RFID label, if any. |
| Col14 | sVerifySKU | Key name literal. |
| Dat14 |  | The verification requirement for a SKU label, if any. |
| Col15 – Col26 | sTypeL1 – sTypeL6 | Key name literal for the six possible labels for the carton. |
| Dat15 – Dat26 |  | Verification requirements for each label, if any. |
| Col27 | sScale | Key name literal. |
| Dat27 | 5.2 | Weight of carton, echoed back from the LRM request. |
| Col28 | sScaleValid | Key name literal. |
| Dat28 | 1 | 0 = Invalid scale reading 1 = Valid scale reading Echoed back from the LRM request. |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

  - LMI – Label Image Message

When the PNA Engine receives an LRM, it will generate zero or more LMI messages in response, in addition to the reply for the LRM.  The number of LMI messages depends on the number of labels to be applied.  The LMI has a pipe-delimited text header similar to the other messages, but the main payload is the label image in the last field, which can be up to 20,000 characters in length.  The LMI also includes a five-digit sequence number that follows the <STX> and is used to identify the acknowledgement message.  The label image is usually in ZPL format.

| Message Content: Label Image Message, Inbound |  |  |  |
| --- | --- | --- | --- |
| Content | Description |  |  |
| <STX> |  | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| 00001 |  | A sequential number starting at 00001 and incrementing to 99999 that uniquely identifies the LMI message. The value is assigned by the PNA Engine. |
| LMI | Identifies this as a Label Image Message |  |  |
| 0123456789 | The carton identifier number from the induct scan, as it appeared in the LRM sContainer field. It could also be set to “TEMPLATE” for template images. This string is written to the RcvLabels[1-5] table LPN field. |  |  |
| 0012345 | The SKU identifier number from the induct scan, as it appeared in the LRM sSKU field. This string is written to the RcvLabels[1-5] table SKU field. |  |  |
| SHIPLBL | The label type associated with this label image, as it appeared in the LRM sType* field. This string is written to the RcvLabels[1-5] table ExtLabelType field. It will be matched to LabelTemplates_ExtLabelType. |  |  |
| 337 | The FPC internal table number for the tracking table, as it appeared in the LRM sTrackTable field. Used to find the correct tracking record for this label. |  |  |
| 17 | The FPC internal row number for the tracking record within the tracking table, as it appeared in the LRM sControlNum field. Used to find the correct tracking record for this label. |  |  |
| <blob> | The label image, up to 20,000 bytes (by default). |  |  |
| <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |  |  |

The PNA AC acknowledges the LMI message with an abbreviated form that contains just the message sequence number, for identification, and the message status.

| Message Content: Label Image Message, Outbound |  |
| --- | --- |
| Content | Description |
| <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| 00001 | A sequential number starting at 00001 and incrementing to 99999 that uniquely identifies the LMI message. The value is assigned by the PNA Engine. Here in the acknowledgement, the value sent from the PNA Engine is echoed back. |
| ACK | In response to an LMI, “ACK” will always be sent. In the unlikely event that a “NAK” message is received from the PNA Engine, it will be echoed back. |
| <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

  - LVM – Label Verify Message

The PNA AC sends a Label Verify Message to the PNA Engine to report the results of the verification.  The PNA Engine echoes the message back as an acknowledgement.  The LVM can report the verification status of up to six labels.  The PNA Engine may make the information available at exception handling workstations to aid in error correction.

Note:  The order of the verification results in the sTypeL[1-6]/sVerifyL[1-6] key/value pairs is based on the order in the tracking table and may not match the order of the verification requirements as delivered in the LRM.

The sStatusMsg field will contain one of these numeric codes:

0 –	Not all labels have been verified.

1 –	All labels have passed verification.
Also used if the carton has no verification requirement.

2 –	A verification error has occurred, or status is included.  See the individual label status values.

The sStatus* fields will contain one of these numeric codes:

0 -	The label has not been verified (yet).

1--	The label has passed verification.

2 -	The label has failed verification because it did not match the expected string.

3 -	The label has failed verification due to a scanner error.

4 -	No attempt was made to verify the label due to a prior verification failure.

The sType* fields will contain the string from the printer tracking table fields ExtTypeL1 or ExtTypeL2.  They are from LabelTemplates_ExtLabelType.

| Message Content: Label Verify Message, Outbound and Inbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | LVM | Identifies this as a Label Verify Message |
| Header | PA1 | Machine_Name from table Machine for this AC. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “iQty” pair is counted as the first pair. |
| Dat1 | 32 | LVM has thirty-two key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcContainer | Table name. |
| Col3 | “sContainerNum” or “sSKU” | Key name literal. The ConfirmScan string from the tracking record. “sContainerNum” is used for LPN configurations, “sSKU” is used for SKU-based configurations. |
| Dat3 | 0123456789 | The carton identifier string, as read by the induct scanner. |
| Col4 | sScanZone | Key name literal. |
| Dat4 | SHIPSCAN | The scan zone identifier, as found in “ScanZoneID” in the tracking table. |
| Col5 | sStatus | Key name literal. |
| Dat5 | Okay | For LVM, this will always be “Okay”. |
| Col6 | sStatusMsg | Key name literal. |
| Dat6 | 1 | The overall error status code number for the carton, as described above for sStatusMsg |
| Col7 | sTrackTable | Key name literal. |
| Dat7 | 337 | The FPC internal table number for the tracking table. |
| Col8 | sControlNum | Key name literal. |
| Dat8 | 17 | The FPC internal row number for the tracking record within sTrackTable. |
| Col9 | sStatusRF | Key name literal for RFID label status. |
| Dat9 | 1 | The sStatus* error code number, as described above. |
| Col10 | sStatusSKU | Key name literal for SKU label status. |
| Dat10 | 1 | The sStatus* error code number, as described above. |
| Col11 | sTypeL1 | Key name literal for the first label type. |
| Dat11 | SHIPLBL | The sType* string, as described above. |
| Col12 | sStatusL1 | Key name literal for the first label status. |
| Dat12 | 1 | The sStatus* error code number, as described above. |
| Col13 | sVerScanL1 | Key name literal for the first label verify scan. |
| Dat13 | 1Z0123456789 | The string from the verification scanner for the first label. |
| Col14 | sTypeL2 | Key name literal for the second label type. |
| Dat14 | CONTLBL | The sType* string, as described above. |
| Col15 | sStatusL2 | Key name literal for the second label status. |
| Dat15 | 1 | The sStatus* error code number, as described above. |
| Col16 | sVerScanL2 | Key name literal for the second label verify scan. |
| Dat16 |  | The string from the verification scanner for the second label. |
| Col17 | sTypeL3 | Key name literal for the third label type. |
| Dat17 |  | The sType* string, as described above. |
| Col18 | sStatusL3 | Key name literal for the third label status. |
| Dat18 | 1 | The sStatus* error code number, as described above. |
| Col19 | sVerScanL3 | Key name literal for the third label verify scan. |
| Dat19 |  | The string from the verification scanner for the third label. |
| Col20 | sTypeL4 | Key name literal for the fourth label type. |
| Dat20 |  | The sType* string, as described above. |
| Col21 | sStatusL4 | Key name literal for the fourth label status. |
| Dat21 | 1 | The sStatus* error code number, as described above. |
| Col22 | sVerScanL4 | Key name literal for the fourth label verify scan. |
| Dat22 |  | The string from the verification scanner for the fourth label. |
| Col23 | sTypeL5 | Key name literal for the fifth label type. |
| Dat23 |  | The sType* string, as described above. |
| Col24 | sStatusL5 | Key name literal for the fifth label status. |
| Dat24 | 1 | The sStatus* error code number, as described above. |
| Col25 | sVerScanL5 | Key name literal for the fifth label verify scan. |
| Dat25 |  | The string from the verification scanner for the fifth label. |
| Col26 | sTypeL6 | Key name literal for the sixth label type. |
| Dat26 |  | The sType* string, as described above. |
| Col27 | sStatusL6 | Key name literal for the sixth label status. |
| Dat27 | 1 | The sStatus* error code number, as described above. |
| Col28 | sVerScanL6 | Key name literal for the sixth label verify scan. |
| Dat28 |  | The string from the verification scanner for the sixth label. |
| Col29 | sHeight | Key name literal. |
| Dat29 |  | The height value. Numeric: nnn.nn |
| Col30 | sWidth | Key name literal. |
| Dat30 |  | The width value. Numeric: nnn.nn |
| Col31 | sLength | Key name literal. |
| Dat31 |  | The length value. Numeric: nnn.nn |
| Col32 | sScale | Key name literal. |
| Dat32 |  | The weight value. Numeric: nnn.nn |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

  - TRM – Template Request Message

A TRM is initiated by the PNA AC to request template images from the PNA Engine.  TRMs are associated with records in table LabelTemplates that use LabelType = “Template”.

The template label image is transmitted by the PNA Engine via an LMI message.  Locally-defined template files are supported via LabelTemplates_LabelType = “Template” and LabelTemplates_LoadType = “File Load”.  The label image is stored in LabelTemplates_pLabelImg, which is a blob data type with room for 20,000 bytes by default.

| Message Content: Template Request Message, Outbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | TRM | Identifies this as a Template Request Message |
| Header | PA1 | Machine_Name from table Machine for this AC. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “iQty” pair is counted as the first pair. |
| Dat1 | 6 | TRM has six key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcTemplates | Table name. |
| Col3 | sLabelType | Key name literal. |
| Dat3 | SHIPTMPLT | From LabelTemplates_ExtLabelType |
| Col4 | sFunction | Key name literal. |
| Dat4 | Read | The function associated with this message. It is always “Read” for TRM. |
| Col5 | sStatus | Key name literal. |
| Dat5 | Okay | The request is always “Okay” |
| Col6 | sStatusMsg | Key name literal. |
| Dat6 |  | The request status is “Okay”, so this field is blank. |
|  |  | Col/Dat pairs 7-32 are not used for TRM. |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

The PNA Engine responds with the same identifier information but may update sStatus and sStatusMsg if an error occurs.

| Message Content: Template Request Message, Inbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | TRM | Identifies this as a Template Request Message |
| Header | PA1 | Machine_Name from table Machine for this AC. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “Qty” pair is counted as the first pair. |
| Dat1 | 6 | TRM has six key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcTemplates | Table name. |
| Col3 | sLabelType | Key name literal. |
| Dat3 | SHIPTMPLT | From LabelTemplates_ExtLabelType |
| Col4 | sFunction | Key name literal. |
| Dat4 | Read | The function associated with this message. It is always “Read” for TRM. |
| Col5 | sStatus | Key name literal. |
| Dat5 | Okay | “Okay” – sStatusMsg will be blank. “Error” – sStatusMsg will contain a description. |
| Col6 | sStatusMsg | Key name literal. |
| Dat6 |  | See Dat5, above. |
|  |  | Col/Dat pairs 7-32 are not used for TRM. |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

  - PRM – Printer Ready Message

Note:  The Printer Ready Message is only needed in systems that send the label image directly to the printer from the PNA Engine or the customer’s host system, and if the applicator does not provide a PrinterBoss_LabelOnTampIn signal.  All modern applicators provide that signal, and most systems now send the label image to the PNA AC via an LMI, so this message is obsolescent.

A PRM is initiated by the PNA Engine to tell the PNA AC that a label has been sent directly to the printer.  The PNA AC can be configured to hold the carton until this message is received before allowing it to advance to the tamp point.  This serves the same function as LabelOnTampIn.

The LRM used to initiate handling of a carton contains references to the carton (sContainerID), tracking table number (sTrackTable) and tracking record number (sControlNum) to uniquely identify the carton.  The PNA Engine will use these identifiers in the PRM.

| Message Content: Printer Ready Message, Inbound and Outbound |  |  |
| --- | --- | --- |
| Field | Content | Description |
|  | <STX> | Start-of-text character (a.k.a., Ctrl-B). Signifies the beginning of a message to the message parser. |
| CmdHdr | PRM | Identifies this as a Printer Ready Message |
| Header | PA1 | The unique name of the print apply line. |
| Col1 | iQty | Literal string for the count of key/value pairs to expect in this message. The “iQty” pair is counted as the first pair. |
| Dat1 | 9 | PRM has nine key/value pairs. |
| Col2 | sTableName | Key name literal. |
| Dat2 | zcContainer | Table name. |
| Col3 | sContainerID | Key name literal. |
| Dat3 | 0123456789 | The carton identifier string, as read by the induct scanner. |
| Col4 | sFunction | Key name literal. |
| Dat4 | Write | The function associated with this message. It is always “Write” for PRM. |
| Col5 | sStatus | Key name literal. |
| Dat5 | “Okay” or “Error” | “Okay” – sStatusMsg will be blank. “Error” – sStatusMsg will contain a description. |
| Col6 | sStatusMsg | Key name literal. |
| Dat6 |  | See Dat5, above. |
| Col7 | sTrackTable | Key name literal. |
| Dat7 | 337 | The FPC internal table number for the tracking table. The PNA Engine gets this value from the LRM message for this carton. |
| Col8 | sControlNum | Key name literal. |
| Dat8 | 17 | The FPC internal row number for the tracking record within sTrackTable. |
| Col9 | sTypeL1 | Key name literal. |
| Dat9 | SHIP | The label type, matching LabelTemplates_ExtLabelType. This is used to determine which printer the message is associated with. |
|  |  | Col/Dat pairs 10-32 are not used for PRM. |
|  | <ETX> | End-of-text character (a.k.a., Ctrl-C). Signifies the end of a message to the message parser. |

The PNA AC echoes the PRM back to the PNA Engine without any changes as an acknowledgement.

# PNA Dimension Sensor Calibration

Height or width sensors are needed when the position of the carton face that will receive the label varies with respect to the tamper.  That would include top-apply labels where more than one box size is used, and side-apply labels if the cartons are justified to the side of the conveyor opposite the applicator.  If the cartons are justified to the same side as the applicator, the distance will be constant, so no height sensor is needed.

NOTE:  A record must be created in the HeightWidth table even if height and width sensors are not used.  The minimal configuration requires Name, SensorType and PrintLine.

  - Using Two Photoeyes

When using two photoeyes for height detection, adjust the first sensor to 1/3 the distance between the height of the shortest carton and the tallest carton.  Adjust the second sensor to 2/3 the distance.

PE 1 Height = Shortest + ((Tallest – Shortest) * 1/3)
PE 2 Height = Shortest + ((Tallest – Shortest) * 2/3)

Example:
Shortest = 3 inches
Tallest = 24 inches
PE 1 Height = 3 + ((24 – 3) * 1/3 = 10 inches from the belt.
PE 2 Height = 3 + ((24 – 3) * 2/3 = 17 inches from the belt.

Tallest Carton

2/3 Height Sensor

Shortest Carton.

1/3 Height Sensor

  - Using One Photoeye

When using a single photoeye for height detection, adjust the sensor position so it is halfway between the height of the shortest and tallest cartons.  In table HeightWidth, use this photoeye in both the Height1In and Height2In fields.

Tallest carton

Shortest carton.

1/2 Height sensor

  - Using Ultrasonic or Laser Height/Width Sensors

Each sensor is calibrated by placing example cartons on the belt and recording the values returned by the sensor and the actual carton dimension measured with a tape measure.  Use the resulting data to configure the HeightWidth table.

Width sensor

Height sensor

The carton is justified to this side

| Height Calibration – Ultrasonic and Laser |  |  |
| --- | --- | --- |
| Description | Height Inches | Height Read by the Sensor |
| Shortest carton | Measure the height with a tape measure, in inches. Enter the measured height into HeightWidth_BoxHgtInches1 | Enter the sensor height value from HeightWidth_pLastSensorHgt into HeightWidth_SensorHgt1. |
| Tallest carton | Measure the height with a tape measure, in inches. Enter the measured height into HeightWidth_BoxHgtInches2. | Enter the sensor height value from HeightWidth_pLastSensorHgt into HeightWidth_SensorHgt2. |

| Width Calibration – Ultrasonic and Laser |  |  |
| --- | --- | --- |
| Description | Width Inches | Height Read by the Sensor |
| Narrowest carton | Measure the width with a tape measure, in inches. Enter the measured width into HeightWidth_BoxWdtInches1. | Enter the sensor width value from HeightWidth_pLastSensorWdt into HeightWidth_SensorWdt1. |
| Widest carton | Measure the width with a tape measure, in inches. Enter the measured width into HeightWidth_BoxWdtInches2. | Enter the sensor width value from HeightWidth_pLastSensorWdt into HeightWidth_SensorWdt2. |

After the configuration is updated, test with a medium sized carton.  Compare the HeightWidth_pLastHeightInches or HeightWidth_pLastWidthInches with your tape measure reading to verify.

# PNA Dimension Sensor Update Point Calibration

For ultrasonic and laser sensors, the update point used to collect the height or width measurements must be adjusted so the reading is taken near the middle of the carton for best accuracy.  Photoeye sensors will trigger as the nose of the box arrives.

Compute an initial setting using:

I =	Induct point location along the belt, in ticks (usually 30).

T =	Ticks-per-inch setting for the belt.

S =	Sensor trigger photoeye location along the belt in inches from the induct photoeye.

C =	Carton length of the smallest carton.

Sensor update point tick location = I + (T * (S + (C / 2)))

Induct photoeye

Sensor trigger photoeye

Sensor update point

Distance to sensor trigger photoeye

For photoeye-based configurations, the value may be adjusted based on “update” shift print messages found in the SORT log files.  Add the photoeye to the Name column in table Updates to activate the shift prints.  The “update” shift print will be generated when the photoeye transitions ON.  In this case, the photoeye must be in the expected state BEFORE the carton image arrives at the update point position, since the state is read at that moment and any state change will NOT be detected after that.  The example below shows a properly adjusted configuration:

update 1 8 ----------+++++++++1-?----------------------------

You can use the graphical shifter to make the adjustments.

NOTE:  The “update” shift print image is only generated when the photoeye transitions ON, so no shift print will be generated for short cartons!

NOTE:  Check the SORT logs for “PrintApplyHeightPhotoeye:”, “PrintApplyHeightUltrasonic:”, “PrintApplyWidthPhotoeye:”, or “PrintApplyWidthUltrasonic:” messages, depending on the sensor type configured.

# PNA RFID Label Setup

Some RFID printers can be configured with ZPL commands.  Things like RFID tag location and number of retries can be configured this way.  A “Template” LabelType can be used in table LabelTemplates to send these commands to the printer.

Label

RFID Tag

Measure from top of the label to the bottom of the tag.

Measure from the top of the label to the bottom of the RFID tag.  Add 0.94 inches to the length.  Convert the value to pixels.  For example:

(4.50 inches) + (0.94 inches) = 5.44 inches.
(5.44 inches) * (300 pixels per inch) = 1632 pixels

Use the calculated value as the basis for testing to determine the best value for repeatable reads.  Create a template file with the following contents based on the calculated value:

^XA^RS,1632,,1^XZ

Determine if the label can be read using this configuration, then increase the value 10 pixels at a time and re-test until it fails.  Repeat, decreasing the value from the calculated starting point 10 pixels at a time until it fails to read again.

| Test | Delta Pixels | Pixel Value | Result |
| --- | --- | --- | --- |
| 9 | -50 | 1582 | FAIL |
| 8 | -40 | 1592 | FAIL |
| 7 | -30 | 1602 | PASS |
| 6 | -20 | 1612 | PASS |
| 5 | -10 | 1622 | PASS |
| 1 | 0 | 1632 | PASS |
| 2 | +10 | 1642 | PASS |
| 3 | +20 | 1652 | FAIL |
| 4 | +30 | 1662 | FAIL |

The optimum value in this example is 1622, taken from the midpoint of the range of successful reads.  Edit the template file to use the optimum value:

^XA^RS,1622,,1^XZ

The value 1622 represents the location of the RFID tag on the label, and the value 1 represents the number of labels to retry if the RFID write fails.  The number of retries must be set to 1 for print apply automatic RFID applicators.

The location of the RFID tag can also be determined using a serial connection to the printer to send a calibrate command.  The printer will scan the tag and report the optimum spot.  The ZPL to issue the calibrate command is:  ^XA^HR^XZ.  The answer will be reported through the serial connection.  There may be several areas where the tags can be read successfully.  Use multiple test tags to repeat the procedure and select the best location where all tags can be successfully read.  Consult the printer’s manual for more details on how to do it this way.

# Print Apply Speed Control

Print apply lines may have several conveyor sections that require speed control.  In applications with multiple printers on the same line, the pitch (space between cartons) must be a multiple of the space between the printers (or the printers must be “on pitch”) to ensure that if the line must stop for one of the printers, the others will not be in the process of tamping while the belt is not running at full speed.  If this is not done, label placement errors (sometimes extreme – like missing the box entirely) will occur.  Sufficient time between cartons is also critical to allow the printers to receive, generate, and feed the labels onto the applicator mechanism.  When two labels are applied to a single carton from a single applicator the conveyor must be configured to slow down to allow the printer and applicator time to produce and apply the second label.

LPN or SKU scanner

Brake / meter belt

Print apply belt

SPEZ Section

Top labeler

Right side labeler

Verification scanners

Left-side labeler

Beacon

PNA AC

The speed control function (configured through the SpdControl table) searches the tracking records for cartons with speed control requirements for the printer.  The belt speed is adjusted if the carton labelling requirements demand it, or if the labeler is not ready.  The belt speed is set to the lowest speed needed after checking all cartons at all printers.  In label-over-label applications, when a “Label 2” is required, the belt speed is set to PrintBelt_SlowSpeed while the label is being applied and will stay there until PrinterUpdPnts_DelayTimer has expired.

# Zebra Printer Configuration

The Zebra printer default configuration will cause the printer to disconnect after 300 seconds (5 minutes) of inactivity.  This is not desirable for our application.  Use the printer’s web interface to set TIMEOUT CHECKING = NO for both “External Wired” and “Internal Wired” connections:

Zebra – Disable the Automatic Disconnect

(Set Timeout Checking to “NO”)

# Test Mode Configuration

  - A Test Label can be defined for each printer in the LabelTemplates table. The system searches for a match on Printer, LabelType of “Test Label”, and a LabelNum of “Label 1”.

  - If VerifyStr is provided the label is also verified. The expected verify scanner string is entered into the following test configuration tables.  The verify strings for the RF and SKU are defined in the TestConfig table VerifyStrRF and VerifyStrSKU fields, respectively.  The verify strings for individual printers are entered into the TestCnfgPrt table VerifyStrL1 and VerifyStrL2 fields.

  - If a container scan is not received, the No Read label is applied if it is defined in the LabelTemplates table.

  - If the print apply line is set to Round Robin, the system will print labels from each printer, assigning the labels using a round-robin algorithm.

Note:  The ExtLabelType1 or ExtLabelType2 field in the TestConfig table and the ExtLabelType field in the LabelTemplates table must match for the printers being used in the round-robin algorithm.

# JSON Message Protocol

With the advent of PNA in FRC, a message protocol was devised for the new PandA Engine that uses JSON formatting instead of the pipe-delimited format used for the traditional PandA Engine and all CommCore messages.

The JSON protocol is supported in FPC via Communication_Protocol = PandA in the Machine table.  ActiveMQ is still used for the connections.

When using the PandA protocol, FPC translates the messages internally into the more traditional message space.

The equivalent to the outbound LRM is LABELREQUEST.  The connection is made using Connection_Type = AMQ_TALK with Machine_Port = /queue/AC.PNA.LABELREQUEST.

The equivalent to the inbound LRM (the response) is LABELDATA.  The connection is made using Connection_Type = AMQ_LISTEN with Machine_Port = /queue/PNA.<ACName>.LABELDATA.

There is no LMI equivalent, as the LABELDATA message contains the label image usually found in the LMI, as well as the verification requirements usually found in the LRM response.

The equivalent to the LVM is LABELVERIFY.  The connection is made using Connection_Type = AMQ_TALK with Machine_Port = /queue/AC.PNA.LABELVERIFY.

NOTE:  The statusCode and statusMessage components of the LABELDATA message, which are mapped into sStatus and sStatusMsg, respectively, in the pipe-delimited format, contain verbose messages that are not compatible with the scheme within FPC.  To compensate, sStatus is set to “OKAY” if statusCode is “OKAY”, and to “ERROR” for all other responses.  The original statusCode string is written to the logs tagged with “RamMsgParsePandA: statusCode”.  Similarly, if statusCode is “OKAY”, sStatusMsg is set to a single space.  If statusCode is any other value, sStatusMsg is set to “ERROR”.  The original statusMessage string is written to the logs tagged with “RamMsgParsePandA: statusMessage”.  If you wish to use the ScnZoneErrIO_CommErrorMsg feature, you must set it to “ERROR”.  There is no provision for multiple error codes when using the PandA protocol.

From the source code:  The statusCode in the PandA message protocol is interpreted by the PandA engine from the container’s perspective – the container is in an error state.  This is primarily intended to allow the PandA engine to indicate the container should be diverted to an error correction lane, regardless of the labeling operation's success or failure.  In FPC, we interpret sStatusMsg and sStatus in a labeling transaction context.  The differences in semantics for statusCode and statusMessage are largely immaterial.  An error is an error and will require the same behavior physically.  Upon an error, statusMessage may contain a long message including carton IDs, which will not work well with the ScnZoneErrIO_CommErrorMsg feature.  Also, statusMessage may contain something even when statusCode = 'OKAY'.  This is not allowed in FPC so:  Force sStatusMsg to ' ' (a single space) when statusCode is “OKAY” and to “ERROR” otherwise.