# FortnaPlus Control Print Apply Fallback

Document Revision: 1.1

Date: 23-Jul-2021

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 01-Sep-2015 | B C Conklin | New document |
| 1.1 | 23-Jul-2021 | T R Kremser | Update and general refresh |

Table of Contents

1.	Introduction	4

1.1.	Definitions	4

2.	High-Level Summary	5

2.1.	Example Conveyor System Layout	5

3.	Process Flow	6

3.1.	Sorter Release	6

3.2.	Print Apply Line	6

3.2.1.	Gapper	7

3.2.2.	Printer	7

4.	Feature Configuration	7

4.1.	Table Batches	7

4.2.	Table PrintApplyFallback	7

5.	Example Configuration	10

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Introduction

This document provides a detailed description of the Print Apply Fallback Module.

# Definitions

| AC | Automation Controller. Computer hardware that runs the QNX operating system and FPC software. It controls the hardware through IO adapters, interfaces with CommCore and the Print Apply Engine, and has an operator interface. |
| --- | --- |
| CommCore | A Fortna product that provides business-logic information, such as Container destinations for Sorters. |
| Container | A general term used to describe cartons or totes that can be conveyed by the system. |
| Encoder | A device that measures belt movement, often in the form of a rotary device that emits a configurable number of electrical pulses (Ticks) per revolution measured by means of a wheel that rides against the underside of the belt. The ticks-per-turn setting, and the wheel circumference determine the resolution and can be converted to ticks-per-distance along the belt. |
| FPC | A Fortna product, FortnaPlus Control runs on an AC. The Print Apply software described in this document is a component of FPC. |
| Gap | The distance between tail of a Container and the nose of the next Container. |
| Gapper | A device that creates distance between containers. |
| New Gapper | A virtual device that creates gap. It is composed of a fixed-speed belt feeding into a second fixed-speed belt. A gap is created by stopping the first belt briefly. |
| Pitch | The distance between the nose of a Container and the nose of the next Container. |
| Print Apply Engine | A Fortna product that provides information to FPC on how to handle Containers on a Print Apply Line, including label image data and verification details. |
| Servo Gapper | A virtual device that creates gap. It is composed of a servo-controlled (variable speed) belt feeding a fixed-speed takeaway belt. A gap is created by slowing the servo belt. |
| Shifter | A shifter is a virtual device created from a belt and an encoder. It is used to track the position of a container. |
| Shifter Count | The Shifter Count is the position of the container in encoder ticks from the start of the belt/shifter. |
| Sorter | The Sorter requests events from a Shifter based on a Container’s position. A Shifter will track a Container on each tick. A Sorter will take action on a Container at certain pre-defined important positions along a Shifter. |
| Tick | A pulse from an Encoder. Each Tick represents distance moved along a Shifter. |

# High-Level Summary

The goal of the Print Apply Fallback design is to create a fault-tolerant printing system.  The system consists of a sorter, one or more print lines, and a controlling AC.  Each print line contains a Gapper, a Scanner, more than one Printer, and a controlling AC.  The Print Apply system will be able to fully utilize all the printers at full speed during normal operation.  If a printer fault occurs, the Print Apply Fallback module will reduce the volume of product flowing on to the line and adjust the Gapper so the remaining printers will operate correctly.

# Example Conveyor System Layout

This abstract example layout shows a section of a sorter feeding three Print and Apply lines, each of which has three printers.  The sorter is controlled by one AC, and each Print and Apply line has its own AC.

Sorter

Printers

Scanner

Gapper

AC

AC

AC

AC

# Process Flow

# Sorter Release

During normal operation, the Sorter will release a set volume of product into each Print Apply Line.  In the event of a printer failure, the Gapper is adjusted to increase the gap between containers.  This will slow the uptake of the Print Apply Line.  To prevent excessive recirculation on a Sorter, the Sorter’s input gap can also be increased.  The Capacity Gapper Module is used to adjust the gap of the Sorter.

The Print Apply Fallback module, described in this document, is used to adjust the gap of a single Print Apply Line.

# Print Apply Line

Each Print Apply line is composed of a Gapper, an LPN Scanner, a printer, and a Verification Scanner.  A local error-correction lane may also be included, or errors may be handled in a common location at a down-stream sorter lane.  The Print Apply Fallback module alters the behavior of the Gapper based on the number of active printers.

# Gapper

The Print Apply Fallback module creates a gap or pitch based on the number of working printers.  Table PrintApplyFallback is used to define the gap needed for each possible configuration of available printers.

The LPN Scanner is located after the Gapper and will scan each container.  The data is sent to the Print Apply Engine to determine which label should be applied by the printer.  The time required to scan a container, communicate with the Print Apply Engine, and send the data to the printer will determine the distance required between the Scanner and the first Printer.

# Printer

The Printer applies the label specified by the Print Apply Engine.  A printer fault will cause the Print Apply Fallback module to adjust the Gapper configuration.  If all available Printers become unavailable, the Print Line will stop.

# Feature Configuration

This section describes the configuration needed to activate and use the Print Apply Fallback feature.

# Table Batches

Activate the Print Apply Fallback feature by adding an entry in the Batches table with Process = CTRL and InternalProcCtrl =  “PAFB  CTRL”.

NOTE:  The PrintApplyFallback_pStatus column is updated by the software even if the Batches entry is not present or not activated!  Always check that the Batches entry is active!

# Table PrintApplyFallback

The PrintApplyFallback table is found under Configuration Menu -> PrintApply Utilities -> Print Apply Fallback.

| PrintApplyFallback Table |  |  |
| --- | --- | --- |
| Field | Type | Value or Description |
| Name | String | A descriptive name for the entry. Each entry is associated with a Print Apply Line and a particular number of printers. For example: PNA1-2PRT for line PNA1 with two printers. |
| PA Name | PopUp | Select the Print Apply Line associated with this Fallback entry. |
| Number Printers | Int | The number of active printers that this record defines. Each Print Apply Line will require at least one more record in this table than the number of printers on the line. For example, if there are two printers, three records are required in this table: Number Printers – 0 (Both printers are offline) Number Printers – 1 (One printer is offline) Number Printers – 2 (Both printers are running) If Pass Through Mode is to be used, additional records will be needed. |
| PassThru | Y/N | Set to “Y” to designate this entry for use with Pass Through Mode. Set to “N” to designate this entry for use when Pass Through Mode is not active. For use with PrintApplyLine_PassThroughModeEnable. |
| pStatus | Radio | The software sets this column to indicate which row is being actively used. Note that this will occur even if the Batch job PAFB CTRL is not active! |
| pDown Printer | PopUp | The software sets this field to call out, by name, one printer that is marked as out-of-service to aid the operator in restoring service. The name will be displayed in the highest Number Printers row. If more than one printer is out of service, each one will appear here as the faults are corrected. |
| New Gapper | PopUp | If the Print Apply Line uses a “New Gapper” for gapping, select the New Gapper record to associate with this line. All rows for the same Print Apply Line must have the same New Gapper record selected or an error will occur. |
| Servo Gapper | PopUp | If the Print Line uses a “Servo Gapper” for gapping, select the Servo Gapper record to associate with this line. All rows for the same Print Apply Line must have the same Servo Gapper record selected or an error will occur. |
| Gap Time | Time | This field indicates the gap time for a New Gapper. Set the time to the value needed for the number of printers defined in Number Printers. When this record is activated, this value will be used to set the Preset_Value for the NewGapper_MinGapTM timer for the associated New Gapper. |
| Pitch Time | Time | This field indicates the pitch time for a New Gapper. Set the time to the value needed for the number of printers defined in Number Printers. When this record is activated, this value will be used to set the Preset_Value for the NewGapper_MinPitchTM timer for the associated New Gapper. |
| Gap Ticks | Float | This field indicates the gap, in ticks, for a Servo Gapper. Set the value to the gap needed for the number of printers defined in Number Printers. When this record is activated, this value will be used to set ServoGapper_Gap_Ticks for the associated Servo Gapper if ServoGapper_PitchFlag = N. |
| Pitch Ticks | Float | This field indicates the pitch, in ticks, for a Servo Gapper. Set the value to the pitch needed for the number of printers defined in Number Printers. When this record is activated, this value will be used to set ServoGapper_Pitch_Ticks for the associated Servo Gapper if ServoGapper_PitchFlag = Y. |
| IO Out | PopUp | Use this to generate an output signal based on the fallback state. In systems where there are multiple Print Apply Lines fed from a single sorter, this can be used with the Capacity Gapper to adjust the volume fed from the sorter into each Print Apply Lane. Select an IO part (usually a memory bit) that will be turned ON when the number of available printers is greater than or equal to the Number Printers value. The IO will be turned OFF when the available printer count is less than the total printers available. The number of available printers is determined from PrinterBoss_Enabled, PrinterBoss_AvailAssign, and PrinterBoss_PrinterErrorIn. Example: A PNA Line has 2 Printers (3 records configured) 2 Printers are in use – All IO Out signals are turned ON. 1 Printer is in use – The IO Out signals in the rows for Number Printers = 0 and 1 are ON. The IO Out signal in the row for Number Printers = 2 is OFF. 0 Printers are in use – The IO Out signal in the row for Number Printers = 0 is ON. The other two are OFF. |
| pConfError | Radio | This field is set by the software to indicate an error is occurring. Possible errors include: - Multiple Print and Apply Line names. - Missing rows for Number Printers. Rows above a gap in the number of printers sequence will be marked invalid. - Different New Gappers or Servo Gappers have been selected for a print line. - A New Gapper and a Servo Gapper have been selected. |
| pError Message | String | This field is updated by the software with a description of the error detected. |

# Example Configuration

Print Apply Line PNA1 has two printers and uses a New Gapper for gapping.

Table Batches:

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | How Often | Incre-ment | Type | Process | Internal Proc | Internal Proc Ctrl | Machine | All Machines | Do File |
| PAFB1 | 1 | Invalid | Count | CTRL |  | PAFB CTRL | PNA1 | N | N |

The fallback behavior will not occur if the Batch is not active!

Table PrintApplyFallback:

| Print Apply Fallback (Part 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | PA Name | Number Printers | Pass Thru | New Gapper | Servo Gapper | Gap Time |
| PNA1-0PRT | PNA1 | 0 | N | PNA1 Gapper | N/A | 6.00 |
| PNA1-1PRT | PNA1 | 1 | N | PNA1 Gapper | N/A | 6.00 |
| PNA1-2PRT | PNA1 | 2 | N | PNA1 Gapper | N/A | 3.00 |

| Print Apply Fallback (Part 2) |  |  |  |
| --- | --- | --- | --- |
| Pitch Time | Gap Ticks | Pitch Ticks | IO Out |
| 6.00 | 0 | 0 | INVALID |
| 6.00 | 0 | 0 | MEM_1_PRINTERS |
| 3.00 | 0 | 0 | MEM_2_PRINTERS |

The Gap and Pitch Times in the PNA1 Gapper will be adjusted as the number of available printers changes.  The IO Out memory bits could be used with a Capacity Gapper on the sorter feeding this line to adjust the volume of cartons sent to this Print Apply Line.