# FortnaPlus Control Print and Apply Simulation

Document Revision: 1.1

Date: 07-June-2023

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 |  | B C Conklin | New document |
| 2.0 | 01-Apr-2014 | B C Conklin | Updates |
| 1.1 | 07-Jun-2023 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	5

1.	Introduction	6

2.	The Print and Apply Process	6

2.1.	Skew Section	7

2.2.	Brake / Meter Belt	7

2.3.	Induct Photoeye	7

2.4.	LPN / SKU Scan Tunnel	7

2.5.	Container Dimensioning	8

2.6.	Label Request Message	8

2.7.	Label Printer Assignment	8

2.8.	Verification Scan Tunnel	9

2.9.	Label Verification Message	9

3.	PNA Simulation Menus and Tables	9

3.1.	PNA Simulation Main Menu	10

3.2.	PASimBoss Table	10

3.3.	PASimScans Table	11

3.4.	PA Sim Wave Data Menu (PASimWaveScans Tables)	12

3.5.	PA Wave Data Msg Menu	13

3.6.	PASimLabels Table	13

3.7.	PASimCartons Table	13

3.8.	PASimTrkTable1	14

3.9.	PNASimScrn	16

4.	PNA Configuration for Simulation Testing	18

4.1.	Machine Table	18

4.2.	MsgMap Table	19

4.3.	Hosts File	20

4.4.	Conveyor Table	20

4.5.	Batches Table	21

4.6.	Sorters Table	21

4.7.	Inpoints Table	22

4.8.	Outpoints Table	22

4.9.	Updates Table	23

4.10.	PrintApplyLine Table	23

4.11.	PrintBelt Table	24

4.12.	PrinterBoss Table	25

4.13.	PrinterUpdPnts Table	25

4.14.	PrinterErrConfig Table	26

4.15.	ScnScanZone Table	26

4.16.	ScnScanDevice Table	27

4.17.	HeightWidth Table	27

4.18.	PAScannerBoss Table	28

4.19.	PAPrinterScanAssign Table	29

4.20.	LabelTemplates Table	29

5.	PNA Simulation Configuration	30

5.1.	PASimCartons Table	30

5.2.	PASimWaveScans Tables	31

5.3.	PASimBoss Table	32

6.	Running The Simulations	34

7.	WSD Message	35

7.1.	WSD Machine Table Entries	37

7.2.	WSD MsgMap Table Entries	38

7.3.	WSD Message Client Connections	38

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

CommCore – The Fortna software package that provides container routing information.

Container, Carton, Tote – The items transported by the conveyor system.

FPC – FortnaPlus Control.  The machine-control software package that is described in this document.  Runs on QNX.

LMI – Label Image Message.  A message from the PandA engine software to FortnaPlus that contains the label data to be transmitted to a printer.

LPN – License Plate Number.  A unique identifier for a container on a conveyor system.

LRM – Label Request Message.  A message initiated by FPC to request the labeling requirements and verification information for a container based on its LPN.

LVM – Label Verification Message.  A message from FPC to the PandA engine that indicates the verification status of a container.

PandA Engine – The Fortna software package that FPC interacts with to obtain container labels and verification data.

PNA, PandA – Common abbreviations for “Print and Apply.”

QNX – The operating system that hosts FPC.

SKU – Stock Keeping Unit.  An identification number for a particular type of item in inventory.

WSD – Wave Simulation Data.  A message that supplies simulation data for a virtual container.

ZPL – Zebra Programming Language.  A programming language for defining label images and commanding Zebra label printers.

# Introduction

This document describes how to use the Print and Apply Simulation feature in FortnaPlus.

Refer to the FPC-PNA-Print-Apply-Configuration and FPC-PNA-ConfigurationChecklist documents for details on configuring a PNA line.

The FortnaPlus Print-and-Apply Simulator can be used to simulate any of the following aspects of the Print and Apply process:

- Container induction onto a shifter

- LPN/SKU scan data

- LRM from the PandA engine

- LMI from the PandA engine

- Verification scan data

- LVM handling

The simulator can be configured to simulate the entire process or just certain desired components.

# The Print and Apply Process

Use this operational overview of a PNA line to help understand the aspects that can be simulated.

## Skew Section

The skew section is a passive component formed by mounting rollers at an angle to drive the containers to one side of the conveyor and align them with that edge.  This helps ensure consistent and accurate label placement.

The skew section has no impact on simulation.

## Brake / Meter Belt

The Brake / Meter Belt section generates appropriate gaps between containers so the downstream printer(s) will have sufficient time for label processing and application.  The required gap is dependent upon the number of printers available.  The Brake / Meter Belt configuration can be dynamically adjusted in response to changes in printer availability using standard gapper functionality.

These adjustments are made for real-world use and for simulation.

## Induct Photoeye

As a container reaches the PNA belt, it blocks the induction photoeye.  When this occurs, the container is inducted into the system, triggering the creation of sorter and PNA tracking records.

For simulation, the induction photoeye input is controlled (switched ON and OFF) by the simulator to generate virtual containers on the PNA shifter.

## LPN / SKU Scan Tunnel

Each container must be identified to determine the actions the PNA line will take on it.  Many systems require only a single barcode to be read, but more elaborate arrangements are supported – often referred to as a “scan tunnel”.  The LPN / SKU Scan Tunnel scans the container for barcodes and reports them to FPC.  The scan tunnel can include a variety of scanners.  PNA systems may be designed to work with either LPN (unique) or SKU (non-unique or product-type) identifiers for the containers.  The difference is mostly in how the PandA engine determines label requirements and keeps track of containers that have been processed.

For simulation, simulated scans are inserted to mimic real scanners.

## Container Dimensioning

To protect downstream equipment and calculate accurate printer tamp points, each container’s height must be known (For top-apply at least.  More accurately, the distance the tamp head must travel to apply the label must be known).  Dimension data can be supplied via height sensing equipment or via container dimensioning devices.

Dimension information can also be supplied for simulation.

## Label Request Message

Upon receipt of scan data, FPC transmits a Label Request Message (LRM) to the PandA engine, which will respond with an LRM reply that includes the label type to be assigned and the verify string for the label that is about to be applied.  If the label is supplied by the PandA engine (it usually is), a Label Image Message (LMI) is also sent to FortnaPlus which includes data for the label printer.  The label image is often in ZPL format, but FPC will accept any printer data and pass it along, unaltered, to the printer at the appropriate time.

When simulating the PandA engine, the simulator creates an LRM message from label data retrieved from simulation records (described later in this document) and inserts them into the appropriate message table.

## Label Printer Assignment

Upon receipt of the LRM message from the PandA engine, FPC will determine which downstream printer will be assigned to print the container’s label.  The system can assign labels to one or more printers using three different algorithms:

- Static Assign – The label is assigned to a specified printer.  This might be useful if a label must be applied in a certain orientation, or if the label must be printed on a different label stock.

- Round-Robin – The label is assigned to one of multiple printers on a Round-Robin load-sharing basis, with the label type determining the next printer selected.  This algorithm can result in the same printer receiving consecutive assignments.  Multiple label types are supported, with each label type potentially having multiple printers available, with round-robin assignments being made within each label type group.

- Line Round-Robin – The label is assigned to one of multiple printers on a Round-Robin load-sharing basis, regardless of label type.  (Not very useful!)

This is the same for real-world use or for simulation.

## Verification Scan Tunnel

The Verification Scan Tunnel will scan the container to determine if the correct label was (or labels were) applied to the container.  Verification results are linked to the printer that generated each label, so specific printers can be removed from service automatically if they are generating errors.

When simulating verification scan data, the simulator will retrieve the appropriate test scan data (described later in this document) and insert it into the scan device/zone record(s) to simulate the receipt of scan data.

## Label Verification Message

The result of the Verification Scan (pass or fail) is reported to the PandA engine in the form of a Label Verification Message (LVM) along with other information such as:

- LPN / SKU scan data

- Container Dimensions (height, width, length)

- Verification scan data

- Container weight (if applicable)

Success / Fail codes, Print-and-Apply verification status (message fields sStatus[1-6]):

| Status Code | Description | Internal Identifier |
| --- | --- | --- |
| 0 | Label not verified | VERIFYSTATUS_OFF |
| 1 | Label verified successfully | VERIFYSTATUS_OKAY |
| 2 | Label mismatch | VERIFYSTATUS_MISSERROR |
| 3 | Verify scan error | VERIFYSTATUS_SCANERROR |
| 4 | Not verified due to prior verify failure | VERIFYSTATUS_NOTVERIFIED |

For simulation, the PandA engine simulation accepts and acknowledges the LVM.

# PNA Simulation Menus and Tables

The simulator uses special menus and tables, as described in this section.

## PNA Simulation Main Menu

You will find the PNA Simulation tables in a sub-menu of the PrintApply Utilities menu at Configuration Menu | PrintApply Utilities | Print Apply Simulation.  The tables are described in the following sections.

## PASimBoss Table

The PASimBoss table (Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Boss) is the top-level table for the simulation configuration.

| PASimBoss Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the PNA simulation. The simulator can accommodate up to five simulation configurations. |
| PALine | PopUp | Choose the PNA line that the simulation will act upon. Each simulation must be linked to its own separate PNA line. |
| Shifter | PopUp | Choose the simulation’s shifter. The PNA simulation process is shifter-based and must be associated with an existing shifter. |
| CartonGenerator | PopUp | If the simulation will generate virtual containers, use this field to choose the carton generator from the PASimCartons table. |
| SimMachineName | String | Set the machine name for use with the simulated LRM and LVM messages. If this is left blank, the FPC name of this machine is used. |
| SimAutoRestart | Radio | If this is selected, when the end of the simulation data is reached, it will ‘wrap around’ and continue from the beginning. This allows a small set of simulation data values to be used with a large set of virtual containers. |
| SimScanType | PopUp | When simulating scan data, use this field to select the method: Static – Retrieve simulation scan data from a pre-constructed list. Dynamic – Create simulation scan data from a set of instructions defined in the PASimScans table. |
| SimScanMenu | PopUp | When using SimScanType = Static, choose the table that contains the scan data. Use tables PASimWaveScans1 through PASimWaveScans5 from the PA Sim Wave Data menu for this purpose. |
| SimLevel2 | Radio | If this is selected, the PandA engine is simulated by generating LRM, LMI and LVM messages. |
| StaticSimScnIndx | Int | When using SimScanType = Static, the program will use this field to record the current index of the row in the table selected in SimScanMenu. |
| LPNScanZone | PopUp | When simulating LPN scan data, use this field to select the scan zone that will receive the simulated scans. |
| LPNScanDevice | PopUp | When simulating LPN scan data with a simple scan, you may use this field to select the scan device directly rather than using the scan zone. |
| LPNScanPoint | PopUp | Choose the update point associated with the LPN scan. |
| VerifyScanZone | PopUp | When simulating verification scan data, use this field to select the scan zone that will receive the simulated scans. |
| VerifyScanDevice | PopUp | When simulating verification scan data with a simple scan, you may use this field to select the scan device directly rather than using the scan zone. |
| VerifyScanPoint | PopUp | Choose the update point associated with the verification scan. |
| ScaleScanDevice | PopUp | When simulating weights, use this field to select the scale device. |
| ScaleScanPoint | PopUp | Choose the update point associated with the scale. |
| pStart | Int | Set by the software when the simulation is started. |
| pStop | Int | Set by the software when the simulation is stopped. |
| pReset | Int | Set by the software when the simulation is reset. |
| pRunning | Int | Set by the software while the simulation is running. |
| pGoodVerifyCount | Int | Set by the software to track the number of good verifications while the simulation is running. |
| pBadVerifyCount | Int | Set by the software to track the number of verification failures while the simulation is running. |
| pClearWaveData | Int | Used by the software when clearing simulation wave data. |
| pResetCounts | Int | Used by the software when resetting the pGoodVerifyCount and pBadVerifyCount fields. |

## PASimScans Table

The PASimScans table (Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Scans) is used to define how to dynamically generate simulation scan data for the simulator.

| PASimScans Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this data generation record. |
| Desc | String | Enter an optional description. |
| PASimBoss | PopUp | Choose the PASimBoss record associated with this record. |
| ScanZone | PopUp | Choose the scan zone that will receive the data from this record. |
| ScanDevice | PopUp | For single-scanner configurations, you may choose the ScnScanDevice record for the LPN scanner here instead of selecting the scan zone. |
| ScanPrefix | String | Enter an optional literal string that will start all scan strings. |
| Field1Type – Field5Type | PopUp | Use the five field type fields to define up to a five-part scan string. You can mix and match field types as needed for multi-part scans. There are two options: Static – The literal strings found in the Field[1-5]Literal fields are used. Dynamic – The data for the scans are defined by the Field[1-5]Auto fields. |
| Field1Literal – Field5Literal | String | The strings in these fields are used exactly as entered when Field[1-5]Type = Static. |
| Field1Auto – Field5Auto | PopUp | Use these fields to auto-generate up to five elements of the simulated scan when Field[1-5]Type = Dynamic. Choose the data you would like to generate for each element: Carton ID – A ten-character numeric string padded with leading zeros. The simulation will start with ‘0000000001’ and increment by 1 for each container. Carton Height – The container’s height. Carton Width – The container’s width. Carton Length – The container’s length. Carton Verify – The container’s verify string as supplied by the LRM. |
| ScanPostfix | String | Enter an optional literal string that will end all scan strings. |
| FieldDelim | String | Optional. Enter a single character to be inserted between the fields. |

## PA Sim Wave Data Menu (PASimWaveScans Tables)

Five tables are available for simulation data:  PASimWaveScans1 – PASimWaveScans5.  They can be accessed from Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Wave Data.  Each simulation can reference one of these tables to retrieve container simulation data.  Each record defines all the data needed for a container’s transit through the PNA line.  You can populate these tables manually, or by using WSD messages, as described in section 7, below.  All the tables have the same structure:

| PASimWaveScans[1-5] Tables |  |  |
| --- | --- | --- |
| Field | Type | Description |
| CartonID | String | Enter the container ID string. This could be the same as LPNScan. |
| LPNScan | String | The LPN or SKU scan string scan, as specified in PASimBoss_LPNScanZone or PASimBoss_LPNScanDevice. |
| LabelType | String | The label type string to be supplied in the LRM message if simulating the PandA engine connection. |
| VerifyString | String | The verify string to be supplied in the LRM message if simulating the PandA engine connection. If VerifyString and VerifyScan match, the verification test will pass. To test verification failure behavior make them differ. |
| VerifyScan | String | The verify scan string, as specified in PASimBoss_VerifyScanZone or PASimBoss_VerifyScanDevice. |
| ScaleData | String | The weight value, as specified in PASimBoss_ScaleScanDevice. |

## PA Wave Data Msg Menu

Nine tables are available for WSD message data:  MsgWSD1 - MsgWSD9.  They can be accessed from Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Wave Data Msg.  These tables are used to receive WSD messages that can optionally be used to populate the PASimWaveScans* tables automatically.  See section 7, below.

## PASimLabels Table

The PASimLabels table is reserved for future use.

| PASimLabels Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String |  |
| Desc | String |  |
| LoadFile | Radio |  |
| Status | String |  |
| LabelNum | PopUp |  |
| LabelType | PopUp |  |
| ExtLabelType | String |  |
| LabelFileName | String |  |
| ScanDataString | String |  |
| pLastUsed | Time |  |
| pNumBytes | Int |  |
| pTimeStamp | Time |  |
| pLabelImg | Blob | 20,000 characters by default. |

## PASimCartons Table

The PASimCartons table is used to generate simulated containers on a specific shifter.

| PASimCartons Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the record. |
| Desc | String | Enter an optional description. |
| Enabled | Radio | Use this to enable or disable this record. |
| Status | String | Set by the software to indicate the state of this record. |
| SimCartonCount | Int | Use this field to define how many simulated containers to generate. A value of 0 indicates an unlimited number and will cause the container generator to run until manually stopped. |
| SimulateApplicator | Y/N | When set to Y, the hardware signals, such as the PrinterBoss LabelOnTampIn and the TamperSolOut output are simulated. |
| CreateSorterTmr | Y/N | When set to Y, a timer will be automatically created in table Sorters, field Encoder tmName for use in the simulation. |
| StopOnError | Y/N | When set to Y, the timer in table Sorters, field Encoder tmName will halt with the simulation when an error occurs, so additional simulation containers are not created. |
| LengthMin | Int | Set the minimum and maximum length of a container, in ticks. If set to the same size, all containers will be generated at that size. If set to different sizes, the first container will be LengthMin ticks long. Subsequent containers will increase in length by one tick until LengthMax is reached, then begin again at LengthMin. |
| LengthMax | Int |  |
| SpacingMin | Int | Set the minimum and maximum gap between containers, in ticks. If set to the same size, all gaps will be generated at that size. If set to different sizes, the first gap will be SpacingMin ticks long. Subsequent gaps will increase in length by one tick until SpacingMax is reached, then begin again at SpacingMin. |
| SpacingMax | Int |  |
| PALine | PopUp | Choose the Print Apply Line that will receive the simulated containers. |
| SimShifter | PopUp | Choose the shifter that will receive the simulated containers. |
| InPoint | PopUp | Choose the induction point where the simulated containers will be created. |
| pCartonIndex | Int | While the carton generator is running, this field shows the index of the most recently generated container. If SimCartonCount is greater than zero, the carton generator will halt when that value is reached. |
| pCartonLength | Int | The length of the most recently generated container, in ticks. |
| pCartonSpacing | Int | The space between the two most recently generated containers. |
| pCurrLength | Int | While the carton generator is running, this field shows the length of the container as it is being generated, in ticks. |
| pCurrGap | Int | While the carton generator is running, this field shows the space between containers as it is being generated, in ticks. |
| pLastShiftCnt | Int | The current shift count for the shifter associated with this record. |
| pCartonCount | Int | The running count of containers generated since the last reset. |
| pStateForcePE | Int | Used by the software to track the state of the induction photoeye. |

## PASimTrkTable1

As simulated containers are generated, special tracking records for the simulation are created in table PASimTrkTable1.  Each tracking record contains all the information needed to move that container through the entire PNA line.

| PASimTrkTable1 Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Optional name for the simulation tracking record. |
| CartonIndex | Int | An index integer value assigned by the simulator. After a reset, this will start at 1, and increment with each new simulated container. |
| CartonID | String | The ID string for the simulated container. It is the CartonIndex value in string form, padded with leading zeros. |
| SortMnuIndx | Int | The table index number (from F2-Menu-Of-Menus) of the sorter tracking table associated with this simulation. |
| SortTrkIndx | Int | The row number in the table pointed to by SortMnuIndx for this container. |
| CartonHeight | Float | The container’s simulated height. At induction, a default value is inserted from HeightWidth_BoxHgtInches1. When real or simulated height data is received, it overwrites the default. |
| CartonWidth | Float | The container’s simulated width. At induction, a default value is inserted from HeightWidth_BoxWdtInches1. When real or simulated width data is received, it overwrites the default. |
| CartonLength | Float | The container’s simulated length. At induction, a default value is inserted from HeightWidth_BoxWdtInches1 (yes, the width). When real or simulated length data is received, it overwrites the default. |
| LPNScanZone | PopUp | The LPN scan zone. |
| LPNScanDevice | PopUp | The LPN scan device (if a simulation scan zone is not defined). |
| LPNScanData | String | The LPN scan data to be inserted when the simulated container reaches the scan point defined in PASimBoss_LPNScanPoint. |
| LabelType | String | The label type string, if defined in the PASimWaveScans tables. |
| LabelVerifyString | String | The label verification string, if defined in the PASimWaveScans VerifyString fields. |
| SimLabelIndex | PopUp | Part of the unfinished PaSimLabels feature. |
| VerifyScanZone | PopUp | The verification scan zone. |
| VerifyScanDevice | PopUp | The verification scan device (if a simulation scan zone is not defined). |
| VerifyScanData | String | The verification scan data to be inserted when the simulated container reaches the PASimBoss_VerifyScanPoint. |
| ScaleScanDevice | PopUp | The scale scan device. |
| ScaleScanData | String | The scale data to be inserted when the simulated container reaches the PASimBoss_ScaleScanPoint. |
| TimeStamp | Time | The timestamp for this record’s creation. This is used to find the next available record. |
| FixedVScan | Int | This flag is set by the software if a verification string is defined in the PASimWaveScans tables in field VerifyString so it will be used instead of a generated verification scan string. Used to create verification scan failure test scenarios. |

## PNASimScrn

The Print and Apply Simulation Operator’s Screen, or PNASimScrn, is used to control and adjust simulations.

| PNASimScrn |  |  |
| --- | --- | --- |
| Field | Type | Description |
| PNA Sim Boss | PopUp | Choose the simulation you wish to control from table PNASimBoss. |
| Auto Restart | Radio | This button controls the PNASimBoss_SimAutoRestart state. It determines what will happen when the end of the simulation scan list is reached. ON = Wrap to the start of the simulation scan list and continue. OFF = Stop the simulation. |
| Start Simulation | Radio | Start the currently-selected simulation. |
| Stop Simulation | Radio | Stop the currently-selected simulation. NOTE: Before stopping a simulation, first click the Stop Carton Sim button to halt the creation of virtual containers. Once the last container has completed its transit through the PNA line, the simulation can be stopped here without any errors. If the simulation is stopped while containers are still on the shifter, errors may occur in the PNA processing as there may still be outstanding events for the simulation to process. |
| Reset Simulation | Radio | Reset the simulation counters: Sim Carton Count, Good Verify Count, Bad Verify Count, Total Good Verify, Total Bad Verify. |
| Simulation Running | Radio | Set by the software to show the current state of the simulation. |
| Start Carton Sim | Radio | Enable the container simulation. |
| Stop Carton Sim | Radio | Disable the container simulation. |
| Sim Carton Count | Int | Shows the number of containers that have been simulated since the last reset. |
| Good Verify Count | Int | Shows the number of containers that have been successfully verified in the currently-selected simulation since the last reset. |
| Bad Verify Count | Int | Shows the number of containers that have failed verification in the currently-selected simulation since the last reset. |
| Clear Track Data | Radio | Clears the following tables related to the currently-selected simulation: - Sorter tracking table for the LPN scan zone. - Sorter tracking table for the verification scan zone. - Device tracking table for the LPN scan zone. - Device tracking table for the verification scan zone. - Print Apply tracking table for the LPN scan zone. - Print Apply tracking table for the verification scan zone. |
| Start ALL | Radio | Start all configured simulations at once. |
| Stop ALL Cartons | Radio | Stop all virtual container generation for all configured simulations. |
| Stop ALL | Radio | Stop all active simulations |
| Reset ALL | Radio | Reset all configured simulations. |
| Clear ALL | Radio | Clears the tables shown in Clear Track Data for all configured simulations, plus table PASimTrkTable1. |
| Total Good Verify | Int | Shows the number of containers that have been successfully verified in all configured simulations since the last reset. |
| Total Bad Verify | Int | Shows the number of containers that have failed verification in all configured simulations since the last reset. |
| Scan Device Sim | PopUp | Choose the scan device from ScnScanDevice that will receive simulated scans. |
| Sim Scan Data | String | Enter a string to be used as the scan value for the Scan Device Sim scanner. |
| Sim Scan | Radio | Use this button to simulate a scan on the Scan Device Sim scanner by inserting the string from Sim Scan Data. Use this to simulate error conditions such as improperly formatted data, scans received outside the scan window, etc. |
| Next Carton LPN No Scan | Radio | Use this button to create a NoScan event for the next simulated container moving through the LPN scan zone. |
| Next Carton LPN No Read | Radio | Use this button to create a NoRead event for the next simulated container moving through the LPN scan zone. |
| Next Carton Verify No Scan | Radio | Use this button to create a NoScan event for the next simulated container moving through the verification scan zone. |
| Next Carton Verify No Read | Radio | Use this button to create a NoRead event for the next simulated container moving through the verification scan zone. |

# PNA Configuration for Simulation Testing

In this section, we will define a PNA configuration that will serve as the basis for the simulation we will create in the following section.  It will represent two independent lines.  Each will have two equivalent printers that will act in a load-sharing round-robin fashion for each line.  This example assumes no prior configuration (starts from a ‘blank’ FPC instance) and entails a significant amount of manual entry.  A pre-configured version can be found in the file PASIM1-RUN.tar.gz on Redmine along with this document.  It is a complete FPC configuration for an AC called PASIM1.  You will still need to make the required changes in your /etc/hosts file!

## Machine Table

Open the Machine table (Configuration Menu | Communication Utilities | Machine) and add the following records to define the connections for LRM and LVM messages for two PNA lines.  These are the ‘normal’ connections:

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine _Name | Connection _Type | Communication _Protocol | Machine _Port | Transmit _Retry | Process | Log Level |
| LRM_CL1 | CONNECT _TCPCLIENT | PaComm | 9002 | 10 | SERV | Level 1 |
| LVM_CL1 | CONNECT _TCPCLIENT | PaComm | 9003 | 10 | SERV | Level 1 |
| LRM_CL2 | CONNECT _TCPCLIENT | PaComm | 9012 | 10 | SERV | Level 1 |
| LVM_CL2 | CONNECT _TCPCLIENT | PaComm | 9013 | 10 | SERV | Level 1 |

Now, define the simulation connections to interact with the ‘normal’ connections.  These server-style connections mimic the PandA Engine:

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine _Name | Connection _Type | Communication _Protocol | Machine _Port | Transmit _Retry | Process | Log Level |
| LRM_SIMLS1 | TCPSERV _LISTEN | PaTest | 9002 | 10 | SERV | Level 1 |
| LRM_SIMRW1 | TCPSERV _RDWR | PaTest | 9002 | 10 | SERV | Level 1 |
| LVM_SIMLS1 | TCPSERV _LISTEN | PaTest | 9003 | 10 | SERV | Level 1 |
| LVM_SIMRW1 | TCPSERV _RDWR | PaTest | 9003 | 10 | SERV | Level 1 |
| LRM_SIMLS2 | TCPSERV _LISTEN | PaTest | 9012 | 10 | SERV | Level 1 |
| LRM_SIMRW2 | TCPSERV _RDWR | PaTest | 9012 | 10 | SERV | Level 1 |
| LVM_SIMLS2 | TCPSERV _LISTEN | PaTest | 9013 | 10 | SERV | Level 1 |
| LVM_SIMRW2 | TCPSERV _RDWR | PaTest | 9013 | 10 | SERV | Level 1 |

Note:  Be sure to restart FPC after adding Machine table records!

## MsgMap Table

Open the MsgMap table (Configuration Menu | Communication Utilities | Message Map) and add the following records to define the connections from the Machine entries to the message tables for the ‘normal’ PNA configuration:

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message _Name | Machine _Name | Recv _Msg _Type | Send _Msg _Type | Ack _Msg _Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| LRM_CL1 | LRM_CL1 | PASIM1 | PASIM1 | PASIM1 | MsgCR1 | 1 | 66 | 67 | 68 | 0 |
| LVM_CL1 | LVM_CL1 | PASIM1 | PASIM1 | PASIM1 | MsgDC1 | 1 | 66 | 67 | 68 | 0 |
| LRM_CL2 | LRM_CL2 | PASIM1 | PASIM1 | PASIM1 | MsgCR2 | 1 | 66 | 67 | 68 | 0 |
| LVM_CL2 | LVM_CL2 | PASIM1 | PASIM1 | PASIM1 | MsgDC2 | 1 | 66 | 67 | 68 | 0 |

Now, create the connections for the simulation machines:

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message _Name | Machine _Name | Recv _Msg _Type | Send _Msg _Type | Ack _Msg _Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| LRM_SIMRW1 | LRM_SIMRW1 | PASIM1 | PASIM1 | PASIM1 | MsgGP1 | 1 | 66 | 67 | 68 | 0 |
| LVM_SIMRW1 | LVM_SIMRW1 | PASIM1 | PASIM1 | PASIM1 | MsgGP2 | 1 | 66 | 67 | 68 | 0 |
| LRM_SIMRW2 | LRM_SIMRW2 | PASIM1 | PASIM1 | PASIM1 | MsgGP3 | 1 | 66 | 67 | 68 | 0 |
| LVM_SIMRW2 | LVM_SIMRW2 | PASIM1 | PASIM1 | PASIM1 | MsgGP4 | 1 | 66 | 67 | 68 | 0 |

## Hosts File

The /etc/hosts file defines the IP addresses associated with the names from Machine_Machine_Name for client connections.  In this case, we want to create a ‘loop back’ connection to our FPC instance for simulation.  Add a line to /etc/hosts using the editor of your choice:

127.0.0.1	LRM_CL1 LVM_CL1 LRM_CL2 LVM_CL2

Check the Machine_Machine_Pid column.  Established connections will have a large integer value.  A value of -1 indicates the connection has not been established.

## Conveyor Table

Use Configuration Menu | Wizard Utilities | Create Conveyor Part to create parts needed for the simulation.  You will need two memory words.  The Word:Bit values are arbitrary.  The parts reside in table Conveyor (Configuration Menu | Standard Controls | IO Conveyor).

| Part Name | IO Word:Bit |
| --- | --- |
| SimInduct01 | 200:0 |
| SimInduct02 | 200:1 |
| EH1-1 | 200:2 |
| EH1-2 | 200:3 |
| EH2-1 | 200:4 |
| EH2-2 | 200:5 |
| PA1Mtr | 200:6 |
| PA2Mtr | 200:7 |
| PA1VerifyLight | 200:8 |
| PA2VerifyLight | 200:9 |
| PA1ResetLine | 200:10 |
| PA2ResetLine | 200:11 |
| PA1ClearLine | 200:12 |
| PA2ClearLine | 200:13 |
| PA1P1ErrIn | 200:14 |
| PA1P2ErrIn | 200:15 |
| PA2P1ErrIn | 201:0 |
| PA2P2ErrIn | 201:1 |
| PA1P1ErrLight | 201:2 |
| PA1P2ErrLight | 201:3 |
| PA2P1ErrLight | 201:4 |
| PA2P2ErrLight | 201:5 |
| PA1P1VerErr | 201:6 |
| PA1P2VerErr | 201:7 |
| PA2P1VerErr | 201:8 |
| PA2P2VerErr | 201:9 |

## Batches Table

Open the Batches table (Configuration Menu | Standard Controls | Batches).  Add or complete the configuration of the following six batches to enable the Print-and-Apply process and the simulation process.

| Batches Table |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| File_ Name | How Often | Increment | Type | Process | Internal Proc | Internal ProcCtrl | Machine | DoFile |
| Scan Process | 1 | Invalid | Count | CTRL |  | ScnAppCTRL | PASIM1 | N |
| PA Sort | 1 | Invalid | Count | SORT |  | PA SORT (CPN) | PASIM1 | N |
| PA Ctrl | 1 | Invalid | Count | CTRL |  | PA CTRL (CPN) | PASIM1 | N |
| PA Serv | 1 | Invalid | Count | SERV | PA SERV (CPN) |  | PASIM1 | N |
| PA GUI | 1 | Invalid | Count | GUI | PA GUI (CPN) |  | PASIM1 | N |
| PA Simulation | 1 | Invalid | Count | SORT |  | PNASim SORT | PASIM1 | N |

## Sorters Table

Each PNA simulation requires its own shifter.  Open the Sorters table (Configuration Menu | Sorter Utilities | Shifter Configurations).  Add the entries as show in the table below:

| Sorters Table (part 1) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sorter Name | Encoder tmName | Encoder Type | Max Cartons | Max Verify Buffer | OK | Machine | Data LowRec | Data High Rec |
| PA1 | tmPA1Enc | 1 | 100 | 100 | Y | PASIM1 | 1 | 500 |
| PA2 | tmPA2Enc | 1 | 100 | 100 | Y | PASIM1 | 1001 | 1500 |

| Sorters Table (part 2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Buffer LowRec | Buffer HighRec | Inpoints LowRec | Inpoints HighRec | Updates LowRec | Updates HighRec | Outpoints LowRec | Outpoints HighRec |
| 1 | 500 | 1 | 1 | 1 | 7 | 1 | 2 |
| 1001 | 1500 | 2 | 2 | 8 | 14 | 3 | 4 |

## Inpoints Table

Each PNA simulation shifter requires an induction point.  Create one for each of the two simulations in table Inpoints (Configuration Menu | Sorter Utilities | Induct Points), as shown:

| Inpoints Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inpoint Name | Sorter | Induct Number | Induct Location | Induct I/O Name | Trailing_ Edge_Pad | Minimum Gap | Longest Package |
| Inpoint1 | PA1 | 0 | 10 | SimInduct01 | 1 | 10 | 20 |
| Inpoint2 | PA2 | 0 | 10 | SimInduct02 | 1 | 10 | 21 |

## Outpoints Table

In table Outpoints (Configuration Menu | Sorter Utilities | Divert Points), we create two divert points at the end of each shifter:

| Outpoints Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Outpoint Name | Sorter | Outpoint Number | Outpoint Location | Outpoint I/O Name | Verify Location | Verify Offset |
| PA1_DV1 | PA1 | 0 | 410 | EH1-1 | 409 | -1 |
| PA1_DV2 | PA1 | 1 | 400 | EH1-2 | 399 | -1 |
| PA2_DV1 | PA2 | 0 | 410 | EH2-1 | 409 | -1 |
| PA2_DV2 | PA2 | 1 | 400 | EH2-2 | 399 | -1 |

| Outpoints Table (part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Verify I/O Name | VerifySENSE | Full_Clr_ Timer_Name | Trigger_ I/O_Name | LoLane | HiLane |
| EH1-1 | IsOn | tmALWAYSON | INVALID | 101 | 101 |
| EH1-2 | IsOn | tmALWAYSON | INVALID | 102 | 102 |
| EH2-1 | IsOn | tmALWAYSON | INVALID | 103 | 103 |
| EH2-2 | IsOn | tmALWAYSON | INVALID | 104 | 104 |

## Updates Table

In table Updates (Configuration Menu | Sorter Utilities | Update Points), we configure all the update points needed to trigger the various tasks for the two PNA lines:

| Updates Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Update Name | Sorter | Update Number | Location | Name | Description |
| PA1LPNScan | PA1 | 0 | 70 | INVALID | PA1-LPN-Scan |
| PA1P1L1Small | PA1 | 1 | 185 | INVALID | PA1-P1-L1-Small |
| PA1P1L1Large | PA1 | 2 | 190 | INVALID | PA1-P1-L1-Large |
| PA1P2L1Small | PA1 | 3 | 220 | INVALID | PA1-P2-L1-Small |
| PA1P2L1Large | PA1 | 4 | 226 | INVALID | PA1-P2-L1-Large |
| PA1VerScan | PA1 | 5 | 348 | INVALID | PA1-Ver-Scan |
| PA1Scale | PA1 | 6 | 355 | INVALID | PA1-Scale |
| PA2LPNScan | PA2 | 0 | 70 | INVALID | PA2-LPN-Scan |
| PA2P1L1Small | PA2 | 1 | 185 | INVALID | PA2-P1-L1-Small |
| PA2P1L1Large | PA2 | 2 | 190 | INVALID | PA2-P1-L1-Large |
| PA2P2L1Small | PA2 | 3 | 220 | INVALID | PA2-P2-L1-Small |
| PA2P2L1Large | PA2 | 4 | 226 | INVALID | PA2-P2-L1-Large |
| PA2VerScan | PA2 | 5 | 348 | INVALID | PA2-Ver-Scan |
| PA2Scale | PA2 | 6 | 355 | INVALID | PA2-Scale |

## PrintApplyLine Table

In table PrintApplyLine (Configuration Menu | PrintApply Utilities | Printer Apply Line), create two PNA lines:

| PrintApplyLine Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | AssignType | TestMode | SendType | LogLevel | ControlMachine | NumPrinters |
| PA1 | Line Round Robin | Sim Mode ON | Scan – Lbl Snd | Level 2 | PASIM1 | 2 |
| PA2 | Line Round Robin | Sim Mode ON | Scan – Lbl Snd | Level 2 | PASIM1 | 2 |

| PrintApplyLine Table (part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| LrmMsgTable | LvmMsgTable | TstMsgTable | RcvLabelTable | SndLabelQueue |
| MsgCR1 | MsgDC1 | MsgGP1 | RcvLabels1 | SndLabelQue1 |
| MsgCR2 | MsgDC2 | MsgGP2 | RcvLabels2 | SndLabelQue2 |

| PrintApplyLine Table (part 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PrintLineCnvMtr | MtrACTION | MtrBIT | VerifyErrLight | VerifyACTION | VerifyBIT |
| PA1Mtr | UnFIre | 0 | PA1VerifyLight | UnFire | 0 |
| PA2Mtr | UnFire | 0 | PA2VerifyLight | UnFire | 0 |

| PrintApplyLine Table (part 4) |  |  |  |
| --- | --- | --- | --- |
| ResetLineSignal | ClearLineSignal | AutoErrHandling | NoPrintersError |
| PA1ResetLine | PA1ClearLine | Y | Shutdown Line |
| PA2ResetLine | PA2ClearLine | Y | Shutdown Line |

## PrintBelt Table

In the PrintBelt table (Configuration Menu | PrintApply Utilities | Printer Belt), create an entry for each simulation line:

| PrintBelt Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | EnableFlag | Sorter | SlowSpeed | MaxSpeed | WaitRdySpeed |
| PA1Belt | Y | PA1 | 0 | 100 | 0 |
| PA2Belt | Y | PA2 | 0 | 100 | 0 |

## PrinterBoss Table

Create two printers per line in table PrinterBoss (Configuration Menu | PrintApply Utilities | Printer Configuration):

| PrintBoss Table (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | AvailAssign | Enabled | PrintLine | PrinterMachine |
| PA1Printer1 | ON | ON | PA1 | PASIM1 |
| PA1Printer2 | ON | ON | PA1 | PASIM1 |
| PA2Printer1 | ON | ON | PA2 | PASIM1 |
| PA2Printer2 | ON | ON | PA2 | PASIM1 |

| PrintBoss Table (part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ApplType | PrinterErrorIn | PrinterErrLight | PrinterErrACTION | PrinterErrBIT | PrintBelt |
| Top | PA1P1ErrIn | PA1P1ErrLight | Fire | 0 | PA1Belt |
| Top | PA1P2ErrIn | PA1P2ErrLight | Fire | 0 | PA1Belt |
| Top | PA2P1ErrIn | PA2P1ErrLight | Fire | 0 | PA2Belt |
| Top | PA2P2ErrIn | PA2P2ErrLight | Fire | 0 | PA2Belt |

## PrinterUpdPnts Table

Create update points that define where the tamper should be fired for each printer on each PNA line in table PrinterUpdPnts (Configuration Menu | PrintApply Utilities | Update Pnts).

| PrinterUpdPnts Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Type | PrintLine | Printer | LabelNum | UpdatePnt |
| PA1P1L1Small | Tamp (1 Label) Small | PA1 | PA1Printer1 | Label 1 | PA1P1L1Small |
| PA1P1L1Large | Tamp (1 Label) Large | PA1 | PA1Printer1 | Label 1 | PA1P1L1Large |
| PA1P2L1Small | Tamp (1 Label) Small | PA1 | PA1Printer2 | Label 1 | PA1P2L1Small |
| PA1P2L1Large | Tamp (1 Label) Large | PA1 | PA1Printer2 | Label 1 | PA1P2L1Large |
| PA2P1L1Small | Tamp (1 Label) Small | PA2 | PA2Printer1 | Label 1 | PA2P1L1Small |
| PA2P1L1Large | Tamp (1 Label) Large | PA2 | PA2Printer1 | Label 1 | PA2P1L1Large |
| PA2P2L1Small | Tamp (1 Label) Small | PA2 | PA2Printer2 | Label 1 | PA2P2L1Small |
| PA2P2L1Large | Tamp (1 Label) Large | PA2 | PA2Printer2 | Label 1 | PA2P2L1Large |

## PrinterErrConfig Table

Create an error handling definition for each printer in table PrinterErrConfig (Configuration Menu | PrintApply Utilities | Printer Error Config):

| PrinterErrConfig Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Enable | Printer | Update History | HRRecord | AutoResetCons | Maximum VerifyErrs |
| PA1P1Err | Y | PA1Printer1 | N | N/A | Y | 3 |
| PA1P2Err | Y | PA1Printer2 | N | N/A | Y | 3 |
| PA2P1Err | Y | PA2Printer1 | N | N/A | Y | 3 |
| PA2P2Err | Y | PA2Printer2 | N | N/A | Y | 3 |

| PrinterErrConfig Table (part 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DisableOn MaxVerifyErrs | DisableLine OnPrinterErr | DisableOn LOTARFail | ErrorIO | ErrorACTION | ErrorBIT |
| Y | N | N | PA1P1VerErr | Fire | 0 |
| Y | N | N | PA1P2VerErr | Fire | 0 |
| Y | N | N | PA2P1VerErr | Fire | 0 |
| Y | N | N | PA2P2VerErr | Fire | 0 |

## ScnScanZone Table

In table ScnScanZone (Configuration Menu | Scanner Utilities | Scan Zone Config), create a LPN scan zone and a verification scan zone for each PNA line.

| ScnScanZone Table, part 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Delimiter | LogLevel | ScanZoneID | CompareScans | Tracking Table | DeviceTrk Table |
| PA1LPNZone | ; | Level 2 | PNA1LPN | N | SrtTrack1 | SrtDevice1 |
| PA1VERZone | ; | Level 2 | PNA1VER | N | SrtTrack1 | SrtDevice1 |
| PA2LPNZone | ; | Level 2 | PNA2LPN | N | SrtTrack1 | SrtDevice1 |
| PA2VERZone | ; | Level 2 | PNA2VER | N | SrtTrack1 | SrtDevice1 |

| ScnScanZone Table, part 2 |  |  |
| --- | --- | --- |
| PrinterTrk Table | CTRL Process | PrintLine |
| PrtTrkTable1 | PA (Scan Process) | PA1 |
| PrtTrkTable1 | PA (Scan Process) | PA1 |
| PrtTrkTable1 | PA (Scan Process) | PA2 |
| PrtTrkTable1 | PA (Scan Process) | PA2 |

## ScnScanDevice Table

In table ScnScanDevice (Configuration Menu | Scanner Utilities | Scan Device Config), we define connections for the devices that deliver data about the containers.  Each line will have an LPN scanner, dimension information and a verification scanner:

| ScnScanDevice Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | Field | UpdatePt | NoReadString |
| PA1LPN | PA1LPNZone | LPN Scan | Field 1A | PA1LPNScan | ??????? |
| PA1LPNHeight | PA1LPNZone | Carton Height | Field 2A | PA1LPNScan | ? |
| PA1LPNWidth | PA1LPNZone | Carton Width | Field 3A | PA1LPNScan | ? |
| PA1LPNLength | PA1LPNZone | Carton Length | Field 4A | PA1LPNScan | ? |
| PA1VER | PA1VERZone | Verify Top | Field 1A | PA1VerScan | ??????? |
| PA1VERHeight | PA1VERZone | Carton Height | Field 2A | PA1VerScan | ? |
| PA1VERWidth | PA1VERZone | Carton Width | Field 3A | PA1VerScan | ? |
| PA1VERLength | PA1VERZone | Carton Length | Field 4A | PA1VerScan | ? |
| PA1Scale | PA1VERZone | Scale | N/A | PA1VerScan | ??????? |
| PA1VERUpdate | PA1VERZone | Scan | Field 1A | PA1VerScan | ??????? |
| PA2LPN | PA2LPNZone | LPN Scan | Field 1A | PA2LPNScan | ??????? |
| PA2LPNHeight | PA2LPNZone | Carton Height | Field 2A | PA2LPNScan | ? |
| PA2LPNWidth | PA2LPNZone | Carton Width | Field 3A | PA2LPNScan | ? |
| PA2LPNLength | PA2LPNZone | Carton Length | Field 4A | PA2LPNScan | ? |
| PA2VER | PA2VERZone | Verify Top | Field 1A | PA2VerScan | ??????? |
| PA2VERHeight | PA2VERZone | Carton Height | Field 2A | PA2VerScan | ? |
| PA2VERWidth | PA2VERZone | Carton Width | Field 3A | PA2VerScan | ? |
| PA2VERLength | PA2VERZone | Carton Length | Field 4A | PA2VerScan | ? |
| PA2Scale | PA2VERZone | Scale | N/A | PA2VerScan | ??????? |
| PA2VERUpdate | PA2VERZone | Scan | Field 1A | PA2VerScan | ??????? |

## HeightWidth Table

The HeightWidth table (Configuration Menu | PrintApply Utilities | Height Width Config) defines acceptable container dimensions to protect downstream equipment and to ensure labels can be properly affixed.  It is also used to adjust tamp points to control the position of applied labels more precisely on the containers.  Photoeyes can be used for rough dimension information (small, medium, large), or special sensing equipment can be used for precise dimensions.  For our simulation, we will use dimension scanning equipment.

| HeightWidth Table (part 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Sensor Type | Print Line | BoxDetectIn | Height1In | Height2In | HeightInDT |
| PA1HW | Scanner Data | PA1 | INVALID | INVALID | INVALID | 0 |
| PA2HW | Scanner Data | PA2 | INVALID | INVALID | INVALID | 0 |

| HeightWidth Table (part 2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BoxHgt Inches1 | BoxHgt Inches2 | BoxWdt Inches1 | BoxWdt Inches2 | Length TicksPerInch | Scaling Factor | Validate Height |
| 6.00 | 14.00 | 5.00 | 18.00 | 2.00 | 100.00 | Y |
| 6.00 | 14.00 | 5.00 | 18.00 | 2.00 | 100.00 | Y |

| HeightWidth Table (part 3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Validate Width | MinHeight Inches | MaxHeight Inches | MinWidth Inches | MaxWidth Inches | LPN ScanZone | Verify ScanZone |
| Y | 6.00 | 14.00 | 5.00 | 18.00 | PA1LPNZone | PA1VERZone |
| Y | 6.00 | 14.00 | 5.00 | 18.00 | PA2LPNZone | PA2VERZone |

## PAScannerBoss Table

In table PaScannerBoss (Configuration Menu | PrintApply Utilities | Scanner Config), we define the scanner devices for each PNA line and their verification method and messaging requirements:

| PAScannerBoss Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | PrintLine | SendLVM | VerifyType | ScanDevice |
| PA1LPNScan | PA1 | N/A | N/A | PA1LPN |
| PA1LPNHeight | PA1 | N/A | N/A | PA1LPNHeight |
| PA1LPNWidth | PA1 | N/A | N/A | PA1LPNWidth |
| PA1LPNLength | PA1 | N/A | N/A | PA1LPNLength |
| PA1VERScan | PA1 | Send LVM Msg | Verify All | PA1VER |
| PA1VERHeight | PA1 | N/A | N/A | PA1VERHeight |
| PA1VERWidth | PA1 | N/A | N/A | PA1VERWidth |
| PA1VERLength | PA1 | N/A | N/A | PA1VERLength |
| PA1ScaleData | PA1 | N/A | N/A | PA1Scale |
| PA1VERUpdate | PA1 | N/A | N/A | PA1VERUpdate |
| PA2LPNScan | PA2 | N/A | N/A | PA2LPN |
| PA2LPNHeight | PA2 | N/A | N/A | PA2LPNHeight |
| PA2LPNWidth | PA2 | N/A | N/A | PA2LPNWidth |
| PA2LPNLength | PA2 | N/A | N/A | PA2LPNLength |
| PA2VERScan | PA2 | Send LVM Msg | Verify All | PA2VER |
| PA2VERHeight | PA2 | N/A | N/A | PA2VERHeight |
| PA2VERWidth | PA2 | N/A | N/A | PA2VERWidth |
| PA2VERLength | PA2 | N/A | N/A | PA2VERLength |
| PA2ScaleData | PA2 | N/A | N/A | PA2Scale |
| PA2VERUpdate | PA2 | N/A | N/A | PA2VERUpdate |  |

## PAPrinterScanAssign Table

Table PAPrinterScanAssign (Configuration Menu | PrintApply Utilities | Printer Scan Assign) defines the relationship between scanners and printers:

| PAPrinterScanAssign Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | LabelNum | PAScanRecord |
| PA1P1LPNScan | PA1Printer1 | Label 1 | PA1LPNScan |
| PA1P2LPNScan | PA1Printer2 | Label 1 | PA1LPNScan |
| PA1P1VERScan | PA1Printer1 | Label 1 | PA1VERScan |
| PA1P2VERScan | PA1Printer2 | Label 1 | PA1VERScan |
| PA2P1LPNScan | PA2Printer1 | Label 1 | PA2LPNScan |
| PA2P2LPNScan | PA2Printer2 | Label 1 | PA2LPNScan |
| PA2P1VERScan | PA2Printer1 | Label 1 | PA2VERScan |
| PA2P2VERScan | PA2Printer2 | Label 1 | PA2VERScan |

## LabelTemplates Table

In table LabelTemplates (Configuration Menu | PrintApply Utilities | Label Configuration), we define some labels to use for testing.  These are stored locally.  A real PNA line will usually receive label images from the Print and Apply Engine.

| LabelTemplates Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Printer | Label Num | Label Type | Load Type | ExtLabel Type | Label FileName |
| PA1P1Test | PA1Printer1 | Label 1 | Standard Label | File Load | SHIP1 | LABELS/PA1P1Test.txt |
| PA1P2Test | PA1Printer2 | Label 1 | Standard Label | File Load | SHIP1 | LABELS/PA1P2Test.txt |
| PA2P1Test | PA2Printer1 | Label 1 | Standard Label | File Load | SHIP2 | LABELS/PA2P1Test.txt |
| PA2P2Test | PA2Printer2 | Label 1 | Standard Label | File Load | SHIP2 | LABELS/PA2P2Test.txt |

In this case, the test files are expected to reside in directory LABELS within the RUN directory.  Create the directory and use a text editor to create the files with some text that matches their purpose, like “PA1 Printer 1 Test Label” for PA1P1Test.txt.  To verify that everything is configured correctly, click the LoadFile button to read the file, then check the Status field, and double-click the pLabelImg field to inspect the contents.

These files would normally contain ZPL code for the printers, but it is all text to FPC.

# PNA Simulation Configuration

Now that we have defined the PNA lines, we can define the simulation containers.

## PASimCartons Table

In table PASimCartons (Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Cartons), we define a virtual container generator for each simulation line:

| PASimCartons Table (part 1) |  |  |  |
| --- | --- | --- | --- |
| Name | SimCartonCount | LengthMin | LengthMax |
| PNASImLine1 | 0 | 15 | 25 |
| PNASimLine2 | 0 | 20 | 20 |

| PASimCartons Table (part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SpacingMin | SpacingMax | PALine | SimShifter | InPoint |
| 30 | 32 | PA1 | PA1 | Inpoint1 |
| 25 | 25 | PA2 | PA2 | Inpoint2 |

## PASimWaveScans Tables

The PNA simulation wave tables, PASimWaveScans1 through PASimWaveScans5, found in Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Wave Data, are used to define data sets for the simulator.  We will define a separate set of data for each of our two simulations, using the first two tables:

For the PA1 simulation:

| PASimWaveScans1 Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CartonID | LPNScan | LabelType | VerifyString | VerifyScan | ScaleData |
| 001 | 0000000000;600;500;800 | SHIP1 | V0000000 | V0000000;600;500;800 | 5.60 |
| 002 | 1111111111;610;510;810 | SHIP1 | V0000001 | V0000001;610;510;810 | 5.61 |
| 003 | 2222222222;620;520;820 | SHIP1 | V0000002 | V0000002;620;520;820 | 5.62 |
| 004 | 3333333333;630;530;830 | SHIP1 | V0000003 | V0000003;630;530;830 | 5.63 |
| 005 | 4444444444;640;540;840 | SHIP1 | V0000004 | V0000004;640;540;840 | 5.64 |
| 006 | 5555555555;650;550;850 | SHIP1 | V0000005 | V0000005;650;550;850 | 5.65 |
| 007 | 6666666666;660;560;860 | SHIP1 | V0000006 | V0000006;660;560;860 | 5.66 |
| 008 | 7777777777;670;570;870 | SHIP1 | V0000007 | V0000007;670;570;870 | 5.67 |
| 009 | 8888888888;680;580;880 | SHIP1 | V0000008 | V0000008;680;580;880 | 5.68 |
| 010 | 9999999999;690;590;890 | SHIP1 | V0000009 | V0000009;690;590;890 | 5.69 |

For the PA2 simulation:

| PASimWaveScans2 Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CartonID | LPNScan | LabelType | VerifyString | VerifyScan | ScaleData |
| 0001 | AAAAAAAAAA;600;500;800 | SHIP2 | VAAAAAAA | VAAAAAAA;600;500;800 | 6.00 |
| 0002 | BBBBBBBBBB;610;510;810 | SHIP2 | VBBBBBBB | VBBBBBBB;610;510;810 | 6.10 |
| 0003 | CCCCCCCCCC;620;520;820 | SHIP2 | VCCCCCCC | VCCCCCCC;620;520;820 | 6.20 |
| 0004 | DDDDDDDDDD;630;530;830 | SHIP2 | VDDDDDDD | VDDDDDDD;630;530;830 | 6.30 |
| 0005 | EEEEEEEEEE;640;540;840 | SHIP2 | VEEEEEEE | VEEEEEEE;640;540;840 | 6.40 |
| 0006 | FFFFFFFFFF;650;550;850 | SHIP2 | VFFFFFFF | VFFFFFFF;650;550;850 | 6.50 |
| 0007 | GGGGGGGGGG;660;560;860 | SHIP2 | VGGGGGGG | VGGGGGGG;660;560;860 | 6.60 |
| 0008 | HHHHHHHHHH;670;570;870 | SHIP2 | VHHHHHHH | VHHHHHHH;670;570;870 | 6.70 |
| 0009 | IIIIIIIIII;680;580;880 | SHIP2 | VIIIIIII | VIIIIIII;680;580;880 | 6.80 |
| 0010 | JJJJJJJJJJ;690;590;890 | SHIP2 | VJJJJJJJ | VJJJJJJJ;690;590;890 | 6.90 |

NOTE:  CartonID in these tables is just a row name for the simulation wave data – it is not a cross-reference to the PNA configuration.

LPNScan is the whole scan, as received by the scanner.  For these simulations, we are creating a multi-part scan, as defined in ScnScanZone and ScnScanDevice, using ScnScanZone_Delimiter.  This single scan, in our example, transmits the LPN identifier and the container dimensions.  VerifyScan is similar.

LabelType, here, is compared with LabelTemplates_ExtLabelType.

Keep in mind that the string found in VerifyString must match the first portion of the string found in VerifyScan to obtain a successful verification result.

ScaleData is used to define simulated weight readings.

See section 7 on the WSD message for a way to populate these tables automatically.

## PASimBoss Table

The PASimBoss table (Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Boss) is the top-level table for definition of the simulation.  We will create a record for each of our two simulation configurations:

| PASimBoss Table (part 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | PALine | Shifter | CartonGenerator | SimAutoRestart | SimScanType |
| PASim1 | PA1 | PA1 | PNASimLine1 | ON | Static |
| PASim2 | PA2 | PA2 | PNASimLine2 | ON | Static |

| PASimBoss Table (part 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| SimScanMenu | SimLevel2 | LPNScanZone | LPNScanDevice | LPNScanPoint |
| PASimWaveScans1 | ON | PA1LPNZone | N/A | PA1LPNScan |
| PASimWaveScans2 | ON | PA2LPNZone | N/A | PA2LPNScan |

| PASimBoss Table (part 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| VerifyScanZone | VerifyScanDevice | VerifyScanPoint | ScaleScanDevice | ScaleScanPoint |
| PA1VERZone | N/A | PA1VerScan | PA1Scale | PA1Scale |
| PA2VERZone | N/A | PA2VerScan | PA2Scale | PA2Scale |

# Running The Simulations

Open the PNA simulation operator’s screen (PNASimScrn) at Configuration Menu | PrintApply Utilities | Print Apply Simulation | PA Sim Screen.

- Click the Stop ALL Cartons button to make sure there are no container simulations active.

- Click the Stop ALL button to stop any active simulations.

- Click the Reset ALL button to reset simulation data.

- Click the Clear ALL button to clear all simulation data from the associated tracking tables.

- Open the PNA System Status Window (Operator Menu | Print Apply Status).  Select the PA1 line in the Print and Apply Line box at the top of the screen.  Click Clear All and Reset All in the Printer Status and Actions box, then Clear in the Print and Apply Last Results box, then Clear in the Print and Apply Line Status box, then Reset Fault at the top of the window.  The line is now completely reset.  All the data fields should be empty, and the Line Status should be RUNNING.

- Open the Graphical Shifters window by clicking View PNA Shifter at the bottom of the PNA System Status Window (or from Configuration Menu | Sorter Utilities | Display Graphical Shifters).  Select the PA1 shifter from the shifter selection box.

- Go back to the PNASimScrn window and choose the PASim1 simulation in the PNA Sim Boss field.

- Click the Start Simulation button to begin simulating PNA line 1.  If all is configured correctly, you should see virtual containers being inducted onto the PA1 Graphical Shifter.  Use the Boxes Info tab to see the scans and scale readings.

- Open the Sorter and Print Apply tracking tables to verify that the containers are being inducted into the system and that simulation scan data is being applied at the appropriate update points.

- Return to the PNA System Status Window and observe the scan data and verification information.  [NOTE:  There is an issue with the simulator that causes the fields in the Print and Apply Last Results section not to fill in properly while the simulation is running!  TRK.]

- Inspect the FPC log files to observe the scans as they arrive (CTRL), and the message simulations (SERV).

- When you wish to stop the simulation, click Stop Carton Sim.  Once the last virtual container has exited the graphical shifter, click the Stop Simulation button.  If you stop the simulation while containers are still moving along the shifter, you may cause scan and/or verification errors as scan data will no longer be simulated.

To use the PASim2 simulation, choose it in the PNA Sim Boss field on the operator’s screen, and select PA2 from the Graphical Shifters selection box.

Click the Start Simulation button to begin the simulation.

To run both simulations simultaneously, switch to PASim1 and click the Start Simulation button again or use the Start ALL button to begin all configured simulations.

# WSD Message

The Wave Simulation Data (WSD) message can be used to deliver container simulation data to FPC for the PNA simulator.  Data for an entire test run can be delivered in this way from CommCore or any other source that can follow the pipe-delimited message format we use for most of our standard messages.  The WSD data is copied to the PASimWaveScans[1-5] tables.  Complete test data sets for a variety of test scenarios can be delivered in this way without requiring a lengthy manual data entry process.

A WSD message is composed of the fields from CmdHdr to Dat14, in order, separated by a “pipe” (|) character, and wrapped with a <STX> and <ETX> character.

Here is an example WSD message for container “0000000001” for simulation number 1 (PASimWaveScan1):

WSD|PASIM1|iQty|14|sCartonID|0000000001|sSimID|1|sLPNScan|

????????,11111111,22222222|sLPNHeight|678|sLPNWidth|567|sLPNLength|800|

sVerifyScan|U80001000,11111111,22222222|sVerifyHeight|678|sVerifyWidth|567|

sVerifyLength|800|sScaleData|3.64|sLabelType|SHIP1|sVerifyString|U00010000

The last four fields in the table are used internally by FPC for message status and tracking and are not included in the message.  Nine identical message tables are defined for use with WSD messages.  They are MsgWSD1 through MsgWSD9, and are defined as follows:

| MsgWSD Tables |  |  |
| --- | --- | --- |
| Field | Type | Description |
| CmdHdr | String | Literal string “WSD” |
| Header | String | Literal string of the AC name. “PASIM1” for our example. |
| Col1 | String | “iQty” |
| Dat1 | String | The quantity of fields in the message, including this one: 14 |
| Col2 | String | “sCartonID” |
| Dat2 | String | The container ID. |
| Col3 | String | “sSimID” |
| Dat3 | String | A one-character numeric string indicating the PASimWaveScan table that should receive this data. Valid values are 1 – 5. |
| Col4 | String | “sLPNScan” |
| Dat4 | String | A comma-delimited list of one or more LPN scan strings. Maximum total length = 120 characters. |
| Col5 | String | “sLPNHeight” |
| Dat5 | String | Optional. The height of the container. Leave blank for no height value. Set to 0 to use the default simulator height value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col6 | String | “sLPNWidth” |
| Dat6 | String | Optional. The width of the container. Leave blank for no width value. Set to 0 to use the default simulator width value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col7 | String | “sLPNLength” |
| Dat7 | String | Optional. The length of the container. Leave blank for no length value. Set to 0 to use the default simulator length value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col8 | String | “sVerifyScan” |
| Dat8 | String | A delimited list of one or more verification scan strings. The delimiter is set by ScnScanZone_Delimiter. Maximum total length = 120 characters. |
| Col9 | String | “sVerifyHeight” |
| Dat9 | String | Optional. The height of the container to report at verification. Leave blank for no height value. Set to 0 to use the default simulator height value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col10 | String | “sVerifyWidth” |
| Dat10 | String | Optional. The width of the container to report at verification. Leave blank for no width value. Set to 0 to use the default simulator width value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col11 | String | “sVerifyLength” |
| Dat11 | String | Optional. The length of the container to report at verification. Leave blank for no length value. Set to 0 to use the default simulator length value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col12 | String | “sScaleData” |
| Dat12 | String | Optional. The weight of the container. Leave blank for no weight value. Set to 0 to use the default simulator weight value. NOTE: The FPC configuration will determine whether this should be an integer or floating-point value. |
| Col13 | String | “sLabelType” |
| Dat13 | String | The LabelType string in the PASimWaveScans tables, for comparison with LabelTemplates_ExtLabelType. |
| Col14 | String | “sVerifyString” |
| Dat14 | String | This defines the expected verification string to compare with the verification scan value. |
| TxStat | PopUp | Transmit status. Used internally. |
| DbKey | Int | Database Key. Used internally. |
| DbTime | String | Database Timestamp. Used internally. |
| iDbTime | Int | Database Timestamp, integer format. Used internally. |

## WSD Machine Table Entries

Add the following two entries to the Machine table (Configuration Menu | Communication Utilities Machine) to create a server-style connection to accept delivery of WSD messages:

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine _Name | Connection _Type | Communication _Protocol | Machine _Port | Transmit _Retry | Process | Log Level |
| WSD_LS | TCPSERV_ LISTEN | PaTest | 9006 | 10 | SERV | Level 1 |
| WSD_RW | TCPSERV_ RDWR | PaTest | 9006 | 10 | SERV | Level 1 |

Note:  You must restart FPC after adding records to table Machine before they become active.

## WSD MsgMap Table Entries

Use the MsgMap Table (Configuration Menu | Communication Utilities | Message Map) to define where the WSD data will go:

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message _Name | Machine _Name | Recv _Msg _Type | Send _Msg _Type | Ack _Msg _Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| WSD _RCV | WSD _RW | PASIM1 | PASIM1 | PASIM1 | MsgWSD1 | 1 | 30 | 31 | 32 | 0 |

## WSD Message Client Connections

Once the WSD_LS and WSD_RW Machine connections are active, any client application can connect and transmit WSD messages.  The data is deposited in the MsgWSD1 table, and the PaTest algorithms place it into the requested PASimWaveScans[1-5] tables, per the sSimID field contents.  The simulation data is then available for use.