# FortnaPlus Control Print Apply Test Plan for Sorters

Document Revision: 1.3

Date: 16-Mar-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 01-May-2006 | B C Conklin | New document |
| 1.1 | 03-Jul-2012 | B C Conklin | updates |
| 1.2 | 07-Jan-2022 | T R Kremser | Update and general refresh |
| 1.3 | 16-Mar-2022 | T R Kremser | Add note regarding SrtTrack and SrtDevice table sizes. |

Table of Contents

Glossary of Terms	6

1.	Print Apply Test Plan	7

2.	Simulation Configuration	8

2.1	Machine Table	8

2.2	MsgMap Table	10

2.3	TestConfig Table	11

2.4	TestCnfgPrt Table	11

3.	One Printer, Static Assign Configuration	12

3.1.	Machine Table	12

3.2.	MsgMap Table	13

3.3.	Configio Table	14

3.4.	Conveyor Table	15

3.5.	timemenu Table	16

3.6.	Batches Table	16

3.7.	Sorters Table	17

3.8.	Inpoints Table	17

3.9.	Outpoints Table	18

3.10.	Updates Table	18

3.11.	PrintApplyLine Table	20

3.12.	SpdControl Table	20

3.13.	PrintBelt Table	21

3.14.	HeightWidth Table	21

3.15.	PrinterBoss Table	22

3.16.	PrinterUpdPnts Table	23

1.1.	PrinterErrConfig Table	23

1.2.	ScnScanZone Table	24

1.3.	ScnScanDevice Table	24

1.4.	PAScannerBoss Table	25

1.5.	PAPrinterScanAssign Table	25

1.6.	LabelTemplates Table	26

2.	One Printer, Static Assign Tests	27

3.	Two Printers, Static Assign Configuration	35

4.	Two Printers, Static Assign Tests	36

5.	Printer Clear Tests	42

6.	Wait Label and ON/OFF Speed Control Tests	43

7.	Wait Label (Error Signal) Tests	44

8.	History Count Test for Verify Scanners	44

9.	LRM Host Message Tests	45

10.	LVM Host Message Tests	46

11.	TRM Message Tests	48

12.	LMI Message Tests	49

13.	PRM Message Tests	49

14.	Two Printers, Round-Robin Tests	51

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

- Glossary of Terms

AC – Automation Controller

FPC – FortnaPlus Control

PNA, PandA – Print and Apply

# Print Apply Test Plan

This document describes how to create a FPC configuration to test the PNA module.  This example uses a sorter (shifter)-based configuration.  Transfer-based PNA is also possible.

FPC now supports three distinct PNA communication methods for the interaction with the PNA Engine:

- The original method uses pipe-delimited messages and TCP connections.  It uses LRM messages to request and receive information about the disposition of cartons, including verification requirements.  An LRM message will precipitate both an LRM response with the verification requirements and cause an LMI message to be created by the PNA Engine with label images to be forwarded to the printer.  A LVM message is initiated by the PNA AC once the newly-applied label is read and compared to the verification requirement.

- The next method uses the same LRM-LMI-LVM message scheme as the original method, but transmits those messages to the PNA Engine via an ActiveMQ message broker.  To switch from the original method to this method requires changes only in the Machine table.

- The newest method also uses the ActiveMQ message broker, but the messages are constructed in JSON format instead of pipe-delimited format.  The LRM message becomes a LabelRequest message.  The LabelRequest message response includes the label image data, so no LMI-equivalent is needed.  The LVM message becomes a LabelVerify message.  Again, to convert to this method requires changes only in the Machine table.  Internal to the FPC code, these messages are translated to and from the LRM-LMI-LVM scheme.

This document describes simulation techniques using the original communication method.  That way, the simulation can be accomplished entirely within FPC without need of an ActiveMQ broker.

We start with a simple configuration for a single printer and verification, then modify and expand on that to add a second printer and verification for two different labels, then two printers printing the same label (round-robin).  Various verification options are used, dimension checking is tested, and messages to the PNA Engine are tested.  Various error detection schemes are configured and tested.

# Simulation Configuration

We can use FPC to simulate printers, and, through the PaTest protocol, the action of the PNA Engine.  These items are only needed for testing and are not part of the PNA configuration.  They are useful for all the configurations we will create in this document, so we will set them up first.

NOTE:  This document uses the original TCP connection method for simulation, which can be set up entirely within FPC.  It is possible to simulate using the ActiveMQ connection method, as well, if an ActiveMQ message broker is available.

## Machine Table

Entries must be made in the Machine table (Configuration Menu | Communication Utilities | Machine) to simulate connections to devices required for the PNA line.  In addition to the fields shown in the table below, all should also have Transmit_Retry = 10 and LogLevel = “Level 2”.

| Machine Table – PNA Simulation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Process | NeedACK |
| Create server connections to simulate two printers. |  |  |  |  |  |
| PRT1LS | TCPSERV_LISTEN | AlphaNum | 9001 | SERV | 0 |
| PRT1RW | TCPSERV_RDWR | AlphaNum | 9001 | SERV | 0 |
| PRT2LS | TCPSERV_LISTEN | AlphaNum | 9002 | SERV | 0 |
| PRT2RW | TCPSERV_RDWR | AlphaNum | 9002 | SERV | 0 |
| Create a server connection to the simulated PNA Engine for LRM messages |  |  |  |  |  |
| LRMSRVLS | TCPSERV_LISTEN | PaTest | 9003 | SERV | 0 |
| LRMSRVRW | TCPSERV_RDWR | PaTest | 9003 | SERV | 0 |
| Create a client connection from the simulated PNA Engine for LMI messages |  |  |  |  |  |
| LMICLI | CONNECT_TCPCLIENT | PaLblImg | 9004 | SERV | 1 |
| Create a server connection to the simulated PNA Engine for LVM messages |  |  |  |  |  |
| LVMSRVLS | TCPSERV_LISTEN | PaTest | 9005 | SERV | 0 |
| LVMSRVRW | TCPSERV_RDWR | PaTest | 9005 | SERV | 0 |

A LRM request will cause a LRM response to be generated, and a LMI message to be initiated.  The LMI is placed in the SendTstLabels send queue and sent via the PaLblImg protocol.  The label image protocol then stores the label in the receive label queue, where it is ready to be sent to the printer.

Be sure to restart FPC after adding rows in the Machine table.

Create entries in the /etc/hosts file for CONNECT_TCPCLIENT connections:

127.0.0.1	LMICLI

Use the loopback address, as shown, or the AC’s IP address.  Normally, these would reference the actual addresses of the printers and PNA Engine, but for simulation, we point them to the AC itself.  Printers normally use port 9100, but for simulation we need to give them unique ports.

## MsgMap Table

The MsgMap table (Configuration Menu | Communication Utilities | Message Map) is used to link Machine connections to message tables.  This one is defined so the PNA Engine simulation can deliver a label image to the AC.

| MsgMap Table – PNA Simulation |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_Name | Recv_ Msg_ Type | Send_ Msg_ Type | Ack_ Msg_ Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| Create a connection from the PNA Engine to the AC for LRM messages |  |  |  |  |  |  |  |  |  |  |
| LRMSRVRWmap | LRMSRVRW | PA1 | PA1 | PA1 | MsgGP1 | 1 | 66 | 67 | 68 | 0 |
| Create a connection from the PNA Engine to the AC for LMI messages |  |  |  |  |  |  |  |  |  |  |
| LMICLImap | LMICLI | PA1 | PA1 | PA1 | SendTstLabels | 1 | 6 | 7 | 8 | 0 |
| Create a connection from the PNA Engine to the AC for LVM messages |  |  |  |  |  |  |  |  |  |  |
| LVMSRVRWmap | LVMSRVRW | PA1 | PA1 | PA1 | MsgGP2 | 1 | 66 | 67 | 68 | 0 |

NOTE:  The Msg_Type fields contain references to the AC name.

## TestConfig Table

Define data and connections for simulation in the TestConfig table (Configuration Menu | PrintApply Utilities | Test Config Line).  This table is in this section because it is associated with testing and simulation.  It cannot be filled in completely until the PrintApplyLine table entry is created see section 3.11.  Return here after completing the setup in section 3.

| TestConfig Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | PrintLine | VerifyStrSKU | VerifyStrRF | ExtLabelType1 | ExtLabelType2 | RtnStatusMsg |
| PA1Test | PA1Line |  |  | SHIP | CONT |  |

## TestCnfgPrt Table

Define data and connections for simulation in the TestCnfgPrt table (Configuration Menu | PrintApply Utilities | Test Config Printer).  This table is in this section because it is associated with testing and simulation.  It cannot be filled in completely until the PrinterBoss table entry is created see section 3.15.  Return here after completing the setup in section 3.

| TestCnfgPrt Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | VerifyStrL1 | VerifyStrL2 |
| PA1P1PrtCfg | PA1Prt1 | 12345678 |  |
| PA1P2PrtCfg | PA1Prt2 | 99999999 |  |

# One Printer, Static Assign Configuration

This is the most basic PNA configuration.  A single printer prints one label.  The label is read with a verification scanner.  We will build upon this configuration throughout the document.

## Machine Table

First, we create the Machine connections for the printers and to the PNA Engine:

| Machine Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Process | NeedACK |
| Create the connections to the printers |  |  |  |  |  |
| PA1PRT1 | CONNECTC_TCPCLIENT or CLOSE_CLIENT | AlphaNum | 9001 * | SERV | 0 |
| PA1PRT2 | CONNECTC_TCPCLIENT or CLOSE_CLIENT | AlphaNum | 9002 * | SERV | 0 |
| Create a connection to the PNA Engine for LRM messages |  |  |  |  |  |
| PA1LRM | CONNECT_TCPCLIENT | PaComm | 9003 | SERV | 1 |
| Create a server connection to receive the LMI messages from the PNA Engine |  |  |  |  |  |
| PA1LMILS | TCPSERV_LISTEN | PaLblImg | 9004 | SERV | 0 |
| PA1LMIRW | TCPSERV_RDWR | PaLblImg | 9004 | SERV | 0 |
| Create a connection to the PNA Engine for LVM messages |  |  |  |  |  |
| PA1LVM | CONNECT_TCPCLIENT | PaComm | 9005 | SERV | 1 |

* Note that printers usually use port 9100, but for this simulation, we are connecting them loop-back style to the simulated printers defined in section 2.1.

Also set Transmit_Retry = 10 and LogLevel = “Level 2” for all these connections.  Be sure to restart FPC after adding rows in the Machine table.

Create entries in the /etc/hosts file for the CONNECT_TCPCLIENT connections:

127.0.0.1	PA1PRT1 PA1PRT2 PA1LRM PA1LVM

You can add these onto the existing line or create a separate line.

Use the loopback address, as shown, or the AC’s IP address.  Normally, these would reference the actual addresses of the printers and PNA Engine, but for simulation, we point them back to the AC itself.

Note:  Once activated, all the entries except for the printers should show as connected.  The “connect close” Connection_Type leaves the connections closed until there is something to send.

## MsgMap Table

Next, we need to map the messages to the appropriate tables:

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_Name | Recv_ Msg_ Type | Send_ Msg_ Type | Ack_ Msg_ Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| Link the LRM messages to table MsgCR1 |  |  |  |  |  |  |  |  |  |  |
| PA1LRMmap | PA1LRM | PA1 | PA1 | PA1 | MsgCR1 | 1 | 66 | 67 | 68 | 0 |
| Link the LVM messages to table MsgCR2 |  |  |  |  |  |  |  |  |  |  |
| PA1LVMmap | PA1LVM | PA1 | PA1 | PA1 | MsgDC1 | 1 | 66 | 67 | 68 | 0 |

NOTE:  The Msg_Type fields contain references to the AC name.

## Configio Table

We must create some IO space in table Configio (Configuration Menu | IO Utilities | IO Configure).  We will use memory bits.  We need three words of input bits and two words of output bits.

| Configio Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bank | Octal_Word | LoHi | In_Out | I_O_Type | Interface | Process |
| 0 | 200 | Both | 0000000000000000 | Digital | Memory | SORT |
| 0 | 201 | Both | 1111111111111111 | Digital | Memory | SORT |
| 0 | 210 | Both | 0000000000000000 | Digital | Memory | SORT |
| 0 | 211 | Both | 0000000000000000 | Digital | Memory | SORT |
| 0 | 212 | Both | 1111111111111111 | Digital | Memory | SORT |

## Conveyor Table

Now we can use the IO bits to create some parts in the Conveyor table (Configuration Menu | Standard Controls | IO Conveyor).

| Conveyor Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| IO_Name | IO_Address_Word | IO_Address_Bit | ProcNum | Machine_Name |
| Inputs |  |  |  |  |
| PA1PE1 | 200 | 00 | SORT | PA1 |
| PA1HT1 | 200 | 01 | CTRL | PA1 |
| PA1HT2 | 200 | 02 | CTRL | PA1 |
| PA1P1LOT | 200 | 03 | CTRL | PA1 |
| PA1ResetLine | 200 | 04 | CTRL | PA1 |
| PA1ClearLine | 200 | 05 | CTRL | PA1 |
| Outputs |  |  |  |  |
| PA1EH1 | 201 | 00 | SORT | PA1 |
| PA1EH2 | 201 | 01 | SORT | PA1 |
| PA1VL1 | 201 | 02 | CTRL | PA1 |
| PA1VL2 | 201 | 03 | CTRL | PA1 |
| PA1Mtr1 | 201 | 04 | CTRL | PA1 |
| PA1ERR1 | 201 | 05 | CTRL | PA1 |
| PA1ERR2 | 201 | 06 | CTRL | PA1 |
| PA1ERR3 | 201 | 07 | CTRL | PA1 |
| PA1ScanErr1 | 201 | 10 | CTRL | PA1 |
| PA1CnflErr1 | 201 | 11 | CTRL | PA1 |
| PA1VrfyErr1 | 201 | 12 | CTRL | PA1 |
| PA1VrfyErr2 | 201 | 13 | CTRL | PA1 |

## timemenu Table

We need a few timers in table timemenu (Configuration Menu | Standard Controls | Timers):

| timemenu Table |  |  |  |
| --- | --- | --- | --- |
| Timer_Name | Timer_Address | Preset_Value | Owner |
| PA1P1TampCycle | 5 | 60.0 | PA1 |
| PA1P1HostWait | 6 | 60.0 | PA1 |

## Batches Table

Activate the print apply modules with these entries in table Batches (Configuration Menu | Standard Controls | Batches):

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| File_Name | HowOften | Increment | Type | Process | InternalProc | InternalProcCtrl | Machine | All Machines | DoFile |
| PA Sort | 1 | INVALID | Count | SORT |  | PA SORT (CPN) | PA1 | N | N |
| PA Ctrl | 1 | INVALID | Count | CTRL |  | PA CTRL (CPN) | PA1 | N | N |
| PA Serv | 1 | INVALID | Count | SERV | PA SERV (CPN) |  | PA1 | N | N |
| PA GUI | 1 | INVALID | Count | GUI | PA GUI (CPN) |  | PA1 | N | N |

## Sorters Table

Define the sorter (shifter) that underlies the print apply line in table Sorters (Configuration Menu | Sorter Utilities | Shifter Configurations):

| Sorters Table, Part 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sorter Name | Max Cartons | Max Verify Buffer | OK | Machine | Data LowRec | Data HighRec | Buffer LowRec | Buffer HighRec |
| PA1Srt | 100 | 100 | Y | PA1 | 1 | 100 | 1 | 500 |

| Sorters Table, Part 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Inpoints LowRec | Inpoints HighRec | Updates LowRec | Updates HighRec | Outpoints LowRec | Outpoints HighRec |
| 1 | 1 | 1 | 22 | 1 | 3 |

## Inpoints Table

Define an induct point where the carton image will be created in table Inpoints (Configuration Menu | Sorter Utilities | Induct Points):

| Inpoints Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inpoint Name | Sorter | Induct Number | Induct Location | Induct I/O Name | Trailing_ Edge_Pad | Minimum Gap | Longest Package |
| PA1Induct | PA1Srt | 0 | 30 | PA1PE1 | 1 | 0 | 0 |

## Outpoints Table

Define divert points for the error handling lanes in table Outpoints (Configuration Menu | Sorter Utilities | Divert Points):

| Outpoints Table, Part 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Outpoint Name | Sorter | Outpoint Number | Outpoint Location | Outpoint I/O Name | Verify Location | VerifyOffset | Verify I/O Name |
| PA1EH1Out | PA1Srt | 1 | 400 | PA1EH1 | 0 | -1 | PA1EH1 |
| PA1EH2Out | PA1Srt | 2 | 420 | PA1EH2 | 0 | -1 | PA1EH2 |

| Outpoints Table, Part 2 |  |  |  |
| --- | --- | --- | --- |
| Full_Clr_Timer_Name | Trigger_I/O_Name | LoLane | HiLane |
| tmALWAYSON | INVALID | 101 | 101 |
| tmALWAYSON | INVALID | 102 | 102 |

## Updates Table

Define points where actions will occur in table Updates (Configuration Menu | Sorter Utilities | Update Points):

| Updates Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Update Name | Sorter | Update Number | Location | Name | Window Size | Description |
| UpdPA1Height | PA1Srt | 1 | 50 | INVALID | 0 | UpdPA1Height |
| UpdPA1Width | PA1Srt | 2 | 60 | INVALID | 0 | UpdPA1Width |
| UpdPA1LPN1Scan | PA1Srt | 3 | 70 | INVALID | 0 | UpdPA1LPN1Scan |
| UpdPA1LPN2Scan | PA1Srt | 4 | 80 | INVALID | 0 | UpdPA1LPN2Scan |
| UpdPA1P1HostWait | PA1Srt | 5 | 120 | INVALID | 0 | UpdPA1P1HostWait |
| UpdPA1P1Send | PA1Srt | 6 | 130 | INVALID | 0 | UpdPA1P1Send |
| UpdPA1P1LabelWait | PA1Srt | 7 | 140 | INVALID | 0 | UpdPA1P1LabelWait |
| UpdPA1P1L1Small | PA1Srt | 8 | 150 | INVALID | 0 | UpdPA1P1L1Small |
| UpdPA1P1L1Large | PA1Srt | 9 | 160 | INVALID | 0 | UpdPA1P1L1Large |
| UpdPA1P1L2Small | PA1Srt | 12 | 190 | INVALID | 0 | UpdPA1P1L2Small |
| UpdPA1P1L2Large | PA1Srt | 13 | 200 | INVALID | 0 | UpdPA1P1L2Large |
| UpdPA1P1Verify | PA1Srt | 14 | 210 | INVALID | 0 | UpdPA1P1Verify |
| UpdPA1P2Send | PA1Srt | 15 | 300 | INVALID | 0 | UpdPA1P2Send |
| UpdPA1P2LabelWait | PA1Srt | 16 | 310 | INVALID | 0 | UpdPA1P2LabelWait |
| UpdPA1P2L1Small | PA1Srt | 17 | 320 | INVALID | 0 | UpdPA1P2L1Small |
| UpdPA1P2L1Large | PA1Srt | 18 | 330 | INVALID | 0 | UpdPA1P2L1Large |
| UpdPA1P2Verify | PA1Srt | 19 | 340 | INVALID | 0 | UpdPA1P2Verify |
| UpdPA1RFVerify | PA1Srt | 20 | 370 | INVALID | 0 | UpdPA1RFVerify |
| UpdPA1SKUVerify | PA1Srt | 21 | 380 | INVALID | 0 | UpdPA1SKUVerify |

## PrintApplyLine Table

Create the print apply line in table PrintApplyLine (Configuration Menu | PrintApply Utilities | Printer Apply Line):

| PrintApplyLine Table, part 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Assign Type | Test Mode | Send Type | Log Level | Control Machine | Num Printers | LrmMsgTable | LvmMsgTable | TrmMsgTable |
| PA1Line1 | Static Assign | Test Mode OFF | Scan – Lbl Snd | Level 2 | PA1 | 1 | MsgCR1 | MsgDC1 | MsgGP1 |

| PrintApplyLine Table, part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TstMsgTable | RcvLabelTable | SndLabelQueue | PrintLineCnvMtr | MtrACTION | MtrBIT | ScanErrLight |
| MsgTST | RcvLabels1 | SndLabelQueue1 | PA1Mtr1 | UnFire | 0 | PA1ScanErr1 |

| PrintApplyLine Table, part 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ScanACTION | ScanBIT | VerifyErrLight | VerifyACTION | VerifyBIT | ResetLineSignal | ClearLineSignal |
| UnFire | 0 | INVALID | N/A | 0 | PA1ResetLine | PA1ClearLine |

Note:  Be sure to return to section 2.3 to complete the configuration there now that these records have been created.

## SpdControl Table

Later in the simulation we will need a SpdControl (Configuration Menu | Standard Controls | Motor Speed Control) entry:

| SpdControl Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable Flag | Maint_IO | IO_Owner | Off Percent | On Percent | IO_Addr_Out | IO_Addr_In | Max Speed | Print Line |
| PA1Speed | Y | INVALID | CTRL | 100 | 100 | 212 | 212 | 1111111 | PA1Line |

## PrintBelt Table

Define the parameters needed for the shifter in the context of Print Apply with the PrintBelt table (Configuration Menu | PrintApply Utilities | Printer Belt):

| PrintBelt Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | EnableFlag | Sorter | SlowSpeed | MaxSpeed | WaitRdySpeed |
| PA1Belt | Y | PA1Srt | 10 | 90 | 0 |

## HeightWidth Table

Set up the HeightWidth table (Configuration Menu | PrintApply Utilities | Height Width Config) to adjust the tamp point based on carton height and width:

| HeightWidth, part 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Sensor Type | PrintLine | Height1In | Height2In | HeightInDT | BoxHgt Inches1 | Sensor Hgt1 | BoxHgt Inches2 |
| PA1HtWt | Photoeye | PA1Line | PA1HT1 | PA1HT2 | 210 | 5.00 | 1000 | 14.00 |

| HeightWidth, part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sensor Hgt2 | WidthInDT | BoxWdt Inches1 | Sensor Wdt1 | BoxWdt Inches2 | Sensor Wdt2 | LengthTicks PerInch |
| 500 | 210 | 6.00 | 1500 | 18.00 | 400 | 2.00 |

## PrinterBoss Table

Define the printers in the PrinterBoss table (Configuration Menu | PrintApply Utilities | Printer Configuration):

| PrinterBoss Table, part 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Avail Assign | Enabled | Print Line | Printer Machine | Appl Type | Trail Edge | Printer ErrorIn | Tamper HomeIn |
| PA1Prt1 | Y | Y | PA1Line | PA1PRT1 | Top | N | INVALID | INVALID |
| PA1Prt2 | Y | Y | PA1Line | PA1PRT2 | Side | N | INVALID | INVALID |

|  |  | PrinterBoss Table, part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LabelOn TampIn | Cycle CompleteIn |  | Printer ResetOut | Tamper SolOut | Printer ErrLight | PrinterErr ACTION | PrintBelt | MinLength |
| INVALID | INVALID |  | INVALID | INVALID | PA1ERR1 | Fire | PA1Belt | 2 |
| INVALID | INVALID |  | INVALID | INVALID | PA1ERR2 | Fire | PA1Belt | 2 |

Note:  Be sure to return to section 2.4 to complete the configuration there now that these records have been created.

## PrinterUpdPnts Table

Define update points for print and apply actions in the PrinterUpdPnts table (Configuration Menu | PrintApply Utilities | Update Pnts):

| PrinterUpdPnts Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Type | Print Line | Printer | Label Num | UpdatePnt | Delay Timer | CycleTmr | Error Signal | Error ACTION |
| PA1UpdHeight | Height Update | PA1Line | N/A | N/A | UpdPA1Height | N/A | N/A | INVALID | N/A |
| PA1UpdWidth | N/A * (Width Update) | N/A * (PA1Line) | N/A | N/A | N/A * (UpdPA1Width) | N/A | N/A | INVALID | N/A |
| PA1P1L1Sm | Tamp (1 Label) Small | PA1Line | PA1Prt1 | Label 1 | UpdPA1P1L1Small | N/A | N/A | INVALID | N/A |
| PA1P1L1Lg | Tamp (1 Label) Large | PA1Line | PA1Prt1 | Label 1 | UpdPA1P1L1Large | N/A | N/A | INVALID | N/A |
| PA1P1LabelWait | Wait Label Feed | PA1Line | PA1Prt1 | Label 1 | UpdPA1P1LabelWait | N/A | PA1P1TampCycle | PA1ERR3 | UnFire |

* Leave at N/A for now.  We will fill in the values in parenthesis later.

## PrinterErrConfig Table

Define how a printer will behave when errors occur using the PrinterErrConfig table (Configuration Menu | PrintApply Utilities | Printer Error Config):

If PrinterErrConfig_Enable = N (manually set), the line may continue to run even if the printer has an active error.

| PrinterErrConfig Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable | Printer | AutoResetCons | Maximum VerifyErrs | ErrorIO | Error ACTION | ErrorSignal |
| Prt1Err | Y | PA1Prt1 | N | 1 | PA1VL1 | Fire | PA1VrfyErr1 |
| Prt2Err | Y | PA1Prt2 | N | 1 | PA1VL2 | Fire | PA1VrfyErr2 |

## ScnScanZone Table

Define the scan zones in the ScnScanZone table (Configuration Menu | Scanner Utilities | Scan Zone Config):

| ScnScanZone Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Scan ZoneID | Compare Scans | Tracking Table | DeviceTrkTable | PrinterTrkTable | CTRLProcess | Print Line |
| PA1LPNZone | PA1LPN | N | SrtTrack1 | SrtDevice1 | PrtTrkTable1 | PA (Scan Process) | PA1Line |
| PA1VerZone | PA1VER | N | SrtTrack1 | SrtDevice1 | PrtTrkTable1 | PA (Scan Process) | PA1Line |

NOTE:  Versions prior to V11.47.0 have a bug that requires the TrackingTable and DeviceTrkTable pair chosen to be defined with the same number of records for Print and Apply applications.  If DeviceTrkTable is larger than TrackingTable, the symptom is LRM messages will stop being generated when DeviceTrkTable row numbers greater than the last row of TrackingTable are used.  This is not a problem for Sorter applications.

## ScnScanDevice Table

Define the scanners in the ScnScanDevice table (Configuration Menu | Scanner Utilities | Scan Device Config):

| ScnScanDevice Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | UpdatePt | NoReadString | ReadConflictStr |
| PA1LPNScan | PA1LPNZone | LPN Scan | UpdPA1LPN1Scan | ???????? | ******** |
| PA1P1Verify | PA1VerZone | Verify Top | UpdPA1P1Verify | ???????? | ******** |
| PA1P2Verify | N/A | Verify Left | UpdPA1P2Verify | ???????? | ******** |

Note:  We will fill in the ScanZone for PA1P2Verify later.

## PAScannerBoss Table

Connect the scanners to the print apply line in table PAScannerBoss (Configuration Menu | PrintApply Utilities | Scanner Config):

| PAScannerBoss Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | PrintLine | SendLVM | VerifyType | ScanDevice |
| PA1LPNSBoss | PA1Line | N/A | N/A | PA1LPNScan |
| PA1Ver1SBoss | PA1Line | N/A | Verify One | PA1P1Verify |

## PAPrinterScanAssign Table

Associate printers with labels and scanners in table PAPrinterScanAssign (Configuration Menu | PrintApply Utilities | Printer Scan Assign):

| PAPrinterScanAssign Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | LabelNum | PAScanRecord |
| PA1P1Assign | PA1Prt1 | Label 1 | PA1Ver1SBoss |

## LabelTemplates Table

Table LabelTemplates (Configuration Menu | PrintApply Utilities | Label Configuration) defines and names the labels, which printers they are associated with, and how they are stored and retrieved.

The path for LabelFileName is relative to the RUN directory.  To implement as shown below, create a directory LABELS beneath the RUN directory and put the zpl files in it.  To verify, click LoadFile, then Apply.  Status will say “File Loaded” on success.  Double-click pLabelImg to see the label image contents.

| LabelTemplates Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Printer | LabelNum | LabelType | LoadType | ExtLabelType | LabelFileName |
| PA1P1NoRead | PA1Prt1 | Label 1 | No Read | File Load | NOREAD | LABELS/NoRead.zpl |
| PA1P1Clear | PA1Prt1 | Label 1 | Clear Printer | File Load | CLEAR | LABELS/ClearPrinter.zpl |
| PA1P1TestLabel | PA1Prt1 | Label 1 | Test Label | File Load | TEST1 | LABELS/Test1.zpl |
| PA1P1Std | PA1Prt1 | Label 1 | Standard Label | N/A | SHIP |  |
| PA1P2NoRead | PA1Prt2 | Label 1 | No Read | File Load | NOREAD | LABELS/NoRead.zpl |
| PA1P2Clear | PA1Prt2 | Label 1 | Clear Printer | File Load | CLEAR | LABELS/ClearPrinter.zpl |
| PA1P2TestLabel | PA1Prt2 | Label 1 | Test Label | File Load | TEST2 | LABELS/Test2.zpl |
| PA1P2Std | PA1Prt2 | Label 1 | Standard Label | N/A | CONT |  |

# One Printer, Static Assign Tests

Table TestConfig (Configuration Menu | PrintApply Utilitites | Test Config Line) defines parameters needed to simulate the PNA Engine.

Table TestCnfgPrt (Configuration Menu | PrintApply Utilities | Test Config Printer) defines test verification strings for each printer and label.  VerifyStrL1 and VerifyStrL2 refer to Label 1 and Label 2, respectively, where Label 2 is rarely used – it is for label-over-label applications where one printer applies two labels to each carton.

To perform the following tests, open the PNA System Status Window (Operator Menu | Print Apply Status).  This is the PNA operator’s normal control console.  Click the “View PNA Shifter” at the bottom of the screen to open the graphical shifter for the sorter that underlies the PNA line.  Use Alt-S to shift the shifter by one tick.  Use Alt-I to toggle the state of the induct photoeye.  To induct a carton, turn the induct eye ON, then shift the shifter a few ticks, then turn it OFF.

NOTE:  Be sure to restart FPC after all of the tables have been populated.  You may need to set Sorters_OK = Y before you can use the graphical shifter.  Check Sorters_pStatus for error messages.

Height Test (Photoeye), Short box:

- Clear the PA1 shifter.

- Use PNA System Status Window, Reset Fault to clear any errors.

- Verify the height sensors PA1HT1 and PA1HT2 are OFF in table HeightWidth.

- Induct a carton and move it to the UpdPA1Height update point.

- Verify that a printer tracking table record (PrtTrkTable1) has been created with a PATrkIndx value that matches the tracking table (SrtTrack1) index for the current record.  The PrtTrkTable1_TmpIndxG1 field will have the Updates_Location value for the UpdPA1P1L1Small update point (150).

Height Test (Photoeye), Medium box:

- Set PA1HT1 ON and PA1HT2 OFF and induct another carton.

- Move the carton to the UpdPA1Height update point.

- Verify that the PrtTrkTable1_TmpIndxG1 value is set to the mid-point between the UpdPA1P1L1Small and UpdPA1P1L1Large update points (155).

Height Test (Photoeye), Large box:

- Set PA1HT1 ON and PA1HT2 ON and induct another carton.

- Move the carton to the UpdPA1Height update point.

- Verify that the PrtTrkTable1_TmpIndxG1 value is set to the Location value for the UpdPA1P1L1Large update point (160).

Height Test (Ultrasonic/Laser):

- Adjust the HeightWidth table configuration to use SensorType Ultrasonic.  Previously-configured values define HeightInDT as the input data word (210) where the sensor will deliver its height reading, BoxHgtInches1 as the minimum box height (5.0 inches) with a sensor reading of SensorHgt1 = 1000 decimal, and BoxHgtInches2 as the maximum height (14.0 inches) with a sensor reading of SensorHgt2 = 500 decimal.

- Clear the PA1 shifter.

- Use PNA System Status Window, Reset Fault to clear any errors.

- Use the View I/O (F3) screen to find IO Word 210 and set it to 0000001110000000 – or 896 in decimal notation – by forcing bits 7,8 and 9 ON..

- Induct a carton.  Count the number of ticks you move the carton while the induct eye is on – you will need this value later.

- Move the carton to the UpdPA1Height update point.

- Verify that a tracking table record has been created (SrtTrack1) and Height has a value of 6.87 inches.  This value is computed by finding the sensor range defined by SensorHgt1 and SensorHgt2, and the range in inches defined by BoxHgtInches1 and BoxHgtInches2, then computing a inches per sensor ‘tick’ value.  Then, the difference between the HeightInDT and SensorHgt1 is taken, and it is multiplied by the inches per sensor ‘tick’ value, and finally, the BoxHgtInches1 value is added in to find the true height of the box, in inches.

- Verify a printer tracking record has been created (PrtTrkTable1) with a PATrkIndx value that matches the tracking record number.  The TmpIndxG1 value will have a value interpolated from the height data according to the box size, using the UpdPA1P1L1Small and UpdPA1P1L1Large update points.  In this case, the box height is 20.8% of the height range, resulting in a tick position of 0.208 * (160 – 150) = 152.

Width Test (Ultrasonic / Laser):

- In table PrinterUpdPnts, fill in the rest of the PA1Width record: Type = “Width Update”, PrintLine = “PA1Line”, UpdatePnt = “UpdPA1Width”.  Restart FPC to make the changes take effect.

- Keep the HeightWidth_SensorType set to Ultrasonic.  For this test, we are using the same sensor input word in WidthInDT.  In real life, a separate sensor would provide the value.  The input value is still 896 in decimal notation.  Previously-configured values define WidthInDT as input data word 210, BoxWdtInches1 = 6.0 inches, SensorWdt1 = 1500, BoxWdtInches2 = 18.0, and SensorWdt2 = 400.

- Advance the carton to the UpdPA1Width update point.

- Verify that the tracking record Width field has a value of 12.59 inches.  This value is computed in the same way as the height, using the aforementioned width configuration values.

Length Test:

- Make sure the ScnApp CTRL batch job is active in table Batches.  The row should already be in the table, named “Scan Process”, but it may need to be activated by populating the Machine column.

- Advance the carton to the UpdPA1LPN1Scan scan induction update point.  Assign a scan value to the carton using the ScnScanDevice table.

- Verify the scan value appears in the tracking record.

- Verify the Length field in the tracking record is updated to show a value that is the number of ticks that you recorded for the length of the carton at induct multiplied by the HeightWidth_LengthTicksPerInch value (2).  Note that Inpoints_Trailing_Edge_Pad and HeightWidth_TampAdjInches are not considered here.

Printer Data Send Test:

- Since PrintApplyLine_SendType is set to “Scan – LblSnd”, the label is sent to the printer as soon as the scan is processed.

- Verify the label data is sent to the printer by inspecting the SERV log.

- Verify the LblL1 status is set to “Sent” in the printer tracking table.

Tamp Test:

- Clear the PA1 shifter.

- Use the Print Apply Status control panel to Reset Fault and verify that any errors have cleared.

- Create a carton and shift it to the UpdPA1Height update point.

- Shift the leading edge of the carton to the location indicated in the printer tracking record TmpIndxG1 field.

- Verify the SORT log contains a message indicating the tamp has been fired.  (There will already be some “Tamped” messages in the log from previous tests).

- Change the PrinterBoss_TrailEdge setting to Y for PA1Prt1.

- Create a carton and advance it to the UpdPA1LPN1Scan update point.

- Assign scan data to the carton.

- Advance the carton until the trailing edge reaches the location indicated in the printer tracking record TmpIndxG1 field.

- Verify the SORT log contains a message indicating the tamp has been fired.

- Change the PrinterBoss_TrailEdge setting back to N.

Label Send Test:

- Clear the PA1 shifter.

- Use the Print Apply Status control panel to Reset Fault and verify that any errors have cleared.

- Change PrintApplyLine_Type to “Upd Pnt - Lbl Snd”

- Add a Printer Update Point

| PrinterUpdPnts Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Type | PrintLine | Printer | LabelNum | UpdatePnt | DelayTimer | CycleTmr |
| PA1P1LabelSend | Send Label | PA1Line | PA1Prt1 | Label 1 | UpdPA1P1Send | N/A | N/A |

- Create a carton, advance it to the scan point and simulate receipt of a scan.

- Check the printer tracking record, LblL1 field.  Verify it is set to “Received”.  Verify LblL2 is set to “Not Required”.

- Advance the carton to the label send update point, UpdPA1P1Send.

- Verify LblL1 has changed to “Sent”.

- Verify that the label data was sent to the printer by checking the SERV log.

LPN Scan Error Test:

- Clear the PA1 shifter.

- Use the Print Apply Status control panel to Reset Fault and verify that any errors have cleared.

- Run the History Config Wizard (Configuration Menu | Wizard Utilities | History Config Wizard).  Click Create for scan zone PA1LPNZone and scan devices PA1LPNScan and PA1P1Verify.  This will create the error records needed in the ScnZoneErrIO and ScnDeviceErrIO tables.  You can safely click Create for all the options presented.  Click Save once all items have been created.

- Verify that PrintApplyLine_PrintLineCnvMtr is set to PA1Mtr1, and MtrACTION is set to UnFire, and that it is turned ON once the Reset Fault has been performed (The PNA System Status Window should show Line Status: RUNNING).

- Set up table ScnDeviceErrIO (Configuration Menu | Scanner Utilities | Device Err Config) as shown by filling in the missing items in the existing “Dv1 NoScan” record that was created by the History Config Wizard:

| ScnDeviceErrIO Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanDevice | ErrorType | MaximumErrors | AppSorter | PrintLine | ErrorLane |
| Dv1 NoScan | PA1LPNScan | No Scan | 1 | N/A | PA1Line | PA1EH1Out |

- Induct a carton and shift it past the LPN scan update point.  Verify Conveyor_PA1Mtr1 goes OFF.

- On the PNA System Status Window, Line Status will be STOPPED.  Click Reset Fault.  Line Status will return to RUNNING.

- Induct another carton and shift it past the LPN scan update point.  Verify Conveyor_PA1Mtr1 goes OFF again.

- In table PrintApplyLine, find column ResetLineSignal.  Force it ON, then clear the force.

- Verify that the Print Apply Status Line Status is RUNNING.  This demonstrates that the input signal (such as from a pushbutton) can be used to reset the line.

NoRead Label and Exception Handling Lane “LPN Scan Error” Test:

- ScnDeviceErrIO_ErrorLane is already set to PA1EH1Out from the previous test.

- Induct a carton and shift it through the LPN scan update point to create a NoScan error.

- Use the SERV log to verify the NoRead label was sent to printer PA1Prt1.  Verify the carton tracking record (SrtTrack1) shows SorterLane = 101, indicating the carton has been assigned to that lane.

- Shift the carton to the PA1EH1Out outpoint and verify that it fires (turns green).

LPN Scan Error Test with Update Point Label Send:

- Clear the PA1 shifter.

- Verify the PrintApplyLine SendType is set to “Upd Pnt - Lbl Snd”.

- Use the PNA System Status Window to Reset Fault and verify that any errors have cleared.

- Induct a carton and shift it past the LPN scan update point.  Verify ScanErrLight turns ON.

- Use the PNA System Status Window Reset Fault again and verify that the error clears.

LPN Scan Conflict Test:

- In table ScnDeviceErrIO, find the row labeled “Dv1 Conflict” with ErrorType = Conflict and adjust it as shown:

| ScnDeviceErrIO Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanDevice | ErrorType | MaximumErrors | AppSorter | PrintLine | ErrorLane |
| Dv1 Conflict | PA1LPNScan | Conflict | 1 | N/A | PA1Line1 | PA1EH1Out |

- Induct a carton and shift it to the LPN scan update point.  Enter “********” (8 asterisks) into the ScnScanDevice_RcvData field for the PA1LPNScan row.  Set RxStat to “Recvd” and click Apply to simulate receipt of a read conflict from the scanner.  Verify that ParsedStatus changes to “Read Conflict”.  Click Save.

- Verify that the resulting scan conflict error stops the conveyor and that the signal in PrintApplyLine_ScanErrLight turns ON.

- Use the PNA System Status Window to Reset Fault and verify that any errors have cleared.

LPN Scan No Read Test:

- In table ScnDeviceErrIO, find the row labeled “Dv1 NoRead” with ErrorType = “No Read” and adjust it as shown:

| ScnDeviceErrIO Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanDevice | ErrorType | MaximumErrors | AppSorter | PrintLine | ErrorLane |
| Dv1 NoRead | PA1LPNScan | No Read | 1 | N/A | PA1Line1 | PA1EH1Out |

- Induct a carton and shift it to the LPN scan update point.  Enter “????????” (8 question marks) into the ScnScanDevice_RcvData field for the PA1LPNScan row.  Set RxStat to “Recvd” and click Apply to simulate receipt of a NoRead from the scanner.  Verify that ParsedStatus changes to “No Read”.  Click Save.

- Verify that the resulting no read error stops the conveyor and that the signal in PrintApplyLine_ScanErrLight turns ON.

- Use the PNA System Status Window to Reset Fault and verify that any errors have cleared.

Verify Error Lane Assignment Tests:

- In table ScnDeviceErrIO, find all the rows with ScanDevice = PA1P1Verify.  Set PrintLine = PA1Line for all of them.  Set ErrorLane = PA1EH1Out for the No Scan, Bad Read, No Read and Conflict entries.

- Clear the shifter.

- Induct a carton and shift it to the LPN scan update point.  Enter any valid numeric scan value into the ScnScanDevice_RcvData field for the PA1LPNScan row.  Set RxStat to “Recvd” and click Apply.  ParsedStatus should change to “Good Scan”.  Click Save.

- Advance the carton to the UpdPA1P1Verify update point.  Enter a scan value on row PA1P1Verify that does not match the “good” value of “12345678” expected in TestCnfgPrt_VerifyStrL1 to simulate a verification failure.  ScnScanDevice_ParsedStatus will show “Good Scan” to indicate that a valid value has been received.

- Verify that the “Print and Apply Last Results:” section shows Result = “MISMATCH ERROR” along with the verification scan you entered in Actual, and the Expected value of 12345678.  In the graphical shifter, the box will turn white, and the Boxes Info tab will show it has been assigned to lane 101 – PA1EH1Out.

- Induct a new box and assign LPN data to it, as before.

- Advance it through the UpdPA1P1Verify update point without assigning a verification scan value to produce a NoScan.

- Verify that Result = “SCAN ERROR”, and Actual = NOSCAN.  The box will turn white and be assigned to lane 101.

# Two Printers, Static Assign Configuration

Let’s add a second printer to the configuration and perform some tests with that.

First, in table PrintApplyLine, change the NumPrinters value from 1 to 2.

We have already defined the second printer, PA1PRT2, in table Machine.

We have already defined the PA1P2Verify printer in table ScnScanDevice, but we need to add it to the verification zone with ScanZone = PA1VerZone.

| ScnScanDevice Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | UpdatePt | NoReadString | ReadConflictStr |
| PA1LPNScan | PA1LPNZone | LPN Scan | UpdPA1LPN1Scan | ???????? | ******** |
| PA1P1Verify | PA1VerZone | Verify Top | UpdPA1P1Verify | ???????? | ******** |
| PA1P2Verify | PA1VerZone | Verify Left | UpdPA1P2Verify | ???????? | ******** |

Connect the second verify scanner to the PNA configuration in table PAScannerBoss by adding the following line:

| PAScannerBoss Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | PrintLine | SendLVM | VerifyType | ScanDevice |
| PA1Ver2SBoss | PA1Line | N/A | Verify All | PA1P2Verify |

Add the second printer in table PAPrinterScanAssign to associate it with the scanner:

| PAPrinterScanAssign Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | LabelNum | PAScanRecord |
| PA1P2Assign | PA1Prt2 | Label 1 | PA1Ver2SBoss |

We have already defined PA1Prt2 in the PrinterBoss table.

We have already defined several UpdPA1P2 update points in table Updates.

Add rows to table PrinterUpdPnts for the second printer:

| PrinterUpdPnts Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Type | PrintLine | Printer | LabelNum | UpdatePnt |
| PA1P2LabelSend | Send Label | PA1Line | PA1Prt2 | Label 1 | UpdPA1P2Send |
| PA1P2L1Sm | Tamp (1 Label) Small | PA1Line | PA1Prt2 | Label 1 | UpdPA1P2L1Small |
| PA1P2L1Lg | Tamp (1 Label) Large | PA1Line | PA1Prt2 | Label 1 | UpdPA1P2L1Large |

We have already included rows in table LabelTemplates for the second printer.

We have already included a row in table TestCnfgPrt for the second printer.

# Two Printers, Static Assign Tests

Immediate Send Type

- Make sure to restore PrintApplyLine_SendType to “Scan – Lbl Snd”.

- Clear the PA1Srt shifter.

- Create a carton and shift it to the LPN scan update point.

- Assign scan data to the carton.

- Verify two printer tracking records are created with a PATrkIndx value that matches the assigned tracking table row number.

- In those printer tracking records, verify that the ExtType1 fields are set to “SHIP” and “CONT”.  These values come from the TestConfig table, ExtLabelType1 and ExtLabelType2 fields.

- Also, in the printer tracking records, verify the ExptL1 fields are set to “12345678” and “99999999”.  These values come from the TestCnfgPrt table, VerifyStrL1 fields.

- Verify the TmpIndxG1 field is updated with interpolated tamp point data based on the height of the carton for the PA1Prt1 printer record and with interpolated tamp point data based on the width of the carton for the PA1Prt2 printer record, as previously described.

- Verify the label status in LblL1 is set to “Sent” for both printer tracking records.

Update Point Send Type, Pass

- Set PrintApplyLine_SendType to “Upd Pnt – Lbl Snd”.

- Clear the PA1Srt shifter.

- Create a carton and shift it to the LPN scan update point.

- Assign scan data to the carton.

- Shift the carton to the UpdPA1P1Send update point.

- Verify the printer tracking record LblL1 field is set to “Sent” for the PA1Prt1 printer only.  LblL1 for the PA2Prt2 printer will be “Received”.

- Verify the SERV log shows the data sent to PA1Prt1.

- Shift the carton to the UpdPA1P1Verify update point.  Assign a verification scan value of “12345678” (the expected good value).

- Shift the carton to the UpdPA1P2Send update point.

- Verify the printer tracking record LblL1 field is set to “Sent” for the PA1Prt2 printer.

- Verify the SERV log shows the data sent to PA1Prt2.

- Shift the carton to the UpdPA1P2Verify update point.  Assign a verification scan value of “99999999” (the expected good value).

- Verify the printer tracking record VerL1 field shows “Okay” for the PA1Prt1 record.

- Verify the printer tracking record VerL1 field shows “Okay” for the PA1Prt2 record.

- Note that these values can also be seen in the ‘Print and Apply Last Results’ box on the PNA System Status Window.

Update Point Send Type, Fail

- Keep PrintApplyLine_SendType set to “Upd Pnt – Lbl Snd”.

- Clear the PA1Srt shifter.

- Repeat the previous test, but do not assign a scan at the UpdPA1P1Verify update point.

- Shift the carton past the UpdPA1P2Verify update point.

- Verify VerL1 in the printer tracking record for printer PA1Prt1 shows “Scan Error”.

- Verify VerL1 for printer PA1Prt2 shows “Not Verified”.  The label is not applied, but the status shows “Not Verified” because of the error from the first label.

Verify One Label Error Test (Motor Stop, Error Light – Two Printers)

Let’s add some more records in table ScnDeviceErrIO.  You can re-run the configuration wizard (Configuration Menu | Wizard Utilities | History Config Wizard) or add the rows manually.  Make sure PrintLine and ErrorLane are set as shown after running the wizard.

| ScnDeviceErrIO Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | ScanDevice | ErrorType | MaximumErrors | AppSorter | PrintLine |  | ErrorLane |
| Existing rows (there may be more): |  |  |  |  |  |  |  |
| Dv1 NoScan | PA1LPNScan | No Scan | 1 | N/A | PA1Line |  | PA1EH1Out |
| Dv1 BadRead | PA1LPNScan | Bad Read | 1 | N/A | PA1Line |  | PA1EH1Out |
| Dv1 NoRead | PA1LPNScan | No Read | 1 | N/A | PA1Line |  | PA1EH1Out |
| Dv1 Conflict | PA1LPNScan | Conflict | 1 | N/A | PA1Line |  | PA1EH1Out |
| Dv2 NoScan | PA1P1Verify | No Scan | 1 | N/A |  | PA1Line | PA1EH1Out |
| Dv2 BadRead | PA1P1Verify | Bad Read | 1 | N/A |  | PA1Line | PA1EH1Out |
| Dv2 NoRead | PA1P1Verify | No Read | 1 | N/A |  | PA1Line | PA1EH1Out |
| Dv2 Conflict | PA1P1Verify | Conflict | 1 | N/A |  | PA1Line | PA1EH1Out |
| Add these rows: |  |  |  |  |  |  |  |
| Dv3 NoScan | PA1P2Verify | No Scan | 1 | N/A |  | PA1Line | PA1EH2Out |
| Dv3 BadRead | PA1P2Verify | Bad Read | 1 | N/A |  | PA1Line | PA1EH2Out |
| Dv3 NoRead | PA1P2Verify | No Read | 1 | N/A |  | PA1Line | PA1EH2Out |
| Dv3 Conflict | PA1P2Verify | Conflict | 1 | N/A |  | PA1Line | PA1EH2Out |

- Clear the PA1Srt shifter and reset any faults.

- Create a carton and shift it to the LPN scan update point.

- Assign scan data to the carton.

- Shift the carton through the UpdPA1P2Verify update point.

- Verify Line Status = STOPPED for a NoScan error on the PNA System Status Window (PrintApplyLine_PrintLineCnvMtr is OFF).

- Use Reset Fault to clear the error.

Verify One Label and Verify One Motor Stop Test

- Clear the PA1Srt shifter.

- Use the PNA System Status Window Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign scan data to the carton.

- Shift the carton past the UpdPA1P1Verify update point.

- Verify PrinterErrConfig_ErrorSignal and PrinterErrConfig_ErrorIO are OFF for both printers.

- Shift the carton past the UpdPA1P2Verify update point.

- Verify PrintErrConfig_ErrorIO and PrintErrConfig_ErrorSignal are ON for the first printer.  The line stops for the verification failure on the first label.

Verify Mismatch Test

- Clear the PA1Srt shifter and reset any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the UpdPA1P1Verify update point.

- Assign verification scan data, but NOT “12345678” as defined in table TestCfgPrt, column VerifyStrL1 for printer 1.

- Advance the carton to the UpdPA1P2Verify update pont.

- Assign scan data “99999999” as defined in table TestCfgPrt, column VerifyStrL1 for printer 2.

- Verify that the line stops for a MISMATCH ERROR, and the value in VerL1 in the printer tracking record is set to “MissMatch Error” [sic].  The operator screen will also indicate Error Message: “Bad Read”, Status Message: “Error” and Comment: “PA1P1Verify”.

Verify Scan Test with Two Scanners – Success Case

- Clear the PA1Srt shifter.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the UpdPA1P1Verify update point.

- Assign verification scan data “12345678” to match TestCfgPrt_VerifyStrL1 for printer 1.

- Advance the carton to the UpdPA1P2Verify update point.

- Assign verification scan data “999999999” to match TestCfgPrt_VerifyStrL1 for printer 2.

- Verify that the values in VerL1 in the printer tracking record are set to “Okay” for both printers.

Scan Zone Test for Verify One, Verify All

- Change the configuration in table ScnDeviceErrIO for the PA1P2Verify NoScan, BadRead, NoRead and Conflict rows (the four rows assigned to ErrorLane PA1EH2Out) to set MaximumErrors from 1 to 0.

- Change the configuration in table ScnZoneErrIO to set MaximumErrors = 1 for the NoScan,BadRead, NoRead and Conflict entries for PA1VerZone.  Set PrintLine = PA1Line1 and ErrorLane = PA1EH2Out for these rows, as well.

- In table PAScannerBoss, set PrintLine = PA1Line1 to activate the PA1Ver2SBoss entry, and make sure VerifyType = “Verify All”.

Demonstrate No Second Label if Error on First

- Clear the PA1Srt Shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Advance the carton through the UpdPA1P1Send update point.

- Use the SERV log to verify that a label was sent to the printer Machine entry PRT1RW.

- Verify the printer tracking record for printer PA1Prt1 has the LblL1 field set to “Sent”.

- Advance the carton through the UpdPA1P1Verify update point without entering a scan value.

- Advance the carton to the UpdPA1P2Send update point.  ***TRK: This doesn’t work – the TEST2 label is sent to the printer!  Must be a misconfiguration, since this SHOULD work!

- Verify the printer tracking record for printer PA1Prt2 has the LblL1 field set to “Error”.  ***TRK: This doesn’t happen, either.

- Shift the carton past the UpdPA1P2Verify update point without entering a scan value.

- Verify the printer tracking record for printer PA1Prt1 has the VerL1 field set to “Scan Error” and printer PA1Prt2 has the VerL1 field set to “Not Verified”.  No verification is done on the second label since there was an error on the first label.

- Verify the PNA System Status Window shows the line stopped for a SCAN ERROR on the first label.

- Use Reset Fault to clear the error.

Demonstrate Second Label is Sent if Success on First

- Create a new carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Advance the carton through the UpdPA1P1Send update point.

- Use the SERV log to verify that a label was sent to the printer Machine entry PRT1RW.

- Verify the printer tracking record for printer PA1Prt1 has the LblL1 field set to “Sent”.

- Advance the carton to the UpdPA1P1Verify update point and enter scan value “12345678” to match TestCfgPrt_VerifyStrL1 for printer 1.

- Advance the carton to the UpdPA1P2Send update point.

- Use the SERV log to verify that a label was sent to the printer Machine entry PRT2RW.

- Verify the printer tracking record for printer PA1Prt2 has the LblL1 field set to “Sent”.

- Advance the carton to the UpdPA1P2Verify update point and enter scan value “99999999” to match TestCfgPrt_VerifyStrL1 for printer 2.

- Verify that the values in VerL1 in the printer tracking record are set to “Okay”.

# Printer Clear Tests

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton through the UpdPA1P1Send update point.

- Use the PNA System Status Window.  Click the Clear All button in the Printer Status and Actions box to initiate the printer clear action.

- Use the SERV log to verify that the Clear Printer label was sent to both printers.

- Check that TampStatus is set to “Cleared” in the tracking record.

- Check that LblL1 is set to “Error” for both records in the printer tracking record.

- Clear the PA1Srt shifter.  Notice that the carton passes off the end of the shifter as though it is not there – it is not diverted to either error correction lane.

- Use PNA System Status Window Reset Fault to clear any errors.

- Create another carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton through the UpdPA1P1Send update point.

- Open the PrintApplyLine table and find the ClearLineSignal column.  Force the PA1P1ClearLine signal ON, then OFF again.

- Use the SERV log to verify that the Clear Printer label was sent to both printers.

- Check that TampStatus is set to “Cleared” in the tracking record.

- Check that LblL1 is set to “Error” in the printer tracking record.

# Wait Label and ON/OFF Speed Control Tests

We have already configured a record in table SpdControl.

We have already added the PA1P1LabelWait entry in table PrinterUpdPnts.

Set PrintApplyLine_PrintLineSpdCtl to PA1Mtr1 – the same as PrintLineCnvMtr and set SpeedACTION = UnFire.

Add a signal in table PrinterBoss for the PA1Prt1 printer:  LabelOnTampIn = PA1P1LOT.

Restart FPC to make the changes take effect.

Timer PA1P1TampCycle in table timemenu has been configured to 60 seconds, which will allow enough time to manually force the LabelOnTampIn signal ON for testing.

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- In table PrinterUpdPnts, verify that pCurrSpeed1 and pCurrSpeed2 are set to 100 for the PA1P1LabelWait entry.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the UpdPA1P1LabelWait update point.

- Verify the pCurrSpeed1 and pCurrSpeed2 values are now set to 0.

- Verify that PA1Mtr1 is OFF.

- Force the PA1P1LOT tamp signal ON to simulate the tamper indicating the label is ready.

- Verify that PA1Mtr1 is now ON and PCurrSpeed1 and pCurrSpeed2 are no longer zero.

# Wait Label (Error Signal) Tests

Verify that an error occurs if the PrinterBoss_LabelOnTampIn signal does not turn ON before the PrinterUpdPnts_CycleTmr expires.

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the UpdPA1P1LabelWait update point.

- Verify timer PA1P1TampCycle is timing.

- Verify that when the timer expires, PrinterBoss_Status = “Error  Label not on Tamp” and PrinterUpdPnts_ErrorSignal (PA1ERR3) turns ON.

- Use Reset Fault to clear the error.

- Verify that PA1ERR3 is now OFF.

# History Count Test for Verify Scanners

Verify that table ScnDeviceErrIO has references filled in in the HRRecord column.  If it does not, use the ConfigHistScrn “wizard” (Configuration Menu | History Utilities | History Config Wizard) to populate them.  We are interested in all the items related to the scanners for this exercise.  Click “Create” for each scanner.  You can safely click “Create” for all items presented.

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Open the “Scanner Statistics” module (Operator Menu | Scanner Statistics, select “this” AC from the list and click the “Scanner Statistics” button).  Click the “Scanners” tab.  Note the values displayed.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton through the UpdPA1P1Verify update point.

- Verify that the value in the “No Scan” column for the PA1P1Verify scanner has increased.

- Create additional cartons and assign various values for the verification scans.  Use “12345678” to produce a good read.  Verify that the “Scanner Statistics” values increment in the appropriate columns.  Note that cartons with no label requirement will not increment the scanner statistics values for the verification scanners.

# LRM Host Message Tests

A LRM (Label Request Message) is initiated by the PNA AC to request information about the work to be performed on a carton.  In actual use, the LRM is sent to the PNA Engine.  The response indicates verification requirements for the labels to be applied.  The PNA Engine will also return the label images in separate LMI (Label Image Message) messages.  For testing, the PNA Engine is simulated using the configuration we have created in this document.

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.  An LRM is created when the scan data is assigned.

- Verify the MsgCR1 table contains an entry with the scan data, and height, width, and length data (if assigned).

- Check the SERV log for the outbound request and the inbound response from the PNA Engine simulation.  The response will be based on the configuration in the TestConfig and TestCfgPrt tables.  Col3 (sContainerNum) will contain the scan value.  sHeight, sWidth and sLength will contain the dimension information.  sTypeL1 will contain the label type (SHIP), sVerifyL1 will have the string for comparison with the verification scan result (12345678), sTypeL2 will have the label type for the second label (CONT), and sVerifyL2 the verification string for the second label (99999999).

- Verify the printer track table (PrtTrkTable1) has a record for each label to be printed.  ExtTypeL1 = SHIP and ExptL1 = 12345678 for PA1Prt1.  ExtTypeL1 = CONT and ExptL1 = 99999999 for PA1Prt2.

If the test is repeated with ScnScanDevice_ScanType = “SKU Scan”, the scan will be found in the sSKU field of the LRM message instead of the sContainerNum field.  Likewise, for ScanType “RFID Scan”, the scan will be found in the sRFID.

# LVM Host Message Tests

A LVM (Label Verify Message) is initiated by the PNA AC to report the verification results to the PNA Engine.

Adjust the configuration in the PAScannerBoss table to add “Send LVM Msg” in the SendLVM column for both verification entries.  Make sure VerifyType is set to “Verify One” for the first verification point, and “Verify All” for the second verification point.

Success Case:

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the PA1P1Verify update point.

- Assign “12345678” as the verification scan to produce a successful verification result.

- An LVM is generated when the verification scan is processed.

- Verify the LVM contains the expected fields.  The LVM message will be sent using the MsgDC1 table.

- The sContainerNum field will contain the LPN scan.  You should also see these values:  sStatusMsg = 0, sTypeL1 = SHIP, sStatusL1 = 1 (verification success), sTypeL2 = CONT, sStatusL2 = 0 (label has not been verified).

- Shift the carton to the PA1P2Verify update point.

- Assign “99999999” as the verification scan to produce a successful verification result.

- A new LVM message will be constructed in table MsgDC1 with sContainer set to the LPN scan, sStatusMsg = 1, sTypeL1 = SHIP, sStatusL1 = 1, sTypeL2 = CONT and sStatusL2 = 1 indicating a successful verify for both labels.

NoScan Case:

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton through the PA1P1Verify update point to create a NoScan. An LVM message is generated when the NoScan occurs.

- The LVM will contain:  sContainerNum = LPN Scan value, sStatusMsg = 0, sTypeL1 = SHIP, sStatusL1 = 3 (verify scan error), sTypeL2 = CONT, sStatusL2 = 0 (label has not been verified).

- Shift the carton to the PA1P2Verify update point.

- Assign “99999999” as the verification scan to produce a successful verification result.

- A new LVM message will be constructed in table MsgDC1 with sContainer set to the LPN scan, sStatusMsg = 2, sTypeL1 = SHIP, sStatusL1 = 3, sTypeL2 = CONT and sStatusL2 = 1 indicating failure for the first label, and success for the second.

Mismatch Case:

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Shift the carton to the PA1P1Verify update point.

- Assign a scan value of “00000000” to generate a mismatch value.

- The LVM will contain:  sContainerNum = LPN Scan value, sStatusMsg = 0, sTypeL1 = SHIP, sStatusL1 = 2 (label mismatch), sTypeL2 = CONT, sStatusL2 = 0 (label has not been verified).

- Shift the carton to the PA1P2Verify update point.

- Assign “99999999” as the verification scan to produce a successful verification result.

- A new LVM message will be constructed in table MsgDC1 with sContainer set to the LPN scan, sStatusMsg = 2, sTypeL1 = SHIP, sStatusL1 = 2, sTypeL2 = CONT and sStatusL2 = 1 indicating failure for the first label, and success for the second.

# TRM Message Tests

A TRM (Template Request Message) requests a label image from the PNA Engine.  It is activated by setting LabelType = “Host Label” and LoadType = “Host Load” in table LabelTemplates.  The PNA Engine will respond with an LMI message containing the requested ExtLabelType.  The test protocol can be used to simulate this.

For simulation, add two test records in LabelTemplates:

| LabelTemplates Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Printer | LabelNum | LabelType | LoadType | ExtLabelType | LabelFileName |
| TRMTest1 | PA1Prt1 | Label 1 | Host Label | Host Load | NOREAD |  |
| TRMTest2 | PA1Prt2 | Label 1 | Host Label | Host Copy | NOREAD |  |

When the test protocol receives a TRM message, it searches the LabelTemplates table for the first entry that matches the ExtLabelType with pNumBytes greater than zero and returns its contents in an LMI message.  In actual use, the PNA Engine would acknowledge the TRM message, and send an LMI with the proper content.

To trigger the send of a TRM, click LoadFile for the TRMTest1 record, then click Apply.

The Status for both test records will indicate “Host Image Received” and the pNumBytes values will match the NOREAD size of the others.  Table MsgGP1 will contain a new entry with sLabelType = NOREAD.  You can inspect and compare the actual label image contents by double-clicking the pLabelImg field for each row.  You will see the LMI message in the SERV log, as well.

You may delete the TRMTest records when finished.

NOTE:  This might not work correctly anymore.  It generates the outbound message in table MsgGP1, but no LMI is received.

# LMI Message Tests

The test protocol sends required label images when a LRM or TRM message is received.

You can see the transactions in the SERV log.

# PRM Message Tests

The PRM (Printer Ready Message) is used for configurations where the PNA Engine or an external host sends labels directly to printers.  The PRM indicates to FPC that the label is ready to be applied, much like the label on tamp signal (TRK:  I think).

NOTE:  PRM messages are not used in any active installation, to my knowledge.  This test procedure is extremely contrived to force the expected response.  It is likely of no value, but I have preserved it here, for posterity.  TRK.

We have already configured PrintApplyLine_PrintLineSpdCtl and the SpdControl table to stop the line when the speed is set to 0.

In table PrinterUpdPnts, change Type from “Send Label” to “N/A” for the PA1P1LabelSend and PA1P2LabelSend rows.  This will prevent FPC from attempting to send the labels to the printers.

Add a row in table PrinterUpdPnts:

| PrinterUpdPnts Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Type | Print Line | Printer | Label Num | UpdatePnt | Delay Timer | CycleTmr | Error Signal | Error ACTION |
| PA1P1HostWait | Wait Host Message | PA1Line | PA1Prt1 | Label 1 | UpdPA1P1HostWait | N/A | PA1P1HostWait | INVALID |  |

- Clear the PA1Srt shifter.

- Use Reset Fault to clear any errors.  This will initialize the speed to 100 in table PrinterUpdPnts, fields pCurrSpeed1 and pCurrSpeed2.

- Create a carton and shift it through the LPN scan update point to create a NoScan.

- Verify the NoRead label is assigned to the carton and is sent to the printer.

- Shift the carton to the PA1P1HostWait update point.

- Verify that pCurrSpeed1 and pCurrSpeed2 remain set to 100, and PA1Mtr1 is ON.

- Induct a new carton and shift it to the LPN scan update point.

- Assign scan data to the carton.

- Shift the carton to the PA1P1HostWait update point.

- Verify that pCurrSpeed1 and pCurrSpeed2 are now set to 0, and PA1Mtr1 is OFF.

- Find the current tracking table index value (SrtTrack1).

- Open table SendTstPRM.  Fill in row 1 as shown, using the tracking table index value in TrackIndx.  Note that only row 1 is active!  Find MnuIndx by right-clicking the arrow next to ScnScanZone_TrackingTable.  The row number shown in the Menu of Menus is the MnuIndx value.  Enter the scan data you assigned to the carton in ContID.  Use the appropriate label type in ExtLabelType.

| SendTstPRM Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Send | Status | MnuIndx | TrackIndx | ContID | ExtLabelType |
| Test |  |  | 327 | 1 |  | SHIP |

- Click the radio button in Send, then click Apply.

- The Status field should indicate the message was added to the message send queue.

- Verify that pCurrSpeed1 and pCurrSpeed2 are now set to 100, and PA1Mtr1 is ON.

- Verify that the label status shown in LblL1 of the printer tracking record for the printer is set to “Sent”.

- Shift the leading edge of the carton to the location indicated in the printer tracking record TampIndxG1 column.

- Check the SORT log for a message indicating the tamp has been attempted.  If PA1P1LOT is ON, the tamp will be fired, otherwise, a No Label On Tamp error will occur.

- Verify the MsgTST table contains an entry with sContainerNum set to the LPN scan value you used, sControlNum matches the SendTstPRM_TrackIndx value you used, and sTypeL1 matches SendTstPRM_ExtLabelType.

# Two Printers, Round-Robin Tests

PNA lines are often configured with more than one printer for the same label type.  This can increase the maximum throughput of the line and allow the line to continue to run at reduced capacity while one of the printers is serviced, such as when labels or ribbon need to be replenished.

First, we will adjust the test configuration to use our two printers for the same label type.

In table PrintApplyLine, change AssignType to “Round Robin”, and make sure NumPrinters = 2.

In table LabelTemplates, change ExtLabelType for PA1P2Std from CONT to SHIP.  This defines both printers to print the same label.

Clear all entries in table RndRobinLast.

Verify Round-Robin Assignment to Two Printers

- Clear the PA1Srt shifter.

- Create a carton and shift it to the LPN scan update point.

- Assign LPN scan data to the carton.

- Check the printer tracking record Printer field is assigned to printer PA1Prt1 and the ExtType1 field is set to SHIP.

- Create another carton and assign LPN scan data to it.

- Check the printer tracking record Printer field is assigned to printer PA1Prt2 and the ExtType1 field is set to SHIP.

- Create another carton and assign LPN scan data to it.

- Check the printer tracking record Printer field is assigned to printer PA1Prt1 and the ExtType1 field is set to SHIP.