# FortnaPlus Control Transfer Print and Apply

Document Revision: 1.2

Date: 10-Aug-2022

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 12-Apr-2012 | B C Conklin | Original Release |
| 1.1 | 21-Apr-2012 | B C Conklin | Additional general edits |
| 1.2 | 10-Aug-2022 | T R Kremser | Update and general refresh |

Table of Contents

Glossary of Terms	6

1.	Document Objective	7

2.	Transfer Print and Apply Events and Messaging	7

3.	Transfer Print and Apply Tables	9

3.1.	Table XfrCfgPnAInput	9

3.2.	Table XfrCfgPnAOutput	10

3.3.	Transfer PnA Table and Column Details	11

3.3.1.	XfrBoss_XfrZoneID	11

3.3.2.	XfrTrack_XfrZoneID	11

3.3.3.	ScnScanZone_XfrZoneID	11

3.4.	Scanner Configuration	11

4.	Transfer PnA Process Flow	13

4.1.	Transfer 01 (Induction Zone)	13

4.2.	Transfer 02 (PNA Zone)	14

4.3.	Transfer 03 (PNA Verification Zone)	15

5.	Example Pallet PNA Line	16

5.1.	Test Configuration – LRM, LMI, LVM	18

5.1.1.	Machine Table	18

5.1.2.	MsgMap Table	19

5.2.	ESM Test Configuration	20

5.2.1.	Machine Table	20

5.2.2.	MsgMap Table	21

5.2.3.	SendTstCRR Table	22

6.	Transfer PNA Monitoring Screen	22

6.1.	Table XfrPNAMonitorConfig	25

6.2.	Induction	27

6.3.	Print and Apply	29

6.4.	Verification	31

6.5.	Operator Controls	32

7.	Transfer PnA Test Plan	33

7.1.	Machine Table	34

7.2.	MsgMap Table	35

7.3.	Flagmenu Table	36

7.4.	Configio Table	36

7.5.	Conveyor Table	37

7.6.	Batches Table	38

7.7.	Transfer Configuration	39

7.8.	ScnScanZone Table	40

7.9.	ScnScanDevice Table	40

7.10.	XfRouteBoss Table	41

7.11.	XfRouteTable Table	41

7.12.	XfrBoss Table	42

7.13.	XfrTree Table	42

7.14.	XfrConfig Table	42

7.15.	XfrCfgDelayTmr Table	43

7.16.	XfrCheckSet Table	43

7.17.	XfrGoSet Table	44

7.18.	PrintApplyLine Table	44

7.19.	XfrTrig Table	45

7.20.	PrinterBoss Table	45

7.21.	XfrCfgPnAInput Table	46

7.22.	XfrCfgPnAOutput Table	47

7.23.	PAScannerBoss Table	47

7.24.	PAPrinterScanAssign Table	47

7.25.	TestConfig Table	48

7.26.	TestCnfgPrt Table	48

7.27.	LabelTemplates Table	49

7.28.	SendTstCRR Table	49

7.29.	Printer Scan Test	50

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Glossary of Terms

AC – Automation Controller

Container - A general term used to describe items that can be transported by the system.

ESM – External Scan Message.  A message from the Level 2 software to the AC that supplies the induction scan data for an incoming container

FPC – FortnaPlus Control

IO – Input – Output.  Electrical signals from/to field devices and the hardware used to transmit them.

LMI – Label Image Message, a message from the Level 2 software to the AC that contains the label data to be transmitted to a destination printer.

LOTAR – Label On Tamp And Ready.  An input from a PNA applicator used to indicate that a label has been successfully printed and positioned onto the tamp head and is ready for application to the container.

LPN – License Plate Number, a unique identifier for a container on a conveyor system.

LRM – Label Request Message.  A message used to request the labeling requirements and specifics for a particular container.

MDR – Motor Driven Roller

PE – Photoeye

PNA – Print and Apply.  Sometimes PnA or PandA.

QNX – The real-time operating system used on the AC.

RDC – Regional Distribution Center

SKU – Stock Keeping Unit.  An identification number for a particular type of item in inventory.  Uniquely identifies a type of product but is not unique to the individual container.

Tamp – A device that applies a printed label onto a container.

UPS – Uninterruptible Power Supply

WM – Warehouse Management.  The software layer that processes product data according to business rules.

ZPL – Zebra Programming Language.  A programming language for commanding label printers and formatting labels.

# Document Objective

This document describes enhancements made to the Transfer module to integrate it with the Print and Apply module, and how to configure and use them.

TRK NOTE:  This document is incomplete.  It still contains a fair amount of useful information on Transfer Print and Apply.  I have consolidated information from several documents, eliminated a lot of redundant information, and fleshed out a lot of details where I was able to.  To my knowledge, the only installation of this anywhere is at McKesson on AC7 and AC7A, which handles pallets.  It doesn’t seem to warrant working through the test plan examples to find what is missing.  Sorry about that.  You will find my notes in this document on where I got stuck and what I think might be needed, for what that’s worth.

# Transfer Print and Apply Events and Messaging

Print and Apply module operation can be described as a sequence of events:

- Container Induction – The container is inducted.  Transfer and Print Apply tracking records are created.

- Container LPN/SKU Scan – Scan results are recorded in the tracking record.

- Label Request Message (LRM) – The LRM is transmitted to the Print and Apply engine application (Level 2).  The message includes the container scan from the LPN/SKU scanner.

- Label Request Message Reply – The PnA engine responds with a message that identifies the type of label(s) that are to be applied and the verification string(s) for those label(s).

- Label Image Message (LMI) – The PnA engine transmits the label(s) as ZPL files.  They are stored by the AC until needed by the printer.

- Image Transmission to Printer – The label image from the LMI message is transmitted to the printer.

- Printer Control – Control signals between the AC and the printer/applicator are used to apply the printed label at the tamp point.

- Container Verify Scan – The applied label is scanned, and the results are reported.  The tracking record is updated.

- Label Verify Message (LVM) – The verification results are reported to the PnA engine via this message.

This sequence is the same for sorter-based and transfer-based configurations.  For transfer-based setups, some additional configuration is required.

The Transfer module triggers PNA events and messaging through configurations found in the XfrCfgPnAInput and XfrCfgPnAOutput tables.  Configuration options provide the ability to trigger PNA events based on specific states within the transfer control sequence.

The transfer module is a linear state machine, in which a transition from one state to another cannot be made until all defined inputs for the next state have been satisfied.  Once a transition has been completed, outputs can be executed.

The following PNA events are integrated into the transfer state machine logic as state INPUTS as defined in table XfrCfgPnAInput:

- Receive Label (LRM and LMI) – This input requires that both LRM and LMI messages have been received in response to an outbound LRM message.

- Label Verification – This input requires that a verification scan has been received and label verification completed (success or failure).

The following PNA events are integrated into the transfer state machine logic as state OUTPUTS as defined in table XfrCfgPnAOutput:

- Get Label (LRM) – Transmit an LRM to the PnA engine.

- Activate Printer – Fire the printer tamp.  The printer ready signals must be ON.

# Transfer Print and Apply Tables

# Table XfrCfgPnAInput

This table contains PNA input conditions and their associated transfer, state, and printer.

| XfrCfgPnAInput Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this input action. |
| Desc | String | Optional description. |
| pStatus | String | Updated by the software as events occur. |
| XfrConfig | PopUp | Choose the XfrConfig record associated with this entry. |
| State | PopUp | Choose the transfer state associated with this entry. |
| PrintLine | PopUp | Choose the print and apply line associated with this entry. |
| Printer | PopUp | Chose the PrinterBoss record associated with this entry |
| LabelNum | PopUp | Choose the label number to associate with this entry. Usually Label 1. Label 2 is used for label-over-label applications. |
| PrintCondition | PopUp | Choose the input condition for this entry: Receive Label (LRM and LMI) – This entry is triggered by receipt of a message. Label Verification – This entry is triggered by receipt of a verification scan. |
| VerifyScanZone | PopUp | Choose the ScnScanZone record associated with this entry. |
| LogLevel | PopUp | Choose the log detail level desired for this entry. |
| Machine | PopUp | Choose the AC associated with this entry. |
| OrSignal1 thru OrSignal7 | PopUp | Choose up to seven input signals. |
| OrSense1 thru OrSense7 | PopUp | Set the sense (ON or OFF) of each input signal defined in the corresponding OrSignal fields that will define the logical TRUE state for that field. |
| pInputState | Int | Set by the software to define the overall state of this entry. |
| pOrState1 thru pOrState7 | Int | Set by the software while determining the logical OrSignal states. |
| pCfgErr | Radio | If set (by the software), indicates there is a configuration error associated with this entry. |
| pCfgMessage | String | The software will display a message here when pCfgErr is set that may help to find configuration errors. |
| pLabelVerified | Int | Set by the software to determine the verification status for the current container. |

# Table XfrCfgPnAOutput

This table contains PNA output actions and their associated transfer, state, and printer.

| XfrCfgPnAOutput Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for this output action. |
| Desc | String | Optional description. |
| pStatus | String | Updated by the software as events occur. |
| XfrConfig | PopUp | Choose the XfrConfig record associated with this entry. |
| State | PopUp | Choose the transfer state associated with this entry. |
| PrintLine | PopUp | Choose the print and apply line associated with this entry. |
| PrintAction | PopUp | Choose the action for this entry: Get Label (LRM) – Send an LRM message Activate Printer – Act upon the LRM data to print and apply labels |
| LogLevel | PopUp | Choose the log detail level desired for this entry. |
| Machine | PopUp | Choose the AC associated with this entry. |
| pCfgErr | Radio | If set (by the software), indicates there is a configuration error associated with this entry. |
| pCfgMessage | String | The software will display a message here when pCfgErr is set that may help to find configuration errors. |

# Transfer PnA Table and Column Details

Points of connection between the sorter-based and the transfer-based PnA.

# XfrBoss_XfrZoneID

The XfrZoneID column in table XfrBoss defines the zones that the transfer boss controls.

# XfrTrack_XfrZoneID

The XfrZoneID column in table XfrTrack is updated when a transfer is triggered.  The value is taken from XfrBoss_XfrZoneID.  You can use this to determine which physical zone a particular container is in at any time.

# ScnScanZone_XfrZoneID

Verification scans are connected to the associated tracking record (table XfrTrack) through ScnScanZone_XfrZoneID so they can be processed.

# Scanner Configuration

The Scanner Configuration table (ScnScanDevice) is used to define the scanners that will be used for the Print and Apply Line.  Scanners may be configured to read an LPN or SKU to identify the container.  Scanners may also be configured to verify an RFID Tag, a SKU, or up to six Labels for any one container.

The Print Apply Scanner configuration table (PAScannerBoss) includes selections to define processing for each Verify Scan event in column VerifyType:

      - N/A – don’t verify any labels at this scan event

      - Verify One - Verify one label – the current label being scanned.  Errors are captured in the Scan Device (ScnDeviceErrIO) and Scan Zone Error (ScnZoneErrIO) tables.  This is most often used in label-over-label configurations to verify each label as it is applied.

      - Verify All - Verify all the labels configured for this line regardless of the zone.  Errors are captured in the Scan Zone Error table (ScnZoneErrIO).  This is most often used for single label configurations or for the final label for a label-over-label line.

To configure to capture specific exception handling events such as line stops and other various errors, and to activate beacons, refer to the FPC-Scanner-Control-Configuration document.

Each Print Apply Line will have a minimum of two scan zones: A container scan zone and a verify scan zone.  There may be a third scan zone for a line with a label-over-label configuration.

Verify Scan Zone

A verify scan zone may have several scanners that verify specific labels or fields, such as:

      - Top Label – Verify that a Top Label was applied.

      - Side Label – Verify that a Side Label was applied.

      - SKU – Verify the correct SKU was labeled with the Shipping Label.

A Scan Zone (or combination of scanners) will record good scan events and verify error scan events.  A verify error scan event is recorded in the Scan Zone Error configuration table (ScnZoneErrIO) as a BadRead.

Verify Device

A scan device will record good scan events, NoReads, Scan Conflicts, No Scans, and verify errors based on the type of device error detected.  A verify error is recorded in the Scan Device Error configuration table (ScnDeviceErrIO) as a BadRead.  A Label mismatch error is recorded as a BadRead.

Note: An error is only recorded in the error statistics if the container has a label requirement and a verify requirement.  If multiple labels are required for a container, the error processing logic will only count the first error found.  This prevents the system from counting errors for labels that have a dependency on a prior label, i.e., in label-over-label applications.

Note: The Print Apply module will activate the “Verify Error” button on the Print Apply Status Screen and will update the error message if the PrintLine column is selected in the ScnDeviceErrIO or the ScnZoneErrIO table.

# Transfer PnA Process Flow

This section describes the process flow for the Transfer Print and Apply process.

# Transfer 01 (Induction Zone)

This transfer is triggered when an induction scan event occurs – either by the scanner sending data to the AC, by an ESM (a scan event detected by Level 2), or, by a NoScan event generated by the AC when the NoScan timer has timed out (if no scan was received).

Transfer Idle

- The receipt of a scan updates the tracking records and triggers the transfer to run.

Transfer State 1

- This transfer triggers another transfer to run using the “Trig Next” configuration option, which will create a new set of tracking records with a copy of the tracking information.

Note: A second transfer is started using “Trig Next” to update the XfrZoneID column.  This is the only action that updates the XfrZoneID column.

# Transfer 02 (PNA Zone)

This transfer will send the LRM to retrieve the label requirements for the container.  The LRM response will determine which printer applicator(s) to use.

Transfer State 1

- A record in the XfrCfgPnaOutput table triggers the LRM.

- The PNA module transmits the LRM to the PnA engine requesting the label information.

Transfer State 2

- A record in table XfrCfgPnAInput associated with State 2 causes the transfer to wait before entering this state until the label requirements are received from the PnA engine.

  - Note: The LOTAR signals should be checked based on the Label Requirements.  This is feature that needs to be added.

- Label requirements are received and the transfer moves to State 2

Transfer State 3

- A record in table XfrCfgPnaOutput associated with State 3 activates the printer(s) based on the LRM response.

Transfer State 4

- The tamper is fired in this state.

- The transfer will wait for the Cycle Complete signal to turn ON.

- The transfer will wait for the Tamp Home inputs to turn ON.  The Tamp Home signal indicates the tamp mechanism is clear – to prevent damage during container movement.

- The transfer moves to State 5

Transfer State 5

- “Trig Next” is used to move the container through the verification scanners.

# Transfer 03 (PNA Verification Zone)

As the pallet moves into the zone, the verification scanners will supply scan data from both the left and right “size” labels.

Transfer State Idle

- The transfer moves to State 1, when activated, and when State 1 inputs are satisfied.

Transfer State 1

- The transfer will start the pallet moving.

Transfer State 2

- The transfer will complete the move when the scan zone photoeye is activated.

Transfer State 3

- A record in table XfrCfgPnAInput associated with State 3 causes the transfer to wait for the left-side scan zone data.

Transfer State 4

- A record in table XfrCfgPnAInput associated with State 4 causes the transfer to wait for the right-side scan zone data.

- Once both scans are received, the scan data is verified and the results are posted into the printer tracking record.

- An LVM is transmitted to the PnA engine with the verification results.

- The transfer moves to state 5.

Note: Even though the transfer checks the verify scans in a specific order, the transmission of the scans and the logic to store the scans in the tracking record is independent and the scans can be received in any order.

Note: The transfer verification process must receive the scans before the “OR” signal is detected.  If the “OR” signal is detected before the scan is received the software will process the scan as an error.

# Example Pallet PNA Line

In this example, a pallet induction line conveys incoming pallets to a storage area.  During induction, pallets pass through five main operations:

- Operator Drop-Off

- Pallet Sizing

- Identification

- Print and Apply

- AGV Pick-Up

The screenshot below shows the pallet induction line.

The example system is controlled using the Transfer and Print and Apply modules.  This section provides details on how this is done.

# Test Configuration – LRM, LMI, LVM

The Transfer Print and Apply module does not support the TestMode flag in the PrintApplyLine table.  To test a transfer, the LRM, LVM and LMI must be explicitly configured with the test protocols, loop-back style.

# Machine Table

The following four entries define port 9001 as a Server/Client to simulate Printer 1 and Printer 2.

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Transmit_Retry | Process | LogLevel |
| MHOST1LS | TCPSERV_LISTEN | AlphaNum | 9001 | 10 | SERV | Level 1 |
| MHOST1RW | TCPSERV_RDWR | AlphaNum | 9001 | 10 | SERV | Level 1 |
| PR1CL | CONNECTC_TCPCLIENT | AlphaNum | 9001 | 10 | SERV | Level 1 |
| PR2CL | CONNECTC_TCPCLIENT | AlphaNum | 9001 | 10 | SERV | Level 1 |

The next three entries define Port 9002 as a Server/Client to simulate Label Request Messages for PNA Line 1.

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Transmit_Retry | Process | LogLevel |
| MHOST2LS | TCPSERV_LISTEN | PaTest | 9002 | 10 | SERV | Level 1 |
| MHOST2RW | TCPSERV_RDWR | PaTest | 9002 | 10 | SERV | Level 1 |
| MHOST2CL | CONNECT_TCPCLIENT | PaComm | 9002 | 10 | SERV | Level 1 |

The next three entries define Port 9003 as a Server/Client to simulate Label Request Messages for PNA Line 2.

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Transmit_Retry | Process | LogLevel |
| MHOST3LS | TCPSERV_LISTEN | PaTest | 9003 | 10 | SERV | Level 1 |
| MHOST3RW | TCPSERV_RDWR | PaTest | 9003 | 10 | SERV | Level 1 |
| MHOST3CL | CONNECT_TCPCLIENT | PaComm | 9003 | 10 | SERV | Level 1 |

The next three entries define Port 9004 as a Server/Client to simulate Label Image Messages for PNA Lines 1 and 2. The test software places label images into the send queue table SendTstLabels.  The messages are sent using the PaLblImg protocol, which stores the labels in the receive queue selected in the PrintApplyLine table.  The LMI message contains the information needed to determine the PNA line associated with message.  Received labels are then sent to the printer when needed.

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Transmit_Retry | Process | LogLevel |
| MHOST4LS | TCPSERV_LISTEN | PaLblImg | 9004 | 10 | SERV | Level 1 |
| MHOST4RW | TCPSERV_RDWR | PaLblImg | 9004 | 10 | SERV | Level 1 |
| MHOST4CL | CONNECT_TCPCLIENT | PaLblImg | 9004 | 10 | SERV | Level 1 |

# MsgMap Table

Open the MsgMap table (Configuration Menu | Communication Utilities | Message Map).

Make the following entries in the MsgMap table to define the connections between the Machine table entries and the tables where message data will be stored.

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_ Name | Recv_ Msg_Type | Send_ Msg_Type | Ack_ Msg_Type | Menu_Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| PA1 Msg Send | MHOST2CL | PA1 | PA1 | PA1 | MsgCR1 | 1 | 54 | 55 | 56 | 0 |
| PA1 Tst Recv | MHOST2RW | PA1 | PA1 | PA1 | MsgGP1 | 1 | 54 | 55 | 0 | 0 |
| PA2 Msg Send | MHOST3CL | PA1 | PA1 | PA1 | MsgCR2 | 1 | 54 | 55 | 56 | 0 |
| PA2 Tst Recv | MHOST3RW | PA1 | PA1 | PA1 | MsgGP2 | 1 | 54 | 55 | 0 | 0 |
| Label Send | MHOST4CL | PA1 | PA1 | PA1 | SendTstLabels | 1 | 6 | 7 | 8 | 0 |

NOTE: PA1 refers to the name of the AC in the table above.  Substitute the appropriate name as needed.

# ESM Test Configuration

Use the following configuration to simulate an ESM message transaction with level 2.  The test protocol understands the ESM message.

# Machine Table

Open the Machine table (Configuration Menu | Communication Utilities | Machine).

The following three entries define Port 9210 as a Server/Client to simulate an ESM connection with Level 2.  The Machine Name must be defined in the /etc/hosts file (127.1     ESM_CL)

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_ Name | Connection_Type | Communication_ Protocol | Machine_Port | Transmit_Retry | Process | LogLevel |
| ESM_LS | TCPSERV_LISTEN | SrtCommSeq | 9210 | 10 | SERV | Level 1 |
| ESM_RW | TCPSERV_RDWR | SrtCommSeq | 9210 | 10 | SERV | Level 1 |
| ESM_CL | CONNECT_TCPCLIENT | SrtTestSeq | 9210 | 10 | SERV | Level 1 |

# MsgMap Table

Open the MsgMap table (Configuration Menu | Communication Utilities | Message Map).

Make the following entries in the MsgMap table to define the connections between the Machine table entries and the tables where message data will be stored.

| MsgMap Table |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_ Name | Machine_ Name | Recv_ Msg_Type | Send_ Msg_Type | Ack_ Msg_Type | Menu_ Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor | Record |
| ESM Recv | ESM_RW | ALL | ALL | ALL | MsgXR1 | 1 | 54 | 67 | 0 | 0 |
| ESM Test | ESM_CL | ALL | ALL | ALL | MsgTST | 1 | 54 | 67 | 68 | 0 |

Note: The test protocol adds messages to the MsgTST table.

# SendTstCRR Table

Use the SendTstCRR table (Configuration Menu | Transfer Utilities | Transfer Messages | Config CRR Test Message) to generate the ESM message.

Enter the scan value (Example: 123456) into the Z1 column.  When you click the “Apply” button the software will create an ESM message in the MsgTST table.  The message will be sent to simulate the Level 2 system.  The software will automatically clear the Z1 column after the message is sent.

| SendTstCRR Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Status | Z1 | Z2 | Z3 | Z4 | Z5 |
| TestESM | Enter Scan | <ScanValue> |  |  |  |  |

# Transfer PNA Monitoring Screen

As a layer on top of the belt PNA module, the Transfer Print and Apply feature generates additional information during use that is not covered by the standard Print and Apply Operator’s interface screen.  The Transfer PNA Monitoring Screen helps to address this issue by consolidating the transfer PNA data on one screen to help recover from errors more quickly.

The screen shows the state of input and output devices by color.  Green indicates the device is ON, red indicates it is OFF.

You will find XfrPNAMonitorScreen at: Configuration Menu | Transfer Utilities | Transfer PNA Monitor | Transfer PNA Monitor Screen.  The companion configuration table, XfrPNAMonitorConfig is found in the same location.  If so configured, the XfrPNAMonitorScreen will appear on the Operator Menu, as well.

The XfrPNAMonitorScreen can be divided into four functional sections for the purpose of this discussion.  The top portion of the screen shows information about the induction portion of the PNA line.  The next group of fields shows information about the Print and Apply portion – the printer and applicator, and associated labels and error signals.  The third group of fields shows information about the verification portion.  The fourth section is composed of the three controls at the bottom of the screen that let you select the area to monitor and to clear the displayed data and enable or disable monitoring.

TRK NOTE:  This feature could stand some improvement.  Selecting the “Transfer PNA Config” on the Transfer PNA Monitoring Screen does NOT update the rest of the fields, such as Induct PE, making it really hard to use if more than one configuration is required on a single AC.  The menu entry for this screen has been dropped from the standard Operator Menu configuration but could be added if needed.  I think it would only be of use during initial setup and for troubleshooting, so it is not something that would be used by an operator, anyway.  Proper configuration of the security settings on the individual fields would be the only means to avoid inadvertent or intentional changes to the configuration by the operator.  As this feature has only ever been used on McKesson AC7 and AC7A, these deficiencies will likely never be corrected.

First, configure the XfrPNAMonitorConfig table, as described in the next section, then proceed to configure the fields in this screen, as described in the following sections.

# Table XfrPNAMonitorConfig

The XfrPNAMonitorConfig table (Configuration Menu | Transfer Utilities | Transfer PNA Monitor | Transfer PNA Monitor Config) is needed to define additional items required by the Transfer PNA Monitoring Screen.  It is used to gather data displayed on that screen.

| XfrPNAMonitorConfig Table |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Name | String | Enter a unique name for the record. This is linked to the Transfer PNA Monitoring Screen with the Transfer PNA Config field. |
| Enabled | Radio | Use this button to manually enable or disable this record. |
| PNALine | PopUp | Choose the Print and Apply line associated with this record. |
| ESM Scan Zone | PopUp | Choose the scan zone associated with this record. |
| ESM Received | String | Set by the software to the scan data received via the ESM. |
| ESS Sent | String | Set by the software to display the status of the most recent ESS sent. |
| LRM Received | String | Set by the software to display the status from the last LRM received. |
| Label 1 | String | Set by the software to display the ExtLabelType for Label 1 from the most recent LRM. |
| Label 2 | String | Set by the software to display the ExtLabelType for Label 2 from the most recent LRM. |
| LabelCount | Int | Set by the software to show the number of LMI messages received with the last LRM. |
| Verify Scan 1 | String | Set by the software to show the last scan received from the first verification scanner. |
| Verify 1 Scan Device | PopUp | Choose the scan device for the first verification scanner. |
| Verify Scan 2 | String | Set by the software to show the last scan received from the second verification scanner. |
| Verify 2 Scan Device | PopUp | Choose the scan device for the second verification scanner. |
| Induct Zone Config | PopUp | Choose the transfer for the induction zone. |
| PNA Zone Config | PopUp | Choose the transfer for the PNA zone. |
| Verify Zone Config | PopUp | Choose the transfer for the verification zone. |
| pProcESM | Int | An internal flag that indicates an ESM is ready for processing. |
| pProcESS | Int | An internal flag that indicates an ESS is ready for processing. |
| pProcLRM | Int | An internal flag that indicates an LRM is ready for processing. |
| pProcVerify1 | Int | An internal flag that indicates scan data from the first verification scanner is ready for processing. |
| pProcVerify2 | Int | An internal flag that indicates scan data from the second verification scanner is ready for processing. |
| pInduct | Int | An internal flag that tracks the state of Induct PE. |

Proceed to configure the Transfer PNA Monitoring Screen fields in the following sections.

# Induction

As a pallet is inducted, its identity is determined by use of an ESM message from the WMS.  The fields in the top portion of the Transfer PNA Monitoring Screen show information regarding induction.

| Field | Use | Configure | Notes |
| --- | --- | --- | --- |
| Induct PE | ON: A pallet is blocking the eye. The transition ON initiates tracking of a new pallet. OFF: No pallet is present. | Choose the induction PE for the line. | When the Induct PE is blocked, the text fields on this screen will be cleared. |
| ESM Received | Shows the “scan” data received in the last ESM message. If this field is blank, no ESM has been received. |  | Level 2 delivers the identification information for the pallet via an ESM. |
| ESS Sent | The status sent in the ESS message for this pallet. Okay: ESM has been received and pallet has been inducted *** No Product Present ***: An ESM was received, but the Induct PE is not ON. *** Induct Failed ***: An ESM was received but the pallet induction failed. *** Transfer Not Ready ***: An ESM was received but the pallet could not be inducted because one or more of the transfer’s prerequisite conditions was not met. If this field is blank, no ESS has been generated. |  | FPC acknowledges receipt of the ESM by sending an ESS. |
| Induct Zone State | The current transfer State for the induction zone. |  |  |
| Induct Zone Status | The text description of the current transfer State. |  |  |
| Induct Zone Motor | ON: The motor for the induction section is being told to run. OFF: The motor for the induction section is not running. | Choose the motor for the induction section. |  |

# Print and Apply

The pallet proceeds to the print and apply section.  This portion of the Transfer PNA Monitoring Screen shows the status key items related to the printer and applicator.

| Field | Use | Configure | Notes |
| --- | --- | --- | --- |
| PNA Zone PE | ON: A pallet is present in the zone. OFF: No pallet is present. | Choose the presence eye for this area. | This eye must be OFF before a pallet will be allowed to move into the zone. |
| P1 Tamp Home | ON: The tamper is in the home position. OFF: The tamper is NOT in the home position. | Choose the input signal from the applicator that indicates the tamper for the first printer is in the home position |  |
| P2 Tamp Home | ON: The tamper is in the home position. OFF: The tamper is NOT in the home position. | Choose the input signal from the applicator that indicates the tamper for the second printer is in the home position |  |
| P1 Error | ON: The applicator is indicating an error has occurred. Corrective action must be taken on the applicator or printer. OFF: No error. | Choose the input signal that the applicator for the first printer will use to report an error condition. | If this signal is ON, a pallet will not be able to move into the zone. |
| P2 Error | ON: The applicator is indicating an error has occurred. Corrective action must be taken on the applicator or printer. OFF: No error. | Choose the input signal that the applicator for the second printer will use to report an error condition. | If this signal is ON, a pallet will not be able to move into the zone. |
| P1 LOTAR | ON: The applicator is indicating that a label is present on the tamp head and ready to be applied. OFF: No label is ready for application. | Choose the input signal from the applicator for the first printer that indicates a label is present on the applicator tamp head. | The LOTAR signal must be ON to allow the pallet to enter the zone. |
| P2 LOTAR | ON: The applicator is indicating that a label is present on the tamp head and ready to be applied. OFF: No label is ready for application. | Choose the input signal from the applicator for the second printer that indicates a label is present on the applicator tamp head. | The LOTAR signal must be ON to allow the pallet to enter the zone. |
| LRM Status |  |  | The status taken from the LRM message response. Also contains the scan zone name. |
| Label 1 |  |  | The ExtLabelType string from the LRM for label 1. |
| Label 2 |  |  | The ExtLabelType string from the LRM for label 2. |
| LabelCount |  |  | The number of labels received in the form of individual LMI messages. |
| PNA Zone State | The current transfer State for the PNA zone. |  |  |
| PNA Zone Status | The text description of the current transfer State. |  |  |
| PNA Zone Motor | ON: The motor for the PNA section is being told to run. OFF: The motor for the PNA section is not running. | Choose the motor for the PNA section. |  |

# Verification

In the verification section, the newly-applied labels are scanned and compared with the expected values.

| Field | Use | Configure | Notes |
| --- | --- | --- | --- |
| Scanner 1 Verify | Shows the result read from the first verification scanner. |  |  |
| Scanner 2 Verify | Shows the result read from the second verification scanner. |  |  |
| Verify Zone State | The current transfer State for the verification zone. |  |  |
| Verify Zone Status | The text description of the current transfer State. |  |  |
| Verify Zone Motor | ON: The motor for the verification section is being told to run. OFF: The motor for the verification section is not running. | Choose the motor for the verification section. |  |

# Operator Controls

The operator can use the three items at the bottom of the screen to control what is displayed.

| Field | Use | Configure | Notes |
| --- | --- | --- | --- |
| Transfer PNA Config |  | Choose the record from table XfrPNAMonitorConfig that will be used to feed data to this screen. | Choosing a different record here does not alter the items selected in the other fields on this screen! |
| Clear Data | Click this button to clear the text fields on this screen |  | The same fields are automatically cleared when a new pallet is introduced in the induction zone (when the Induct PE turns ON) |
| Monitoring Enabled | Selected: Monitoring is active. De-selected: Monitoring stops. |  | Select this to enable the feature. |

# Transfer PnA Test Plan

This section describes how to create a test configuration for a Transfer Print and Apply line with one printer, using static label assignments, then manually simulate the motion of a container through that line.

In the following sections, the contents of various tables are described.  Only the fields needed for this test configuration are shown.  Leave all the other fields at their default values.

NOTE:  Define an entry in the /etc/hosts file for the client connections like this:
  127.0.0.1     P1CL LRM LMI CRR

# Machine Table

Open the Machine table (Configuration Menu | Communication Utilities | Machine).

Make the following entries to define a connection on port 9001 as a Server/Client to simulate Printer 1.  Port 9001 is the same port used by the printer for “Raw TCP/IP Input”.

| Machine Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Transmit_Retry | Process | Log Level |
| P1LS | TCPSERV_LISTEN | AlphaNum | 9001 | 10 | SERV | Level 1 |
| P1RW | TCPSERV_RDWR | AlphaNum | 9001 | 10 | SERV | Level 1 |
| P1CL | CLOSE_CLIENT | AlphaNum | 9001 | 10 | SERV | Level 1 |

Make the following entries to define a connection on port 9002 as a Server/Client to simulate Level 2 Label Request Messages (LRM) for PNA Line 1.

| Machine Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Transmit_Retry | Process | Log Level |
| LRMLS | TCPSERV_LISTEN | PaTest | 9002 | 10 | SERV | Level 1 |
| LRMRW | TCPSERV_RDWR | PaTest | 9002 | 10 | SERV | Level 1 |
| LRM | CONNECT_TCPCLIENT | PaComm | 9002 | 10 | SERV | Level 1 |

Make the following entries to define a connection on port 9003 as a Server/Client to simulate Level 2 Label Images Messages (LMI) for PNA Line 1.  The test software places label images into the send queue table SendTstLabels.  The messages are sent using the PaLblImg protocol, which then stores the labels in the receive queue selected in the PrintApplyLine table (RcvLabels1).  The LMI message contains information about the print line the message is associated with.  Received Labels are then sent to the printer through the Send Label Queue table (SndLabelQue1).

| Machine Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Transmit_Retry | Process | Log Level |
| LMILS | TCPSERV_LISTEN | PaLblImg | 9003 | 10 | SERV | Level 1 |
| LMIRW | TCPSERV_RDWR | PaLblImg | 9003 | 10 | SERV | Level 1 |
| LMI | CONNECT_TCPCLIENT | PaLblImg | 9003 | 10 | SERV | Level 1 |

Make the following entries to define a connection on port 9004 as a Server/Client to simulate Level 2 Container Routing Requests (CRR) that will trigger the transfer to run.

| Machine Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine_Name | Connection_Type | Communication_Protocol | Machine_Port | Transmit_Retry | Process | Log Level |
| CRRLS | TCPSERV_LISTEN | SrtTest | 9004 | 10 | SERV | Level 1 |
| CRRRW | TCPSERV_RDWR | SrtTest | 9004 | 10 | SERV | Level 1 |
| CRR | CONNECT_TCPCLIENT | SrtComm | 9004 | 10 | SERV | Level 1 |

# MsgMap Table

Open the MsgMap table (Configuration Menu | Communication Utilities | Message Map).

Make the following entries in the MsgMap table to define the connections between the Machine table entries and the tables where message data will be stored.

| MsgMap Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message_Name | Machine_Name | Recv_ Msg_Type | Send_ Msg_Type | Ack_ Msg_Type | Menu_Name | Low_ Cursor | High_ Cursor | Stat_ Cursor | Key_ Cursor |
| PA1LRM | LRM | CP2 | CP2 | CP2 | MsgCR1 | 1 | 66 | 67 | 68 |
| PA1TstRecv | LRMRW | CP2 | CP2 | CP2 | MsgTst | 1 | 66 | 67 | 68 |
| PA1LabelSend | LMI | CP2 | CP2 | CP2 | SendTstLabels | 1 | 6 | 7 | 8 |
| CRRMsgSend | CRR | CP2 | CP2 | CP2 | MsgCR2 | 1 | 66 | 67 | 68 |
| CRRTstRecv | CRRRW | CP2 | CP2 | CP2 | MsgTst | 1 | 66 | 67 | 68 |

NOTE: CP2 refers to the name of the AC in the table above.  Substitute the appropriate name if needed.

# Flagmenu Table

Open the flagmenu table (Configuration Menu | Flag Utilities | Integer Flags).

Find row 101, OCTAL_MODE, and make sure Flag_Value = 1.  Once set, IO addresses will be represented in Octal notation.

Exit FortnaPlus and restart so the new entries will take effect.

# Configio Table

Open the Configio table (Configuration Menu | IO Utilities | IO Configure).

Make the following entry in the Configio table, in row 0, to define a word of memory bits.

| Configio Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Desc | Bank | Octal_Word | LoHi | In_Out | I_O_Type | Interface | Process |
| MEM200 | 0 | 200 | Both | 1111111111111111 | Memory | Memory | SORT |

# Conveyor Table

Use the PartPfgScrn wizard (Configuration Menu | Wizard Utilities | Create Conveyor Part) to add 16 memory bits.  Find the IOMemoryWord field about half-way down the screen and enter 200.  Click Add16MemoryParts (the other end of the same line).  This will create 16 new memory bits in table Conveyor with word address 200.  Right-click the down arrow next to ConveyorPart to open the Conveyor table (Configuration Menu | Standard Controls | IO Conveyor) to see the results.  As you step through each row in table Conveyor, notice that IO_Address_Word = 200 and IO_Address_Bit ranges from 0 to 17 (Octal) for the new MEM0200 parts.  Rename the first ten parts as shown in the table.

| Conveyor Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| IO_Name | IO_Address_Word | IO_Address_Bit | ProcNum | Machine_Name |
| TransferPresence1 | 200 | 0 | CTRL | CP2 |
| Line1VerifyPresence | 200 | 1 | CTRL | CP2 |
| Line1ErrLight | 200 | 2 | CTRL | CP2 |
| PA1Run | 200 | 3 | CTRL | CP2 |
| P1ErrIn | 200 | 4 | CTRL | CP2 |
| P1TampHome | 200 | 5 | CTRL | CP2 |
| P1LOTAR | 200 | 6 | CTRL | CP2 |
| P1CycleComplete | 200 | 7 | CTRL | CP2 |
| P1ResetOut | 200 | 10 | CTRL | CP2 |
| P1TampSolOut | 200 | 11 | CTRL | CP2 |
| Mem0200-12 | 200 | 12 | CTRL | CP2 |
| Mem0200-13 | 200 | 13 | CTRL | CP2 |
| Mem0200-14 | 200 | 14 | CTRL | CP2 |
| Mem0200-15 | 200 | 15 | CTRL | CP2 |
| Mem0200-16 | 200 | 16 | CTRL | CP2 |
| Mem0200-17 | 200 | 17 | CTRL | CP2 |

# Batches Table

Open the Batches table (Configuration Menu | Standard Controls | Batches).

Make the following entries to activate the Scanner Control module, the Sorter Control module, the Transfer module, and the Print and Apply module.  The first three will have entries already defined – just activate them by defining the Machine entries.  You will need to add the Print and Apply items.

| Batches Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| File_Name | HowOften | Increment | Type | Process | Internal Proc | Internal ProcCtrl | Machine | All Machines | Do File |
| Scan Process | 1 | INVALID | Count | CTRL |  | ScnApp CTRL | CP2 | N | N |
| Srt Ctrl | 1 | INVALID | Count | CTRL |  | SrtApp CTRL (CPN) | CP2 | N | N |
| Srt Serv | 1 | INVALID | Count | SERV | SrtApp SERV (CPN) |  | CP2 | N | N |
| Xfr Serv | 1 | INVALID | Count | SERV | XfrApp SERV (CPN) |  | CP2 | N | N |
| Xfr Ctrl | 1 | INVALID | Count | CTRL |  | XfrApp CTRL (CPN) | CP2 | N | N |
| Xfr Run | 1 | INVALID | Count | CTRL |  | XfrProcess (CTRL) | CP2 | N | N |
| PNA Sort | 1 | INVALID | Count | SORT |  | PA SORT (CPN) | CP2 | N | N |
| PNA Ctrl | 1 | INVALID | Count | CTRL |  | PA CTRL (CPN) | CP2 | N | N |
| PNA Serv | 1 | INVALID | Count | SERV | PA SERV (CPN) |  | CP2 | N | N |
| PNA GUI | 1 | INVALID | Count | GUI | PA GUI (CPN) |  | CP2 | N | N |

# Transfer Configuration

Use the transfer configuration wizard, ConfigXfrScrn (Configuration Menu | Transfer Utilities | Configure Transfers), to complete the following steps:

Add a new transfer (click the buttons in the order shown):

- AddScanZone

- AddRouteConfig

- AddXfr

- AddTree

- AddRoute

- AddConfig

- AddCheckSet

- AddGoSet

- AddDelayTimer for State 1 (SelectedState should be State1 by default).

Add another new transfer, following the same sequence as before (click the buttons in the order shown).  Use AddDelayTimer on State1, State2, State3, and State4.  Select each state in SelectedState before clicking the button.

Restart FPC again to allow timers and other items to be automatically generated.

# ScnScanZone Table

Open the ScnScanZone table (Configuration Menu | Scanner Utilities | Scan Zone Config).

Define a scan zone for each of the two transfers we just created.  The transfer wizard created the entries, but you must complete them.

| ScnScanZone Table |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Desc | Scan ZoneId | Compare Scans | Tracking Table | Device Trk Table | Printer Trk Table | Ctrl Process | PrintLine |
| SZ01 |  | SZ01 | N | XfrTrack | XfrDevice |  |  | PA1 |
| SZ02 |  | SZ02 | N | XfrTrack | XfrDevice | PrtTrkTable1 |  | PA1 |

NOTE:  The PA1 entry for PrintLine won’t be defined until later.  You will have to come back to this table to complete the entries then.  TRK: AC7 doesn’t have PrintLine defined!

# ScnScanDevice Table

Open the ScnScanDevice table (Configuration Menu | Scanner Utilities | Scan Device Config).

Define a scan device for the LPN scan and verification scan.

| ScnScanDevice Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | ScanZone | ScanType | UpdatePt | NoRead String | ReadConflict String | StartChar | QtyChars | ChkLength | MaxWeight |
| PNA1 LPN Scan | SZ01 | Scan | Invalid | ????? | ***** | 0 | 0 | N | 0.0 |
| PNA1 P1 Verify | SZ02 | Verify Top | Invalid | ????? | ***** | 0 | 0 | N | 0.0 |

# XfRouteBoss Table

Open the XfRouteBoss table (Configuration Menu | Transfer Utilities | Transfer Scan Processing).  Verify the settings created by the wizard:

| XfRouteBoss Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | LogLevel | ScanZone | CrrMsgTable | SendAvailStatus | SystemRun | RunNewXfr |
| RB01 | Level 1 | SZ01 | MsgCR1 | N | PA1Run | Y |
| RB02 | Level 1 | SZ02 | None | N | PA1Run | Y |

Entries will also have been created automatically for NoScanTmr, NoRouteTmr, NoCartonTmr and ClrQueDlyTmr.

Force PA1Run ON.

# XfRouteTable Table

Open the XfRouteTable table (Configuration Menu | Transfer Utilities | Transfer Destination Config).  Verify the settings created by the wizard:

| XfRouteTable Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Enabled | BossRecord | HostRouteStr | Route | NewRoute |
| R01-RB01-Tr01-X01 | Checked | RB01 | D01 | N/A | Tr01-X01 |
| R02-RB02-Tr02-X02 | Checked | RB02 | D02 | N/A | Tr02-X02 |

# XfrBoss Table

Open the XfrBoss table (Configuration Menu | Transfer Utilities | Transfer Boss).  Verify the settings created by the wizard:

| XfrBoss Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | LogLevel | MaxWaitTM | ProductINPUT | ProductSENSE | Machine |
| X01 | Level 1 | tm_CP2_X01MaxWait | TransferPresence1 | Is On | CP2 |
| X02 | Level 1 | tm_CP2_X02MaxWait | TransferPresence1 | Is On | CP2 |

# XfrTree Table

Open the XfrTree table (Configuration Menu | Transfer Utilities | Transfer Tree).  Verify the settings created by the wizard:

| XfrTree Table |  |  |  |
| --- | --- | --- | --- |
| Name | Boss | Dest | DoConfirm |
| Tr01-X01 | X01 | D01 | Y |
| TR02-X02 | X02 | D02 | Y |

# XfrConfig Table

Open the XfrConfig table (Configuration Menu | Transfer Utilities | Transfer Config).  Verify the settings created by the wizard:

| XfrConfig Table |  |  |
| --- | --- | --- |
| Name | pStatus | pState |
| C01-X01 | Idle | Idle |
| C02-X02 | Idle | Idle |

Note that if the Batches entries are activated, the values in pScrnUpdateTmr in the last column will be changing.

# XfrCfgDelayTmr Table

Open the XfrCfgDelayTmr table (Configuration Menu | Transfer Utilities | Transfer Delay Timers).  Verify the settings created by the wizard:

| XfrCfgDelayTmr Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | XfrConfig | State | Timer | TimerAction |
| Dly01-C01-State1 | C01-X01 | State1 | tmrCnf01 State1 Delay_CP2 | Run Timer |
| Dly02-C02-State1 | C02-X02 | State1 | tmCnf02 State1 Delay_CP2 | Run Timer |
| Dly03-C02-State2 | C02-X02 | State2 | tmCnf02 State2 Delay_CP2 | Run Timer |
| Dly04-C02-State3 | C02-X02 | State3 | tmCnf02 State3 Delay_CP2 | Run Timer |
| Dly05-C02-State4 | C02-X02 | State4 | tmCnf02 State4 Delay_CP2 | Run Timer |

NOTE:  Set the timers to longer intervals – at least 5 seconds – to provide extra time to view activities during testing.

# XfrCheckSet Table

Open the XfrCheckSet table (Configuration Menu | Transfer Utilities | Transfer Check Set Recs).  Verify the settings created by the wizard:

| XfrCheckSet Table |  |  |
| --- | --- | --- |
| Name | Tree | XfrConfig |
| Ck01-C01-Tr01-X01 | Tr01-X01 | C01-X01 |
| Ck02-C02-Tr02-X02 | Tr02-X02 | C02-X02 |

# XfrGoSet Table

Open the XfrGoSet table (Configuration Menu | Transfer Utilities | Transfer Go Set Recs).  Verify the settings created by the wizard:

| XfrGoSet Table |  |  |  |
| --- | --- | --- | --- |
| Name | Tree | XfrConfig | InitialState |
| Go01-C01-Tr01-X01 | Tr01-X01 | C01-X01 | Idle |
| Go02-C02-Tr02-X02 | Tr02-X02 | C02-X02 | Idle |

# PrintApplyLine Table

Open the PrintApplyLine table (Configuration Menu | PrintApply Utilities | Print Apply Line).  Create the following entry:

| PrintApplyLine Table, Part 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | AssignType | TestMode | SendType | LogLevel | ControlMachine | NumPrinters |
| PA1 | Static Assign | Test Mode OFF | Scan - Lbl Snd | Level 2 | CP2 | 1 |

| PrintApplyLine Table, Part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ScanErr SendLRM | TampClr SendLVM | LrmMsgTable | LvmMsgTable | RcvLabelTable | SndLabelQueue |
| PA1 | Unchecked | Unchecked | MsgCR1 | MsgCR2 | RcvLabels1 | SndLabelQue1 |

Note:  Go back to table ScnScanZone and complete the PrintLine entry there referring to this new record.

# XfrTrig Table

Open the XfrTrig table (Configuration Menu | Transfer Utilities | Transfer DCM Trigger).  Verify the setting created by the wizard:

| XfrTrig Table |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Enable | XfrConfig | State | ProcessType | CrrXfrBoss | NextXfrRoute | CopyParmData | PNALine |
| Trg01-C01-State2 | Y | C01-X01 | State2 | NextXfrTrig | N/A | R02-RB02-Tr02-X02 | Y | PA1 |

NOTE:  When the first transfer is triggered, it will activate the second transfer.

# PrinterBoss Table

Open the PrinterBoss table (Configuration Menu | PrintApply Utilities | Printer Configuration).  Create the following entry:

| PrinterBoss Table, Part 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | AvalAssign | Enabled | PrintLine | PrinterMachine | ApplType | VerifyScans | TrailEdge |
| PA1Boss | Checked | Checked | PA1 | P1CL | Top | N | N |

| PrinterBoss Table, Part 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | PrinterErrorIn | TamperHomeIn | LabelOnTampIn | CycleCompleteIn | PrinterResetOut | TamperSolOut |
| PA1Boss | P1ErrIn | P1TampHome | P1LOTAR | P1CycleComplete | P1ResetOut | P1TampSolOut |

| PrinterBoss Table, Part 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | PrinterErrLight | PrintBelt | TampPulseTimer | TampCycleTimer | TampRetractTimer | ErrorResetTimer | MinLength |
| PA1Boss | Line1ErrLight | PA1Blt |  |  |  |  | 2 |

TRK: We need a PrintBelt entry – or do we?  This is a transfer.  Hmm…What about those timers in part 3???

# XfrCfgPnAInput Table

Open the XfrCfgPnAInput table (Configuration Menu | Transfer Utilities | Transfer PnA Input).  Verify the setting created by the wizard:

| XfrCfgPnAInput Table, Part 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | State | PrintLine | Printer | LabelNum | PrintCondition |
| Line 1 Wait | C02-X02 | State2 | PA1 | PA1Boss | Label 1 | Receive Label (LRMA |
| Line 1 Send LVM | C02-X02 | State4 | PA1 | PA1Boss | Label 1 | Label Verification |

| XfrCfgPnAInput Table, Part 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | VerifyScanZone | LogLevel | Machine | OrSignal1 | OrSense1 |
| Line 1 Wait | N/A | Level 1 | CP2 | INVALID | N/A |
| Line 1 Send LVM | SZ02 | Level 1 | CP2 | Line 1VerifyPresence | Is On |

# XfrCfgPnAOutput Table

Open the XfrCfgPnAOutput table (Configuration Menu | Transfer Utilities | Transfer PnA Output).  Verify the setting created by the wizard:

| XfrCfgPnAOutput Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | XfrConfig | State | PrintLine | PrintAction | LogLevel | Machine |
| Line 1 LRM Send | C02-X02 | State1 | PA1 | Get Label (LRM) | Level 1 | CP2 |
| Line 1 Apply Label | C02-X02 | State3 | PA1 | Activate Printers (T | Level 1 | CP2 |

# PAScannerBoss Table

Open the PAScannerBoss table (Configuration Menu | PrintApply Utilities | Scanner Config).  Create the following entries:

NOTE:  These entries are required to link the scanners to the PrintApplyLine!

| PAScannerBoss Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | PrintLine | SendLVM | VerifyType | ScanDevice |
| PA1 LPN1 Scan | PA1 | NO LVM Msg | N/A | PNA1 LPN Scan |
| PA1 P1 Verify | PA1 | Send LVM Msg | Verify All | PNA1 P1 Verify |

# PAPrinterScanAssign Table

Open the PAPrinterScanAssign table (Configuration Menu | PrintApply Utilities | Printer Scan Assign).  Create the following entry:

| PAPrinterScanAssign Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | LabelNum | PAScanRecord |
| PA1 P1 Verify Scan | PA1Boss | Label 1 | PA1 P1 Verify |

# TestConfig Table

Open the TestConfig table (Configuration Menu | PrintApply Utilities | Test Config Line).  Create the following entry:

| TestConfig Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | PrintLine | VerifyStrSKU | VerifyStrRF | ExtLabelType1 | ExtLabelType2 | RtnStatusMsg |
| PA1 Ship | PA1 |  |  | Ship |  |  |

# TestCnfgPrt Table

Open the TestCnfgPrt table (Configuration Menu | PrintApply Utilities | Test Config Printer).  Create the following entry:

| TestCnfgPrt Table |  |  |  |
| --- | --- | --- | --- |
| Name | Printer | VerifyStrL1 | VerifyStrL2 |
| PA1 P1 | PA1Boss | 123456 |  |

# LabelTemplates Table

Open the LabelTemplates table (Configuration Menu | PrintApply Utilities | Label Configuration).  Create the following entries:

| LabelTemplates Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Printer | LabelNum | LabelType | LoadType | ExtLabelType | VerifyStr | LabelFileName |
| PA1 P1 No Read | PA1Boss | Label 1 | No Read | File Load | NoRead | NoRead | LABELS/NoRead.txt |
| PA1 P1 Clear | PA1Boss | Label 1 | Clear Printer | File Load | Clear |  | LABELS/Clear.txt |
| PA1 P1 Test Label | PA1Boss | Label 1 | Test Label | File Load | TestLabel | Test | LABELS/Test.txt |
| PA1 P1 Std | PA1Boss | Label 1 | Standard Label | N/A | Ship |  |  |

NOTE:  Create a directory named LABELS in /SortPlus/RUN and add the three text files as described in the table.  Each should contain a string that describes its function.  For example, NoRead.txt should contain something like “NoRead Label Printed”.  When these files are sent to the simulated printer, the string will be written to the log file.

# SendTstCRR Table

Open the SendTstCRR table (Configuration Menu | Sorter Utilities | Sorter Messages | Config CRR Test Message).  Create the following entry:

| SendTstCRR Table |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | Status | Z1 | Z2 | Z3 | Z4 | Z5 |
| TestCRR |  | D01 |  |  |  |  |

# Printer Scan Test

Now, let’s use the configuration to simulate some activity:

- Open the MsgCR1 table (Configuration Menu | Sorter Utilities | Sorter Messages | CR1 Message Table) and make sure the table is empty.  You can delete rows using the Alt-D key.  Repeat this for the MsgCR2 table.

- Open the ConfigXfgScrn screen (Configuration Menu | Transfer Utilities | Configure Transfers – Bottom of the list).  Select ScanZone “SZ01”.

- The PrsPE field should display “TransferPresence1”.  Click on the field to select it and press the Delete key to force the signal OFF.  It will show as red text with a yellow background.  Click the button by ResetXfr.  The Status field will show “Xfr Boss: X01, has reset”.  XfrStatus will show Idle.

- Force “TransferPresence1” ON by pressing the Insert key.  It will now show as green text with a yellow background.  XfrStatus will change to “Idle – Waiting for Routing”.

- Open the ScnScanDevice table (Configuration Menu | Scanner Utilities | Scan Device Config).  In row 1 (PNA1 LPN Scan) enter “12345” in the RcvData field.  In the RxStat field, use the pull-down list to select Recvd.  Click Apply.  The string will be copied to the ParsedData field and ParsedStatus will be set to Good Scan.  Click Save.

- Click StartXfr.  Transfer X01 will run.  XfrStatus will change to Done to indicate it ran, then back to Idle.  Status will show “Xfr Boss: X01, is Done”.
Note:  Set the timers longer if you need more time to see this occur.

- Verify the MsgCR1 table contains the CRR Message, and the routing to D01.
TRK:  I see the outbound scan in the logs, but no destination is set in the response.
2022/08/05 09:40:55.482014|M01|SERV|L01|tcp_recv: LRMRW(12): Len: 357, Recv: >>^BCRR|CP2|iQty|20|sTableName|zcContainer|sContainer|12345|sFunction|Read|sStatus|Okay|sStatusMsg| |sScanner|SZ01|sTrackTable|325|sTrackIndx|59|sScaleData| |sToteID| |sZone1| |sZone2| |sZone3| |sZone4| |sZone5| |sZone6| |sZone7| |sZone8| |sZone9| |sZone10| |Param1| |Param2| |Param3| |Param4| |Param5| |sHeight|  0.00|sWidth|  0.00|sLength|  0.00| | | | | | ^C<<
2022/08/05 09:41:13.536321|M01|SERV|L00|XfDivertConfirmMsg: DcmMsgTable not selected in Transfer Configuration
2022/08/05 09:41:13.536321|M01|SERV|L01|XfRouteProcessData: Cont:  , Confirmed:
2022/08/05 09:41:43.575841|M01|SERV|L00|XfrBarCodeProcessingBasedScanType: Undefined scan type: 12
2022/08/05 09:41:43.575841|M01|SERV|L01|XfGetBarCodeErrRoute: Scan Error Route, Not Selected, RouteBoss: RB02

- Verify XfrTrack contains a row (should be prior to the row with the ------> last updated indicator), that has the D01 Routing assignment.
TRK: I see assignment to D02.

- -------------------------------

- Select the ScanZone “SZ02”.

- PrsPE should be “TransferPresence1”

- Select the field, and press the delete key to force the signal OFF

- Press the insert key to force the signal ON.

- Open the ScnScanDevice table again (Configuration Menu | Scanner Utilities | Scan Device Config).

- Click on the RxStat row 1 column and select “Recvd”. Press enter and click “Save”.

- The Transfer X01 should RUN, Complete and trigger X02, the display will only show that X02 has RUN.
TRK: I had to click StartXfr here, too.

- X02 should RUN until the Reset Timer times out, then display the message X02 has reset.

- Verify the LRM Message was created in the MsgCR1 Table.  There should be 2 messages since the Transfer was run 2 times.
TRK: Nope.  All CRR messages in MsgCR1!

- ---------------------------------------------

- Open the Scanner Utilities/ Scan Device Config and in row 2 enter a RcvData string of “123456”, click apply.

- Open the menu ConfigXfgScrn, Scan Zone SZ02 should display the PrsPE as “Transfer Presence 1”

- Select the field, and press the delete key to force the signal OFF

- Select the PrsPE “TransferPresence1” and press the insert key to force the signal ON.

- Open the Scanner Utilities/ Scan Device Config

- Click on the RxStat row 2 column and select “Recvd”. Press enter and click “Save”.

- This will start the transfer.
TRK: Again, StartXfr was needed.

TRK:  I see no PNA messages, nothing is set up for PNA tamping, etc.  It seems this document was never completed!