Zebra Print Engine Automatic Disconnection

The Zebra Print Engine has a feature to automatically disconnect a TCP/IP connection after 5 minutes.  To disable this feature use a web browser to connect to the printer’s ip address.

Click View and Modify Printer Settings.  If you get a login screen, Zebra’s default password is 1234.

Click Network Configuration.

Click TCP/IP Settings.  Set Timeout Checking to NO.  Click Submit Changes.

Click “Save Current Configuration” from the “View and Modify Printer Settings” page to make the change permanent!