# FortnaPlus Control Zebra Printer Configuration

Document Revision: 1.2

Date: 23-Jul-2021

| Revision | Date | Author | Revision Summary |
| --- | --- | --- | --- |
| 1.0 | 26-Mar-2013 | P Rotter | New document |
| 1.1 | 18-Jun-2020 | T R Kremser | Updates and general refresh |
| 1.2 | 23-Jul-2021 | T R Kremser | Format refresh |

Table of Contents

1.	Configuring a Zebra 110PAX4 Printer for Diagnostic Reporting	4

2.	Configuring FortnaPlus as the Diagnostic Server	4

3.	Configuring the Printer as a Diagnostic Client	4

4.	Using the Diagnostic Messages	5

Confidential/Proprietary Statement

This document contains trade secrets and information that is considered confidential and proprietary to Fortna, Inc.  It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document.  Fortna Material Handling reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

# Configuring a Zebra 110PAX4 Printer for Diagnostic Reporting

The Zebra 110PAX4 printer features the ability to enable diagnostic messaging from the printer to a TCP/IP server.  This document describes how to configure a printer for diagnostic reporting and to configure FortnaPlus as the server for the diagnostic messages.

# Configuring FortnaPlus as the Diagnostic Server

Configure a server-style connection in table Machine.

- Configure a listener entry:

  - Use Connection_Type = TCPSERV_LISTEN

  - Use Communication_Protocol = HOST_ALPHA

  - Choose an unused Machine_Port value.  Suggested port:  4000

  - Use Process = SRV1

- Configure a reader-writer entry: machine in the FortnaPlus Machine table:

  - Use Connection_Type = TCPSERVER_RDRW

  - Use Communication_Protocol = HOST_ALPHA

  - Use the same Machine_Port value chosen for the TCPSERV_LISTEN entry.

  - Use Process = SRV1

- Set LogLevel for both entries to Level 5

- Restart FortnaPlus

# Configuring the Printer as a Diagnostic Client

The printer must be configured to send diagnostic information.

- From the QNX workstation, open the BonEcho web browser

- Enter the printer’s IP Address in the URL box.

- Click Alert Setup

- Click Add Alert

- Configure a diagnostic entry with the following parameters:

  - Condition: 	All Messages

  - Destination: 	TCP

  - SET:		YES

  - CLR:		YES

  - Address:	Enter the QNX node IP Address

  - Port:		Use the port value configured in Machine_Port, as described above.  (4000)

  - Password:	Enter printer password

  - Click Add Alert Message

# Using the Diagnostic Messages

The printer diagnostic messages will be written to the FortnaPlus SRV1 log file.  Search the log file for “ALERT” to find the relevant messages
 (grep ALERT <logfile>)