# Fortna

# West Reading, PA

# Ink Jet - Internal Design Document

Document Version: 0.4

Date: 12/17/2012

This page intentionally left blank.

Table of Contents

1	Introduction	1

1.1	Document Objective	1

1.2	References	1

1.3	Document Conventions	1

1.3.1	Standard Definitions	1

1.3.2	Acronyms and Abbreviations	1

1.3.3	Requirements Numbering	1

2	Project Overview	2

2.1	Conveyor System Layout	2

2.1.1	Typical PnA Conveyor Layout	2

2.1.2	Recommended Ink Jet Conveyor Layout	2

2.2	High Level Summary	3

3	Detailed Design	4

3.1	Scanner to Second PE Distance	4

3.2	Update Point to Printer Distance	4

3.3	Gapping	4

3.4	Pitch	4

4	Ink Jet Printer Requirements	5

4.1	Print Triggering	5

4.2	Label Communication	5

4.3	Label Queue Flushing	5

4.3.1	Automation Controller Message	5

4.3.2	Printer User Interface	5

4.3.3	I/O Signal	5

4.3.4	Timer	6

4.3.5	Queue Size of 1	6

4.4	Error IO	6

4.5	Printing Distance	6

5	Error Conditions	7

5.1	Box Removal	7

5.2	Bad Gap	8

6	Messaging	9

7	VideoJet 2120 Interface	10

7.1	High Level Description	10

7.2	Label Communication	10

7.3	Label Flushing	11

7.3.1	None	11

7.3.2	Clear Print Queue I/O	11

7.3.3	Clear Print Command	12

7.4	Print Activation	12

7.4.1	Printer Photo-eye	12

7.4.2	Software Control	13

7.5	Printer Error Detection	13

7.6	Label Ready Detection	13

7.7	Videojet Questions and Answers	13

# Introduction

This Internal Design Document (IDD) contains the software design details and Ink Jet requirements for using an Ink Jet Printer.  It provides the low level design for Conveyor layout, Automation Controller configuration, operation, and messaging.

## Document Objective

The purpose of this document is to serve as a vehicle for reviewing the design of an Ink Jet based PnA system and using the standard PnA functionally without software changes.

The intended audience for this document is predominantly the software development team, the Controls team, Hardware and Electrical groups.  This is an internal document and is not intended to be distributed to the customer.

## References

These requirements were gathered through meetings and other communications methods with Client personnel. The final SDR was conducted <location, date> with the following attendance:

Fortna:

Client:

WMS:

Vendor: 	[1] Videojet 2120 Service Manual Revision AA, December 2010

[2] Zipher Text Communication Protocol.

Shawn Bert and John Revecki of Videojet technical support.

## Document Conventions

### Standard Definitions

### Acronyms and Abbreviations

PnA – Print and Apply

### Requirements Numbering

# Project Overview

Several customers have requested the ability to print a human readable (no barcode) Zone number on each carton.  The zone number allows associates to sort containers for cart building and put away.

Designing a reliable system using an Ink Jet Printer has challenges over a standard PnA system.  A regular PnA system validates the label to ensure each container is track properly.  The human readable zone number will not be validated.  This could create issues especial if the cartons become out of sync.

## Conveyor System Layout

### Typical PnA Conveyor Layout

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Label Printer

Verification Scanner

### Recommended Ink Jet Conveyor Layout

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Ink Jet Printer

Induction Photo-eye

Update

Point

Ink Jet

Printer PE

Note: Swap of Servo Gapper section and Skewed Conveyor.  Induction Photo-eyes are angled.

## High Level Summary

The goal of the conveyor layout is to guarantee good justification for the Ink Jet Printer and that only one print job is sent to the printer at a time.

The container is gapped at the Servo Gapper and is then justified to the side with the Ink Jet Printer.  The Servo Gapper and the skewed rollers are swapped to prevent skewing when containers back up before the Servo Gapper and to correct the skew introduced by the container going over the Servo Gapper.

The PnA conveyor will be split into two logical Shifters in the Automation Controller software.  Physically a single belt can be used.  The first Shifter will be from the first Induction Photo-eye to the second Induction Photo-eye.  The second Shifter will be from the second Induction Photo-eye to the end of the PnA belt.

The container passes the first Induction Photo-eye and the Automation Controller software will start tracking the container.  The scanner will scan the LPN or SKU of the container and send a LRM (Label Request Message) to Level 2.  The LRM message response will inform the Automation Controller if the container requires printing.  If printing is required an LMI (Label Image Message) message will also be received containing the data to be sent to the printer.  The LPN and the printer data will be held in the tracking information by the Automation Controller software.

The container will cross the second Induction Photo-eye and pick up the tracking from the first Shifter.  The container will cross the logical update point and transmit the data to the Ink Jet Printer.

The container will cross the Ink Jet Printer’s photo-eye and the printer will print the Zone number on the container as it passes.

Two significant differences between a standard Print and Apply Line and an Inkjet Print and Apply Line are

- The need to either not send a LVM message or always send a successful LVM message.

- The need to purge the printer’s buffer between cartons.

# Detailed Design

This section provides the detailed designed consideration for the Conveyor

## Scanner to Second PE Distance

The recommended distance between the Scanner and the second Photo-eye is based on the length of the box and the time required for processing a scan.  Typically a Scanner will send data to the Automation Controller when the box tails out of the Scanner’s Photo-eye.  About 4 seconds should be given to process the scanner data.  The distance from the Scanner to the second PE should = (4 seconds * the conveyor speed) + the length of the longest box.

## Update Point to Printer Distance

To prevent out of sync issues a small distance between the Automation Controller update point and the Printer is desirable.  The distance should be based on the worst case time to transmit the Printer data plus the worst case time to process the printer data.  The total time should be multiplied by the conveyor speed.

The worst case processing time should be specified by the printer manufacturer and 200 ms should be added for Automation Controller processing.

## Gapping

The minimum gap between boxes should be the distance between the second Induction Photo-eye and the Printer Photo-eye.  The goal is to only send one print job to the printer at a time.  To avoid errors it is recommended the gap be set larger than the longest container.

## Pitch

It is critical that the box not stop in front of the printer.  Unless the Printer has its own encoder it will not be aware the belt has stopped.  Downstream condition may cause the belt to stop.  It is recommended the minimum pitch be set so the box is stopped before the second Induction Photo-eye.  Once the box crosses the second Induction Photo-eye the PnA line will send the Printer information.  If the box is then removed the line could become out of sync.  Selecting an Ink Jet Printer that can clear its print queue can help mitigate this issue.

If the container is stopped in front of the Printer, it is assumed the Zone number will not be readable.

# Ink Jet Printer Requirements

This section details requirements for Ink Jet Printers.

## Print Triggering

The Ink Jet Printer must have either its own Photo-eye to detect the edge of the box or it must have an IO input to trigger printing.  If an IO input is provided, the Controls group can use the Tamp IO in the PrinterBoss to trigger the printing.

## Label Communication

The Ink Jet Printer must have a documented label format and communication standard.  Use of another software package to transmit the printer data will cause issues.  Normally the printer data is generated by the WM.  Level 2 will request the printer data from the WM and sent the printer data to the Automation Controller.  In this case, the Automation Controller will send the printer data just before the container reaches the printer so only one print job is being processed by the printer at a time.

The printer must support communication through a serial connection or TCP/IP Ethernet socket.

## Label Queue Flushing

In the event of an error it is possible that more than one print job could be contained in the Printer’s queue.  The Printer should have the ability to flush its queue.

### Automation Controller Message

The Automation Controller contains a file that can be sent to the printer when errors are cleared.  This message will clear a Printer’s queue.  It is recommended the printer support clearing its queue by a message that is delivered in the same manner as a printer data.

### Printer User Interface

The Printer’s user interface should have the ability to clear the Printer’s queue.

### I/O Signal

The Printer could provide an I/O input to clear the Printer’s queue.  This requires an Automation Controller software change.

### Timer

The Ink Jet Printer could provide a timer for automatically clearing the printer’s queue.  If print data is received and the printer is not activated for printing within the time period, the printer’s queue would be cleared.

### Queue Size of 1

The Ink Jet Printer could provide a queue size of 1.  If printer data was already in the buffer and the printer received another print job then the first print job would be deleted.

## Error IO

The Printer must support signaling an error through an IO signal.  The error IO signal should be trigger whenever the printer is incapable of printing.  (Out of ink, door open, power off, etc.)  This allows the Automation Controller software to stop the conveyor and not continue to send Containers.

## Printing Distance

The printing distance tolerance should be specified by the Printer manufacturer and should be communicated to the customer.  Containers are not always square and Fortna should specify to the customer when the Zone number will not be readable.

# Error Conditions

This section describes error conditions and recommendation on mitigating issues.  The most serious issue is printing out of sync.  Because there is no verification scanner, an out of sync issue could go on indefinitely.

Because associates will stock locations based on the Zone number, an error may require the customer to inventory the entire warehouse.

## Box Removal

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Ink Jet Printer

Induction Photo-eye

Update

Point

Ink Jet

Printer PE

Box 2

Box 1

In the event of a Jam or other issue a box may need to be removed from the conveyor.  The standard procedure is to clear the boxes from the scanner to the printer and restart the line.  When the errors are cleared from the Automation Controller, the Automation Controller will delete the label and tracking information for all the boxes with printer data.  In the event an Associate doesn’t follow this procedure and removes Box 1 from the line, Box 1 will not cross the second Induction Photo-eye and the data will not be transferred to the second Shifter.  No data will then be sent to the printer and the system will operate correctly.

If Box 1 crosses the second Induction Photo-eye, Box 1 is removed, and the Printer buffer is not cleared then Box 2 will get the Zone number for Box 1.  The next container will then also be out of sync and this will continue until the Printer’s buffer is cleared.

## Bad Gap

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Ink Jet Printer

Induction Photo-eye

Update

Point

Ink Jet

Printer PE

Box 2

Box 1

Box 3

Issues with Bad Gap can be avoided in the following scenarios.  In the event 3 boxes are properly inducted with a gap greater than the length of the maximum size box and the first box jams, the first box will no longer be married to the tracking record from the first Shifter when it crosses the second Induction Photo-eye.  The tracking data will be dropped and the printer data for the first box will be lost.  As the image of the second box hits the dump point the combine boxes may get the second box’s printer data.  The Software group will need to test if the second box printer data gets sent.  The third box should receive the correct data.

If the boxes are uniform in size, the Longest Package can be set in the Sorter Inpoints menu.  A software change would be required to have the PnA subsystem ignore boxes with bad gap.

If the pitch is set to less than the length of the longest container, the second Induction Photo-eye may still marry the data to the first and second box.  If the Printer only sees one box then the Printer will only trigger once and box 3 will get the second boxes’ Zone number.  At this point the printer will be out of sync.

# Messaging

Below is the message sequence diagram for the Hardware layout in Section 2.1.2.  This is based on the standard Print and Apply code in the Automation Controller software.

Scanner

Printer

Automation

Controller

Level II

Scan Data

LRM Message

LRM ACK

LMI Message

Box crosses Label Update Pt.

Label Data

# VideoJet 2120 Interface

This section provides a design based on the VideoJet 2120 Inkjet Interface.

## High Level Description

The VideoJet 2120 Inkjet printer can be interfaced with using a serial connection or TCP/IP; and also provides 6 configurable I/O input pins and 3 configurable output pins.  A USB port is provided to load the printer configuration and templates.

In order to support the printer, the printer provides web based interface the can be used to controller the printer.  The web based interface should be useful to the support group.

Printer setup, configuration, and template creation is provided by the CLARiSOFT software and can be saved on a USB drive.  The USB drive is then used to load the configuration and templates on the printer.

The label data from Level 2 will consist of a Zipher Text Communication Protocol command with the data.  The Automation Controller software will forward the printer command to the Videojet printer unchanged.  The use of ZPL (Zebra Printer Language) can also be investigated.  The printer has a ZPL compatibility mode however Videojet technical support was unable to provide any information about what version was supported or if the entire command structure was implemented.

To prevent out of sync issues a method of flushing the printer’s queue will be used.

## Label Communication

The printer command containing the data will be sent to the Automation Controller software in the standard LMI message.  The printer command will be a Zipher Text command.  Templates are referred to as “jobs” in the documentation.  There are several commands to send fields to the printer, but the SEL or SLA are the recommended commands.

The SEL and SLA command is documented on page 8 reference [2].  The SEL command is formatted as

SEL|<jobname>|<data>|<next data>|….|<CR> where <CR> is ASCII byte 0x0D.

The jobname is the name of the template.  The data fields start at the named fields VarField00, VarField01 …

If the named fields in the template do not use the pattern VarField## then the SLA command should be used.  The SLA command is formatted as

SLA|<jobname>|<fieldname>=<data>|<next fieldname>=<next data>|…|<CR>

Both of these commands will return ACK<CR> or ERR<CR> to the Automation Controller software.  The status returned will be ignored by the Automation Controller software.

Note: It is suggested that the ZPL emulation be investigated if the system is prototyped.

## Label Flushing

This section contains options for preventing out of sync issues and misprinting cartons.

### None

Under this option no Label Flushing would occur.  The VideoJet 2120 controller has an option to limit the number of labels in the queue.  The MaxQueueLength value would be set to 1.  One issue with this is it does not prevent the incorrect data from being printed on a carton.  If a carton is removed after the second induction photo-eye that cartons data will remain in the printer’s queue.  When the printer data for the next box is sent the new printer data will be dropped and not the old printer data.  The old data will be printed on the new box.

### Clear Print Queue I/O

The Clear Print Queue I/O is an I/O input on the Videojet 2120 controller that clears the printer’s queue.  A new feature would be added to create an update point that activates the Clear Print Queue I/O just before sending the label data.

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Ink Jet Printer

Induction Photo-eye

Clear Queue Update Point

Ink Jet

Printer PE

Box 2

Box 1

Send Label Update Point

Under the Clear Print Queue I/O the MaxQueueLength would still be set to 1.

### Clear Print Command

The Zipher Text Communication Protocol version 4 or greater supports a Clear Queue Item Command (CQI).  This command can removes print items from the printer’s queue.  This command would be added to a label file on the Automation Controller.  An update point would be created to send the CQI command prior to sending the label data.  This may require changes to the Automation Controller software or Level II.  Currently the Automation Controller does not send labels unless instructed to by Level II or the operator using the PrintApplyScrn menu.  The change on the Automation Controller would be to have an option to always send a label.  This option is architecturally the cleanest.  The second option is to have Level II send the clear label id as part of the LRM message.  The second option requires Level II to know what type of printer is being used.

Skewed Conveyor

Servo Gapper

Induction Photo-eye

Induction Scanner

Ink Jet Printer

Induction Photo-eye

Send Clear Queue Label Update Point

Ink Jet

Printer PE

Box 2

Box 1

Send Label Update Point

The Send Clear Queue Label Update Point is configured in the PrinterUpdPnts and LabelTemplates menu on the Automation Controller Software.  Under the Send Clear Print Queue Label the MaxQueueLength would still be set to 1.

## Print Activation

The Videojet 2120 uses an I/O input to active the printing on the carton.  There are 2 ways the I/O can be activated.

### Printer Photo-eye

The Videojet 2120 comes with a photo-eye to connect directly to the printer.  When the carton blocks the photo-eye the printer will start printing after a delay.  In this configuration only the front edge of the box can be printed unless the boxes are a consistent size.  This method is the most reliable.

### Software Control

The Automation Controller software can operate the printer’s print trigger.  The TamperSol I/O in the PrinterBoss can be used to active the printer’s print trigger.  By using the Automation Controller to trigger the printer, the data can be printed on the leading or trailing edge of the box.  The TampPulseTimer in the PrinterBoss would be set for a long enough time period to completely print the data.  One disadvantage of this method is if the box is stopped while in front of the printer then the TampPulseTimer will expire and the remaining print string will not be printed.

## Printer Error Detection

The Automation Controller has the ability to stop the Print and Apply line if a printer error is detected.  The detection occurs through and I/O input and is configured in the PrinterErrorIn field in the PrinterBoss.  The Videojet 2120 provides two outputs that can be used to indicate the printer is not functional.  The “Printer is Offline” and the “Printer Enters Fault” can be used.  It is not clear from the documentation when the “Printer Enters Fault” I/O is deactived.

## Label Ready Detection

A standard Print and Apply line will wait for a label to be printed.  The Videojet 2120 can provide an I/O signal to indicate the printer is finish rendering the label.  The I/O signal “New Job Allocation Rendered” can be used in the LabelOnTampIn field in the PrinterBoss.  Note: It is not clear what deactivates this input from the documentation.

## Videojet Questions and Answers

The answer to these questions was provided by John Revecki and Shawn Bert of Videojet technical support.  The page numbers are from the Videojet 2120 Service Manual Revision AA, December 2010.

- Do we need to use the Zipher Text Communication Protocol?

  - Yes the printer must use the Zipher Text Communication Protocol.

- Page 4-6.  Does the Pin 1 - AUX Power Out provide the power state of the printer?

  - Yes.

- Page 4-6.  What faults do pins 7 and 8 provide – Fault O/P RLY (COM) and (N/O).

  - Inputs are

  - Outputs are

- Page 4-6.  What is the warning provided by pin 9.

  - See output list.

- Page 4-7.  What are the standard range of CLARiTY outputs.

  - See output list.

- Page 4-8.  What does the clear print queue exactly do?  Does it clear templates?  It does wipe out the template.

  - No.

- Page 5-11.  OutputConfiguration – What are the available outputs.

  - See outputs.

- Page 5-13.  Can the UpdateQueue – MaxQueueLength be used to limit the number of print jobs to 1?

  - Yes but doesn’t wipe out previous print job.  Prevents the new one from being queueing.

- Page 5-14.  Does Usage control the printer protocol?  Can it be used over TCP/IP?

  - Yes

- Does Text Communication allow us to just print character?

  - No it uses Zipher Text Communications Protocol.

- Is ZPL the same ZPL as used by the Zebra print engine?

  - Yes.

- What set of ZPL is supported?

  - Don’t know.  Not documented.

- Text Communication used the Zipher Text Communication Protocol.

- What does ZPL Emulation do?

  - Talk to like a Zebra printer.  Unknown version.

- What does the Transmitted Field do?

  - Send back the field that was printed.  Only output.  You can use another COM port for the job data.

- Page 5-15.  Explain PrintMode.

  - Sets the mode for downloading templates.

- Page 5-15.  What does the Add WebServer Enabled do?

  - Provides a remote interface to the panel.

- Page 6-13.  Can the CIFF file be sent directly to the printer or is a protocol needed?

  - Doesn’t know.  Usually use ClariSOFT or USB drive.