# Print and Apply Design

This document contains trade secrets and information which is considered confidential and proprietary to Fortna Inc. It may not be duplicated, used, or disclosed outside the original recipient organization, and it may not be duplicated, in whole or in part, for any purpose other than to evaluate this document. Fortna Inc. reserves the right to modify this document as necessary to reflect any terms and conditions negotiated and included as part of a subsequent contract or purchase order.

Bar Code

Scanner

Verification

Scanner

Print and Apply

Labeler

Print and Apply  Belt

Skew Section

Brake/Meter Belt

Tracking PE  Induct Point

Scanner Tail Out Point

Nose of the Longest Box from Scan Tail Out

Labeler Tamp Point

Labeler End Point, Beginning of the scan tunnel

Verify Scanner Tail Out Point

Nose of the Longest Box from Verify Tail Out

Height Detection

- Printer Type – Customers typically already use either a Zebra or Sato print engine. When selecting a printer verify whether the customer needs a Sato (SPL printing language) or a Zebra (ZPL printing language) for their application. The resolution of the desk top units must also match the applicator units, if Fortna is controlling a desk top label printer (for the exception handling area).

- If a desktop Print and Apply workstation is used to reprint labels (for example at the Exception Handling Lane), the desktop unit should be the same resolution (for example 200 dpi) as the Print and Apply Applicators. Printers with different resolutions require different label formats (label generation software would have to create multiple labels for the different resolutions).

- Each Print and Apply Printer has a stand and mechanism that requires 48 in. (4 ft) of space on the belt. The Tamp mechanism is located on one side and for design purposes can be assumed to be in the worst-case location. The actual Print and Apply unit should be purchased with the applicator mounted on the downstream side (this will allow the maximum amount of time to transmit, create, and feed the label). Typically the conveyor is designed to justify the carton to one side of the conveyor, the Print and Apply unit must be located on the same side.

- Tracking Induct Point – The Tracking Induct Point is typically located (.75 * Max Carton Length)

- Height Detection Sensor – located past the Tracking PE Induct Point and prior to the Scanner Tail Out Point. The Height information is captured prior to receiving the scan information. The Height and Scan Information are sent to Level 2.

- Scanner Tail Out Point - is located at (A combination of the Length of the Scan Tunnel and the Length of the Longest Carton).

- The Nose of the Longest Box from Tail Out Point - is located by adding in the length of the longest box to the Scanner Tail Out Point.

- Labeler Tamp Point - is located (4 seconds) down stream from the “Nose of the Longest Box from Scanner Tail Out”.

- Labeler End Point, located at the beginning of the verify scan tunnel – located at the end of the 48 in. foot print of Labeler.

- Verify Scanner Tail Out Point - is located at (A combination of the Length of the Scan Tunnel and the Length of the Longest Carton).

- Nose of the Longest Box from Verify Tail Out Point – is located by adding in the length of the longest box to the Verify Scanner Tail Out Point.

Example:

- The Longest Carton is 34 in.

- The Tallest Carton is 18 in.

- The throughput required is 30 cartons per second.

- The scanner is scanning the front edge of the carton (requires a minimum gap of 18 in., use 26 to include a safety factor).

- The conveyor speed = (34 in. + 26 in. in 2 seconds, 150 ft per minute)

- Tracking Induct Point = 34 * .75 = 25 in.

- Scanner Tail Out (scan tunnel combined with longest carton) = 48 in.

- The Nose of the Longest Box from Scanner Tail Out Point = 34 in.

- Labeler Tamp Point = 30 in. sec * 4 sec = 120 in.

- Print and Apply Printer requires 48 in. of space.

- Verify Scanner Tail Out Point (scan tunnel combined with longest carton) = 48 in.

- The Nose of the Longest Box from Verify Tail Out Point = 34 in.

# Label Placement

- Top Label Placement

Top of the Carton with the Largest Width

Conveyor Side Rail (cartons are justified to this side)

Applicator is also on this side or mounted overhead.

Label

Labels are placed the same distance from the front edge and the same distance from the side closest to the guardrail. (If the carton is consistently justified to the guardrail).

Label

The label is placed on the top of the carton, justified from the side closest to the side rail. The placement of the label will vary based on the consistency of the carton justification to the side rail, and based on the speed of the carton. For example: the conveyor may be stopped for traffic or it may be stopped because of a system stop.  If the conveyor is decelerating (or slowing down) and the “tamp” is activated, the label will move forward on the box. As the box slows down, the distance traveled while the applicator travels is reduced.

- Side Label Placement

Label

Labels are placed the same distance from the front edge and the same distance from the conveyor surface. (If the carton is consistently justified to the guardrail).

Carton

Conveyor

Label

Conveyor

The label is placed on the side, justified from the bottom and the front of the carton. The placement of the label will vary based on the consistency of the carton justification to the side rail, and based on the speed of the carton. For example: if a carton is (2) inches from the side rail, the side label applicator will travel an additional (2) inches to reach the carton. The additional (2) inches of applicator travel will take additional time and the carton will travel further on the belt. The label will be placed further away from the front edge of the carton.

# Print and Apply Rate Calculation

The rate of each Print and Apply Line is constrained by several factors:

- The number of printers for each label type.

- The size of the label.

- The label feed rate of the printer.

- The amount of time needed to generate the bit image of the label and begin printing. In applications with RFID this time may vary based on the write/read cycles of the RFID encoder. In applications with Hazmat labels the time may be extended to process a much larger format file.

- The amount of time needed to send the label to the printer. In most systems the memory of the printer is large enough to store the next label and the label is transmitted to the printer before the carton arrives.

- The location of the label used to identify the carton. If the label is on the front of the carton then the gap between cartons must be large enough to read the label. The gap is set to the height of the largest carton.

- The travel distance of the applicator. The travel distance can vary significantly.

  - A top apply may travel (6 inches for clearance + 20 inches in carton height variance)

  - A side apply may travel (2 inches for clearance)

- The maximum speed of the Print and Apply Line. The maximum speed is constrained by the label placement accuracy. The slower the line the more accurate the label placement.

  - If a 6 in. label is placed on a carton with +1/2 in. accuracy, the speed is limited to 166 ft/min (constrained by the encoder pulse rate).

  - If a 6 in. label is placed on a 6.5 in. carton the accuracy is +1/4 in., the speed is limited to 84 ft/min.

  - The surface variation between different cartons (of the same height) will effect how fast the label will be removed from the applicator, this variation will increase as the speed of the carton increases. The end result is that the accuracy of label placement decreases as the speed increases.

  - Note: the maximum speed is based on 5ms update time of the SORT process and the I/O update time of 5 ms or less, with the latest computer equipment and with an I/O system of 2 ms or less the speed can be increased to 332 ft/min.

- The response time of the computer and network sending the label to the printer.

For example: A Print and Apply Line is designed with (1) printer that has a feed rate of 8 in. per second and will be printing a 6 in. label. The height variation of the cartons is 10 in. and the applicator can travel at 24 in. per second (the speed of the applicator is adjusted to a speed that does not damage the carton or its contents). The label accuracy is +1/2 in. The longest carton is 36 in. The label identifying the carton is on it side.

- The gap between cartons can be set to a minimum distance (4 in.). The gap plus the longest carton = 40 in., a carton can feed every 1.2 seconds.

- The applicator can generate a label in .25 seconds and the label will require .75 seconds to feed out onto the applicator. (This assumes that the memory in the printer is large enough to store a label, and the label has been sent by the computer to the printer before the carton arrives).

- The applicator will travel 10 in. plus 2 in. (for carton clearance). The travel time out and back 1 second.

- The applicator time plus the travel time = 2 seconds plus 10% safety factor. The line can run at 27 cartons per minute.

- If a second printer is added the line is constrained by the max rate of a carton every 1.2 seconds plus 10% or 45 cartons per minute.

# Applications where there are 2 Printers

In an application where there are 2 printers, the space between the printers should match the pitch of the cartons (this may also be multiples of the pitch as well). In addition if the print apply line must stop due to downstream traffic, the location of the stop point should also be based on the pitch of the carton.

When a belt is required to stop, the belt must come to a complete stop and the belt must restart after the stop, accelerate to full speed, before the label applicator can begin its cycle. If the belt is not at full speed when the cycle begins, the label will be placed ahead of the box, possibly missing the box.

The location of the printers is critical in preventing the start/stop events from one printer (or start/stop events from downstream traffic) from affecting the label placement of the other printer.

The printer and stop point placement does not address all cases (for example: emergency stops or system stops, may cause label placement errors) and only prevents label placement errors created by start/stop events associated with automatic control of the Print Apply Line.

Print and Apply

Labelers

Verification

Scanner

Bar Code

Scanner

Labeler 1 Wait Update Point

Distance to the Wait Update Point

Tracking Induct Point

Labeler 2 Wait Update Point

Wait Update Point for downstream traffic

Distance from Printer 1 to Printer 2

Labeler 2 Tamp Point

Labeler 1 Tamp Point

For example: There are 2 printers, the longest carton plus the gap is equal to 48 inches. The pitch between cartons would be set to 48 inches. The typical space between 2 printers is 48 inches (the footprint of an applicator is typically 48 inches).

The location of the “Wait Update Point” for downstream traffic would be set at (48 inches * 2) – 96 inches.

This configuration where the “Labeler 1 Wait Point”, “Labeler 2 Wait Point” and the “Distance to the Wait Update Point” are all either set to the pitch of the cartons or are multiples of the pitch, allow the system to start/stop the belt without causing label placement errors.

# Printers and Line Equipment

- Applicators should be purchased with beacons.

- Each Line should have a Beacon

- Each line should have the Automation Controller located close to the Applicator and Verify scanner. The operator would be able to restart and to identify errors without needing to walk more than a few steps.

# Host Communications Data Flow

PNA  (COMM) Communication Server PC

Oracle

DataBase

Carton Data

(Host Messages)

CP(1-N)

Machines

Carton Labels

(Host Files)

###### EH

Machine

PNA (DATA) Database Server PC

The Host sends carton data and label data to the PNA Communications Server (COMM). The COMM machine stores the data in the Oracle database located on the DATA PC. The Control PCs process the scanner data, sensor data, and control the conveyor and applicators.

The EH (Exception Handling PC) is optional and is used to re-print labels or resolve exceptions.